## Relay services

### Relay service provision

Where ICT provides bidirectional real-time continuous voice communication, and one or more relay services are available to the ICT users,

the ICT shall enable users with limited, and users with no, speech communication capabilities, to have the ability to communicate with voice users via each of the relay services available.

NOTE 1: "Available" means here that the ICT user has the right to use an existing relay service and this right can  be provided by an action by any internal or external means, e.g. an authority or the relay service provider or the ICT provider.

NOTE 2: The types of relay services recognized when the present document was authored are briefly described in the following paragraphs. For a comprehensive description, see ETSI ES 202 975 []

- Text relay services provide conversion between real-time text and voice. It is common that text relay services on user request only operate in one direction and let voice through in the other.

- Captioned telephony services convert speech to real-time text towards the user while letting voice through in both directions.

- Video relay services convert between sign language in video and spoken language.

- Speech-to-speech relay services convert between unclear speech and clear speech.

- Lip-speaking relay services convert from speech in a voice channel to lip-speaking in a video channel plus speech in a voice channel in one direction and lets voice through in the other direction.

- Cognitive support services provide support for memory and other cognitive functions in the voice channel in one direction and lets voice through in the other.

NOTE 3: The modality conversion provided by the relay service may require support of more media between the relay service and the user of the relay service than what the communication system supports.

NOTE 4: The relay service may act as a separate communication system with which the ICT has interoperability, or act as an integrated function of the ICT

NOTE 5: The ICT and any communication client may support and use more media in the communication than what is used for the voice communication.

### C.13.1.1 Relay service provision

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client C1 that supports continuous bidirectional voice communication, and is specified for use with a relay service. 2 The specified relay service is available. 3 Another communication client C2 that supports continuous bidirectional voice communication with the ICT is available. 4 If multiparty voice communications are supported by ICT, a third communication client C3 that supports continuous bidirectional voice communication with the ICT is available. |
| Procedure | If preconditions 1-3 are met: A communication session is established from communication client C1 that is specified for use with relay services to communication client C2 with an indication that relay service is wanted in the communication.. Check 1: The specified relay service enables users who specified the need for the relay service and speech users to interact by providing the requested conversion between the used modes of communication. If preconditions 1-4 are met: A communication session is established involving communication client 1 that is specified for use with relay services to communication clients 2 and 3. Check 2: The specified relay service enables users who specified the need for the relay service and speech users to interact by providing the requested conversion between the used modes of communication. |
| Result | Pass: When preconditions 1-3 are met, check 1 is true Pass: When preconditions 1-4 are met, check 2 is true Fail: Any check is false. Not applicable: At least one of 1,2 and 3 preconditions is not met Note: If the communication system is specified to connect to a number of relay services, the test needs to be repeated separately for each of the relay services. |

### 13.1.2 One step connection of outgoing communications

Where ICT is or contains a communication system providing accessibility to voice communication by supporting relay service use,
invocation of a relay service in an outgoing communication establishment from the relay service user of the ICT shall be supported to be done in a single user operation including providing the identification (number or address) of the communication destination.

#### C.13.1.2 One step connection of outgoing communications

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client C1 that support continuous bidirectional voice communication, and is specified for use with a relay service. - The specified relay service is available. - Another communication client C2 that supports continuous bi-directional voice communication is available. |
| Procedure | Initiation: A communication session is initiated from communication client 1 that is specified for use with relay services to communication client 2. Check 1: The specified relay service is invoked Check 2: A connection is established between C1 and C2 supported by the relay service |
| Result | Pass: : Check 1 and 2 are true Fail: At least one Check is not true. Not applicable: At least one precondition is not met Note: If the communication system is specified to connect to a number of relay services, the test needs to be repeated separately for each of the relay services. |

### 13.1.3	One step connection of incoming communications

Where ICT is or contains a communications system providing accessibility to voice communication by supporting relay service use,
invocation of a relay service in an incoming communication establishment to the user of a relay service shall be supported to be done in a single user operation, where the communications user who wants to reach the user of a relay service makes use of an identifier (number or address) referring to the user of the relay service.

NOTE 1: The relay service identification can be configured in the ICT of the user or the ICT service to make the invocation easy.

NOTE 2: This requirement is intended to provide a more accessible alternative to a traditional way to invoke relay services by first calling the relay service and there ask to be connected to the user of the relay service. One step incoming calling makes it possible for the user of the relay service to be called back in case of no connection on outgoing communications and to leave a simple phone number or address to be called by.

#### C.13.1.3	One step connection of incoming communications

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client C1 that supports continuous bidirectional voice communication, and is specified for use with a relay service. - The specified relay service is available. - Another communication client C2 that supports continuous bi-directional voice communication is available. |
| Procedure | Initiation: A communication session is established from communication client C2 to communication client C1 that is specified for use with a relay service by use of an identifier for the user of the relay service. Check 1: The specified relay service is invoked automatically or by the same user action as when the user answers the communication. Check 2: Relayed communication can take place |
| Result | Pass: : Check 1 and 2 are true Fail: At least one of the checks is not true. Not applicable: At least one precondition is not met Note: If the communication system is specified to connect to a number of relay services, the test needs to be repeated separately for each of the relay services. |

### 13.1.4	Establishment of relay service characteristics in relay service communications

Where ICT is or contains a communications system supporting relay service use,
the communication system shall support methods for addressing the relay service and selecting type of relay service and detailing indications of on which modalities and directions the relay service actions are wanted when the relay service is invoked in one step invocations.

#### C.13.1.4 Establishment of relay service characteristics in relay service communications

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client 1 that supports continuous bidirectional voice communication, and is specified for use with a relay service - The specified relay service is available. - Another communication client that supports continuous bi-directional voice communication is available. |
| Procedure | Step 1: Settings are done for the user of communications client 1 containing relay service address and type, and the wanted directions of the modality conversions of the user. Step 1: A communication session is established from communication client 1 that is specified for use with relay services to communication client 2 including an indication that relay service is wanted Check 1: The specified relay service is invoked supporting the requested relay service support for the preferred conversions.. Step 2: A communication session is established from communication client 2 to communication client 1 that is specified for use with relay services. Check 2 The user of the relay service gets an indication about an incoming communication and gets an opportunity to indicate that the relay service invocation is wanted and selects that option when answering. Check 3: The specified relay service is invoked supporting the requested relay service support for modality conversion. |
| Result | Pass: : Check 1, Check 2 and Check 3 are true Fail: One or more checks are false. Not applicable: At least one precondition is not met |

### 13.1.5	Relay service support in remote conferences

Where ICT is or includes a conference system for remote participation,
the ICT shall support participation of relay services in conferences and enable conference participation of users of the relay service.

NOTE 1: The extra modalities required for the conversion may be conveyed via the conference system or directly between the relay service and the user of the relay service.

NOTE 2: Access for the relay service to not only hear the voice channel of the conference but also view all contents of the conference provides an opportunity to successfully convey the conference contributions.

#### C.13.1.5	Relay service support in remote conferences

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a conference system for remote participation. - A conference participant device for taking part in the conference is available - A specified relay service is available. |
| Procedure | Step 1: The relay service connects to the conference. Step 2: The user of the relay service connects to the conference Step 3: The media channels required for the relay service and the user of the relay service to communicate in real time are established via the conference system or directly whichever is feasible for the type of conference. Check 1: The communication is established and the user of the relay service is enabled to both take part of the conference contents and contribute. |
| Result | Pass: : Check is true Fail: Check 1 is false Not applicable: At least one precondition is not met |

### 13.1.6 Invocation of a relay service during ongoing communication

Where ICT is or contains a communications system providing accessibility to voice communication by supporting relay service use,
invocation of a relay service in an ongoing communication shall be supported to be done by request of the user of the relay service.

NOTE: The functionality is intended for cases when the initial assumption of no need of a relay service initially in the communication turned out to be wrong, or for situations when the need for a relay service in the communications appears during the communication.

### C.13.1.6 Invocation of a relay service during ongoing communication

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client C1 that support continuous bidirectional voice communication, and is specified for use with a relay service. - The specified relay service is available. - Another communication client C2 that supports continuous bi-directional voice communication is available. |
| Procedure | - Initiation: A communication session is initiated from communication client 2 to communication client 1 that is specified for use with relay services. - The communication is answered by communication client without invoking the relay service - During the communication, invocation of the relay service is requested from communication client 1. Check 1: The specified relay service is invoked Check 2: A connection is established between C1 and C2 supported by the relay service |
| Result | Pass: : Check 1 and 2 are true Fail: At least one Check is not true. Not applicable: At least one precondition is not met Note: If the communication system is specified to connect to a number of relay services, the test needs to be repeated separately for each of the relay services. |

### 13.1.7 Disconnection of a relay service during ongoing communication

Where ICT is or contains a communications system providing accessibility to voice communication by supporting relay service use,
disconnection of a relay service already invoked in an ongoing communication shall be supported to be done by request of the user of the relay service.

NOTE: The functionality is intended for cases when the initial assumption of need of a relay service initially in the communication turned out to be wrong, or for situations when the need for a relay service in the communications disappears during the communication.

### C.13.1.7 Disconnection of a relay service during ongoing communication

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client C1 that supports continuous bidirectional voice communication, and is specified for use with a relay service. - The specified relay service is available. - Another communication client C2 that supports continuous bi-directional voice communication is available. |
| Procedure | - Initiation: A communication session is initiated from communication client C2 to communication client 1 that is specified for use with relay services. - The communication is answered by communication client C1 including invoking the relay service (automatically or by user action) Check 1: The specified relay service is invoked and acts on the communication 3 During the communication, disconnection of the relay service is requested from communication client C1. Check 2: The relay service is disconnected and communication between C1 and C2 can continue without support by the relay service |
| Result | Pass: : Check 1 and 2 are true Fail: At least one Check is not true. Not applicable: At least one precondition is not met Note: If the communication system is specified to connect to a number of relay services, the test needs to be repeated separately for each of the relay services. |

### 13.1.8 Provision of user identification of a user of the relay service

Where ICT is or contains a communications system providing accessibility to voice communication by supporting relay service use,
the user of the relay service shall be provided with an identifier (e.g. a number or an address) that can be used for reaching the user of the relay service.

NOTE: The identifier can preferably be the same for reaching the user with as well as without relay service invocation and the invocation being decided by other means.

#### C.13.1.8 Provision of user identification of a user of the relay service

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client C1 that supports continuous bidirectional voice communication, and is specified for use with a relay service. - The specified relay service is available - Another communication client C2 that supports continuous bi-directional voice communication is available. |
| Procedure | - Initiation: A communication session is initiated from communication client C2 to communication client C1 that is specified for use with relay services by use of the identifier for C1 - The communication is answered by communication client C1 invoking the relay service Check 1: The specified relay service is invoked Check 2: A connection is established between C1 and C2 supported by the relay service |
| Result | Pass: : Check 1 and 2 are true Fail: At least one Check is not true. Not applicable: At least one precondition is not met Note: If the communication system is specified to connect to a number of relay services, the test needs to be repeated separately for each of the relay services. |

### 13.1.9 Calling line identification of a user of the relay service

Where ICT is or contains a communications system providing accessibility to voice communication by supporting relay service use,
the initiation of a communication including the relay service by user of the relay service shall result in a calling line identification provided to the destination of the communication, useful for reaching the user of the relay service with a relay service supported communication if such identifications are provided in general in the communication system and the user has not requested the communication to be anonymous.

NOTE: The identifier can preferably be the same for reaching the user with as well as without relay service invocation and the invocation being decided by other means.

### C.13.1.9 Calling line identification of a user of the relay service

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client C1 that supports continuous bidirectional voice communication, and is specified for use with a relay service. - The specified relay service is available. - Another communication client C2 that supports continuous bi-directional voice communication is available. |
| Procedure | - Initiation: A communication session from communication client C1 with an indication to invoke the relay service is initiated to communication client C2 - The communication is answered by communication client C2 Check 1: The specified relay service is invoked Check 2: A connection is established between C1 and C2 supported by the relay service Check 3: A calling line identification is provided to C2. - The communication is disconnected and a mew communication initiated from C2 to C1, using the received calling line identification. Check 4: the communication is received in C1, either with the relay service already invoked or with a possibility for the user of C1 to answer and invoke the relay service in the same operation. |
| Result | Pass: : All checks are true are true Fail: At least one Check is not true. Not applicable: At least one precondition is not met Note: If the communication system is specified to connect to a number of relay services, the test needs to be repeated separately for each of the relay services. |

### 13.1.10 Establishment of modality, language and action directions in relay service communications

Where ICT is or contains a communication system supporting continuous bidirectional communication, and the communication system provides accessibility to voice communication by supporting use with relay services,

the communication system should support methods for indicating relay service address and type and negotiation of preferred modality and language of the user of the relay service, including detailing this information for production versus perception of communication contents including indications of on which modalities the relay service actions are wanted.

#### C.13.1.10 Establishment of modality, language and action directions in relay service communications

| Type of assessment | Inspection |
| --- | --- |
| Pre-conditions | - ICT under test is or includes a communication system that supports continuous bidirectional communication with a communication client that supports continuous bidirectional voice communication, and is specified for use with a relay service by indications containing relay service address and type, and the preferred languages and modalities of the user including their directions. - The specified relay service is available. - Another communication client that supports continuous bi-directional voice communication is available. |
| Procedure | Step 1: Make settings for the user of the relay service containing relay service address and type, and the preferred languages and modalities of the user including their directions. Step 2: A communication session is established from communication client 1 that is specified for use with relay services to communication client 2. Check 1: The specified relay service is invoked supporting the requested relay service support for input and outputs. Step 2: A communication session is established from communication client 2 to communication client 1 that is specified for use with relay services. Check 2: The specified relay service is invoked supporting the requested relay service support for input and output. |
| Result | Pass: : Check 1 and Check 2 are true Fail: One or more checks are false. Not applicable: At least one precondition is not met |