## 3.1	Terms

New term:
user-control: component of ICT used to activate, deactivate, or adjust the ICT

Note:	An on-screen button is an example of a user-control provided by software. Physical switches, latches, scanner covers, and notebook docking stations and lids are examples of user controls provided in hardware. This the same definition that used to define the term “operable part”, but without that definition’s notes.

The note on this definition is a minor edit of Note 1 from the previous definition of “operable part”.

Redefined term:
operable part: hardware element that needs to be touched or manipulated in order to operate the ICT

Note :	Operable parts do not include parts involved only in maintenance or repair or other actions that are not expected of a typical user if the product is not malfunctioning. These actions include: clearing paper jams internal to the machine, replacing items or parts internal to the machine that may expose the end user to sharp or hot surfaces, replacing or repairing items designated by manufacturers as service or maintenance items in user documentation.

This replaces the current definition of operable part and aligns it with the Section 508 definition of a hardware control.

New term:
tactilely discernible operable part: an operable part that can be located via the sense of touch without activating it

Note: 	Physical properties of the tactilely discernible operable part, or its surroundings, that enable an operable part to be tactilely discerned include, but are not limited to:

some of the operable part being above or below the surface that surrounds it;

the operable part having a raised feature that differentiates it from its surroundings;

a detectable gap that surrounds the tactile part;

having a different surface texture to surrounding surfaces;

an on-screen user-control being located adjacent to a tactilely discernible edge, corner, tactile markings on a screen frame;

a touchscreen or touchpad that is tactilely discernible that is used as a whole as a gesture surface for non-location-specific gestures.

The term “mechanically operable part” is no longer required.

# 5	Generic requirements

The previous 5.5 “Operable parts” had two requirements, 5.5.1 “Means of operation” and 5.5.2 Operable parts discernability. This is replaced by a single 5.5 as below. The software parts of the old 5.5.1 are addressed by 11.2.5.1 “Pointer gestures”, and the hardware parts are addressed in the new 8.4.2.1 “Means of operation”

## 5.5	Control using tactilely discernible operable parts

Where ICT includes user-controls,
the ICT shall provide a mode of operation that allows all the functionality of the ICT to be controlled without requiring vision using only user-controls that are implemented using tactilely discernible operable parts.

Note:	As noted in the definition of tactilely discernible operable part, a touchscreen or touchpad that can be tactilely discerned, or an-on-screen user-control that is located adjacent to a tactilely discernible edge, corner, or tactile markings on screen frame is a tactilely discernible operable part.

Clause 8 revisions incorporate the improved organisation of clause 8 proposed in GitLab Issue #327. That gives:

# 8	Hardware

## 8.1	General

The "generic requirements" of clause 5 also apply to ICT that is hardware.

## 8.2	Hardware products with speech output

## 8.3	Stationary ICT

## 8.4	Operable parts

### 8.4.1	Numeric keys

### 8.4.2	Operation of operable parts

#### 8.4.2.1	Means of operation

Where ICT is, or includes, hardware, and has operable parts that require grasping, pinching, or twisting of the wrist to operate,
the ICT shall provide at least one mode of operation that does not require these actions.

#### 8.4.2.2	Force of operation of mechanical parts

### 8.4.3	Keys, tickets and fare cards

## 8.5	Tactile indication of speech mode

## 8.6	Standard connections

## 8.7	Colour on hardware