ETSI GS NFV-SOL 001 V2.6.1 (2019-05)

Network Functions Virtualisation (NFV) Release 2;

Protocols and Data Models;

NFV descriptors based on TOSCA specification

<

Disclaimer

The present document has been produced and approved by the Network Functions Virtualisation (NFV) ETSI Industry Specification Group (ISG) and represents the views of those members who participated in this ISG.
It does not necessarily represent the views of the entire ETSI membership.

Group Specification

Reference

RGS/NFV-SOL001ed261

Keywords

data, information model, model, NFV

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2019.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPPTM and LTETM are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and
of the oneM2M Partners.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	16

Foreword	16

Modal verbs terminology	16

1	Scope	17

2	References	17

2.1	Normative references	17

2.2	Informative references	18

3	Definition of terms, symbols and abbreviations	18

3.1	Terms	18

3.2	Symbols	19

3.3	Abbreviations	19

4	Overview of TOSCA model	19

5	General concept of using TOSCA to model NFV descriptors	19

5.1	Introduction	19

5.2	Network Service Descriptor	19

5.3	Virtualised Network Function Descriptor	20

5.4	Physical Network Function Descriptor	20

5.5	tosca_definitions_version and Namespace prefix	20

5.6	Imports statement	21

5.6.1	VNFD TOSCA service template	21

5.6.2	NSD TOSCA service template	21

5.6.3	PNFD TOSCA service template	22

5.7	Type extension	23

5.7.1	Introduction	23

5.7.2	Rules	23

5.7.3	VNFD Types	23

5.7.4	NSD types	24

6	VNFD TOSCA model	24

6.1	Introduction	24

6.2	Data Types	25

6.2.1	tosca.datatypes.nfv.L2AddressData	25

6.2.1.1	Description	25

6.2.1.2	Properties	25

6.2.1.3	Definition	25

6.2.1.4	Examples	26

6.2.1.5	Additional Requirements	26

6.2.2	tosca.datatypes.nfv.L3AddressData	26

6.2.2.1	Description	26

6.2.2.2	Properties	26

6.2.2.3	Definition	27

6.2.2.4	Examples	27

6.2.2.5	Additional Requirements	27

6.2.3	tosca.datatypes.nfv.AddressData	27

6.2.3.1	Description	27

6.2.3.2	Properties	28

6.2.3.3	Definition	28

6.2.3.4	Examples	29

6.2.3.5	Additional Requirements	29

6.2.4	tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements	29

6.2.4.1	Description	29

6.2.4.2	Properties	29

6.2.4.3	Definition	30

6.2.4.4	Examples	30

6.2.4.5	Additional Requirements	30

6.2.5	tosca.datatypes.nfv.ConnectivityType	31

6.2.5.1	Description	31

6.2.5.2	Properties	31

6.2.5.3	Definition	31

6.2.5.4	Examples	32

6.2.5.5	Additional Requirements	32

6.2.6	tosca.datatypes.nfv.RequestedAdditionalCapability	32

6.2.6.1	Description	32

6.2.6.2	Properties	32

6.2.6.3	Definition	32

6.2.6.4	Examples	33

6.2.6.5	Additional Requirements	33

6.2.7	tosca.datatypes.nfv.VirtualMemory	33

6.2.7.1	Description	33

6.2.7.2	Properties	33

6.2.7.3	Definitions	34

6.2.7.4	Examples	34

6.2.7.5	Additional Requirements	34

6.2.8	tosca.datatypes.nfv.VirtualCpu	35

6.2.8.1	Description	35

6.2.8.2	Properties	35

6.2.8.3	Definition	35

6.2.8.4	Examples	36

6.2.8.5	Additional Requirements	36

6.2.9	tosca.datatypes.nfv.VirtualCpuPinning	36

6.2.9.1	Description	36

6.2.9.2	Properties	36

6.2.9.3	Definition	37

6.2.9.4	Examples	37

6.2.9.5	Additional Requirements	37

6.2.10	tosca.datatypes.nfv.VnfcConfigurableProperties	37

6.2.10.1	Description	37

6.2.10.2	Properties	38

6.2.10.3	Definition	38

6.2.10.4	Examples	38

6.2.10.5	Additional Requirements	39

6.2.11	tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties	39

6.2.11.1	Description	39

6.2.11.2	Properties	39

6.2.11.3	Definition	39

6.2.12	tosca.datatypes.nfv.VduProfile	40

6.2.12.1	Description	40

6.2.12.2	Properties	40

6.2.12.3	Definition	40

6.2.12.4	Examples	41

6.2.12.5	Additional requirements	42

6.2.13	tosca.datatypes.nfv.VlProfile	42

6.2.13.1	Description	42

6.2.13.2	Properties	42

6.2.13.3	Definition	43

6.2.13.4	Examples	43

6.2.13.5	Additional Requirements	43

6.2.14	tosca.datatypes.nfv.VirtualLinkProtocolData	43

6.2.14.1	Description	43

6.2.14.2	Properties	44

6.2.14.3	Definition	44

6.2.14.4	Examples	44

6.2.14.5	Additional Requirements	44

6.2.15	tosca.datatypes.nfv.L2ProtocolData	45

6.2.15.1	Description	45

6.2.15.2	Properties	45

6.2.15.3	Definition	45

6.2.15.4	Examples	46

6.2.15.5	Additional Requirements	46

6.2.16	tosca.datatypes.nfv.L3ProtocolData	46

6.2.16.1	Description	46

6.2.16.2	Properties	46

6.2.16.3	Definition	47

6.2.16.4	Examples	48

6.2.16.5	Additional Requirements	48

6.2.17	tosca.datatypes.nfv.IpAllocationPool	48

6.2.17.1	Description	48

6.2.17.2	Properties	48

6.2.17.3	Definition	48

6.2.17.4	Examples	48

6.2.17.5	Additional Requirements	49

6.2.18	tosca.datatypes.nfv.InstantiationLevel	49

6.2.18.1	Description	49

6.2.18.2	Properties	49

6.2.18.3	Definition	49

6.2.18.4	Examples	49

6.2.18.5	Additional Requirements	50

6.2.19	tosca.datatypes.nfv.VduLevel	50

6.2.19.1	Description	50

6.2.19.2	Properties	50

6.2.19.3	Definition	50

6.2.19.4	Examples	50

6.2.19.5	Additional Requirements	50

6.2.20	tosca.datatypes.nfv.VnfLcmOperationsConfiguration	51

6.2.20.1	Description	51

6.2.20.2	Properties	51

6.2.20.3	Definition	52

6.2.20.4	Examples	52

6.2.20.5	Additional Requirements	52

6.2.21	tosca.datatypes.nfv.VnfInstantiateOperationConfiguration	52

6.2.21.1	Description	52

6.2.21.2	Properties	53

6.2.21.3	Definition	53

6.2.21.4	Examples	53

6.2.21.5	Additional Requirements	53

6.2.22	tosca.datatypes.nfv.VnfScaleOperationConfiguration	53

6.2.22.1	Description	53

6.2.22.2	Properties	53

6.2.22.3	Definition	54

6.2.22.4	Examples	54

6.2.22.5	Additional Requirements	54

6.2.23	tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration	54

6.2.23.1	Description	54

6.2.23.2	Properties	54

6.2.23.3	Definition	54

6.2.23.4	Examples	55

6.2.23.5	Additional Requirements	55

6.2.24	tosca.datatypes.nfv.VnfHealOperationConfiguration	55

6.2.24.1	Description	55

6.2.24.2	Properties	55

6.2.24.3	Definition	55

6.2.24.4	Examples	56

6.2.24.5	Additional Requirements	56

6.2.25	tosca.datatypes.nfv.VnfTerminateOperationConfiguration	56

6.2.25.1	Description	56

6.2.25.2	Properties	56

6.2.25.3	Definition	56

6.2.25.4	Examples	57

6.2.25.5	Additional Requirements	57

6.2.26	tosca.datatypes.nfv.VnfOperateOperationConfiguration	57

6.2.26.1	Description	57

6.2.26.2	Properties	57

6.2.26.3	Definition	57

6.2.26.4	Examples	58

6.2.26.5	Additional Requirements	58

6.2.27	tosca.datatypes.nfv.ScaleInfo	58

6.2.27.1	Description	58

6.2.27.2	Properties	58

6.2.27.3	Definition	58

6.2.27.4	Examples	59

6.2.27.5	Additional Requirements	59

6.2.28	tosca.datatypes.nfv.ScalingAspect	59

6.2.28.1	Description	59

6.2.28.2	Properties	59

6.2.28.3	Definition	59

6.2.28.4	Examples	60

6.2.28.5	Additional Requirements	60

6.2.29	tosca.datatypes.nfv.LinkBitrateRequirements	60

6.2.29.1	Description	60

6.2.29.2	Properties	60

6.2.29.3	Definition	61

6.2.29.4	Examples	61

6.2.29.5	Additional Requirements	61

6.2.30	tosca.datatypes.nfv.Qos	61

6.2.30.1	Description	61

6.2.30.2	Properties	61

6.2.30.3	Definition	62

6.2.30.4	Examples	62

6.2.30.5	Additional Requirements	62

6.2.31	tosca.datatypes.nfv.VnfConfigurableProperties	62

6.2.31.1	Description	62

6.2.31.2	Properties	63

6.2.31.3	Definition	63

6.2.31.4	Examples	64

6.2.31.5	Additional Requirements	64

6.2.32	tosca.datatypes.nfv.VnfAdditionalConfigurableProperties	64

6.2.32.1	Description	64

6.2.32.2	Properties	64

6.2.32.3	Definition	65

6.2.32.4	Examples	65

6.2.32.5	Additional Requirements	65

6.2.33	tosca.datatypes.nfv.VnfInfoModifiableAttributes	65

6.2.33.1	Description	65

6.2.33.2	Properties	65

6.2.33.3	Definition	65

6.2.33.4	Examples	66

6.2.33.5	Additional Requirements	66

6.2.34	tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions	66

6.2.34.1	Description	66

6.2.34.2	Properties	66

6.2.34.3	Definition	66

6.2.34.4	Examples	66

6.2.34.5	Additional Requirements	66

6.2.35	tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata	67

6.2.35.1	Description	67

6.2.35.2	Properties	67

6.2.35.3	Definition	67

6.2.35.4	Examples	67

6.2.35.5	Additional Requirements	68

6.2.36	tosca.datatypes.nfv.CpProtocolData	68

6.2.36.1	Description	68

6.2.36.2	Properties	68

6.2.36.3	Definition	68

6.2.36.4	Examples	69

6.2.36.5	Additional Requirements	69

6.2.37	tosca.datatypes.nfv.LogicalNodeData	69

6.2.37.1	Description	69

6.2.37.2	Properties	69

6.2.37.3	Definition	69

6.2.37.4	Examples	70

6.2.37.5	Additional Requirements	70

6.2.38	tosca.datatypes.nfv.SwImageData	70

6.2.38.1	Description	70

6.2.38.2	Properties	70

6.2.38.3	Definition	71

6.2.38.4	Examples	72

6.2.38.5	Additional Requirements	72

6.2.39	tosca.datatypes.nfv.VirtualBlockStorageData	72

6.2.39.1	Description	72

6.2.39.2	Properties	73

6.2.39.3	Definition	73

6.2.39.4	Examples	73

6.2.39.5	Additional Requirements	73

6.2.40	tosca.datatypes.nfv.VirtualObjectStorageData	74

6.2.40.1	Description	74

6.2.40.2	Properties	74

6.2.40.3	Definition	74

6.2.40.4	Examples	74

6.2.40.5	Additional Requirements	74

6.2.41	tosca.datatypes.nfv.VirtualFileStorageData	74

6.2.41.1	Description	74

6.2.41.2	Properties	75

6.2.41.3	Definition	75

6.2.41.4	Examples	75

6.2.41.5	Additional Requirements	75

6.2.42	tosca.datatypes.nfv.VirtualLinkBitrateLevel	75

6.2.42.1	Description	75

6.2.42.2	Properties	76

6.2.42.3	Definition	76

6.2.42.4	Examples	76

6.2.42.5	Additional Requirements	76

6.2.43	tosca.datatypes.nfv.VnfOperationAdditionalParameters	76

6.2.43.1	Description	76

6.2.43.2	Properties	77

6.2.43.3	Definition	77

6.2.43.4	Examples	77

6.2.43.5	Additional Requirements	77

6.2.44	tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration	77

6.2.44.1	Description	77

6.2.44.2	Properties	78

6.2.44.3	Definition	78

6.2.44.4	Examples	78

6.2.44.5	Additional Requirements	78

6.2.45	tosca.datatypes.nfv.VnfChangeExtConnectivityOperation Configuration	78

6.2.45.1	Description	78

6.2.45.2	Properties	78

6.2.45.3	Definition	78

6.2.45.4	Examples	79

6.2.45.5	Additional Requirements	79

6.2.46	tosca.datatypes.nfv.VnfMonitoringParameter	79

6.2.46.1	Description	79

6.2.46.2	Properties	79

6.2.46.3	Definition	80

6.2.46.4	Examples	80

6.2.46.5	Additional Requirements	80

6.2.47	tosca.datatypes.nfv.VnfcMonitoringParameter	80

6.2.47.1	Description	80

6.2.47.2	Properties	80

6.2.47.3	Definition	81

6.2.47.4	Examples	82

6.2.47.5	Additional Requirements	82

6.2.48	tosca.datatypes.nfv.VirtualLinkMonitoringParameter	82

6.2.48.1	Description	82

6.2.48.2	Properties	82

6.2.48.3	Definition	82

6.2.48.4	Examples	83

6.2.48.5	Additional Requirements	83

6.2.49	tosca.datatypes.nfv.InterfaceDetails	83

6.2.49.1	Description	83

6.2.49.2	Properties	83

6.2.49.3	Definition	84

6.2.49.4	Examples	84

6.2.49.5	Additional Requirements	84

6.2.50	tosca.datatypes.nfv.UriComponents	84

6.2.50.1	Description	84

6.2.50.2	Properties	84

6.2.50.3	Definition	85

6.2.50.4	Examples	85

6.2.50.5	Additional Requirements	85

6.2.51	tosca.datatypes.nfv.UriAuthority	86

6.2.51.1	Description	86

6.2.51.2	Properties	86

6.2.51.3	Definition	86

6.2.51.4	Examples	87

6.2.51.5	Additional Requirements	87

6.2.52	tosca.datatypes.nfv.VnfProfile	87

6.2.52.1	Description	87

6.2.52.2	Properties	87

6.2.52.3	Definition	88

6.2.52.4	Example	88

6.2.52.5	Additional Requirements	88

6.2.53	tosca.datatypes.nfv.ChecksumData	88

6.2.53.1	Description	88

6.2.53.2	Properties	89

6.2.53.3	Definition	89

6.2.53.4	Examples	89

6.2.53.5	Additional Requirements	89

6.3	Artifact Types	89

6.3.1	tosca.artifacts.nfv.SwImage	89

6.3.1.1	Description	89

6.3.1.2	Description	90

6.3.2	tosca.artifacts.Implementation.nfv.Mistral	90

6.3.2.1	Description	90

6.3.2.2	Definition	90

6.4	Capability Types	90

6.4.1	tosca.capabilities.nfv.VirtualBindable	90

6.4.1.1	Description	90

6.4.1.2	Properties	91

6.4.1.3	Definition	91

6.4.2	tosca.capabilities.nfv.VirtualLinkable	91

6.4.2.1	Description	91

6.4.2.2	Properties	91

6.4.2.3	Definition	91

6.4.3	tosca.capabilities.nfv.VirtualCompute	91

6.4.3.1	Description	91

6.4.3.2	Properties	92

6.4.3.3	Definition	92

6.4.4	tosca.capabilities.nfv.VirtualStorage	93

6.4.4.1	Description	93

6.4.4.2	Definition	93

6.5	Requirements Types	93

6.6	Relationship Types	93

6.6.1	tosca.relationships.nfv.VirtualBindsTo	93

6.6.1.1	Description	93

6.6.1.2	Properties	94

6.6.1.3	Definition	94

6.6.2	tosca.relationships.nfv.VirtualLinksTo	94

6.6.2.1	Description	94

6.6.2.2	Properties	94

6.6.2.3	Definition	94

6.6.3	tosca.relationships.nfv.AttachesTo	94

6.6.3.1	Description	94

6.6.3.2	Properties	95

6.6.3.3	Definition	95

6.7	Interface Types	95

6.7.1	tosca.interfaces.nfv.Vnflcm	95

6.7.1.1	Description	95

6.7.1.2	Definition	96

6.7.1.3	Additional Requirements	98

6.7.1.4	Support of LCM scripts	98

6.7.1.5	Examples	99

6.8	Node Types	101

6.8.1	tosca.nodes.nfv.VNF	101

6.8.1.1	Description	101

6.8.1.2	Properties	101

6.8.1.3	Attributes	103

6.8.1.4	Requirements	103

6.8.1.5	Capabilities	103

6.8.1.6	Definition	104

6.8.1.7	Artifact	105

6.8.1.8	Additional Requirements	105

6.8.1.9	Example	106

6.8.2	tosca.nodes.nfv.VnfExtCp	108

6.8.2.1	Description	108

6.8.2.2	Properties	108

6.8.2.3	Attributes	108

6.8.2.4	Requirements	109

6.8.2.5	Definition	109

6.8.2.6	Additional Requirements	109

6.8.2.7	Example	109

6.8.3	tosca.nodes.nfv.Vdu.Compute	110

6.8.3.1	Description	110

6.8.3.2	Properties	110

6.8.3.3	Attributes	111

6.8.3.4	Requirements	111

6.8.3.5	Capabilities	111

6.8.3.6	Definition	112

6.8.3.7	Additional requirements	113

6.8.3.8	Example	113

6.8.4	tosca.nodes.nfv.Vdu.VirtualBlockStorage	116

6.8.4.1	Description	116

6.8.4.2	Properties	116

6.8.4.3	Attributes	116

6.8.4.4	Requirements	116

6.8.4.5	Capabilities	116

6.8.4.6	Definition	116

6.8.4.7	Additional requirements	117

6.8.5	tosca.nodes.nfv.Vdu.VirtualObjectStorage	117

6.8.5.1	Description	117

6.8.5.2	Properties	117

6.8.5.3	Attributes	117

6.8.5.4	Requirements	117

6.8.5.5	Capabilities	117

6.8.5.6	Definition	118

6.8.5.7	Additional requirements	118

6.8.6	tosca.nodes.nfv.Vdu.VirtualFileStorage	118

6.8.6.1	Description	118

6.8.6.2	Properties	118

6.8.6.3	Attributes	118

6.8.6.4	Requirements	119

6.8.6.5	Capabilities	119

6.8.6.6	Definition	119

6.8.6.7	Additional requirements	119

6.8.7	tosca.nodes.nfv.Cp	119

6.8.7.1	Description	119

6.8.7.2	Properties	120

6.8.7.3	Attributes	120

6.8.7.4	Requirements	120

6.8.7.5	Capabilities	120

6.8.7.6	Definition	121

6.8.7.7	Additional requirements	121

6.8.8	tosca.nodes.nfv.VduCp	121

6.8.8.1	Description	121

6.8.8.2	Properties	122

6.8.8.3	Attributes	122

6.8.8.4	Requirements	122

6.8.8.5	Definition	123

6.8.9	tosca.nodes.nfv.VnfVirtualLink	123

6.8.9.1	Description	123

6.8.9.2	Properties	124

6.8.9.3	Requirements	124

6.8.9.4	Capabilities	124

6.8.9.5	Definition	124

6.9	Group Types	125

6.9.1	tosca.groups.nfv.PlacementGroup	125

6.9.1.1	Description	125

6.9.1.2	Properties	125

6.9.1.3	Definition	125

6.9.1.4	Additional Requirements	126

6.9.1.5	Examples	126

6.10	Policy Types	126

6.10.1	tosca.policies.nfv.InstantiationLevels	126

6.10.1.1	Description	126

6.10.1.2	Properties	126

6.10.1.3	Definition	127

6.10.2	tosca.policies.nfv.VduInstantiationLevels	127

6.10.2.1	Description	127

6.10.2.2	Properties	127

6.10.2.3	Definition	128

6.10.2.4	Additional Requirements	128

6.10.3	tosca.policies.nfv.VirtualLinkInstantiationLevels	128

6.10.3.1	Description	128

6.10.3.2	Properties	128

6.10.3.3	Definition	129

6.10.3.4	Additional Requirements	129

6.10.4	Void	129

6.10.5	tosca.policies.nfv.ScalingAspects	129

6.10.5.1	Description	129

6.10.5.2	Properties	129

6.10.5.3	Definition	130

6.10.5.4	Examples	130

6.10.6	tosca.policies.nfv.VduScalingAspectDeltas	130

6.10.6.1	Description	130

6.10.6.2	Properties	130

6.10.6.3	Definition	131

6.10.6.4	Examples	131

6.10.7	tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas	131

6.10.7.1	Description	131

6.10.7.2	Properties	131

6.10.7.3	Definition	132

6.10.7.4	Examples	132

6.10.8	tosca.policies.nfv.VduInitialDelta	132

6.10.8.1	Description	132

6.10.8.2	Properties	132

6.10.8.3	Definition	133

6.10.8.4	Examples	133

6.10.9	tosca.policies.nfv.VirtualLinkBitrateInitialDelta	133

6.10.9.1	Description	133

6.10.9.2	Properties	133

6.10.9.3	Definition	134

6.10.9.4	Examples	134

6.10.10	AffinityRule, AntiAffinityRule	134

6.10.10.1	Description	134

6.10.10.2	Properties	134

6.10.10.3	targets	135

6.10.10.4	Definition	135

6.10.10.5	Examples	136

6.10.11	tosca.policies.nfv.SecurityGroupRule	136

6.10.11.1	Description	136

6.10.11.2	Properties	137

6.10.11.3	Definition	137

6.10.11.4	Additional Requirements	139

6.10.12	tosca.policies.nfv.SupportedVnfInterface	139

6.10.12.1	Description	139

6.10.12.2	Properties	139

6.10.12.3	Definition	139

6.10.12.4	Additional requirements	140

6.10.12.5	Example	140

6.11	VNFD TOSCA service template design	140

6.11.1	General	140

6.11.2	Single or multiple deployment flavour design with TOSCA-Simple-Profile-YAML-v1.2	140

6.11.3	Single deployment flavour design with TOSCA-Simple-Profile-YAML-v1.1	141

7	NSD TOSCA model	142

7.1	Introduction	142

7.2	Data Types	143

7.2.1	tosca.datatype.nfv.ServiceAvailability	143

7.2.1.1	Description	143

7.2.1.2	Properties	143

7.2.1.3	Definition	143

7.2.1.4	Examples	143

7.2.2	tosca.datatypes.nfv.VnfProfile	143

7.2.2.1	Description	143

7.2.3	tosca.datatype.nfv.NsVlProfile	144

7.2.3.1	Description	144

7.2.3.2	Properties	144

7.2.3.3	Definition	144

7.2.3.4	Examples	145

7.2.3.5	Additional Requirements	145

7.2.4	tosca.datatypes.nfv.ConnectivityType	145

7.2.4.1	Description	145

7.2.5	tosca.datatypes.nfv. NsVirtualLinkQos	145

7.2.5.1	Description	145

7.2.5.2	Properties	145

7.2.5.3	Definition	145

7.2.5.4	Examples	146

7.2.5.5	Additional Requirements	146

7.2.6	tosca.datatypes.nfv.LinkBitrateRequirements	146

7.2.6.1	Description	146

7.2.7	Void	146

7.2.8	Void	146

7.2.9	Void	146

7.2.10	Void	146

7.2.11	tosca.datatypes.nfv.CpProtocolData	146

7.2.11.1	Description	146

7.2.12	tosca.datatypes.nfv.AddressData	146

7.2.12.1	Description	146

7.2.13	tosca.datatypes.nfv.L2AddressData	146

7.2.13.1	Description	146

7.2.14	tosca.datatypes.nfv.L3AddressData	146

7.2.14.1	Description	146

7.2.15	tosca.datatypes.nfv.Qos	147

7.2.15.1	Description	147

7.3	Artifact Types	148

7.4	Capabilities Types	148

7.4.1	tosca.capabilities.nfv.VirtualLinkable	148

7.4.1.1	Description	148

7.5	Requirements Types	148

7.6	Relationship Types	148

7.6.1	tosca.relationships.nfv.VirtualLinksTo	148

7.6.1.1	Description	148

7.7	Interface Types	149

7.7.1	tosca.interfaces.nfv.Nslcm	149

7.7.1.1	Description	149

7.7.1.2	Definition	149

7.7.1.3	Additional Requirements	150

7.7.1.4	Support of LCM scripts	150

7.7.1.5	Examples	151

7.8	Node Types	151

7.8.1	tosca.nodes.nfv.NS	151

7.8.1.1	Description	151

7.8.1.2	Properties	152

7.8.1.3	Attributes	152

7.8.1.4	Requirements	152

7.8.1.5	Capabilities	152

7.8.1.6	Definition	153

7.8.1.7	Artifact	153

7.8.1.8	Additional requirements	153

7.8.2	tosca.nodes.nfv.Sap	154

7.8.2.1	Description	154

7.8.2.2	Properties	154

7.8.2.3	Attributes	154

7.8.2.4	Requirements	154

7.8.2.5	Capabilities	154

7.8.2.6	Definition	154

7.8.2.7	Additional requirements	155

7.8.2.8	Example	155

7.8.3	tosca.nodes.nfv.NsVirtualLink	155

7.8.3.1	Description	155

7.8.3.2	Properties	155

7.8.3.3	Attributes	156

7.8.3.4	Requirements	156

7.8.3.5	Capabilities	156

7.8.3.6	Definition	156

7.8.3.7	Artifact	157

7.8.3.8	Additional Requirements	157

7.8.3.9	Example	157

7.8.4	tosca.nodes.nfv.Cp	157

7.8.4.1	Description	157

7.9	Group Types	157

7.9.1	tosca.groups.nfv.PlacementGroup	157

7.9.1.1	Description	157

7.10	Policy Types	157

7.10.1	AffinityRule, AntiAffinityRule	157

7.10.1.1	Description	157

7.10.1.2	Targets	158

7.10.1.3	Examples	158

7.10.2	tosca.policies.nfv.SecurityGroupRule	159

7.10.2.1	Description	159

7.10.2.2	Additional Requirements	159

7.11	NSD TOSCA service template design	159

7.11.1	General	159

7.11.2	Single or multiple deployment flavour design with TOSCA-Simple-Profile-YAML-v1.2	159

7.11.3	Single deployment flavour design with TOSCA-Simple-Profile-YAML-v1.1	160

8	PNFD TOSCA model	161

8.1	Introduction	161

8.2	Data Types	162

8.2.1	tosca.datatypes.nfv.CpProtocolData	162

8.2.1.1	Description	162

8.2.2	tosca.datatypes.nfv.AddressData	162

8.2.2.1	Description	162

8.2.3	tosca.datatypes.nfv.L2AddressData	162

8.2.3.1	Description	162

8.2.4	tosca.datatypes.nfv.L3AddressData	162

8.2.4.1	Description	162

8.2.5	tosca.datatypes.nfv.LocationInfo	162

8.2.5.1	Description	162

8.2.5.2	Properties	162

8.2.5.3	Definition	163

8.2.5.4	Examples	163

8.2.5.5	Additional Requirements	163

8.2.6	tosca.datatypes.nfv.CivicAddressElement	163

8.2.6.1	Description	163

8.2.6.2	Properties	163

8.2.6.3	Definition	164

8.2.6.4	Examples	164

8.2.6.5	Additional Requirements	164

8.3	Artifact Types	164

8.4	Capabilities Types	164

8.4.1	tosca.capabilities.nfv.VirtualLinkable	164

8.4.1.1	Description	164

8.5	Requirements Types	164

8.6	Relationship Types	165

8.6.1	tosca.relationships.nfv.VirtualLinksTo	165

8.6.1.1	Description	165

8.7	Interface Types	165

8.8	Node Types	165

8.8.1	tosca.nodes.nfv.PNF	165

8.8.1.1	Description	165

8.8.1.2	Properties	165

8.8.1.3	Attributes	166

8.8.1.4	Requirements	166

8.8.1.5	Capabilities	166

8.8.1.6	Definition	166

8.8.1.7	Artifact	166

8.8.1.8	Additional Requirements	167

8.8.1.9	Example	167

8.8.2	tosca.nodes.nfv.PnfExtCp	167

8.8.2.1	Description	167

8.8.2.2	Properties	167

8.8.2.3	Attributes	167

8.8.2.4	Requirements	167

8.8.2.5	Capabilities	167

8.8.2.6	Definition	168

8.8.3	tosca.nodes.nfv.Cp	168

8.8.3.1	Description	168

8.9	Group Types	168

8.10	Policy Types	168

8.10.1	tosca.policies.nfv.SecurityGroupRule	168

8.10.1.1	Description	168

8.10.1.2	Additional Requirements	168

8.11	PNFD TOSCA service template design	168

8.11.1	General	168

Annex A (informative):	Examples	170

A.1	Deployment flavour design mapping	170

A.1.1	Introduction	170

A.1.2	Design principle	170

A.2	VNFD with deployment flavour modelling design example	171

A.3	VNF external connection point	179

A.3.1	General	179

A.3.2	External connection point re-exposing an internal connection point	180

A.3.3	External connection point connected to an internal virtual link	181

A.4	VNFD modelling design example by using TOSCA composition	182

A.5	VNFD with Single deployment flavour modelling design example	185

A.6	Scaling and Instantiation Level examples	189

A.6.1	ScalingAspect and InstantiationLevels policies with uniform delta	189

A.6.2	ScalingAspect and InstantationLevels policies with non-uniform deltas	194

A.7	Service Access Point	198

A.7.1	General	198

A.7.2	VNF External connection point exposing as a SAP	198

A.7.3	SAP connected to an NS virtual link	200

A.8	NSD with Single deployment flavour modelling design example	201

A.9	Mapping between NFV IM and TOSCA concepts	203

A.9.1	Introduction	203

A.9.2	Mapping between ETSI GS NFV-IFA 011 IM and TOSCA concepts	203

A.9.3	Mapping between ETSI GS NFV-IFA 014 IM and TOSCA concepts	204

A.10	PNFD modelling design example	205

A.11	NSD with Multiple deployment flavour modelling design example	207

A.12	NSD TOSCA service template design	213

Annex B (normative):	etsi_nfv_sol001_type definitions	218

B.1	Purpose	218

B.2	VNFD type definitions file	218

B.3	NSD type definitions file	218

B.4	PNFD type definitions file	219

Annex C (normative):	Conformance	220

C.1	Purpose	220

C.2	NFV TOSCA YAML service template	220

C.3	NFV TOSCA processor	221

Annex D (informative):	Mapping between properties of TOSCA types and API attributes	222

D.1	Introduction	222

D.2	VNFD-related constructs	222

Annex E (informative):	Authors & contributors	229

Annex F (informative):	Change History	230

History	232

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org/).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This Group Specification (GS) has been produced by ETSI Industry Specification Group (ISG) Network Functions Virtualisation (NFV).

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# 1	Scope

The present document specifies a data model for NFV descriptors, using the TOSCA Simple Profile in YAML [3], fulfilling the requirements specified in ETSI GS NFV-IFA 011 [1] and ETSI GS NFV-IFA 014 [2] for a Virtualised Network Function Descriptor (VNFD), a Network Service Descriptor (NSD) and a Physical Network Function Descriptor (PNFD). The present document also specifies requirements on the VNFM and NFVO specific to the handling of NFV descriptors based on the TOSCA Simple Profile in YAML specification [3].

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference/.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ETSI GS NFV-IFA 011: "Network Functions Virtualisation (NFV) Release 2; Management and Orchestration; VNF Descriptor and Packaging Specification".

[2]	ETSI GS NFV-IFA 014: "Network Functions Virtualisation (NFV) Release 2; Management and Orchestration; Network Service Templates Specification".

[3]	TOSCA-Simple-Profile-yaml-v1.2: "TOSCA Simple Profile in YAML Version 1.2".

[4]	TOSCA-Simple-Profile-yaml-v1.1: "TOSCA Simple Profile in YAML Version 1.1".

[5]	Private Enterprise Numbers registry at IANA.

NOTE:	Available at https://www.iana.org/assignments/enterprise-numbers/enterprise-numbers.

[6]	IETF RFC 5234: "Augmented BNF for Syntax Specifications: ABNF".

[7]	ETSI GS NFV-IFA 027: "Network Functions Virtualisation (NFV) Release 2; Management and Orchestration; Performance Measurements Specification".

[8]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[9]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[10]	ISO 3166 (all parts): "Codes for the representation of names of countries and their subdivisions".

[11]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

NOTE:	Available at https://tools.ietf.org/html/rfc4776.

[12]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[13]	IETF RFC 5646: "Tags for Identifying Languages".

[14]	Hash Function Textual Names registry at IANA.

NOTE:	Available at https://www.iana.org/assignments/hash-function-text-names.

[15]	The Open Group Base Specifications Issue 7, 2018 edition IEEETM Std 1003.1-2017 (Revision of IEEE Std 1003.1-2008).

NOTE:	Available at http://pubs.opengroup.org/onlinepubs/9699919799/basedefs/V1_chap09.html.

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ETSI GS NFV-IFA 007: "Network Functions Virtualisation (NFV); Management and Orchestration; Or-Vnfm reference point - Interface and Information Model Specification".

[i.2]	ETSI GS NFV 003: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[i.3]	ETSI GS NFV-REL 001: "Network Functions Virtualisation (NFV); Resiliency Requirements".

[i.4]	ETSI GS NFV-SOL 002: "Network Functions Virtualisation (NFV) Release 2; Protocols and Data Models; RESTful protocols specification for the Ve-Vnfm Reference Point".

[i.5]	Assigned Internet Protocol Numbers.

NOTE:	Available at https://www.iana.org/assignments/protocol-numbers/protocol-numbers.xhtml.

[i.6]	ETSI GS NFV-SOL 004: "Network Functions Virtualisation (NFV) Release 2; Protocols and Data Models; VNF Package specification".

[i.7]	Mistral Workflow Language v2 specification.

NOTE:	Available at https://docs.openstack.org/mistral/latest/user/wf_lang_v2.html.

[i.8]	ETSI GS NFV-IFA 013: "Network Functions Virtualisation (NFV); Management and Orchestration; Os-Ma-Nfvo reference point - Interface and Information Model Specification".

[i.9]	ETSI GS NFV-SOL 003: "Network Functions Virtualisation (NFV) Release 2; Protocols and Data Models; RESTful protocols specification for the Or-Vnfm Reference Point".

[i.10]	ETSI GS NFV-SOL 005: "Network Functions Virtualisation (NFV) Release 2; Protocols and Data Models; RESTful protocols specification for the Os-Ma-nfvo Reference Point".

[i.11]	ETSI GS NFV-SOL 007: "Network Functions Virtualisation (NFV) Release 2; Protocols and Data Models; Network Service Descriptor File Structure Specification".

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in ETSI GS NFV 003 [i.2] and the following apply:

TOSCA interface type: reusable entity that describes a set of TOSCA operations that can be included as part of a Node type or Relationship Type definition

NOTE:	See TOSCA-Simple-Profile-YAML-v1.2 [3].

TOSCA operation: behavioural lifecycle procedure in a TOSCA node or relationship definition that can be invoked by an orchestration engine, whose implementation definition can be provided in the service template as part of a node template definition or a relationship template definition, or rely on an implementation of the operation built in the orchestration engine

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in ETSI GS NFV 003 [i.2] apply.

# 4	Overview of TOSCA model

TOSCA (Topology and Orchestration Specification for Cloud Applications) is a modelling language for describing the components of a cloud application and their relationships. TOSCA uses the concept of service templates to describe cloud workloads. TOSCA further provides means of associating standard or user-defined lifecycle operations to a cloud application component or to a relationship between components. The present document is based on TOSCA-Simple-Profile-yaml v1.2 [3], which describes a YAML rendering for TOSCA.

# 5	General concept of using TOSCA to model NFV descriptors

## 5.1	Introduction

An NFV deployment template is modelled by using one or more TOSCA service template as defined in TOSCA-Simple-Profile-yaml-v1.2 [3].

Three main deployment templates are identified in the present document:

The Virtualised Network Function Descriptor (VNFD).

The Network Service Descriptor (NSD).

The Physical Network Function Descriptor (PNFD).

## 5.2	Network Service Descriptor

The Network Service Descriptor (NSD) is a deployment template which consists of information used by the NFVO for lifecycle management of an NS as defined in ETSI GS NFV-IFA 014 [2]. The NSD:

References zero, one or more Virtualised Network Function Descriptors (VNFD).

References zero, one or more Physical Network Functions Descriptors (PNFD).

References zero, one or more nested NSD.

Includes zero, one or more Virtual Link Descriptors (VLD).

Includes zero, one or more VNF Forwarding Graph Descriptors (VNFFGD).

A VNFFGD describes a topology of the Network Service or a portion of the Network Service.

A VLD describes the resource for deploying and managing the lifecycle of virtual links between the constituents of an NS.

A PNFD describes the connectivity requirements to integrate PNFs in an NS.

A nested NSD is an NSD from which a nested NS can be instantiated within a parent NS instance.

## 5.3	Virtualised Network Function Descriptor

The VNFD is a component of a VNF package. It is used by both the NFVO and the VNFM.

A VNFD is a deployment template which describes a VNF in terms of deployment and operational behaviour requirements. It also contains Virtualised Deployment Units (VDUs), internal virtual link descriptors, external connection point descriptors, software image descriptors, and deployment flavour descriptors, as defined in ETSI GS NFV-IFA 011 [1].

A VNFD contains the following main pieces of information, as shown in figure 5.3-1:

Virtualisation Deployment Unit (VDU) is a construct supporting the description of the deployment and operational behaviour of a VNF Component (VNFC). A VNFC instance created based on the VDU maps to a single virtualisation container (e.g. a VM). AVDU describes the resources needed to deploy and manage the lifecycle of a VNFC. A VDU includes internal Connection Point Descriptors (CPDs) that describe internal connection points that can either be used to connect a VNFC to an internal virtual link or be re-exposed outside the VNF as external connection points.

External CPD: describes an external connection point of a VNF, where either an internal connection point of a VDU is exposed as external connection point or the external connection point is directly connected to an internal virtual link.

Internal VLD: describes the resource requirements for deploying and managing the lifecycle of virtual links between one or more VNFC instances created based on one or more VDUs.

Figure 5.3-1: Overview of VNF descriptor

The information within a VNFD is structured according to one or more VNF deployment flavours (VnfDf) that specify different deployment configuration of a VNF, in terms of its internal topology and resource needs.

## 5.4	Physical Network Function Descriptor

The Physical Network Function Descriptor (PNFD) information element is a deployment template enabling on-boarding PNFs and referencing them from an NSD. It focuses on connectivity aspects only.

## 5.5	tosca_definitions_version and Namespace prefix

The "tosca_definitions_version" keyword when used in the present document shall comply with the definition as specified in section 3.1.2 of TOSCA-Simple-Profile-YAML-v1.2 [3] with the associated Namespace Alias value defined in the TOSCA-Simple-Profile-YAML-v1.2 specification [3].

Table 5.5-1 defines the TOSCA Namespace prefix that shall be used to declare the namespace of all the TOSCA types as specified in the present document.

Table 5.5-1

| Namespace Prefix | Specification Description |
| --- | --- |
| toscanfv | The TOSCA namespace prefix of all the TOSCA types as specified in the present document. |

## 5.6	Imports statement

### 5.6.1	VNFD TOSCA service template

A VNFD TOSCA service template as specified in clause 6.11 shall include a TOSCA import definition referencing the following files:

The file defined in clause B.2 that includes all the type definitions from clause 6 of the present document.

Others, as described in clause 6.11.

As specified in [3], the import statement can use a single-line or multi-line grammar.

The single-line grammar supports the import of single or multiple uniquely named VNFD types and other template definition files.

EXAMPLE 1:

imports: - https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_vnfd_types.yaml - any_other_files.yaml - custom_vnfd_datatypes_extension.yaml

The multi-line grammar also supports the import of single or multiple uniquely named VNFD types and other template definition files. The "file" keyword is a mandatory parameter in this grammar.

EXAMPLE 2:

imports: - file: https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_vnfd_types.yaml - file: any_other_files.yaml - file: custom_vnfd_datatypes_extension.yaml

### 5.6.2	NSD TOSCA service template

A NSD TOSCA service template as specified in clause 7.11 shall include a TOSCA import definition referencing the following files:

The file defined in clause B.3 that includes all the type definitions from clause 7 of the present document.

Others, as described in clause 7.11.

As specified in [3], the import statement can use a single-line or multi-line grammar.

The single-line grammar supports the import of single or multiple uniquely named NSD types and other template definition files.

EXAMPLE 1:

imports: - https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_nsd_types.yaml - any_other_files.yaml - custom_nsd_node_types_extension.yaml

The multi-line grammar also supports the import of single or multiple uniquely named NSD types and other template definition files. The "file" keyword is a mandatory parameter in this grammar.

EXAMPLE 2:

imports: - file: https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_nsd_types.yaml - file: any_other_files.yaml - file: custom_nsd_node_types_extension.yaml

### 5.6.3	PNFD TOSCA service template

A PNFD TOSCA service template as specified in clause 8.11 shall include a TOSCA import definition referencing the following files:

The file defined in clause B.4 that includes all the type definitions from clause 8 of the present document.

Others, as described in clause 8.11.

As specified in [3], the import statement can use a single-line or multi-line grammar.

The single-line grammar supports the import of single or multiple uniquely named PNFD types and other template definition files.

EXAMPLE 1:

imports: - https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_pnfd_types.yaml - any_other_files.yaml - custom_pnfd_node_types_extension.yaml

The multi-line grammar also supports the import of single or multiple uniquely named PNFD types and other template definition files. The "file" keyword is a mandatory parameter in this grammar.

EXAMPLE 2:

imports: - file: https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_pnfd_types.yaml - file: any_other_files.yaml - file: custom_pnfd_node_types_extension.yaml

## 5.7	Type extension

### 5.7.1	Introduction

Type extension is used when VNF-specific type information is introduced in the VNFD (e.g. modifiable attributes, configurable properties and additional parameters to LCM operations) or NSD.

### 5.7.2	Rules

Type extension may be applied to NFV types defined in the present document within the limits specified table 5.7.3-1, adhering to the following rule.

A derived type shall extend the base type in such a way that it remains substitutable for the base type with the following requirements.

New properties may be introduced with no restriction within the limits specified table 5.7.3-1.

Existing properties may be extended according to the following rules:

A scalar property may not be extended to another type (e.g. a string property may not be replaced with an integer property or with a complex property of the same name).

A complex property of data type "X" may only be extended to a property of type "Y" where "Y" is derived from "X" according to the present rules (recursive rule: present rules applied to each property of the derived data type.

A property of type list with entry schema "X" may only be extended to a list with entry schema "Y" where "Y" is an extension of "X" according to the present rules (recursive rule: present rules applied to the elements of the list).

A property of type map with entry schema "X" may only be extended to a map with entry schema "Y" where "Y" is an extension of "X" according to the present rules (recursive rule: present rules applied to the values of the map).

In general, the same rules (1 and 2) apply to introducing/extending other elements beyond properties such as capabilities, requirements, interfaces, operations, inputs, etc. as well; in the current version of the specification, however, there are only examples of extending properties and the tosca.nodes.nfv.VNF node type.

### 5.7.3	VNFD Types

Table 5.7.3-1 specifies the extension point where VNFD author may extend the pre-defined types.

Table 5.7.3-1: VNFD type extension points

| Type | Keyname | Property name |
| --- | --- | --- |
| tosca.nodes.nfv.VNF | properties | modifiable_attributes (as a new property) configurable_properties (as a new property) |
|  | interfaces | Vnflcm.{operation_name}.inputs.additional_parameters (as a new property) Vnflcm.{operation_name}.inputs (as new properties) |
| tosca.nodes.nfv.Vdu.Compute | properties | configurable_properties (as a new property) |
| tosca.datatypes.nfv.VnfInfoModifiableAttributes | properties | extensions (as a new property) metadata (as a new property) |
| tosca.datatypes.nfv.VnfConfigurableProperties | properties | additional_configurable_properties (as a new property) |
| tosca.datatypes.nfv.VnfcConfigurableProperties | properties | additional_vnfc_configurable_properties (as a new property) |
| tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions | properties | (new properties) |
| tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata | properties | (new properties) |
| tosca.datatypes.nfv.VnfAdditionalConfigurableProperties | properties | (new properties) |
| tosca.datatypes.nfv.VnfAdditionalVnfcConfigurableProperties | properties | (new properties) |
| tosca.datatypes.nfv.VnfOperationAdditionalParameters | properties | (new properties) |

### 5.7.4	NSD types

None.

# 6	VNFD TOSCA model

## 6.1	Introduction

The VNFD information model specified by ETSI GS NFV-IFA 011 [1] is mapped to the TOSCA concepts. The VNFD is represented as one or more TOSCA service templates to be used by the VNFM for deploying and managing the lifecycle of a VNF instance.

Table 6.1-1 describes the mapping of the main information elements defined in ETSI GS NFV-IFA 011 [1] applicable to a VNFD and the corresponding NFV-specific TOSCA Types, as well the basic TOSCA types defined in TOSCA Simple YAML Profile [3] from which they are derived from. The full definition of all types can be found in the following clauses.

NOTE:	There is only part of VNFD features as specified in ETSI GS NFV-IFA 011 [1] is included in this version of the present document, the following are features not included:

VnfIndicators

autoScale

Table 6.1-1: Mapping of ETSI GS NFV-IFA 011 [1] information elements with TOSCA types

| ETSI GS NFV-IFA 011 [1] Elements | VNFD TOSCA types | Derived from |
| --- | --- | --- |
| VNFD | tosca.nodes.nfv.VNF | tosca.nodes.Root |
| Vdu | n/a (see note 1) | n/a |
| Cpd (Connection Point) | tosca.nodes.nfv.Cp | tosca.nodes.Root |
| VduCpd (internal connection point) | tosca.nodes.nfv.VduCp | tosca.nodes.nfv.Cp |
| VnfVirtualLinkDesc (Virtual Link) | tosca.nodes.nfv.VnfVirtualLink | tosca.nodes.Root |
| VnfExtCpd (External Connection Point) | tosca.nodes.nfv.VnfExtCp tosca.nodes.nfv.VduCp | tosca.nodes.nfv.Cp |
| VirtualStorageDesc | tosca.nodes.nfv.Vdu.VirtualBlockStorage tosca.nodes.nfv.Vdu.VirtualObjectStorage tosca.nodes.nfv.Vdu.VirtualFileStorage | tosca.nodes.Root |
| VirtualComputeDesc | tosca.nodes.nfv.Vdu.Compute | tosca.nodes.Root |
| SwImageDesc | tosca.artifacts.nfv.SwImage | tosca.artifacts.Deployment.Image |
| VnfDf | n/a (see note 2) | n/a |
| SecurityGroupRule | tosca.policies.nfv.SecurityGroupRule | tosca.policies.Root |
| VnfConfigurableProperties | tosca.datatypes.nfv.VnfConfigurableProperties | tosca.datatypes.Root |
| VnfInfoModifiableAttributes | tosca.datatypes.nfv.VnfInfoModifiableAttributes | tosca.datatypes.Root |
| NOTE 1: The Vdu information element is represented as a collection of tosca.nodes.nfv.VduCp, tosca.nodes.nfv.Vdu.Compute, tosca.nodes.nfv.Vdu.VirtualBlockStorage, tosca.nodes.nfv.Vdu.VirtualObjectStorage and tosca.nodes.nfv.Vdu.VirtualFileStorage types. NOTE 2: The VnfDf information element is represented as a TOSCA service template. |  |  |

Figure 6.1-1 provides an overview of the TOSCA node types used to build a service template representing a VNFD for a specific deployment flavour, and of the relationship between them. The figure shows one of the three types of virtual storage. A detailed description is provided in clause 6.11.

Figure 6.1-1: Service template VNFD overview

## 6.2	Data Types

### 6.2.1	tosca.datatypes.nfv.L2AddressData

#### 6.2.1.1	Description

The L2AddressData data type describes the information on the MAC addresses to be assigned to a connection point as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.1.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.1.1-1: Type name, shorthand, and URI

| Shorthand Name | L2AddressData |
| --- | --- |
| Type Qualified Name | toscanfv:L2AddressData |
| Type URI | tosca.datatypes.nfv.L2AddressData |

#### 6.2.1.2	Properties

The properties of the L2AddressData data type shall comply with the provisions set out in table 6.2.1.2-1.

Table 6.2.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| mac_address_assignment | yes | boolean |  | Specify if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility. |

#### 6.2.1.3	Definition

The syntax of the L2AddressData data type shall comply with the following definition:

tosca.datatypes.nfv.L2AddressData: derived_from: tosca.datatypes.Root description: Describes the information on the MAC addresses to be assigned to a connection point. properties: mac_address_assignment: type: boolean description: Specifies if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility required: true

#### 6.2.1.4	Examples

<some_tosca_entity>: properties: l2_address_data: mac_address_assignment: true

#### 6.2.1.5	Additional Requirements

None.

### 6.2.2	tosca.datatypes.nfv.L3AddressData

#### 6.2.2.1	Description

The L3AddressData data type supports providing information about Layer 3 level addressing scheme and parameters applicable to a CP, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.2.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.2.1-1: Type name, shorthand, and URI

| Shorthand Name | L3AddressData |
| --- | --- |
| Type Qualified Name | toscanfv:L3AddressData |
| Type URI | tosca.datatypes.nfv.L3AddressData |

#### 6.2.2.2	Properties

The properties of the L3AddressData data type shall comply with the provisions set out in table 6.2.2.2-1.

Table 6.2.2.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ip_address_assignment | yes | boolean |  | Specify if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility. |
| floating_ip_activated | yes | boolean |  | Specify if the floating IP scheme is activated on the Connection Point or not. |
| ip_address_type | no | string | Valid values: ipv4, ipv6 | Define address type. The address type should be aligned with the address type supported by the layer_protocols properties of the parent VnfExtCp. |
| number_of_ip_address | no | Integer | greater_than: 0 | Minimum number of IP addresses to be assigned. |

#### 6.2.2.3	Definition

The syntax of the L3AddressData data type shall comply with the following definition:

tosca.datatypes.nfv.L3AddressData: derived_from: tosca.datatypes.Root description: Provides information about Layer 3 level addressing scheme and parameters applicable to a CP properties: ip_address_assignment: type: boolean description: Specifies if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility required: true floating_ip_activated: type: boolean description: Specifies if the floating IP scheme is activated on the Connection Point or not required: true ip_address_type: type: string description: Defines address type. The address type should be aligned with the address type supported by the layer_protocols properties of the parent VnfExtCp required: false constraints: - valid_values: [ ipv4, ipv6 ] number_of_ip_address: type: integer description: Minimum number of IP addresses to be assigned required: false constraints: - greater_than: 0

#### 6.2.2.4	Examples

<some_tosca_entity>: properties: l3_address_data: ip_address_assignment: true floating_ip_activated: true ip_address_type: ipv4 number_of_ip_address: 4

#### 6.2.2.5	Additional Requirements

None.

### 6.2.3	tosca.datatypes.nfv.AddressData

#### 6.2.3.1	Description

The AddressData data type describes information about the addressing scheme and parameters applicable to a CP, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.3.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.3.1-1: Type name, shorthand, and URI

| Shorthand Name | AddressData |
| --- | --- |
| Type Qualified Name | toscanfv:AddressData |
| Type URI | tosca.datatypes.nfv.AddressData |

#### 6.2.3.2	Properties

The properties of the AddressData data type shall comply with the provisions set out in table 6.2.3.2-1.

Table 6.2.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| address_type | yes | string | Valid values: mac_address ip_address. | Describes the type of the address to be assigned to a connection point The content type shall be aligned with the address type supported by the layerProtocol property of the connection point. |
| l2_address_data | no | tosca.datatypes.nfv.L2AddressData | Shall be present when the address_type is mac_address. | Provides the information on the MAC addresses to be assigned to a connection point. |
| l3_address_data | no | tosca.datatypes.nfv. L3AddressData | Shall be present when the address_type is ip_address. | Provides the information on the IP addresses to be assigned to a connection point. |

#### 6.2.3.3	Definition

The syntax of the AddressData data type shall comply with the following definition:

tosca.datatypes.nfv.AddressData: derived_from: tosca.datatypes.Root description: Describes information about the addressing scheme and parameters applicable to a CP properties: address_type: type: string description: Describes the type of the address to be assigned to a connection point. The content type shall be aligned with the address type supported by the layerProtocol property of the connection point required: true constraints: - valid_values: [ mac_address, ip_address ] l2_address_data: type: tosca.datatypes.nfv.L2AddressData description: Provides the information on the MAC addresses to be assigned to a connection point. required: false l3_address_data: type: tosca.datatypes.nfv.L3AddressData description: Provides the information on the IP addresses to be assigned to a connection point required: false

#### 6.2.3.4	Examples

<some_tosca_entity>: properties: address_data: address_type: ip_address l3_address_data: ip_address_assignment: true floating_ip_activated: true ip_address_type: ipv4 number_of_ip_address: 4

#### 6.2.3.5	Additional Requirements

None.

### 6.2.4	tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements

#### 6.2.4.1	Description

The VirtualNetworkInterfaceRequirements data type describes requirements on a virtual network interface, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.4.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.4.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualNetworkInterfaceRequirements |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualNetworkInterfaceRequirements |
| Type URI | tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements |

#### 6.2.4.2	Properties

The properties of the VirtualNetworkInterfaceRequirements data type shall comply with the provisions set out in table 6.2.4.2-1.

Table 6.2.4.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Provides a human readable name for the requirement. |
| description | no | string |  | Provides a human readable description of the requirement. |
| support_mandatory | yes | boolean |  | Indicates whether fulfilling the constraint is mandatory (TRUE) for successful operation or desirable (FALSE). |
| network_interface_requirements | yes | map of string |  | The network interface requirements. A map of strings that contain a set of key-value pairs that describes the hardware platform specific network interface deployment requirements. More information regarding the usage of this property is available at: https://register.etsi.org |
| nic_io_requirements | no | tosca.datatypes.nfv.LogicalNodeData |  | This references (couples) the CP with any logical node I/O requirements (for network devices) that may have been created. Linking these attributes is necessary so that so that I/O requirements that need to be articulated at the logical node level can be associated with the network interface requirements associated with the CP. |

#### 6.2.4.3	Definition

The syntax of the VirtualNetworkInterfaceRequirements data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements: derived_from: tosca.datatypes.Root description: Describes requirements on a virtual network interface properties: name: type: string description: Provides a human readable name for the requirement. required: false description: type: string description: Provides a human readable description of the requirement. required: false support_mandatory: type: boolean description: Indicates whether fulfilling the constraint is mandatory (TRUE) for successful operation or desirable (FALSE). required: true network_interface_requirements: type: map description: The network interface requirements. A map of strings that contain a set of key-value pairs that describes the hardware platform specific network interface deployment requirements. required: true entry_schema: type: string nic_io_requirements: type: tosca.datatypes.nfv.LogicalNodeData description: references (couples) the CP with any logical node I/O requirements (for network devices) that may have been created. Linking these attributes is necessary so that so that I/O requirements that need to be articulated at the logical node level can be associated with the network interface requirements associated with the CP. required: false

#### 6.2.4.4	Examples

None.

#### 6.2.4.5	Additional Requirements

None.

### 6.2.5	tosca.datatypes.nfv.ConnectivityType

#### 6.2.5.1	Description

The ConnectivityType data type describes additional connectivity information of a virtualLink, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.5.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.5.1-1: Type name, shorthand, and URI

| Shorthand Name | ConnectivityType |
| --- | --- |
| Type Qualified Name | toscanfv:ConnectivityType |
| Type URI | tosca.datatypes.nfv.ConnectivityType |

#### 6.2.5.2	Properties

The properties of the ConnectivityType shall comply with the provisions set out in table 6.2.5.2-1.

Table 6.2.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| layer_protocols | yes | list of string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | Identifies the protocol a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire). The top layer protocol of the virtualLink protocol stack shall always be provided. The lower layer protocols may be included when there are specific requirements on these layers. |
| flow_pattern | no | string | Valid values: line, tree, mesh | Identifies the flow pattern of the connectivity. |

#### 6.2.5.3	Definition

The syntax of the ConnectivityType data type shall comply with the following definition:

tosca.datatypes.nfv.ConnectivityType: derived_from: tosca.datatypes.Root description: describes additional connectivity information of a virtualLink properties: layer_protocols: type: list description: Identifies the protocol a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire). The top layer protocol of the virtualLink protocol stack shall always be provided. The lower layer protocols may be included when there are specific requirements on these layers. required: true entry_schema: type: string constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] flow_pattern: type: string description: Identifies the flow pattern of the connectivity required: false constraints: - valid_values: [ line, tree, mesh ]

#### 6.2.5.4	Examples

<some_tosca_entity>: properties: connectivity_type: layer_protocol: - ipv4 flow_pattern: mesh

#### 6.2.5.5	Additional Requirements

None.

### 6.2.6	tosca.datatypes.nfv.RequestedAdditionalCapability

#### 6.2.6.1	Description

The RequestedAdditionalCapability data type describes requested additional capability for a particular VDU, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.6.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.6.1-1: Type name, shorthand, and URI

| Shorthand Name | RequestedAdditionalCapability |
| --- | --- |
| Type Qualified Name | toscanfv:RequestedAdditionalCapability |
| Type URI | tosca.datatypes.nfv.RequestedAdditionalCapability |

#### 6.2.6.2	Properties

The properties of the RequestedAdditionalCapability data type shall comply with the provisions set out in table 6.2.6.2-1.

Table 6.2.6.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| requested_additional_capability_name | yes | string |  | Identifies a requested additional capability for the VDU. |
| support_mandatory | yes | boolean |  | Indicates whether the requested additional capability is mandatory for successful operation. |
| min_requested_additional_capability_version | no | string |  | Identifies the minimum version of the requested additional capability. |
| preferred_requested_additional_capability_version | no | string |  | Identifies the preferred version of the requested additional capability. |
| target_performance_parameters | yes | map of string |  | Identifies specific attributes, dependent on the requested additional capability type. |

#### 6.2.6.3	Definition

The syntax of the RequestedAdditionalCapability data type shall comply with the following definition:

tosca.datatypes.nfv.RequestedAdditionalCapability: derived_from: tosca.datatypes.Root description: describes requested additional capability for a particular VDU properties: requested_additional_capability_name: type: string description: Identifies a requested additional capability for the VDU. required: true support_mandatory: type: boolean description: Indicates whether the requested additional capability is mandatory for successful operation. required: true min_requested_additional_capability_version: type: string description: Identifies the minimum version of the requested additional capability. required: false preferred_requested_additional_capability_version: type: string description: Identifies the preferred version of the requested additional capability. required: false target_performance_parameters: type: map description: Identifies specific attributes, dependent on the requested additional capability type. required: true entry_schema: type: string

#### 6.2.6.4	Examples

None.

#### 6.2.6.5	Additional Requirements

None.

### 6.2.7	tosca.datatypes.nfv.VirtualMemory

#### 6.2.7.1	Description

The VirtualMemory data type supports the specification of requirements related to virtual memory of a virtual compute resource, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.7.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.7.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualMemory |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualMemory |
| Type URI | tosca.datatypes.nfv.VirtualMemory |

#### 6.2.7.2	Properties

The properties of the VirtualMemory data type shall comply with the provisions set out in table 6.2.7.2-1.

Table 6.2.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_mem_size | yes | scalar-unit.size |  | Amount of virtual memory. |
| virtual_mem_oversubscription_policy | no | string |  | The memory core oversubscription policy in terms of virtual memory to physical memory on the platform. |
| vdu_mem_requirements | no | map of string |  | The hardware platform specific VDU memory requirements. A map of strings that contains a set of key-value pairs that describes hardware platform specific VDU memory requirements. More information regarding the usage of this property is available at: https://register.etsi.org |
| numa_enabled | no | boolean | default: false | It specifies the memory allocation to be cognisant of the relevant process/core allocation. |

#### 6.2.7.3	Definitions

The syntax of the VirtualMemory data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualMemory: derived_from: tosca.datatypes.Root description: supports the specification of requirements related to virtual memory of a virtual compute resource properties: virtual_mem_size: type: scalar-unit.size description: Amount of virtual memory. required: true virtual_mem_oversubscription_policy: type: string description: The memory core oversubscription policy in terms of virtual memory to physical memory on the platform. required: false vdu_mem_requirements: type: map description: The hardware platform specific VDU memory requirements. A map of strings that contains a set of key-value pairs that describes hardware platform specific VDU memory requirements. required: false entry_schema: type: string numa_enabled: type: boolean description: It specifies the memory allocation to be cognisant of the relevant process/core allocation. required: false default: false

#### 6.2.7.4	Examples

None.

#### 6.2.7.5	Additional Requirements

None.

### 6.2.8	tosca.datatypes.nfv.VirtualCpu

#### 6.2.8.1	Description

The VirtualCpu data type supports the specification of requirements related to virtual CPU(s) of a virtual compute resource, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.8.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.8.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualCpu |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualCpu |
| Type URI | tosca.datatypes.nfv.VirtualCpu |

#### 6.2.8.2	Properties

The properties of the VirtualCpu data type shall comply with the provisions set out in table 6.2.8.2-1.

Table 6.2.8.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| cpu_architecture | no | string |  | CPU architecture type. Examples are x86, ARM. |
| num_virtual_cpu | yes | integer | greater_than: 0 | Number of virtual CPUs. |
| virtual_cpu_clock | no | scalar-unit.frequency |  | Minimum virtual CPU clock rate. |
| virtual_cpu_oversubscription_policy | no | string |  | CPU core oversubscription policy e.g. the relation of virtual CPU cores to physical CPU cores/threads. |
| vdu_cpu_requirements | no | map of string |  | The hardware platform specific VDU CPU requirements. A map of strings that contains a set of key-value pairs describing VDU CPU specific hardware platform requirements. More information regarding the usage of this property is available at: https://register.etsi.org. |
| virtual_cpu_pinning | no | tosca.datatypes.nfv.VirtualCpuPinning |  | The virtual CPU pinning configuration for the virtualised compute resource. |

#### 6.2.8.3	Definition

The syntax of the VirtualCpu data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualCpu: derived_from: tosca.datatypes.Root description: Supports the specification of requirements related to virtual CPU(s) of a virtual compute resource properties: cpu_architecture: type: string description: CPU architecture type. Examples are x86, ARM required: false num_virtual_cpu: type: integer description: Number of virtual CPUs required: true constraints: - greater_than: 0 virtual_cpu_clock: type: scalar-unit.frequency description: Minimum virtual CPU clock rate required: false virtual_cpu_oversubscription_policy: type: string description: CPU core oversubscription policy e.g. the relation of virtual CPU cores to physical CPU cores/threads. required: false vdu_cpu_requirements: type: map description: The hardware platform specific VDU CPU requirements. A map of strings that contains a set of key-value pairs describing VDU CPU specific hardware platform requirements. required: false entry_schema: type: string virtual_cpu_pinning: type: tosca.datatypes.nfv.VirtualCpuPinning description: The virtual CPU pinning configuration for the virtualised compute resource. required: false

#### 6.2.8.4	Examples

None.

#### 6.2.8.5	Additional Requirements

None.

### 6.2.9	tosca.datatypes.nfv.VirtualCpuPinning

#### 6.2.9.1	Description

The VirtualCpuPinning data type supports the specification of requirements related to the virtual CPU pinning configuration of a virtual compute resource, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.9.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.9.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualCpuPinning |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualCpuPinning |
| Type URI | tosca.datatypes.nfv.VirtualCpuPinning |

#### 6.2.9.2	Properties

The properties of the VirtualCpuPinning data type shall comply with the provisions set out in table 6.2.9.2-1.

Table 6.2.9.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_cpu_pinning_policy | no | string | Valid values: static, dynamic | Indicates the policy for CPU pinning. The policy can take values of "static" or "dynamic". In case of "dynamic" the allocation of virtual CPU cores to logical CPU cores is decided by the VIM. (e.g. SMT (Simultaneous Multi-Threading) requirements). In case of "static" the allocation is requested to be according to the virtual_cpu_pinning_rule. |
| virtual_cpu_pinning_rule | no | list of string |  | Provides the list of rules for allocating virtual CPU cores to logical CPU cores/threads. |

#### 6.2.9.3	Definition

The syntax of the VirtualCpuPinning data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualCpuPinning: derived_from: tosca.datatypes.Root description: Supports the specification of requirements related to the virtual CPU pinning configuration of a virtual compute resource properties: virtual_cpu_pinning_policy: type: string description: Indicates the policy for CPU pinning. The policy can take values of "static" or "dynamic". In case of "dynamic" the allocation of virtual CPU cores to logical CPU cores is decided by the VIM. (e.g. SMT (Simultaneous Multi-Threading) requirements). In case of "static" the allocation is requested to be according to the virtual_cpu_pinning_rule. required: false constraints: - valid_values: [ static, dynamic ] virtual_cpu_pinning_rule: type: list description: Provides the list of rules for allocating virtual CPU cores to logical CPU cores/threads required: false entry_schema: type: string

#### 6.2.9.4	Examples

None.

#### 6.2.9.5	Additional Requirements

The virtual_cpu_pinning_rule shall be included if the virtual_cpu_pinning_policy property is set to "static" and shall be absent otherwise.

### 6.2.10	tosca.datatypes.nfv.VnfcConfigurableProperties

#### 6.2.10.1	Description

The VnfcConfigurableProperties data type defines the configurable properties of a VNFC, as defined in ETSI GS NFV-IFA 011 [1].

Table 6.2.10.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.10.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfcConfigurableProperties |
| --- | --- |
| Type Qualified Name | toscanfv:VnfcConfigurableProperties |
| Type URI | tosca.datatypes.nfv.VnfcConfigurableProperties |

#### 6.2.10.2	Properties

The properties of the VnfcconfigurableProperties shall comply with the provisions set out in table 6.2.10.2-1.

Table 6.2.10.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| additional_vnfc_configurable_properties | no | tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties |  | Describes additional configuration for VNFC that can be modified using the ModifyVnfInfo operation. |

#### 6.2.10.3	Definition

The syntax of the VnfcConfigurableProperties data type shall comply with the following definition:

tosca.datatypes.nfv.VnfcConfigurableProperties: derived_from: tosca.datatypes.Root description: Defines the configurable properties of a VNFC #properties: # additional_vnfc_configurable_properties: # type: tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties # description: Describes additional configuration for VNFC that can be modified using the ModifyVnfInfo operation # required: false # derived types are expected to introduce # additional_vnfc_configurable_properties with its type derived from # tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties

#### 6.2.10.4	Examples

tosca_definitions_version: tosca_simple_yaml_1_2 node_types: MyCompany.nodes.nfv.Vdu.Aux: derived_from: tosca.nodes.nfv.Vdu.Compute properties: configurable_properties: type: MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties data_types: MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties: derived_from: tosca.datatypes.nfv.VnfcConfigurableProperties properties: additional_vnfc_configurable_properties: type: MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties required: true MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties: derived_from: tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties properties: name_prefix_in_vim: type: string required: false dns_server: type: string required: true topology_template: .. node_templates: aux: type: MyCompany.nodes.nfv.Vdu.Aux properties: ..

#### 6.2.10.5	Additional Requirements

None.

### 6.2.11	tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties

#### 6.2.11.1	Description

The VnfcAdditionalConfigurableProperties type is an empty base type for deriving data types for describing additional configurable properties for a given VNFC. Table 6.2.11.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.11.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfcAdditionalConfigurableProperties |
| --- | --- |
| Type Qualified Name | toscanfv:VnfcAdditionalConfigurableProperties |
| Type URI | tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties |

#### 6.2.11.2	Properties

None.

#### 6.2.11.3	Definition

The syntax of the VnfcAdditionalConfigurableProperties data type shall comply with the following definition:

tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties: derived_from: tosca.datatypes.Root description: VnfcAdditionalConfigurableProperties type is an empty base type for deriving data types for describing additional configurable properties for a given VNFC.

### 6.2.12	tosca.datatypes.nfv.VduProfile

#### 6.2.12.1	Description

The VduProfile data type describes additional instantiation data for a given Vdu.Compute used in a specific deployment flavour. Table 6.2.12.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.12.1-1: Type name, shorthand, and URI

| Shorthand Name | VduProfile |
| --- | --- |
| Type Qualified Name | toscanfv:VduProfile |
| Type URI | tosca.datatypes.nfv.VduProfile |

#### 6.2.12.2	Properties

The properties of the VduProfile data type shall comply with the provisions set out in table 6.2.12.2-1.

Table 6.2.12.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| min_number_of_instances | yes | integer | greater_or_equal: 0 | Minimum number of instances of the VNFC based on this Vdu.Compute that is permitted to exist for a particular VNF deployment flavour. |
| max_number_of_instances | yes | integer | greater_or_equal: 0 | Maximum number of instances of the VNFC based on this Vdu.Compute that is permitted to exist for a particular VNF deployment flavour. |
| NOTE: A vduId property, which exists in ETSI GS NFV-IFA 011 [1] is not needed, as the VduProfile is contained in the Vdu.Compute node. |  |  |  |  |

#### 6.2.12.3	Definition

The syntax of the VduProfile data type shall comply with the following definition:

tosca.datatypes.nfv.VduProfile: derived_from: tosca.datatypes.Root description: describes additional instantiation data for a given Vdu.Compute used in a specific deployment flavour. properties: min_number_of_instances: type: integer description: Minimum number of instances of the VNFC based on this Vdu.Compute that is permitted to exist for a particular VNF deployment flavour. required: true constraints: - greater_or_equal: 0 max_number_of_instances: type: integer description: Maximum number of instances of the VNFC based on this Vdu.Compute that is permitted to exist for a particular VNF deployment flavour. required: true constraints: - greater_or_equal: 0

#### 6.2.12.4	Examples

tosca_definitions_version: tosca_simple_yaml_1_2 topology template: … node_templates: VDU_A: type: tosca.nodes.nfv.Vdu.Compute properties: vdu_profile: min_number_of_instances: 2 max_number_of_instances: 6 # other properties omitted for brevity requirements: - virtual_storage: VirtualStorage_A1 - virtual_storage: VirtualStorage_A2 capabilities: virtual_binding: VirtualStorage_A1: type: tosca.nodes.nfv.Vdu.VirtualBlockStorage properties: # omitted for brevity capabilities: virtual_storage VirtualStorage_A2: type: tosca.nodes.nfv.Vdu.VirtualBlockStorage properties: # omitted for brevity capabilities: virtual_storage VduCp_A1: type: tosca.nodes.nfv.VduCp properties: # omitted for brevity requirements: - virtual_binding: VDU_A - virtual_link VduCp_A2: type: tosca.nodes.nfv.VduCp properties: # omitted for brevity requirements: - virtual_binding: VDU_A - virtual_link

Above snippet shows part of a topology template. The VDU_A node template is a Vdu.Compute node that is connected to two VirtualBlockStorage nodes: VirtualStorage_A1 and VirtualStorage_A2. It also has two VduCps: VduCp_A1 and Vdu_CpA2.

The minimum number of instances of VDU_A that are permitted to exist is 2. Likewise, the minimum number of instances of VirtualStorage_A1, VirtualStorage_A2, VduCp_A1 and VduCp_A2 that are permitted to exist is 2.

The maximum number of instances of VDU_A that are permitted to exist is 6. Likewise, the maximum number of instances of VirtualStorage_A1, VirtualStorage_A2, VduCp_A1 and VduCp_A2 that are permitted to exist is 6.

#### 6.2.12.5	Additional requirements

The properties of the vdu_profile indicate the maximum and minimum number of Vdu.Compute instances that are permitted to exist, created from a given Vdu.Compute node template during its lifecycle, as well as:

The maximum and minimum number of instances of each VirtualBlockStorage, VirtualObjectStorage and VirtualFileStorage nodes connected to the Vdu.Compute via one particular occurrence of the virtual_storage requirement.

The maximum and minimum number instances of each VduCp node connected to the Vdu.Compute via one particular occurrence of the virtual_binding capability.

### 6.2.13	tosca.datatypes.nfv.VlProfile

#### 6.2.13.1	Description

The VlProfile data type describes additional instantiation data for a given VL used in a specific VNF deployment flavour. Table 6.2.13.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.13.1-1: Type name, shorthand, and URI

| Shorthand Name | VlProfile |
| --- | --- |
| Type Qualified Name | toscanfv:VlProfile |
| Type URI | tosca.datatypes.nfv.VlProfile |

#### 6.2.13.2	Properties

The properties of the VlProfile data type shall comply with the provisions set out in table 6.2.13.2-1.

Table 6.2.13.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| max_bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Specifies the maximum bitrate requirements for a VL instantiated according to this profile. |
| min_bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Specifies the minimum bitrate requirements for a VL instantiated according to this profile. |
| qos | no | tosca.datatypes.nfv.Qos |  | Specifies the QoS requirements of a VL instantiated according to this profile. |
| virtual_link_protocol_data | no | list of tosca.datatypes.nfv.VirtualLinkProtocolData |  | Specifies the protocol data for a virtual link. |
| NOTE: A vnfVirtualLinkDescId property, which exists in ETSI GS NFV-IFA 011 [1] is not needed, as the VLProfile is contained in the VL node. |  |  |  |  |

#### 6.2.13.3	Definition

The syntax of the VlProfile data type shall comply with the following definition:

tosca.datatypes.nfv.VlProfile: derived_from: tosca.datatypes.Root description: Describes additional instantiation data for a given VL used in a specific VNF deployment flavour. properties: max_bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Specifies the maximum bitrate requirements for a VL instantiated according to this profile. required: true min_bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Specifies the minimum bitrate requirements for a VL instantiated according to this profile. required: true qos: type: tosca.datatypes.nfv.Qos description: Specifies the QoS requirements of a VL instantiated according to this profile. required: false virtual_link_protocol_data: type: list description: Specifies the protocol data for a virtual link. required: false entry_schema: type: tosca.datatypes.nfv.VirtualLinkProtocolData

#### 6.2.13.4	Examples

None.

#### 6.2.13.5	Additional Requirements

None.

### 6.2.14	tosca.datatypes.nfv.VirtualLinkProtocolData

#### 6.2.14.1	Description

The VirtualLinkProtocolData data type describes one protocol layer and associated protocol data for a given virtual link used in a specific VNF deployment flavour. Table 6.2.14.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.14.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkProtocolData |
| Type URI | tosca.datatypes.nfv.VirtualLinkProtocolData |

#### 6.2.14.2	Properties

The properties of the VirtualLinkProtocolData data type shall comply with the provisions set out in table 6.2.14.2-1.

Table 6.2.14.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| associated_layer_protocol | yes | string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | Identifies one of the protocols a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire) as specified by the connectivity_type property. |
| l2_protocol_data | no | tosca.datatypes.nfv.L2ProtocolData |  | Specifies the L2 protocol data for a virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L2 protocol and shall be absent otherwise. |
| l3_protocol_data | no | tosca.datatypes.nfv.L3ProtocolData |  | Specifies the L3 protocol data for this virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L3 protocol and shall be absent otherwise. |

#### 6.2.14.3	Definition

The syntax of the VirtualLinkProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualLinkProtocolData: derived_from: tosca.datatypes.Root description: describes one protocol layer and associated protocol data for a given virtual link used in a specific VNF deployment flavour properties: associated_layer_protocol: type: string description: Identifies one of the protocols a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire) as specified by the connectivity_type property. required: true constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] l2_protocol_data: type: tosca.datatypes.nfv.L2ProtocolData description: Specifies the L2 protocol data for a virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L2 protocol and shall be absent otherwise. required: false l3_protocol_data: type: tosca.datatypes.nfv.L3ProtocolData description: Specifies the L3 protocol data for this virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L3 protocol and shall be absent otherwise. required: false

#### 6.2.14.4	Examples

None.

#### 6.2.14.5	Additional Requirements

None.

### 6.2.15	tosca.datatypes.nfv.L2ProtocolData

#### 6.2.15.1	Description

The L2ProtocolData data type escribes L2 protocol data for a given virtual link used in a specific VNF deployment flavour. Table 6.2.15.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.15.1-1: Type name, shorthand, and URI

| Shorthand Name | L2ProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:L2ProtocolData |
| Type URI | tosca.datatypes.nfv.L2ProtocolData |

#### 6.2.15.2	Properties

The properties of the L2ProtocolData data type shall comply with the provisions set out in table 6.2.15.2-1.

Table 6.2.15.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Identifies the network name associated with this L2 protocol. |
| network_type | no | string | Valid values: flat, vlan, vxlan, gre | Specifies the network type for this L2 protocol. The value may be overridden at run-time. |
| vlan_transparent | no | boolean | default: false | Specifies whether to support VLAN transparency for this L2 protocol or not. |
| mtu | no | integer | greater_than : 0 | Specifies the maximum transmission unit (MTU) value for this L2 protocol. |

#### 6.2.15.3	Definition

The syntax of the L2ProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.L2ProtocolData: derived_from: tosca.datatypes.Root description: describes L2 protocol data for a given virtual link used in a specific VNF deployment flavour. properties: name: type: string description: Identifies the network name associated with this L2 protocol. required: false network_type: type: string description: Specifies the network type for this L2 protocol. The value may be overridden at run-time. required: false constraints: - valid_values: [ flat, vlan, vxlan, gre ] vlan_transparent: type: boolean description: Specifies whether to support VLAN transparency for this L2 protocol or not. required: false default: false mtu: type: integer description: Specifies the maximum transmission unit (MTU) value for this L2 protocol. required: false constraints: - greater_than: 0

#### 6.2.15.4	Examples

None.

#### 6.2.15.5	Additional Requirements

None.

### 6.2.16	tosca.datatypes.nfv.L3ProtocolData

#### 6.2.16.1	Description

The L3ProtocolData data type describes L3 protocol data for a given virtual link used in a specific VNF deployment flavour. Table 6.2.16.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.16.1-1: Type name, shorthand, and URI

| Shorthand Name | L3ProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:L3ProtocolData |
| Type URI | tosca.datatypes.nfv.L3ProtocolData |

#### 6.2.16.2	Properties

The properties of the L3ProtocolData data type shall comply with the provisions set out in table 6.2.16.2-1.

Table 6.2.16.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Identifies the network name associated with this L3 protocol. |
| ip_version | yes | string | Valid values: ipv4 ipv6 | Specifies IP version of this L3 protocol. The value of the ip_version property shall be consistent with the value of the layer_protocol in the connectivity_type property of the virtual link node. |
| cidr | yes | string |  | Specifies the CIDR (Classless Inter-Domain Routing) of this L3 protocol. The value may be overridden at run-time. |
| ip_allocation_pools | no | list of tosca.datatypes.nfv.IpAllocationPool |  | Specifies the allocation pools with start and end IP addresses for this L3 protocol. The value may be overridden at run-time. |
| gateway_ip | no | string |  | Specifies the gateway IP address for this L3 protocol. The value may be overridden at run-time. |
| dhcp_enabled | no | boolean |  | Indicates whether DHCP (Dynamic Host Configuration Protocol) is enabled or disabled for this L3 protocol. The value may be overridden at run-time. |
| ipv6_address_mode | no | string | Valid values: slaac dhcpv6-stateful dhcpv6-stateless | Specifies IPv6 address mode. May be present when the value of the ipVersion attribute is "ipv6" and shall be absent otherwise. The value may be overridden at run-time. |

#### 6.2.16.3	Definition

The syntax of the L3ProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.L3ProtocolData: derived_from: tosca.datatypes.Root description: describes L3 protocol data for a given virtual link used in a specific VNF deployment flavour. properties: name: type: string description: Identifies the network name associated with this L3 protocol. required: false ip_version: type: string description: Specifies IP version of this L3 protocol. The value of the ip_version property shall be consistent with the value of the layer_protocol in the connectivity_type property of the virtual link node. required: true constraints: - valid_values: [ ipv4, ipv6 ] cidr: type: string description: Specifies the CIDR (Classless Inter-Domain Routing) of this L3 protocol. The value may be overridden at run-time. required: true ip_allocation_pools: type: list description: Specifies the allocation pools with start and end IP addresses for this L3 protocol. The value may be overridden at run-time. required: false entry_schema: type: tosca.datatypes.nfv.IpAllocationPool gateway_ip: type: string description: Specifies the gateway IP address for this L3 protocol. The value may be overridden at run-time. required: false dhcp_enabled: type: boolean description: Indicates whether DHCP (Dynamic Host Configuration Protocol) is enabled or disabled for this L3 protocol. The value may be overridden at run-time. required: false ipv6_address_mode: type: string description: Specifies IPv6 address mode. May be present when the value of the ipVersion attribute is "ipv6" and shall be absent otherwise. The value may be overridden at run-time. required: false constraints: - valid_values: [ slaac, dhcpv6-stateful, dhcpv6-stateless ]

#### 6.2.16.4	Examples

None.

#### 6.2.16.5	Additional Requirements

None.

### 6.2.17	tosca.datatypes.nfv.IpAllocationPool

#### 6.2.17.1	Description

The IpAllocationPool data type specifies a range of IP addresses. Table 6.2.17.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.17.1-1: Type name, shorthand, and URI

| Shorthand Name | IpAllocationPool |
| --- | --- |
| Type Qualified Name | toscanfv:IpAllocationPool |
| Type URI | tosca.datatypes.nfv.IpAllocationPool |

#### 6.2.17.2	Properties

The properties of the IpAllocationPool data type shall comply with the provisions set out in table 6.2.17.2-1.

Table 6.2.17.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| start_ip_address | yes | string |  | The IP address to be used as the first one in a pool of addresses derived from the cidr block full IP range |
| end_ip_address | yes | string |  | The IP address to be used as the last one in a pool of addresses derived from the cidr block full IP range |

#### 6.2.17.3	Definition

The syntax of the IpAllocationPool data type shall comply with the following definition:

tosca.datatypes.nfv.IpAllocationPool: derived_from: tosca.datatypes.Root description: Specifies a range of IP addresses properties: start_ip_address: type: string description: The IP address to be used as the first one in a pool of addresses derived from the cidr block full IP range required: true end_ip_address: type: string description: The IP address to be used as the last one in a pool of addresses derived from the cidr block full IP range required: true

#### 6.2.17.4	Examples

None.

#### 6.2.17.5	Additional Requirements

None.

### 6.2.18	tosca.datatypes.nfv.InstantiationLevel

#### 6.2.18.1	Description

The InstantiationLevel data type describes the scale level for each aspect that corresponds to a given level of resources to be instantiated within a deployment flavour in term of the number VNFC instances . Table 6.2.18.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.18.1-1: Type name, shorthand, and URI

| Shorthand Name | InstantiationLevel |
| --- | --- |
| Type Qualified Name | toscanfv:InstantiationLevel |
| Type URI | tosca.datatypes.nfv.InstantiationLevel |

#### 6.2.18.2	Properties

The properties of the InstantiationLevel data type shall comply with the provisions set out in table 6.2.18.2-1.

Table 6.2.18.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | yes | string |  | Human readable description of the level. |
| scale_info | no | map of tosca.datatypes.nfv.ScaleInfo |  | Represents for each aspect the scale level that corresponds to this instantiation level. scale_info shall be present if the VNF supports scaling. |

#### 6.2.18.3	Definition

The syntax of the InstantiationLevel data type shall comply with the following definition:

tosca.datatypes.nfv.InstantiationLevel: derived_from: tosca.datatypes.Root description: Describes the scale level for each aspect that corresponds to a given level of resources to be instantiated within a deployment flavour in term of the number VNFC instances properties: description: type: string description: Human readable description of the level required: true scale_info: type: map # key: aspectId description: Represents for each aspect the scale level that corresponds to this instantiation level. scale_info shall be present if the VNF supports scaling. required: false entry_schema: type: tosca.datatypes.nfv.ScaleInfo

#### 6.2.18.4	Examples

See clause A.6.

#### 6.2.18.5	Additional Requirements

None.

### 6.2.19	tosca.datatypes.nfv.VduLevel

#### 6.2.19.1	Description

The VduLevel data type indicates for a given Vdu.Compute in a given level the number of instances to deploy. Table 6.2.19.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.19.1-1: Type name, shorthand, and URI

| Shorthand Name | VduLevel |
| --- | --- |
| Type Qualified Name | toscanfv:VduLevel |
| Type URI | tosca.datatypes.nfv.VduLevel |

#### 6.2.19.2	Properties

The properties of the VduLevel data type shall comply with the provisions set out in table 6.2.19.2-1.

Table 6.2.19.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| number_of_instances | yes | integer | greater_or_equal : 0 | Number of instances of VNFC based on this VDU to deploy for this level. |

#### 6.2.19.3	Definition

The syntax of the VduLevel data type shall comply with the following definition:

tosca.datatypes.nfv.VduLevel: derived_from: tosca.datatypes.Root description: Indicates for a given Vdu.Compute in a given level the number of instances to deploy properties: number_of_instances: type: integer description: Number of instances of VNFC based on this VDU to deploy for this level. required: true constraints: - greater_or_equal: 0

#### 6.2.19.4	Examples

See clause A.6.

#### 6.2.19.5	Additional Requirements

None.

### 6.2.20	tosca.datatypes.nfv.VnfLcmOperationsConfiguration

#### 6.2.20.1	Description

The VnfLcmOperationsConfiguration data type represents information to configure lifecycle management operations as specified in ETSI GS NFV-IFA 007 [i.1]. Each VNF LCM operations configuration property represents a container for all attributes that affect the invocation of the corresponding VNF Lifecycle Management operation. Table 6.2.20.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.20.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfLcmOperationsConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfLcmOperationsConfiguration |
| Type URI | tosca.datatypes.nfv.VnfLcmOperationsConfiguration |

#### 6.2.20.2	Properties

The properties of the VnfLcmOperationsConfiguration data type shall comply with the provisions set out in table 6.2.20.2-1.

Table 6.2.20.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| instantiate | no | tosca.datatypes.nfv.VnfInstantiateOperationConfiguration |  | Configuration parameters for the InstantiateVnf operation. |
| scale | no | tosca.datatypes.nfv.VnfScaleOperationConfiguration |  | Configuration parameters for the ScaleVnf operation. |
| scale_to_level | no | tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration |  | Configuration parameters for the ScaleVnfToLevel operation. |
| change_flavour | no | tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration |  | Configuration parameters for the changeVnfFlavourOpConfig operation. |
| heal | no | tosca.datatypes.nfv.VnfHealOperationConfiguration |  | Configuration parameters for the HealVnf operation. |
| terminate | no | tosca.datatypes.nfv.VnfTerminateOperationConfiguration |  | Configuration parameters for the TerminateVnf operation. |
| operate | no | tosca.datatypes.nfv.VnfOperateOperationConfiguration |  | Configuration parameters for the OperateVnf operation. |
| change_ext_connectivity | no | tosca.datatypes.nfv.VnfChangeExtConnectivityOperationConfiguration |  | Configuration parameters for the changeExtVnfConnectivityOpConfig operation. |

#### 6.2.20.3	Definition

The syntax of the VnfLcmOperationsConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfLcmOperationsConfiguration: derived_from: tosca.datatypes.Root description: Represents information to configure lifecycle management operations properties: instantiate: type: tosca.datatypes.nfv.VnfInstantiateOperationConfiguration description: Configuration parameters for the InstantiateVnf operation required: false scale: type: tosca.datatypes.nfv.VnfScaleOperationConfiguration description: Configuration parameters for the ScaleVnf operation required: false scale_to_level: type: tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration description: Configuration parameters for the ScaleVnfToLevel operation required: false change_flavour: type: tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration description: Configuration parameters for the changeVnfFlavourOpConfig operation required: false heal: type: tosca.datatypes.nfv.VnfHealOperationConfiguration description: Configuration parameters for the HealVnf operation required: false terminate: type: tosca.datatypes.nfv.VnfTerminateOperationConfiguration description: Configuration parameters for the TerminateVnf operation required: false operate: type: tosca.datatypes.nfv.VnfOperateOperationConfiguration description: Configuration parameters for the OperateVnf operation required: false change_ext_connectivity: type: tosca.datatypes.nfv.VnfChangeExtConnectivityOperationConfiguration description: Configuration parameters for the changeExtVnfConnectivityOpConfig operation required: false

#### 6.2.20.4	Examples

None.

#### 6.2.20.5	Additional Requirements

None.

### 6.2.21	tosca.datatypes.nfv.VnfInstantiateOperationConfiguration

#### 6.2.21.1	Description

The VnfInstantiateOperationConfiguration data type represents information that affect the invocation of the InstantiateVnf operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.21.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.21.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfInstantiateOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfInstantiateOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfInstantiateOperationConfiguration |

#### 6.2.21.2	Properties

None.

#### 6.2.21.3	Definition

The syntax of the VnfInstantiateOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfInstantiateOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the InstantiateVnf operation. # properties:

#### 6.2.21.4	Examples

See clause 6.8.1.9.

#### 6.2.21.5	Additional Requirements

None.

### 6.2.22	tosca.datatypes.nfv.VnfScaleOperationConfiguration

#### 6.2.22.1	Description

VnfScaleOperationConfiguration represents information that affect the invocation of the ScaleVnf operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.22.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.22.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfScaleOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfScaleOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfScaleOperationConfiguration |

#### 6.2.22.2	Properties

The properties of the VnfScaleOperationConfiguration data type shall comply with the provisions set out in table 6.2.22.2-1.

Table 6.2.22.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scaling_by_more_than_one_step_supported | no | boolean |  | Signals whether passing a value larger than one in the numScalingSteps parameter of the ScaleVnf operation is supported by this VNF. Default is FALSE, i.e. "not supported". |

#### 6.2.22.3	Definition

The syntax of the VnfScaleOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfScaleOperationConfiguration: derived_from: tosca.datatypes.Root description: Represents information that affect the invocation of the ScaleVnf operation properties: scaling_by_more_than_one_step_supported: type: boolean description: Signals whether passing a value larger than one in the numScalingSteps parameter of the ScaleVnf operation is supported by this VNF. required: false default: false

#### 6.2.22.4	Examples

See clause 6.8.1.9.

#### 6.2.22.5	Additional Requirements

None.

### 6.2.23	tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration

#### 6.2.23.1	Description

The VnfScaleToLevelOperationConfiguration data type represents information that affect the invocation of the ScaleVnfToLevel operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.23.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.23.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfScaleToLevelOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfScaleToLevelOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration |

#### 6.2.23.2	Properties

The properties of the VnfScaleToLevelOperationConfiguration data type shall comply with the provisions set out in table 6.2.23.2-1.

Table 6.2.23.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| arbitrary_target_levels_supported | yes | boolean |  | Signals whether scaling according to the parameter "scaleInfo" is supported by this VNF. |

#### 6.2.23.3	Definition

The syntax of the VnfScaleToLevelOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the ScaleVnfToLevel operation properties: arbitrary_target_levels_supported: type: boolean description: Signals whether scaling according to the parameter "scaleInfo" is supported by this VNF required: true

#### 6.2.23.4	Examples

See clause 6.8.1.9.

#### 6.2.23.5	Additional Requirements

None.

### 6.2.24	tosca.datatypes.nfv.VnfHealOperationConfiguration

#### 6.2.24.1	Description

The VnfHealOperationConfiguration data type represents information that affect the invocation of the HealVnf operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.24.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.24.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfHealOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfHealOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfHealOperationConfiguration |

#### 6.2.24.2	Properties

The properties of the VnfHealOperationConfiguration data type shall comply with the provisions set out in table 6.2.24.2-1.

Table 6.2.24.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| causes | no | list of string |  | Supported "cause" parameter values. |

#### 6.2.24.3	Definition

The syntax of the VnfHealOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfHealOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the HealVnf operation properties: causes: type: list description: Supported "cause" parameter values required: false entry_schema: type: string

#### 6.2.24.4	Examples

See clause 6.8.1.9.

#### 6.2.24.5	Additional Requirements

None.

### 6.2.25	tosca.datatypes.nfv.VnfTerminateOperationConfiguration

#### 6.2.25.1	Description

The VnfTerminateOperationConfiguration data type represents information that affect the invocation of the TerminateVnf, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.25.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.25.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfTerminateOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfTerminateOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfTerminateOperationConfiguration |

#### 6.2.25.2	Properties

The properties of the VnfTerminateOperationConfiguration data type shall comply with the provisions set out in table 6.2.25.2-1.

Table 6.2.25.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| min_graceful_termination_timeout | yes | scalar-unit.time |  | Minimum timeout value for graceful termination of a VNF instance. |
| max_recommended_graceful_termination_timeout | no | scalar-unit.time |  | Maximum recommended timeout value that can be needed to gracefully terminate a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint. |

#### 6.2.25.3	Definition

The syntax of the VnfTerminateOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfTerminateOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the TerminateVnf properties: min_graceful_termination_timeout: type: scalar-unit.time description: Minimum timeout value for graceful termination of a VNF instance required: true max_recommended_graceful_termination_timeout: type: scalar-unit.time description: Maximum recommended timeout value that can be needed to gracefully terminate a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint required: false

#### 6.2.25.4	Examples

None.

#### 6.2.25.5	Additional Requirements

None.

### 6.2.26	tosca.datatypes.nfv.VnfOperateOperationConfiguration

#### 6.2.26.1	Description

The VnfOperateOperationConfiguration data type represents information that affect the invocation of the OperateVnf operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.26.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.26.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfOperateOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfOperateOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfOperateOperationConfiguration |

#### 6.2.26.2	Properties

The properties of the VnfOperateOperationConfiguration data type shall comply with the provisions set out in table 6.2.26.2-1.

Table 6.2.26.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| min_graceful_stop_timeout | yes | scalar-unit.time |  | Minimum timeout value for graceful stop of a VNF instance. |
| max_recommended_graceful_stop_timeout | no | scalar-unit.time |  | Maximum recommended timeout value that can be needed to gracefully stop a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint. |

#### 6.2.26.3	Definition

The syntax of the VnfOperateOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfOperateOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the OperateVnf operation properties: min_graceful_stop_timeout: type: scalar-unit.time description: Minimum timeout value for graceful stop of a VNF instance required: true max_recommended_graceful_stop_timeout: type: scalar-unit.time description: Maximum recommended timeout value that can be needed to gracefully stop a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint required: false

#### 6.2.26.4	Examples

None.

#### 6.2.26.5	Additional Requirements

None.

### 6.2.27	tosca.datatypes.nfv.ScaleInfo

#### 6.2.27.1	Description

The scaleInfo data type indicates for a given scaleAspect the corresponding scaleLevel. Table 6.2.27.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.27.1-1: Type name, shorthand, and URI

| Shorthand Name | ScaleInfo |
| --- | --- |
| Type Qualified Name | toscanfv:ScaleInfo |
| Type URI | tosca.datatypes.nfv.ScaleInfo |

#### 6.2.27.2	Properties

The properties of the ScaleInfo data type shall comply with the provisions set out in table 6.2.27.2-1.

Table 6.2.27.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scale_level | yes | integer | greater_or_equal : 0 | The scale level for a particular aspect. |

#### 6.2.27.3	Definition

The syntax of the ScaleInfo data type shall comply with the following definition:

tosca.datatypes.nfv.ScaleInfo: derived_from: tosca.datatypes.Root description: Indicates for a given scaleAspect the corresponding scaleLevel properties: scale_level: type: integer description: The scale level for a particular aspect required: true constraints: - greater_or_equal: 0

#### 6.2.27.4	Examples

See clause 6.8.1.9.

#### 6.2.27.5	Additional Requirements

None.

### 6.2.28	tosca.datatypes.nfv.ScalingAspect

#### 6.2.28.1	Description

The ScalingAspect data type describes the details of an aspect used for horizontal scaling. Table 6.2.28.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.28.1-1: Type name, shorthand, and URI

| Shorthand Name | ScalingAspect |
| --- | --- |
| Type Qualified Name | toscanfv:ScalingAspect |
| Type URI | tosca.datatypes.nfv.ScalingAspect |

#### 6.2.28.2	Properties

The properties of the ScalingAspect data type shall comply with the provisions set out in table 6.2.28.2-1.

Table 6.2.28.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | yes | string |  | Human readable name of the aspect. |
| description | yes | string |  | Human readable description of the aspect. |
| max_scale_level | yes | integer | positiveInteger | Total number of scaling steps that can be applied w.r.t. this aspect. The value of this property corresponds to the number of scaling steps can be applied to this aspect when scaling it from the minimum scale level (i.e. 0) to the maximum scale level defined by this property. |
| step_deltas | no | list of string |  | List of scaling deltas to be applied for the different subsequent scaling steps of this aspect. The first entry in the array shall correspond to the first scaling step (between scale levels 0 to 1) and the last entry in the array shall correspond to the last scaling step (between maxScaleLevel-1 and maxScaleLevel). |

#### 6.2.28.3	Definition

The syntax of the ScalingAspect data type shall comply with the following definition:

tosca.datatypes.nfv.ScalingAspect: derived_from: tosca.datatypes.Root properties: name: type: string required: true description: type: string required: true max_scale_level: type: integer # positiveInteger required: true constraints: - greater_or_equal: 0 step_deltas: type: list required: false entry_schema: type: string # Identifier

#### 6.2.28.4	Examples

See clause A.6.

#### 6.2.28.5	Additional Requirements

None.

### 6.2.29	tosca.datatypes.nfv.LinkBitrateRequirements

#### 6.2.29.1	Description

The LinkBitrateRequirements data type describes the requirements in terms of bitrate for a virtual link. Table 6.2.29.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.29.1-1: Type name, shorthand, and URI

| Shorthand Name | LinkBitrateRequirements |
| --- | --- |
| Type Qualified Name | toscanfv:LinkBitrateRequirements |
| Type URI | tosca.datatypes.nfv.LinkBitrateRequirements |

#### 6.2.29.2	Properties

The properties of the LinkBitrateRequirements data type shall comply with the provisions set out in table 6.2.29.2-1.

Table 6.2.29.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| root | yes | integer | greater_or_equal: 0 | Specifies the throughput requirement in bits per second of the link (e.g. bitrate of E-Line, root bitrate of E-Tree, aggregate capacity of E-LAN). |
| leaf | no | integer | greater_or_equal: 0 | Specifies the throughput requirement in bits per second of leaf connections to the link when applicable to the connectivity type (e.g. for E-Tree and E-LAN branches). |

#### 6.2.29.3	Definition

The syntax of the LinkBitrateRequirements data type shall comply with the following definition:

tosca.datatypes.nfv.LinkBitrateRequirements: derived_from: tosca.datatypes.Root description: describes the requirements in terms of bitrate for a virtual link properties: root: type: integer # in bits per second description: Specifies the throughput requirement in bits per second of the link (e.g. bitrate of E-Line, root bitrate of E-Tree, aggregate capacity of E-LAN). required: true constraints: - greater_or_equal: 0 leaf: type: integer # in bits per second description: Specifies the throughput requirement in bits per second of leaf connections to the link when applicable to the connectivity type (e.g. for E-Tree and E LAN branches). required: false constraints: - greater_or_equal: 0

#### 6.2.29.4	Examples

None.

#### 6.2.29.5	Additional Requirements

None.

### 6.2.30	tosca.datatypes.nfv.Qos

#### 6.2.30.1	Description

The QoS describes QoS data type a given VL used in a VNF deployment flavour. Table 6.2.30.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.30.1-1: Type name, shorthand, and URI

| Shorthand Name | Qos |
| --- | --- |
| Type Qualified Name | toscanfv:Qos |
| Type URI | tosca.datatypes.nfv.Qos |

#### 6.2.30.2	Properties

The properties of the Qos data type shall comply with the provisions set out in table 6.2.30.2-1.

Table 6.2.30.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| latency | yes | scalar-unit.time | greater_than: 0 s | Specifies the maximum latency. |
| packet_delay_variation | yes | scalar-unit.time |  | Specifies the maximum jitter. |
| packet_loss_ratio | no | float | in_range: [0,1] | Specifies the maximum packet loss ratio. |

#### 6.2.30.3	Definition

The syntax of the Qos data type shall comply with the following definition:

tosca.datatypes.nfv.Qos: derived_from: tosca.datatypes.Root description: describes QoS data for a given VL used in a VNF deployment flavour properties: latency: type: scalar-unit.time #Number description: Specifies the maximum latency required: true constraints: - greater_than: 0 s packet_delay_variation: type: scalar-unit.time #Number description: Specifies the maximum jitter required: true constraints: - greater_or_equal: 0 s packet_loss_ratio: type: float description: Specifies the maximum packet loss ratio required: false constraints: - in_range: [ 0.0, 1.0 ]

#### 6.2.30.4	Examples

None.

#### 6.2.30.5	Additional Requirements

None.

### 6.2.31	tosca.datatypes.nfv.VnfConfigurableProperties

#### 6.2.31.1	Description

The VnfConfigurableProperties data type describes configurable properties for a given VNF (e.g. related to auto scaling and auto healing).

Table 6.2.31.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.31.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfConfigurableProperties |
| --- | --- |
| Type Qualified Name | toscanfv:VnfConfigurableProperties |
| Type URI | tosca.datatypes.nfv.VnfConfigurableProperties |

#### 6.2.31.2	Properties

The properties of the VnfConfigurableProperties data type shall comply with the provisions set out in table 6.2.31.2-1.

Table 6.2.31.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| is_autoscale_enabled | no | boolean |  | It permits to enable (TRUE)/disable (FALSE) the auto-scaling functionality. If the properties is not present for configuring, then VNF property is not supported |
| is_autoheal_enabled | no | boolean |  | It permits to enable (TRUE)/disable (FALSE) the auto-healing functionality. If the properties is not present for configuring, then VNF property is not supported |
| additional_configurable_property | no | tosca.datatypes.nfv.VnfAdditionalConfigurableProperties |  | It provides VNF specific configurable properties that can be modified using the ModifyVnfInfo operation |

#### 6.2.31.3	Definition

The syntax of the VnfConfigurableProperties data type shall comply with the following definition:

tosca.datatypes.nfv.VnfConfigurableProperties: derived_from: tosca.datatypes.Root description: indicates configuration properties for a given VNF (e.g. related to auto scaling and auto healing). properties: is_autoscale_enabled: type: boolean description: It permits to enable (TRUE)/disable (FALSE) the auto-scaling functionality. If the properties is not present for configuring, then VNF property is not supported required: false is_autoheal_enabled: type: boolean description: It permits to enable (TRUE)/disable (FALSE) the auto-healing functionality. If the properties is not present for configuring, then VNF property is not supported required: false # additional_configurable_properties: # description: It provides VNF specific configurable properties that can be # modified using the ModifyVnfInfo operation # required: false # type: tosca.datatypes.nfv.VnfAdditionalConfigurableProperties # derived types are expected to introduce # additional_configurable_properties with its type derived from # tosca.datatypes.nfv.VnfAdditionalConfigurableProperties

#### 6.2.31.4	Examples

Example definition of configurable properties.

tosca_definitions_version: tosca_simple_yaml_1_2 node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: constraints: - valid_values: [ simple, complex ] configurable_properties: type: MyCompany.datatypes.nfv.VnfConfigurableProperties data_types: Mycompany.datatypes.nfv.VnfConfigurableProperties: derived_from: tosca.datatypes.nfv.VnfConfigurableProperties properties: additional_configurable_properties: type: MyCompany.datatypes.nfv.VnfAdditionalConfigurableProperties MyCompany.datatypes.nfv.VnfAdditionalConfigurableProperties: derived_from: tosca.datatypes.nfv.VnfAdditionalConfigurableProperties properties: name_prefix_in_vim: type: string required: false dns_server: type: string required: true

#### 6.2.31.5	Additional Requirements

None.

### 6.2.32	tosca.datatypes.nfv.VnfAdditionalConfigurableProperties

#### 6.2.32.1	Description

The VnfAdditionalConfigurableProperties data type is an empty base type for deriving data types for describing additional configurable properties for a given VNF. Table 6.2.32.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.32.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfAdditionalConfigurableProperties |
| --- | --- |
| Type Qualified Name | toscanfv:VnfAdditionalConfigurableProperties |
| Type URI | tosca.datatypes.nfv.VnfAdditionalConfigurableProperties |

#### 6.2.32.2	Properties

None.

#### 6.2.32.3	Definition

The syntax of the VnfAdditionalConfigurableProperties data type shall comply with the following definition:

tosca.datatypes.nfv.VnfAdditionalConfigurableProperties: derived_from: tosca.datatypes.Root description: is an empty base type for deriving data types for describing additional configurable properties for a given VNF

#### 6.2.32.4	Examples

See clause 6.2.31.4.

#### 6.2.32.5	Additional Requirements

None.

### 6.2.33	tosca.datatypes.nfv.VnfInfoModifiableAttributes

#### 6.2.33.1	Description

The VnfInfoModifiableAttributes data type describes VNF-specific extension and metadata for a given VNF. Table 6.2.33.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.33.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfInfoModifiableAttributes |
| --- | --- |
| Type Qualified Name | toscanfv:VnfInfoModifiableAttributes |
| Type URI | tosca.datatypes.nfv.VnfInfoModifiableAttributes |

#### 6.2.33.2	Properties

The properties of the VnfInfoModifiableAttributes data type shall comply with the provisions set out in table 6.2.33.2-1.

Table 6.2.33.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| extensions | no | tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions |  | "Extension" properties of VnfInfo that are writeable. |
| metadata | no | tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata |  | "Metadata" properties of VnfInfo that are writeable. |

#### 6.2.33.3	Definition

The syntax of the VnfInfoModifiableAttributes data type shall comply with the following definition:

tosca.datatypes.nfv.VnfInfoModifiableAttributes: derived_from: tosca.datatypes.Root description: Describes VNF-specific extension and metadata for a given VNF #properties: #extensions: #type: tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions #description: "Extension" properties of VnfInfo that are writeable #required: false # derived types are expected to introduce # extensions with its type derived from # tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions #metadata: #type: tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata #description: "Metadata" properties of VnfInfo that are writeable #required: false # derived types are expected to introduce # metadata with its type derived from # tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata

#### 6.2.33.4	Examples

See clause 6.8.1.9.

#### 6.2.33.5	Additional Requirements

None.

### 6.2.34	tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions

#### 6.2.34.1	Description

The VnfInfoModifiableAttributesExtensions data type is an empty base type for deriving data types for describing VNF-specific extension. Table 6.2.34.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.34.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfInfoModifiableAttributesExtensions |
| --- | --- |
| Type Qualified Name | toscanfv:VnfInfoModifiableAttributesExtensions |
| Type URI | tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions |

#### 6.2.34.2	Properties

None.

#### 6.2.34.3	Definition

The syntax of the VnfInfoModifiableAttributesExtensions data type shall comply with the following definition:

tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions: derived_from: tosca.datatypes.Root description: is an empty base type for deriving data types for describing VNF-specific extension

#### 6.2.34.4	Examples

See clause 6.8.1.9.

#### 6.2.34.5	Additional Requirements

None.

### 6.2.35	tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata

#### 6.2.35.1	Description

The VnfInfoModifiableAttributesMetadata data type is an empty base type for deriving data types for describing VNF-specific metadata. Table 6.2.35.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.35.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfInfoModifiableAttributesMetadata |
| --- | --- |
| Type Qualified Name | toscanfv:VnfInfoModifiableAttributesMetadata |
| Type URI | tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata |

#### 6.2.35.2	Properties

None.

#### 6.2.35.3	Definition

The syntax of the VnfInfoModifiableAttributesMetadata data type shall comply with the following definition:

tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata: derived_from: tosca.datatypes.Root description: is an empty base type for deriving data types for describing VNF-specific metadata

#### 6.2.35.4	Examples

Example metadata definition:

tosca_definitions_version: tosca_simple_yaml_1_2 node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: constraints: - valid_values: [ simple, complex ] modifiable_attributes: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes data_types: mycompany.datatypes.nfv.VnfInfoModifiableAttributes: derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributes properties: metadata: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesMetadata mycompany.datatypes.nfv.VnfInfoModifiableAttributesMetadata: derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata properties: metadata_key_1: type: string required: false metadata_key_2: type: string required: false

#### 6.2.35.5	Additional Requirements

None.

### 6.2.36	tosca.datatypes.nfv.CpProtocolData

#### 6.2.36.1	Description

The CpProtocolData data type describes and associates the protocol layer that a CP uses together with other protocol and connection point information. Table 6.2.36.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.36.1-1: Type name, shorthand, and URI

| Shorthand Name | CpProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:CpProtocolData |
| Type URI | tosca.datatypes.nfv.CpProtocolData |

#### 6.2.36.2	Properties

The properties of the CpProtocolData data type shall comply with the provisions set out in table 6.2.36.2-1.

Table 6.2.36.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| associated_layer_protocol | yes | string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | One of the values of the property layer_protocols of the CP. |
| address_data | no | list of tosca.datatypes.nfv.AddressData |  | Provides information on the addresses to be assigned to the CP. |

#### 6.2.36.3	Definition

The syntax of the CpProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.CpProtocolData: derived_from: tosca.datatypes.Root description: Describes and associates the protocol layer that a CP uses together with other protocol and connection point information . properties: associated_layer_protocol: type: string required: true description: One of the values of the property layer_protocols of the CP constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] address_data: type: list description: Provides information on the addresses to be assigned to the CP entry_schema: type: tosca.datatypes.nfv.AddressData required: false

#### 6.2.36.4	Examples

None.

#### 6.2.36.5	Additional Requirements

None.

### 6.2.37	tosca.datatypes.nfv.LogicalNodeData

#### 6.2.37.1	Description

The LogicalNodeData data type describes compute, memory and I/Orequirements associated with a particular VDU. Table 6.2.37.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.37.1-1: Type name, shorthand, and URI

| Shorthand Name | LogicalNodeData |
| --- | --- |
| Type Qualified Name | toscanfv:LogicalNodeData |
| Type URI | tosca.datatypes.nfv.LogicalNodeData |

#### 6.2.37.2	Properties

The properties of the LogicalNodeData data type shall comply with the provisions set out in table 6.2.37.2-1.

Table 6.2.37.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| logical_node_requirements | no | map of string |  | The logical node-level compute, memory and I/O requirements. A map of strings that contains a set of key-value pairs that describes hardware platform specific deployment requirements, including the number of CPU cores on this logical node, a memory configuration specific to a logical node or a requirement related to the association of an I/O device with the logical node. More information regarding the usage of this property is available at: https://register.etsi.org |

#### 6.2.37.3	Definition

The syntax of the LogicalNodeData data type shall comply with the following definition:

tosca.datatypes.nfv.LogicalNodeData: derived_from: tosca.datatypes.Root description: Describes compute, memory and I/O requirements associated with a particular VDU. properties: logical_node_requirements: type: map description: The logical node-level compute, memory and I/O requirements. A map of strings that contains a set of key-value pairs that describes hardware platform specific deployment requirements, including the number of CPU cores on this logical node, a memory configuration specific to a logical node or a requirement related to the association of an I/O device with the logical node. required: false entry_schema: type: string

#### 6.2.37.4	Examples

None.

#### 6.2.37.5	Additional Requirements

None.

### 6.2.38	tosca.datatypes.nfv.SwImageData

#### 6.2.38.1	Description

The SwImageData data type describes information related to a software image artifact. Table 6.2.38.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.38.1-1: Type name, shorthand, and URI

| Shorthand Name | SwImageData |
| --- | --- |
| Type Qualified Name | toscanfv:SwImageData |
| Type URI | tosca.datatypes.nfv.SwImageData |

#### 6.2.38.2	Properties

The properties of the SwImageData data type shall comply with the provisions set out in table 6.2.38.2-1.

Table 6.2.38.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | yes | string |  | Name of this software image. |
| version | yes | string |  | Version of this software image. |
| checksum | yes | tosca.datatypes.nfv.ChecksumData |  | Checksum of the software image file. |
| container_format | yes | string | Valid values: [aki, ami, ari, bare, docker, ova, ovf] | The container format describes the container file format in which software image is provided. Description of valid values: aki: a kernel image ami: a machine image ari: a ramdisk image bare: the image does not have a container or metadata envelope docker: docker container format ova: OVF package in a tarfile ovf: OVF container format Future versions of the present document may extend the list of possible values. |
| disk_format | yes | string | Valid values: [aki, ami, ari, iso, qcow2, raw, vdi, vhd, vhdx, vmdk] | The disk format of a software image is the format of the underlying disk image. Description of valid values: aki: a kernel image ami: a machine image ari: a ramdisk image iso: an archive format for the data contents of an optical disc, such as CD-ROM qcow2: a common disk image format, which can expand dynamically and supports copy on write raw: an unstructured disk image format vdi: a common disk image format vhd: a common disk image format vhdx: enhanced version of VHD format vmdk: a common disk image format Future versions of the present document may extend the list of possible values. |
| min_disk | yes | scalar-unit.size | greater_or_equal: 0 B | The minimal disk size requirement for this software image. |
| min_ram | no | scalar-unit.size | greater_or_equal: 0 B | The minimal RAM requirement for this software image. |
| size | yes | scalar-unit.size |  | The size of this software image. |
| operating_system | no | string |  | Identifies the operating system used in the software image. |
| supported _virtualisation_environments | no | list of string |  | Identifies the virtualisation environments (e.g. hypervisor) compatible with this software image. |

#### 6.2.38.3	Definition

The syntax of the SwImageData data type shall comply with the following definition:

tosca.datatypes.nfv.SwImageData: derived_from: tosca.datatypes.Root description: describes information related to a software image artifact properties: # in SOL001 v0.8.0: "properties or metadata:" name: type: string description: Name of this software image required: true version: type: string description: Version of this software image required: true checksum: type: tosca.datatypes.nfv.ChecksumData description: Checksum of the software image file required: true container_format: type: string description: The container format describes the container file format in which software image is provided required: true constraints: - valid_values: [ aki, ami, ari, bare, docker, ova, ovf ] disk_format: type: string description: The disk format of a software image is the format of the underlying disk image required: true constraints: - valid_values: [ aki, ami, ari, iso, qcow2, raw, vdi, vhd, vhdx, vmdk ] min_disk: type: scalar-unit.size # Number description: The minimal disk size requirement for this software image required: true constraints: - greater_or_equal: 0 B min_ram: type: scalar-unit.size # Number description: The minimal RAM requirement for this software image required: false constraints: - greater_or_equal: 0 B size: type: scalar-unit.size # Number description: The size of this software image required: true operating_system: type: string description: Identifies the operating system used in the software image required: false supported_virtualisation_environments: type: list description: Identifies the virtualisation environments (e.g. hypervisor) compatible with this software image required: false entry_schema: type: string

#### 6.2.38.4	Examples

None.

#### 6.2.38.5	Additional Requirements

None.

### 6.2.39	tosca.datatypes.nfv.VirtualBlockStorageData

#### 6.2.39.1	Description

The VirtualBlockStorageData data type describes block storage requirements associated with compute resources in a particular VDU, either as a local disk or as virtual attached storage. Table 6.2.39.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.39.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualBlockStorageData |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualBlockStorageData |
| Type URI | tosca.datatypes.nfv.VirtualBlockStorageData |

#### 6.2.39.2	Properties

The properties of the VirtualBlockStorageData data type shall comply with the provisions set out in table 6.2.39.2-1.

Table 6.2.39.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| size_of_storage | yes | scalar-unit.size | greater_or_equal:0 B | Size of virtualised storage resource |
| vdu_storage_requirements | no | map of string |  | The hardware platform specific storage requirements. A map of strings that contains a set of key-value pairs that represents the hardware platform specific storage deployment requirements More information regarding the usage of this property is available at: https://register.etsi.org |
| rdma_enabled | no | boolean | default: false | Indicate if the storage support RDMA |

#### 6.2.39.3	Definition

The syntax of the VirtualBlockStorageData data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualBlockStorageData: derived_from: tosca.datatypes.Root description: VirtualBlockStorageData describes block storage requirements associated with compute resources in a particular VDU, either as a local disk or as virtual attached storage properties: size_of_storage: type: scalar-unit.size description: Size of virtualised storage resource required: true constraints: - greater_or_equal: 0 B vdu_storage_requirements: type: map description: The hardware platform specific storage requirements. A map of strings that contains a set of key-value pairs that represents the hardware platform specific storage deployment requirements required: false entry_schema: type: string rdma_enabled: type: boolean description: Indicates if the storage support RDMA required: false default: false

#### 6.2.39.4	Examples

None.

#### 6.2.39.5	Additional Requirements

None.

### 6.2.40	tosca.datatypes.nfv.VirtualObjectStorageData

#### 6.2.40.1	Description

The VirtualObjectStorageData data type describes object storage requirements associated with compute resources in a particular VDU. Table 6.2.40.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.40.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualObjectStorageData |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualObjectStorageData |
| Type URI | tosca.datatypes.nfv.VirtualObjectStorageData |

#### 6.2.40.2	Properties

The properties of the VirtualObjectStorageData data type shall comply with the provisions set out in table 6.2.40.2-1.

Table 6.2.40.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| max_size_of_storage | no | scalar-unit.size | greater_or_equal:0 B | Maximum size of virtualised storage resource |

#### 6.2.40.3	Definition

The syntax of the VirtualObjectStorageData data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualObjectStorageData: derived_from: tosca.datatypes.Root description: VirtualObjectStorageData describes object storage requirements associated with compute resources in a particular VDU properties: max_size_of_storage: type: scalar-unit.size description: Maximum size of virtualized storage resource required: false constraints: - greater_or_equal: 0 B

#### 6.2.40.4	Examples

None.

#### 6.2.40.5	Additional Requirements

None.

### 6.2.41	tosca.datatypes.nfv.VirtualFileStorageData

#### 6.2.41.1	Description

The VirtualObjectFileData data type describes file storage requirements associated with compute resources in a particular VDU. Table 6.2.41.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.41.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualObjectFileData |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualFileStorageData |
| Type URI | tosca.datatypes.nfv.VirtualFileStorageData |

#### 6.2.41.2	Properties

The properties of the VirtualFileStorageData data type shall comply with the provisions set out in table 6.2.41.2-1.

Table 6.2.41.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| size_of_storage | yes | scalar-unit.size | greater_or_equal:0 B | Size of virtualised storage resource |
| file_system_protocol | yes | string |  | The shared file system protocol (e.g. NFS, CIFS). |

#### 6.2.41.3	Definition

The syntax of the VirtualFileStorageData data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualFileStorageData: derived_from: tosca.datatypes.Root description: VirtualFileStorageData describes file storage requirements associated with compute resources in a particular VDU properties: size_of_storage: type: scalar-unit.size description: Size of virtualized storage resource required: true constraints: - greater_or_equal: 0 B file_system_protocol: type: string description: The shared file system protocol (e.g. NFS, CIFS) required: true

#### 6.2.41.4	Examples

None.

#### 6.2.41.5	Additional Requirements

None.

### 6.2.42	tosca.datatypes.nfv.VirtualLinkBitrateLevel

#### 6.2.42.1	Description

The VirtualLinkBitrateLevel data type describes bitrate requirements applicable to the virtual link instantiated from a particular VnfVirtualLink. Table 6.2.42.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.42.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkBitrateLevel |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkBitrateLevel |
| Type URI | tosca.datatypes.nfv.VirtualLinkBitrateLevel |

#### 6.2.42.2	Properties

The properties of the VirtualLinkBitrateLevel data type shall comply with the provisions set out in table 6.2.42.2-1.

Table 6.2.42.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Virtual link bitrate requirements for an instantiation level or bitrate delta for a scaling step. |

#### 6.2.42.3	Definition

The syntax of the VirtualLinkBitrateLevel data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualLinkBitrateLevel: derived_from: tosca.datatypes.Root description: Describes bitrate requirements applicable to the virtual link instantiated from a particicular VnfVirtualLink properties: bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Virtual link bitrate requirements for an instantiation level or bitrate delta for a scaling step required: true

#### 6.2.42.4	Examples

See clause A.6.

#### 6.2.42.5	Additional Requirements

None.

### 6.2.43	tosca.datatypes.nfv.VnfOperationAdditionalParameters

#### 6.2.43.1	Description

The VnfOperationAdditionalParameters data type is an empty base type for deriving data type for describing VNF-specific parameters to be passed when invoking lifecycle management operations as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.43.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.43.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfOperationAdditionalParameters |
| --- | --- |
| Type Qualified Name | toscanfv:VnfOperationAdditionalParameters |
| Type URI | tosca.datatypes.nfv.VnfOperationAdditionalParameters |

#### 6.2.43.2	Properties

None.

#### 6.2.43.3	Definition

The syntax of the VnfOperationAdditionalParameters data type shall comply with the following definition:

tosca.datatypes.nfv.VnfOperationAdditionalParameters: derived_from: tosca.datatypes.Root description: Is an empty base type for deriving data type for describing VNF-specific parameters to be passed when invoking lifecycle management operations # properties:

#### 6.2.43.4	Examples

tosca_definitions_version: tosca_simple_yaml_1_2 node_types: MyCompany.nodes.nfv.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: .. interfaces: Vnflcm: instantiate: inputs: additional_parameters: type: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters data_types: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters: derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters properties: parameter_1: type: string required: true parameter_2: type: string required: false default: value_2

#### 6.2.43.5	Additional Requirements

None.

### 6.2.44	tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration

#### 6.2.44.1	Description

The VnfChangeFlavourOperationConfiguration data type represents information that affect the invocation of the ChangeVnfFlavour operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.44.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.44.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfChangeFlavourOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfChangeFlavourOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration |

#### 6.2.44.2	Properties

None.

#### 6.2.44.3	Definition

The syntax of the VnfChangeFlavourOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the ChangeVnfFlavour operation # properties:

#### 6.2.44.4	Examples

None.

#### 6.2.44.5	Additional Requirements

None.

### 6.2.45	tosca.datatypes.nfv.VnfChangeExtConnectivityOperation
Configuration

#### 6.2.45.1	Description

The VnfChangeExtConnectivityOperationConfiguration data type represents information that affect the invocation of the ChangeExtVnfConnectivity operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.45.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.45.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfChangeExtConnectivityOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfChangeExtConnectivityOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfChangeExtConnectivityOperationConfiguration |

#### 6.2.45.2	Properties

None.

#### 6.2.45.3	Definition

The syntax of the VnfChangeExtConnectivityOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfChangeExtConnectivityOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the ChangeExtVnfConnectivity operation # properties:

#### 6.2.45.4	Examples

None.

#### 6.2.45.5	Additional Requirements

None.

### 6.2.46	tosca.datatypes.nfv.VnfMonitoringParameter

#### 6.2.46.1	Description

This data type provides information on virtualised resource related performance metrics applicable to VNF. Table 6.2.46.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.46.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfMonitoringParameter |
| --- | --- |
| Type Qualified Name | toscanfv:VnfMonitoringParameter |
| Type URI | tosca.datatypes.nfv.VnfMonitoringParameter |

#### 6.2.46.2	Properties

The properties of the VnfMonitoringParameter data type shall comply with the provisions set out in table 6.2.46.2-1.

Table 6.2.46.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Human readable name of the monitoring parameter. |
| performance_metric | yes | string | valid values: v_cpu_usage_mean_vnf v_cpu_usage_peak_vnf v_memory_usage_mean_vnf v_memory_usage_peak_vnf v_disk_usage_mean_vnf v_disk_usage_peak_vnf byte_incoming_vnf_ext_cp byte_outgoing_vnf_ext_cp packet_incoming_vnf_ext_cp packet_outgoing_vnf_ext_cp | Identifies the performance metric, according to ETSI GS NFV-IFA 027 [7]. Performance metric values shall be set to the corresponding measurement name defined in clause 7.2 of ETSI GS NFV-IFA 027 [7], without appending a sub-counter. |
| collection_period | no | scalar-unit.time |  | Describes the periodicity at which to collect the performance information. |

#### 6.2.46.3	Definition

The syntax of the VnfMonitoringParameter data type shall comply with the following definition:

tosca.datatypes.nfv.VnfMonitoringParameter: derived_from: tosca.datatypes.Root description: Represents information on virtualised resource related performance metrics applicable to the VNF. properties: name: type: string description: Human readable name of the monitoring parameter required: true performance_metric: type: string description: Identifies the performance metric, according to ETSI GS NFV-IFA 027. required: true constraints: - valid_values: [ v_cpu_usage_mean_vnf, v_cpu_usage_peak_vnf, v_memory_usage_mean_vnf, v_memory_usage_peak_vnf, v_disk_usage_mean_vnf, v_disk_usage_peak_vnf, byte_incoming_vnf_ext_cp, byte_outgoing_vnf_ext_cp, packet_incoming_vnf_ext_cp, packet_outgoing_vnf_ext_cp ] collection_period: type: scalar-unit.time description: Describes the periodicity at which to collect the performance information. required: false constraints: - greater_than: 0 s

#### 6.2.46.4	Examples

None.

#### 6.2.46.5	Additional Requirements

None.

### 6.2.47	tosca.datatypes.nfv.VnfcMonitoringParameter

#### 6.2.47.1	Description

This data type provides information on virtualised resource related performance metrics applicable to a VNFC. Table 6.2.47.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.47.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfcMonitoringParameter |
| --- | --- |
| Type Qualified Name | toscanfv:VnfcMonitoringParameter |
| Type URI | tosca.datatypes.nfv.VnfcMonitoringParameter |

#### 6.2.47.2	Properties

The properties of the VnfcMonitoringParameter data type shall comply with the provisions set out in table 6.2.47.2-1.

Table 6.2.47.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Human readable name of the monitoring parameter. |
| performance_metric | yes | string | valid values: v_cpu_usage_mean_vnf v_cpu_usage_peak_vnf v_memory_usage_mean_vnf v_memory_usage_peak_vnf v_disk_usage_mean_vnf v_disk_usage_peak_vnf byte_incoming_vnf_int_cp byte_outgoing_vnf_int_cp packet_incoming_vnf_int_cp packet_outgoing_vnf_int_cp | Identifies the performance metric, according to ETSI GS NFV-IFA 027 [7]. Performance metric values shall be set to the corresponding measurement name defined in clause 7.2 of ETSI GS NFV-IFA 027 [7], without appending a sub-counter. |
| collection_period | no | scalar-unit.time |  | Describes the periodicity at which to collect the performance information. |

#### 6.2.47.3	Definition

The syntax of the VnfcMonitoringParameter data type shall comply with the following definition:

tosca.datatypes.nfv.VnfcMonitoringParameter: derived_from: tosca.datatypes.Root description: Represents information on virtualised resource related performance metrics applicable to the VNF. properties: name: type: string description: Human readable name of the monitoring parameter required: true performance_metric: type: string description: Identifies the performance metric, according to ETSI GS NFV-IFA 027. required: true constraints: - valid_values: [ v_cpu_usage_mean_vnf, v_cpu_usage_peak_vnf, v_memory_usage_mean_vnf, v_memory_usage_peak_vnf, v_disk_usage_mean_vnf, v_disk_usage_peak_vnf, byte_incoming_vnf_int_cp, byte_outgoing_vnf_int_cp, packet_incoming_vnf_int_cp, packet_outgoing_vnf_int_cp ] collection_period: type: scalar-unit.time description: Describes the periodicity at which to collect the performance information. required: false constraints: - greater_than: 0 s

#### 6.2.47.4	Examples

None.

#### 6.2.47.5	Additional Requirements

None.

### 6.2.48	tosca.datatypes.nfv.VirtualLinkMonitoringParameter

#### 6.2.48.1	Description

This data type provides information on virtualised resource related performance metrics. Table 6.2.48.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.48.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkMonitoringParameter |
| --- | --- |
| Type Qualified Name | toscanfv: VirtualLinkMonitoringParameter |
| Type URI | tosca.datatypes.nfv.VirtualLinkMonitoringParameter |

#### 6.2.48.2	Properties

The properties of the VirtualLinkMonitoringParameter data type shall comply with the provisions set out in table 6.2.48.2-1.

Table 6.2.48.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Human readable name of the monitoring parameter. |
| performance_metric | yes | string | valid values: packet_outgoing-virtual_link packet_incoming_virtual_link | Identifies a performance metric derived from those defined in ETSI GS NFV-IFA 027 [7]. The packet_outgoing_virtual_link and packet_incoming_virtual_link metrics shall be obtained by aggregation the PacketOutgoing and PacketIncoming measurements defined in clause 7.1 of ETSI GS NFV-IFA 027 [7] of all virtual link ports attached to the virtual link to which the metrics apply. |
| collection_period | no | scalar-unit.time |  | Describes the periodicity at which to collect the performance information. |

#### 6.2.48.3	Definition

The syntax of the VirtualLinkMonitoringParameter data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualLinkMonitoringParameter: derived_from: tosca.datatypes.Root description: Represents information on virtualised resource related performance metrics applicable to the VNF. properties: name: type: string description: Human readable name of the monitoring parameter required: true performance_metric: type: string description: Identifies a performance metric derived from those defined in ETSI GS NFV-IFA 027.The packetOutgoingVirtualLink and packetIncomingVirtualLink metrics shall be obtained by aggregation the PacketOutgoing and PacketIncoming measurements defined in clause 7.1 of ETSI GS NFV-IFA 027 of all virtual link ports attached to the virtual link to which the metrics apply. required: true constraints: - valid_values: [ packet_outgoing_virtual_link, packet_incoming_virtual_link ] collection_period: type: scalar-unit.time description: Describes the periodicity at which to collect the performance information. required: false constraints: - greater_than: 0 s

#### 6.2.48.4	Examples

None.

#### 6.2.48.5	Additional Requirements

None.

### 6.2.49	tosca.datatypes.nfv.InterfaceDetails

#### 6.2.49.1	Description

The InterfaceDetails data type describes information used to access an interface exposed by a VNF. It corresponds to the interfaceDetails attribute of the VnfInterfaceDetails information element defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.49.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.49.1-1: Type name, shorthand, and URI

| Shorthand Name | InterfaceDetails |
| --- | --- |
| Type Qualified Name | toscanfv:InterfaceDetails |
| Type URI | tosca.datatypes.nfv.InterfaceDetails |

#### 6.2.49.2	Properties

The properties of the InterfaceDetails data type shall comply with the provisions set out in table 6.2.49.2-1.

Table 6.2.49.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| uri_components | no | tosca.datatypes.nfv.UriComponents |  | Provides components to build a Uniform Resource Identifier (URI) where to access the interface end point. |
| interface_specific_data | no | map of string |  | Provides additional details that are specific to the type of interface considered. |

#### 6.2.49.3	Definition

The syntax of the InterfaceDetails data type shall comply with the following definition:

tosca.datatypes.nfv.InterfaceDetails: derived_from: tosca.datatypes.Root description: information used to access an interface exposed by a VNF properties: uri_components: type: tosca.datatypes.nfv.UriComponents description: Provides components to build a Uniform Ressource Identifier (URI) where to access the interface end point. required: false interface_specific_data: type: map description: Provides additional details that are specific to the type of interface considered. required: false entry_schema: type: string

#### 6.2.49.4	Examples

See clause 6.10.12.

#### 6.2.49.5	Additional Requirements

None.

### 6.2.50	tosca.datatypes.nfv.UriComponents

#### 6.2.50.1	Description

The UriComponents data type describes information used to build a URI that complies with IETF RFC 3986 [8]. Table 6.2.50.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.50.1-1: Type name, shorthand, and URI

| Shorthand Name | UriComponents |
| --- | --- |
| Type Qualified Name | toscanfv:UriComponents |
| Type URI | tosca.datatypes.nfv.UriComponents |

#### 6.2.50.2	Properties

The properties of the UriComponents data type shall comply with the provisions set out in table 6.2.50.2-1.

Table 6.2.50.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scheme | yes | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the scheme component of a URI, as per IETF RFC 3986 [8] |
| authority | no | tosca.datatypes.nfv.UriAuthority |  | Corresponds to the authority component of a URI, as per IETF RFC 3986 [8] See note |
| path | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the path component of a URI, as per IETF RFC 3986 [8] |
| query | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the query component of a URI, as per IETF RFC 3986 [8] |
| fragment | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the fragment component of a URI, as per IETF RFC 3986 [8] |
| NOTE: If this property is not included while the URI scheme requires it, the VNFM is expected to generate it, based on knowledge of the network configuration of the external CP instance that provides the connectivity for this interface. |  |  |  |  |

#### 6.2.50.3	Definition

The syntax of the UriComponents data type shall comply with the following definition:

tosca.datatypes.nfv.UriComponents: derived_from: tosca.datatypes.Root description: information used to build a URI that complies with IETF RFC 3986 [8]. properties: scheme: type: string # shall comply with IETF RFC 3986 description: scheme component of a URI. required: true authority: type: tosca.datatypes.nfv.UriAuthority description: Authority component of a URI required: false path: type: string # shall comply with IETF RFC 3986 description: path component of a URI. required: false query: type: string # shall comply with IETF RFC 3986 description: query component of a URI. required: false fragment: type: string # shall comply with IETF RFC 3986 description: fragment component of a URI. required: false

#### 6.2.50.4	Examples

See clause 6.10.12.

#### 6.2.50.5	Additional Requirements

When this datatype is used to provide information for accessing APIs defined in ETSI GS NFV-SOL 002 [i.4], the path property may be included and the query and fragment properties shall be absent. The values of the scheme, authority and path properties form the {apiRoot} of the URI prefix.

### 6.2.51	tosca.datatypes.nfv.UriAuthority

#### 6.2.51.1	Description

The UriAuthority data type corresponds to the authority component of a URI as specified in IETF RFC 3986 [8]. Table 6.2.51.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.51.1-1: Type name, shorthand, and URI

| Shorthand Name | UriAuthority |
| --- | --- |
| Type Qualified Name | toscanfv:UriAuthority |
| Type URI | tosca.datatypes.nfv.UriAuthority |

#### 6.2.51.2	Properties

The properties of the UriAuthority data type shall comply with the provisions set out in table 6.2.51.2-1.

Table 6.2.51.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| user_info | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the user_info field of the authority component of a URI, as per IETF RFC 3986 [8]. For HTTP and HTTPS URIs, the provisions in sections 2.7.1 and 2.7.2 of IETF RFC 7230 [12] apply, respectively. |
| host | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the host field of the authority component of a URI, as per IETF RFC 3986 [8]. See note 1. |
| port | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the port field of the authority component of a URI, as per IETF RFC 3986 [8]. See note 2. |
| NOTE 1: If this property is not included the VNFM is expected to generate it, based on knowledge of the network configuration of the external CP instance that provides the connectivity for this interface. NOTE 2: If this property is not included the default port for the protocol declared by the scheme property of the parent UriComponents structure shall be used unless there are configuration mechanisms applied that are outside the scope of the present document. If no default port exists for the URI scheme, the port property shall be included unless there are configuration mechanisms applied that are outside the scope of the present document. |  |  |  |  |

#### 6.2.51.3	Definition

The syntax of the UriAuthority data type shall comply with the following definition:

tosca.datatypes.nfv.UriAuthority: derived_from: tosca.datatypes.Root description: information that corresponds to the authority component of a URI as specified in IETF RFC 3986 [8] properties: user_info: type: string # shall comply with IETF RFC 3986 description: user_info field of the authority component of a URI required: false host: type: string # shall comply with IETF RFC 3986 description: host field of the authority component of a URI required: false port: type: string # shall comply with IETF RFC 3986 description: port field of the authority component of a URI required: false

#### 6.2.51.4	Examples

See clause 6.10.12.

#### 6.2.51.5	Additional Requirements

When this datatype is used to provide information for accessing APIs defined in ETSI GS NFV-SOL 002 [i.4], the host property and port properties may be included and the user_info property shall not be included. If the host property is included and the value is a registered name, it is assumed that means are in place to resolve the host name to the correct IP address. If the host property is not included, it is assumed that the VNFM will use the IP address associated to one of the connection point instances created from the VnfExpCp and VduCp node types declared as a target of the SupportedVnfInterface policy.

NOTE:	This means that if multiple CP instances exist that were created from a particular VnfExtCp or VduCp node template, the VNFM may use any of them to attempt accessing the interface. If no reply is received because the selected CP instance if out of service or is not reachable, the VNFM is expected to try reaching the interface through another CP instance.

### 6.2.52	tosca.datatypes.nfv.VnfProfile

#### 6.2.52.1	Description

The VnfProfile data type describes a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF as defined in ETSI GS NFV-IFA 014 [2]. Table 6.2.52.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.52.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfProfile |
| --- | --- |
| Type Qualified Name | toscanfv:VnfProfile |
| Type URI | tosca.datatypes.nfv.VnfProfile |

#### 6.2.52.2	Properties

The properties of the VnfProfile data type shall comply with the provisions set out in table 6.2.52.2-1.

Table 6.2.52.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| instantiation_Level | no | string |  | Identifier of the instantiation level of the VNF DF to be used for instantiation. If not present, the default instantiation level as declared in the VNFD shall be used. |
| min_number_of_instances | yes | integer | greater_or_equal : 0 | Minimum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. |
| max_number_of_instances | yes | integer | greater_or_equal : 0 | Maximum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. |

#### 6.2.52.3	Definition

The syntax of the VnfProfile data type shall comply with the following definition:

tosca.datatypes.nfv.VnfProfile: derived_from: tosca.datatypes.Root description: describes a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF. properties: instantiation_level: type: string description: Identifier of the instantiation level of the VNF DF to be used for instantiation. If not present, the default instantiation level as declared in the VNFD shall be used. required: false min_number_of_instances: type: integer description: Minimum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. required: true constraints: - greater_or_equal: 0 max_number_of_instances: type: integer description: Maximum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. required: true constraints: - greater_or_equal: 0

#### 6.2.52.4	Example

None.

#### 6.2.52.5	Additional Requirements

None.

### 6.2.53	tosca.datatypes.nfv.ChecksumData

#### 6.2.53.1	Description

The ChecksumData data type describes information about the result of performing a checksum operation over some arbitrary data. Table 6.2.53.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.2.53.1-1: Type name, shorthand, and URI

| Shorthand Name | ChecksumData |
| --- | --- |
| Type Qualified Name | toscanfv:ChecksumData |
| Type URI | tosca.datatypes.nfv.ChecksumData |

#### 6.2.53.2	Properties

The properties of the ChecksumData data type shall comply with the provisions set out in table 6.2.53.2-1.

Table 6.2.53.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| algorithm | yes | string | Valid values: sha-224, sha-256, sha-384, sha-512 | Describes the algorithm used to obtain the checksum value, as described in [14]. |
| hash | yes | string |  | Contains the result of applying the algorithm indicated by the algorithm property to the data to which this ChecksumData refers. |

#### 6.2.53.3	Definition

The syntax of the ChecksumData data type shall comply with the following definition:

tosca.datatypes.nfv.ChecksumData: derived_from: tosca.datatypes.Root description: Describes information about the result of performing a checksum operation over some arbitrary data properties: algorithm: type: string description: Describes the algorithm used to obtain the checksum value required: true constraints: - valid_values: [sha-224, sha-256, sha-384, sha-512 ] hash: type: string description: Contains the result of applying the algorithm indicated by the algorithm property to the data to which this ChecksumData refers required: true

#### 6.2.53.4	Examples

<some_tosca_entity>: properties: checksum: algorithm: sha-256 hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

#### 6.2.53.5	Additional Requirements

None.

## 6.3	Artifact Types

### 6.3.1	tosca.artifacts.nfv.SwImage

#### 6.3.1.1	Description

The SwImage artifact describes the software image which is directly loaded on the virtualisation container realizing of the VDU or is to be loaded on a virtual storage resource, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.3.1.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.3.1.1-1: Type name, shorthand, and URI

| Shorthand Name | SwImage |
| --- | --- |
| Type Qualified Name | toscanfv:SwImage |
| Type URI | tosca.artifacts.nfv.SwImage |

#### 6.3.1.2	Description

tosca.artifacts.nfv.SwImage: derived_from: tosca.artifacts.Deployment.Image description: describes the software image which is directly loaded on the virtualisation container realizing of the VDU or is to be loaded on a virtual storage resource

### 6.3.2	tosca.artifacts.Implementation.nfv.Mistral

#### 6.3.2.1	Description

This artifact type represents a Mistral file that contains Mistral language [i.7] constructs that can be executed within a Mistral workbook. Support of this type is optional.

Table 6.3.2.1-1: Type name, shorthand, and URI

| Shorthand Name | Mistral |
| --- | --- |
| Type Qualified Name | toscanfv:Mistral |
| Type URI | tosca.artifacts.Implementation.nfv.Mistral |
| derived_from | tosca.artifacts.Implementation |

#### 6.3.2.2	Definition

The syntax of the Mistral artifact type shall comply with the following definition:

tosca.artifacts.Implementation.nfv.Mistral: derived_from: tosca.artifacts.Implementation description: artifacts for Mistral workflows mime_type: application/x-yaml file_ext: [ yaml ]

## 6.4	Capability Types

### 6.4.1	tosca.capabilities.nfv.VirtualBindable

#### 6.4.1.1	Description

The VirtualBindable capability indicates that the node that includes it can be pointed by a tosca.relationships.nfv.VirtualBindsTo relationship type which is used to model the VduHasCpd association illustrated in ETSI GS NFV-IFA 011 [1]. Table 6.4.1.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.4.1.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualBindable |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualBindable |
| Type URI | tosca.capabilities.nfv.VirtualBindable |

#### 6.4.1.2	Properties

None.

#### 6.4.1.3	Definition

The syntax of the VirtualBindable capability type shall comply with the following definition:

tosca.capabilities.nfv.VirtualBindable: derived_from: tosca.capabilities.Node description: Indicates that the node that includes it can be pointed by a tosca.relationships.nfv.VirtualBindsTo relationship type which is used to model the VduHasCpd association

### 6.4.2	tosca.capabilities.nfv.VirtualLinkable

#### 6.4.2.1	Description

A node type that includes the VirtualLinkable capability indicates that it can be pointed by tosca.relationships.nfv.VirtualLinksTo relationship type. which is used to model the VduCpdReferencesVirtualLinkDesc and the VnfExtCpdMapsToVirtualLinkDesc associations illustrated in ETSI GS NFV-IFA 011 [1]. Table 6.4.2.1-1 specifies the declared names for this capability type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.4.2.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkable |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkable |
| Type URI | tosca.capabilities.nfv.VirtualLinkable |

#### 6.4.2.2	Properties

None.

#### 6.4.2.3	Definition

The syntax of the VirtualLinkable capability type shall comply with the following definition:

tosca.capabilities.nfv.VirtualLinkable: derived_from: tosca.capabilities.Node description: A node type that includes the VirtualLinkable capability indicates that it can be pointed by tosca.relationships.nfv.VirtualLinksTo relationship type

### 6.4.3	tosca.capabilities.nfv.VirtualCompute

#### 6.4.3.1	Description

The VirtualCompute capability type describes the capabilities related to virtual compute resources, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.4.3.1-1 specifies the declared names for this capability type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.4.3.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualCompute |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualCompute |
| Type URI | tosca.capabilities.nfv.VirtualCompute |

#### 6.4.3.2	Properties

The properties of the VirtualCompute capability type shall comply with the provisions set out in table 6.4.3.2-1.

Table 6.4.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| logical_node | no | map of tosca.datatypes.nfv.LogicalNodeData |  | The Logical Node requirements. |
| request_additional_capabilities | no | map of tosca.datatypes.nfv.RequestedAdditionalCapability |  | Describes additional capability for a particular VDU. |
| compute_requirements | no | map of string |  | Describes compute requirements. |
| virtual_memory | yes | tosca.datatypes.nfv.VirtualMemory |  | Describes virtual memory of the virtualised compute. |
| virtual_cpu | yes | tosca.datatypes.nfv.VirtualCpu |  | Describes virtual CPU(s) of the virtualised compute. |
| virtual_local_storage | no | list of tosca.datatypes.nfv.VirtualBlockStorageData |  | A list of virtual system disks created and destroyed as part of the VM lifecycle. |

#### 6.4.3.3	Definition

The syntax of the VirtualCompute capability type shall comply with the following definition:

tosca.capabilities.nfv.VirtualCompute: derived_from: tosca.capabilities.Node description: Describes the capabilities related to virtual compute resources properties: logical_node: type: map description: Describes the Logical Node requirements required: false entry_schema: type: tosca.datatypes.nfv.LogicalNodeData requested_additional_capabilities: type: map description: Describes additional capability for a particular VDU required: false entry_schema: type: tosca.datatypes.nfv.RequestedAdditionalCapability compute_requirements: type: map required: false entry_schema: type: string virtual_memory: type: tosca.datatypes.nfv.VirtualMemory description: Describes virtual memory of the virtualized compute required: true virtual_cpu: type: tosca.datatypes.nfv.VirtualCpu description: Describes virtual CPU(s) of the virtualized compute required: true virtual_local_storage: type: list description: A list of virtual system disks created and destroyed as part of the VM lifecycle required: false entry_schema: type: tosca.datatypes.nfv.VirtualBlockStorageData description: virtual system disk definition

### 6.4.4	tosca.capabilities.nfv.VirtualStorage

#### 6.4.4.1	Description

The VirtualStorage capability indicates that the node that includes it can be pointed by a tosca.relationships.nfv.AttachesTo relationship type which is used to model the VduHasVirtualStorageDesc association illustrated in ETSI GS NFV-IFA 011 [1]. Table 6.4.4.1-1 specifies the declared names for this capability type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.4.4.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualStorage |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualStorage |
| Type URI | tosca.capabilities.nfv.VirtualStorage |

#### 6.4.4.2	Definition

The syntax of the VirtualStorage capability type shall comply with the following definition:

tosca.capabilities.nfv.VirtualStorage: derived_from: tosca.capabilities.Root description: Describes the attachment capabilities related to Vdu.Storage

## 6.5	Requirements Types

None.

## 6.6	Relationship Types

### 6.6.1	tosca.relationships.nfv.VirtualBindsTo

#### 6.6.1.1	Description

This relationship type represents an association between Vdu.Compute and VduCp node types. Table 6.6.1.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.6.1.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualBindsTo |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualBindsTo |
| Type URI | tosca.relationships.nfv.VirtualBindsTo |

#### 6.6.1.2	Properties

None.

#### 6.6.1.3	Definition

The syntax of the VirtualBindsTo relationship type shall comply with the following definition:

tosca.relationships.nfv.VirtualBindsTo: derived_from: tosca.relationships.DependsOn description: Represents an association relationship between Vdu.Compute and VduCp node types valid_target_types: [ tosca.capabilities.nfv.VirtualBindable ]

### 6.6.2	tosca.relationships.nfv.VirtualLinksTo

#### 6.6.2.1	Description

This relationship type represents an association between the VduCp and VnfVirtualLink node types. Table 6.6.2.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.6.2.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinksTo |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinksTo |
| Type URI | tosca.relationship.nfv.VirtualLinksTo |

#### 6.6.2.2	Properties

None.

#### 6.6.2.3	Definition

The syntax of the VnfVirtualLinksTo relationship type shall comply with the following definition:

tosca.relationships.nfv.VirtualLinksTo: derived_from: tosca.relationships.DependsOn description: Represents an association relationship between the VduCp and VnfVirtualLink node types valid_target_types: [ tosca.capabilities.nfv.VirtualLinkable ]

### 6.6.3	tosca.relationships.nfv.AttachesTo

#### 6.6.3.1	Description

This relationship type represents an association between the Vdu.Compute and one of the following node types: Vdu.VirtualBlockStorage, Vdu.VirtualObjectStorage or Vdu.VirtualFileStorage. Table 6.6.3.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.6.3.1-1: Type name, shorthand, and URI

| Shorthand Name | AttachesTo |
| --- | --- |
| Type Qualified Name | toscanfv:AttachesTo |
| Type URI | tosca.relationships.nfv.AttachesTo |

#### 6.6.3.2	Properties

None.

#### 6.6.3.3	Definition

The syntax of the AttachesTo relationship type shall comply with the following definition:

tosca.relationships.nfv.AttachesTo: derived_from: tosca.relationships.Root description: Represents an association relationship between the Vdu.Compute and one of the node types, Vdu.VirtualBlockStorage, Vdu.VirtualObjectStorage or Vdu.VirtualFileStorage valid_target_types: [ tosca.capabilities.nfv.VirtualStorage ]

## 6.7	Interface Types

### 6.7.1	tosca.interfaces.nfv.Vnflcm

#### 6.7.1.1	Description

The tosca.interfaces.nfv.Vnflcm contains a set of TOSCA operations corresponding to the following VNF LCM operations defined in ETSI GS NFV-IFA 007 [i.1]:

Instantiate VNF

Terminate VNF

Modify VNF information

Change VNF Flavour

Change External VNF Connectivity

Operate VNF

Heal VNF

Scale VNF

Scale VNF To Level

In addition, the VNFM shall also support TOSCA operations corresponding to preamble and postamble to the execution of the base operation. The name of these operations is constructed according to the following pattern:

<base_operation_name>_start for a preamble

<base_operation_name>_end for a postamble

The designations ("_start", "_end") in the name of TOSCA operations are postfixes so that related operations are adjacent in an alphabetical listing.

Table 6.7.1.1-1 specifies the declared names for this interface type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.7.1.1-1: Type name, shorthand, and URI

| Shorthand Name | Vnflcm |
| --- | --- |
| Type Qualified Name | toscanfv:Vnflcm |
| Type URI | tosca.interfaces.nfv.Vnflcm |

#### 6.7.1.2	Definition

The syntax of the Vnflcm interface type shall comply with the following definition:

tosca.interfaces.nfv.Vnflcm: derived_from: tosca.interfaces.Root description: This interface encompasses a set of TOSCA operations corresponding to the VNF LCM operations defined in ETSI GS NFV-IFA 007 as well as to preamble and postamble procedures to the execution of the VNF LCM operations. instantiate: description: Invoked upon receipt of an Instantiate VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters instantiate_start: description: Invoked before instantiate instantiate_end: description: Invoked after instantiate terminate: description: Invoked upon receipt Terminate VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters terminate_start: description: Invoked before terminate terminate_end: description: Invoked after terminate modify_information: description: Invoked upon receipt of a Modify VNF Information request modify_information_start: description: Invoked before modify_information modify_information_end: description: Invoked after modify_information change_flavour: description: Invoked upon receipt of a Change VNF Flavour request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters change_flavour_start: description: Invoked before change_flavour change_flavour_end: description: Invoked after change_flavour change_external_connectivity: description: Invoked upon receipt of a Change External VNF Connectivity request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters change_external_connectivity_start: description: Invoked before change_external_connectivity change_external_connectivity_end: description: Invoked after change_external_connectivity operate: description: Invoked upon receipt of an Operate VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters operate_start: description: Invoked before operate operate_end: description: Invoked after operate heal: description: Invoked upon receipt of a Heal VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters heal_start: description: Invoked before heal heal_end: description: Invoked after heal scale: description: Invoked upon receipt of a Scale VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters scale_start: description: Invoked before scale scale_end: description: Invoked after scale scale_to_level: description: Invoked upon receipt of a Scale VNF to Level request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters scale_to_level_start: description: Invoked before scale_to_level scale_to_level_end: description: Invoked after scale_to_level

#### 6.7.1.3	Additional Requirements

All VNF supported LCM operations shall be listed in the service template, except "instantiate" and "terminate" that may be omitted, as specified in ETSI GS NFV-IFA 011 [1] for the supportedOperation attribute of a deployment flavour.

The implementation and inputs keynames specified in TOSCA-Simple-Profile-YAML-v1.2 [3] for an operation definition may be included for each operation listed in the Vnflcm interface definition.

If a TOSCA operation representing a VNF LCM operation is listed in the service template without an associated implementation, then it means that:

the VNF LCM operation is supported (i.e. this is the manifestation of the supportedOperation attribute as per ETSI GS NFV-IFA 011 [1]); and

the processing logic associated with the LCM operation is the default implementation provided by the VNFM.

If an implementation is associated to a TOSCA operation that represents a preamble or a postamble to a VNF LCM operation, the implementation logic is executed before or after the execution of the VNF LCM operation implementation, respectively.

The VNFM shall make available all parameters from the message invoking the VNF LCM operation as inputs to the corresponding TOSCA interface operations.

In the operation definitions on the Vnflcm interface, the additional_parameters (VNF-specific extension of the tosca.datatypes.nfv.VnfOperationAdditionalParameters) of the inputs section describes the name and type of the additional parameters (additionalParams) that can be submitted in the VNF LCM operation request. See an example in clause 6.2.43 (tosca.datatypes.nfv.VnfOperationAdditionalParameters).

The inputs keyname can also be used to specify additional input parameters for executing the TOSCA operation, beyond those received in the VNF LCM operation request. To distinguish them from the latter ones, such input parameters shall not be named "additional_parameters".

The implementation of the operation corresponding to preamble and postamble TOSCA operations (instantiate_start, instantiate_end, scale_start, scale_end, etc.), if present, shall be invoked with the same parameters as the corresponding base operations ones (instantiate, scale, etc.). The inputs of the operations corresponding to the postamble and preamble operations shall not be defined in the VNFD.

#### 6.7.1.4	Support of LCM scripts

In ETSI GS NFV-IFA 011 [1], the definition of the "LifeCycleManagementScript" information element of the VNFD associates scripts with events, where an event can be an external or an internal stimulus. These events are mapped to TOSCA operations of the VNF node type in the following way:

external stimuli are mapped to TOSCA operations corresponding to the VNF LCM operations defined in ETSI GS NFV-IFA 007 [i.1];

internal stimuli are mapped to preamble and postamble of these TOSCA operations;

events that cannot be mapped to these TOSCA operations (lcmTransitionEvent as described in ETSI GS NFV-IFA 011 [1], clause 7.1.13) can be mapped to further TOSCA operations by extending the TOSCA interface.

The LCM scripts can be regarded as artifacts that provide a VNF-specific implementation of the TOSCA operation corresponding to the stimulus. The script input parameters shall be provided to the script according to the declaration in the inputs field of the operation definition. The artifact type definition shall enable identifying the DSL used by the script. The artifact type definition for Python is provided in section 5.4.4.1 of TOSCA-Simple-Profile-YAML-v1.2 [3]. The artifact definition for Mistral is provided in clause 6.3.2 of the present document.

NOTE:	As all input parameters needed for operations corresponding to external and internal stimuli are defined in the "input parameters of the external stimuli operations", the VNF Designer is expected to make the list of parameters as complete as needed to handle not only the external stimuli but also the internal stimuli.

#### 6.7.1.5	Examples

The following example template fragments illustrate the concepts.

tosca_definitions_version: tosca_simple_yaml_1_2 imports: - .. node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF .. topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 .. interfaces: Vnflcm: instantiate: ..

In the above example, as there is no implementation and inputs specified to the operations, the built-in implementation of the operation is invoked when the Instantiate VNF request is received on the LCM interface of the Or-Vnfm reference point. The received parameters (flavourId, instantiationLevelId, etc.) are passed to the built-in implementation (as flavour_id, instantiation_level_id).

tosca_definitions_version: tosca_simple_yaml_1_2 imports: - .. node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF .. topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 .. interfaces: Vnflcm: instantiate: implementation: instantiate-script ..

In the above example, the instantiate-script is invoked when the Instantiate VNF request is received, passing the received parameters to it similarly to the previous example. This example does not imply a one-to-one mapping between operations and script names.

tosca_definitions_version: tosca_simple_yaml_1_2 imports: - .. node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF .. topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 .. interfaces: Vnflcm: .. scale_start: pre-scale-script scale_end: post-scale-script ..

In the above example, LCM scripts are associated with the "scale start" and "scale end" internal stimuli. As no script is associated to the scale operation, its default implementation runs (after running the pre-scale-script, and before running the post-scale-script).

tosca_definitions_version: tosca_simple_yaml_1_2 imports: - .. node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF .. topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 .. interfaces: Vnflcm: instantiate: implementation: instantiate-script inputs: script_input_1: value_1 script_input_2: value_2 artifacts: instantiate-script: description: Instantiate workflow script type: tosca.artifacts.Implementation.Python file: instantiate.py #repository: .. #deploy_path: .. ..

In the above example:

The inputs section provides additional input values to the instantiate-script (i.e. the manifestation of the scriptInput attribute of LifeCycleManagementScript as defined in ETSI GS NFV-IFA 011 [1]).

NOTE:	There is another kind of input called additional_parameters dedicated to the additional parameters (additionalParams) received in the message invoking the VNF LCM operation; this input is not illustrated by the above examples; see clause 6.2.43.4 on how to declare additional_parameters in the derived VNF node type.

TOSCA artifacts definition is used to convey the type of DSL used as a scripting language that is associated with an operation (i.e. the manifestation of the scriptDsl attribute of LifeCycleManagementScript as per ETSI GS NFV-IFA 011 [1]).

## 6.8	Node Types

### 6.8.1	tosca.nodes.nfv.VNF

#### 6.8.1.1	Description

The VNF node type is the generic abstract type from which all VNF specific abstract node types shall be derived to form, together with other node types, the TOSCA service template(s) representing the VNFD information element as defined in ETSI GS NFV-IFA 011 [1]. Table 6.8.1.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.1.1-1: Type name, shorthand, and URI

| Shorthand Name | VNF |
| --- | --- |
| Type Qualified Name | toscanfv:VNF |
| Type URI | tosca.nodes.nfv.VNF |

#### 6.8.1.2	Properties

The properties of the VNF node type shall comply with the provisions set out in table 6.8.1.2-1.

Table 6.8.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| descriptor_id | yes | string |  | Identifier of this VNFD information element. This attribute shall be globally unique. The VNFD Identifier shall be used as the unique identifier of the VNF Package that contains this VNFD. Any modification of the content of the VNFD or the VNF Package shall result in a new VNFD Identifier. |
| descriptor_version | yes | string |  | Identifies the version of the VNFD. |
| provider | yes | string |  | Provider of the VNF and of the VNFD. |
| product_name | yes | string |  | Name to identify the VNF Product. Invariant for the VNF Product lifetime. |
| software_version | yes | string |  | Software version of the VNF. This is changed when there is any change to the software that is included in the VNF Package. |
| product_info_name | no | string |  | Human readable name for the VNF Product. Can change during the VNF Product lifetime. |
| product_info_description | no | string |  | Human readable description of the VNF Product. Can change during the VNF Product lifetime. |
| vnfm_info | yes | list of string |  | Identifies VNFM(s) compatible with the VNF described in this version of the VNFD. To indicate that a VNF can be managed by any ETSI NFV compliant VNFM, the string value shall be the concatenation of the string "etsivnfm" and the minimum version of ETSI GS NFV-SOL 002 [i.4] to be supported by this VNFM (e.g. etsivnfm:v2.3.1). If the VNF is compatible with multiple versions, multiple values may be included. See note 1. To indicate a specific VNFM product, the string value shall be the concatenation of the IANA enterprise number of the VNFM provider [5], followed by a product-specific string. |
| localization_languages | no | list of string | Valid values: string values that comply with IETF RFC 5646 [13] | Information about localization languages of the VNF (includes e.g. strings in the VNFD). This allows to provide one or more localization languages to support selecting a specific localization language at VNF instantiation time. |
| default_localization_language | no | string | Valid values: string values that comply with IETF RFC 5646 [13] | Default localization language that is instantiated if no information about selected localization language is available. Shall be present if "localizationLanguage" is present and shall be absent otherwise. |
| configurable_properties | no | tosca.datatypes.nfv.VnfConfigurableProperties |  | Describes the configurable properties of the VNF (e.g. related to auto scaling and auto healing). |
| modifiable_attributes | no | tosca.datatypes.nfv.VnfInfoModifiableAttributes |  | Describes the modifiable attributes of the VNF. |
| lcm_operations_configuration | no | tosca.datatypes.nfv.VnfLcmOperationsConfiguration |  | Describes the configuration parameters for the VNF LCM operations. |
| monitoring_parameters | no | list of tosca.datatypes.nfv.VnfMonitoringParameter |  | Describes monitoring parameters applicable to the VNF. |
| flavour_id | yes | string |  | Identifier of this DF within the VNFD. |
| flavour_description | yes | string |  | Human readable description of the DF. |
| vnf_profile | no | tosca.datatypes.nfv.VnfProfile |  | Describes a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF. See note 2. |
| NOTE 1: When LCM scripts are used, the support of this minimum version might not be sufficient to ensure that the VNF can be managed by a VNFM. The support of the domain specific language(s) used by these LCM scripts is another criterion for determining the compatibility of the VNF with a VNFM. NOTE 2: This property is only used in a NSD service template when describing a VNF node template with the corresponding VnfProfile information. |  |  |  |  |

The value of the descriptor_id string shall comply with an UUID format as specified in section 3 of [9].

The syntax of the vnfm_info string values shall comply with the following ABNF [6] snippet:

value = any_etsi_nfv_compliant_product| product_specific

any_etsi_nfv_compliant_product = "etsivnfm" SEP "version"

version = "v" version_identifier

version_identifier = 1*2DIGIT DOT 1*2DIGIT DOT 1*2DIGIT

; the version identifier is encoded as a sequence of items of 1 or 2 digits separated by dots representing the 3 fields (major, technical and editorial) of the version of an ETSI deliverable.

product_specific = enterprise_number SEP product_specific_string

enterprise_number = 1*DIGIT

product_specific_string = *(ALPHA / DIGIT / "-" / ".")

SEP = ":"

DOT = "."

This implies that vnfm_info string values shall also comply with the pattern defined by the following regular expression [15]: (^etsivnfm:v[0-9]?[0-9]\.[0-9]?[0-9]\.[0-9]?[0-9]$)|(^[0-9]+:[a-zA-Z0-9.-]+$)

#### 6.8.1.3	Attributes

None.

#### 6.8.1.4	Requirements

The requirements of the VNF node type shall comply with the provisions set out in table 6.8.1.4-1.

Table 6.8.1.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_link | yes | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link |

#### 6.8.1.5	Capabilities

None.

#### 6.8.1.6	Definition

The syntax of the VNF node type shall comply with the following definition:

tosca.nodes.nfv.VNF: derived_from: tosca.nodes.Root description: The generic abstract type from which all VNF specific abstract node types shall be derived to form, together with other node types, the TOSCA service template(s) representing the VNFD properties: descriptor_id: # instead of vnfd_id type: string # GUID description: Globally unique identifier of the VNFD required: true descriptor_version: # instead of vnfd_version type: string description: Identifies the version of the VNFD required: true provider: # instead of vnf_provider type: string description: Provider of the VNF and of the VNFD required: true product_name: # instead of vnf_product_name type: string description: Human readable name for the VNF Product required: true software_version: # instead of vnf_software_version type: string description: Software version of the VNF required: true product_info_name: # instead of vnf_product_info_name type: string description: Human readable name for the VNF Product required: false product_info_description: # instead of vnf_product_info_description type: string description: Human readable description of the VNF Product required: false vnfm_info: type: list required: true description: Identifies VNFM(s) compatible with the VNF entry_schema: type: string constraints: - pattern: (^etsivnfm:v[0-9]?[0-9]\.[0-9]?[0-9]\.[0-9]?[0-9]$)¦(^[0-9]+:[a-zA-Z0-9.-]+$) localization_languages: type: list description: Information about localization languages of the VNF required: false entry_schema: type: string #IETF RFC 5646 string default_localization_language: type: string #IETF RFC 5646 string description: Default localization language that is instantiated if no information about selected localization language is available required: false #configurable_properties: #type: tosca.datatypes.nfv.VnfConfigurableProperties #description: Describes the configurable properties of the VNF #required: false # derived types are expected to introduce configurable_properties # with its type derived from tosca.datatypes.nfv.VnfConfigurableProperties #modifiable_attributes: #type: tosca.datatypes.nfv.VnfInfoModifiableAttributes #description: Describes the modifiable attributes of the VNF #required: false # derived types are expected to introduce modifiable_attributes # with its type derived from # tosca.datatypes.nfv.VnfInfoModifiableAttributes lcm_operations_configuration: type: tosca.datatypes.nfv.VnfLcmOperationsConfiguration description: Describes the configuration parameters for the VNF LCM operations required: false monitoring_parameters: type: list entry_schema: type: tosca.datatypes.nfv.VnfMonitoringParameter description: Describes monitoring parameters applicable to the VNF. required: false flavour_id: type: string description: Identifier of the Deployment Flavour within the VNFD required: true flavour_description: type: string description: Human readable description of the DF required: true vnf_profile: type: tosca.datatypes.nfv.VnfProfile description: Describes a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF required: false requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [ 0, 1 ] # Additional requirements shall be defined in the VNF specific node type (deriving from tosca.nodes.nfv.VNF) corresponding to NS virtual links that need to connect to VnfExtCps interfaces: Vnflcm: type: tosca.interfaces.nfv.Vnflcm

#### 6.8.1.7	Artifact

None.

#### 6.8.1.8	Additional Requirements

For a given VNFD, a new VNF node type shall be defined following the below requirements:

- The node type shall be derived from: tosca.nodes.nfv.VNF.

The following properties listed in tosca.nodes.nfv.VNF where the "required:" field is set to "true" shall be included with their values indicated as constraints and as default values:

  - descriptor_id

  - descriptor_version

  - provider

  - product_name

  - software_version

  - vnfm_info

  - flavour_id

  - flavour_description

NOTE 1:	Indicating their values as default allows not to include them in property assignments in node templates, e.g. in the NSD.

An empty string shall be indicated as the default value of the flavour_description property.

The capabilities, requirements, interfaces of tosca.nodes.nfv.VNF shall be preserved.

Depending on the number of external connection points of the VNF that need to connect to NS virtual links, additional requirements for VirtualLinkable capability shall be defined. In this case, it is the VNFD author's choice to use the requirement for VirtualLinkable capability defined in the tosca.nodes.nfv.VNF node type or use only the additional requirements defined in the derived VNF specific node type. In the latter case, the virtual_link requirement should be included in the node type definition with occurrences [ 0, 0 ].

The rule for naming this node type in the service template should be:

provider.product_name.software_version.descriptor_version, by concatenating the values of the corresponding properties of the created VNF node type.

NOTE 2:	If the software_version value or descriptor_version value contains a dot (i.e. "."), this character should be replaced with an underscore (i.e. "_").

VNF Providers shall use the following types to derive the VNF specific modifiable attributes and additional configurable properties:

tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions.

tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata.

tosca.datatypes.nfv.VnfAdditionalConfigurableProperties.

tosca.datatypes.nfv.VnfInfoModifiableAttributes.

tosca.datatypes.nfv.VnfConfigurableProperties.

See illustrative examples in clauses 6.8.1.9, 6.2.35.4 and 6.2.31.4.

Node templates of the VNF specific node type shall not include the vnf_profile property when they are part of a VNFD service template.

For a given NSD, when describing a referenced VNFD as a node templates, the vnf_profile property shall be included with a valid value.

#### 6.8.1.9	Example

Example usage of modifiable_attributes:

tosca_definitions_version: tosca_simple_yaml_1_2 node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: constraints: - valid_values: [ simple, complex ] modifiable_attributes: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes data_types: mycompany.datatypes.nfv.VnfInfoModifiableAttributes: derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributes properties: extensions: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions: derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions properties: http_proxy: type: string required: true https_proxy: type: string required: false

Example usage of lcm_operations_configuration:

Top level service template:

tosca_definitions_version: tosca_simple_yaml_1_2 node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: type: string constraints: - valid_values: [ simple, complex ] lcm_operations_configuration: scale: scaling_by_more_than_one_step_supported: true scale_to_level: arbitrary_target_levels_supported: true heal: causes: - service_unavailable - performance_degraded terminate: min_graceful_termination_timeout: 60 s max_recommended_graceful_termination_timeout: 600 s operate: min_graceful_stop_timeout: 60 s max_recommended_graceful_stop_timeout: 600 s

Example usage of describing a VNF node template with vnf_profile in a NSD TOSCA service template:

tosca_definitions_version: tosca_simple_yaml_1_2 description: an example of NSD TOSCA service template topology template: … node_templates: VNF_1: type: tosca.nodes.nfv.exampleVNF properties: flavour_id: small descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 provider: MyCompany product_name: SunshineDB software_version: 1.0 descriptor_version: 1.0 vnf_profile: instantiation_Level: level_1 min_number_of_instances: 2 max_number_of_instances: 6 # other properties omitted for brevity requirements: - virtual_link: NsVirtualLink

### 6.8.2	tosca.nodes.nfv.VnfExtCp

#### 6.8.2.1	Description

The VnfExtCp node type represents the VnfExtCpd information element as defined in ETSI GS NFV-IFA 011 [1], which describes a logical external connection point, exposed by this VNF enabling connecting with an external Virtual Link. Table 6.8.2.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.2.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfExtCp |
| --- | --- |
| Type Qualified Name | toscanfv:VnfExtCp |
| Type URI | tosca.nodes.nfv.VnfExtCp |

#### 6.8.2.2	Properties

The properties of the VnfExtCp node type shall comply with the provisions set out in table 6.8.2.2-1.

Table 6.8.2.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_network_interface_requirements | no | list of tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements |  | The actual virtual NIC requirements that is been assigned when instantiating the connection point. |

#### 6.8.2.3	Attributes

None.

#### 6.8.2.4	Requirements

The requirements of the VnfExtCp node type shall comply with the provisions set out in table 6.8.2.4-1.

Table 6.8.2.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| external_virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |
| internal_virtual_link | yes | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |

#### 6.8.2.5	Definition

The syntax of the VnfExtCp node type shall comply with the following definition:

tosca.nodes.nfv.VnfExtCp: derived_from: tosca.nodes.nfv.Cp description: Describes a logical external connection point, exposed by the VNF enabling connection with an external Virtual Link properties: virtual_network_interface_requirements: type: list description: The actual virtual NIC requirements that is been assigned when instantiating the connection point required: false entry_schema: type: tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements requirements: - external_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo - internal_virtual_link: #name in ETSI NFV IFA 011 v0.7.3: intVirtualLinkDesc capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo

#### 6.8.2.6	Additional Requirements

A node template of this type is used to represent a VNF external connection point only in the case the VnfExtCp is connected to an internal virtual link. The node template has the following requirements:

internal_virtual_link requirement to allow to connect it to an internal virtual link;

external_virtual_link requirement to allow to connect it to an external virtual link.

In the case where a VNF external connection point is re-exposing a VduCp (internal connection point), the VduCp node type shall be used in the service template, instead of the VnfExtCp node type.

#### 6.8.2.7	Example

In a typical scenario, the VnfExtCp node template will be part of a service template representing a certain VNF deployment flavour. The service template substitutes for an abstract VNF node type. In this substitution, the virtual_link requirement of the abstract VNF node type is mapped to the external_virtual_link requirement of the VnfExtCp node. This example is illustrated in clause A.3.2.

When a VNF external connection point re-exposes a Vdu connection point, the service template does not require an explicit node template of type VnfExtCp in a typical scenario where an abstract VNF node type is substituted by a service template representing a certain VNF deployment flavour. In this substitution, the virtual_link requirement of the abstract VNF node type is mapped to the virtual_link requirement of the VduCp node. This example is illustrated in clause A.3.1.

### 6.8.3	tosca.nodes.nfv.Vdu.Compute

#### 6.8.3.1	Description

The Vdu.Compute node type describes the virtual compute part of a VDU which is a construct supporting the description of the deployment and operational behaviour of a VNFC, as defined in ETSI GS NFV-IFA 011 [1].

Table 6.8.3.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.3.1-1: Type name, shorthand, and URI

| Shorthand Name | Vdu.Compute |
| --- | --- |
| Type Qualified Name | toscanfv:Vdu.Compute |
| Type URI | tosca.nodes.nfv.Vdu.Compute |

#### 6.8.3.2	Properties

The properties of the Vdu.Compute node type shall comply with the provisions set out in table 6.8.3.2-1.

Table 6.8.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | yes | string |  | Human readable name of the Vdu. |
| description | yes | string |  | Human readable description of the Vdu. |
| boot_order | no | list of string |  | The key indicates the boot index (lowest index defines highest boot priority). The Value references a node template name from which a valid boot device is created e.g. VirtualBlockStorage node template from which a VirtualBlockStorage instance is created. If no boot order is defined the default boot order defined in the VIM or NFVI shall be used. |
| nfvi_constraints | no | list of string |  | Describes constraints on the NFVI for the VNFC instance(s) created from this Vdu. For example, aspects of a secure hosting environment for the VNFC instance that involve additional entities or processes. |
| monitoring_parameters | no | list of tosca.datatypes.nfv.VnfcMonitoringParameter |  | Describes monitoring parameters applicable to a VNFC based on this VDU. |
| configurable_properties | no | map of tosca.datatypes.nfv.VnfcConfigurableProperties |  | Describes the configurable properties of all VNFC instances based on this VDU. |
| boot_data | no | string |  | Contains a string or a URL to a file contained in the VNF package used to customize a virtualised compute resource at boot time. The bootData may contain variable parts that are replaced by deployment specific values before being sent to the VIM. For the parameters with "volatile" values, the parameters of each variable part shall be declared in the tosca.datatypes.nfv.VnfOperationAdditionalParameters; for parameters with "persistent" values during the lifetime of the VNF instance, the parameters shall be declared in the tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions. |
| vdu_profile | yes | tosca.datatypes.nfv.VduProfile |  | Defines additional instantiation data for the VDU.Compute node. |
| sw_image_data | no | tosca.datatypes.nfv.SwImageData |  | Defines information related to a SwImage artifact used by this Vdu.Compute node. |

#### 6.8.3.3	Attributes

None.

#### 6.8.3.4	Requirements

The requirements of the Vdu.Compute node type shall comply with the provisions set out in table 6.8.3.4-1.

Table 6.8.3.4-1: Requirements

| Name | Required | Required capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_storage | no | tosca.capabilities.nfv.VirtualStorage |  | Describes storage requirements for a virtual_storage instance attached to the virtualisation container created from virtual_compute defined for this vdu |

#### 6.8.3.5	Capabilities

The capabilities of the Vdu.Compute node type shall comply with the provisions set out in table 6.8.3.5-1.

Table 6.8.3.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_compute | tosca.capabilities.nfv.VirtualCompute |  | Describes virtual compute resources capabilities. |
| virtual_binding | tosca.capabilities.nfv.VirtualBindable |  | Defines ability of VirtualBindable. See note. |
| NOTE: Due to current grammar limitations in TOSCA-Simple-Profile-YAML-v1.2 [3], the Vdu.Compute node template cannot limit the number of the virtual_binding capability occurrences. It is the VNFD author's responsibility to make sure that the number of VduCps connected to a particular Vdu.Compute node is compatible with the virtualised resources assigned to this node. |  |  |  |

#### 6.8.3.6	Definition

The syntax of the Vdu.Compute node type shall comply with the following definition:

tosca.nodes.nfv.Vdu.Compute: derived_from: tosca.nodes.Root description: Describes the virtual compute part of a VDU which is a construct supporting the description of the deployment and operational behavior of a VNFC properties: name: type: string description: Human readable name of the VDU required: true description: type: string description: Human readable description of the VDU required: true boot_order: type: list # explicit index (boot index) not necessary, contrary to IFA011 description: References a node template name from which a valid boot device is created required: false entry_schema: type: string nfvi_constraints: type: list description: Describes constraints on the NFVI for the VNFC instance(s) created from this VDU required: false entry_schema: type: string monitoring_parameters: type: list description: Describes monitoring parameters applicable to a VNFC instantiated from this VDU required: false entry_schema: type: tosca.datatypes.nfv.VnfcMonitoringParameter #configurable_properties: #type: tosca.datatypes.nfv.VnfcConfigurableProperties #required: false # derived types are expected to introduce # configurable_properties with its type derived from # tosca.datatypes.nfv.VnfcConfigurableProperties vdu_profile: type: tosca.datatypes.nfv.VduProfile description: Defines additional instantiation data for the VDU.Compute node required: true sw_image_data: type: tosca.datatypes.nfv.SwImageData description: Defines information related to a SwImage artifact used by this Vdu.Compute node required: false # property is required when the node template has an associated artifact of type tosca.artifacts.nfv.SwImage and not required otherwise boot_data: type: string description: Contains a string or a URL to a file contained in the VNF package used to customize a virtualised compute resource at boot time. The bootData may contain variable parts that are replaced by deployment specific values before being sent to the VIM. required: false capabilities: virtual_compute: type: tosca.capabilities.nfv.VirtualCompute occurrences: [ 1, 1 ] virtual_binding: type: tosca.capabilities.nfv.VirtualBindable occurrences: [ 1, UNBOUNDED ] requirements: - virtual_storage: capability: tosca.capabilities.nfv.VirtualStorage relationship: tosca.relationships.nfv.AttachesTo occurrences: [ 0, UNBOUNDED ]

#### 6.8.3.7	Additional requirements

Node templates of type tosca.nodes.nfv.Vdu.Compute may contain an artifact definition of type tosca.artifacts.nfv.SwImage. There shall be a maximum number of one such artifact definition in a tosca.nodes.nfv.Vdu.Compute node template. The node template name of type tosca.nodes.nfv.Vdu.Compute fulfills the purpose of the "SwImageDescId" attribute of the SwImageDesc information element in ETSI GS NFV-IFA 011 [1] and hence it will be used in APIs to identify the software image id from the VNFD perspective. See example in clause 6.8.3.8.

VNF Providers shall use the datatype tosca.datatypes.nfv.VnfcConfigurableProperties to derive the type of configurable_properties when introducing this property with VDU-specific elements in a Vdu.Compute specific node type.

The node template name of type tosca.nodes.nfv.Vdu.Compute fulfills the purpose of the 'virtualComputeDescId' attribute of the virtualComputeDesc information element in ETSI GS NFV-IFA 011 [1] and hence it will be used in APIs to identify the virtual compute id (vnfdVirtualComputeDescId).

NOTE:	The use of the node template name of type tosca.nodes.nfv.Vdu.Compute for the 'virtualComputeDescId' attribute of the virtualComputeDesc information element in ETSI GS NFV-IFA 011 [1] implies in the present document version a one-to-one mapping of virtualComputeDesc with VDU. This deviates from the ETSI GS NFV-IFA 011 [1] modelling that defines a mapping where a virtualComputeDesc can be reused by one or more VDU, i.e. it implies a one-to-many mapping of virtualComputeDesc with VDU. This can have an impact in the determination of the number of compute flavours needed to be created with the VIM.

#### 6.8.3.8	Example

This example illustrates boot data values as string.

tosca_definitions_version: tosca_simple_yaml_1_2

..

node_types:

mycompany.nodes.nfv.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

..

modifiable_attributes:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes

..

data_types:

mycompany.datatypes.nfv.VnfInfoModifiableAttributes:

derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributes

properties:

extensions:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions

required: false

mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions:

derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions

properties:

http_proxy:

type: string

required: true

https_proxy:

type: string

required: false

topology_template:

inputs:

modifiable_attributes:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes

substitution_mappings:

node_type: mycompany.nodes.nfv.SunshineDB.1_0.1_0

..

node_templates:

vnf:

type: mycompany.nodes.nfv.SunshineDB.1_0.1_0

properties:

..

modifiable_attributes: { get_input: modifiable_attributes }

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

boot_data: { concat: [

"#!/bin/bash\n",

"echo setting HTTP proxy to: ", { get_property: [vnf, modifiable_attributes, extensions, http_proxy ] }, "\n",

"..."

] }

..

This example illustrates fetching the boot data value from a file.

tosca_definitions_version: tosca_simple_yaml_1_2

..

topology_template:

..

node_templates:

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

boot_data: { get_artifact: [ SELF, user_data ] }

..

artifacts:

user_data:

file: db-backend-init.yaml

type: tosca.artifacts.File

..

This example illustrates the association of a software image artifact to a Vdu.Compute node. The name of the artifact "dbBackend" will be used in external APIs to identify the image.

tosca_definitions_version: tosca_simple_yaml_1_2

..

topology_template:

..

node_templates:

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

sw_image_data:

..

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: images/dbBackend.v1.0.1.qcow2

..

This example illustrates the association of a software image artifact to more than Vdu.Compute nodes. The name of the artifact "dbBackend and oamService" will be used in external APIs to identify the image of each Vdu.Compute node.

tosca_definitions_version: tosca_simple_yaml_1_2

..

topology_template:

..

node_templates:

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

sw_image_data:

..

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: images/dbBackend.v1.0.1.qcow2

oamService:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

sw_image_data:

..

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: images/dbBackend.v1.0.1.qcow2

..

### 6.8.4	tosca.nodes.nfv.Vdu.VirtualBlockStorage

#### 6.8.4.1	Description

The VirtualBlockStorage node type describes the specifications of requirements related to virtual block storage resources, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.8.4.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.4.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualBlockStorage |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualBlockStorage |
| Type URI | tosca.nodes.nfv.Vdu.VirtualBlockStorage |

#### 6.8.4.2	Properties

The properties of the VirtualBlockStorage node type shall comply with the provisions set out in table 6.8.4.2-1.

Table 6.8.4.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_block_storage_data | yes | tosca.datatypes.nfv.VirtualBlockStorage |  | Describes the block storage characteristics. |
| sw_image_data | no | tosca.datatypes.nfv.SwImageData |  | Defines information related to a SwImage artifact used by this Vdu.Compute node. |

#### 6.8.4.3	Attributes

None

#### 6.8.4.4	Requirements

None.

#### 6.8.4.5	Capabilities

The capabilities of the VirtualBlockStorage node type shall comply with the provisions set out in table 6.8.4.5-1.

Table 6.8.4.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_storage | tosca.capabilities.nfv.VirtualStorage |  | Defines the capabilities of virtual_storage. |

#### 6.8.4.6	Definition

The syntax of the VirtualBlockStorage node type shall comply with the following definition:

tosca.nodes.nfv.Vdu.VirtualBlockStorage: derived_from: tosca.nodes.Root description: This node type describes the specifications of requirements related to virtual block storage resources properties: virtual_block_storage_data: type: tosca.datatypes.nfv.VirtualBlockStorageData description: Describes the block storage characteristics. required: true sw_image_data: type: tosca.datatypes.nfv.SwImageData description: Defines information related to a SwImage artifact used by this Vdu.Compute node. required: false # property is required when the node template has an associated artifact of type tosca.artifacts.nfv.SwImage and not required otherwise capabilities: virtual_storage: type: tosca.capabilities.nfv.VirtualStorage description: Defines the capabilities of virtual_storage.

#### 6.8.4.7	Additional requirements

Node templates of type tosca.nodes.nfv.Vdu.VirtualBlockStorage may contain an artifact definition of type tosca.artifacts.nfv.SwImage. There shall be a maximum number of one such artifact definition in a tosca.nodes.nfv.Vdu.VirtualBlockStorage node template. The node template name of type tosca.nodes.nfv.Vdu.VirtualBlockStorage fulfills the purpose of the "SwImageDescId" attribute of the SwImageDesc information element in ETSI GS NFV-IFA 011 [1] and hence it will be used in APIs to identify the software image id from the VNFD descriptor. See example in clause 6.8.3.8.

### 6.8.5	tosca.nodes.nfv.Vdu.VirtualObjectStorage

#### 6.8.5.1	Description

The VirtualObjectStorage node type describes the specifications of requirements related to virtual object storage resources, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.8.5.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.5.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualObjectStorage |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualObjectStorage |
| Type URI | tosca.nodes.nfv.Vdu.VirtualObjectStorage |

#### 6.8.5.2	Properties

The properties of the VirtualObjectStorage node type shall comply with the provisions set out in table 6.8.5.2-1.

Table 6.8.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_object_storage_data | yes | tosca.datatypes.nfv.VirtualObjectStorageData |  | Describes the object storage characteristics. |

#### 6.8.5.3	Attributes

None.

#### 6.8.5.4	Requirements

None.

#### 6.8.5.5	Capabilities

The capabilities of the VirtualObjectStorage node type shall comply with the provisions set out in table 6.8.5.5-1.

Table 6.8.5.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_storage | tosca.capabilities.nfv.VirtualStorage |  | Defines the capabilities of virtual_storage. |

#### 6.8.5.6	Definition

The syntax of the VirtualObjectStorage node type shall comply with the following definition:

tosca.nodes.nfv.Vdu.VirtualObjectStorage: derived_from: tosca.nodes.Root description: This node type describes the specifications of requirements related to virtual object storage resources properties: virtual_object_storage_data: type: tosca.datatypes.nfv.VirtualObjectStorageData description: Describes the object storage characteristics. required: true capabilities: virtual_storage: type: tosca.capabilities.nfv.VirtualStorage description: Defines the capabilities of virtual_storage.

#### 6.8.5.7	Additional requirements

None.

### 6.8.6	tosca.nodes.nfv.Vdu.VirtualFileStorage

#### 6.8.6.1	Description

The VirtualFileStorage node type describes the specifications of requirements related to virtual file storage resources, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.8.6.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.6.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualFileStorage |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualFileStorage |
| Type URI | tosca.nodes.nfv.Vdu.VirtualFileStorage |

#### 6.8.6.2	Properties

The properties of the VirtualFileStorage node type shall comply with the provisions set out in table 6.8.6.2-1.

Table 6.8.6.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_file_storage_data | yes | tosca.datatypes.nfv.VirtualFileStorageData |  | Describes the file storage characteristics. |

#### 6.8.6.3	Attributes

None.

#### 6.8.6.4	Requirements

The requirements of the VirtualFileStorage node type shall comply with the provisions set out in table 6.8.6.4-1.

Table 6.8.6.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_link | yes | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link |

#### 6.8.6.5	Capabilities

The capabilities of the VirtualFileStorage node type shall comply with the provisions set out in table 6.8.6.5-1.

Table 6.8.6.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_storage | tosca.capabilities.nfv.VirtualStorage |  | Defines the capabilities of virtual_storage. |

#### 6.8.6.6	Definition

The syntax of the VirtualFileStorage node type shall comply with the following definition:

tosca.nodes.nfv.Vdu.VirtualFileStorage: derived_from: tosca.nodes.Root description: This node type describes the specifications of requirements related to virtual file storage resources properties: virtual_file_storage_data: type: tosca.datatypes.nfv.VirtualFileStorageData description: Describes the file storage characteristics. required: true capabilities: virtual_storage: type: tosca.capabilities.nfv.VirtualStorage #description: Defines the capabilities of virtual_storage. requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo # description: Describes the requirements for linking to virtual link

#### 6.8.6.7	Additional requirements

None.

### 6.8.7	tosca.nodes.nfv.Cp

#### 6.8.7.1	Description

A Cp node type represents the Cpd information element as defined in ETSI GS NFV-IFA 011 [1], which describes network connectivity to a compute resource or a VL. This is an abstract type used as parent for the various Cp node types. Table 6.8.7.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.7.1-1: Type name, shorthand, and URI

| Shorthand Name | Cp |
| --- | --- |
| Type Qualified Name | toscanfv:Cp |
| Type URI | tosca.nodes.nfv.Cp |

#### 6.8.7.2	Properties

The properties of the Cp node type shall comply with the provisions set out in table 6.8.7.2-1.

Table 6.8.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| layer_protocols | yes | list of string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | Identifies which protocol the connection point uses for connectivity purposes. |
| role | no | string | valid values: root, leaf | Identifies the role of the port in the context of the traffic flow patterns in the VNF or parent NS. For example a VNF with a tree flow pattern within the VNF will have legal cpRoles of ROOT and LEAF. |
| description | no | string |  | Provides human-readable information on the purpose of the connection point (e.g. connection point for control plane traffic). |
| protocol | no | list of tosca.datatypes.nfv.CpProtocolData |  | Provides information on the addresses to be assigned to the connection point(s) instantiated from this Connection Point Descriptor. |
| trunk_mode | no | boolean |  | Information about whether the CP instantiated from this Cp is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. Absence of this property implies that trunkMode is not configured for the Cp i.e. It is equivalent to boolean value "false". |

#### 6.8.7.3	Attributes

None.

#### 6.8.7.4	Requirements

None.

#### 6.8.7.5	Capabilities

None.

#### 6.8.7.6	Definition

The syntax of the Cp node type shall comply with the following definition:

tosca.nodes.nfv.Cp: derived_from: tosca.nodes.Root description: Provides information regarding the purpose of the connection point properties: layer_protocols: type: list description: Identifies which protocol the connection point uses for connectivity purposes required: true entry_schema: type: string constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] role: #Name in ETSI GS NFV-IFA 011 v0.7.3: cpRole type: string description: Identifies the role of the port in the context of the traffic flow patterns in the VNF or parent NS required: false constraints: - valid_values: [ root, leaf ] description: type: string description: Provides human-readable information on the purpose of the connection point required: false protocol: type: list description: Provides information on the addresses to be assigned to the connection point(s) instantiated from this Connection Point Descriptor required: false entry_schema: type: tosca.datatypes.nfv.CpProtocolData trunk_mode: type: boolean description: Provides information about whether the CP instantiated from this Cp is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. Absence of this property implies that trunkMode is not configured for the Cp i.e. It is equivalent to boolean value "false". required: false

#### 6.8.7.7	Additional requirements

The 'protocol' property shall not be included in a derived PnfExtCp node and shall be included in all other cases.

### 6.8.8	tosca.nodes.nfv.VduCp

#### 6.8.8.1	Description

A VduCp node type represents the VduCpd information element as defined in ETSI GS NFV-IFA 011 [1], which describes network connectivity between a VNFC instance (based on VDU) and an internal VL. Table 6.8.8.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.8.1-1: Type name, shorthand, and URI

| Shorthand Name | VduCp |
| --- | --- |
| Type Qualified Name | toscanfv:VduCp |
| Type URI | tosca.nodes.nfv.VduCp |

#### 6.8.8.2	Properties

The properties of the VduCp node type shall comply with the provisions set out in table 6.8.8.2-1.

Table 6.8.8.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| bitrate_requirement | no | integer | greater_or_equal: 0 | Bitrate requirement in bit per second on this connection point. |
| virtual_network_interface_requirements | no | list of tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements |  | Specifies requirements on a virtual network interface realizing the CPs instantiated from this CPD. |
| order | no | integer | greater_or_equal: 0 | The order of the NIC on the compute instance (e.g. eth2). See note. compute (aka multi vNICs) and ordering is desired, it is *mandatory* that all ports will be set with an order value and. The order values shall represent a positive, arithmetic progression that starts with 0 (e.g. 0, 1, 2, …, n). If the property is not present, it shall be left to the VIM to assign the value when creating the instance. |
| vnic_type | no | string | Allowed values: normal, virtio, direct-physical | Describes the type of the virtual network interface realizing the CPs instantiated from this CPD. This is used to determine which mechanism driver(s) to be used to bind the port. |
| Note: When binding more than one port to a single. |  |  |  |  |

#### 6.8.8.3	Attributes

None.

#### 6.8.8.4	Requirements

The requirements of the VduCp node type shall comply with the provisions set out in table 6.8.8.4-1.

Table 6.8.8.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_binding | yes | tosca.capabilities.nfv.VirtualBindable |  | Describe the requirement for binding with VDU |
| virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link |

#### 6.8.8.5	Definition

The syntax of the VduCp node type shall comply with the following definition:

tosca.nodes.nfv.VduCp: derived_from: tosca.nodes.nfv.Cp description: describes network connectivity between a VNFC instance based on this VDU and an internal VL properties: bitrate_requirement: type: integer # in bits per second description: Bitrate requirement in bit per second on this connection point required: false constraints: - greater_or_equal: 0 virtual_network_interface_requirements: type: list description: Specifies requirements on a virtual network interface realising the CPs instantiated from this CPD required: false entry_schema: type: tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements order: type: integer description: The order of the NIC on the compute instance (e.g.eth2) required: false constraints: - greater_or_equal: 0 vnic_type: type: string description: Describes the type of the virtual network interface realizing the CPs instantiated from this CPD required: false constraints: - valid_values: [ normal, virtio, direct-physical ] requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo - virtual_binding: capability: tosca.capabilities.nfv.VirtualBindable relationship: tosca.relationships.nfv.VirtualBindsTo node: tosca.nodes.nfv.Vdu.Compute

### 6.8.9	tosca.nodes.nfv.VnfVirtualLink

#### 6.8.9.1	Description

The VnfVirtualLink node type represents the VnfVirtualLinkDesc information element as defined in ETSI GS NFV-IFA 011 [1], which describes the information about an internal VNF VL. Table 6.8.9.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.8.9.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfVirtualLink |
| --- | --- |
| Type Qualified Name | toscanfv:VnfVirtualLink |
| Type URI | tosca.nodes.nfv.VnfVirtualLink |

#### 6.8.9.2	Properties

The properties of the VnfVirtualLink node type shall comply with the provisions set out in table 6.8.9.2-1.

Table 6.8.9.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| connectivity_type | yes | ConnectivityType |  | Specifies the protocol exposed by the VL and the flow pattern supported by the VL. |
| description | no | string |  | Provides human-readable information on the purpose of the VL (e.g. control plane traffic). |
| test_access | no | list of string | Valid values: passive_monitoring, active_loopback | Test access facilities available on the VL. |
| vl_profile | yes | tosca.datatypes.nfv.VlProfile |  | Defines additional data for the VL: maximum and minimum bit rate requirements and QoS. |
| monitoring_parameters | no | list of tosca.datatypes.nfv.VirtualLinkMonitoringParameter |  | Describe monitoring parameters applicable to a VL instantiated from this node type. |

#### 6.8.9.3	Requirements

None.

#### 6.8.9.4	Capabilities

The capabilities of the VnfVirtualLink node type shall comply with the provisions set out in table 6.8.9.4-1.

Table 6.8.9.4-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_linkable | tosca.capabilities.nfv.VirtualLinkable |  | Defines ability of VirtualLinkable. |

#### 6.8.9.5	Definition

The syntax of the VnfVirtualLink node type shall comply with the following definition:

tosca.nodes.nfv.VnfVirtualLink: derived_from: tosca.nodes.Root description: Describes the information about an internal VNF VL properties: connectivity_type: type: tosca.datatypes.nfv.ConnectivityType description: Specifies the protocol exposed by the VL and the flow pattern supported by the VL required: true description: type: string description: Provides human-readable information on the purpose of the VL required: false test_access: type: list description: Test access facilities available on the VL required: false entry_schema: type: string constraints: - valid_values: [ passive_monitoring, active_loopback ] vl_profile: type: tosca.datatypes.nfv.VlProfile description: Defines additional data for the VL required: true monitoring_parameters: type: list entry_schema: type: tosca.datatypes.nfv.VirtualLinkMonitoringParameter description: Describes monitoring parameters applicable to the VL required: false capabilities: virtual_linkable: type: tosca.capabilities.nfv.VirtualLinkable

## 6.9	Group Types

### 6.9.1	tosca.groups.nfv.PlacementGroup

#### 6.9.1.1	Description

PlacementGroup is used for describing the affinity or anti-affinity relationship applicable between the virtualisation containers to be created based on different VDUs, or between internal VLs to be created based on different VnfVirtualLinkDesc(s). Table 6.9.1.1-1 specifies the declared names for this group type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.9.1.1-1: Type name, shorthand, and URI

| Shorthand Name | PlacementGroup |
| --- | --- |
| Type Qualified Name | toscanfv:PlacementGroup |
| Type URI | tosca.groups.nfv.PlacementGroup |

#### 6.9.1.2	Properties

The properties of the PlacementGroup group type shall comply with the provisions set out in table 6.9.1.2-1.

Table 6.9.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | yes | string |  | Human readable description of the group |

#### 6.9.1.3	Definition

The syntax of the PlacementGroup group type shall comply with the following definition:

tosca.groups.nfv.PlacementGroup: derived_from: tosca.groups.Root description: PlacementGroup is used for describing the affinity or anti-affinity relationship applicable between the virtualization containers to be created based on different VDUs, or between internal VLs to be created based on different VnfVirtualLinkDesc(s) properties: description: type: string description: Human readable description of the group required: true members: [ tosca.nodes.nfv.Vdu.Compute, tosca.nodes.nfv.VnfVirtualLink ]

#### 6.9.1.4	Additional Requirements

A group with type tosca.groups.nfv.PlacementGroup shall contain more than one member when used as the target of an AffinityRule or AntiAffinityRule policy.

#### 6.9.1.5	Examples

See clause 6.10.10.5.

## 6.10	Policy Types

### 6.10.1	tosca.policies.nfv.InstantiationLevels

#### 6.10.1.1	Description

The InstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour and including default instantiation level in term of the number of VNFC instances to be created as defined in ETSI GS NFV-IFA 011 [1]. This policy concerns the whole VNF (deployment flavour) represented by the topology_template and thus has no explicit target list. Table 6.10.1.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.1.1-1: Type name, shorthand, and URI

| Shorthand Name | InstantiationLevels |
| --- | --- |
| Type Qualified Name | toscanfv:InstantiationLevels |
| Type URI | tosca.policies.nfv.InstantiationLevels |

#### 6.10.1.2	Properties

The properties of the InstantiationLevels policy type shall comply with the provisions set out in table 6.10.1.2-1.

Table 6.10.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| levels | yes | map of tosca.datatypes.nfv.InstantiationLevel |  | Describes the various levels of resources that can be used to instantiate the VNF using this flavour. |
| default_level | no | string |  | The default instantiation level for this flavour. |

#### 6.10.1.3	Definition

The syntax of the InstantiationLevels policy type shall comply with the following definition:

tosca.policies.nfv.InstantiationLevels: derived_from: tosca.policies.Root description: The InstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour and including default instantiation level in term of the number of VNFC instances to be created as defined in ETSI GS NFV-IFA 011 [1]. properties: levels: type: map # key: levelId description: Describes the various levels of resources that can be used to instantiate the VNF using this flavour. required: true entry_schema: type: tosca.datatypes.nfv.InstantiationLevel constraints: - min_length: 1 default_level: type: string # levelId description: The default instantiation level for this flavour. required: false # required if multiple entries in levels

### 6.10.2	tosca.policies.nfv.VduInstantiationLevels

#### 6.10.2.1	Description

The VduInstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour in term of the number of VNFC instances to be created from each vdu.Compute. as defined in ETSI GS NFV-IFA 011 [1].

Table 6.10.2.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.2.1-1: Type name, shorthand, and URI

| Shorthand Name | VduInstantiationLevels |
| --- | --- |
| Type Qualified Name | toscanfv:VduInstantiationLevels |
| Type URI | tosca.policies.nfv.VduInstantiationLevels |

#### 6.10.2.2	Properties

The properties of the VduInstantiationLevels policy type shall comply with the provisions set out in table 6.10.2.2-1.

Table 6.10.2.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| levels | yes | map of tosca.datatypes.nfv.VduLevel |  | Describes the Vdu.Compute levels of resources that can be used to instantiate the VNF using this flavour. |

#### 6.10.2.3	Definition

The syntax of the VduInstantiationLevels policy type shall comply with the following definition:

tosca.policies.nfv.VduInstantiationLevels: derived_from: tosca.policies.Root description: The VduInstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour in term of the number of VNFC instances to be created from each vdu.Compute. as defined in ETSI GS NFV-IFA 011 [1] properties: levels: type: map # key: levelId description: Describes the Vdu.Compute levels of resources that can be used to instantiate the VNF using this flavour required: true entry_schema: type: tosca.datatypes.nfv.VduLevel constraints: - min_length: 1 targets: [ tosca.nodes.nfv.Vdu.Compute ]

#### 6.10.2.4	Additional Requirements

A VduInstantiationLevels policy shall contain an entry for each instantiation level (and only for them) defined in the InstantiationLevels policy.

### 6.10.3	tosca.policies.nfv.VirtualLinkInstantiationLevels

#### 6.10.3.1	Description

The VirtualLinkInstantiationLevels type is a policy type representing all the instantiation levels of virtual link resources to be instantiated within a deployment flavour as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.3.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.3.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkInstantiationLevels |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkInstantiationLevels |
| Type URI | tosca.policies.nfv.VirtualLinkInstantiationLevels |

#### 6.10.3.2	Properties

The properties of the VirtualLinkInstantiationLevels policy type shall comply with the provisions set out in table 6.10.3.2-1.

Table 6.10.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| levels | yes | map of tosca.datatypes.nfv.VirtualLinkBitrateLevel |  | Describes the virtual link levels of resources that can be used to instantiate the VNF using this flavour. |

#### 6.10.3.3	Definition

The syntax of the VirtualLinkInstantiationLevels policy type shall comply with the following definition:

tosca.policies.nfv.VirtualLinkInstantiationLevels: derived_from: tosca.policies.Root description: The VirtualLinkInstantiationLevels type is a policy type representing all the instantiation levels of virtual link resources to be instantiated within a deployment flavour as defined in ETSI GS NFV-IFA 011 [1]. properties: levels: type: map # key: levelId description: Describes the virtual link levels of resources that can be used to instantiate the VNF using this flavour. required: true entry_schema: type: tosca.datatypes.nfv.VirtualLinkBitrateLevel constraints: - min_length: 1 targets: [ tosca.nodes.nfv.VnfVirtualLink ]

#### 6.10.3.4	Additional Requirements

A VirtualLinkInstantiationLevels policy shall contain an entry for each instantiation level (and only for them) defined in the InstantiationLevels policy.

### 6.10.4	Void

### 6.10.5	tosca.policies.nfv.ScalingAspects

#### 6.10.5.1	Description

The ScalingAspects type is a policy type representing the scaling aspects used for horizontal scaling as defined in ETSI GS NFV-IFA 011 [1]. This policy concerns the whole VNF (deployment flavour) represented by the topology_template and thus has no explicit target list. Table 6.10.5.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.5.1-1: Type name, shorthand, and URI

| Shorthand Name | ScalingAspects |
| --- | --- |
| Type Qualified Name | toscanfv:ScalingAspects |
| Type URI | tosca.policies.nfv.ScalingAspects |

#### 6.10.5.2	Properties

The properties of the ScalingAspects policy type shall comply with the provisions set out in table 6.10.5.2-1.

Table 6.10.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspects | yes | Map of tosca.datatypes.nfv.ScalingAspect |  | Describe maximum scale level for total number of scaling steps that can be applied to a particular aspect. |

#### 6.10.5.3	Definition

The syntax of the ScalingAspects policy type shall comply with the following definition:

tosca.policies.nfv.ScalingAspects: derived_from: tosca.policies.Root description: The ScalingAspects type is a policy type representing the scaling aspects used for horizontal scaling as defined in ETSI GS NFV-IFA 011 [1] properties: aspects: type: map # key: aspectId description: Describe maximum scale level for total number of scaling steps that can be applied to a particular aspect required: true entry_schema: type: tosca.datatypes.nfv.ScalingAspect constraints: - min_length: 1

#### 6.10.5.4	Examples

See clause A.6.

### 6.10.6	tosca.policies.nfv.VduScalingAspectDeltas

#### 6.10.6.1	Description

The VduScalingAspectDeltas type is a policy type representing the Vdu.Compute detail of an aspect deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.6.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.6.1-1: Type name, shorthand, and URI

| Shorthand Name | VduScalingAspectDeltas |
| --- | --- |
| Type Qualified Name | toscanfv:VduScalingAspectDeltas |
| Type URI | tosca.policies.nfv.VduScalingAspectDeltas |

#### 6.10.6.2	Properties

The properties of the VduScalingAspectDeltas policy type shall comply with the provisions set out in table 6.10.6.2-1.

Table 6.10.6.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspect: | yes | string |  | Represents the scaling aspect to which this policy applies. |
| deltas | yes | map of tosca.datatypes.nfv.VduLevel |  | Describes the Vdu.Compute scaling deltas to be applied for every scaling steps of a particular aspect. |

#### 6.10.6.3	Definition

The syntax of the VduScalingAspectDeltas policy type shall comply with the following definition:

tosca.policies.nfv.VduScalingAspectDeltas: derived_from: tosca.policies.Root description: The VduScalingAspectDeltas type is a policy type representing the Vdu.Compute detail of an aspect deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1] properties: aspect: type: string description: Represents the scaling aspect to which this policy applies required: true deltas: type: map # key: scalingDeltaId description: Describes the Vdu.Compute scaling deltas to be applied for every scaling steps of a particular aspect. required: true entry_schema: type: tosca.datatypes.nfv.VduLevel constraints: - min_length: 1 targets: [ tosca.nodes.nfv.Vdu.Compute ]

#### 6.10.6.4	Examples

See clause A.6.

### 6.10.7	tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas

#### 6.10.7.1	Description

The VirtualLinkBitrateScalingAspectDeltas type is a policy type representing the VnfVirtualLink detail of an aspect deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.7.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.7.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkBitrateScalingAspectDeltas |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkBitrateScalingAspectDeltas |
| Type URI | tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas |

#### 6.10.7.2	Properties

The properties of the VirtualLinkBitrateScalingAspectDeltas policy type shall comply with the provisions set out in table 6.10.7.2-1.

Table 6.10.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspect | yes | string |  | Represents the scaling aspect to which this policy applies. |
| aspect_deltas | yes | map of tosca.datatypes.nfv.VirtualLinkBitrateLevel |  | Describes the VnfVirtualLink scaling deltas to be applied for every scaling steps of a particular aspect. |

#### 6.10.7.3	Definition

The syntax of the VirtualLinkBitrateScalingAspectDeltas policy type shall comply with the following definition:

tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas: derived_from: tosca.policies.Root description: The VirtualLinkBitrateScalingAspectDeltas type is a policy type representing the VnfVirtualLink detail of an aspect deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. properties: aspect: type: string description: Represents the scaling aspect to which this policy applies. required: true deltas: type: map # key: scalingDeltaId description: Describes the VnfVirtualLink scaling deltas to be applied for every scaling steps of a particular aspect. required: true entry_schema: type: tosca.datatypes.nfv.VirtualLinkBitrateLevel constraints: - min_length: 1 targets: [ tosca.nodes.nfv.VnfVirtualLink ]

#### 6.10.7.4	Examples

See clause A.6.

### 6.10.8	tosca.policies.nfv.VduInitialDelta

#### 6.10.8.1	Description

The VduInitialDelta type is a policy type representing the Vdu.Compute detail of an initial delta used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.8.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.8.1-1: Type name, shorthand, and URI

| Shorthand Name | VduInitialDelta |
| --- | --- |
| Type Qualified Name | toscanfv:VduInitialDelta |
| Type URI | tosca.policies.nfv.VduInitialDelta |

#### 6.10.8.2	Properties

The properties of the VduInitialDelta policy type shall comply with the provisions set out in table 6.10.8.2-1.

Table 6.10.8.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| initial_delta: | yes | tosca.datatypes.nfv.VduLevel |  | Represents the initial minimum size of the VNF. |

#### 6.10.8.3	Definition

The syntax of the VduInitialDelta policy type shall comply with the following definition:

tosca.policies.nfv.VduInitialDelta: derived_from: tosca.policies.Root description: The VduInitialDelta type is a policy type representing the Vdu.Compute detail of an initial delta used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. properties: initial_delta: type: tosca.datatypes.nfv.VduLevel description: Represents the initial minimum size of the VNF. required: true targets: [ tosca.nodes.nfv.Vdu.Compute ]

#### 6.10.8.4	Examples

See clause A.6.

### 6.10.9	tosca.policies.nfv.VirtualLinkBitrateInitialDelta

#### 6.10.9.1	Description

The VirtualLinkBitrateInitialDelta type is a policy type representing the VnfVirtualLink detail of an initial deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.9.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.9.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkBitrateInitialDelta |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkBitrateInitialDelta |
| Type URI | tosca.policies.nfv.VirtualLinkBitrateInitialDelta |

#### 6.10.9.2	Properties

The properties of the VirtualLinkBitrateInitialDelta policy type shall comply with the provisions set out in table 6.10.9.2-1.

Table 6.10.9.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| initial_delta: | yes | tosca.datatypes.nfv.VirtualLinkBitrateLevel |  | Represents the initial minimum size of the VNF. |

#### 6.10.9.3	Definition

The syntax of the VirtualLinkBitrateInitialDelta policy type shall comply with the following definition:

tosca.policies.nfv.VirtualLinkBitrateInitialDelta: derived_from: tosca.policies.Root description: The VirtualLinkBitrateInitialDelta type is a policy type representing the VnfVirtualLink detail of an initial deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. properties: initial_delta: type: tosca.datatypes.nfv.VirtualLinkBitrateLevel description: Represents the initial minimum size of the VNF. required: true targets: [ tosca.nodes.nfv.VnfVirtualLink ]

#### 6.10.9.4	Examples

See clause A.6.

### 6.10.10	AffinityRule, AntiAffinityRule

#### 6.10.10.1	Description

The AffinityRule or AntiAffinityRule describes the affinity or anti-affinity rules applicable for the defined targets:

If there is only one node template with node type tosca.nodes.nfv.Vdu.Compute or tosca.nodes.nfv.VnfVirtualLink set as the targets, the AffinityRule or AntiAffinityRule applies between the virtualisation containers to be created based on a particular VDU, or between internal VLs to be created based on a particular VnfVirtualLinkDesc as described in ETSI GS NFV-IFA 011 [1].

If there are more than one node templates with node type tosca.nodes.nfv.Vdu.Compute or tosca.nodes.nfv.VnfVirtualLink set as the targets, or a group with type tosca.groups.nfv.PlacementGroup which contains more than one members set as targets, the AffinityRule or AntiAffinityRule applies between the virtualisation containers to be created based on different VDUs, or between internal VLs to be created based on different VnfVirtualLinkDesc(s) as described in ETSI GS NFV-IFA 011 [1].

Tables 6.10.10.1-1 and 6.10.10.1-2 specify the declared names for the policy types. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.10.1-1: Type name, shorthand, and URI

| Shorthand Name | AffinityRule |
| --- | --- |
| Type Qualified Name | toscanfv:AffinityRule |
| Type URI | tosca.policies.nfv.AffinityRule |

Table 6.10.10.1-2: Type name, shorthand, and URI

| Shorthand Name | AntiAffinityRule |
| --- | --- |
| Type Qualified Name | toscanfv:AntiAffinityRule |
| Type URI | tosca.policies.nfv.AntiAffinityRule |

#### 6.10.10.2	Properties

The properties of the AffinityRule and AntiAffinityRule types shall comply with the provisions set out in table 6.10.10.2-1.

Table 6.10.10.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scope | Yes | String | Possible values are "nfvi_pop", "zone", "zone_group", "nfvi_node". | Specifies the scope of the local affinity rule |

#### 6.10.10.3	targets

The targets of the AffinityRule and AntiAffinityRule policy types shall comply with the provisions set out in table 6.10.10.3-1.

Table 6.10.10.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | Yes | string[] | Valid types: tosca.nodes.nfv.Vdu.Compute tosca.nodes.nfv.VnfVirtualLink tosca.groups.nfv.PlacementGroup | In case of LocalAffinityOrAntiAffinityRule as defined in ETSI GS NFV-IFA 011 [1], the valid type of the targets is tosca.nodes.nfv.Vdu.Compute or tosca.nodes.nfv.VnfVirtualLink. In case of affinityOrAntiAffinityGroup as defined in ETSI GS NFV-IFA 011 [1], the valid types of the targets are: tosca.nodes.nfv.Vdu.Compute and tosca.nodes.nfv.VnfVirtualLink or a tosca.groups.nfv.PlacementGroup. |

#### 6.10.10.4	Definition

The syntax of the AffinityRule policy type shall comply with the following definition:

tosca.policies.nfv.AffinityRule: derived_from: tosca.policies.Placement description: The AffinityRule describes the affinity rules applicable for the defined targets properties: scope: type: string description: scope of the rule is an NFVI_node, an NFVI_PoP, etc. required: true constraints: - valid_values: [ nfvi_node, zone, zone_group, nfvi_pop ] targets: [ tosca.nodes.nfv.Vdu.Compute, tosca.nodes.nfv.VnfVirtualLink, tosca.groups.nfv.PlacementGroup ]

The syntax of the AntiAffinityRule policy type shall comply with the following definition:

tosca.policies.nfv.AntiAffinityRule: derived_from: tosca.policies.Placement description: The AntiAffinityRule describes the anti-affinity rules applicable for the defined targets properties: scope: type: string description: scope of the rule is an NFVI_node, an NFVI_PoP, etc. required: true constraints: - valid_values: [ nfvi_node, zone, zone_group, nfvi_pop ] targets: [ tosca.nodes.nfv.Vdu.Compute, tosca.nodes.nfv.VnfVirtualLink, tosca.groups.nfv.PlacementGroup ]

#### 6.10.10.5	Examples

The following example template fragments illustrate the concepts:

node_templates: VDU_1: type: tosca.nodes.nfv.Vdu.Compute policies: policy_affinity_local_VDU_1: type: tosca.policies.nfv.AffinityRule targets: [ VDU_1 ] properties: scope: nfvi_node

The above example illustrates a local affinity rule for VDU_1.

node_template: VDU_1: type: tosca.nodes.nfv.Vdu.Compute VDU_2: type: tosca.nodes.nfv.Vdu.Compute groups: affinityOrAntiAffinityGroup_1: type: tosca.groups.nfv.PlacementGroup members: [ VDU_1, VDU_2 ] policies: policy_antiaffinity_group_1: type: tosca.policies.nfv.AntiAffinityRule targets: [ affinityOrAntiAffinityGroup_1 ] properties: scope: nfvi_node

The above example illustrates an anti-affinity policy among a group which contains VDU_1 and VDU_2 as members.

### 6.10.11	tosca.policies.nfv.SecurityGroupRule

#### 6.10.11.1	Description

The SecurityGroupRule type is a policy type specifying the matching criteria for the ingress and/or egress traffic to and from visited connection points as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.11.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.11.1-1: Type name, shorthand, and URI

| Shorthand Name | SecurityGroupRule |
| --- | --- |
| Type Qualified Name | toscanfv:SecurityGroupRule |
| Type URI | tosca.policies.nfv.SecurityGroupRule |

#### 6.10.11.2	Properties

The properties of the SecurityGroupRule policy type shall comply with the provisions set out in table 6.10.11.2-1.

Table 6.10.11.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | no | string |  | Human readable description of the security group rule. |
| direction | no | string | ingress, egress | The direction in which the security group rule is applied. The direction of 'ingress' or 'egress' is specified against the associated CP. I.e. 'ingress' means the packets entering a CP, while 'egress' means the packets sent out of a CP. |
| ether_type | no | string | ipv4, ipv6 | Indicates the protocol carried over the Ethernet layer. |
| protocol | no | string | see note | Indicates the protocol carried over the IP layer. Permitted values: any protocol defined in the IANA protocol registry [i.5], e.g. TCP, UDP, ICMP, etc. |
| port_range_min | no | integer | 0 - 65535 | Indicates minimum port number in the range that is matched by the security group rule. If a value is provided at design-time, this value may be overridden at run-time based on other deployment requirements or constraints. |
| port_range_max | no | integer | 0 – 65535 | Indicates maximum port number in the range that is matched by the security group rule. If a value is provided at design-time, this value may be overridden at run-time based on other deployment requirements or constraints. |
| NOTE: "protocol" constraints values: hopopt, icmp, igmp, ggp, ipv4, st, tcp, cbt, egp, igp, bbn_rcc_mon, nvp_ii, pup, argus, emcon, xnet, chaos, udp, mux, dcn_meas, hmp, prm, xns_idp, trunk_1, trunk_2, leaf_1, leaf_2, rdp, irtp, iso_tp4, netblt, mfe_nsp, merit_inp, dccp, 3pc, idpr, xtp, ddp, idpr_cmtp, tp++, il, ipv6, sdrp, ipv6_route, ipv6_frag, idrp, rsvp, gre, dsr, bna, esp, ah, i_nlsp, swipe, narp, mobile, tlsp, skip, ipv6_icmp, ipv6_no_nxt, ipv6_opts, cftp, sat_expak, kryptolan, rvd, ippc, sat_mon, visa, ipcv, cpnx, cphb, wsn, pvp, br_sat_mon, sun_nd, wb_mon, wb_expak, iso_ip, vmtp, secure_vmtp, vines, ttp, iptm, nsfnet_igp, dgp, tcf, eigrp, ospfigp, sprite_rpc, larp, mtp, ax.25, ipip, micp, scc_sp, etherip, encap, gmtp, ifmp, pnni, pim, aris, scps, qnx, a/n, ip_comp, snp, compaq_peer, ipx_in_ip, vrrp, pgm, l2tp, ddx, iatp, stp, srp, uti, smp, sm, ptp, isis, fire, crtp, crudp, sscopmce, iplt, sps, pipe, sctp, fc, rsvp_e2e_ignore, mobility, udp_lite, mpls_in_ip, manet, hip, shim6, wesp, rohc. |  |  |  |  |

#### 6.10.11.3	Definition

The syntax of the SecurityGroupRule policy type shall comply with the following definition:

tosca.policies.nfv.SecurityGroupRule: derived_from: tosca.policies.Root description: The SecurityGroupRule type is a policy type specified the matching criteria for the ingress and/or egress traffic to/from visited connection points as defined in ETSI GS NFV-IFA 011 [1]. properties: description: type: string description: Human readable description of the security group rule. required: false direction: type: string description: The direction in which the security group rule is applied. The direction of 'ingress' or 'egress' is specified against the associated CP. I.e., 'ingress' means the packets entering a CP, while 'egress' means the packets sent out of a CP. required: false constraints: - valid_values: [ ingress, egress ] default: ingress ether_type: type: string description: Indicates the protocol carried over the Ethernet layer. required: false constraints: - valid_values: [ ipv4, ipv6 ] default: ipv4 protocol: type: string description: Indicates the protocol carried over the IP layer. Permitted values include any protocol defined in the IANA protocol registry, e.g. TCP, UDP, ICMP, etc. required: false constraints: - valid_values: [ hopopt, icmp, igmp, ggp, ipv4, st, tcp, cbt, egp, igp, bbn_rcc_mon, nvp_ii, pup, argus, emcon, xnet, chaos, udp, mux, dcn_meas, hmp, prm, xns_idp, trunk_1, trunk_2, leaf_1, leaf_2, rdp, irtp, iso_tp4, netblt, mfe_nsp, merit_inp, dccp, 3pc, idpr, xtp, ddp, idpr_cmtp, tp++, il, ipv6, sdrp, ipv6_route, ipv6_frag, idrp, rsvp, gre, dsr, bna, esp, ah, i_nlsp, swipe, narp, mobile, tlsp, skip, ipv6_icmp, ipv6_no_nxt, ipv6_opts, cftp, sat_expak, kryptolan, rvd, ippc, sat_mon, visa, ipcv, cpnx, cphb, wsn, pvp, br_sat_mon, sun_nd, wb_mon, wb_expak, iso_ip, vmtp, secure_vmtp, vines, ttp, iptm, nsfnet_igp, dgp, tcf, eigrp, ospfigp, sprite_rpc, larp, mtp, ax.25, ipip, micp, scc_sp, etherip, encap, gmtp, ifmp, pnni, pim, aris, scps, qnx, a/n, ip_comp, snp, compaq_peer, ipx_in_ip, vrrp, pgm, l2tp, ddx, iatp, stp, srp, uti, smp, sm, ptp, isis, fire, crtp, crudp, sscopmce, iplt, sps, pipe, sctp, fc, rsvp_e2e_ignore, mobility, udp_lite, mpls_in_ip, manet, hip, shim6, wesp, rohc ] default: tcp port_range_min: type: integer description: Indicates minimum port number in the range that is matched by the security group rule. If a value is provided at design-time, this value may be overridden at run-time based on other deployment requirements or constraints. required: false constraints: - greater_or_equal: 0 - less_or_equal: 65535 default: 0 port_range_max: type: integer description: Indicates maximum port number in the range that is matched by the security group rule. If a value is provided at design-time, this value may be overridden at run-time based on other deployment requirements or constraints. required: false constraints: - greater_or_equal: 0 - less_or_equal: 65535 default: 65535 targets: [ tosca.nodes.nfv.VduCp, tosca.nodes.nfv.VnfExtCp ]

#### 6.10.11.4	Additional Requirements

If an ingress traffic is accepted, the corresponding egress response shall be allowed, regardless of the security group rules on the egress direction. If an egress traffic is allowed, the corresponding ingress response shall be accepted, regardless of the security group rules on the ingress direction. The design of security group rule follows a permissive model where all security group rules applied to a connection point are dealt with in an "OR" logic fashion, i.e. the traffic is allowed if it matches any security group rule applied to this connection point.

### 6.10.12	tosca.policies.nfv.SupportedVnfInterface

#### 6.10.12.1	Description

The SupportedVnfInterface policy type represents interfaces produced by a VNF, the details to access them and the applicable connection points to use to access these interfaces. It corresponds to the VnfInterfaceDetails information element defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.12.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 6.10.12.1-1: Type name, shorthand, and URI

| Shorthand Name | SupportedVnfInterface |
| --- | --- |
| Type Qualified Name | toscanfv:SupportedVnfInterface |
| Type URI | tosca.policies.nfv.SupportedVnfInterface |

#### 6.10.12.2	Properties

The properties of the SupportedVnfInterface policy type shall comply with the provisions set out in table 6.10.12.2-1.

Table 6.10.12.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| interface_name | yes | string | Valid values vnf_indicator vnf_configuration | Identifies an interface produced by the VNF. |
| details | no | tosca.datatypes.nfv.InterfaceDetails |  | Provide additional data to access the interface endpoint (e.g. API URI prefix). |

#### 6.10.12.3	Definition

The syntax of the SupportedVnfInterface policy type shall comply with the following definition:

tosca.policies.nfv.SupportedVnfInterface: derived_from: tosca.policies.Root description: this policy type represents interfaces produced by a VNF, the details to access them and the applicable connection points to use to access these interfaces properties: interface_name: type: string description: Identifies an interface produced by the VNF. required: true constraints: - valid_values: [ vnf_indicator, vnf_configuration ] details: type: tosca.datatypes.nfv.InterfaceDetails description: Provide additional data to access the interface endpoint required: false targets: [ tosca.nodes.nfv.VnfExtCp, tosca.nodes.nfv.VduCp ]

#### 6.10.12.4	Additional requirements

The valid targets for this policy type shall be the node templates representing the connection point descriptors from which to instantiate the connection point instances through which the interfaces can be accessed. This may be a VnfExtCp node template or a VduCp node template when an internal connection point is re-exposed externally.

#### 6.10.12.5	Example

policies: policy_interface_1: type: tosca.policies.nfv.SupportedVnfInterface targets: [ MyVnfmFacingExtCp ] properties: interface_name: vnf_indicator details: uri_components: scheme: https authority: host: myvnf.example.com

## 6.11	VNFD TOSCA service template design

### 6.11.1	General

The TOSCA service template design for a VNFD in the general case uses two levels of service templates as described in clause 6.11.2. In this design, the top level contains an abstract node template, i.e. without an implementation of the creation operation and is therefore substituted by one of the lower level service templates. This design is applicable regardless of whether the VNF has one or multiple deployment flavours.

In the particular case of a VNF with only one deployment flavour there is an alternative design which is described in clause 6.11.3 and which uses only one service template.

### 6.11.2	Single or multiple deployment flavour design with TOSCA-Simple-Profile-YAML-v1.2

VNFD shall be implemented as one top-level service template and one or multiple lower level service templates, where each lower level service template represents a deployment flavour. In addition, an abstract node type representing the VNF with multiple deployment flavours is also required. The top level service template shall be the main entry point of the VNF package as specified in ETSI GS NFV-SOL 004 [i.6], i.e. the Entry-definitions file. The node type definition is included in a separate YAML file imported from the top and from the lower level files.

See clause A.2 for an example of VNFD design with multiple deployment flavours.

The top level service template shall comply with TOSCA-Simple-Profile-YAML-v1.2 [3] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.2;

an import statement referencing the definitions of all low level service template(s);

an import statement referencing a yaml file which contains a VNF specific node type definition which shall be derived from tosca.nodes.nfv.VNF node type as defined in clause 6.8.1;

Optionally, import statements referencing additional VNF-specific files containing type definitions used by this TOSCA service template; and

a topology template with a node template of the VNF specific node type, which shall include the flavour_id and other properties and the requirements as defined in clause 6.8.1.

The deployment and lifecycle management of instances of this VNF node type is done by means of substitution by any of the lower level service templates.

The lower level service template is an implementable TOSCA service template for the deployment of a specific deployment flavour. The lower level service template shall comply with TOSCA-Simple-Profile-YAML-v1.2 [3] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.2;

an import statement referencing a yaml file which contains a VNF specific node type definition which shall be derived from tosca.nodes.nfv.VNF node type as defined in clause 6.8.1;

Optionally, import statements referencing additional VNF-specific files containing type definitions used by this TOSCA service template; and

a topology template describing the internal topology of the VNF with:

substitution_mappings indicating:

the same node type as defined in the top level service template;

a flavour_id property value which identifies the DF corresponding to this low level template within the VNFD; and

the mapping of the virtual_link requirements on external connection points;

a node template referencing the VNF specific node type, implementations of the operations of the LCM interface to be executed by the VNFM, if applicable; and

additional node templates of type Vdu.Compute, Vdu.VirtualBlockStorage, Vdu.VirtualObjectStorage, Vdu.VirtualFileStorage VduCp, etc. that define the topology and composition of the VNF flavour.

The yaml file with the VNF specific node type definition shall be included in the VNF package.

NOTE:	The format and structure of a VNF package is defined in ETSI GS NFV-SOL 004 [i.6].

When the flavour_id of a VNF has been chosen (e.g. through an input parameter of a VNF instantiation request received by a VNFM) among the values included in the VNF node type imported into the top level service template, it is then used as the property constraint for selecting a particular lower level TOSCA service template inside the VNF package by using abstract node template matching with substitution as described in TOSCA-Simple-Profile-YAML-v1.2 [3].

### 6.11.3	Single deployment flavour design with TOSCA-Simple-Profile-YAML-v1.1

In case of single deployment flavour scenario, TOSCA-Simple-Profile-YAML-v1.1 [4] may be used to describe the VNFD with a single TOSCA service template.

NOTE 1:	The present document does not preclude this pattern using TOSCA-Simple-Profile-YAML-v1.2 [3]. In this case one single service template is used as specified in TOSCA-Simple-Profile-YAML-v1.1 [4] and includes:

- an import statement referencing the TOSCA types definition file as defined in clause B.2;

- Optionally, import statements referencing additional VNF-specific files containing type definitions used by this TOSCA service template;

c)	an abstract VNF node type definition derived from the tosca.nodes.nfv.VNF node type, as defined in clause 6.8.1; and

d)	a topology template describing the internal topology of the VNF with:

-	substitution_mappings indicating the same abstract VNF node type and the mapping of the virtual_link requirements on external connection points;

-	a node template of this abstract VNF node type with the flavour_id and other properties and, if applicable, implementations of the operations of the LCM interface to be executed by the VNFM; and

-	additional node templates of type Vdu.Compute, Vdu.VirtualBlockStorage, Vdu.VirtualObjectStorage, Vdu.VirtualFileStorage, VduCp, etc. that define the topology and composition of the VNF flavour.

See clause A.5 for an example of VNFD design with single deployment flavour.

NOTE 2:	The service template is deployed stand-alone, i.e. without performing a substitution. However including the substitution_mappings rule indicate its ability to substitute a node template of the abstract VNF node type, which may appear in an NSD.

# 7	NSD TOSCA model

## 7.1	Introduction

The NSD information model specified by ETSI GS NFV-IFA 014 [2] is mapped to the TOSCA concepts. NSD occurrences are represented as TOSCA service templates, as defined in the TOSCA Simple YAML Profile v1.2 [3], to be used by the NFVO for managing the lifecycle of NS instances.

Table 7.1-1 shows an overview of the mapping between the main NSD information elements defined in ETSI GS NFV-IFA 014 [2] and TOSCA types defined in the present document. The definition of all TOSCA types for representing all information elements is described in the following clauses.

NOTE:	There is only part of NSD features as specified in ETSI GS NFV-IFA 014 [2] included in this version of the present document, the following are features not included:

Multiple NS deployment flavours

NS scaling Aspect

Multiple Ns instantiation levels

Nested NS design

Vnffgd

NS Monitoring

VnfIndicators

Dependencies

Table 7.1-1: Mapping of ETSI GS NFV-IFA 014 [2] information elements with TOSCA types

| ETSI NFV Information Element ETSI GS NFV-IFA 014 [2] | TOSCA type | Derived from |
| --- | --- | --- |
| NSD | tosca.nodes.nfv.NS | tosca.nodes.Root |
| Sapd | tosca.nodes.nfv.Sap | tosca.nodes.Root |
| NsVirtualLinkDesc | tosca.nodes.nfv.NsVirtualLink | tosca.nodes.Root |
| Pnfd | tosca.nodes.nfv.PNF | tosca.nodes.Root |
| Vnfd | tosca.nodes.nfv.VNF | tosca.nodes.Root |

## 7.2	Data Types

### 7.2.1	tosca.datatype.nfv.ServiceAvailability

#### 7.2.1.1	Description

ServiceAvailability describes the information on the Service Availability Level for a particular virtual link flavour, as defined in ETSI GS NFV-IFA 014 [2] and ETSI GS NFV-REL 001 [i.3]. Table 7.2.1.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 7.2.1.1-1: Type name, shorthand, and URI

| Shorthand Name | ServiceAvailability |
| --- | --- |
| Type Qualified Name | toscanfv:ServiceAvailability |
| Type URI | tosca.datatypes.nfv.ServiceAvailability |

#### 7.2.1.2	Properties

The properties of the ServiceAvailability data type shall comply with the provisions set out in table 7.2.1.2-1.

Table 7.2.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| level | yes | string | valid_values: [level1, level2, level3] | Service available level as defined in [i.3] |

#### 7.2.1.3	Definition

The syntax of the ServiceAvailability data type shall comply with the following definition:

tosca.datatypes.nfv.ServiceAvailability: derived_from: tosca.datatypes.Root description: service availability properties: level: type: string description: service availability levels required: true constraints: - valid_values: [ level1, level2, level3 ]

#### 7.2.1.4	Examples

None.

### 7.2.2	tosca.datatypes.nfv.VnfProfile

#### 7.2.2.1	Description

The VnfProfile data type is defined in clause 6.2.52 of the present document.

### 7.2.3	tosca.datatype.nfv.NsVlProfile

#### 7.2.3.1	Description

The NsVlProfile data type describes additional instantiation data for a given NsVirtualLink used in a specific NS deployment flavour. Table 7.2.3.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 7.2.3.1-1: Type name, shorthand, and URI

| Shorthand Name | NsVlProfile |
| --- | --- |
| Type Qualified Name | toscanfv:NsVlProfile |
| Type URI | tosca.datatypes.nfv.NsVlProfile |

#### 7.2.3.2	Properties

The properties of the NsVlProfile data type shall comply with the provisions set out in table 7.2.3.2-1.

Table 7.2.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| max_bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Specifies the maximum bitrate requirements for a VL instantiated according to this profile. |
| min_bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Specifies the minimum bitrate requirements for a VL instantiated according to this profile. |
| qos | no | tosca.datatypes.nfv.NsVirtualLinkQos |  | Specifies the QoS requirements of a VL instantiated according to this profile. |
| service_availability | no | tosca.datatypes.nfv.ServiceAvailability |  | Network service virtual link service availability levels, as described in ETSI GS NFV-REL 001 [i.3]. |
| NOTE: A virtualLinkDescId property, which exists in ETSI GS NFV-IFA 014 [2] is not needed, as the NsVlProfile is contained in the NsVirtualLink node. |  |  |  |  |

#### 7.2.3.3	Definition

The syntax of the NsVlProfile data type shall comply with the following definition:

tosca.datatypes.nfv.NsVlProfile: derived_from: tosca.datatypes.Root description: Describes additional instantiation data for a given NsVirtualLink used in a specific NS deployment flavour. properties: max_bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Specifies the maximum bitrate requirements for a VL instantiated according to this profile. required: true min_bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Specifies the minimum bitrate requirements for a VL instantiated according to this profile. required: true qos: type: tosca.datatypes.nfv.NsVirtualLinkQos description: Specifies the QoS requirements of a VL instantiated according to this profile. required: false service_availability: type: tosca.datatypes.nfv.ServiceAvailability description: Network service virtual link service availability levels, as described in ETSI GS NFV-REL 001 required: false

#### 7.2.3.4	Examples

None.

#### 7.2.3.5	Additional Requirements

None.

### 7.2.4	tosca.datatypes.nfv.ConnectivityType

#### 7.2.4.1	Description

The ConnectivityType data type is defined in clause 6.2.5 of the present document.

### 7.2.5	tosca.datatypes.nfv. NsVirtualLinkQos

#### 7.2.5.1	Description

The NsVirtualLinkQoS describes QoS data type a given NsVirtualLink used in an NS deployment flavour. Table 7.2.5.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 7.2.5.1-1: Type name, shorthand, and URI

| Shorthand Name | NsVirtualLinkQos |
| --- | --- |
| Type Qualified Name | toscanfv:NsVirtualLinkQos |
| Type URI | tosca.datatypes.nfv.NsVirtualLinkQos |

#### 7.2.5.2	Properties

The properties of the NsVirtualLinkQos data type shall comply with the provisions set out in table 7.2.5.2-1.

Table 7.2.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| priority | no | integer | greater_or_equal: 0 | Specifies the priority level in case of congestion on the underlying physical links. |

#### 7.2.5.3	Definition

The syntax of the NsVirtualLinkQos data type shall comply with the following definition:

tosca.datatypes.nfv.NsVirtualLinkQos: derived_from: tosca.datatypes.nfv.Qos description: describes QoS data for a given VL used in a VNF deployment flavour properties: priority: type: integer constraints: - greater_or_equal: 0 description: Specifies the priority level in case of congestion on the underlying physical links required: false

#### 7.2.5.4	Examples

None.

#### 7.2.5.5	Additional Requirements

None.

### 7.2.6	tosca.datatypes.nfv.LinkBitrateRequirements

#### 7.2.6.1	Description

The LinkBitrateRequirements data type is defined in clause 6.2.29 of the present document.

### 7.2.7	Void

### 7.2.8	Void

### 7.2.9	Void

### 7.2.10	Void

### 7.2.11	tosca.datatypes.nfv.CpProtocolData

#### 7.2.11.1	Description

The CpProtocolData data type is defined in clause 6.2.36 of the present document.

### 7.2.12	tosca.datatypes.nfv.AddressData

#### 7.2.12.1	Description

The AddressData data type is defined in clause 6.2.3 of the present document.

### 7.2.13	tosca.datatypes.nfv.L2AddressData

#### 7.2.13.1	Description

The L2AddressData data type is defined in clause 6.2.1 of the present document.

### 7.2.14	tosca.datatypes.nfv.L3AddressData

#### 7.2.14.1	Description

The L3AddressData data type is defined in clause 6.2.2 of the present document.

### 7.2.15	tosca.datatypes.nfv.Qos

#### 7.2.15.1	Description

The Qos data type is defined in clause 6.2.30 of the present document.

7.2.16	tosca.datatypes.nfv.NsProfile

7.2.16.1	Description

The NsProfile data type describes a profile for instantiating nested NSs which are constituents of a NS with a particular NS DF as defined in ETSI GS NFV-IFA 014 [2]. Table 7.2.16.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 7.2.16.1-1: Type name, shorthand, and URI

| Shorthand Name | NsProfile |
| --- | --- |
| Type Qualified Name | toscanfv:NsProfile |
| Type URI | tosca.datatypes.nfv.NsProfile |

7.2.16.2	Properties

The properties of the NsProfile data type shall comply with the provisions set out in table 7.2.16.2-1.

Table 7.2.16.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ns_instantiation_level | no | string |  | Identifier of the instantiation level of the NS DF to be used for instantiation. If not present, the default instantiation level as declared in the NSD shall be used. |
| min_number_of_instances | yes | integer | greater_or_equal : 0 | Minimum number of instances of the NS based on this NSD that is permitted to exist for this NsProfile. |
| max_number_of_instances | yes | integer | greater_or_equal : 0 | Maximum number of instances of the NS based on this NSD that is permitted to exist for this NsProfile. |
| flavour_id | yes | string |  | Identifies the applicable network service DF within the scope of the NSD. |

7.2.16.3	Definition

The syntax of the NsProfile data type shall comply with the following definition:

tosca.datatypes.nfv.NsProfile: derived_from: tosca.datatypes.Root description: describes a profile for instantiating NSs of a particular NS DF according to a specific NSD and NS DF. properties: ns_instantiation_level: type: string description: Identifier of the instantiation level of the NS DF to be used for instantiation. If not present, the default instantiation level as declared in the NSD shall be used. required: false min_number_of_instances: type: integer description: Minimum number of instances of the NS based on this NSD that is permitted to exist for this NsProfile. required: true constraints: - greater_or_equal: 0 max_number_of_instances: type: integer description: Maximum number of instances of the NS based on this NSD that is permitted to exist for this NsProfile. required: true constraints: - greater_or_equal: 0 flavour_id: type: string description: Identifies the applicable network service DF within the scope of the NSD. required: true

7.2.16.4	Example

None.

7.2.16.5	Additional Requirements

None.

## 7.3	Artifact Types

None.

## 7.4	Capabilities Types

### 7.4.1	tosca.capabilities.nfv.VirtualLinkable

#### 7.4.1.1	Description

The VirtualLinkable capability type is defined in clause 6.4.2 of the present document.

## 7.5	Requirements Types

None.

## 7.6	Relationship Types

### 7.6.1	tosca.relationships.nfv.VirtualLinksTo

#### 7.6.1.1	Description

The VirtualLinksTo relationship type is defined in clause 6.6.2 of the present document representing an association relationship between the VNF or PNF or Sap of a Nested NS and NsVirtualLink node types when used in a NSD.

## 7.7	Interface Types

### 7.7.1	tosca.interfaces.nfv.Nslcm

#### 7.7.1.1	Description

The tosca.interfaces.nfv.Nslcm interface type contains a set of TOSCA operations corresponding to the following NS LCM operations defined in ETSI GS NFV-IFA 013 [i.8]:

Instantiate NS

Scale NS

Update NS

Heal NS

Terminate NS

The interface also contains TOSCA operations corresponding to preamble and postamble to the execution of the aforementioned base operations. The name of these operations is constructed according to the following pattern:

<base_operation_name>_start for a preamble

<base_operation_name>_end for a postamble

The designations ("_start", "_end") in the name of TOSCA operations are postfixes so that related operations are adjacent in an alphabetical listing.

Table 7.7.1.1-1 specifies the declared names for this interface type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 7.7.1.1-1: Type name, shorthand, and URI

| Shorthand Name | Nslcm |
| --- | --- |
| Type Qualified Name | toscanfv:Nslcm |
| Type URI | tosca.interfaces.nfv.Nslcm |

#### 7.7.1.2	Definition

The syntax of the Nslcm interface type shall comply with the following definition:

tosca.interfaces.nfv.Nslcm: derived_from: tosca.interfaces.Root description: This interface encompasses a set of TOSCA operations corresponding to NS LCM operations defined in ETSI GS NFV-IFA 013. as well as to preamble and postamble procedures to the execution of the NS LCM operations. instantiate_start: description: Preamble to execution of the instantiate operation instantiate: description: Base procedure for instantiating an NS, corresponding to the Instantiate NS operation defined in ETSI GS NFV-IFA 013. instantiate_end: description: Postamble to the execution of the instantiate operation terminate_start: description: Preamble to execution of the terminate operation terminate: description: Base procedure for terminating an NS, corresponding to the Terminate NS operation defined in ETSI GS NFV-IFA 013. terminate_end: description: Postamble to the execution of the terminate operation update_start: description: Preamble to execution of the update operation update: description: Base procedure for updating an NS, corresponding to the Update NS operation defined in ETSI GS NFV-IFA 013. update_end: description: Postamble to the execution of the update operation scale_start: description: Preamble to execution of the scale operation scale: description: Base procedure for scaling an NS, corresponding to the Scale NS operation defined in ETSI GS NFV-IFA 013. scale_end: description: Postamble to the execution of the scale operation heal_start: description: Preamble to execution of the heal operation heal: description: Base procedure for healing an NS, corresponding to the Heal NS operation defined in ETSI GS NFV-IFA 013. heal_end: description: Postamble to the execution of the heal operation

#### 7.7.1.3	Additional Requirements

The implementation and inputs keynames specified in TOSCA-Simple-Profile-YAML-v1.2 [3] for an operation definition may be included for each operation listed in the Nslcm interface definition.

When a TOSCA operation representing an NS LCM operation does not have an associated implementation keyname, the default implementation provided by the NFVO for this NS LCM operation applies.

The NSD consumer shall make available all parameters from the message invoking the NS LCM operation as inputs to the corresponding TOSCA interface operations. The inputs keyname can be used to specify additional input parameters for executing the operation.

The implementation of preamble and postamble TOSCA operations (e.g. instantiate_start), if present, is invoked with the same parameters as the corresponding base TOSCA operation (e.g. instantiate). The inputs of the preamble and postamble operations shall not be defined in the NSD.

If an implementation is associated to a TOSCA operation that represents a preamble or a postamble to a VNF LCM operation, the implementation logic is executed before or after the execution of the VNF LCM operation implementation, respectively.

#### 7.7.1.4	Support of LCM scripts

In ETSI GS NFV-IFA 014 [2], the definition of the "LifeCycleManagementScript" information element of the NSD associates LCM scripts with events, where an event can be an external or an internal stimulus. These events are mapped to TOSCA operations of the NS node type in the following way:

external stimuli are mapped to TOSCA operations corresponding to the NS LCM operations defined in ETSI GS NFV-IFA 013 [i.8];

internal stimuli are mapped to preamble and postamble of these TOSCA operations.

LCM scripts can be regarded as artifacts that provide an NS-specific implementation of the TOSCA operation corresponding to the stimulus.

The script input parameters shall be provided to the script according to the declaration in the inputs field of the operation definition. The artifact type definition shall enable identifying the DSL used by the script. The artifact type definition for Python is provided in section 5.4.4.1 of TOSCA-Simple-Profile-YAML-v1.2 [3]. The artifact definition for Mistral is provided in clause A.7.2 of the present document.

#### 7.7.1.5	Examples

The following example template fragments illustrate the concept. An LCM script is associated with the instantiate_end operation. As no LCM script is associated to the instantiate operation, its default implementation runs and before running the post-instantiate-script. The inputs section of the instantiate_end operation definition provides additional input values to the post-instantiate-script, and the TOSCA artifacts definition conveys the type of DSL used as a scripting language.

tosca_definitions_version: tosca_simple_yaml_1_2 imports: - .. node_types: MyCompany.SunshineVPN.1_0.1_0: derived_from: tosca.nodes.nfv.NS .. topology_template: substitution_mappings: node_type: MyCompany.SunshineVPN.1_0.1_0 .. node_templates: SunshineVPN: type: MyCompany.SunshineVPN.1_0.1_0 .. interfaces: Nslcm: instantiate_end: implementation: post-instantiate-script inputs: script_input_1: value_1 script_input_2: value_2 artifacts: post-instantiate-script: description: Instantiate workflow script type: tosca.artifacts.Implementation.Python file: instantiate.py #repository: .. #deploy_path: .. ..

## 7.8	Node Types

### 7.8.1	tosca.nodes.nfv.NS

#### 7.8.1.1	Description

The NFV Network Service (NS) node type describes a NS in terms of deployment, operational behaviour, and requirements, as defined by ETSI GS NFV-IFA 014 [2]. Table 7.8.1.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 7.8.1.1-1: Type name, shorthand, and URI

| Shorthand Name | NS |
| --- | --- |
| Type Qualified Name | toscanfv:NS |
| Type URI | tosca.nodes.nfv.NS |

#### 7.8.1.2	Properties

The properties of the NS node type shall comply with the provisions set out in table 7.8.1.2-1.

Table 7.8.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| descriptor_id | yes | string |  | Identifier of this NS descriptor. |
| designer | yes | string |  | Identifies the designer of the NSD. |
| version | yes | string |  | Identifies the version of the NSD. |
| name | yes | string |  | Provides the human readable name of the NSD. |
| invariant_id | yes | string |  | Identifies an NSD in a version independent manner. This attribute is invariant across versions of NSD. |
| flavour_id | yes | string |  | Identifier of this NS DF within the NSD. |
| ns_profile | no | tosca.datatypes.nfv.NsProfile |  | Specifies a profile of a NS, when this NS is used as nested NS within another NS. See note. |
| NOTE : This property is only used in an NS node template, when it is representing a nested NS within another NS. |  |  |  |  |

The value of the descriptor_id string shall comply with an UUID format as specified in section 3 of [9].

#### 7.8.1.3	Attributes

None.

#### 7.8.1.4	Requirements

The requirements of the NS node type shall comply with the provisions set out in table 7.8.1.4-1.

Table 7.8.1.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_link | yes | tosca.nodes.nfv.NsVirtualLink |  | Describes the requirements for linking to virtual link |

#### 7.8.1.5	Capabilities

None.

#### 7.8.1.6	Definition

The syntax of the NS node type shall comply with the following definition:

tosca.nodes.nfv.NS: derived_from: tosca.nodes.Root properties: descriptor_id: type: string # GUID description: Identifier of this NS descriptor required: true designer: type: string description: Identifies the designer of the NSD. required: true version: type: string description: Identifies the version of the NSD. required: true name: type: string description: Provides the human readable name of the NSD. required: true invariant_id: type: string description: Identifies an NSD in a version independent manner. This attribute is invariant across versions of NSD required: true flavour_id: type: string description: Identifier of the NS Deployment Flavour within the NSD required: true ns_profile: type: tosca.datatypes.nfv.NsProfile description: Specifies a profile of a NS, when this NS is used as nested NS within another NS. required: false requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo node: tosca.nodes.nfv.NsVirtualLink occurrences: [ 0, 1 ] interfaces: Nslcm: type: tosca.interfaces.nfv.Nslcm

#### 7.8.1.7	Artifact

None.

#### 7.8.1.8	Additional requirements

For a given NSD, a new VNF node type shall be defined following the below requirements:

- The node type shall be derived from: tosca.nodes.nfv.NS.

All properties listed in tosca.nodes.nfv.NS where the "required:" field is set to "true" shall be included with their values indicated as constraints.

Properties listed in in tosca.nodes.nfv.NS where the "required:" field is set to "false" may be included.

The capabilities, requirements, interfaces of tosca.nodes.nfv.VNF shall be preserved.

Depending on the number of SAPs of the NS, additional requirements for VirtualLinkable capability shall be defined. In this case, it is the NSD author's choice to use the requirement for VirtualLinkable capability defined in the tosca.nodes.nfv.NS node type or use only the additional requirements defined in the derived NS specific node type. In the latter case, the virtual_link requirement should be included in the node type definition with occurrences [ 0, 0 ].

### 7.8.2	tosca.nodes.nfv.Sap

#### 7.8.2.1	Description

The Service Access Point (SAP) node type describes a connection point where a NS can be accessed, as defined by ETSI GS NFV-IFA 014 [2]. Table 7.8.2.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 7.8.2.1-1: Type name, shorthand, and URI

| Shorthand Name | Sap |
| --- | --- |
| Type Qualified Name | toscanfv:Sap |
| Type URI | tosca.nodes.nfv.Sap |

#### 7.8.2.2	Properties

The properties applied to Sap node are derived from Cp node type as defined in clause 6.8.7 of the present document.

#### 7.8.2.3	Attributes

None.

#### 7.8.2.4	Requirements

The requirements of the Sap node type shall comply with the provisions set out in table 7.8.2.4-1.

Table 7.8.2.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| external_virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |
| internal_virtual_link | yes | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |

#### 7.8.2.5	Capabilities

None.

#### 7.8.2.6	Definition

The syntax of the Sap node type shall comply with the following definition:

tosca.nodes.nfv.Sap: derived_from: tosca.nodes.nfv.Cp description: node definition of SAP. requirements: - external_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo - internal_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo

#### 7.8.2.7	Additional requirements

A node template of this type is used to represent a SAP only in the case the Sap is connected to a NsVirtualLink inside a NSD. The node template has the following requirements:

internal_virtual_link requirement to allow to connect it to a NsVirtualLink inside a NSD;

external_virtual_link requirement to allow to connect it to a NsVirtualLink outside a NSD.

In the case where a Sap is exposed by a VNF external connection point, a PNF external connection point or a Sap of the nested NS, the Sap node type does not apply.

#### 7.8.2.8	Example

In a typical scenario, the Sap node template will be part of a service template representing a certain NS deployment flavour. The service template substitutes for an abstract NS node type. In this substitution, the virtual_link requirement of the abstract NS node type is mapped to the external_virtual_link requirement of the Sap node. This example is illustrated in clause A.7.3.

When a Sap re-exposes a VNF external connection point, the service template does not require an explicit node template of type Sap in a typical scenario where an abstract NS node type is substituted by a service template representing a certain NS deployment flavour. In this substitution, the virtual_link requirement of the abstract NS node type is mapped to the virtual_link requirement of the VNF node. This example is illustrated in clause A.7.2.

### 7.8.3	tosca.nodes.nfv.NsVirtualLink

#### 7.8.3.1	Description

The NsVirtualLink node type represents the NsVirtualLinkDesc information element as defined in ETSI GS NFV-IFA 014 [2], which describes the requirements for a virtual link of a network service. Table 7.8.3.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 7.8.3.1-1: Type name, shorthand, and URI

| Shorthand Name | NsVirtualLink |
| --- | --- |
| Type Qualified Name | toscanfv: NsVirtualLink |
| Type URI | tosca.nodes.nfv.NsVirtualLink |

#### 7.8.3.2	Properties

The properties of the NsVirtualLink node type shall comply with the provisions set out in table 7.8.3.2-1.

Table 7.8.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| vl_profile | yes | tosca.datatype.nfv.NsVlProfile |  | Specifies instantiation parameters for a virtual link of a particular NS deployment flavour. |
| connectivity_type | yes | tosca.datatypes.nfv.ConnectivityType |  | Network service virtual link connectivity type. |
| test_access | no | list of string | Valid values: passive_monitoring, active_loopback | Test access facilities available on the VL. |
| description | no | string |  | Human readable information on the purpose of the virtual link (e.g. VL for control plane traffic). |

#### 7.8.3.3	Attributes

None.

#### 7.8.3.4	Requirements

None.

#### 7.8.3.5	Capabilities

The capabilities of the NsVirtualLink node type shall comply with the provisions set out in table 7.8.3.5-1.

Table 7.8.3.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_linkable | tosca.capabilities.nfv.VirtualLinkable |  | VirtualLinkable capability |

#### 7.8.3.6	Definition

The syntax of the NsVirtualLink node type shall comply with the following definition:

tosca.nodes.nfv.NsVirtualLink: derived_from: tosca.nodes.Root description: node definition of Virtual Links properties: type: tosca.datatypes.nfv.NsVlProfile # only covers min/max bitrate requirements description: Specifies instantiation parameters for a virtual link of a particular NS deployment flavour. required: true connectivity_type: type: tosca.datatypes.nfv.ConnectivityType required: true test_access: type: list description: Test access facilities available on the VL required: false entry_schema: type: string constraints: - valid_values: [ passive_monitoring, active_loopback ] description: type: string required: false description: Human readable information on the purpose of the virtual link (e.g. VL for control plane traffic). capabilities: virtual_linkable: type: tosca.capabilities.nfv.VirtualLinkable

#### 7.8.3.7	Artifact

None.

#### 7.8.3.8	Additional Requirements

None.

#### 7.8.3.9	Example

None.

### 7.8.4	tosca.nodes.nfv.Cp

#### 7.8.4.1	Description

The Cp node type is defined in clause 6.8.7 of the present document.

## 7.9	Group Types

### 7.9.1	tosca.groups.nfv.PlacementGroup

#### 7.9.1.1	Description

The PlacementGroup group type is defined in clause 6.9.1 of the present document describing the affinity or anti-affinity relationship applicable between VNF instances created using different VNFDs, the Virtual Link instances created using different VLDs or the nested NS instances created using different NSDs when used in a NSD.

## 7.10	Policy Types

### 7.10.1	AffinityRule, AntiAffinityRule

#### 7.10.1.1	Description

The AffinityRule and AntiAffinityRule policy types as defined in clause 6.10.10 when used in a NSD TOSCA service template specify the affinity or anti-affinity rules applicable:

If there is only one node template with node type tosca.nodes.nfv.VNF or tosca.nodes.nfv.NsVirtualLink set as the targets, the AffinityRule or AntiAffinityRule applies between the instances to be created based on the same VNFD, or between VLs to be created based on the same VLD as described in ETSI GS NFV-IFA 014 [2].

If there are more than one node templates with node type tosca.nodes.nfv.VNF or tosca.nodes.nfv.NsVirtualLink or tosca.nodes.nfv.NS set as the targets, or a group with type tosca.groups.nfv.PlacementGroup which contains more than one members set as targets, the AffinityRule or AntiAffinityRule applies between VNF instances created using different VNFD, the Virtual Link instances created using different VLD or the nested NS instances created using different NSD. as described in ETSI GS NFV-IFA 014 [2].

#### 7.10.1.2	Targets

The targets of the AffinityRule and AntiAffinityRule policy types shall comply with the provisions set out in table 7.10.1.2-1 when used in a NSD.

Table 7.10.1.2-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | Yes | string[] | Valid types: tosca.nodes.nfv.VNF, tosca.nodes.nfv.NsVirtualLink, tosca.nodes.nfv.NS, tosca.groups.nfv.PlacementGroup | In case of LocalAffinityOrAntiAffinityRule as defined in ETSI GS NFV-IFA 014 [2], the valid type of the targets is tosca.nodes.nfv.VNF or tosca.nodes.nfv.NsVirtualLink. In case of affinityOrAntiAffinityGroup as defined in ETSI GS NFV-IFA 014 [2], the valid types of the targets are: tosca.nodes.nfv.VNF, tosca.nodes.nfv.NsVirtualLink and tosca.nodes.nfv.NS or a tosca.groups.nfv.PlacementGroup. |

#### 7.10.1.3	Examples

The following example template fragments illustrate the concepts:

node_templates: VNF_1: type: tosca.nodes.nfv.exampleVNF policies: policy_affinity_local_VNF_1: type: tosca.policies.nfv.AffinityRule targets: [ VNF_1 ] properties: scope: nfvi_node

The above example illustrates a local affinity rule for all the instances of VNF_1.

node_template: VNF_1: type: tosca.nodes.nfv.Vdu.exampleVNF_1 VNF_2: type: tosca.nodes.nfv.Vdu.exampleVNF_2 groups: affinityOrAntiAffinityGroup_1: type: tosca.groups.nfv.PlacementGroup members: [ VNF_1, VNF_2 ] policies: policy_antiaffinity_group_1: type: tosca.policies.nfv.AntiAffinityRule targets: [ affinityOrAntiAffinityGroup_1 ] properties: scope: nfvi_node

The above example illustrates an anti-affinity policy among a group which contains VNF_1 and VNF_2 as members.

### 7.10.2	tosca.policies.nfv.SecurityGroupRule

#### 7.10.2.1	Description

The SecurityGroupRule policy type as defined in clause 6.10.11 of the present document when used in a NSD specifies the matching criteria for the ingress and/or egress traffic to and from visited SAPs.

#### 7.10.2.2	Additional Requirements

The valid type of the targets of the SecurityGroupRule policy when used in a NSD shall set to tosca.nodes.nfv.Sap.

## 7.11	NSD TOSCA service template design

### 7.11.1	General

The TOSCA service template design for a NSD in the general case uses two levels of service templates as described in clause 7.11.2. In this design, the top level contains an abstract node template, i.e. without an implementation of the creation operation and is therefore substituted by one of the lower level service templates. This design is applicable regardless of whether the NS has one or multiple deployment flavours.

In the particular case of a NS with only one deployment flavour there is an alternative design which is described in clause 7.11.3 and which uses only one service template.

### 7.11.2	Single or multiple deployment flavour design with TOSCA-Simple-Profile-YAML-v1.2

NSD shall be implemented as one top-level service template and one or multiple lower level service templates, where each lower level service template represents a deployment flavour. In addition, an abstract node type representing the NS with multiple deployment flavours is also required. The top level service template shall be the main entry point of the NSD file structure as specified in ETSI GS NFV-SOL 007 [i.11], i.e. the Entry-definitions file. The node type definition is included in a separate YAML file imported from the top and from the lower level files.

See clause A.11 for an example of NSD design with multiple deployment flavours.

The top level service template shall comply with TOSCA-Simple-Profile-YAML-v1.2 [3] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.3;

an import statement referencing the definitions of all low level service template(s);

an import statement referencing a yaml file which contains a NS specific node type definition which shall be derived from tosca.nodes.nfv.NS node type as defined in clause 7.8.1;

a topology template with a node template of the NS-specific node type, which shall include the flavour_id and other properties and the requirements as defined in clause 7.8.1; and

Optionally, import statements referencing additional NS-specific files containing type definitions used by this TOSCA service template.

The deployment and lifecycle management of instances of this NS node type is done by means of substitution by any of the lower level service templates.

The lower level service template is an implementable TOSCA service template for the deployment of a specific deployment flavour.

The lower level service template shall comply with TOSCA-Simple-Profile-YAML-v1.2 [3] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.3;

an import statement referencing a yaml file which contains a NS specific node type definition which shall be derived from tosca.nodes.nfv.NS node type as defined in clause 7.8.1;

one of more import statements respectively referencing the yaml file which contains the included VNF specific node type definition if any or the included PNF specific node type definition if any or the included NS specific node type definition if any;

Optionally, import statements referencing additional NS-specific files containing type definitions used by this TOSCA service template; and

a topology template describing the internal topology of the NS with:

substitution_mappings indicating:

the same node type as defined in the top level service template;

a flavour_id property value which identifies the DF corresponding to this low level template within the NSD;

the mapping of the virtual_link requirements on SAPs;

a node template referencing the NS specific node type, implementations of the operations of the LCM interface to be executed by the NFVO, if applicable; and

additional node templates of type VNF, PNF, NS, NsVirtualLink, Sap, etc. that define the topology and composition of the NS flavour, the dependency requirements as defined in TOSCA-Simple-Profile-YAML-v1.2 [3] may be used between different VNF node templates, or between a VNF node template and a nested NS node template, or between different nested NS node templates to specify the order in which instances of the VNFs and/or nested NSs have to be created.

The yaml file with the NS-specific node type definition shall be included in the NSD file structure.

NOTE:	The format and structure of a NSD file structure is defined in ETSI GS NFV-SOL 007 [i.11].

When the flavour_id of a NS has been chosen (e.g. through an input parameter of a NS instantiation request received by a NFVO) among the values included in the NS node type imported into the top level service template, it is then used as the property constraint for selecting a particular lower level TOSCA service template inside the NSD file structure by using abstract node template matching with substitution as described in TOSCA-Simple-Profile-YAML-v1.2 [3].

### 7.11.3	Single deployment flavour design with TOSCA-Simple-Profile-YAML-v1.1

In case of single deployment flavour scenario, TOSCA-Simple-Profile-YAML-v1.1 [4] may be used to describe the NSD with a single TOSCA service template.

NOTE 1:	The present document does not preclude this pattern using TOSCA-Simple-Profile-YAML-v1.2 [3].

In this case one single service template is used as specified in TOSCA-Simple-Profile-YAML-v1.1 [4] and includes:

- an import statement referencing the TOSCA types definition file as defined in clause B.3;

- one of more import statements respectively referencing the yaml file which contains the included VNF specific node type definition if any or the included PNF specific node type definition if any or the included NS specific node type definition if any;

- optionally, import statements referencing additional NS-specific files containing type definitions used by this TOSCA service template;

- an abstract NS node type definition derived from the tosca.nodes.nfv.NS node type, as defined in clause 7.8.1; and

- a topology template describing the internal topology of the NS with:

substitution_mappings indicating the same abstract NS node type and the mapping of the virtual_link requirements on SAPs;

a node template of this abstract NS node type with the flavour_id and other properties and, if applicable, implementations of the operations of the LCM interface to be executed by the NFVO; and

additional node templates of type VNF, PNF, NS, NsVirtualLink, Sap, etc. that define the topology and composition of the NS flavour, the dependency requirements as defined in TOSCA-Simple-Profile-YAML-v1.2 [3] may be used between different VNF node templates, or between a VNF node template and a nested NS node template, or between different nested NS node templates to specify the order in which instances of the VNFs and/or nested NSs have to be created.

See clause A.8 for an example of NSD design with single deployment flavour.

NOTE 2:	The service template is deployed stand-alone, i.e. without performing a substitution. However including the substitution_mappings rule indicate its ability to substitute a node template of the abstract NS node type, which may appear in an NSD.

# 8	PNFD TOSCA model

## 8.1	Introduction

The PNFD information model specified by ETSI GS NFV-IFA 014 [2] is mapped to the TOSCA concepts. It represents as TOSCA topology template to be used by NFVO for preparing network connection.

Table 8.1-1 shows the TOSCA Type "derived from" values used when applying the TOSCA Simple YAML Profile [3] to the PNFD.

Table 8.1-1: Mapping of ETSI GS NFV-IFA 014 [2] information elements with TOSCA types

| ETSI NFV Information Element ETSI GS NFV-IFA 014 [2] | TOSCA type | Derived from |
| --- | --- | --- |
| Pnf | tosca.nodes.nfv.PNF | tosca.nodes.Root |
| PnfExtCpd (PNF External Connection Point) | tosca.nodes.nfv.PnfExtCp | tosca.nodes.Root |

Figure 8.1-1 provides an overview of the TOSCA node types used to build a service template for a PNFD.

Figure 8.1-1: Service template PNFD overview

## 8.2	Data Types

### 8.2.1	tosca.datatypes.nfv.CpProtocolData

#### 8.2.1.1	Description

The CpProtocolData data type is defined in clause 6.2.36 of the present document.

### 8.2.2	tosca.datatypes.nfv.AddressData

#### 8.2.2.1	Description

The AddressData data type is defined in clause 6.2.3 of the present document.

### 8.2.3	tosca.datatypes.nfv.L2AddressData

#### 8.2.3.1	Description

The L2AddressData data type is defined in clause 6.2.1 of the present document.

### 8.2.4	tosca.datatypes.nfv.L3AddressData

#### 8.2.4.1	Description

The L3AddressData data type is defined in clause 6.2.2 of the present document.

### 8.2.5	tosca.datatypes.nfv.LocationInfo

#### 8.2.5.1	Description

The LocationInfo data type represents geographical information on the location where a PNF is deployed as specified in ETSI GS NFV-IFA 011 [1]. Table 8.2.5.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 8.2.5.1-1: Type name, shorthand, and URI

| Shorthand Name | LocationInfo |
| --- | --- |
| Type Qualified Name | toscanfv:LocationInfo |
| Type URI | tosca.datatypes.nfv. LocationInfo |

#### 8.2.5.2	Properties

The properties of the LocationInfo data type shall comply with the provisions set out in table 8.2.5.2-1.

Table 8.2.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| country_code | yes | string |  | Shall be a two-letter ISO 3166 [10] country code in capital letters. |
| civic_address_element | no | list of tosca.datatypes.nfv.CivicAddressElement |  | Elements composing the civic address where the PNF is deployed. |

#### 8.2.5.3	Definition

The syntax of the LocationInfo data type shall comply with the following definition:

tosca.datatypes.nfv.LocationInfo: derived_from: tosca.datatypes.Root description: Represents geographical information on the location where a PNF is deployed. properties: country_code: type: string # two-letter ISO 3166 country code description: Country code required: true civic_address_element: type: list entry_schema type: tosca.datatypes.nfv.CivicAddressElement description: Elements composing the civic address where the PNF is deployed. required: false

#### 8.2.5.4	Examples

<some_tosca_entity>: properties: geographical_location_info: country_code: FR civic_address_element: - element_1 ca_type: 3 ca_value: Paris

#### 8.2.5.5	Additional Requirements

None.

### 8.2.6	tosca.datatypes.nfv.CivicAddressElement

#### 8.2.6.1	Description

The CivicAddressElement data type represents an element of a civic location as specified in IETF RFC 4776 [11]. Table 8.2.6.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 8.2.6.1-1: Type name, shorthand, and URI

| Shorthand Name | CivicAddressElement |
| --- | --- |
| Type Qualified Name | toscanfv: CivicAddressElement |
| Type URI | tosca.datatypes.nfv. CivicAddressElement |

#### 8.2.6.2	Properties

The properties of the CivicAddressElement data type shall comply with the provisions set out in table 8.2.6.2-1.

Table 8.2.6.2-1

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ca_type | yes | string |  | Describe the content type of caValue. The value of caType shall comply with section 3.4 of IETF RFC 4776 [11]. |
| ca_value | yes | string |  | Content of civic address element corresponding to the caType. The format caValue shall comply with section 3.4 of IETF RFC 4776 [11]. |

#### 8.2.6.3	Definition

The syntax of the CivicAddressElement data type shall comply with the following definition:

tosca.datatypes.nfv. CivicAddressElement: derived_from: tosca.datatypes.Root description: Represents an element of a civic location as specified in IETF RFC 4776 [11]. properties: ca_type: type: string # RFC4776 description: caType as per RFC4776 required: true ca_value: type: string # RFC4776 description: caValue as per RFC4776. required: true

#### 8.2.6.4	Examples

See clause 8.2.5.4.

#### 8.2.6.5	Additional Requirements

None.

## 8.3	Artifact Types

None.

## 8.4	Capabilities Types

### 8.4.1	tosca.capabilities.nfv.VirtualLinkable

#### 8.4.1.1	Description

The VirtualLinkable capability type is defined in clause 6.4.2 of the present document.

## 8.5	Requirements Types

None.

## 8.6	Relationship Types

### 8.6.1	tosca.relationships.nfv.VirtualLinksTo

#### 8.6.1.1	Description

The VirtualLinksTo relationship type is defined in clause 6.6.2 of the present document representing an association relationship between a PNF external connection point and an NsVirtualLink node type.

## 8.7	Interface Types

None.

## 8.8	Node Types

### 8.8.1	tosca.nodes.nfv.PNF

#### 8.8.1.1	Description

The Physical Network Function (PNF) node type describes a PNF in terms of deployment behaviour requirements, which it contains PNFD identifier, version and functional description and so on as defined by ETSI GS NFV-IFA 014 [2]. Table 8.8.1.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 8.8.1.1-1: Type name, shorthand, and URI

| Shorthand Name | PNF |
| --- | --- |
| Type Qualified Name | tosca:PNF |
| Type URI | tosca.nodes.nfv.PNF |

#### 8.8.1.2	Properties

The properties of the PNF node type shall comply with the provisions set out in table 8.8.1.2-1.

Table 8.8.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| descriptor_id | yes | string |  | Identifier of this PNFD information element. It uniquely identifies the PNFD. |
| function_description | yes | string |  | Describes the PNF function. |
| provider | yes | string |  | Identifies the provider of the PNFD. |
| version | yes | string |  | Identifies the version of the PNFD. |
| descriptor_invariant_id | yes | string |  | Identifier of this PNFD in a version independent manner. This attribute is invariant across versions of PNFD. |
| name | yes | string |  | Name to identify the PNFD. |
| geographical_location_info | no | tosca.datatype.nfv.LocationInfo |  | Provides information about the geographical location (e.g. geographic coordinates or address of the building, etc.) of the PNF. |

The value of the descriptor_id string and the value of the descriptor_invariant_id string shall comply with an UUID format as specified in section 3 of [8].

#### 8.8.1.3	Attributes

None.

#### 8.8.1.4	Requirements

The requirements of the VNF node type shall comply with the provisions set out in table 8.8.1.4-1.

Table 8.8.1.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_link | yes | tosca.nodes.nfv.VnfVirtualLink |  | Describes the requirements for linking to virtual link |

#### 8.8.1.5	Capabilities

None.

#### 8.8.1.6	Definition

tosca.nodes.nfv.PNF: derived_from: tosca.nodes.Root properties: descriptor_id: # instead of pnfd_id type: string # GUID required: true function_description: type: string required: true provider: type: string required: true version: type: string required: true descriptor_invariant_id: # instead of pnfd-invariant-id type: string # GUID required: true name: type: string required: true geographical_location_info: type: tosca.datatypes.nfv.LocationInfo required: false requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [ 0, 1 ] # Additional requirements shall be defined in the PNF specific node type (deriving from tosca.nodes.nfv.PNF) corresponding to NS virtual links that need to connect to PnfExtCps

#### 8.8.1.7	Artifact

None.

#### 8.8.1.8	Additional Requirements

For a given PNFD, a new PNF node type shall be defined following the below requirements:

- The node type shall be derived from: tosca.nodes.nfv.PNF.

All properties listed in tosca.nodes.nfv.PNF where the "required:" field is set to "true" shall be included with their values indicates as constraints.

The requirements of tosca.nodes.nfv.PNF shall be preserved.

Depending on the number of external connection points of the PNF that need to connect to NS virtual links, additional requirements for VirtualLinkable capability shall be defined. In this case, it is the PNFD author's choice to use the requirement for VirtualLinkable capability defined in the tosca.nodes.nfv.PNF node type or use only the additional requirements defined in the derived PNF specific node type. In the latter case, the virtual_link requirement should be included in the node type definition with occurrences [ 0, 0 ].

#### 8.8.1.9	Example

See clause A.10.

### 8.8.2	tosca.nodes.nfv.PnfExtCp

#### 8.8.2.1	Description

The PnfExtCp node type describes the characteristics of an external interface, a.k.a. an external CP, where to connect the PNF to a VL, as defined by ETSI GS NFV-IFA 014 [2]. Table 8.8.2.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.2 [3].

Table 8.8.2.1-1: Type name, shorthand, and URI

| Shorthand Name | PnfExtCp |
| --- | --- |
| Type Qualified Name | toscanfv:PnfExtCp |
| Type URI | tosca.nodes.nfv.PnfExtCp |

#### 8.8.2.2	Properties

The properties applied to PnfExtCp node are derived from Cp node type.

#### 8.8.2.3	Attributes

None.

#### 8.8.2.4	Requirements

The requirements of the PnfExtCp node type shall comply with the provisions set out in table 8.8.2.4-1.

Table 8.8.2.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| external_virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |

#### 8.8.2.5	Capabilities

None.

#### 8.8.2.6	Definition

The syntax of the PnfExtCp node type shall comply with the following definition:

tosca.nodes.nfv.PnfExtCp: derived_from: tosca.nodes.nfv.Cp description: node definition of PnfExtCp. requirements: - external_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo

### 8.8.3	tosca.nodes.nfv.Cp

#### 8.8.3.1	Description

The Cp node type is defined in clause 6.8.7 of the present document.

## 8.9	Group Types

None.

## 8.10	Policy Types

### 8.10.1	tosca.policies.nfv.SecurityGroupRule

#### 8.10.1.1	Description

The SecurityGroupRule policy type as defined in clause 6.10.11 of the present document when used in a PNFD specifies the matching criteria for the ingress and/or egress traffic to and from visited PNF external connection points.

#### 8.10.1.2	Additional Requirements

The valid type of the targets of the SecurityGroupRule policy when used in a PNFD shall set to tosca.nodes.nfv.PnfExtCp.

## 8.11	PNFD TOSCA service template design

### 8.11.1	General

One single TOSCA service template is used to design a PNFD which shall comply with TOSCA-Simple-Profile-YAML-v1.2 [3] and includes:

NOTE 1:	The present document does not preclude this pattern using TOSCA-Simple-Profile-YAML-v1.1 [4].

- an import statement referencing the TOSCA types definition file as defined in clause B.4;

- an abstract PNF node type definition derived from the tosca.nodes.nfv.PNF node type, as defined in clause 8.8.1; and

- a topology template describing the internal topology of the PNF with:

substitution_mappings indicating the same abstract PNF node type and the mapping of the virtual_link requirements on PNF external connection point;

a node template of this abstract PNF node type with the properties as defined in tosca.nodes.nfv.PNF; and

additional node templates of type PnfExtCp that define the connection information of the PNF.

See clause A.10 for an example of PNFD design.

NOTE 2:	The service template is deployed stand-alone, i.e. without performing a substitution. However including the substitution_mappings rule indicate its ability to substitute a node template of the abstract PNF node type, which may appear in an NSD.

Annex A (informative):
Examples

# A.1	Deployment flavour design mapping

## A.1.1	Introduction

This annex describes the main design principle for VNF deployment flavour and the mapping between VNF deployment flavour elements to TOSCA types.

## A.1.2	Design principle

Each Deployment flavour as specified in ETSI GS NFV-IFA 011 [1] describes a given deployment configuration of a VNF in terms of its internal topology and resource needs. Different deployment flavours can define different topologies of the same VNF, with different scalingAspect, different VDUs and different internal connectivity requirements. The idea of VNF deployment flavour as specified in [1] is that each deployment flavour describes the required vduProfiles and virtualLinkProfiles which are the additional instantiation data for the given VDUs and virtualLinks, once a specific deployment flavour has been chosen at the instantiation time, in order to successfully deploy the given VDUs and virtualLinks, the bindings between the VDU with the corresponding VduProfile and the virtualLinkDesc with the corresponding virtualLinkProfile are required.

To achieve the concept of deployment flavour by using TOSCA, the main design principle is to describe each deployment flavour as a standalone implementable TOSCA service template, and binding VDU and virtualLink with the corresponding VduProfile and virtualLinkProfile, respectively, together at the design time. Once a specific deployment flavour has been chosen at the instantiation time, the corresponding TOSCA service template will be used for deploying the VNF with the given deployment flavour.

Figure A.1.2-1 shows the general principle for a VNF deployment flavour design.

Figure A.1.2-1: General principle for a VNF deployment flavour design

# A.2	VNFD with deployment flavour modelling design example

Deployment flavours are represented as deployable TOSCA topology templates. This way one VNF service template represents one deployment flavour, and different deployment flavours are described by different VNF service templates. This is in line with the idea that different deployment flavours can define different topologies of the same VNF, with different scaling aspects, different VDUs and different internal connectivity.

In order to represent a VNF a top-level service template is used. This top-level service template contains a topology template with only an abstract VNF node which defines the common parts of the different deployment flavours (such as product information, modifiable attributes and parts of the lifecycle management interface definition). It also sets a constraint on the deployment flavour property (the required value of the flavour_id property); this constraint comes from the VNF instantiation request which contains a flavour_id selected among those available in the VNFD.

As a result, the VNFM will look into the available further service templates representing the different VNF deployment flavours of the VNF and use the one that has the matching flavour_id property value to substitute for the abstract VNF. These are the low-level service templates.

Figure A.2-1: VNFD overview with deployment flavour

A VNFD contains a TOSCA top-level Service Template as entry point in the VNF package and one or more TOSCA low-level Service templates representing the different deployment flavours (see figure A.2-1). The VNFD is interpreted by an NFVO or VNF manager. In this example, the templates describe two variants of the VNF each corresponding to a deployment flavour: a simple one and a complex one. The simple VNF consists of one server: a DB backend whereas, the complex VNF variant consists of minimum three DB backend servers and one serviceNode, which may be scaled out in one-size increments.

SunshineDB: VNFD-top level

sunshine.vnfd.tosca.yaml

tosca_definitions_version: tosca_simple_yaml_1_2

description: Relational database, non-scalable

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS

- sunshineVNF.yaml # contains the VNF node type definition

- sunshinedbsimple.vnfd.tosca.yaml

- sunshinedbcomplex.vnfd.tosca.yaml

topology_template:

inputs:

selected_flavour:

type: string

description: VNF deployment flavour selected by the consumer. It is provided in the API

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

properties:

flavour_id: { get_input: selected_flavour }

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider: MyCompany

product_name: SunshineDB

software_version: '1.0'

descriptor_version: '1.0'

vnfm_info:

- '0:MyCompany-1.0.0'

requirements:

#- virtual_link_backend # mapped in lower-level templates

#- virtual_link_service # mapped in lower-level templates

# get_input function would be used by a TOSCA orchestrator at run time to access the selected flavour id. If the deployment is not done by a TOSCA orchestrator above function may not be needed

The sunshineVNF.yaml file has the following content:

tosca_definitions_version: tosca_simple_yaml_1_2

description: Relational database, non-scalable

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS

data_types:

MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters:

derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters

properties:

parameter_1:

type: string

required: true

default: value_1

parameter_2:

type: string

required: false

default: value_2

node_types:

MyCompany.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider:

type: string

constraints: equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: SunshineDB ]

default: SunshineDB

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ valid_values: [ simple, complex ] ]

default: simple

flavour_description:

type: string

default: "" #empty string

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

occurrences: [ 0, 0 ]

- virtual_link_backend:

capability: tosca.capabilities.nfv.VirtualLinkable

- virtual_link_service:

capability: tosca.capabilities.nfv.VirtualLinkable

interfaces:

Vnflcm:

type: tosca.interfaces.nfv.Vnflcm

instantiate:

inputs:

additional_parameters:

type: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters

The vnf node template in the sunshine.vnfd.tosca.yaml file is abstract and is subject to substitution; the lower-level templates in the subsequent sections provide these substitutions. The actual lower-level template is selected based on the node type and a value constraint on the flavour_id property.

Each low level service template contains a node template of type MyCompany.SunshineDB.1_0.1_0 with implementation of the LCM interfaces.

SunshineDB (simple): Lower level

sunshinedbsimple.vnfd.tosca.yaml

This example illustrates one Vdu.Compute nodes (dbBackend) with two connection points and two virtual links (see figure A.2-2). The flavour_id is "Simple".

dbBackend

dbBackendInternalCp

internalVl

dbBackendCp

externalVl

VNF

Figure A.2-2: SunshineDB (simple): Lower level

tosca_definitions_version: tosca_simple_yaml_1_2

description: Relational database, simple

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS

- sunshineVNF.yaml # contains the VNF node type definition

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

properties:

..

flavour_id: simple

requirements:

virtual_link_backend: [ dbBackendCp, virtual_link ]  # IPv4 for SQL

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

properties:

flavour_description: A simple flavour

interfaces:

Vnflcm:

instantiate:

implementation: instantiate.workbook.mistral.yaml

terminate:

implementation: terminate.workbook.mistral.yaml

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: dbBackend

description: dbBackend compute node

boot_order: []

nfvi_constraints: []

configurable_properties: {}

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 8192 MiB

virtual_cpu:

cpu_architecture: x86

num_virtual_cpu: 2

virtual_cpu_clock: 1800 MHz

requirements:

- virtual_storage: mariaDbStorage

mariaDbStorage:

type: tosca.nodes.nfv.Vdu.VirtualBlockStorage

properties:

virtual_block_storage_data:

size_of_storage: 100 GB

rdma_enabled: true

sw_image_data:

name: Software of Maria Db

version: '1.0'

checksum:

algorithm: sha-256

hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

container_format: bare

disk_format: qcow2

min_disk: 2 GB

min_ram: 8192 MiB

size: 2 GB

operating_system: Linux

supported_virtualisation_environments:

- KVM

..

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: maria.db.image.v1.0.qcow2

dbBackendCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access the DB on IPv4

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: dbBackend

# - virtual_link: # the target node is determined in the NSD

dbBackendInternalCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: dbBackend

- virtual_link: internalVl

internalVl:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

flow_pattern: mesh

description: Internal Virtual link in the VNF

vl_profile:

max_bitrate_requirements:

root: 100000

leaf: 20000

min_bitrate_requirements:

root: 10000

leaf: 10000

SunshineDB (complex): Lower level

This example illustrates two Vdu.Compute nodes (dbBackend, and serviceNode) with two connection points and two virtual links (see figure A.2-3). The flavour_id is "complex".

dbBackend

dbBackendInternalCp

internalVl

serviceNode

serviceNodeInternalCp

serviceNodeCp

dbBackendCp

externalVl

VNF

NOTE:	The single external VL above illustrates that both serviceNodeCp and dbBackendCp are connected to the same external VL. Alternatively, external connection points can be connected to separate external VLs.

Figure A.2-3: SunshineDB (complex): lower level

sunshinedbcomplex.vnfd.tosca.yaml

tosca_definitions_version: tosca_simple_yaml_1_2

description: Relational database, complex

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS

- sunshineVNF.yaml # contains the VNF node type definition

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

properties:

..

flavour_id: complex

requirements:

virtual_link_backend: [ dbBackendCp, virtual_link ]  # IPv4 for SQL

virtual_link_service: [ serviceNodeCp, virtual_link ]  # IPv4 for SSH

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

properties:

flavour_description: A complex flavour

interfaces:

Vnflcm:

instantiate:

implementation: instantiate.workbook.mistral.yaml

terminate:

implementation: terminate.workbook.mistral.yaml

heal:

implementation: heal.workbook.mistral.yaml

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: dbBackend

description: dbBackend compute node

boot_order: []

nfvi_constraints: []

configurable_properties: {}

vdu_profile:

min_number_of_instances: 3

max_number_of_instances: 4

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 8192 MiB

virtual_cpu:

cpu_architecture: x86

num_virtual_cpu: 2

virtual_cpu_clock: 1800 MHz

requirements:

- virtual_storage: mariaDbStorage

serviceNode:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: serviceNode

description: brief description about serviceNode

boot_order: []

nfvi_constraints: []

configurable_properties: {}

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

sw_image_data:

name: Software of Maria Db

version: '1.0'

checksum:

algorithm: sha-256

hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

container_format: bare

disk_format: qcow2

min_disk: 2 GB

min_ram: 8192 MiB

size: 2 GB

operating_system: Linux

supported_virtualisation_environments:

- KVM

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 8192 MiB

virtual_cpu:

cpu_architecture: x86

num_virtual_cpu: 2

virtual_cpu_clock: 1800 MHz

requirements:

- virtual_storage: mariaDbStorage

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: maria.db.image.v1.0.qcow2

mariaDbStorage:

type: tosca.nodes.nfv.Vdu.VirtualBlockStorage

properties:

virtual_block_storage_data:

size_of_storage: 100 GB

rdma_enabled: true

sw_image_data:

name: Software of Maria Db

version: '1.0'

checksum:

algorithm: sha-256

hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

container_format: bare

disk_format: qcow2

min_disk: 2 GB

min_ram: 8192 MiB

size: 2 GB

operating_system: Linux

supported_virtualisation_environments:

- KVM

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: maria.db.image.v1.0.qcow2

dbBackendCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access the DB on IPv4

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: dbBackend

#- virtual_link: # the target node is determined in the NSD

dbBackendInternalCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: dbBackend

- virtual_link: internalVl

serviceNodeCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access the DB on IPv4

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: serviceNode

#- virtual_link: # the target node is determined in the NSD

serviceNodeInternalCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on VL

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: serviceNode

- virtual_link: internalVl

internalVl:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

flow_pattern: mesh

description: Internal VL

vl_profile:

max_bitrate_requirements:

root: 100000

leaf: 20000

min_bitrate_requirements:

root: 10000

leaf: 10000

# A.3	VNF external connection point

## A.3.1	General

A VNF external connection point may either be an internal connection point (a VDU connection point) which is re-exposed externally, i.e. it may be connected to an external virtual link, or a virtual link port. In the latter case the external connection point node has an association relationship of type VirtualLinksTo to the internal virtual link node.

The following clauses illustrate the use of both models of VNF external connection point, re-exposure of an internal connection point and connected to an internal virtual link.

## A.3.2	External connection point re-exposing an internal connection point

In this case there is no need for a VnfExtCp node template. When substituting the VNFD low level service template for the abstract VNF node, the virtual_link requirement of the abstract VNF node is mapped to the VDU connection point's virtual_link requirement. This is shown in figure A.3.2-1. In this case the VNF Service Template does not include a node template of type tosca.nodes.nfv.VnfExtCp, but the functionality of it is provided by a VDU connection point.

Figure A.3.2-1: VNFD with an VDU connection point acting as VnfExtCp

The following snippet shows the relevant part of the service template:

tosca_definitions_version: tosca_simple_yaml_1_2

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS

node_types:

tosca.nodes.nfv.exampleVNF:

derived from: tosca.nodes.nfv.VNF

properties:

# omitted for brevity

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

...

topology_template:

...

substitution_mappings:

node_type: tosca.nodes.nfv.exampleVNF

requirements:

virtual_link: [vduCp_B2, virtual_link]

node_templates:

vduCp_B2:

type: tosca.nodes.nfv.VduCp

# properties omitted for brevity

requirements:

- virtual_binding: VDU-B

- virtual_link: # mapped to virtual_link requirement of VNF node

# other node template definitions (VDU-A, VDU-B, intVL, etc.):

...

## A.3.3	External connection point connected to an internal virtual link

In this case a VnfExtCp node template is needed. When substituting the VNFD low level service template for the abstract VNF node type, the virtual_link requirement of the abstract VNF node maps to the external_virtual_link requirement of the VnfExtCp node. The internal_virtual_link requirement of the VnfExtCp node is fulfilled with the corresponding capability of the internal VirtualLink node. This is shown in figure A.3.3-1.

Figure A.3.3-1: VNFD with a VnfExtCp connected to an internal virtual link

The following snippet shows the corresponding node template definition:

tosca_definitions_version: tosca_simple_yaml_1_2

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS

node_types:

tosca.nodes.nfv.exampleVNF:

derived from: tosca.nodes.nfv.VNF

properties:

# omitted for brevity

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

topology_template:

...

substitution_mappings:

node_type: tosca.nodes.nfv.exampleVNF

requirements:

virtual_link: [myMRFExtCp, external_virtual_link]

node_templates:

myMRFExtCp:

type: tosca.nodes.nfv.VnfExtCp

# properties omitted for brevity

...

requirements:

- external_virtual_link: # mapped to virtual_link requirement of VNF node

- internal_virtual_link: intVL-A

intVL-A:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

flow_pattern: mesh

description: Internal VL

vl_profile:

max_bitrate_requirements:

root: 1000000

leaf: 100000

min_bitrate_requirements:

root: 100000

leaf: 10000

qos:

latency: 100 ms

packet_delay_variation: 80 ms

packet_loss_ratio: 0.00001

capabilities:

virtual_linkable:

# Other node templates e.g. VDU-A, VDU-B, VduCP_A, etc.:

...

# A.4	VNFD modelling design example by using TOSCA composition

The following example in figure A.4-1 shows a VNF descriptor contains three VDUs, which are interconnected by two virtualLinks.

Figure A.4-1: Example of using substitution mapping for the VNFD design

In this example, a separate service template is used to describe the composition of Vdu.Compute, Vdu.VirtualBlockStorage and VduCp, and then substituted as a node template in a VNFD service template. tosca.nodes.nfv.groupVDU_A, tosca.nodes.nfv.groupVDU_B, tosca.nodes.nfv.groupVDU_C types are used for substitution_mapping.

tosca_definitions_version: tosca_simple_yaml_1_2 imports: - etsi_nfv_sol001_vnfd_2_6_1_types.yaml # all of TOSCA VNFD types as defined in NFV SOL 001 GS topology_template: description: Template of a VNFD example node_templates: Group_VDU_A: type: tosca.nodes.nfv.groupVDU_A requirements: - virtual_link: VL_2 Group_VDU_B: type: tosca.nodes.nfv.groupVDU_B requirements: - virtual_link: VL_2 - virtual_link1: VL_1 Group_VDU_C: type: tosca.nodes.nfv.groupVDU_C # the description of this type is described # in another service template. requirements: - virtual_link: VL_1 VL_1: type: tosca.nodes.nfv.VnfVirtualLink properties: # omitted here for brevity capabilities: virtual_linkable: VL_2: type: tosca.nodes.nfv.VnfVirtualLink properties: # omitted here for brevity capabilities: virtual_linkable:

The TOSCA service template example of Group_VDU C is showing in figure A.4-2.

Figure A.4-2: Example of composition Vdu.Compute, Vdu.VirtualBlockStorage and VduCp together using TOSCA substitution mapping

tosca_definitions_version: tosca_simple_yaml_1_2 description: composition of Vdu.Compute, Vdu.VirtualBlockStorage and VduCp. topology_template: substitution_mappings: node_type: tosca.nodes.nfv.groupVDU_C # substituted as a node type requirements: virtual_link: [ internalCpd, virtual_link ] capabilities: virtual_compute: [vduC_compute, virtual_compute] virtual_storage: [vduC_storage, virtual_storage] node_templates: vduC_compute: type: tosca.nodes.nfv.Vdu.Compute properties: # omitted here for brevity capabilities: virtual_compute: artifacts: sw_image: requirements: - virtual_storage: vduC_storage vduC_storage: type: tosca.nodes.nfv.Vdu.VirtualBlockStorage properties: # omitted here for brevity capabilities: virtual_storage: internalCpd: type: tosca.nodes.nfv.VduCp properties: # omitted here for brevity requirements: - virtual_link: - virtual_binding: vduC_compute

# A.5	VNFD with Single deployment flavour modelling design example

In this example, there is one deployment flavour applied to this VNFD, and TOSCA-Simple-Profile-yaml-v1.1 is used for designing and processing this VNFD TOSCA model.

The one service template design illustrated by this example is only applicable when the VNF has only one deployment flavour.

The service template is the main entry point in the VNF Package, i.e. the Entry-definitions file, and is deployed as a stand-alone service template, i.e. without substituting for a node template. However, the service template still contains substitution_mappings to indicate its ability to substitute for a node template of the specific node type.

The service template contains a node template of type MyCompany.SunshineDB.1_0.1_0. The node template contains the properties defined in the node type definition and implementations for the LCM interfaces.

sunshinesimple.vnfd.tosca.yaml

This example illustrates a VNF with one VDU.Compute nodes (dbBackend) with two VDU connection points and one VNF virtual link (see figure A.5-1). The VNF is also connected to an external virtual link. The flavour_id is "simple".

Figure A.5-1: SunshineDB (simple)

tosca_definitions_version: tosca_simple_yaml_1_1

description: Relational database, simple

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS for a VNFD

data_types:

MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters:

derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters

properties:

parameter_1:

type: string

required: true

default: value_1

parameter_2:

type: string

required: false

default: value_2

node_types:

MyCompany.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: SunshineDB ]

default: SunshineDB

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ equal: simple ]

default: simple

flavour_description:

type: string

default: ""

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

interfaces:

Vnflcm:

type: tosca.interfaces.nfv.Vnflcm

instantiate:

inputs:

additional_parameters:

type: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters

#terminate:

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

requirements:

virtual_link: [ dbBackendIpv4, virtual_link ] # IPv4 for SQL

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider: MyCompany

product_name: SunshineDB

software_version: '1.0'

descriptor_version: '1.0'

vnfm_info:

- '0:MyCompany-1.0.0'

flavour_id: simple

flavour_description: 'vnf simple flavour description'

interfaces:

Vnflcm:

instantiate:

implementation: instantiate.workbook.mistral.yaml

terminate:

implementation: terminate.workbook.mistral.yaml

heal:

implementation: heal.workbook.mistral.yaml

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: dbbackend

description: dbBackend

boot_order: []

nfvi_constraints: []

configurable_properties: {}

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 4

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 8192 MiB

virtual_cpu:

cpu_architecture: x86

num_virtual_cpu: 2

virtual_cpu_clock: 1800 MHz

requirements:

- virtual_storage: mariaDbStorage

mariaDbStorage:

type: tosca.nodes.nfv.Vdu.VirtualBlockStorage

properties:

virtual_block_storage_data:

size_of_storage: '200 GB'

rdma_enabled: true

sw_image_data:

name: Software of Maria Db

version: '1.0'

checksum:

algorithm: sha-256

hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

container_format: bare

disk_format: qcow2

min_disk: 2 GB

min_ram: 8192 MiB

size: 2 GB

operating_system: Linux

supported_virtualisation_environments:

- KVM

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: maria.db.image.v1.0.qcow2

dbBackendInternalCp:

type: tosca.nodes.nfv.VduCp

properties:

protocol: [associated_layer_protocol: ipv4 ]

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

requirements:

- virtual_binding: dbBackend

- virtual_link: internalVl

internalVl:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

flow_pattern: mesh

test_access: []

description: Internal Virtual link in the VNF

vl_profile:

max_bitrate_requirements:

root: 100000

leaf: 20000

min_bitrate_requirements:

root: 10000

leaf: 10000

dbBackendIpv4:

type: tosca.nodes.nfv.VduCp

properties:

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access the DB on IPv4

requirements:

#- virtual_link: # the target node is determined in the NSD

- virtual_binding: dbBackend

# A.6	Scaling and Instantiation Level examples

## A.6.1	ScalingAspect and InstantiationLevels policies with uniform delta

This example shows instantiationLevels, and ScalingAspect policies types in the complex scaling scenario where scaling aspect delta is based on "uniform delta" values.

Figure A.6.1-1: Complex scaling example with uniform delta

tosca_definitions_version: tosca_simple_yaml_1_2

description: Complex scaling example (uniform delta value) described with policies

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS

topology_template:

node_templates:

vdu_1:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

boot_order:

- ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 5

nfvi_constraints:

- ..

configurable_properties:

additional_vnfc_configurable_properties: {}

vdu_2:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

boot_order:

- ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 7

nfvi_constraints:

- ..

configurable_properties:

additional_vnfc_configurable_properties: {}

vdu_3:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

boot_order:

- ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 5

nfvi_constraints:

- ..

configurable_properties:

additional_vnfc_configurable_properties: {}

vdu_4:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

boot_order:

- ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 9

nfvi_constraints:

- ..

configurable_properties:

additional_vnfc_configurable_properties: {}

vl_1:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

vl_profile:

min_bitrate_requirements:

root: 1000000

max_bitrate_requirements:

root: 2000000

vl_2:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

vl_profile:

min_bitrate_requirements:

root: 1000000

max_bitrate_requirements:

root: 4000000

policies:

- scaling_aspects:

type: tosca.policies.nfv.ScalingAspects

properties:

aspects:

database:

name: ..

description: ..

max_scale_level: 2

step_deltas:

- delta_1

call_proc:

name: ..

description: ..

max_scale_level: 4

step_deltas:

- delta_1

- vdu_1_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ vdu_1 ]

- vdu_1_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: database

deltas:

delta_1:

number_of_instances: 2

targets: [ vdu_1 ]

- vdu_2_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ vdu_2 ]

- vdu_2_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingDeltas

properties:

aspect: database

deltas:

delta_1:

number_of_instances: 3

targets: [ vdu_2 ]

- vl_1_bitrate_initial_delta:

type: tosca.policies.nfv.VirtualLinkBitrateInitialDelta

properties:

initial_delta:

bitrate_requirements:

root: 1000000

targets: [ vl_1 ]

- vl_1_bitrate_scaling_aspect_deltas:

type: tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas

properties:

aspect: database

deltas:

delta_1:

bitrate_requirements:

root: 1000000

targets: [ vl_1 ]

- vdu_3_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ vdu_3 ]

- vdu_3_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: call_proc

deltas:

delta_1:

number_of_instances: 1

targets: [ vdu_3 ]

- vdu_4_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ vdu_4 ]

- vdu_4_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: call_proc

deltas:

delta_1:

number_of_instances: 2

targets: [ vdu_4 ]

- vl_2_bitrate_initial_delta:

type: tosca.policies.nfv.VirtualLinkBitrateInitialDelta

properties:

initial_delta:

bitrate_requirements:

root: 1000000

targets: [ vl_2 ]

- vl_2_bitrate_scaling_aspect_deltas:

type: tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas

properties:

aspect: call_proc

deltas:

delta_1:

bitrate_requirements:

root: 1000000

targets: [ vl_2 ]

- instantiation_levels:

type: tosca.policies.nfv.InstantiationLevels

properties:

levels:

instantiation_level_1:

description: ..

scale_info:

database:

scale_level: 0

call_proc:

scale_level: 0

instantiation_level_2:

description: ..

scale_info:

database:

scale_level: 2

call_proc:

scale_level: 3

default_level: instantiation_level_1

- vdu_1_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

instantiation_level_2:

number_of_instances: 5

targets: [ vdu_1 ]

- vdu_2_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

instantiation_level_2:

number_of_instances: 7

targets: [ vdu_2 ]

- vl_1_instantiation_levels:

type: tosca.policies.nfv.VirtualLinkInstantiationLevels

..

targets: [ vl_1 ]

- vdu_3_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

instantiation_level_2:

number_of_instances: 4

targets: [ vdu_3 ]

- vdu_4_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

instantiation_level_2:

number_of_instances: 7

targets: [ vdu_4 ]

- vl_2_instantiation_levels:

type: tosca.policies.nfv.VirtualLinkInstantiationLevels

..

targets: [ vl_2 ]

## A.6.2	ScalingAspect and InstantationLevels policies with non-uniform deltas

This example shows instantiationLevel, ScalingAspect policies and group types in the complex scaling scenario where scaling aspect delta is based on "delta" (non-uniform) values for Processing Auxiliary VNFC.

Figure A.6.2-1: Complex scaling example with non-uniform delta

For simplicity, virtual link is not illustrated and instantiation level is one level (default) in this example, as these properties are already illustrated in clause A.6.1.

The "Proc" aspect contains a uniform delta part ("processing" and a non-uniform part ("processing_auxiliary").

The "DB" aspect contains a non-uniform part ("db"), and no delta information for "oam".

tosca_definitions_version: tosca_simple_yaml_1_2

description: Complex example (uniform and non-uniform delta value) described with policies

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in NFV SOL 001 GS

topology_template:

node_templates:

db:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

boot_order:

- ..

vdu_profile:

min_number_of_instances: 2

max_number_of_instances: 6

nfvi_constraints:

- ..

configurable_properties:

additional_vnfc_configurable_properties: {}

oam:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

boot_order:

- ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

nfvi_constraints:

- ..

configurable_properties:

additional_vnfc_configurable_properties: {}

processing:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

boot_order:

- ..

vdu_profile:

min_number_of_instances: 4

max_number_of_instances: 12

nfvi_constraints:

- ..

configurable_properties:

additional_vnfc_configurable_properties: {}

processing_auxiliary:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

boot_order:

- ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 3

nfvi_constraints:

- ..

configurable_properties:

additional_vnfc_configurable_properties: {}

policies:

- scaling_aspects:

type: tosca.policies.nfv.ScalingAspects

properties:

aspects:

database:

name: ..

description: ..

max_scale_level: 2

step_deltas:

- delta_1

- delta_1

proc:

name: ..

description: ..

max_scale_level: 4

step_deltas:

- delta_1

- delta_2

- delta_1

- delta_2

- db_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 2

targets: [ db ]

- db_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: database

deltas:

delta_1:

number_of_instances: 2

targets: [ db ]

- oam_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ oam ]

- processing_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 4

targets: [ processing ]

- processing_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: proc

deltas:

delta_1:

number_of_instances: 2

delta_2:

number_of_instances: 2

targets: [ processing ]

- processing_auxiliary_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ processing_auxiliary ]

- processing_auxiliary_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: proc

deltas:

delta_1:

number_of_instances: 1

delta_2:

number_of_instances: 0

targets: [ processing_auxiliary ]

- instantiation_levels:

type: tosca.policies.nfv.InstantiationLevels

properties:

levels:

instantiation_level_1:

description: ..

scale_info:

database:

scale_level: 0

proc:

scale_level: 0

- db_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 2

targets: [ db ]

- oam_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

targets: [ oam ]

- processing_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 4

targets: [ processing ]

- processing_auxiliary_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

targets: [ processing_auxiliary ]

# A.7	Service Access Point

## A.7.1	General

A SAP may either be a VNF external connection point (or a PNF external connection points or a Sap of a nested NS of this NS) which is exposed as CP of one of the NS constituents (VnfExtCp, PnfExtCp, nested NS Sap externally, or be exposed as a new CP on NS virtual link. This is shown in figure A.7.1-1. In the latter case the Sap node has an association relationship of type VirtualLinksTo to the NsVirtualLink node.

Figure A.7.1-1: Overview of SAP options

The following clauses illustrate the use of both models of SAP, re-exposure of a VNF external connection point and connected to an NS virtual link.

## A.7.2	VNF External connection point exposing as a SAP

In this case, Sap node template is not required. When substituting the NSD service template for the abstract NS node, the virtual_link requirement of the abstract NS node is mapped to the VNF external connection point's virtual_link requirement. This is shown in figure A.7.2-1. In this case the NS Service Template does not include a node template of type tosca.nodes.nfv.Sap, but the functionality of it is provided by a VNF external connection point.

Figure A.7.2-1: VNF_2 External connection point (VnfExtCp) exposed as a Sap

The following snippet shows the relevant part of the NS service template:

tosca_definitions_version: tosca_simple_yaml_1_2

imports:

- etsi_nfv_sol001_nsd__types.yaml  # all of TOSCA NSD types as defined in NFV SOL 001 GS

node_types:

tosca.nodes.nfv.exampleNS

derived from: tosca.nodes.nfv.NS

properties:

# omitted for brevity

requirements:

- virtual_link:

capability:

tosca.capabilitites.nfv.VirtualLinkable

...

topology_template:

...

substitution_mappings:

node_type: tosca.nodes.nfv.exampleNS

requirements:

virtual_link: [VNF_2, virtual_link_2] # the External connection point of VNF_2

# will be used as the Sap of this NS

node_templates:

VNF_2:

type: tosca.nodes.nfv.exampleVNF2

# properties omitted for brevity

requirements:

- virtual_link_1: NsVirtualLink_1 # connects to the External connection

# point which maps to the

# virtual_link_1 requirement of VNF_2

- virtual_link_2: # mapped to virtual_link requirement of NS node

# other node template definitions (VNF_1, NsVirtualLink_1 etc.):

...

## A.7.3	SAP connected to an NS virtual link

In this case, a Sap node template is required. When substituting the NSD service template for the abstract NS node type, the virtual_link requirement of the abstract NS node maps to the virtual_link requirement of the Sap node. This is shown in figure A.7.3-1.

Figure A.7.3-1: SAP_1 exposed as new CP on an internal virtual link

The following snippet shows the corresponding node template definition:

tosca_definitions_version: tosca_simple_yaml_1_2

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in NFV SOL 001 GS

node_types:

tosca.nodes.nfv.exampleNS

derived from: tosca.nodes.nfv.NS

properties:

# omitted for brevity

requirements:

- virtual_link:

capability:

tosca.capabilitites.nfv.VirtualLinkable

topology_template:

...

substitution_mappings:

node_type: tosca.nodes.nfv.exampleNS

requirements:

virtual_link: [SAP_1, external_virtual_link] #SAP_1 is the Sap of the NS

node_templates:

SAP_1:

type: tosca.nodes.nfv.Sap

# properties omitted for brevity

...

requirements:

internal_virtual_link: NsVirtualLink_1

external_virtual_link: # map to virtual_link requirement of the NS node

# Other node templates e.g. VNF_1, VNF_2, NsVirtualLink_1, etc.:

...

# A.8	NSD with Single deployment flavour modelling design example

In this example, there is one deployment flavour applied to this NSD, and TOSCA-Simple-Profile-yaml-v1.1 is used for designing and processing this NSD TOSCA model.

The one service template design illustrated by this example is only applicable when the NS has only one deployment flavour.

The service template is the main entry point in the NSD file structure, i.e. the Entry-definitions file, and is deployed as a stand-alone service template, i.e. without substituting for a node template. However, the service template still contains substitution_mappings to indicate its ability to substitute for a node template of the specific node type.

example_NS.yaml

This example illustrates a NS contains two VNF: VNF_1 and VNF_2, they connect through NsVirtualLink_1. One of the VnfExtCp of VNF_2 is exposed as the Sap of this NS. The flavour_id is "simple". In the VNF_2 node template, it defines a dependency requirement to VNF_1, which indicates that all the instances of VNF_1 have to be deployed first before deploy the instances of VNF_2.

VNF_1

NsVirtualLink_1

Ext_NsVirtualLink

NS

VNF_2

Figure A.8-1: example_NS (simple)

tosca_definitions_version: tosca_simple_yaml_1_1

description: Relational database, simple

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of NSD related TOSCA types as defined in NFV SOL 001 GS

node_types:

tosca.example_NS:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

name:

type: string

constraints: [ equal: ExampleService ]

default: ExampleService

version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

invariant_id:

type: string

constraints: [ equal: 1111-2222-aaaa-bbbb ]

default: 1111-2222-aaaa-bbbb

flavour_id:

type: string

constraints: [ equal: simple ]

default: simple

topology_template:

substitution_mappings:

node_type: tosca.example_NS

requirements:

virtual_link: [ VNF_2, virtual_link_2 ] # the External connection point of

# VNF_2 is exposed as the Sap

node_templates:

my_service:

type: tosca.example_NS

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer: MyCompany

name: ExampleService

version: '1.0'

invariant_id: 1111-2222-aaaa-bbbb

flavour_id: simple

interfaces:

Nslcm:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: NsVirtualLink_1

VNF_2:

type: tosca.nodes.nfv.example_VNF2

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: NsVirtualLink_1

- virtual_link_2: # map to virtual_link requirement of the NS node

- dependency: VNF_1

NsVirtualLink_1:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

# A.9	Mapping between NFV IM and TOSCA concepts

## A.9.1	Introduction

This annex describes the mapping between the NFV information model and the TOSCA concepts.

## A.9.2	Mapping between ETSI GS NFV-IFA 011 IM and TOSCA concepts

Table A.9.2-1 illustrates the mapping between the information model as specified in ETSI GS NFV-IFA 011 [1] and the corresponding TOSCA concepts.

Table A.9.2-1: Mapping between ETSI GS NFV-IFA 011 [1] IM and TOSCA concepts

| ETSI GS NFV-IFA 011 [1] information element | TOSCA concept |
| --- | --- |
| VNFD (clause 7.1.2) | TOSCA service template(s) in the VNF package |
| VnfExtCpd (clause 7.1.4) | node template with type tosca.nodes.nfv.VnfExtCp or tosca.nodes.nfv.VduCp |
| VnfLcmOperationsConfiguration (clause 7.1.5) | property of VNF node type with data type tosca.datatypes.nfv.VnfConfigurableProperties |
| Vdu (clause 7.1.6) | node template with type tosca.nodes.nfv.Vdu.Compute |
| VLd (clause 7.1.7) | node template with type tosca.nodes.nfv.VnfVirtualLink |
| DeploymentFlavour (clause 7.1.8) | lower level service template(s) in the VNF package |
| VduProfile (clause 7.1.8.3) | property of Vdu.Compute node type with data type tosca.datatypes.nfv.VduProfile |
| VirtualLinkProfile (clause 7.1.8.4) | property of VnfVirtualLink node type with data type tosca.datatypes.nfv.VlProfile |
| VirtualLinkDescFlavour (clause 7.1.8.5) | Only qos attribute in VirtualLinkDescFlavour has been defined as property of the VlProfile data type |
| InstantiationLevel (clause 7.1.8.7) | policy with type tosca.policies.nfv.InstantiationLevels |
| VduLevel (clause 7.1.8.9) | policy with type tosca.policies.nfv.VduInstantiationLevels |
| LocalAffinityOrAntiAffinityRule (clause 7.1.8.11) | policy with type tosca.policies.nfv.AffinityRule or tosca.policies.nfv.AntiAffinityRule |
| AffinityOrAntiAffinityGroup (clause 7.1.8.12) | policy with type tosca.policies.nfv.AffinityRule or tosca.policies.nfv.AntiAffinityRule |
| VirtualComputeDesc (clause 7.1.9.2.2) | VirtualCompute capability of the Vdu.Compute node template |
| VirtualStorageDesc with BlockStorageData (clause 7.1.9.4.3) | node template with type tosca.nodes.nfv.Vdu.VirtualBlockStorage |
| VirtualStorageDesc with ObjectStorageData (clause 7.1.9.4.4) | node template with type tosca.nodes.nfv.Vdu.VirtualObjectStorage |
| VirtualStorageDesc with FileStorageData (clause 7.1.9.4.5) | node template with type tosca.nodes.nfv.Vdu.VirtualFileStorage |
| ScalingAspect (clause 7.1.10.2) | policy with type tosca.policies.nfv.ScalingAspects |
| ScalingDelta (clause 7.1.10.4) | policy with type tosca.policies.nfv.VduScalingAspectDeltas and/or tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas |
| VnfIndicator (clause 7.1.11.2) | Not supported in this version of the present document |
| MonitoringParameter (clause 7.1.11.3) | property of the VNF, Vdu.Compute and VnfVirtualLink node types |
| VnfConfigurableProperties (clause 7.1.12) | property of VNF node template with data type tosca.datatypes.nfv.VnfConfigurableProperties |
| LifeCycleManagementScript (clause 7.1.13) | interface with type tosca.interfaces.nfv.Vnflcm |
| VnfInfoModifiableAttributes (clause 7.1.14) | property of VNF node template with data type tosca.datatypes.nfv.VnfInfoModifiableAttributes |

## A.9.3	Mapping between ETSI GS NFV-IFA 014 IM and TOSCA concepts

Table A.9.3-1 illustrates the mapping between the information model as specified in ETSI GS NFV-IFA 014 [2] and the corresponding TOSCA concepts.

Table A.9.3-1: Mapping between ETSI GS NFV-IFA 014 [2] IM and TOSCA concepts

| ETSI GS NFV-IFA 014 [2] information element | TOSCA concept |
| --- | --- |
| NSD (clause 6.2.2) | TOSCA service template(s) in the NSD file structure |
| Sapd (clause 6.2.3) | node template with type tosca.nodes.nfv.Sap or the virtual_link requirement in the substitution_mapping |
| MonitoredData (clause 6.2.6) | Not supported in this version of the present document |
| VnfIndicatorData (clause 6.2.7) | Not supported in this version of the present document |
| LifeCycleManagementScript (clause 6.2.9) | Interface with type Nslcm |
| NsDf (clause 6.3) | lower level service template(s) in the NSD file structure |
| VnfProfile (clause 6.3.3) | property of VNF node template with data type tosca.datatypes.nfv.VnfProfile |
| VirtualLinkProfile (clause 6.3.4) | property of NsVirtualLink node template with data type tosca.datatypes.nfv.VlProfile |
| AffinityOrAntiAffinityGroup (clause 6.3.5) | policy with type tosca.policies.nfv.AffinityRule or tosca.policies.nfv.AntiAffinityRule |
| NsVirtualLinkConnectivity (clause 6.3.7) | requirement and capability between PnfExtCp or VnfExtCp and NsVirtualLink node templates |
| NsLevel (clause 6.3.9) | Not supported in this version of the present document |
| NsScaleInfo (clause 6.3.10) | Not supported in this version of the present document |
| NsProfile (clause 6.3.11) | property of NS node template with data type tosca.datatypes.nfv.NsProfile |
| Dependencies (clause 6.3.12) | dependency requirements |
| Vnffgd (clause 6.4.2) | Not supported in this version of the present document |
| Nfpd (clause 6.4.3) | Not supported in this version of the present document |
| CpdPool (clause 6.4.4) | Not supported in this version of the present document |
| NfpPositionDesc(clause 6.4.5) | Not supported in this version of the present document |
| CpProfile (clause 6.4.6) | Not supported in this version of the present document |
| NsVirtualLinkDesc (clause 6.5.2) | node template with type tosca.nodes.nfv.NsVirtualLink |
| VirtualLinkDf (clause 6.5.4) | Only the serviceAvaibilityLevel attribute in VirtualLinkDf has been defined as property of the NsVirtualLink node type |
| Pnfd (clause 6.6.2) | A standalone TOSCA service template |
| PnfExtCpd (clause 6.6.4) | node template with type tosca.nodes.nfv.PnfExtCp |
| NsScalingAspect (clause 6.7.2) | Not supported in this version of the present document |
| VnfToLevelMapping (clause 6.7.4) | Not supported in this version of the present document |
| VirtualLinkToLevelMapping (clause 6.7.5) | Not supported in this version of the present document |
| NsToLevelMapping (clause 6.7.6) | Not supported in this version of the present document |

# A.10	PNFD modelling design example

In this example, TOSCA-Simple-Profile-yaml-v1.1 is used for designing and processing a PNFD TOSCA model.

The service template contains a node template of type MyCompany.examplePnf.1_0 which represents the main part of the PNF and a node template of type tosca.nodes.nfv.PnfExtCp representing the PNF external connection point.

examplePnf.yaml

This example illustrates a PNF with one external connection point, pnfExtCp_1. All the rest parts of the PNF is described as a single box, e.g. called pnf_mainPart in the PNFD.

Figure A.10-1: examplePnf

tosca_definitions_version: tosca_simple_yaml_1_1

description: the service template of a PNFD

imports:

- etsi_nfv_sol001_pnfd_types.yaml

node_types:

MyCompany.examplePnf.1_0:

derived_from: tosca.nodes.nfv.PNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a2233 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a2233

function_description:

type: string

default: an example PNF

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_invariant_id:

type: string

constraints: [ equal: 1111-2222-ccaa-bbdd ]

default: 1111-2222-ccaa-bbdd

name:

type: string

constraints: [ equal: ExamplePnf ]

default: ExamplePnf

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

topology_template:

substitution_mappings:

node_type: MyCompany.examplePnf.1_0

requirements:

virtual_link: [ pnfExtCp_1, external_virtual_link ]

node_templates:

pnf_mainPart:

type: MyCompany.examplePnf.1_0

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a2233

function_description: an example PNF

provider: MyCompany

version: 1.0

descriptor_invariant_id: 1111-2222-ccaa-bbdd

name: ExamplePnf

pnfExtCp_1:

type: tosca.nodes.nfv.PnfExtCp

properties:

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access this pnf

requirements:

# - external_virtual_link:

- dependency: pnf_mainPart

# A.11	NSD with Multiple deployment flavour modelling design example

Deployment flavours are represented as deployable TOSCA topology templates. This way one NS service template represents one NS deployment flavour, and different deployment flavours are described by different NS service templates. This is in line with the concept that different deployment flavours can define different topologies of the same NS, with different scaling aspects, different constituent VNFs, PNFs and nested NSs, and different internal connectivity.

In order to represent a NS, a top-level service template is used. This top-level service template contains a topology template with only an abstract NS node which defines the common parts of the different deployment flavours (such as designer, version and parts of the lifecycle management interface definition). It also sets a constraint on the deployment flavour property (the required value of the flavour_id property); this constraint comes from the NS instantiation request which contains a flavour_id selected among those available in the NSD.

As a result, the NFVO will look into the available further service templates representing the different NS deployment flavours of the NS and use the one that has the matching flavour_id property value to substitute for the abstract NS. These are the low-level service templates.

Figure A.11-1: NSD overview with multiple deployment flavour

A NSD contains a TOSCA top-level Service Template as entry point in the NSD file structure and one or more TOSCA low-level Service templates representing the different deployment flavours (see figure A.11-1). The NSD is interpreted by an NFVO. In this example, the templates describe two variants of the NS each corresponding to a deployment flavour: a small and a big one. The small NS consists of two VNFs one NS Virtual link and, the big VNF variant consists of three VNFs and one NS Virtual link.

NSD-top levelMyExampleNs.yaml

tosca_definitions_version: tosca_simple_yaml_1_2

description: my service

imports:

- etsi_nfv_sol001_nsd_types.yaml

- MyExampleNs_Type.yaml # contains the NS node type definition

- MyExampleNs_small.yaml

- MyExampleNs_big.yaml

topology_template:

inputs:

nsFlavourId:

type: string

description: NS deployment flavour selected by the consumer. It is provided in the SOL005 API

node_templates:

myexampleNs:

type: tosca.MyExampleNS

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer: MyCompany

name: ExampleService

version: '1.0'

invariant_id: 1111-2222-aaaa-bbbb

flavour_id: {get_input: nsFlavourId }

# get_input function would be used by a TOSCA orchestrator at run time to access the nsFlavourId. If the deployment is not done by a TOSCA orchestrator above function may not be needed

requirements:

#- virtual_link # mapped in lower-level templates

The MyExampleNs_Type.yaml file has the following content:

tosca_definitions_version: tosca_simple_yaml_1_2

description: type definition of tosca.MyExampleNS

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in NFV SOL 001 GS

node_types:

tosca.MyExampleNS:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ valid_values: [ b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ] ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer:

type: string

constraints: [ valid_values: [ MyCompany] ]

default: MyCompany

name:

type: string

constraints: [ valid_values: [ ExampleService ] ]

default: ExampleService

version:

type: string

constraints: [ valid_values: [ '1.0' ] ]

default: '1.0'

invariant_id:

type: string

constraints: [ valid_values: [ 1111-2222-aaaa-bbbb ] ]

default: 1111-2222-aaaa-bbbb

flavour_id:

type: string

constraints: [ valid_values: [ small, big ] ]

default: small

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

interfaces:

Nslcm:

type: tosca.interfaces.nfv.Nslcm

The NS node template in the myexample_NS.yaml file is abstract and is subject to substitution; the lower-level templates in the subsequent sections provide these substitutions. The actual lower-level template is selected based on the node type and a value constraint on the flavour_id property.

Each low level service template contains a node template of type tosca.MyExampleNS with implementation of the LCM interfaces.

MyExampleNs (small): Lower level, contains 2 VNFs and 1 NS virtual link.

Figure A.11-2: MyExampleNs (simple): Lower level

MyExampleNs_small.yaml

tosca_definitions_version: tosca_simple_yaml_1_2

description: myExampleNs with small flavour

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in NFV SOL 001 GS

- MyExampleNs_Type.yaml # contains the NS node type definition

- example_VNF1_Type.yaml # contains the tosca.nodes.nfv.example_VNF1 node type definition

- example_VNF2_Type.yaml # contains the tosca.nodes.nfv.example_VNF2 node type definition

topology_template:

substitution_mappings:

node_type: tosca.MyExampleNS

properties:

..

flavour_id: small

requirements:

virtual_link: [ VNF_1, virtual_link_2 ]

node_templates:

MyExampleNS:

type: tosca.MyExampleNS

properties:

#

interfaces:

Nslcm:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: NsVirtualLink_1

- virtual_link_2: # map to virtual_link requirement of the NS node

VNF_2:

type: tosca.nodes.nfv.example_VNF2

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: NsVirtualLink_1

Ns_VL:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

MyExampleNs (big): Lower level, contains 3 VNFs and 1 NS virtual link.

Figure A.11-3: MyExampleNs (big): Lower level

MyExampleNs_big.yaml

tosca_definitions_version: tosca_simple_yaml_1_2

description: myExampleNs with big flavour

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in NFV SOL 001 GS

- MyExampleNs_Type.yaml # contains the NS node type definition

- example_VNF1_Type.yaml # contains the tosca.nodes.nfv.example_VNF1 node type definition

- example_VNF2_Type.yaml # contains the tosca.nodes.nfv.example_VNF2 node type definition

- example_VNF3_Type.yaml # contains the tosca.nodes.nfv.example_VNF3 node type definition

topology_template:

substitution_mappings:

node_type: tosca.MyExampleNS

properties:

..

flavour_id: small

requirements:

virtual_link: [ VNF_1, virtual_link_2 ]

node_templates:

MyExampleNS:

type: tosca.MyExampleNS

properties:

#

interfaces:

Nslcm:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

scale:

implementation: scale.workbook.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: NsVirtualLink_1

- virtual_link_2: # map to virtual_link requirement of the NS node

VNF_2:

type: tosca.nodes.nfv.example_VNF2

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: NsVirtualLink_1

VNF_3:

type: tosca.nodes.nfv.example_VNF3

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: NsVirtualLink_1

Ns_VL:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

# A.12	NSD TOSCA service template design

A TOSCA service template representing a NSD may contain a node template of some specific NS node type as one of its constituents. The latter is a nested NS. When the containing NS is deployed, the node template of the nested NS is substituted by the topology template representing the nested NS.

Figure A.12-1 illustrates a network service NS_1 that consists of one VNF (VNF_1), one NsVirtualLink (NS_VL_1_and one nested NS (NS_2).

The nested NS consists of two VNFs (VNF_3 and VNF_4) and one NsVirtualLink (NS_VL_2).

Figure A.12-1: Example of a network service containing a nested network service

The following snipet shows the topology template representing a particular flavour of NS_1.

tosca_definitions_version: tosca_simple_yaml_1_2

description: myExampleNs with small flavour

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in NFV SOL 001 GS

- MyExampleNs_Type.yaml # contains the NS node type definition

- MyExampleNS_2.yaml

topology_template:

substitution_mappings:

node_type: tosca.MyExampleNS

properties:

..

flavour_id: small

requirements:

- virtual_link: [ VNF_1, virtual_link_2 ]

node_templates:

NS_1:

type: tosca.MyExampleNS

interfaces:

Nslcm:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link_1: NS_VL_1

- virtual_link_2: # map to virtual_link requirement of the NS node

NS_2:

type: tosca.myExample.NS_2

properties:

descriptor_id: c1bb0ab8-deab-4fa7-95ed-4840d70a3574

designer: MyCompany

version: 1.0.0.0

name: myExample2Service

invariant_id: aaaa-bbbb-cccc-dddd

ns_profile:

ns_instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

flavour_id: simple

NS_VL_1:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

The contents of MyExampleNs_Type.yaml file with the node type definition are as follows:

tosca_definitions_version: tosca_simple_yaml_1_2

description: type definition of tosca.MyExampleNS

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA types as defined in NFV SOL 001 GS

node_types:

tosca.MyExampleNS:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ valid_values: [ b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ] ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer:

type: string

constraints: [ valid_values: [ MyCompany] ]

default: MyCompany

name:

type: string

constraints: [ valid_values: [ ExampleService ] ]

default: ExampleService

version:

type: string

constraints: [ valid_values: [ '1.0' ] ]

default: '1.0'

invariant_id:

type: string

constraints: [ valid_values: [ 1111-2222-aaaa-bbbb ] ]

default: 1111-2222-aaaa-bbbb

flavour_id:

type: string

constraints: [ valid_values: [ small, big ] ]

default: small

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

interfaces:

Nslcm:

type: tosca.interfaces.nfv.Nslcm

The following snippet shows the service template representing the NSD NS_2. In this example, NS_2 supports one single deployment flavour.

MyExampleNS_2.yaml:

tosca_definitions_version: tosca_simple_yaml_1_1

description: Relational database, simple

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of NSD related TOSCA types as defined in NFV SOL 001 GS

node_types:

tosca.myExample.NS_2:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ valid_values: [ c1bb0ab8-deab-4fa7-95ed-4840d70a3574 ] ]

default: c1bb0ab8-deab-4fa7-95ed-4840d70a3574

designer:

type: string

constraints: [ valid_values: [ MyCompany] ]

default: MyCompany

name:

type: string

constraints: [ valid_values: [ myExample2Service ] ]

default: myExample2Service

version:

type: string

constraints: [ valid_values: [ '1.0.0.0' ] ]

default: '1.0.0.0'

invariant_id:

type: string

constraints: [ valid_values: [ aaaa-bbbb-cccc-dddd ] ]

default: aaaa-bbbb-cccc-dddd

flavour_id:

type: string

constraints: [ valid_values: [ simple ] ]

default: simple

topology_template:

substitution_mappings:

node_type: tosca.myExample.NS_2

requirements:

- virtual_link: [ VNF_4, virtual_link_2 ] # the External connection point of

# VNF_2 is exposed as the Sap

node_templates:

NS_2:

type: tosca.myExample.NS_2

interfaces:

Nslcm:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_3:

type: tosca.nodes.nfv.example_VNF3

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: NS_VL_2

VNF_4:

type: tosca.nodes.nfv.example_VNF4

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: NS_VL_2

- virtual_link_2: # map to virtual_link requirement of the NS node

NS_VL_2:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

Annex B (normative):
etsi_nfv_sol001_type definitions

# B.1	Purpose

All type definitions specified in clauses 6, 7 and 8 of the present document are gathered in three definition files.

The file names are structured as follows:

etsi_nfv_sol001_common_types.yaml, for the common type definitions which are used by at least two types of deployment templates among those identified in clause 5.1, the content of this yaml file is described in B.5;

etsi_nfv_sol001_vnfd_types.yaml, for the definitions provided in clause 6 and only used in a VNFD service template design;

etsi_nfv_sol001_nsd_types.yaml, for the definitions provided in clause 7 and only used in a NSD service template design;

etsi_nfv_sol001_pnfd _types.yaml, for the definitions provided in clause 8 and only used in a PNFD service template design.

# B.2	VNFD type definitions file

The file etsi_nfv_sol001_vnfd_types.yaml, attached to the present document, contains all type definitions specified in clause 6 of the present document. This file is also available at the following URL:

https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_vnfd_types.yaml

NOTE 1:	The file etsi_nfv_sol001_vnfd_types.yaml includes a TOSCA import definition referencing etsi_nfv_sol001_common_types.yaml file. If the later file is included in the VNF package, the import definition can reference the local file using appropriate path in the VNF package.

This file is a TOSCA service template that only contains definitions. The template_version in the metadata section within this template is structured as x.y.z, where x, y and z represent the version of this file and are set respectively to "2", "6" and "1" for this version of the present document. In subsequent versions of the present document, "x", "y" and "z" in the template_version will be incremented only if there are changes in the VNFD type definitions.

A TOSCA service template representing a VNFD complying with the present document shall contain import statement referencing this file, as defined in clause 5.6.1.

NOTE 2:	This file may, but need not, be included in the VNF Package.

# B.3	NSD type definitions file

The file etsi_nfv_sol001_nsd_types.yaml, attached to the present document, contains all type definitions specified in clause 7 of the present document. This file is also available at the following URL:

https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_nsd_types.yaml

NOTE 1:	The file etsi_nfv_sol001_nsd_types.yaml includes a TOSCA import definition referencing etsi_nfv_sol001_common_types.yaml file. If the later file is included in the NSD file archive, the import definition can reference the local file using appropriate path in the NSD file archive.

This file is a TOSCA service template that only contains definitions. The template_version in the metadata section within this template is structured as x.y.z, where x, y and z represent the version of this file and are set respectively to "2", "6" and "1" for this version of the present document. In subsequent versions of the present document, "x", "y" and "z" in the template_version will be incremented only if there are changes in the NSD type definitions.

A TOSCA service template representing a NSD complying with the present document shall contain import statement referencing this file, as defined in clause 5.6.2.

NOTE 2:	This file may, but need not, be included in the NSD file archive.

# B.4	PNFD type definitions file

The file etsi_nfv_sol001_pnfd_types.yaml, attached to the present document, contains all type definitions specified in clause 8 of the present document. This file is also available at the following URL:

https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_pnfd_types.yaml

NOTE:	The file etsi_nfv_sol001_pnfd_types.yaml includes a TOSCA import definition referencing etsi_nfv_sol001_common_types.yaml file.

This file is a TOSCA service template that only contains definitions. The template_version in the metadata section  within this template is structured as x.y.z, where x, y and z represent the version of this file and are set respectively to "2", "6" and "1" for this version of the present document. In subsequent versions of the present document, "x", "y" and "z" in the template_version will be incremented only if there are changes in the PNFD type definitions.

A TOSCA service template representing a PNFD complying with the present document shall contain import statement referencing this file, as defined in clause 5.6.3.

B.5	Common type definitions file

The file etsi_nfv_sol001_common_types.yaml, attached to the present document, contains the type definitions as specified in clause 6 and used by at least two types of deployment templates. This file is also available at the following URL:

https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_common_types.yaml

NOTE 1:	This file includes the following type definitions:

tosca.datatypes.nfv.L2AddressData

tosca.datatypes.nfv.L3AddressData

tosca.datatypes.nfv.AddressData

tosca.datatypes.nfv.ConnectivityType

tosca.datatypes.nfv.LinkBitrateRequirements

tosca.datatypes.nfv.CpProtocolData

tosca.datatypes.nfv.VnfProfile

tosca.datatypes.nfv.Qos

tosca.capabilities.nfv.VirtualLinkable

tosca.relationships.nfv.VirtualLinksTo

tosca.nodes.nfv.Cp

This file is a TOSCA service template that only contains definitions. The template_version in the metadata section within this template is structured as x.y.z, where x, y and z represent the version of this file and are set respectively to "2", "6" and "1" for this version of the present document. In subsequent versions of the present document, "x", "y" and "z" in the template_version will be incremented only if there are changes in the common type definitions.

NOTE 2:	This file may, but need not, be included in the VNF package or NSD file archive.

Annex C (normative):
Conformance

# C.1	Purpose

The present document specifies a data model for the VNFD, the NSD and the PNFD, by using the grammar defined in the TOSCA Simple Profile in YAML specification [3]. This annex specifies the requirements to be fulfilled for claiming conformance to the present document.

# C.2	NFV TOSCA YAML service template

A VNFD, an NSD or a PNFD conforms to the present document if it complies will all the requirements below:

1)	A VNFD conformant to the present document shall comply with the requirements in clause 6 of the present document and to the specification of the elements of the TOSCA Simple Profile in YAML specification [3] it uses, unless otherwise stated in clause 6 of the present document.

2)	A NSD conformant to the present document shall comply with the requirements in clause 7 of the present document and to the specification of the elements of the TOSCA Simple Profile in YAML specification [3] it uses, unless otherwise stated in clause 7 of the present document.

3)	A PNFD conformant to the present document shall comply with the requirements in clause 8 of the present document and to the specification of the elements of the TOSCA Simple Profile in YAML specification [3] it uses, unless otherwise stated in clause 8 of the present document.

4)	When using or referring to the TOSCA normative types listed in table C.2-1, it is valid according to the definitions given in clauses 6 and 7 of the present document and to section 5 of the TOSCA Simple Profile in YAML specification [3].

Table C.2-1: TOSCA normative types used in the present document

| Types | VNFD | NSD |
| --- | --- | --- |
| tosca.datatypes.Root | X | X |
| tosca.artifacts.File | X |  |
| tosca.artifacts.Deployment.Image | X |  |
| tosca.artifacts.Implementation | X |  |
| tosca.capabilities.Root | X |  |
| tosca.relationships.Root | X |  |
| tosca.relationships.DependsOn | X | X |
| tosca.interfaces.Root | X | X |
| tosca.nodes.Root | X | X |
| tosca.groups.Root | X | X |
| tosca.policies.Root | X | X |
| tosca.policies.Placement | X | X |

5)	A VNFD conformant to the present document shall comply with VNFD TOSCA service template design specified in clause 6.11 of the present document.

6)	A VNFD and NSD conformant to the present document shall comply with rules for Type extension defined in clause 5.7 of the present document.

# C.3	NFV TOSCA processor

A processor or program conforms to the present document as NFV TOSCA processor for VNFD, NFV TOSCA processor for NSD or NFV TOSCA processor for PNFD if it complies will all the requirements below:

1)	It can parse and recognize the elements of any VNFD, NSD or PNFD that conform to the present document, and shall generate errors for those documents that fail to conform to the present document.

2)	It shall comply with all requirements and implement the semantics associated with the definitions specified in clauses 6, 7 and 8 of the present document.

3)	It shall resolve the import definitions, either explicit or implicit, as described in clause 5.5 of the present document.

Annex D (informative):
Mapping between properties of TOSCA types and API attributes

# D.1	Introduction

This annex provides the mapping between properties of TOSCA types defined in the present document and defined in the following API specifications: ETSI GS NFV-SOL 002 [i.4], ETSI GS NFV-SOL 003 [i.9], and ETSI GS NFV-SOL 005 [i.10].

NOTE:	See also annex A "Mapping operations to protocol elements" of ETSI GS NFV-SOL 002 [i.4], ETSI GS NFV-SOL 003 [i.9] and ETSI GS NFV-SOL 005 [i.10] for each operation.

# D.2	VNFD-related constructs

Table D.2-1 provides the mapping between the properties of TOSCA types related to the VNFD and API attributes, which include: resource or notification data type (and referenced structured data type when available), attribute name and type in the resource or notification data type, and the interface operation in which the data type is used.

NOTE:	In the "Data model" column of the table, an arrow "->" indicates the navigation through the resource, notification and referenced structured data types.

Table D.2-1: Mapping of API attributes and TOSCA constructs

| ETSI GS NFV-SOL 001 | SOL APIs |  |  |
| --- | --- | --- | --- |
| Type and Property or entity name | Type and attribute name | Data model | Operation (see note) |
| tosca.nodes.nfv.VNF -> descriptor_id | (Identifier) vnfdId | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfInfoModificationRequest VnfInfoModifications | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query VNF Package Info (see SOL005) |
|  |  | VnfPackageOnboardingNotification | Notify about VNF Package (see SOL003 and SOL005) |
|  |  | VnfPackageChangeNotification | Notify about VNF Package (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | UpdateNsRequest -> InstantiateVnfData | Update NS (see SOL005) |
|  |  | NsLcmOperationOccurrenceNotification -> AffectedVnf | Notify about NS LCM (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> provider | (String) vnfProvider | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query VNF Package Info (see SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter -> vnfProductsFromProviders | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter -> vnfProductsFromProviders | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> product_name | (String) vnfProductName | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query VNF Package Info (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> software_version | (Version) vnfSoftwareVersion | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query VNF Package Info (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts -> versions | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts -> versions | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> descriptor_version | (Version) vnfdVersion | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfInfoModifications | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query VNF Package Info (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts -> versions | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts -> versions | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> configurable_properties | (KeyValuePairs) vnfConfigurableProperties | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | VnfInfoModificationRequest | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfInfoModifications |  |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
| tosca.nodes.nfv.VNF ->flavour_id | (IdentifierInVnfd) flavourId | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | InstantiateVnfRequest | Instantiate VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) vnfFlavourId | UpdateNsRequest -> instantiateVnfData | Update NS (see SOL005) |
|  | (IdentifierInVnfd) newFlavourId | ChangeVnfFlavourRequest | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | UpdateNsRequest -> ChangeVnfFlavourData | Update NS (see SOL005) |
| tosca.datatypes.nfv.ScalingAspect ->name | (IdentifierInVnfd) aspectId | VnfInstance -> instantiatedVnfInfo -> ScaleInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | ScaleVnfRequest | Scale VNF (see SOL002 and SOL003) |
|  |  | ScaleVnfToLevelRequest -> ScaleInfo | Scale VNF to level (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> ScaleInfo | Query NS (see SOL005) |
| tosca.datatypes.nfv.ScaleInfo ->scale_level | (Integer) scaleLevel | VnfInstance -> instantiatedVnfInfo -> ScaleInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | ScaleVnfToLevelRequest -> ScaleInfo | Scale VNF to level (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> ScaleInfo | Query NS (see SOL005) |
| Node template name of type tosca.nodes.nfv.VnfExtCp | (IdentifierInVnfd) cpdId | VnfInstance -> instantiatedVnfInfo -> VnfExtCpInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | ChangeVnfFlavourRequest -> ExtVirtualLinkData -> VnfExtCpData | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | ChangeExtVnfConnectivityRequest -> ExtVirtualLinkData -> VnfExtCpData | Change External VNF Connectivity (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> VnfExtCpInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) resourceTemplateId | GrantRequest -> ResourceDefinition | Grant Lifecycle Operation (see SOL003) |
| Node template name of type tosca.nodes.nfv.VnfVirtualLink | (IdentifierInVnfd) vnfVirtualLinkDescId | VnfInstance -> instantiatedVnfInfo -> ExtManagedVirtualLinkInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | InstantiateVnfRequest -> ExtManagedVirtualLinkData | Instantiate VNF (see SOL002 and SOL003) |
|  |  | ChangeVnfFlavourRequest -> ExtManagedVirtualLinkData | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> resourceChanges -> AffectedVirtualLink | Get Operation Status (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> ExtManagedVirtualLinkInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) vnfVirtualLinkDescId | UpdateNsRequest -> InstantiateVnfData -> ExtManagedVirtualLinkData | Update NS (see SOL005) |
| TBD tosca.datatypes.nfv.VnfMonitoringParameter | (IdentifierInVnfd) id | VnfInstance -> instantiatedVnfInfo -> MonitoringParameter | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> MonitoringParameter | Query NS (see SOL005) |
| tosca.datatypes.nfv.VnfMonitoringParameter ->name | (String) name | VnfInstance -> instantiatedVnfInfo -> MonitoringParameter | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> MonitoringParameter | Query NS (see SOL005) |
| tosca.nodes.nfv.VNF ->localization_languages | (String) localizationLanguage | VnfInstance -> instantiatedVnfInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo | Query NS (see SOL005) |
| Node template name of type tosca.nodes.nfv.Vdu.Compute | (IdentifierInVnfd)vduId | VnfInstance -> instantiatedVnfInfo -> VnfcResourceInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfInstance -> instantiatedVnfInfo -> VnfcInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> resourceChanges -> AffectedVnfc | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> AffectedVnfc | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> VnfcResourceInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) vnfdVirtualComputeDescId | Grant -> vimAssets -> VimConputeResourceFlavour | Grant Lifecycle Operation (see SOL003) |
| Node template name of type tosca.nodes.nfv.VduCp | (IdentifierInVnfd) cpdId | VnfInstance -> instantiatedVnfInfo -> VnfcResourceInfo -> vnfcCpInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
| Node template name of type tosca.nodes.nfv.Vdu.VirtualBlockStorage Node template name of type tosca.nodes.nfv.Vdu.VirtualObjectStorage Node template name of type tosca.nodes.nfv.Vdu.VirtualFileStorage | (IdentifierInVnfd) virtualStorageDescId | VnfInstance -> instantiatedVnfInfo -> VirtualStorageResourceInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> resourceChanges -> AffectedVirtualStorage | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> AffectedVirtualStorage | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> VirtualStorageResourceInfo | Query NS (see SOL005) |
| tosca.datatypes.nfv.VnfInfoModifiableAttributes ->metadata | (KeyValuePairs) metadata | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | CreateVnfRequest | Create VNF Identifier (see SOL002 and SOL003) |
|  |  | VnfInfoModificationRequest | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfInfoModifications |  |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
| tosca.datatypes.nfv.VnfInfoModifiableAttributes ->extensions | (KeyValuePairs) extensions | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | InstantiateVnfRequest | Instantiate VNF (see SOL002 and SOL003) |
|  |  | VnfInfoModificationRequest | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfInfoModifications |  |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
| tosca.policies.nfv.InstantiationLevel ->levels[key] | (IdentifierInVnfd) instantiationLevelId | InstantiateVnfRequest | Instantiate VNF (see SOL002 and SOL003) |
|  |  | ScaleVnfToLevelRequest | Scale VNF to level (see SOL002 and SOL003) |
|  |  | ChangeVnfFlavourRequest | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | UpdateNsRequest -> changeVnfFlavourData | Update NS (see SOL005) |
|  | (IdentifierInVnfd) vnfInstantiationLevelId | UpdateNsRequest -> InstantiateVnfData | Update NS (see SOL005) |
|  |  | ScaleNsRequest -> ScaleVnfData -> ScaleToLevelData | Scale NS (see SOL005) |
| tosca.datatypes.nfv.VnfLcmOperationsConfiguration -> instantiate | (KeyValuePairs) additionalParams | InstantiateVnfRequest | Instantiate VNF (see SOL002 and SOL003) |
| TBD tosca.datatypes.nfv.VnfLcmOperationsConfiguration -> scale | (KeyValuePairs) additionalParams | ScaleVnfRequest | Scale VNF (see SOL002 and SOL003) |
| TBD tosca.datatypes.nfv.VnfLcmOperationsConfiguration -> scale_to_level | (KeyValuePairs) additionalParams | ScaleVnfToLevelRequest | Scale VNF to level (see [i.4] and [i.9])l |
| tosca.datatypes.nfv.VnfLcmOperationsConfiguration -> change_flavour | (KeyValuePairs) additionalParams | ChangeVnfFlavourRequest | Change VNF Flavour (see SOL002 and SOL003) |
| TBD tosca.datatypes.nfv.VnfLcmOperationsConfiguration -> operate | (KeyValuePairs) additionalParams | OperateVnfRequest | Operate VNF (see SOL002 and SOL003) |
| TBD tosca.datatypes.nfv.VnfLcmOperationsConfiguration -> heal | (KeyValuePairs) additionalParams | HealVnfRequest | Heal VNF (see SOL002 and SOL003) |
| tosca.datatypes.nfv.VnfLcmOperationsConfiguration -> change_ext_connectivity | (KeyValuePairs) additionalParams | ChangeExtVnfConnectivityRequest | Change External VNF Connectivity (see SOL002 and SOL003) |
| TBD tosca.datatypes.nfv.VnfLcmOperationsConfiguration -> terminate | (KeyValuePairs) additionalParams | TerminateVnfRequest | Terminate VNF (see SOL002 and SOL003) |
| TBD tosca.artifacts.nfv.SwImage | (IdentifierInVnfd) id | VnfPkgInfo -> VnfPackageSoftwareImageInfo | Create VNF Package Info (see SOL005) Query VNF Package Info (see SOL005) |
| Corresponding property for this attribute is not supported in the present document of SOL001. | (IdentifierInVnfd) Id | VnfIndicator | Get indicator value (see SOL002 and SOL003) |
| Corresponding property for this attribute is not supported in the present document of SOL001. | (String) Name | VnfIndicator | Get indicator value (see SOL002 and SOL003) |
| Corresponding property for this attribute is not supported in the present document of SOL001. | (Object) value | VnfIndicator | Get indicator value (see SOL002 and SOL003) |
| TBD | (IdentifierInVnfd) vnfdSoftwareImageId | Grant -> vimAssets -> VimSoftwareImage | Grant Lifecycle Operation (see SOL003) |
| NOTE: The entry "SOL002" in the "Operation" column refers to ETSI GS NFV-SOL 002 [i.4], "SOL003" refers to ETSI GS NFV-SOL 003 [i.9] and "SOL005" refers to ETSI GS NFV-SOL 005 [i.10]. |  |  |  |

              - Annex E (informative):
Authors & contributors

The following people have contributed to the present document:

Rapporteur:
Shitao Li, Huawei

Other contributors:
Shitao Li, Huawei

Thinh Nguyenphu, Nokia

Andrei Kojukhov, Amdocs

Arturo Martin De Nicolas, Ericsson

Yuya Kuno, NTTDOCOMO

Gabor Marton, Nokia

Manchang Ju, ZTE Corporation

Bruno Chatras, Orange

Raquel Morera, Verizon

Satish Vasamsetti, Verizon Wireless

Philip Joseph, RIFT.io

Alex Vul, Intel

Priya T G, Netcracker Technologies Ltd

Li xiang, China Mobile US Research Center

Ren rongwei, China Mobile US Research Center

Yuan yannan, China Mobile US Research Center

Uwe Rauschenbach, Nokia Germany

Nagesha Subramanya, Nokia Germany

kaliyaperumal Ramalingam. Verizon UK Ltd

Abhishek Kumar, Verizon UK Ltd

Philip Joseph, Riftio

Wangyaoguang, Huawei

Lijuan Chen, ZTE Corporation

Annex F (informative):
Change History

| Date | Version | Information about changes |
| --- | --- | --- |
| 2016.05 | 0.0.1 | Implemented NFVSOL(16)000005r1_GS_NFV_SOL001_ToC |
| 2016.07 | 0.02 | Implemented NFVSOL(16)000026r1, NFVSOL(16)000027r1, NFVSOL(16)000028r2 |
| 2017.09 | 0.1.0 | Implemented NFVSOL(17)000539r3, NFVSOL(17)000540r3, NFVSOL(17)000542r2, NFVSOL(17)000544r1 |
| 2017.10 | 0.2.0 | Implemented NFVSOL(17)000543r3, NFVSOL(17)000566, NFVSOL(17)000567r1, NFVSOL(17)000568r1, NFVSOL(17)000569 Editorial modification for clause numbering and format. |
| 2017.11 | 0.3.0 | Clean-up done by editHelp Implemented NFVSOL(17)000545r4, NFVSOL(17)000559r2, NFVSOL(17)000560r2, NFVSOL(17)000570r1, NFVSOL(17)000575r1, NFVSOL(17)000616r1, NFVSOL(17)000641 |
| 2017.12 | 0.4.0 | Implemented NFVSOL(17)000642r1, NFVSOL(17)000675r1, NFVSOL(17)000756 |
| 2018.01 | 0.5.0 | Implemented NFVSOL(17)000621r6, NFVSOL(17)000676r3, NFVSOL(17)000677r2, NFVSOL(17)000736r1, NFVSOL(18)000004r2 |
| 2018.03 | 0.6.0 | Implemented NFVSOL(18)000049, NFVSOL(18)000025r1, NFVSOL(18)00048R1, NFVSOL(18)00040, NFVSOL(18)00023r2, NFVSOL(18)00024r2, NFVSOL(18)00029r1, NFVSOL(18)00063r5, NFVSOL(18)000077, NFVSOL(18)000046r2, NFVSOL(18)000074r2, NFVSOL(17)0000611r5, NFVSOL(18)000041r1, NFVSOL(18)000038r3, NFVSOL(18)000028, NFVSOL(18)000044r2, NFVSOL(18)000055r2, NFVSOL(18)000045r1 |
| 2018.03 | 0.6.1 | Implemented NFVSOL(18)000052r1, NFVSOL(18)000094r5, NFVSOL(18)000043r3, NFVSOL(18)0000112r1, NFVSOL(18)0000117r2, NFVSOL(18)0000124r1, NFVSOL(18)0000129r1, NFVSOL(18)000042r2, NFVSOL(18)0000115r2 |
| 2018.04 | 0.6.2 | Implemented NFVSOL(18)000113r2, NFVSOL(18)000119r2, NFVSOL(18)000121, NFVSOL(18)000135, NFVSOL(18)000116r1, NFVSOL(18)000157, NFVSOL(18)000158r1 |
| 2018.05 | 0.6.3 | Implemented NFVSOL(18)000168, NFVSOL(18)000169r2, NFVSOL(18)000173, NFVSOL(18)000174r2 |
| 2018.05 | 0.7.0 | Implemented NFVSOL(18)000156r7, NFVSOL(18)000142r3, NFVSOL(18)000147, NFVSOL(18)000193r2, NFVSOL(18)000201, NFVSOL(18)000202, NFVSOL(18)000203r2, NFVSOL(18)000205r1, NFVSOL(18)000160r1, NFVSOL(18)000199r2, NFVSOL(18)000200r3, NFVSOL(18)000192r1, NFVSOL(18)000194r2, NFVSOL(18)000231r1, NFVSOL(18)000223r1, NFVSOL(18)000183r2 |
| 2018.06 | 0.8.0 | Implemented NFVSOL(18)000287, NFVSOL(18)00012r11, NFVSOL(18)000197r2, NFVSOL(18)000198r3, NFVSOL(18)000286, NFVSOL(18)000292, NFVSOL(18)000294r1, NFVSOL(18)000295r1, NFVSOL(18)000301r2, NFVSOL(18)000302r2, NFVSOL(18)000240r1, NFVSOL(18)000253r3, NFVSOL(18)000254r2, NFVSOL(18)000256r2, |
| 2018.06 | 0.9.0 | Implemented NFVSOL(18)000288r2 Adding etsi_nfv_sol001_vnfd_0_9_0_type.yaml and SOL001 Graphics v0_9_0.pptx in the draft GS zip package Editorial changes for all the TOSCA type definitions. |
| 2018.08 | 0.10.0 | Implemented NFVSOL(18)000331r4, NFVSOL(18)000335r2, NFVSOL(18)000351r1, NFVSOL(18)000374r1, NFVSOL(18)000376, NFVSOL(18)000382, NFVSOL(18)000395r1, NFVSOL(18)000402r2, NFVSOL(18)000404, NFVSOL(18)000405r2, NFVSOL(18)000406r1, NFVSOL(18)000408r2, NFVSOL(18)000409r1, NFVSOL(18)000411r1, NFVSOL(18)000413, NFVSOL(18)000416r2, NFVSOL(18)000422r4, NFVSOL(18)000423r4, NFVSOL(18)000424r2, NFVSOL(18)000425r2, NFVSOL(18)000430r1, NFVSOL(18)000336r4, NFVSOL(18)000380, NFVSOL(18)000387r2, NFVSOL(18)000394r1, NFVSOL(18)000410r2, NFVSOL(18)000447r3, NFVSOL(18)000427, NFVSOL(18)000403r3, NFVSOL(18)000420r4, NFVSOL(18)000393r1, NFVSOL(18)000415, NFVSOL(18)000398r1, NFVSOL(18)000399r3, NFVSOL(18)000414r2, NFVSOL(18)000379r3, NFVSOL(18)000428r5, NFVSOL(18)000429r4, NFVSOL(18)000465r1, NFVSOL(18)000479, NFVSOL(18)000375r4, NFVSOL(18)000488r2, NFVSOL(18)000492r1 |
| 2018.09 | 0.11.0 | Implemented NFVSOL(18)000486r7, NFVSOL(18)000495r1, NFVSOL(18)000497r3, NFVSOL(18)000498, NFVSOL(18)000500r3, NFVSOL(18)000503r1, NFVSOL(18)000504r1, NFVSOL(18)000505r1, NFVSOL(18)000508, NFVSOL(18)000514r2, NFVSOL(18)000515r1, NFVSOL(18)000516r1, NFVSOL(18)000524r1, NFVSOL(18)000529r2, NFVSOL(18)000530r1, NFVSOL(18)000536r1, NFVSOL(18)000538r1, NFVSOL(18)000541r3, NFVSOL(18)000544r1，NFVSOL(18)000545r1, |
| 2018.10 | 0.12.0 | Implemented NFVSOL(18)000507r3, NFVSOL(18)000621r4, NFVSOL(18)000547r1, NFVSOL(18)000567r1, NFVSOL(18)000574, NFVSOL(18)000579, NFVSOL(18)000590r1 |
| 2018.11 | 0.13.0 | Implemented NFVSOL(18)000563r5, NFVSOL(18)000575r5, NFVSOL(18)000586r2, NFVSOL(18)000587r1, NFVSOL(18)000589r2, NFVSOL(18)000591, NFVSOL(18)000592, NFVSOL(18)000604r2, NFVSOL(18)000606r1, NFVSOL(18)000607r5, NFVSOL(18)000608, NFVSOL(18)000609, NFVSOL(18)000612, NFVSOL(18)000614r1, NFVSOL(18)000615, NFVSOL(18)000616r2, NFVSOL(18)000617r3, NFVSOL(18)000619r3, NFVSOL(18)000620r5, NFVSOL(18)000628r2, NFVSOL(18)000629r1, NFVSOL(18)000630, NFVSOL(18)000631r1, NFVSOL(18)000632, NFVSOL(18)000634, NFVSOL(18)000635, NFVSOL(18)000636r3, NFVSOL(18)000637, NFVSOL(18)000655r2, NFVSOL(18)000658r3, NFVSOL(18)000659, NFVSOL(18)000660, NFVSOL(18)000665r1, NFVSOL(18)000666, NFVSOL(18)000678r1, NFVSOL(18)000679, NFVSOL(18)000682, NFVSOL(18)000684r1, NFVSOL(18)000687 |
| 2019.03 | 2.5.2 | Implemented NFVSOL(19)000063r4, NFVSOL(19)000068r1, NFVSOL(19)000069r2, NFVSOL(19)000070r1, NFVSOL(19)00080, NFVSOL(19)000107r1, NFVSOL(19)000120r1, NFVSOL(19)000121, NFVSOL(19)00039r5, NFVSOL(19)00087r2, NFVSOL(19)00067r5, NFVSOL(19)00077, NFVSOL(19)00082r3, NFVSOL(19)00085, NFVSOL(19)00086, NFVSOL(19)000106r1 |
| 2019.03 | 2.5.3 | Implemented NFVSOL(19)000084r4, NFVSOL(19)0000101r2, NFVSOL(19)0000165, NFVSOL(19)0000166, NFVSOL(19)0000167r1, NFVSOL(19)0000170r4, NFVSOL(19)0000173, NFVSOL(19)0000163, NFVSOL(19)0000119 |
| 2019.03.22 | 2.5.4 | Editorial modification made by ETSI Secretariat allowing to structure the SOL repository on ETSI Forge in a future proof and maintainable way: Forge structure updated: "v2.6.1" tag created. Yaml filenames updated: version numbers removed from filenames (still included in file header). Import statements updated: version number removed from imported filenames. Draft updated: Updated all references to yaml files Updated the forge URLs In B.1: removed the sentence explaining the meaning of x_y_z_ in the filename structure |
| 2019.04.23 | 2.5.5 | 2 comments were raised during the Remote Consensus approval: both requesting to implement the WG SOL approved Change Request in NFVSOL(19)000229r3 onto the final SOL001 draft (see these 2 comments in the RC report). The present version implements NFVSOL(19)000229r3: adding machine readable meta info inside the yaml file indicating the SOL001 release version to which they apply + other editorial changes. |

# History

| Document history |  |  |
| --- | --- | --- |
| V2.5.1 | December 2018 | Publication |
| V2.6.1 | May 2019 | Publication |