Draft ETSI GS NFV-SOL 001 V3.3.3 (2020-09)

Network Functions Virtualisation (NFV) Release 3;

Protocols and Data Models;

NFV descriptors based on TOSCA specification

<

Disclaimer

The present document has been produced and approved by the Network Functions Virtualisation (NFV) ETSI Industry Specification Group (ISG) and represents the views of those members who participated in this ISG.
It does not necessarily represent the views of the entire ETSI membership.

Group Specification

Reference

RGS/NFV-SOL001ed331

Keywords

data, information model, model, NFV

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2020.

All rights reserved.

DECT™, PLUGTESTS™, UMTS™ and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPP™ and LTE™ are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and
of the oneM2M Partners.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	20

Foreword	20

Modal verbs terminology	20

1	Scope	21

2	References	21

2.1	Normative references	21

2.2	Informative references	22

3	Definition of terms, symbols and abbreviations	23

3.1	Terms	23

3.2	Symbols	23

3.3	Abbreviations	23

4	Overview of TOSCA model	24

5	General concept of using TOSCA to model NFV descriptors	24

5.1	Introduction	24

5.2	Network Service Descriptor	24

5.3	Virtualised Network Function Descriptor	24

5.4	Physical Network Function Descriptor	25

5.5	tosca_definitions_version and Namespace prefix	25

5.6	Imports statement	26

5.6.1	VNFD TOSCA service template	26

5.6.2	NSD TOSCA service template	26

5.6.3	PNFD TOSCA service template	27

5.7	Type extension	27

5.7.1	Introduction	27

5.7.2	Rules	28

5.7.3	VNFD Types	28

5.7.4	NSD types	29

5.8	Non-Backward Compatible changes	30

6	VNFD TOSCA model	30

6.1	Introduction	30

6.2	Data Types	31

6.2.1	tosca.datatypes.nfv.CpProtocolData	31

6.2.1.1	Description	31

6.2.2	tosca.datatypes.nfv.AddressData	32

6.2.2.1	Description	32

6.2.3	tosca.datatypes.nfv.L2AddressData	32

6.2.3.1	Description	32

6.2.4	tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements	32

6.2.4.1	Description	32

6.2.4.2	Properties	32

6.2.4.3	Definition	33

6.2.4.4	Examples	33

6.2.4.5	Additional Requirements	33

6.2.5	tosca.datatypes.nfv.L3AddressData	33

6.2.5.1	Description	33

6.2.6	tosca.datatypes.nfv.RequestedAdditionalCapability	34

6.2.6.1	Description	34

6.2.6.2	Properties	34

6.2.6.3	Definition	35

6.2.6.4	Examples	35

6.2.6.5	Additional Requirements	35

6.2.7	tosca.datatypes.nfv.VirtualMemory	35

6.2.7.1	Description	35

6.2.7.2	Properties	36

6.2.7.3	Definitions	36

6.2.7.4	Examples	36

6.2.7.5	Additional Requirements	37

6.2.8	tosca.datatypes.nfv.VirtualCpu	37

6.2.8.1	Description	37

6.2.8.2	Properties	37

6.2.8.3	Definition	37

6.2.8.4	Examples	38

6.2.8.5	Additional Requirements	38

6.2.9	tosca.datatypes.nfv.VirtualCpuPinning	38

6.2.9.1	Description	38

6.2.9.2	Properties	38

6.2.9.3	Definition	39

6.2.9.4	Examples	39

6.2.9.5	Additional Requirements	39

6.2.10	tosca.datatypes.nfv.VnfcConfigurableProperties	39

6.2.10.1	Description	39

6.2.10.2	Properties	40

6.2.10.3	Definition	40

6.2.10.4	Examples	40

6.2.10.5	Additional Requirements	41

6.2.11	tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties	41

6.2.11.1	Description	41

6.2.11.2	Properties	41

6.2.11.3	Definition	41

6.2.12	tosca.datatypes.nfv.VduProfile	41

6.2.12.1	Description	41

6.2.12.2	Properties	42

6.2.12.3	Definition	42

6.2.12.4	Examples	43

6.2.12.5	Additional requirements	44

6.2.13	tosca.datatypes.nfv.VlProfile	44

6.2.13.1	Description	44

6.2.13.2	Properties	44

6.2.13.3	Definition	45

6.2.13.4	Examples	45

6.2.13.5	Additional Requirements	45

6.2.14	tosca.datatypes.nfv.VirtualLinkProtocolData	45

6.2.14.1	Description	45

6.2.14.2	Properties	46

6.2.14.3	Definition	46

6.2.14.4	Examples	46

6.2.14.5	Additional Requirements	46

6.2.15	tosca.datatypes.nfv.L2ProtocolData	47

6.2.15.1	Description	47

6.2.15.2	Properties	47

6.2.15.3	Definition	47

6.2.15.4	Examples	48

6.2.15.5	Additional Requirements	48

6.2.16	tosca.datatypes.nfv.L3ProtocolData	48

6.2.16.1	Description	48

6.2.16.2	Properties	48

6.2.16.3	Definition	49

6.2.16.4	Examples	50

6.2.16.5	Additional Requirements	50

6.2.17	tosca.datatypes.nfv.IpAllocationPool	50

6.2.17.1	Description	50

6.2.17.2	Properties	50

6.2.17.3	Definition	51

6.2.17.4	Examples	51

6.2.17.5	Additional Requirements	51

6.2.18	tosca.datatypes.nfv.InstantiationLevel	51

6.2.18.1	Description	51

6.2.18.2	Properties	51

6.2.18.3	Definition	52

6.2.18.4	Examples	52

6.2.18.5	Additional Requirements	52

6.2.19	tosca.datatypes.nfv.VduLevel	52

6.2.19.1	Description	52

6.2.19.2	Properties	52

6.2.19.3	Definition	53

6.2.19.4	Examples	53

6.2.19.5	Additional Requirements	53

6.2.20	tosca.datatypes.nfv.VnfLcmOperationsConfiguration	53

6.2.20.1	Description	53

6.2.20.2	Properties	53

6.2.20.3	Definition	54

6.2.20.4	Examples	55

6.2.20.5	Additional Requirements	55

6.2.21	tosca.datatypes.nfv.VnfInstantiateOperationConfiguration	56

6.2.21.1	Description	56

6.2.21.2	Properties	56

6.2.21.3	Definition	56

6.2.21.4	Examples	56

6.2.21.5	Additional Requirements	56

6.2.22	tosca.datatypes.nfv.VnfScaleOperationConfiguration	56

6.2.22.1	Description	56

6.2.22.2	Properties	56

6.2.22.3	Definition	57

6.2.22.4	Examples	57

6.2.22.5	Additional Requirements	57

6.2.23	tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration	57

6.2.23.1	Description	57

6.2.23.2	Properties	57

6.2.23.3	Definition	58

6.2.23.4	Examples	58

6.2.23.5	Additional Requirements	58

6.2.24	tosca.datatypes.nfv.VnfHealOperationConfiguration	58

6.2.24.1	Description	58

6.2.24.2	Properties	58

6.2.24.3	Definition	58

6.2.24.4	Examples	59

6.2.24.5	Additional Requirements	59

6.2.25	tosca.datatypes.nfv.VnfTerminateOperationConfiguration	59

6.2.25.1	Description	59

6.2.25.2	Properties	59

6.2.25.3	Definition	59

6.2.25.4	Examples	60

6.2.25.5	Additional Requirements	60

6.2.26	tosca.datatypes.nfv.VnfOperateOperationConfiguration	60

6.2.26.1	Description	60

6.2.26.2	Properties	60

6.2.26.3	Definition	60

6.2.26.4	Examples	61

6.2.26.5	Additional Requirements	61

6.2.27	tosca.datatypes.nfv.ScaleInfo	61

6.2.27.1	Description	61

6.2.27.2	Properties	61

6.2.27.3	Definition	61

6.2.27.4	Examples	62

6.2.27.5	Additional Requirements	62

6.2.28	tosca.datatypes.nfv.ScalingAspect	62

6.2.28.1	Description	62

6.2.28.2	Properties	62

6.2.28.3	Definition	62

6.2.28.4	Examples	63

6.2.28.5	Additional Requirements	63

6.2.29	tosca.datatypes.nfv.LinkBitrateRequirements	63

6.2.29.1	Description	63

6.2.30	tosca.datatypes.nfv.ConnectivityType	63

6.2.30.1	Description	63

6.2.31	tosca.datatypes.nfv.VnfConfigurableProperties	63

6.2.31.1	Description	63

6.2.31.2	Properties	64

6.2.31.3	Definition	65

6.2.31.4	Examples	66

6.2.31.5	Additional Requirements	68

6.2.32	tosca.datatypes.nfv.VnfAdditionalConfigurableProperties	68

6.2.32.1	Description	68

6.2.32.2	Properties	68

6.2.32.3	Definition	68

6.2.32.4	Examples	69

6.2.32.5	Additional Requirements	69

6.2.33	tosca.datatypes.nfv.VnfInfoModifiableAttributes	69

6.2.33.1	Description	69

6.2.33.2	Properties	69

6.2.33.3	Definition	69

6.2.33.4	Examples	70

6.2.33.5	Additional Requirements	70

6.2.34	tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions	70

6.2.34.1	Description	70

6.2.34.2	Properties	70

6.2.34.3	Definition	71

6.2.34.4	Examples	71

6.2.34.5	Additional Requirements	71

6.2.35	tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata	71

6.2.35.1	Description	71

6.2.35.2	Properties	71

6.2.35.3	Definition	71

6.2.35.4	Examples	71

6.2.35.5	Additional Requirements	72

6.2.36	tosca.datatypes.nfv.Qos	72

6.2.36.1	Description	72

6.2.37	tosca.datatypes.nfv.LogicalNodeData	72

6.2.37.1	Description	72

6.2.37.2	Properties	72

6.2.37.3	Definition	73

6.2.37.4	Examples	73

6.2.37.5	Additional Requirements	73

6.2.38	tosca.datatypes.nfv.SwImageData	73

6.2.38.1	Description	73

6.2.38.2	Properties	74

6.2.38.3	Definition	75

6.2.38.4	Examples	76

6.2.38.5	Additional Requirements	76

6.2.39	tosca.datatypes.nfv.VirtualBlockStorageData	76

6.2.39.1	Description	76

6.2.39.2	Properties	76

6.2.39.3	Definition	76

6.2.39.4	Examples	77

6.2.39.5	Additional Requirements	77

6.2.40	tosca.datatypes.nfv.VirtualObjectStorageData	77

6.2.40.1	Description	77

6.2.40.2	Properties	77

6.2.40.3	Definition	78

6.2.40.4	Examples	78

6.2.40.5	Additional Requirements	78

6.2.41	tosca.datatypes.nfv.VirtualFileStorageData	78

6.2.41.1	Description	78

6.2.41.2	Properties	78

6.2.41.3	Definition	79

6.2.41.4	Examples	79

6.2.41.5	Additional Requirements	79

6.2.42	tosca.datatypes.nfv.VirtualLinkBitrateLevel	79

6.2.42.1	Description	79

6.2.42.2	Properties	79

6.2.42.3	Definition	80

6.2.42.4	Examples	80

6.2.42.5	Additional Requirements	80

6.2.43	tosca.datatypes.nfv.VnfOperationAdditionalParameters	80

6.2.43.1	Description	80

6.2.43.2	Properties	80

6.2.43.3	Definition	80

6.2.43.4	Examples	81

6.2.43.5	Additional Requirements	82

6.2.44	tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration	82

6.2.44.1	Description	82

6.2.44.2	Properties	82

6.2.44.3	Definition	82

6.2.44.4	Examples	82

6.2.44.5	Additional Requirements	82

6.2.45	tosca.datatypes.nfv.VnfChangeExtConnectivityOperation Configuration	82

6.2.45.1	Description	82

6.2.45.2	Properties	83

6.2.45.3	Definition	83

6.2.45.4	Examples	83

6.2.45.5	Additional Requirements	83

6.2.46	tosca.datatypes.nfv.VnfMonitoringParameter	83

6.2.47	tosca.datatypes.nfv.VnfcMonitoringParameter	83

6.2.47.1	Description	83

6.2.47.2	Properties	83

6.2.47.3	Definition	84

6.2.47.4	Examples	85

6.2.47.5	Additional Requirements	85

6.2.48	tosca.datatypes.nfv.VirtualLinkMonitoringParameter	85

6.2.48.1	Description	85

6.2.48.2	Properties	85

6.2.48.3	Definition	86

6.2.48.4	Examples	86

6.2.48.5	Additional Requirements	86

6.2.49	tosca.datatypes.nfv.InterfaceDetails	87

6.2.49.1	Description	87

6.2.49.2	Properties	87

6.2.49.3	Definition	87

6.2.49.4	Examples	87

6.2.49.5	Additional Requirements	87

6.2.50	tosca.datatypes.nfv.UriComponents	88

6.2.50.1	Description	88

6.2.50.2	Properties	88

6.2.50.3	Definition	88

6.2.50.4	Examples	89

6.2.50.5	Additional Requirements	89

6.2.51	tosca.datatypes.nfv.UriAuthority	89

6.2.51.1	Description	89

6.2.51.2	Properties	89

6.2.51.3	Definition	90

6.2.51.4	Examples	90

6.2.51.5	Additional Requirements	90

6.2.52	tosca.datatypes.nfv.VnfProfile	90

6.2.52.1	Description	90

6.2.53	tosca.datatypes.nfv.ChecksumData	90

6.2.53.1	Description	90

6.2.53.2	Properties	91

6.2.53.3	Definition	91

6.2.53.4	Examples	91

6.2.53.5	Additional Requirements	91

6.2.54	tosca.datatypes.nfv.VnfmInterfaceInfo	92

6.2.54.1	Description	92

6.2.54.2	Properties	92

6.2.54.3	Definition	92

6.2.54.4	Examples	93

6.2.54.5	Additional Requirements	93

6.2.55	tosca.datatypes.nfv.OauthServerInfo	93

6.2.55.1	Description	93

6.2.55.2	Properties	93

6.2.55.3	Definition	93

6.2.55.4	Examples	93

6.2.55.5	Additional Requirements	93

6.2.56	tosca.datatypes.nfv.BootData	93

6.2.56.1	Description	93

6.2.56.2	Properties	94

6.2.56.3	Definition	95

6.2.56.4	Examples	95

6.2.56.5	Additional Requirements	95

6.2.57	tosca.datatypes.nfv.KvpData	95

6.2.57.1	Description	95

6.2.57.2	Properties	96

6.2.57.3	Definition	96

6.2.57.4	Examples	96

6.2.57.5	Additional Requirements	96

6.2.58	tosca.datatypes.nfv.ContentOrFileData	96

6.2.58.1	Description	96

6.2.58.2	Properties	96

6.2.58.3	Definition	97

6.2.58.4	Examples	98

6.2.58.5	Additional Requirements	98

6.2.59	tosca.datatypes.nfv.BootDataVimSpecificProperties	98

6.2.59.1	Description	98

6.2.59.2	Properties	98

6.2.59.3	Definition	99

6.2.59.4	Examples	99

6.2.59.5	Additional Requirements	99

6.2.60	tosca.datatypes.nfv.VnfPackageChangeSelector	99

6.2.60.1	Description	99

6.2.60.2	Properties	99

6.2.60.3	Definition	100

6.2.60.4	Examples	100

6.2.60.5	Additional Requirements	100

6.2.61	tosca.datatypes.nfv.VnfPackageChangeComponentMapping	100

6.2.61.1	Description	100

6.2.61.2	Properties	101

6.2.61.3	Definition	101

6.2.61.4	Examples	102

6.2.61.5	Additional Requirements	102

6.2.62	tosca.datatypes.nfv.VnfChangeCurrentPackageOperation Configuration	102

6.2.62.1	Description	102

6.2.62.2	Properties	102

6.2.62.3	Definition	102

6.2.62.4	Examples	102

6.2.62.5	Additional Requirements	102

6.2.63	tosca.datatypes.nfv.VnfCreateSnapshotOperationConfiguration	103

6.2.63.1	Description	103

6.2.63.2	Properties	103

6.2.63.3	Definition	103

6.2.63.4	Examples	103

6.2.63.5	Additional Requirements	103

6.2.64	tosca.datatypes.nfv.VnfRevertToSnapshotOperationConfiguration	103

6.2.64.1	Description	103

6.2.64.2	Properties	103

6.2.64.3	Definition	104

6.2.64.4	Examples	104

6.2.64.5	Additional Requirements	104

6.3	Artifact Types	104

6.3.1	tosca.artifacts.nfv.SwImage	104

6.3.1.1	Description	104

6.3.1.2	Properties	104

6.3.1.3	Description	105

6.3.2	tosca.artifacts.Implementation.nfv.Mistral	106

6.3.2.1	Description	106

6.3.2.2	Definition	107

6.4	Capability Types	107

6.4.1	tosca.capabilities.nfv.VirtualBindable	107

6.4.1.1	Description	107

6.4.1.2	Properties	107

6.4.1.3	Definition	107

6.4.2	tosca.capabilities.nfv.VirtualLinkable	107

6.4.2.1	Description	107

6.4.3	tosca.capabilities.nfv.VirtualCompute	108

6.4.3.1	Description	108

6.4.3.2	Properties	108

6.4.3.3	Definition	108

6.4.4	tosca.capabilities.nfv.VirtualStorage	109

6.4.4.1	Description	109

6.4.4.2	Definition	109

6.5	Requirements Types	110

6.6	Relationship Types	110

6.6.1	tosca.relationships.nfv.VirtualBindsTo	110

6.6.1.1	Description	110

6.6.1.2	Properties	110

6.6.1.3	Definition	110

6.6.2	tosca.relationships.nfv.VirtualLinksTo	111

6.6.2.1	Description	111

6.6.3	tosca.relationships.nfv.AttachesTo	111

6.6.3.1	Description	111

6.6.3.2	Properties	111

6.6.3.3	Definition	111

6.7	Interface Types	112

6.7.1	tosca.interfaces.nfv.Vnflcm	112

6.7.1.1	Description	112

6.7.1.2	Definition	113

6.7.1.3	Additional Requirements	116

6.7.1.4	Support of LCM scripts	117

6.7.1.5	Examples	117

6.7.2	tosca.interfaces.nfv.VnfIndicator	120

6.7.2.1	Description	120

6.7.2.2	Definition	120

6.7.2.3	Examples	120

6.7.3	tosca.interfaces.nfv.ChangeCurrentVnfPackage	120

6.7.3.1	Description	120

6.7.3.2	Definition	120

6.7.3.3	Examples	121

6.8	Node Types	121

6.8.1	tosca.nodes.nfv.VNF	121

6.8.1.1	Description	121

6.8.1.2	Properties	121

6.8.1.3	Attributes	123

6.8.1.4	Requirements	124

6.8.1.5	Capabilities	124

6.8.1.6	Definition	124

6.8.1.7	Artifact	126

6.8.1.8	Additional Requirements	126

6.8.1.9	Example	127

6.8.2	tosca.nodes.nfv.VnfExtCp	130

6.8.2.1	Description	130

6.8.2.2	Properties	130

6.8.2.3	Attributes	130

6.8.2.4	Requirements	130

6.8.2.5	Capabilities	131

6.8.2.6	Definition	131

6.8.2.7	Additional Requirements	131

6.8.2.8	Example	131

6.8.3	tosca.nodes.nfv.Vdu.Compute	132

6.8.3.1	Description	132

6.8.3.2	Properties	132

6.8.3.3	Attributes	133

6.8.3.4	Requirements	133

6.8.3.5	Capabilities	133

6.8.3.6	Definition	134

6.8.3.7	Additional requirements	135

6.8.3.8	Example	136

6.8.4	tosca.nodes.nfv.Vdu.VirtualBlockStorage	141

6.8.4.1	Description	141

6.8.4.2	Properties	141

6.8.4.3	Attributes	142

6.8.4.4	Requirements	142

6.8.4.5	Capabilities	142

6.8.4.6	Definition	142

6.8.4.7	Additional requirements	142

6.8.5	tosca.nodes.nfv.Vdu.VirtualObjectStorage	143

6.8.5.1	Description	143

6.8.5.2	Properties	143

6.8.5.3	Attributes	143

6.8.5.4	Requirements	143

6.8.5.5	Capabilities	143

6.8.5.6	Definition	143

6.8.5.7	Additional requirements	144

6.8.6	tosca.nodes.nfv.Vdu.VirtualFileStorage	144

6.8.6.1	Description	144

6.8.6.2	Properties	144

6.8.6.3	Attributes	144

6.8.6.4	Requirements	144

6.8.6.5	Capabilities	145

6.8.6.6	Definition	145

6.8.6.7	Additional requirements	145

6.8.7	tosca.nodes.nfv.Cp	145

6.8.7.1	Description	145

6.8.8	tosca.nodes.nfv.VduCp	145

6.8.8.1	Description	145

6.8.8.2	Properties	146

6.8.8.3	Attributes	146

6.8.8.4	Requirements	146

6.8.8.6	Definition	147

6.8.9	tosca.nodes.nfv.VnfVirtualLink	148

6.8.9.1	Description	148

6.8.9.2	Properties	148

6.8.9.3	Requirements	148

6.8.9.4	Capabilities	148

6.8.9.5	Definition	149

6.8.10	tosca.nodes.nfv.VipCp	149

6.8.10.1	Description	149

6.8.10.2	Properties	150

6.8.10.3	Attributes	150

6.8.10.4	Requirements	150

6.8.10.5	Definition	150

6.8.10.6	Example	151

6.9	Group Types	152

6.9.1	tosca.groups.nfv.PlacementGroup	152

6.9.1.1	Description	152

6.9.1.2	Properties	152

6.9.1.3	Definition	153

6.9.1.4	Additional Requirements	153

6.9.1.5	Examples	153

6.10	Policy Types	153

6.10.1	tosca.policies.nfv.InstantiationLevels	153

6.10.1.1	Description	153

6.10.1.2	Properties	153

6.10.1.3	Definition	154

6.10.2	tosca.policies.nfv.VduInstantiationLevels	154

6.10.2.1	Description	154

6.10.2.2	Properties	154

6.10.2.3	Definition	155

6.10.2.4	Additional Requirements	155

6.10.3	tosca.policies.nfv.VirtualLinkInstantiationLevels	155

6.10.3.1	Description	155

6.10.3.2	Properties	155

6.10.3.3	Definition	156

6.10.3.4	Additional Requirements	156

6.10.4	Void	156

6.10.5	tosca.policies.nfv.ScalingAspects	156

6.10.5.1	Description	156

6.10.5.2	Properties	156

6.10.5.3	Definition	157

6.10.5.5	Examples	157

6.10.6	tosca.policies.nfv.VduScalingAspectDeltas	157

6.10.6.1	Description	157

6.10.6.2	Properties	157

6.10.6.3	Definition	158

6.10.6.5	Examples	158

6.10.7	tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas	158

6.10.7.1	Description	158

6.10.7.2	Properties	159

6.10.7.3	Definition	159

6.10.7.5	Examples	159

6.10.8	tosca.policies.nfv.VduInitialDelta	159

6.10.8.1	Description	159

6.10.8.2	Properties	160

6.10.8.3	Definition	160

6.10.8.4	Examples	160

6.10.9	tosca.policies.nfv.VirtualLinkBitrateInitialDelta	160

6.10.9.1	Description	160

6.10.9.2	Properties	160

6.10.9.3	Definition	161

6.10.9.4	Examples	161

6.10.10	AffinityRule, AntiAffinityRule	161

6.10.10.1	Description	161

6.10.10.2	Properties	162

6.10.10.3	targets	162

6.10.10.4	Definition	162

6.10.10.5	Examples	163

6.10.11	tosca.policies.nfv.Abstract.SecurityGroupRule	163

6.10.11.1	Description	163

6.10.12	tosca.policies.nfv.SupportedVnfInterface	164

6.10.12.1	Description	164

6.10.12.2	Properties	164

6.10.12.3	Definition	164

6.10.12.4	Additional requirements	164

6.10.12.5	Example	165

6.10.13	tosca.policies.nfv.SecurityGroupRule	165

6.10.13.1	Description	165

6.10.13.2	Properties	165

6.10.13.3	targets	165

6.10.13.4	Definition	165

6.10.13.5	Additional Requirements	166

6.10.14	tosca.policies.nfv.VnfIndicator	166

6.10.14.1	Description	166

6.10.14.2	Properties	166

6.10.14.3	Definition	166

6.10.14.4	Additional requirements	166

6.10.15	tosca.policies.nfv.VnfPackageChange	167

6.10.15.1	Description	167

6.10.15.2	Properties	167

6.10.15.3	Definition	168

6.10.15.4	Additional Requirements	168

6.10.15.5	Example	169

6.11	VNFD TOSCA service template design	175

6.11.1	General	175

6.11.2	Single or multiple deployment flavour design with two levels of service templates	175

6.11.3	Single deployment flavour design with one service template	176

7	NSD TOSCA model	177

7.1	Introduction	177

7.2	Data Types	178

7.2.1	Void	178

7.2.2	tosca.datatypes.nfv.VnfProfile	178

7.2.2.1	Description	178

7.2.3	tosca.datatype.nfv.NsVlProfile	178

7.2.3.1	Description	178

7.2.3.2	Properties	178

7.2.3.3	Definition	179

7.2.3.4	Examples	179

7.2.3.5	Additional Requirements	179

7.2.4	tosca.datatypes.nfv.ConnectivityType	180

7.2.4.1	Description	180

7.2.5	tosca.datatypes.nfv.NsVirtualLinkQos	180

7.2.5.1	Description	180

7.2.5.2	Properties	180

7.2.5.3	Definition	180

7.2.5.4	Examples	180

7.2.5.5	Additional Requirements	180

7.2.6	tosca.datatypes.nfv.LinkBitrateRequirements	181

7.2.6.1	Description	181

7.2.7	Void	181

7.2.8	Void	181

7.2.9	Void	181

7.2.10	Void	181

7.2.11	tosca.datatypes.nfv.CpProtocolData	181

7.2.11.1	Description	181

7.2.12	tosca.datatypes.nfv.AddressData	181

7.2.12.1	Description	181

7.2.13	tosca.datatypes.nfv.L2AddressData	181

7.2.13.1	Description	181

7.2.14	tosca.datatypes.nfv.L3AddressData	181

7.2.14.1	Description	181

7.2.15	tosca.datatypes.nfv.Qos	181

7.2.15.1	Description	181

7.2.16	tosca.datatypes.nfv.NsProfile	181

7.2.16.1	Description	181

7.2.16.2	Properties	182

7.2.16.3	Definition	182

7.2.16.4	Example	183

7.2.16.5	Additional Requirements	183

7.2.17	tosca.datatypes.nfv.Mask	183

7.2.17.1	Description	183

7.2.17.2	Properties	183

7.2.17.3	Definition	183

7.2.17.4	Examples	184

7.2.18	tosca.datatypes.nfv.NsOperationAdditionalParameters	184

7.2.18.1	Description	184

7.2.18.2	Properties	184

7.2.18.3	Definition	184

7.2.18.4	Examples	184

7.2.19	tosca.datatypes.nfv.NsMonitoringParameter	185

7.2.19.1	Description	185

7.2.19.2	Properties	185

7.2.19.3	Definition	186

7.2.19.4	Examples	186

7.2.19.5	Additional Requirements	186

7.2.20	tosca.datatypes.nfv.VnfMonitoringParameter	187

7.3	Artifact Types	194

7.4	Capabilities Types	194

7.4.1	tosca.capabilities.nfv.VirtualLinkable	194

7.4.1.1	Description	194

7.4.2	tosca.capabilities.nfv.Forwarding	194

7.4.2.1	Description	194

7.4.2.2	Properties	194

7.4.2.3	Definition	194

7.5	Requirements Types	194

7.6	Relationship Types	195

7.6.1	tosca.relationships.nfv.VirtualLinksTo	195

7.6.1.1	Description	195

7.6.2	tosca.relationships.nfv.ForwardTo	195

7.6.2.1	Description	195

7.6.2.2	Properties	195

7.6.2.3	Definition	195

7.7	Interface Types	195

7.7.1	tosca.interfaces.nfv.Nslcm	195

7.7.1.1	Description	195

7.7.1.2	Definition	196

7.7.1.3	Additional Requirements	197

7.7.1.4	Support of LCM scripts	197

7.7.1.5	Examples	198

7.8	Node Types	199

7.8.1	tosca.nodes.nfv.NS	199

7.8.1.1	Description	199

7.8.1.2	Properties	199

7.8.1.3	Attributes	199

7.8.1.4	Requirements	199

7.8.1.5	Capabilities	200

7.8.1.6	Definition	200

7.8.1.7	Artifact	201

7.8.1.8	Additional requirements	201

7.8.2	tosca.nodes.nfv.Sap	201

7.8.2.1	Description	201

7.8.2.2	Properties	201

7.8.2.3	Attributes	201

7.8.2.4	Requirements	201

7.8.2.5	Capabilities	202

7.8.2.6	Definition	202

7.8.2.7	Additional requirements	202

7.8.2.8	Example	202

7.8.3	tosca.nodes.nfv.NsVirtualLink	202

7.8.3.1	Description	202

7.8.3.2	Properties	203

7.8.3.3	Attributes	203

7.8.3.4	Requirements	203

7.8.3.5	Capabilities	203

7.8.3.6	Definition	203

7.8.3.7	Artifact	204

7.8.3.8	Additional Requirements	204

7.8.3.9	Example	204

7.8.4	tosca.nodes.nfv.Cp	204

7.8.4.1	Description	204

7.8.5	tosca.nodes.nfv.NfpPositionElement	204

7.8.5.1	Description	204

7.8.5.2	Properties	204

7.8.5.3	Attributes	205

7.8.5.4	Requirements	205

7.8.5.5	Capabilities	205

7.8.5.6	Definition	205

7.8.5.7	Artifact	205

7.8.5.8	Additional Requirements	205

7.8.5.9	Example	205

7.8.6	tosca.nodes.nfv.NFP	206

7.8.6.1	Description	206

7.8.6.2	Properties	206

7.8.6.3	Attributes	206

7.8.6.4	Requirements	206

7.8.6.5	Capabilities	206

7.8.6.6	Definition	206

7.8.7	tosca.nodes.nfv.NfpPosition	207

7.8.7.1	Description	207

7.8.7.2	Properties	207

7.8.7.3	Attributes	207

7.8.7.4	Requirements	207

7.8.7.5	Capabilities	208

7.8.7.6	Definition	208

7.8.7.7	Artifact	208

7.8.7.8	Additional Requirements	208

7.8.7.9	Example	209

7.8.8	tosca.nodes.nfv.Forwarding	209

7.8.8.1	Description	209

7.8.8.2	Properties	209

7.8.8.3	Attributes	209

7.8.8.4	Requirements	209

7.8.8.5	Capabilities	209

7.8.8.6	Definition	210

7.8.8.7	Artifact	210

7.8.8.8	Additional Requirements	210

7.8.8.9	Example	210

7.9	Group Types	210

7.9.1	tosca.groups.nfv.NsPlacementGroup	210

7.9.1.1	Description	210

7.9.1.2	Properties	211

7.9.1.3	Definition	211

7.9.1.4	Additional Requirements	211

7.9.2	tosca.groups.nfv.VNFFG	211

7.9.2.1	Description	211

7.9.2.2	Properties	211

7.9.2.3	Definition	212

7.9.2.4	Additional Requirements	212

7.9.2.5	Example	212

7.10	Policy Types	212

7.10.1	NsAffinityRule, NsAntiAffinityRule	212

7.10.1.1	Description	212

7.10.1.2	Properties	213

7.10.1.3	Targets	213

7.10.1.4	Definition	213

7.10.1.5	Examples	214

7.10.2	tosca.policies.nfv.NsSecurityGroupRule	215

7.10.2.1	Description	215

7.10.2.2	Properties	215

7.10.2.3	targets	215

7.10.2.4	Definition	215

7.10.2.5	Additional Requirements	215

7.10.3	tosca.policies.nfv.NfpRule	215

7.10.3.1	Description	215

7.10.3.2	Properties	216

7.10.3.3	Targets	216

7.10.3.4	Definition	217

7.10.3.5	Example	218

7.10.4	tosca.policies.nfv.NsMonitoring	218

7.10.4.1	Description	218

7.10.4.2	Properties	218

7.10.4.3	targets	218

7.10.4.4	Definition	219

7.10.4.5	Additional Requirements	219

7.10.5	tosca.policies.nfv.VnfMonitoring	219

7.10.5.1	Description	219

7.10.5.2	Properties	219

7.10.5.3	targets	220

7.10.5.4	Definition	220

7.10.5.5	Additional Requirements	220

7.10.6	tosca.policies.nfv.Abstract.SecurityGroupRule	220

7.10.6.1	Description	220

7.11	NSD TOSCA service template design	228

7.11.1	General	228

7.11.2	Single or multiple deployment flavour design with two levels of service templates	229

7.11.3	Single deployment flavour design with one service template	230

8	PNFD TOSCA model	231

8.1	Introduction	231

8.2	Data Types	232

8.2.1	tosca.datatypes.nfv.CpProtocolData	232

8.2.1.1	Description	232

8.2.2	tosca.datatypes.nfv.AddressData	232

8.2.2.1	Description	232

8.2.3	tosca.datatypes.nfv.L2AddressData	232

8.2.3.1	Description	232

8.2.4	tosca.datatypes.nfv.L3AddressData	232

8.2.4.1	Description	232

8.2.5	tosca.datatypes.nfv.LocationInfo	233

8.2.5.1	Description	233

8.2.5.2	Properties	233

8.2.5.3	Definition	233

8.2.5.4	Examples	233

8.2.5.5	Additional Requirements	234

8.2.6	tosca.datatypes.nfv.CivicAddressElement	234

8.2.6.1	Description	234

8.2.6.2	Properties	234

8.2.6.3	Definition	234

8.2.6.4	Examples	235

8.2.6.5	Additional Requirements	235

8.2.7	tosca.datatypes.nfv.GeographicCoordinates	235

8.2.7.1	Description	235

8.2.7.2	Properties	235

8.2.7.3	Definition	235

8.2.7.4	Examples	236

8.2.7.5	Additional Requirements	236

8.3	Artifact Types	236

8.4	Capabilities Types	236

8.4.1	tosca.capabilities.nfv.VirtualLinkable	236

8.4.1.1	Description	236

8.5	Requirements Types	236

8.6	Relationship Types	237

8.6.1	tosca.relationships.nfv.VirtualLinksTo	237

8.6.1.1	Description	237

8.7	Interface Types	237

8.8	Node Types	237

8.8.1	tosca.nodes.nfv.PNF	237

8.8.1.1	Description	237

8.8.1.2	Properties	237

8.8.1.3	Attributes	238

8.8.1.4	Requirements	238

8.8.1.5	Capabilities	238

8.8.1.6	Definition	238

8.8.1.7	Artifact	239

8.8.1.8	Additional Requirements	239

8.8.1.9	Example	239

8.8.2	tosca.nodes.nfv.PnfExtCp	239

8.8.2.1	Description	239

8.8.2.2	Properties	239

8.8.2.3	Attributes	239

8.8.2.4	Requirements	239

8.8.2.5	Capabilities	240

8.8.2.6	Definition	240

8.8.3	tosca.nodes.nfv.Cp	240

8.8.3.1	Description	240

8.9	Group Types	240

8.10	Policy Types	240

8.10.1	tosca.policies.nfv.PnfSecurityGroupRule	240

8.10.1.1	Description	240

8.10.1.2	Properties	240

8.10.1.3	targets	241

8.10.1.4	Definition	241

8.10.1.5	Additional Requirements	241

8.10.2	tosca.policies.nfv.Abstract.SecurityGroupRule	241

8.10.2.1	Description	241

8.11	PNFD TOSCA service template design	241

8.11.1	General	241

9	Common Definitions	242

9.1	Introduction	242

9.2	Data Types	242

9.2.1	tosca.datatypes.nfv.L2AddressData	242

9.2.1.1	Description	242

9.2.1.2	Properties	242

9.2.1.3	Definition	242

9.2.1.4	Examples	243

9.2.1.5	Additional Requirements	243

9.2.2	tosca.datatypes.nfv.L3AddressData	243

9.2.2.1	Description	243

9.2.2.2	Properties	243

9.2.2.3	Definition	243

9.2.2.4	Examples	244

9.2.2.5	Additional Requirements	244

9.2.3	tosca.datatypes.nfv.AddressData	244

9.2.3.1	Description	244

9.2.3.2	Properties	244

9.2.3.3	Definition	245

9.2.3.4	Examples	245

9.2.3.5	Additional Requirements	246

9.2.4	tosca.datatypes.nfv.ConnectivityType	246

9.2.4.1	Description	246

9.2.4.2	Properties	246

9.2.4.3	Definition	246

9.2.4.4	Examples	247

9.2.4.5	Additional Requirements	247

9.2.5	tosca.datatypes.nfv.LinkBitrateRequirements	247

9.2.5.1	Description	247

9.2.5.2	Properties	247

9.2.5.3	Definition	247

9.2.5.4	Examples	248

9.2.5.5	Additional Requirements	248

9.2.6	tosca.datatypes.nfv.CpProtocolData	248

9.2.6.1	Description	248

9.2.6.2	Properties	248

9.2.6.3	Definition	249

9.2.6.4	Examples	249

9.2.6.5	Additional Requirements	249

9.2.7	tosca.datatypes.nfv.Qos	249

9.2.7.1	Description	249

9.2.7.2	Properties	249

9.2.7.3	Definition	250

9.2.7.4	Examples	250

9.2.7.5	Additional Requirements	250

9.2.8	tosca.datatypes.nfv.VnfProfile	250

9.2.8.1	Description	250

9.2.8.2	Properties	250

9.2.8.3	Definition	251

9.2.8.4	Example	251

9.2.8.5	Additional Requirements	252

9.2.9	tosca.datatypes.nfv.VnfMonitoringParameter	252

9.2.9.1	Description	252

9.2.9.2	Properties	252

9.2.9.3	Definition	252

9.2.9.4	Examples	253

9.2.9.5	Additional Requirements	253

9.3	Artifact Types	253

9.4	Capabilities Types	253

9.4.1	tosca.capabilities.nfv.VirtualLinkable	253

9.4.1.1	Description	253

9.4.1.2	Properties	253

9.4.1.3	Definition	254

9.4.2	Void	254

9.5	Requirements Types	254

9.6	Relationship Types	254

9.6.1	tosca.relationships.nfv.VirtualLinksTo	254

9.6.1.1	Description	254

9.6.1.2	Properties	254

9.6.1.3	Definition	254

9.6.2	Void	255

9.6.3	tosca.relationships.nfv.VipVirtualLinksTo	255

9.6.3.1	Description	255

9.6.3.2	Properties	255

9.6.3.3	Definition	255

9.7	Interface Types	255

9.8	Node Types	255

9.8.1	tosca.nodes.nfv.Cp	255

9.8.1.1	Description	255

9.8.1.2	Properties	255

9.8.1.3	Attributes	256

9.8.1.4	Requirements	256

9.8.1.5	Capabilities	256

9.8.1.6	Definition	256

9.8.1.7	Additional requirements	257

9.9	Group Types	257

9.10	Policy Types	257

9.10.1	tosca.policies.nfv.Abstract.SecurityGroupRule	257

9.10.1.1	Description	257

9.10.1.2	Properties	258

9.10.1.3	Definition	258

9.10.1.4	Additional Requirements	260

Annex A (informative): Examples	261

A.1	Deployment flavour design mapping	261

A.1.1	Introduction	261

A.1.2	Design principle for VNF deployment flavour	261

A.1.3	Design principle for NS deployment flavour	262

A.2	VNFD with deployment flavour modelling design example	262

A.3	VNF external connection point	272

A.3.1	General	272

A.3.2	External connection point re-exposing an internal connection point	272

A.3.3	External connection point connected to an internal virtual link	276

A.4	VNFD modelling design example by using TOSCA composition	277

A.5	VNFD with Single deployment flavour modelling design example	282

A.6	Scaling and Instantiation Level examples	286

A.6.1	ScalingAspect and InstantiationLevels policies with uniform delta	286

A.6.2	ScalingAspect and InstantationLevels policies with non-uniform deltas	292

A.7	Service Access Point	296

A.7.1	General	296

A.7.2	VNF External connection point exposing as a SAP	297

A.7.3	SAP connected to an NS virtual link	298

A.8	NSD with Single deployment flavour modelling design example	300

A.9	Mapping between NFV IM and TOSCA concepts	303

A.9.1	Introduction	303

A.9.2	Mapping between ETSI GS NFV-IFA 011 IM and TOSCA concepts	303

A.9.3	Mapping between ETSI GS NFV-IFA 014 IM and TOSCA concepts	305

A.10	PNFD modelling design example	306

A.11	NSD with Multiple deployment flavour modelling design example	307

A.12	NSD with nested NS design example	313

A.13	Virtual IP address connection point	319

A.14	NSD VNF Forwarding Graph design example	325

A.15	Auto-scale and auto-heal design with the use of VNF indicators	334

Annex B (normative): etsi_nfv_sol001_type definitions	346

B.1	Purpose	346

B.2	VNFD type definitions file	346

B.3	NSD type definitions file	346

B.4	PNFD type definitions file	347

B.5	Common type definitions file	347

Annex C (normative): Conformance	348

C.1	Purpose	348

C.2	NFV TOSCA YAML service template	348

C.3	NFV TOSCA processor	349

Annex D (informative): Mapping between properties of TOSCA types and API attributes	350

D.1	Introduction	350

D.2	VNFD-related constructs	350

D.3	NSD-related constructs	358

Annex E (informative): TOSCA Imperative workflows	363

E.1	Purpose	363

E.2	TOSCA Imperative workflows for the NSD	363

E.2.1	Introduction	363

E.2.2	Definition of an NS workflow	363

E.2.3	Examples	364

Annex F (informative): Non-Backward Compatible Changes in the GS	369

F.1	Introduction	369

F.2	Non-Backward Compatible changes between version 2.7.1 and 2.6.1	369

F.3	Non-Backward Compatible changes between version 2.8.1 and 3.3.1	369

Annex G (informative): Change History	371

History	375

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org/).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This Group Specification (GS) has been produced by ETSI Industry Specification Group (ISG) Network Functions Virtualisation (NFV).

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# 1	Scope

The present document specifies a data model for NFV descriptors, using the TOSCA-Simple-Profile-YAML-v1.3 [20], fulfilling the requirements specified in ETSI GS NFV-IFA 011 [1] and ETSI GS NFV-IFA 014 [2] for a Virtualised Network Function Descriptor (VNFD), a Network Service Descriptor (NSD) and a Physical Network Function Descriptor (PNFD). The present document also specifies requirements on the VNFM and NFVO specific to the handling of NFV descriptors based on the TOSCA-Simple-Profile-YAML-v1.3 [20].

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference/.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ETSI GS NFV-IFA 011: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; VNF Descriptor and Packaging Specification".

[2]	ETSI GS NFV-IFA 014: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Network Service Templates Specification".

[3]	Void.

[4]	Void.

[5]	Private Enterprise Numbers registry at IANA.

NOTE:	Available at https://www.iana.org/assignments/enterprise-numbers/enterprise-numbers.

[6]	IETF RFC 5234: "Augmented BNF for Syntax Specifications: ABNF".

[7]	ETSI GS NFV-IFA 027: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Performance Measurements Specification".

[8]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[9]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[10]	ISO 3166 (all parts): "Codes for the representation of names of countries and their subdivisions".

[11]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

NOTE:	Available at https://tools.ietf.org/html/rfc4776.

[12]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[13]	IETF RFC 5646: "Tags for Identifying Languages".

[14]	Hash Function Textual Names registry at IANA.

NOTE:	Available at https://www.iana.org/assignments/hash-function-text-names.

[15]	The Open Group Base Specifications Issue 7, 2018 edition IEEETM Std 1003.1-2017 (Revision of IEEETM Std 1003.1-2008).

NOTE:	Available at http://pubs.opengroup.org/onlinepubs/9699919799/basedefs/V1_chap09.html.

[16]	IEEETM 802.1Q-2014: "IEEE Standard for Local and metropolitan area networks - Bridges and Bridged Networks".

[17]	IETF RFC 791: "Internet Protocol".

NOTE:	Available at https://tools.ietf.org/html/rfc791.

[18]	IETF RFC 8200: "Internet Protocol, Version 6 (IPv6) Specification".

NOTE:	Available at https://tools.ietf.org/html/rfc8200.

[19]	IANA: "Assigned Internet Protocol Numbers".

NOTE:	Available at https://www.iana.org/assignments/protocol-numbers/protocol-numbers.xhtml.

[20]	TOSCA-Simple-Profile-yaml-v1.3: "TOSCA Simple Profile in YAML Version 1.3".

[21]	IETF RFC 6225: "Dynamic Host Configuration Protocol Options for Coordinate-Based Location Configuration Information".

NOTE:	Available at https://tools.ietf.org/html/rfc6225.

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ETSI GS NFV-IFA 007: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Or-Vnfm reference point - Interface and Information Model Specification".

[i.2]	ETSI GS NFV 003: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[i.3]	Void.

[i.4]	ETSI GS NFV-SOL 002: "Network Functions Virtualisation (NFV) Release 3; Protocols and Data Models; RESTful protocols specification for the Ve-Vnfm Reference Point".

[i.5]	Void.

[i.6]	ETSI GS NFV-SOL 004: "Network Functions Virtualisation (NFV) Release 3; Protocols and Data Models; VNF Package and PNFD Archive specification".

[i.7]	Mistral Workflow Language v2 specification.

NOTE:	Available at https://docs.openstack.org/mistral/latest/user/wf_lang_v2.html.

[i.8]	ETSI GS NFV-IFA 013: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Os-Ma-Nfvo reference point - Interface and Information Model Specification".

[i.9]	ETSI GS NFV-SOL 003: "Network Functions Virtualisation (NFV) Release 3; Protocols and Data Models; RESTful protocols specification for the Or-Vnfm Reference Point".

[i.10]	ETSI GS NFV-SOL 005: "Network Functions Virtualisation (NFV) Release 3; Protocols and Data Models; RESTful protocols specification for the Os-Ma-nfvo Reference Point".

[i.11]	ETSI GS NFV-SOL 007: "Network Functions Virtualisation (NFV) Release 3; Protocols and Data Models; Network Service Descriptor File Structure Specification".

[i.12]	OpenStack® documentation: "Disk and container formats for images".

NOTE 1:	Available at https://docs.openstack.org/glance/pike/user/formats.html.

NOTE 2:	The OpenStack® Word Mark and OpenStack Logo are either registered trademarks/service marks or trademarks/service marks of the OpenStack Foundation, in the United States and other countries and are used with the OpenStack Foundation's permission. ETSI is not affiliated with, endorsed or sponsored by the OpenStack Foundation, or the OpenStack community.

[i.13]	Openstack Metadata service.

NOTE:	Available at https://docs.openstack.org/nova/rocky/user/metadata-service.html.

[i.14]	Openstack User-data service.

NOTE:	Available at https://docs.openstack.org/nova/rocky/user/user-data.html.

[i.15]	Openstack Personality service.

NOTE:	Available at http://www.openstacknetsdk.org/docs/html/T_net_openstack_Core_Domain_Personality.htm.

[i.16]	ETSI NFV registry of VimConnectionInfo information.

NOTE:	Available at http://register.etsi.org/NFV.

[i.17]	IETF RFC 4090: "Fast Reroute Extensions to RSVP-TE for LSP Tunnels".

[i.18]	TOSCA-Simple-Profile-yaml-v1.2: "TOSCA Simple Profile in YAML Version 1.2".

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in ETSI GS NFV 003 [i.2] and the following apply:

TOSCA interface type: reusable entity that describes a set of TOSCA operations that can be included as part of a Node type or Relationship Type definition

NOTE:	See TOSCA-Simple-Profile-YAML-v1.3 [20].

TOSCA operation: behavioural lifecycle procedure in a TOSCA node or relationship definition that can be invoked by an orchestration engine, whose implementation definition can be provided in the service template as part of a node template definition or a relationship template definition, or rely on an implementation of the operation built in the orchestration engine

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in ETSI GS NFV 003 [i.2] apply.

# 4	Overview of TOSCA model

TOSCA (Topology and Orchestration Specification for Cloud Applications) is a modelling language for describing the components of a cloud application and their relationships. TOSCA uses the concept of service templates to describe cloud workloads. TOSCA further provides means of associating standard or user-defined lifecycle operations to a cloud application component or to a relationship between components. The present document is based on TOSCA-Simple-Profile-YAML-v1.3 [20], which describes a YAML rendering for TOSCA.

# 5	General concept of using TOSCA to model NFV descriptors

## 5.1	Introduction

An NFV deployment template is modelled by using one or more TOSCA service template as defined in TOSCA-Simple-Profile-YAML-v1.3 [20].

Three main deployment templates are identified in the present document:

The Virtualised Network Function Descriptor (VNFD).

The Network Service Descriptor (NSD).

The Physical Network Function Descriptor (PNFD).

## 5.2	Network Service Descriptor

The Network Service Descriptor (NSD) is a deployment template which consists of information used by the NFVO for lifecycle management of an NS as defined in ETSI GS NFV-IFA 014 [2]. The NSD:

References zero, one or more Virtualised Network Function Descriptors (VNFD).

References zero, one or more Physical Network Functions Descriptors (PNFD).

References zero, one or more nested NSD.

Includes zero, one or more Virtual Link Descriptors (VLD).

Includes zero, one or more VNF Forwarding Graph Descriptors (VNFFGD).

A VNFFGD describes a topology of the Network Service or a portion of the Network Service.

A VLD describes the resource for deploying and managing the lifecycle of virtual links between the constituents of an NS.

A PNFD describes the connectivity requirements to integrate PNFs in an NS.

A nested NSD is an NSD from which a nested NS can be instantiated within a parent NS instance.

## 5.3	Virtualised Network Function Descriptor

The VNFD is a component of a VNF package. It is used by both the NFVO and the VNFM.

A VNFD is a deployment template which describes a VNF in terms of deployment and operational behaviour requirements. It also contains Virtualised Deployment Units (VDUs), internal virtual link descriptors, external connection point descriptors, software image descriptors, and deployment flavour descriptors, as defined in ETSI GS NFV-IFA 011 [1].

A VNFD contains the following main pieces of information, as shown in figure 5.3-1:

Virtualisation Deployment Unit (VDU) is a construct supporting the description of the deployment and operational behaviour of a VNF Component (VNFC). A VNFC instance created based on the VDU maps to a single virtualisation container (e.g. a VM). AVDU describes the resources needed to deploy and manage the lifecycle of a VNFC. A VDU includes internal Connection Point Descriptors (CPDs) that describe internal connection points that can either be used to connect a VNFC to an internal virtual link or be re-exposed outside the VNF as external connection points.

External CPD: describes an external connection point of a VNF, where either an internal connection point of a VDU is exposed as external connection point or the external connection point is directly connected to an internal virtual link.

Internal VLD: describes the resource requirements for deploying and managing the lifecycle of virtual links between one or more VNFC instances created based on one or more VDUs.

Figure 5.3-1: Overview of VNF descriptor

The information within a VNFD is structured according to one or more VNF deployment flavours (VnfDf) that specify different deployment configuration of a VNF, in terms of its internal topology and resource needs.

## 5.4	Physical Network Function Descriptor

The Physical Network Function Descriptor (PNFD) information element is a deployment template enabling on-boarding PNFs and referencing them from an NSD. It focuses on connectivity aspects only.

## 5.5	tosca_definitions_version and Namespace prefix

The "tosca_definitions_version" keyword when used in the present document shall comply with the definition as specified in section 3.1.2 of TOSCA-Simple-Profile-YAML-v1.3 [20] with the associated Namespace Alias value defined in the TOSCA-Simple-Profile-YAML-v1.3 [20].

NOTE 1:	This implies that service templates complying to the present document can only import service templates that reference the same version - and thus use the same grammar - as the importing service template. This is a restriction compared to the TOSCA-Simple-Profile-YAML-v1.3 specification [20].

NOTE 2:	As specified in TOSCA-Simple-Profile-YAML-v1.3 [20], the grammar used in TOSCA-Simple-Profile-YAML-v1.2 [i.18] is still supported with deprecation. The present document indicates the cases in which support with deprecation is applicable.

Table 5.5-1 defines the TOSCA Namespace prefix that shall be used to declare the namespace of all the TOSCA types as specified in the present document.

Table 5.5-1

| Namespace Prefix | Specification Description |
| --- | --- |
| toscanfv | The TOSCA namespace prefix of all the TOSCA types as specified in the present document. |

## 5.6	Imports statement

### 5.6.1	VNFD TOSCA service template

A VNFD TOSCA service template as specified in clause 6.11 shall include a TOSCA import definition referencing the following files:

The file defined in clause B.2 that includes all the type definitions from clause 6 of the present document.

Others, as described in clause 6.11.

As specified in TOSCA-Simple-Profile-YAML-v1.3 [20], the import statement can use a single-line or multi-line grammar.

The single-line grammar supports the import of single or multiple uniquely named VNFD types and other template definition files.

EXAMPLE 1:

imports: - https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_vnfd_types.yaml - any_other_files.yaml - custom_vnfd_datatypes_extension.yaml

The multi-line grammar also supports the import of single or multiple uniquely named VNFD types and other template definition files. The "file" keyword is a mandatory parameter in this grammar.

EXAMPLE 2:

imports: - file: https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_vnfd_types.yaml - file: any_other_files.yaml - file: custom_vnfd_datatypes_extension.yaml

### 5.6.2	NSD TOSCA service template

An NSD TOSCA service template as specified in clause 7.11 shall include a TOSCA import definition referencing the following files:

The file defined in clause B.3 that includes all the type definitions from clause 7 of the present document.

Others, as described in clause 7.11.

As specified in TOSCA-Simple-Profile-YAML-v1.3 [20], the import statement can use a single-line or multi-line grammar.

The single-line grammar supports the import of single or multiple uniquely named NSD types and other template definition files.

EXAMPLE 1:

imports: - https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_nsd_types.yaml - any_other_files.yaml - custom_nsd_node_types_extension.yaml

The multi-line grammar also supports the import of single or multiple uniquely named NSD types and other template definition files. The "file" keyword is a mandatory parameter in this grammar.

EXAMPLE 2:

imports: - file: https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_nsd_types.yaml - file: any_other_files.yaml - file: custom_nsd_node_types_extension.yaml

### 5.6.3	PNFD TOSCA service template

A PNFD TOSCA service template as specified in clause 8.11 shall include a TOSCA import definition referencing the following files:

The file defined in clause B.4 that includes all the type definitions from clause 8 of the present document.

Others, as described in clause 8.11.

As specified in TOSCA-Simple-Profile-YAML-v1.3 [20], the import statement can use a single-line or multi-line grammar.

The single-line grammar supports the import of single or multiple uniquely named PNFD types and other template definition files.

EXAMPLE 1:

imports: - https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_pnfd_types.yaml - any_other_files.yaml - custom_pnfd_node_types_extension.yaml

The multi-line grammar also supports the import of single or multiple uniquely named PNFD types and other template definition files. The "file" keyword is a mandatory parameter in this grammar.

EXAMPLE 2:

imports: - file: https://forge.etsi.org/rep/nfv/SOL001/raw/v2.6.1/etsi_nfv_sol001_pnfd_types.yaml - file: any_other_files.yaml - file: custom_pnfd_node_types_extension.yaml

## 5.7	Type extension

### 5.7.1	Introduction

Type extension is used when VNF-specific type information is introduced in the VNFD (e.g. modifiable attributes, configurable properties and additional parameters to LCM operations) or NSD (e.g. additional parameters to LCM operations).

### 5.7.2	Rules

Type extension may be applied to NFV types defined in the present document within the limits specified in table 5.7.3-1 and table 5.7.4-1, adhering to the following rule.

A derived type shall extend the base type in such a way that it remains substitutable for the base type with the following requirements:

- New properties and attributes may be introduced with no restriction within the limits specified in table 5.7.3-1 and table 5.7.4-1.

- Existing properties may be extended according to the following rules:

A scalar property shall not be extended to another type (e.g. a string property shall not be replaced with an integer property or with a complex property of the same name).

A complex property of data type "X" may only be extended to a property of type "Y" where "Y" is derived from "X" according to the present rules (recursive rule: present rules applied to each property of the derived data type.

A property of type list with entry schema "X" may only be extended to a list with entry schema "Y" where "Y" is an extension of "X" according to the present rules (recursive rule: present rules applied to the elements of the list).

A property of type map with entry schema "X" may only be extended to a map with entry schema "Y" where "Y" is an extension of "X" according to the present rules (recursive rule: present rules applied to the values of the map).

In general, the above rules apply to introducing/extending other elements beyond properties such as capabilities, requirements, interfaces, operations, inputs, etc. as well.

### 5.7.3	VNFD Types

Table 5.7.3-1 specifies the extension point where VNFD author may extend the pre-defined types.

Table 5.7.3-1: VNFD type extension points

| Type | Keyname | Property name |
| --- | --- | --- |
| tosca.nodes.nfv.VNF | properties | modifiable_attributes (as a new property) configurable_properties (as a new property). See note 4. |
|  | requirements | New requirements with capability type VirtualLinkable (as new requirements). |
|  | interfaces | Vnflcm.{operation_name}.inputs.additional_parameters (as a new property) Vnflcm.{operation_name}.inputs (as new properties). VnfIndicator.notifications (as new notifications). See note 4. |
|  | attributes | One attribute of primitive type per VNF indicator may be added. One attribute of type integer per scaling aspect may be added. It holds the value of the current scale level. |
| tosca.nodes.nfv.Vdu.Compute | properties | configurable_properties (as a new property). See note 1 and note 4. |
| tosca.datatypes.nfv.VnfInfoModifiableAttributes | properties | extensions (as a new property) metadata (as a new property). See note 2 and note 4. |
| tosca.datatypes.nfv.VnfConfigurableProperties | properties | additional_configurable_properties (as a new property). See note 2 and note 4. |
| tosca.datatypes.nfv.VnfcConfigurableProperties | properties | additional_vnfc_configurable_properties (as a new property). See note 2 and note 4. |
| tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions | properties | (new properties). See note 2 and note 4. |
| tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata | properties | (new properties). See note 2 and note 4. |
| tosca.datatypes.nfv.VnfAdditionalConfigurableProperties | properties | (new properties). See note 2 and note 4. |
| tosca.datatypes.nfv.VnfAdditionalVnfcConfigurableProperties | properties | (new properties). See note 2 and note 4. |
| tosca.datatypes.nfv.VnfOperationAdditionalParameters | properties | (new properties). See note 2 and note 4. |
| tosca.interfaces.nfv.VnfIndicator | notifications | one notification may be added per Vnf indicator. See note 3. |
| tosca.interfaces.nfv.ChangeCurrentVnfPackage | operations | {change_current_package_script} (as new operation). {change_current_package_script}.inputs.additional_parameters (as new property). |
| NOTE 1: VNF specific Vdu.Compute node types should be given names starting by the provider name followed by a dot (".") in order to avoid collisions if these node types are imported in an NSD service template. The provider name may, but need not, be identical to the value of the provider property of the VNF node type. Furthermore, it is the VNF provider's responsibility to ensure the uniqueness of the names of its Vdu.Compute node types, i.e. the Vdu.Compute node type names starting with its provider name. NOTE 2: VNF specific extension datatypes should be given names starting by the provider name followed by a dot (".") in order to avoid collisions when importing these datatypes in an NSD service template. The provider name may, but need not, be identical to the value of the provider property of the VNF node type. Furthermore, it is the VNF provider's responsibility to ensure the uniqueness of the names of its datatypes, i.e. the datatype names starting with its provider name. NOTE 3: VNF specific interface types should be given names starting by the provider name followed by a dot (".") in order to avoid collisions when importing these types in an NSD service template. The provider name may, but need not, be identical to the value of the provider property of the VNF node type. Furthermore, it is the VNF provider's responsibility to ensure the uniqueness of the names of its interface types, i.e. the interface type names starting with its provider name. NOTE 4: If a property is defined with a required value equal to false, the default value shall not be present in VNFD. This also applies to any new datatypes introduced in the VNFD. |  |  |

### 5.7.4	NSD types

Table 5.7.4-1 specifies the extension points where NSD author may extend the pre-defined types.

Table 5.7.4-1: NSD type extension points

| Type | Keyname | Property name |
| --- | --- | --- |
| tosca.nodes.nfv.NS | requirements | New requirements with capability type VirtualLinkable (as new requirements). |
|  | interfaces | Nslcm.{operation_name}.inputs.additional_parameters (as a new property) |
| tosca.datatypes.nfv.NsOperationAdditionalParameters | properties | (new properties). See note. |
| NOTE: If a property is defined with a required value equal to false, the default value shall not be present in NSD. This also applies to any new datatypes introduced in the NSD. |  |  |

### 5.7.x	Security-sensitive properties in extended data types

The definition of the properties of some of the data types derived from the data types specified in the present document may include the following metadata:

sensitive: ”true”

NOTE: double quotes are needed to avoid that the parser interprets it as the Boolean value true.

Editor’s note: It is FFS whether certain properties defined in SOL001 may also be tagged as security-sensitive.

This metadata indicates that the property holds security-sensitive information (e.g. passwords).

It is out of the scope of the present document to specify the exact behaviour of a functional block handling security-sensitive properties. The intent of this metadata is to signal not to expose the value of the property by means such as user interfaces, logging files, programmatic interfaces, etc.

Editor’s note: Specific handling for these properties will be defined in the affected API specifications, e.g. ETSI GS NFV-SOL 003 [i.9].

Extension of the types listed in table 5.7.x-1 may include properties with this metadata.

Table 5.7.x-1: VNFD and NSD extensible data types

Data type

tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions

tosca.datatypes.nfv.VnfAdditionalConfigurableProperties

tosca.datatypes.nfv.VnfAdditionalVnfcConfigurableProperties

tosca.datatypes.nfv.VnfOperationAdditionalParameters

tosca.datatypes.nfv.NsOperationAdditionalParameters

Editor’s note: It should be clarified whether to add a requirement that “No value shall be assigned in the VNFD to a property tagged with sensitive: ”true” metadata.”

## 5.8	Non-Backward Compatible changes

Annex F provides the list of non-backward compatible changes during the development of the present document.

# 6	VNFD TOSCA model

## 6.1	Introduction

The VNFD information model specified by ETSI GS NFV-IFA 011 [1] is mapped to the TOSCA concepts. The VNFD is represented as one or more TOSCA service templates to be used by the VNFM for deploying and managing the lifecycle of a VNF instance.

Table 6.1-1 describes the mapping of the main information elements defined in ETSI GS NFV-IFA 011 [1] applicable to a VNFD and the corresponding NFV-specific TOSCA Types, as well the basic TOSCA types defined in TOSCA-Simple-Profile-YAML-v1.3 [20] from which they are derived from. The full definition of all types can be found in the following clauses.

NOTE:	The autoScale feature specified in ETSI GS NFV-IFA 011 [1] is not supported in this version of the present document.

Table 6.1-1: Mapping of ETSI GS NFV-IFA 011 [1] information elements with TOSCA types

| ETSI GS NFV-IFA 011 [1] Elements | VNFD TOSCA types | Derived from |
| --- | --- | --- |
| VNFD | tosca.nodes.nfv.VNF | tosca.nodes.Root |
| Vdu | n/a (see note 1) | n/a |
| Cpd (Connection Point) | tosca.nodes.nfv.Cp | tosca.nodes.Root |
| VduCpd (internal connection point) | tosca.nodes.nfv.VduCp | tosca.nodes.nfv.Cp |
| VnfVirtualLinkDesc (Virtual Link) | tosca.nodes.nfv.VnfVirtualLink | tosca.nodes.Root |
| VnfExtCpd (External Connection Point) | tosca.nodes.nfv.VnfExtCp tosca.nodes.nfv.VduCp | tosca.nodes.nfv.Cp |
| VirtualStorageDesc | tosca.nodes.nfv.Vdu.VirtualBlockStorage tosca.nodes.nfv.Vdu.VirtualObjectStorage tosca.nodes.nfv.Vdu.VirtualFileStorage | tosca.nodes.Root |
| VirtualComputeDesc | tosca.nodes.nfv.Vdu.Compute | tosca.nodes.Root |
| SwImageDesc | tosca.artifacts.nfv.SwImage | tosca.artifacts.Deployment.Image |
| VnfDf | n/a (see note 2) | n/a |
| SecurityGroupRule | tosca.policies.nfv.SecurityGroupRule | tosca.policies.Root |
| VnfConfigurableProperties | tosca.datatypes.nfv.VnfConfigurableProperties | tosca.datatypes.Root |
| VnfInfoModifiableAttributes | tosca.datatypes.nfv.VnfInfoModifiableAttributes | tosca.datatypes.Root |
| NOTE 1: The Vdu information element is represented as a collection of tosca.nodes.nfv.VduCp, tosca.nodes.nfv.Vdu.Compute, tosca.nodes.nfv.Vdu.VirtualBlockStorage, tosca.nodes.nfv.Vdu.VirtualObjectStorage and tosca.nodes.nfv.Vdu.VirtualFileStorage types. NOTE 2: The VnfDf information element is represented as a TOSCA service template. |  |  |

Figure 6.1-1 provides an overview of the TOSCA node types used to build a service template representing a VNFD for a specific deployment flavour, and of the relationship between them. The figure shows one of the three types of virtual storage. A detailed description is provided in clause 6.11.

Figure 6.1-1: Service template VNFD overview

## 6.2	Data Types

### 6.2.1	tosca.datatypes.nfv.CpProtocolData

#### 6.2.1.1	Description

The CpProtocolData data type is defined in clause 9.2.6 of the present document.

### 6.2.2	tosca.datatypes.nfv.AddressData

#### 6.2.2.1	Description

The AddressData data type is defined in clause 9.2.3 of the present document.

### 6.2.3	tosca.datatypes.nfv.L2AddressData

#### 6.2.3.1	Description

The L2AddressData data type is defined in clause 9.2.1 of the present document.

### 6.2.4	tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements

#### 6.2.4.1	Description

The VirtualNetworkInterfaceRequirements data type describes requirements on a virtual network interface, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.4.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.4.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualNetworkInterfaceRequirements |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualNetworkInterfaceRequirements |
| Type URI | tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements |

#### 6.2.4.2	Properties

The properties of the VirtualNetworkInterfaceRequirements data type shall comply with the provisions set out in table 6.2.4.2-1.

Table 6.2.4.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Provides a human readable name for the requirement. |
| description | no | string |  | Provides a human readable description of the requirement. |
| support_mandatory | yes | boolean |  | Indicates whether fulfilling the constraint is mandatory (TRUE) for successful operation or desirable (FALSE). |
| network_interface_requirements | yes | map of string |  | The network interface requirements. A map of strings that contain a set of key-value pairs that describes the hardware platform specific network interface deployment requirements. More information regarding the usage of this property is available at: https://register.etsi.org. |
| nic_io_requirements | no | tosca.datatypes.nfv.LogicalNodeData |  | This references (couples) the CP with any logical node I/O requirements (for network devices) that may have been created. Linking these attributes is necessary so that so that I/O requirements that need to be articulated at the logical node level can be associated with the network interface requirements associated with the CP. |

#### 6.2.4.3	Definition

The syntax of the VirtualNetworkInterfaceRequirements data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements: derived_from: tosca.datatypes.Root description: Describes requirements on a virtual network interface properties: name: type: string description: Provides a human readable name for the requirement. required: false description: type: string description: Provides a human readable description of the requirement. required: false support_mandatory: type: boolean description: Indicates whether fulfilling the constraint is mandatory (TRUE) for successful operation or desirable (FALSE). required: true network_interface_requirements: type: map description: The network interface requirements. A map of strings that contain a set of key-value pairs that describes the hardware platform specific network interface deployment requirements. required: true entry_schema: type: string nic_io_requirements: type: tosca.datatypes.nfv.LogicalNodeData description: references (couples) the CP with any logical node I/O requirements (for network devices) that may have been created. Linking these attributes is necessary so that so that I/O requirements that need to be articulated at the logical node level can be associated with the network interface requirements associated with the CP. required: false

#### 6.2.4.4	Examples

None.

#### 6.2.4.5	Additional Requirements

None.

### 6.2.5	tosca.datatypes.nfv.L3AddressData

#### 6.2.5.1	Description

The L3AddressData data type is defined in clause 9.2.2 of the present document.

### 6.2.6	tosca.datatypes.nfv.RequestedAdditionalCapability

#### 6.2.6.1	Description

The RequestedAdditionalCapability data type describes requested additional capability for a particular VDU, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.6.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.6.1-1: Type name, shorthand, and URI

| Shorthand Name | RequestedAdditionalCapability |
| --- | --- |
| Type Qualified Name | toscanfv:RequestedAdditionalCapability |
| Type URI | tosca.datatypes.nfv.RequestedAdditionalCapability |

#### 6.2.6.2	Properties

The properties of the RequestedAdditionalCapability data type shall comply with the provisions set out in table 6.2.6.2-1.

Table 6.2.6.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| requested_additional_capability_name | yes | string |  | Identifies a requested additional capability for the VDU. |
| support_mandatory | yes | boolean |  | Indicates whether the requested additional capability is mandatory for successful operation. |
| min_requested_additional_capability_version | no | string |  | Identifies the minimum version of the requested additional capability. |
| preferred_requested_additional_capability_version | no | string |  | Identifies the preferred version of the requested additional capability. |
| target_performance_parameters | yes | map of string |  | Identifies specific attributes, dependent on the requested additional capability type. |

#### 6.2.6.3	Definition

The syntax of the RequestedAdditionalCapability data type shall comply with the following definition:

tosca.datatypes.nfv.RequestedAdditionalCapability: derived_from: tosca.datatypes.Root description: describes requested additional capability for a particular VDU properties: requested_additional_capability_name: type: string description: Identifies a requested additional capability for the VDU. required: true support_mandatory: type: boolean description: Indicates whether the requested additional capability is mandatory for successful operation. required: true min_requested_additional_capability_version: type: string description: Identifies the minimum version of the requested additional capability. required: false preferred_requested_additional_capability_version: type: string description: Identifies the preferred version of the requested additional capability. required: false target_performance_parameters: type: map description: Identifies specific attributes, dependent on the requested additional capability type. required: true entry_schema: type: string

#### 6.2.6.4	Examples

None.

#### 6.2.6.5	Additional Requirements

None.

### 6.2.7	tosca.datatypes.nfv.VirtualMemory

#### 6.2.7.1	Description

The VirtualMemory data type supports the specification of requirements related to virtual memory of a virtual compute resource, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.7.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.7.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualMemory |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualMemory |
| Type URI | tosca.datatypes.nfv.VirtualMemory |

#### 6.2.7.2	Properties

The properties of the VirtualMemory data type shall comply with the provisions set out in table 6.2.7.2-1.

Table 6.2.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_mem_size | yes | scalar-unit.size |  | Amount of virtual memory. |
| virtual_mem_oversubscription_policy | no | string |  | The memory core oversubscription policy in terms of virtual memory to physical memory on the platform. |
| vdu_mem_requirements | no | map of string |  | The hardware platform specific VDU memory requirements. A map of strings that contains a set of key-value pairs that describes hardware platform specific VDU memory requirements. More information regarding the usage of this property is available at: https://register.etsi.org. |
| numa_enabled | yes | boolean | default: false | It specifies the memory allocation to be cognisant of the relevant process/core allocation. |

#### 6.2.7.3	Definitions

The syntax of the VirtualMemory data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualMemory: derived_from: tosca.datatypes.Root description: supports the specification of requirements related to virtual memory of a virtual compute resource properties: virtual_mem_size: type: scalar-unit.size description: Amount of virtual memory. required: true virtual_mem_oversubscription_policy: type: string description: The memory core oversubscription policy in terms of virtual memory to physical memory on the platform. required: false vdu_mem_requirements: type: map description: The hardware platform specific VDU memory requirements. A map of strings that contains a set of key-value pairs that describes hardware platform specific VDU memory requirements. required: false entry_schema: type: string numa_enabled: type: boolean description: It specifies the memory allocation to be cognisant of the relevant process/core allocation. required: true default: false

#### 6.2.7.4	Examples

None.

#### 6.2.7.5	Additional Requirements

None.

### 6.2.8	tosca.datatypes.nfv.VirtualCpu

#### 6.2.8.1	Description

The VirtualCpu data type supports the specification of requirements related to virtual CPU(s) of a virtual compute resource, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.8.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.8.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualCpu |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualCpu |
| Type URI | tosca.datatypes.nfv.VirtualCpu |

#### 6.2.8.2	Properties

The properties of the VirtualCpu data type shall comply with the provisions set out in table 6.2.8.2-1.

Table 6.2.8.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| cpu_architecture | no | string |  | CPU architecture type. Examples are x86, ARM. |
| num_virtual_cpu | yes | integer | greater_than: 0 | Number of virtual CPUs. |
| virtual_cpu_clock | no | scalar-unit.frequency |  | Minimum virtual CPU clock rate. |
| virtual_cpu_oversubscription_policy | no | string |  | CPU core oversubscription policy e.g. the relation of virtual CPU cores to physical CPU cores/threads. |
| vdu_cpu_requirements | no | map of string |  | The hardware platform specific VDU CPU requirements. A map of strings that contains a set of key-value pairs describing VDU CPU specific hardware platform requirements. More information regarding the usage of this property is available at: https://register.etsi.org. |
| virtual_cpu_pinning | no | tosca.datatypes.nfv.VirtualCpuPinning |  | The virtual CPU pinning configuration for the virtualised compute resource. |

#### 6.2.8.3	Definition

The syntax of the VirtualCpu data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualCpu: derived_from: tosca.datatypes.Root description: Supports the specification of requirements related to virtual CPU(s) of a virtual compute resource properties: cpu_architecture: type: string description: CPU architecture type. Examples are x86, ARM required: false num_virtual_cpu: type: integer description: Number of virtual CPUs required: true constraints: - greater_than: 0 virtual_cpu_clock: type: scalar-unit.frequency description: Minimum virtual CPU clock rate required: false virtual_cpu_oversubscription_policy: type: string description: CPU core oversubscription policy e.g. the relation of virtual CPU cores to physical CPU cores/threads. required: false vdu_cpu_requirements: type: map description: The hardware platform specific VDU CPU requirements. A map of strings that contains a set of key-value pairs describing VDU CPU specific hardware platform requirements. required: false entry_schema: type: string virtual_cpu_pinning: type: tosca.datatypes.nfv.VirtualCpuPinning description: The virtual CPU pinning configuration for the virtualised compute resource. required: false

#### 6.2.8.4	Examples

None.

#### 6.2.8.5	Additional Requirements

None.

### 6.2.9	tosca.datatypes.nfv.VirtualCpuPinning

#### 6.2.9.1	Description

The VirtualCpuPinning data type supports the specification of requirements related to the virtual CPU pinning configuration of a virtual compute resource, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.9.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.9.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualCpuPinning |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualCpuPinning |
| Type URI | tosca.datatypes.nfv.VirtualCpuPinning |

#### 6.2.9.2	Properties

The properties of the VirtualCpuPinning data type shall comply with the provisions set out in table 6.2.9.2-1.

Table 6.2.9.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_cpu_pinning_policy | no | string | Valid values: static, dynamic | Indicates the policy for CPU pinning. The policy can take values of "static" or "dynamic". In case of "dynamic" the allocation of virtual CPU cores to logical CPU cores is decided by the VIM. (e.g. SMT (Simultaneous Multi-Threading) requirements). In case of "static" the allocation is requested to be according to the virtual_cpu_pinning_rule. |
| virtual_cpu_pinning_rule | no | list of string |  | Provides the list of rules for allocating virtual CPU cores to logical CPU cores/threads. |

#### 6.2.9.3	Definition

The syntax of the VirtualCpuPinning data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualCpuPinning: derived_from: tosca.datatypes.Root description: Supports the specification of requirements related to the virtual CPU pinning configuration of a virtual compute resource properties: virtual_cpu_pinning_policy: type: string description: Indicates the policy for CPU pinning. The policy can take values of "static" or "dynamic". In case of "dynamic" the allocation of virtual CPU cores to logical CPU cores is decided by the VIM. (e.g. SMT (Simultaneous Multi-Threading) requirements). In case of "static" the allocation is requested to be according to the virtual_cpu_pinning_rule. required: false constraints: - valid_values: [ static, dynamic ] virtual_cpu_pinning_rule: type: list description: Provides the list of rules for allocating virtual CPU cores to logical CPU cores/threads required: false entry_schema: type: string

#### 6.2.9.4	Examples

None.

#### 6.2.9.5	Additional Requirements

The virtual_cpu_pinning_rule shall be included if the virtual_cpu_pinning_policy property is set to "static" and shall be absent otherwise.

### 6.2.10	tosca.datatypes.nfv.VnfcConfigurableProperties

#### 6.2.10.1	Description

The VnfcConfigurableProperties data type defines the configurable properties of a VNFC, as defined in ETSI GS NFV-IFA 011 [1].

Table 6.2.10.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.10.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfcConfigurableProperties |
| --- | --- |
| Type Qualified Name | toscanfv:VnfcConfigurableProperties |
| Type URI | tosca.datatypes.nfv.VnfcConfigurableProperties |

#### 6.2.10.2	Properties

The properties of the VnfcconfigurableProperties shall comply with the provisions set out in table 6.2.10.2-1.

Table 6.2.10.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| additional_vnfc_configurable_properties | no | tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties |  | Describes additional configuration for VNFC that can be modified using the ModifyVnfInfo operation. |

#### 6.2.10.3	Definition

The syntax of the VnfcConfigurableProperties data type shall comply with the following definition:

tosca.datatypes.nfv.VnfcConfigurableProperties: derived_from: tosca.datatypes.Root description: Defines the configurable properties of a VNFC #properties: # additional_vnfc_configurable_properties: # type: tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties # description: Describes additional configuration for VNFC that can be modified using the ModifyVnfInfo operation # required: false # derived types are expected to introduce # additional_vnfc_configurable_properties with its type derived from # tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties

#### 6.2.10.4	Examples

tosca_definitions_version: tosca_simple_yaml_1_3 node_types: MyCompany.nodes.nfv.Vdu.Aux: derived_from: tosca.nodes.nfv.Vdu.Compute properties: configurable_properties: type: MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties data_types: MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties: derived_from: tosca.datatypes.nfv.VnfcConfigurableProperties properties: additional_vnfc_configurable_properties: type: MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties required: true MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties: derived_from: tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties properties: name_prefix_in_vim: type: string required: false dns_server: type: string required: true topology_template: .. node_templates: aux: type: MyCompany.nodes.nfv.Vdu.Aux properties: ..

#### 6.2.10.5	Additional Requirements

None.

### 6.2.11	tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties

#### 6.2.11.1	Description

The VnfcAdditionalConfigurableProperties type is an empty base type for deriving data types for describing additional configurable properties for a given VNFC. Table 6.2.11.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.11.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfcAdditionalConfigurableProperties |
| --- | --- |
| Type Qualified Name | toscanfv:VnfcAdditionalConfigurableProperties |
| Type URI | tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties |

#### 6.2.11.2	Properties

None.

#### 6.2.11.3	Definition

The syntax of the VnfcAdditionalConfigurableProperties data type shall comply with the following definition:

tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties: derived_from: tosca.datatypes.Root description: VnfcAdditionalConfigurableProperties type is an empty base type for deriving data types for describing additional configurable properties for a given VNFC.

### 6.2.12	tosca.datatypes.nfv.VduProfile

#### 6.2.12.1	Description

The VduProfile data type describes additional instantiation data for a given Vdu.Compute used in a specific deployment flavour. Table 6.2.12.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.12.1-1: Type name, shorthand, and URI

| Shorthand Name | VduProfile |
| --- | --- |
| Type Qualified Name | toscanfv:VduProfile |
| Type URI | tosca.datatypes.nfv.VduProfile |

#### 6.2.12.2	Properties

The properties of the VduProfile data type shall comply with the provisions set out in table 6.2.12.2-1.

Table 6.2.12.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| min_number_of_instances | yes | integer | greater_or_equal: 0 | Minimum number of instances of the VNFC based on this Vdu.Compute that is permitted to exist for a particular VNF deployment flavour. |
| max_number_of_instances | yes | integer | greater_or_equal: 0 | Maximum number of instances of the VNFC based on this Vdu.Compute that is permitted to exist for a particular VNF deployment flavour. |
| NOTE: A vduId property, which exists in ETSI GS NFV-IFA 011 [1] is not needed, as the VduProfile is contained in the Vdu.Compute node. |  |  |  |  |

#### 6.2.12.3	Definition

The syntax of the VduProfile data type shall comply with the following definition:

tosca.datatypes.nfv.VduProfile: derived_from: tosca.datatypes.Root description: describes additional instantiation data for a given Vdu.Compute used in a specific deployment flavour. properties: min_number_of_instances: type: integer description: Minimum number of instances of the VNFC based on this Vdu.Compute that is permitted to exist for a particular VNF deployment flavour. required: true constraints: - greater_or_equal: 0 max_number_of_instances: type: integer description: Maximum number of instances of the VNFC based on this Vdu.Compute that is permitted to exist for a particular VNF deployment flavour. required: true constraints: - greater_or_equal: 0

#### 6.2.12.4	Examples

tosca_definitions_version: tosca_simple_yaml_1_3 topology template: … node_templates: VDU_A: type: tosca.nodes.nfv.Vdu.Compute properties: vdu_profile: min_number_of_instances: 2 max_number_of_instances: 6 # other properties omitted for brevity requirements: - virtual_storage: VirtualStorage_A1 - virtual_storage: VirtualStorage_A2 capabilities: virtual_binding: VirtualStorage_A1: type: tosca.nodes.nfv.Vdu.VirtualBlockStorage properties: # omitted for brevity capabilities: virtual_storage VirtualStorage_A2: type: tosca.nodes.nfv.Vdu.VirtualBlockStorage properties: # omitted for brevity capabilities: virtual_storage VduCp_A1: type: tosca.nodes.nfv.VduCp properties: # omitted for brevity requirements: - virtual_binding: VDU_A - virtual_link VduCp_A2: type: tosca.nodes.nfv.VduCp properties: # omitted for brevity requirements: - virtual_binding: VDU_A - virtual_link

Above snippet shows part of a topology template. The VDU_A node template is a Vdu.Compute node that is connected to two VirtualBlockStorage nodes: VirtualStorage_A1 and VirtualStorage_A2. It also has two VduCps: VduCp_A1 and Vdu_CpA2.

The minimum number of instances of VDU_A that are permitted to exist is 2. Likewise, the minimum number of instances of VirtualStorage_A1, VirtualStorage_A2, VduCp_A1 and VduCp_A2 that are permitted to exist is 2.

The maximum number of instances of VDU_A that are permitted to exist is 6. Likewise, the maximum number of instances of VirtualStorage_A1, VirtualStorage_A2, VduCp_A1 and VduCp_A2 that are permitted to exist is 6.

#### 6.2.12.5	Additional requirements

The properties of the vdu_profile indicate the maximum and minimum number of Vdu.Compute instances that are permitted to exist, created from a given Vdu.Compute node template during its lifecycle, as well as:

The maximum and minimum number of instances of each VirtualBlockStorage, VirtualObjectStorage and VirtualFileStorage nodes connected to the Vdu.Compute via one particular occurrence of the virtual_storage requirement.

The maximum and minimum number instances of each VduCp node connected to the Vdu.Compute via one particular occurrence of the virtual_binding capability.

### 6.2.13	tosca.datatypes.nfv.VlProfile

#### 6.2.13.1	Description

The VlProfile data type describes additional instantiation data for a given VL used in a specific VNF deployment flavour. Table 6.2.13.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.13.1-1: Type name, shorthand, and URI

| Shorthand Name | VlProfile |
| --- | --- |
| Type Qualified Name | toscanfv:VlProfile |
| Type URI | tosca.datatypes.nfv.VlProfile |

#### 6.2.13.2	Properties

The properties of the VlProfile data type shall comply with the provisions set out in table 6.2.13.2-1.

Table 6.2.13.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| max_bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Specifies the maximum bitrate requirements for a VL instantiated according to this profile. |
| min_bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Specifies the minimum bitrate requirements for a VL instantiated according to this profile. |
| qos | no | tosca.datatypes.nfv.Qos |  | Specifies the QoS requirements of a VL instantiated according to this profile. |
| virtual_link_protocol_data | no | list of tosca.datatypes.nfv.VirtualLinkProtocolData |  | Specifies the protocol data for a virtual link. |
| NOTE: A vnfVirtualLinkDescId property, which exists in ETSI GS NFV-IFA 011 [1] is not needed, as the VLProfile is contained in the VL node. |  |  |  |  |

#### 6.2.13.3	Definition

The syntax of the VlProfile data type shall comply with the following definition:

tosca.datatypes.nfv.VlProfile: derived_from: tosca.datatypes.Root description: Describes additional instantiation data for a given VL used in a specific VNF deployment flavour. properties: max_bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Specifies the maximum bitrate requirements for a VL instantiated according to this profile. required: true min_bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Specifies the minimum bitrate requirements for a VL instantiated according to this profile. required: true qos: type: tosca.datatypes.nfv.Qos description: Specifies the QoS requirements of a VL instantiated according to this profile. required: false virtual_link_protocol_data: type: list description: Specifies the protocol data for a virtual link. required: false entry_schema: type: tosca.datatypes.nfv.VirtualLinkProtocolData

#### 6.2.13.4	Examples

None.

#### 6.2.13.5	Additional Requirements

None.

### 6.2.14	tosca.datatypes.nfv.VirtualLinkProtocolData

#### 6.2.14.1	Description

The VirtualLinkProtocolData data type describes one protocol layer and associated protocol data for a given virtual link used in a specific VNF deployment flavour. Table 6.2.14.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.14.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkProtocolData |
| Type URI | tosca.datatypes.nfv.VirtualLinkProtocolData |

#### 6.2.14.2	Properties

The properties of the VirtualLinkProtocolData data type shall comply with the provisions set out in table 6.2.14.2-1.

Table 6.2.14.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| associated_layer_protocol | yes | string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | Identifies one of the protocols a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire) as specified by the connectivity_type property. |
| l2_protocol_data | no | tosca.datatypes.nfv.L2ProtocolData |  | Specifies the L2 protocol data for a virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L2 protocol and shall be absent otherwise. |
| l3_protocol_data | no | tosca.datatypes.nfv.L3ProtocolData |  | Specifies the L3 protocol data for this virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L3 protocol and shall be absent otherwise. |

#### 6.2.14.3	Definition

The syntax of the VirtualLinkProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualLinkProtocolData: derived_from: tosca.datatypes.Root description: describes one protocol layer and associated protocol data for a given virtual link used in a specific VNF deployment flavour properties: associated_layer_protocol: type: string description: Identifies one of the protocols a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire) as specified by the connectivity_type property. required: true constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] l2_protocol_data: type: tosca.datatypes.nfv.L2ProtocolData description: Specifies the L2 protocol data for a virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L2 protocol and shall be absent otherwise. required: false l3_protocol_data: type: tosca.datatypes.nfv.L3ProtocolData description: Specifies the L3 protocol data for this virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L3 protocol and shall be absent otherwise. required: false

#### 6.2.14.4	Examples

None.

#### 6.2.14.5	Additional Requirements

None.

### 6.2.15	tosca.datatypes.nfv.L2ProtocolData

#### 6.2.15.1	Description

The L2ProtocolData data type escribes L2 protocol data for a given virtual link used in a specific VNF deployment flavour. Table 6.2.15.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.15.1-1: Type name, shorthand, and URI

| Shorthand Name | L2ProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:L2ProtocolData |
| Type URI | tosca.datatypes.nfv.L2ProtocolData |

#### 6.2.15.2	Properties

The properties of the L2ProtocolData data type shall comply with the provisions set out in table 6.2.15.2-1.

Table 6.2.15.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Identifies the network name associated with this L2 protocol. |
| network_type | no | string | Valid values: flat, vlan, vxlan, gre | Specifies the network type for this L2 protocol. The value may be overridden at run-time. |
| vlan_transparent | yes | boolean | default: false | Specifies whether to support VLAN transparency for this L2 protocol or not. |
| mtu | no | integer | greater_than : 0 | Specifies the maximum transmission unit (MTU) value for this L2 protocol. |
| segmentation_id | no | string |  | If present, specifies a specific virtualised network segment, which depends on the network type. For e.g. VLAN ID for VLAN network type and tunnel ID for GRE/VXLAN network types. See note. |
| NOTE: If this property is included in the VNFD, the property value shall be provided at run-time, unless a default value is provided at design time in the VNFD. If a default value is provided at design-time, this value may be overridden at run-time. |  |  |  |  |

#### 6.2.15.3	Definition

The syntax of the L2ProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.L2ProtocolData: derived_from: tosca.datatypes.Root description: describes L2 protocol data for a given virtual link used in a specific VNF deployment flavour. properties: name: type: string description: Identifies the network name associated with this L2 protocol. required: false network_type: type: string description: Specifies the network type for this L2 protocol. The value may be overridden at run-time. required: false constraints: - valid_values: [ flat, vlan, vxlan, gre ] vlan_transparent: type: boolean description: Specifies whether to support VLAN transparency for this L2 protocol or not. required: true default: false mtu: type: integer description: Specifies the maximum transmission unit (MTU) value for this L2 protocol. required: false constraints: - greater_than: 0 segmentation_id: type: string description: Specifies a specific virtualised network segment, which depends on the network type. For e.g., VLAN ID for VLAN network type and tunnel ID for GRE/VXLAN network types required: false

#### 6.2.15.4	Examples

See example in clause A.5.

#### 6.2.15.5	Additional Requirements

None.

### 6.2.16	tosca.datatypes.nfv.L3ProtocolData

#### 6.2.16.1	Description

The L3ProtocolData data type describes L3 protocol data for a given virtual link used in a specific VNF deployment flavour. Table 6.2.16.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.16.1-1: Type name, shorthand, and URI

| Shorthand Name | L3ProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:L3ProtocolData |
| Type URI | tosca.datatypes.nfv.L3ProtocolData |

#### 6.2.16.2	Properties

The properties of the L3ProtocolData data type shall comply with the provisions set out in table 6.2.16.2-1.

Table 6.2.16.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Identifies the network name associated with this L3 protocol. |
| ip_version | yes | string | Valid values: ipv4 ipv6 | Specifies IP version of this L3 protocol. The value of the ip_version property shall be consistent with the value of the layer_protocol in the connectivity_type property of the virtual link node. |
| cidr | yes | string |  | Specifies the CIDR (Classless Inter-Domain Routing) of this L3 protocol. The value may be overridden at run-time. |
| ip_allocation_pools | no | list of tosca.datatypes.nfv.IpAllocationPool |  | Specifies the allocation pools with start and end IP addresses for this L3 protocol. The value may be overridden at run-time. |
| gateway_ip | no | string |  | Specifies the gateway IP address for this L3 protocol. The value may be overridden at run-time. |
| dhcp_enabled | no | boolean |  | Indicates whether DHCP (Dynamic Host Configuration Protocol) is enabled or disabled for this L3 protocol. The value may be overridden at run-time. |
| ipv6_address_mode | no | string | Valid values: slaac dhcpv6-stateful dhcpv6-stateless | Specifies IPv6 address mode. May be present when the value of the ipVersion attribute is "ipv6" and shall be absent otherwise. The value may be overridden at run-time. |

#### 6.2.16.3	Definition

The syntax of the L3ProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.L3ProtocolData: derived_from: tosca.datatypes.Root description: describes L3 protocol data for a given virtual link used in a specific VNF deployment flavour. properties: name: type: string description: Identifies the network name associated with this L3 protocol. required: false ip_version: type: string description: Specifies IP version of this L3 protocol. The value of the ip_version property shall be consistent with the value of the layer_protocol in the connectivity_type property of the virtual link node. required: true constraints: - valid_values: [ ipv4, ipv6 ] cidr: type: string description: Specifies the CIDR (Classless Inter-Domain Routing) of this L3 protocol. The value may be overridden at run-time. required: true ip_allocation_pools: type: list description: Specifies the allocation pools with start and end IP addresses for this L3 protocol. The value may be overridden at run-time. required: false entry_schema: type: tosca.datatypes.nfv.IpAllocationPool gateway_ip: type: string description: Specifies the gateway IP address for this L3 protocol. The value may be overridden at run-time. required: false dhcp_enabled: type: boolean description: Indicates whether DHCP (Dynamic Host Configuration Protocol) is enabled or disabled for this L3 protocol. The value may be overridden at run-time. required: false ipv6_address_mode: type: string description: Specifies IPv6 address mode. May be present when the value of the ipVersion attribute is "ipv6" and shall be absent otherwise. The value may be overridden at run-time. required: false constraints: - valid_values: [ slaac, dhcpv6-stateful, dhcpv6-stateless ]

#### 6.2.16.4	Examples

None.

#### 6.2.16.5	Additional Requirements

None.

### 6.2.17	tosca.datatypes.nfv.IpAllocationPool

#### 6.2.17.1	Description

The IpAllocationPool data type specifies a range of IP addresses. Table 6.2.17.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.17.1-1: Type name, shorthand, and URI

| Shorthand Name | IpAllocationPool |
| --- | --- |
| Type Qualified Name | toscanfv:IpAllocationPool |
| Type URI | tosca.datatypes.nfv.IpAllocationPool |

#### 6.2.17.2	Properties

The properties of the IpAllocationPool data type shall comply with the provisions set out in table 6.2.17.2-1.

Table 6.2.17.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| start_ip_address | yes | string |  | The IP address to be used as the first one in a pool of addresses derived from the cidr block full IP range |
| end_ip_address | yes | string |  | The IP address to be used as the last one in a pool of addresses derived from the cidr block full IP range |

#### 6.2.17.3	Definition

The syntax of the IpAllocationPool data type shall comply with the following definition:

tosca.datatypes.nfv.IpAllocationPool: derived_from: tosca.datatypes.Root description: Specifies a range of IP addresses properties: start_ip_address: type: string description: The IP address to be used as the first one in a pool of addresses derived from the cidr block full IP range required: true end_ip_address: type: string description: The IP address to be used as the last one in a pool of addresses derived from the cidr block full IP range required: true

#### 6.2.17.4	Examples

None.

#### 6.2.17.5	Additional Requirements

None.

### 6.2.18	tosca.datatypes.nfv.InstantiationLevel

#### 6.2.18.1	Description

The InstantiationLevel data type describes the scale level for each aspect that corresponds to a given level of resources to be instantiated within a deployment flavour in term of the number VNFC instances . Table 6.2.18.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.18.1-1: Type name, shorthand, and URI

| Shorthand Name | InstantiationLevel |
| --- | --- |
| Type Qualified Name | toscanfv:InstantiationLevel |
| Type URI | tosca.datatypes.nfv.InstantiationLevel |

#### 6.2.18.2	Properties

The properties of the InstantiationLevel data type shall comply with the provisions set out in table 6.2.18.2-1.

Table 6.2.18.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | yes | string |  | Human readable description of the level. |
| scale_info | no | map of tosca.datatypes.nfv.ScaleInfo |  | Represents for each aspect the scale level that corresponds to this instantiation level. scale_info shall be present if the VNF supports scaling. |

#### 6.2.18.3	Definition

The syntax of the InstantiationLevel data type shall comply with the following definition:

tosca.datatypes.nfv.InstantiationLevel: derived_from: tosca.datatypes.Root description: Describes the scale level for each aspect that corresponds to a given level of resources to be instantiated within a deployment flavour in term of the number VNFC instances properties: description: type: string description: Human readable description of the level required: true scale_info: type: map # key: aspectId description: Represents for each aspect the scale level that corresponds to this instantiation level. scale_info shall be present if the VNF supports scaling. required: false entry_schema: type: tosca.datatypes.nfv.ScaleInfo

#### 6.2.18.4	Examples

See clause A.6.

#### 6.2.18.5	Additional Requirements

None.

### 6.2.19	tosca.datatypes.nfv.VduLevel

#### 6.2.19.1	Description

The VduLevel data type indicates for a given Vdu.Compute in a given level the number of instances to deploy. Table 6.2.19.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.19.1-1: Type name, shorthand, and URI

| Shorthand Name | VduLevel |
| --- | --- |
| Type Qualified Name | toscanfv:VduLevel |
| Type URI | tosca.datatypes.nfv.VduLevel |

#### 6.2.19.2	Properties

The properties of the VduLevel data type shall comply with the provisions set out in table 6.2.19.2-1.

Table 6.2.19.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| number_of_instances | yes | integer | greater_or_equal : 0 | Number of instances of VNFC based on this VDU to deploy for this level. |

#### 6.2.19.3	Definition

The syntax of the VduLevel data type shall comply with the following definition:

tosca.datatypes.nfv.VduLevel: derived_from: tosca.datatypes.Root description: Indicates for a given Vdu.Compute in a given level the number of instances to deploy properties: number_of_instances: type: integer description: Number of instances of VNFC based on this VDU to deploy for this level. required: true constraints: - greater_or_equal: 0

#### 6.2.19.4	Examples

See clause A.6.

#### 6.2.19.5	Additional Requirements

None.

### 6.2.20	tosca.datatypes.nfv.VnfLcmOperationsConfiguration

#### 6.2.20.1	Description

The VnfLcmOperationsConfiguration data type represents information to configure lifecycle management operations as specified in ETSI GS NFV-IFA 007 [i.1]. Each VNF LCM operations configuration property represents a container for all attributes that affect the invocation of the corresponding VNF Lifecycle Management operation. Table 6.2.20.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.20.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfLcmOperationsConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfLcmOperationsConfiguration |
| Type URI | tosca.datatypes.nfv.VnfLcmOperationsConfiguration |

#### 6.2.20.2	Properties

The properties of the VnfLcmOperationsConfiguration data type shall comply with the provisions set out in table 6.2.20.2-1.

Table 6.2.20.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| instantiate | no | tosca.datatypes.nfv.VnfInstantiateOperationConfiguration |  | Configuration parameters for the InstantiateVnf operation. |
| scale | no | tosca.datatypes.nfv.VnfScaleOperationConfiguration |  | Configuration parameters for the ScaleVnf operation. |
| scale_to_level | no | tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration |  | Configuration parameters for the ScaleVnfToLevel operation. |
| change_flavour | no | tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration |  | Configuration parameters for the changeVnfFlavourOpConfig operation. |
| heal | no | tosca.datatypes.nfv.VnfHealOperationConfiguration |  | Configuration parameters for the HealVnf operation. |
| terminate | no | tosca.datatypes.nfv.VnfTerminateOperationConfiguration |  | Configuration parameters for the TerminateVnf operation. |
| operate | no | tosca.datatypes.nfv.VnfOperateOperationConfiguration |  | Configuration parameters for the OperateVnf operation. |
| change_ext_connectivity | no | tosca.datatypes.nfv.VnfChangeExtConnectivityOperationConfiguration |  | Configuration parameters for the changeExtVnfConnectivityOpConfig operation. |
| change_current_package | no | tosca.datatypes.nfv.VnfChangeCurrentPackageOperationConfiguration |  | Configuration parameters for the ChangeCurrentVnfPackage operation. |
| create_snapshot | no | tosca.datatypes.nfv.VnfCreateSnapshotOperationConfiguration |  | Configuration parameters for the CreateVnfSnapshot operation. |
| revert_to_snapshot | no | tosca.datatypes.nfv.VnfRevertToSnapshotOperationConfiguration |  | Configuration parameters for the RevertToVnfSnapshot operation. |

#### 6.2.20.3	Definition

The syntax of the VnfLcmOperationsConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfLcmOperationsConfiguration: derived_from: tosca.datatypes.Root description: Represents information to configure lifecycle management operations properties: instantiate: type: tosca.datatypes.nfv.VnfInstantiateOperationConfiguration description: Configuration parameters for the InstantiateVnf operation required: false scale: type: tosca.datatypes.nfv.VnfScaleOperationConfiguration description: Configuration parameters for the ScaleVnf operation required: false scale_to_level: type: tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration description: Configuration parameters for the ScaleVnfToLevel operation required: false change_flavour: type: tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration description: Configuration parameters for the changeVnfFlavourOpConfig operation required: false heal: type: tosca.datatypes.nfv.VnfHealOperationConfiguration description: Configuration parameters for the HealVnf operation required: false terminate: type: tosca.datatypes.nfv.VnfTerminateOperationConfiguration description: Configuration parameters for the TerminateVnf operation required: false operate: type: tosca.datatypes.nfv.VnfOperateOperationConfiguration description: Configuration parameters for the OperateVnf operation required: false change_ext_connectivity: type: tosca.datatypes.nfv.VnfChangeExtConnectivityOperationConfiguration description: Configuration parameters for the changeExtVnfConnectivityOpConfig operation required: false change_current_package: type: tosca.datatypes.nfv.VnfChangeCurrentPackageOperationConfiguration description: Configuration parameters for the ChangeCurrentVnfPackage operation required: false # derived types are expected to introduce new properties, with their type derived from # tosca.datatypes.nfv.VnfChangeCurrentPackageOperationConfiguration, # with the same name as the operation designated to the ChangeCurrentVnfPackage request create_snapshot: type: tosca.datatypes.nfv.VnfCreateSnapshotOperationConfiguration description: Configuration parameters for the CreateVnfSnapshot operation required: false revert_to_snapshot: type: tosca.datatypes.nfv.VnfRevertToSnapshotOperationConfiguration description: Configuration parameters for the RevertToVnfSnapshot operation required: false

#### 6.2.20.4	Examples

None.

#### 6.2.20.5	Additional Requirements

None.

### 6.2.21	tosca.datatypes.nfv.VnfInstantiateOperationConfiguration

#### 6.2.21.1	Description

The VnfInstantiateOperationConfiguration data type represents information that affect the invocation of the InstantiateVnf operation, as specified in ETSI GS NFV-IFA 011 [1]. This data type definition is reserved for future use in the present document. Table 6.2.21.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.21.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfInstantiateOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfInstantiateOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfInstantiateOperationConfiguration |

#### 6.2.21.2	Properties

None.

#### 6.2.21.3	Definition

The syntax of the VnfInstantiateOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfInstantiateOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the InstantiateVnf operation. # This data type definition is reserved for future use in the present document. # properties:

#### 6.2.21.4	Examples

None.

#### 6.2.21.5	Additional Requirements

None.

### 6.2.22	tosca.datatypes.nfv.VnfScaleOperationConfiguration

#### 6.2.22.1	Description

VnfScaleOperationConfiguration represents information that affect the invocation of the ScaleVnf operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.22.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.22.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfScaleOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfScaleOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfScaleOperationConfiguration |

#### 6.2.22.2	Properties

The properties of the VnfScaleOperationConfiguration data type shall comply with the provisions set out in table 6.2.22.2-1.

Table 6.2.22.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scaling_by_more_than_one_step_supported | yes | boolean |  | Signals whether passing a value larger than one in the numScalingSteps parameter of the ScaleVnf operation is supported by this VNF. Default is FALSE, i.e. "not supported". |

#### 6.2.22.3	Definition

The syntax of the VnfScaleOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfScaleOperationConfiguration: derived_from: tosca.datatypes.Root description: Represents information that affect the invocation of the ScaleVnf operation properties: scaling_by_more_than_one_step_supported: type: boolean description: Signals whether passing a value larger than one in the numScalingSteps parameter of the ScaleVnf operation is supported by this VNF. required: true default: false

#### 6.2.22.4	Examples

See clause 6.8.1.9.

#### 6.2.22.5	Additional Requirements

None.

### 6.2.23	tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration

#### 6.2.23.1	Description

The VnfScaleToLevelOperationConfiguration data type represents information that affect the invocation of the ScaleVnfToLevel operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.23.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.23.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfScaleToLevelOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfScaleToLevelOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration |

#### 6.2.23.2	Properties

The properties of the VnfScaleToLevelOperationConfiguration data type shall comply with the provisions set out in table 6.2.23.2-1.

Table 6.2.23.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| arbitrary_target_levels_supported | yes | boolean |  | Signals whether scaling according to the parameter "scaleInfo" is supported by this VNF. |

#### 6.2.23.3	Definition

The syntax of the VnfScaleToLevelOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfScaleToLevelOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the ScaleVnfToLevel operation properties: arbitrary_target_levels_supported: type: boolean description: Signals whether scaling according to the parameter "scaleInfo" is supported by this VNF required: true

#### 6.2.23.4	Examples

See clause 6.8.1.9.

#### 6.2.23.5	Additional Requirements

None.

### 6.2.24	tosca.datatypes.nfv.VnfHealOperationConfiguration

#### 6.2.24.1	Description

The VnfHealOperationConfiguration data type represents information that affect the invocation of the HealVnf operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.24.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.24.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfHealOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfHealOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfHealOperationConfiguration |

#### 6.2.24.2	Properties

The properties of the VnfHealOperationConfiguration data type shall comply with the provisions set out in table 6.2.24.2-1.

Table 6.2.24.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| causes | no | list of string |  | Supported "cause" parameter values. |

#### 6.2.24.3	Definition

The syntax of the VnfHealOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfHealOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the HealVnf operation properties: causes: type: list description: Supported "cause" parameter values required: false entry_schema: type: string

#### 6.2.24.4	Examples

See clause 6.8.1.9.

#### 6.2.24.5	Additional Requirements

None.

### 6.2.25	tosca.datatypes.nfv.VnfTerminateOperationConfiguration

#### 6.2.25.1	Description

The VnfTerminateOperationConfiguration data type represents information that affect the invocation of the TerminateVnf, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.25.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.25.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfTerminateOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfTerminateOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfTerminateOperationConfiguration |

#### 6.2.25.2	Properties

The properties of the VnfTerminateOperationConfiguration data type shall comply with the provisions set out in table 6.2.25.2-1.

Table 6.2.25.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| min_graceful_termination_timeout | yes | scalar-unit.time |  | Minimum timeout value for graceful termination of a VNF instance. |
| max_recommended_graceful_termination_timeout | no | scalar-unit.time |  | Maximum recommended timeout value that can be needed to gracefully terminate a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint. |

#### 6.2.25.3	Definition

The syntax of the VnfTerminateOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfTerminateOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the TerminateVnf properties: min_graceful_termination_timeout: type: scalar-unit.time description: Minimum timeout value for graceful termination of a VNF instance required: true max_recommended_graceful_termination_timeout: type: scalar-unit.time description: Maximum recommended timeout value that can be needed to gracefully terminate a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint required: false

#### 6.2.25.4	Examples

None.

#### 6.2.25.5	Additional Requirements

None.

### 6.2.26	tosca.datatypes.nfv.VnfOperateOperationConfiguration

#### 6.2.26.1	Description

The VnfOperateOperationConfiguration data type represents information that affect the invocation of the OperateVnf operation, as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.26.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.26.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfOperateOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfOperateOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfOperateOperationConfiguration |

#### 6.2.26.2	Properties

The properties of the VnfOperateOperationConfiguration data type shall comply with the provisions set out in table 6.2.26.2-1.

Table 6.2.26.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| min_graceful_stop_timeout | yes | scalar-unit.time |  | Minimum timeout value for graceful stop of a VNF instance. |
| max_recommended_graceful_stop_timeout | no | scalar-unit.time |  | Maximum recommended timeout value that can be needed to gracefully stop a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint. |

#### 6.2.26.3	Definition

The syntax of the VnfOperateOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfOperateOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the OperateVnf operation properties: min_graceful_stop_timeout: type: scalar-unit.time description: Minimum timeout value for graceful stop of a VNF instance required: true max_recommended_graceful_stop_timeout: type: scalar-unit.time description: Maximum recommended timeout value that can be needed to gracefully stop a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint required: false

#### 6.2.26.4	Examples

None.

#### 6.2.26.5	Additional Requirements

None.

### 6.2.27	tosca.datatypes.nfv.ScaleInfo

#### 6.2.27.1	Description

The scaleInfo data type indicates for a given scaleAspect the corresponding scaleLevel. Table 6.2.27.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.27.1-1: Type name, shorthand, and URI

| Shorthand Name | ScaleInfo |
| --- | --- |
| Type Qualified Name | toscanfv:ScaleInfo |
| Type URI | tosca.datatypes.nfv.ScaleInfo |

#### 6.2.27.2	Properties

The properties of the ScaleInfo data type shall comply with the provisions set out in table 6.2.27.2-1.

Table 6.2.27.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scale_level | yes | integer | greater_or_equal : 0 | The scale level for a particular aspect. |

#### 6.2.27.3	Definition

The syntax of the ScaleInfo data type shall comply with the following definition:

tosca.datatypes.nfv.ScaleInfo: derived_from: tosca.datatypes.Root description: Indicates for a given scaleAspect the corresponding scaleLevel properties: scale_level: type: integer description: The scale level for a particular aspect required: true constraints: - greater_or_equal: 0

#### 6.2.27.4	Examples

See clause 6.8.1.9.

#### 6.2.27.5	Additional Requirements

None.

### 6.2.28	tosca.datatypes.nfv.ScalingAspect

#### 6.2.28.1	Description

The ScalingAspect data type describes the details of an aspect used for horizontal scaling. Table 6.2.28.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.28.1-1: Type name, shorthand, and URI

| Shorthand Name | ScalingAspect |
| --- | --- |
| Type Qualified Name | toscanfv:ScalingAspect |
| Type URI | tosca.datatypes.nfv.ScalingAspect |

#### 6.2.28.2	Properties

The properties of the ScalingAspect data type shall comply with the provisions set out in table 6.2.28.2-1.

Table 6.2.28.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | yes | string |  | Human readable name of the aspect. |
| description | yes | string |  | Human readable description of the aspect. |
| max_scale_level | yes | integer | positiveInteger | Total number of scaling steps that can be applied with regards to this aspect. The value of this property corresponds to the number of scaling steps can be applied to this aspect when scaling it from the minimum scale level (i.e. 0) to the maximum scale level defined by this property. |
| step_deltas | no | list of string |  | List of scaling deltas to be applied for the different subsequent scaling steps of this aspect. The first entry in the array shall correspond to the first scaling step (between scale levels 0 to 1) and the last entry in the array shall correspond to the last scaling step (between maxScaleLevel-1 and maxScaleLevel). |

#### 6.2.28.3	Definition

The syntax of the ScalingAspect data type shall comply with the following definition:

tosca.datatypes.nfv.ScalingAspect: derived_from: tosca.datatypes.Root description: describes the details of an aspect used for horizontal scaling properties: name: type: string description: Human readable name of the aspect required: true description: type: string description: Human readable description of the aspect required: true max_scale_level: type: integer # positiveInteger description: Total number of scaling steps that can be applied w.r.t. this aspect. The value of this property corresponds to the number of scaling steps can be applied to this aspect when scaling it from the minimum scale level (i.e. 0) to the maximum scale level defined by this property required: true constraints: - greater_or_equal: 0 step_deltas: type: list description: List of scaling deltas to be applied for the different subsequent scaling steps of this aspect. The first entry in the array shall correspond to the first scaling step (between scale levels 0 to 1) and the last entry in the array shall correspond to the last scaling step (between maxScaleLevel-1 and maxScaleLevel) required: false entry_schema: type: string # Identifier

#### 6.2.28.4	Examples

See clause A.6.

#### 6.2.28.5	Additional Requirements

None.

### 6.2.29	tosca.datatypes.nfv.LinkBitrateRequirements

#### 6.2.29.1	Description

The LinkBitrateRequirements data type is defined in clause 9.2.5 of the present document.

### 6.2.30	tosca.datatypes.nfv.ConnectivityType

#### 6.2.30.1	Description

The ConnectivityType data type is defined in clause 9.2.4 of the present document.

### 6.2.31	tosca.datatypes.nfv.VnfConfigurableProperties

#### 6.2.31.1	Description

The VnfConfigurableProperties data type describes configurable properties for a given VNF. Configurable properties can be standardized as listed below (e.g. related to auto scaling, auto healing and interface configuration) or can be VNF-specific as defined by the VNF provider.

The value of all VNF configurable properties listed in table 6.2.31.2-1 shall be modifiable anytime (including after instantiation of the VNF) via the Modify VNF information operation, unless stated otherwise in the description of the specific VNF configurable property.

Table 6.2.31.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.31.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfConfigurableProperties |
| --- | --- |
| Type Qualified Name | toscanfv:VnfConfigurableProperties |
| Type URI | tosca.datatypes.nfv.VnfConfigurableProperties |

#### 6.2.31.2	Properties

The properties of the VnfConfigurableProperties data type shall comply with the provisions set out in table 6.2.31.2-1.

Table 6.2.31.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| is_autoscale_enabled | no | boolean |  | It permits to enable (TRUE)/disable (FALSE) the auto-scaling functionality. If the property is not present , then configuring this VNF property is not supported. |
| is_autoheal_enabled | no | boolean |  | It permits to enable (TRUE)/disable (FALSE) the auto-healing functionality. If the property is not present, then configuring this VNF property is not supported. |
| vnfm_interface_info | no | tosca.datatypes.nfv.VnfmInterfaceInfo |  | Contains information enabling the VNF instance to access to the NFV-MANO interfaces produced by the VNFM (e.g. URIs and credentials). If the property is not present, then configuring this VNF property is not supported. If this attribute is declared for a VNF, its initial value shall be set prior to or at instantiation time (as initial value in the VNFD or via the VNF LCM interface). Its value shall be further modifiable after instantiation via the Modify VNF information operation. |
| vnfm_oauth_server_info | no | tosca.datatypes.nfv.OauthServerInfo |  | Contains information to enable discovery of the authorization server protecting access to VNFM interfaces. If the property is not present, then configuring this VNF property is not supported. If this attribute is declared for a VNF, its initial value shall be set prior to or at instantiation time (as initial value in the VNFD or via the VNF LCM interface). Its value shall be further modifiable after instantiation via the Modify VNF information operation. |
| vnf_oauth_server_info | no | tosca.datatypes.nfv.OauthServerInfo |  | Contains information to enable discovery of the authorization server to validate the access tokens provided by the VNFM when the VNFM accesses the VNF interfaces, if that functionality (token introspection) is supported by the authorization server. If the property is not present, then configuring this VNF property is not supported. If this attribute is declared for a VNF, its initial value shall be set prior to or at instantiation time (as initial value in the VNFD or via the VNF LCM interface). Its value shall be further modifiable after instantiation via the Modify VNF information operation. |
| additional_configurable_property | no | tosca.datatypes.nfv.VnfAdditionalConfigurableProperties |  | It provides VNF specific configurable properties that can be modified using the ModifyVnfInfo operation. If some of these properties are declared as required, their values shall be set prior to or at instantiation time (as initial value in the VNFD or via the VNF LCM interface). Their values may be modifiable after instantiation via the Modify VNF information operation if such modification of individual attributes is supported by the VNF and declared per attribute in the VNFD. |

#### 6.2.31.3	Definition

The syntax of the VnfConfigurableProperties data type shall comply with the following definition:

tosca.datatypes.nfv.VnfConfigurableProperties: derived_from: tosca.datatypes.Root description: indicates configuration properties for a given VNF (e.g. related to auto scaling and auto healing). properties: is_autoscale_enabled: type: boolean description: It permits to enable (TRUE)/disable (FALSE) the auto-scaling functionality. If the property is not present, then configuring this VNF property is not supported. required: false is_autoheal_enabled: type: boolean description: It permits to enable (TRUE)/disable (FALSE) the auto-healing functionality. If the property is not present, then configuring this VNF property is not supported. required: false vnfm_interface_info: type: tosca.datatypes.nfv.VnfmInterfaceInfo description: Contains information enabling access to the NFV-MANO interfaces produced by the VNFM (e.g. URIs and credentials). If the property is not present, then configuring this VNF property is not supported. required: false vnfm_oauth_server_info: type: tosca.datatypes.nfv.OauthServerInfo description: Contains information to enable discovery of the authorization server protecting access to VNFM interfaces. If the property is not present, then configuring this VNF property is not supported. required: false vnf_oauth_server_info: type: tosca.datatypes.nfv.OauthServerInfo description: Contains information to enable discovery of the authorization server to validate the access tokens provided by the VNFM when the VNFM accesses the VNF interfaces, if that functionality (token introspection) is supported by the authorization server. If the property is not present, then configuring this VNF property is not supported. required: false # additional_configurable_properties: # description: It provides VNF specific configurable properties that can be # modified using the ModifyVnfInfo operation # required: false # type: tosca.datatypes.nfv.VnfAdditionalConfigurableProperties # derived types are expected to introduce # additional_configurable_properties with its type derived from # tosca.datatypes.nfv.VnfAdditionalConfigurableProperties

#### 6.2.31.4	Examples

Example definition of configurable properties without properties assignment value.

tosca_definitions_version: tosca_simple_yaml_1_3 node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: constraints: - valid_values: [ simple, complex ] configurable_properties: type: MyCompany.datatypes.nfv.VnfConfigurableProperties data_types: Mycompany.datatypes.nfv.VnfConfigurableProperties: derived_from: tosca.datatypes.nfv.VnfConfigurableProperties properties: additional_configurable_properties: type: MyCompany.datatypes.nfv.VnfAdditionalConfigurableProperties MyCompany.datatypes.nfv.VnfAdditionalConfigurableProperties: derived_from: tosca.datatypes.nfv.VnfAdditionalConfigurableProperties properties: name_prefix_in_vim: type: string required: false dns_server: type: string required: true

In the above example, properties definitions are provided and properties assignment values are not necessary. The properties values are available in the API.

Example definition of configurable properties with properties assignment value.

tosca_definitions_version: tosca_simple_yaml_1_3

...

node_types:

MyCompany.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

interfaces:

Vnflcm:

type: tosca.interfaces.nfv.Vnflcm

MyCompany.nodes.nfv.Vdu.Aux:

derived_from: tosca.nodes.nfv.Vdu.Compute

properties:

configurable_properties:

type: MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties

required: false

data_types:

MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfcConfigurableProperties

properties:

additional_vnfc_configurable_properties:

type: MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties

required: true

MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties

properties:

name_prefix_in_vim:

type: string

required: true

default: "MyCustomer"

dns_server:

type: string

required: true

default: "90.200.250.57"

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

requirements:

virtual_link: [ dbBackendIpv4, virtual_link ] # IPv4 for SQL

inputs:

name_prefix_in_vim:

type: string

dns_server:

type: string

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

dbBackend:

type: MyCompany.nodes.nfv.Vdu.Aux

properties:

...

configurable_properties:

additional_vnfc_configurable_properties:

name_prefix_in_vim: { get_input: name_prefix_in_vim }

dns_server: { get_input: dns_server }

In the above example, default values are provided in the node type definition, properties assignment by using TOSCA get_input function is described in the node template. The properties values from the API will override the default values.

#### 6.2.31.5	Additional Requirements

None.

### 6.2.32	tosca.datatypes.nfv.VnfAdditionalConfigurableProperties

#### 6.2.32.1	Description

The VnfAdditionalConfigurableProperties data type is an empty base type for deriving data types for describing additional configurable properties for a given VNF. Table 6.2.32.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.32.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfAdditionalConfigurableProperties |
| --- | --- |
| Type Qualified Name | toscanfv:VnfAdditionalConfigurableProperties |
| Type URI | tosca.datatypes.nfv.VnfAdditionalConfigurableProperties |

#### 6.2.32.2	Properties

The properties of the VnfAdditionalConfigurableProperties data type shall comply with the provisions set out in table 6.2.32.2-1.

Table 6.2.32.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| is_writable_anytime | yes | boolean |  | It specifies whether these additional configurable properties are writeable (TRUE) at anytime (i.e. prior to / at instantiation time as well as after instantiation).or (FALSE) only prior to / at instantiation time. If this property is not present, the additional configurable properties are writable anytime. |

#### 6.2.32.3	Definition

The syntax of the VnfAdditionalConfigurableProperties data type shall comply with the following definition:

tosca.datatypes.nfv.VnfAdditionalConfigurableProperties: derived_from: tosca.datatypes.Root description: is an empty base type for deriving data types for describing additional configurable properties for a given VNF properties: is_writable_anytime: type: boolean description: It specifies whether these additional configurable properties are writeable (TRUE) at any time (i.e. prior to / at instantiation time as well as after instantiation).or (FALSE) only prior to / at instantiation time. If this property is not present, the additional configurable properties are writable anytime. required: true default: true

#### 6.2.32.4	Examples

See clause 6.2.31.4.

#### 6.2.32.5	Additional Requirements

None.

### 6.2.33	tosca.datatypes.nfv.VnfInfoModifiableAttributes

#### 6.2.33.1	Description

The VnfInfoModifiableAttributes data type describes VNF-specific extension and metadata for a given VNF. Table 6.2.33.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.33.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfInfoModifiableAttributes |
| --- | --- |
| Type Qualified Name | toscanfv:VnfInfoModifiableAttributes |
| Type URI | tosca.datatypes.nfv.VnfInfoModifiableAttributes |

#### 6.2.33.2	Properties

The properties of the VnfInfoModifiableAttributes data type shall comply with the provisions set out in table 6.2.33.2-1.

Table 6.2.33.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| extensions | no | tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions |  | "Extension" properties of VnfInfo that are writeable. |
| metadata | no | tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata |  | "Metadata" properties of VnfInfo that are writeable. |

#### 6.2.33.3	Definition

The syntax of the VnfInfoModifiableAttributes data type shall comply with the following definition:

tosca.datatypes.nfv.VnfInfoModifiableAttributes: derived_from: tosca.datatypes.Root description: Describes VNF-specific extension and metadata for a given VNF #properties: #extensions: #type: tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions #description: "Extension" properties of VnfInfo that are writeable #required: false # derived types are expected to introduce # extensions with its type derived from # tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions #metadata: #type: tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata #description: "Metadata" properties of VnfInfo that are writeable #required: false # derived types are expected to introduce # metadata with its type derived from # tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata

#### 6.2.33.4	Examples

The following example shows an example of a derived VnfInfoModifiableAttributesExtensions data type that contains one security-sensitive property.

tosca_definitions_version: tosca_simple_yaml_1_3

data_types:

MyCompany.datatypes.nfv.VnfInfoModifiableAttributes:

derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributes

properties:

extensions:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions

mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions:

derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions

properties:

password:

type: string

metadata:

sensitive: "true"

See clause 6.8.1.9 for other examples.

#### 6.2.33.5	Additional Requirements

None.

### 6.2.34	tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions

#### 6.2.34.1	Description

The VnfInfoModifiableAttributesExtensions data type is an empty base type for deriving data types for describing VNF-specific extension. Table 6.2.34.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.34.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfInfoModifiableAttributesExtensions |
| --- | --- |
| Type Qualified Name | toscanfv:VnfInfoModifiableAttributesExtensions |
| Type URI | tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions |

#### 6.2.34.2	Properties

None.

#### 6.2.34.3	Definition

The syntax of the VnfInfoModifiableAttributesExtensions data type shall comply with the following definition:

tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions: derived_from: tosca.datatypes.Root description: is an empty base type for deriving data types for describing VNF-specific extension

#### 6.2.34.4	Examples

See clause 6.8.1.9.

#### 6.2.34.5	Additional Requirements

None.

### 6.2.35	tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata

#### 6.2.35.1	Description

The VnfInfoModifiableAttributesMetadata data type is an empty base type for deriving data types for describing VNF-specific metadata. Table 6.2.35.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.35.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfInfoModifiableAttributesMetadata |
| --- | --- |
| Type Qualified Name | toscanfv:VnfInfoModifiableAttributesMetadata |
| Type URI | tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata |

#### 6.2.35.2	Properties

None.

#### 6.2.35.3	Definition

The syntax of the VnfInfoModifiableAttributesMetadata data type shall comply with the following definition:

tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata: derived_from: tosca.datatypes.Root description: is an empty base type for deriving data types for describing VNF-specific metadata

#### 6.2.35.4	Examples

Example metadata definition:

tosca_definitions_version: tosca_simple_yaml_1_3 node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: constraints: - valid_values: [ simple, complex ] modifiable_attributes: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes data_types: mycompany.datatypes.nfv.VnfInfoModifiableAttributes: derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributes properties: metadata: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesMetadata mycompany.datatypes.nfv.VnfInfoModifiableAttributesMetadata: derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata properties: metadata_key_1: type: string required: false metadata_key_2: type: string required: false

#### 6.2.35.5	Additional Requirements

None.

### 6.2.36	tosca.datatypes.nfv.Qos

#### 6.2.36.1	Description

The Qos data type is defined in clause 9.2.7 of the present document.

### 6.2.37	tosca.datatypes.nfv.LogicalNodeData

#### 6.2.37.1	Description

The LogicalNodeData data type describes compute, memory and I/O requirements associated with a particular VDU. Table 6.2.37.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.37.1-1: Type name, shorthand, and URI

| Shorthand Name | LogicalNodeData |
| --- | --- |
| Type Qualified Name | toscanfv:LogicalNodeData |
| Type URI | tosca.datatypes.nfv.LogicalNodeData |

#### 6.2.37.2	Properties

The properties of the LogicalNodeData data type shall comply with the provisions set out in table 6.2.37.2-1.

Table 6.2.37.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| logical_node_requirements | no | map of string |  | The logical node-level compute, memory and I/O requirements. A map of strings that contains a set of key-value pairs that describes hardware platform specific deployment requirements, including the number of CPU cores on this logical node, a memory configuration specific to a logical node or a requirement related to the association of an I/O device with the logical node. More information regarding the usage of this property is available at: https://register.etsi.org. |

#### 6.2.37.3	Definition

The syntax of the LogicalNodeData data type shall comply with the following definition:

tosca.datatypes.nfv.LogicalNodeData: derived_from: tosca.datatypes.Root description: Describes compute, memory and I/O requirements associated with a particular VDU. properties: logical_node_requirements: type: map description: The logical node-level compute, memory and I/O requirements. A map of strings that contains a set of key-value pairs that describes hardware platform specific deployment requirements, including the number of CPU cores on this logical node, a memory configuration specific to a logical node or a requirement related to the association of an I/O device with the logical node. required: false entry_schema: type: string

#### 6.2.37.4	Examples

None.

#### 6.2.37.5	Additional Requirements

None.

### 6.2.38	tosca.datatypes.nfv.SwImageData

#### 6.2.38.1	Description

The SwImageData data type describes information related to a software image artifact. Table 6.2.38.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.38.1-1: Type name, shorthand, and URI

| Shorthand Name | SwImageData |
| --- | --- |
| Type Qualified Name | toscanfv:SwImageData |
| Type URI | tosca.datatypes.nfv.SwImageData |

#### 6.2.38.2	Properties

The properties of the SwImageData data type shall comply with the provisions set out in table 6.2.38.2-1.

Table 6.2.38.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | yes | string |  | Name of this software image. |
| version | yes | string |  | Version of this software image. |
| provider | no | string |  | Provider of this software image. |
| checksum | yes | tosca.datatypes.nfv.ChecksumData |  | Checksum of the software image file. |
| container_format | yes | string | Valid values: [aki, ami, ari, bare, docker, ova, ovf] | The container format describes the container file format in which software image is provided. Description of valid values: aki: a kernel image ami: a machine image ari: a ramdisk image bare: the image does not have a container or metadata envelope docker: docker container format ova: OVF package in a tarfile ovf: OVF container format Future versions of the present document may extend the list of possible values. See note 1. |
| disk_format | yes | string | Valid values: [aki, ami, ari, iso, qcow2, raw, vdi, vhd, vhdx, vmdk] | The disk format of a software image is the format of the underlying disk image. Description of valid values: aki: a kernel image ami: a machine image ari: a ramdisk image iso: an archive format for the data contents of an optical disc, such as CD-ROM qcow2: a common disk image format, which can expand dynamically and supports copy on write raw: an unstructured disk image format vdi: a common disk image format vhd: a common disk image format vhdx: enhanced version of VHD format vmdk: a common disk image format Future versions of the present document may extend the list of possible values. See note 2. |
| min_disk | yes | scalar-unit.size | greater_or_equal: 0 B | The minimal disk size requirement for this software image. |
| min_ram | no | scalar-unit.size | greater_or_equal: 0 B | The minimal RAM requirement for this software image. |
| size | yes | scalar-unit.size |  | The size of this software image. |
| operating_system | no | string |  | Identifies the operating system used in the software image. |
| supported _virtualisation_environments | no | list of string |  | Identifies the virtualisation environments (e.g. hypervisor) compatible with this software image. |
| NOTE 1: The list of permitted values was taken from "Container formats" in [i.12]. NOTE 2: The list of permitted values was adapted from "Disk formats" in [i.12]. |  |  |  |  |

#### 6.2.38.3	Definition

The syntax of the SwImageData data type shall comply with the following definition:

tosca.datatypes.nfv.SwImageData: derived_from: tosca.datatypes.Root description: describes information related to a software image artifact properties: name: type: string description: Name of this software image required: true version: type: string description: Version of this software image required: true provider: type: string description: Provider of this software image required: false checksum: type: tosca.datatypes.nfv.ChecksumData description: Checksum of the software image file required: true container_format: type: string description: The container format describes the container file format in which software image is provided required: true constraints: - valid_values: [ aki, ami, ari, bare, docker, ova, ovf ] disk_format: type: string description: The disk format of a software image is the format of the underlying disk image required: true constraints: - valid_values: [ aki, ami, ari, iso, qcow2, raw, vdi, vhd, vhdx, vmdk ] min_disk: type: scalar-unit.size # Number description: The minimal disk size requirement for this software image required: true constraints: - greater_or_equal: 0 B min_ram: type: scalar-unit.size # Number description: The minimal RAM requirement for this software image required: false constraints: - greater_or_equal: 0 B size: type: scalar-unit.size # Number description: The size of this software image required: true operating_system: type: string description: Identifies the operating system used in the software image required: false supported_virtualisation_environments: type: list description: Identifies the virtualisation environments (e.g. hypervisor) compatible with this software image required: false entry_schema: type: string

#### 6.2.38.4	Examples

None.

#### 6.2.38.5	Additional Requirements

None.

### 6.2.39	tosca.datatypes.nfv.VirtualBlockStorageData

#### 6.2.39.1	Description

The VirtualBlockStorageData data type describes block storage requirements associated with compute resources in a particular VDU, either as a local disk or as virtual attached storage. Table 6.2.39.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.39.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualBlockStorageData |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualBlockStorageData |
| Type URI | tosca.datatypes.nfv.VirtualBlockStorageData |

#### 6.2.39.2	Properties

The properties of the VirtualBlockStorageData data type shall comply with the provisions set out in table 6.2.39.2-1.

Table 6.2.39.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| size_of_storage | yes | scalar-unit.size | greater_or_equal:0 B | Size of virtualised storage resource. |
| vdu_storage_requirements | no | map of string |  | The hardware platform specific storage requirements. A map of strings that contains a set of key-value pairs that represents the hardware platform specific storage deployment requirements. More information regarding the usage of this property is available at: https://register.etsi.org. |
| rdma_enabled | yes | boolean | default: false | Indicate if the storage support RDMA. |

#### 6.2.39.3	Definition

The syntax of the VirtualBlockStorageData data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualBlockStorageData: derived_from: tosca.datatypes.Root description: VirtualBlockStorageData describes block storage requirements associated with compute resources in a particular VDU, either as a local disk or as virtual attached storage properties: size_of_storage: type: scalar-unit.size description: Size of virtualised storage resource required: true constraints: - greater_or_equal: 0 B vdu_storage_requirements: type: map description: The hardware platform specific storage requirements. A map of strings that contains a set of key-value pairs that represents the hardware platform specific storage deployment requirements required: false entry_schema: type: string rdma_enabled: type: boolean description: Indicates if the storage support RDMA required: true default: false

#### 6.2.39.4	Examples

None.

#### 6.2.39.5	Additional Requirements

None.

### 6.2.40	tosca.datatypes.nfv.VirtualObjectStorageData

#### 6.2.40.1	Description

The VirtualObjectStorageData data type describes object storage requirements associated with compute resources in a particular VDU. Table 6.2.40.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.40.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualObjectStorageData |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualObjectStorageData |
| Type URI | tosca.datatypes.nfv.VirtualObjectStorageData |

#### 6.2.40.2	Properties

The properties of the VirtualObjectStorageData data type shall comply with the provisions set out in table 6.2.40.2-1.

Table 6.2.40.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| max_size_of_storage | no | scalar-unit.size | greater_or_equal:0 B | Maximum size of virtualised storage resource. |

#### 6.2.40.3	Definition

The syntax of the VirtualObjectStorageData data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualObjectStorageData: derived_from: tosca.datatypes.Root description: VirtualObjectStorageData describes object storage requirements associated with compute resources in a particular VDU properties: max_size_of_storage: type: scalar-unit.size description: Maximum size of virtualised storage resource required: false constraints: - greater_or_equal: 0 B

#### 6.2.40.4	Examples

None.

#### 6.2.40.5	Additional Requirements

None.

### 6.2.41	tosca.datatypes.nfv.VirtualFileStorageData

#### 6.2.41.1	Description

The VirtualObjectFileData data type describes file storage requirements associated with compute resources in a particular VDU. Table 6.2.41.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.41.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualObjectFileData |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualFileStorageData |
| Type URI | tosca.datatypes.nfv.VirtualFileStorageData |

#### 6.2.41.2	Properties

The properties of the VirtualFileStorageData data type shall comply with the provisions set out in table 6.2.41.2-1.

Table 6.2.41.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| size_of_storage | yes | scalar-unit.size | greater_or_equal:0 B | Size of virtualised storage resource. |
| file_system_protocol | yes | string |  | The shared file system protocol (e.g. NFS, CIFS). |

#### 6.2.41.3	Definition

The syntax of the VirtualFileStorageData data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualFileStorageData: derived_from: tosca.datatypes.Root description: VirtualFileStorageData describes file storage requirements associated with compute resources in a particular VDU properties: size_of_storage: type: scalar-unit.size description: Size of virtualised storage resource required: true constraints: - greater_or_equal: 0 B file_system_protocol: type: string description: The shared file system protocol (e.g. NFS, CIFS) required: true

#### 6.2.41.4	Examples

None.

#### 6.2.41.5	Additional Requirements

None.

### 6.2.42	tosca.datatypes.nfv.VirtualLinkBitrateLevel

#### 6.2.42.1	Description

The VirtualLinkBitrateLevel data type describes bitrate requirements applicable to the virtual link instantiated from a particular VnfVirtualLink. Table 6.2.42.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.42.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkBitrateLevel |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkBitrateLevel |
| Type URI | tosca.datatypes.nfv.VirtualLinkBitrateLevel |

#### 6.2.42.2	Properties

The properties of the VirtualLinkBitrateLevel data type shall comply with the provisions set out in table 6.2.42.2-1.

Table 6.2.42.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Virtual link bitrate requirements for an instantiation level or bitrate delta for a scaling step. |

#### 6.2.42.3	Definition

The syntax of the VirtualLinkBitrateLevel data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualLinkBitrateLevel: derived_from: tosca.datatypes.Root description: Describes bitrate requirements applicable to the virtual link instantiated from a particular VnfVirtualLink properties: bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Virtual link bitrate requirements for an instantiation level or bitrate delta for a scaling step required: true

#### 6.2.42.4	Examples

See clause A.6.

#### 6.2.42.5	Additional Requirements

None.

### 6.2.43	tosca.datatypes.nfv.VnfOperationAdditionalParameters

#### 6.2.43.1	Description

The VnfOperationAdditionalParameters data type is an empty base type for deriving data type for describing VNF-specific parameters to be passed when invoking lifecycle management operations as specified in ETSI GS NFV-IFA 011 [1]. Table 6.2.43.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.43.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfOperationAdditionalParameters |
| --- | --- |
| Type Qualified Name | toscanfv:VnfOperationAdditionalParameters |
| Type URI | tosca.datatypes.nfv.VnfOperationAdditionalParameters |

#### 6.2.43.2	Properties

None.

#### 6.2.43.3	Definition

The syntax of the VnfOperationAdditionalParameters data type shall comply with the following definition:

tosca.datatypes.nfv.VnfOperationAdditionalParameters: derived_from: tosca.datatypes.Root description: Is an empty base type for deriving data type for describing VNF-specific parameters to be passed when invoking lifecycle management operations #properties:

#### 6.2.43.4	Examples

tosca_definitions_version: tosca_simple_yaml_1_3 node_types: MyCompany.nodes.nfv.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: .. interfaces: Vnflcm: operations: instantiate: inputs: additional_parameters: type: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters data_types: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters: derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters properties: parameter_1: type: string required: true parameter_2: type: string required: true default: value_2

The following example shows the declaration of an additional parameter to receive a password in the instantiate operation. The metadata sensitive: “true” indicates that this property holds security-sensitive information.

tosca_definitions_version: tosca_simple_yaml_1_3 node_types: MyCompany.nodes.nfv.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: .. interfaces: Vnflcm: operations: instantiate: inputs: additional_parameters: type: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters data_types: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters: derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters properties: inst_password: type: string required: true metadata: sensitive: “true”

#### 6.2.43.5	Additional Requirements

None.

### 6.2.44	tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration

#### 6.2.44.1	Description

The VnfChangeFlavourOperationConfiguration data type represents information that affect the invocation of the ChangeVnfFlavour operation, as specified in ETSI GS NFV-IFA 011 [1]. This data type definition is reserved for future use in the present document. Table 6.2.44.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.44.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfChangeFlavourOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfChangeFlavourOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration |

#### 6.2.44.2	Properties

None.

#### 6.2.44.3	Definition

The syntax of the VnfChangeFlavourOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfChangeFlavourOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the ChangeVnfFlavour operation # This data type definition is reserved for future use in the present document. # properties:

#### 6.2.44.4	Examples

None.

#### 6.2.44.5	Additional Requirements

None.

### 6.2.45	tosca.datatypes.nfv.VnfChangeExtConnectivityOperation
Configuration

#### 6.2.45.1	Description

The VnfChangeExtConnectivityOperationConfiguration data type represents information that affect the invocation of the ChangeExtVnfConnectivity operation, as specified in ETSI GS NFV-IFA 011 [1]. This data type definition is reserved for future use in the present document. Table 6.2.45.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.45.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfChangeExtConnectivityOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfChangeExtConnectivityOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfChangeExtConnectivityOperationConfiguration |

#### 6.2.45.2	Properties

None.

#### 6.2.45.3	Definition

The syntax of the VnfChangeExtConnectivityOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfChangeExtConnectivityOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the ChangeExtVnfConnectivity operation # This data type definition is reserved for future use in the present document. # properties:

#### 6.2.45.4	Examples

None.

#### 6.2.45.5	Additional Requirements

None.

### 6.2.46	tosca.datatypes.nfv.VnfMonitoringParameter

The VnfMonitoringParameter data type is defined in clause 9.2.9 of the present document.

### 6.2.47	tosca.datatypes.nfv.VnfcMonitoringParameter

#### 6.2.47.1	Description

This data type provides information on virtualised resource related performance metrics applicable to a VNFC. Table 6.2.47.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.47.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfcMonitoringParameter |
| --- | --- |
| Type Qualified Name | toscanfv:VnfcMonitoringParameter |
| Type URI | tosca.datatypes.nfv.VnfcMonitoringParameter |

#### 6.2.47.2	Properties

The properties of the VnfcMonitoringParameter data type shall comply with the provisions set out in table 6.2.47.2-1.

Table 6.2.47.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Human readable name of the monitoring parameter. |
| performance_metric | yes | string | valid values: v_cpu_usage_mean_vnf v_cpu_usage_peak_vnf v_memory_usage_mean_vnf v_memory_usage_peak_vnf v_disk_usage_mean_vnf v_disk_usage_peak_vnf byte_incoming_vnf_int_cp byte_outgoing_vnf_int_cp packet_incoming_vnf_int_cp packet_outgoing_vnf_int_cp v_cpu_usage_mean v_cpu_usage_peak v_memory_usage_mean v_memory_usage_peak v_disk_usage_mean v_disk_usage_peak v_net_byte_incoming v_net_byte_outgoing v_net_packet_incoming v_net_packet_outgoing usage_mean_vStorage usage_peak_vStorage | Identifies a performance metric to be monitored. Performance metric values shall be either set to: A corresponding measurement name defined in clause 7.2 of ETSI GS NFV-IFA 027 [7], without appending a sub-counter. In this case the VNFM computes these measurements from lower-level metrics collected from the VIM. See note. A corresponding measurement name defined in clause 7.1 of ETSI GS NFV-IFA 027 [7], without appending a sub-counter. In this case the VNFM collects these metrics from the VIM for all compute, storage and network resources allocated to the VNFC instance. |
| collection_period | no | scalar-unit.time |  | Describes the periodicity at which to collect the performance information. |
| NOTE: The measured object type for _cpu_usage_mean_vnf, v_cpu_usage_peak_vnf, v_memory_usage_mean_vnf, v_memory_usage_peak_vnf, v_disk_usage_mean_vnf and v_disk_usage_peak_vnf is the VNFC. |  |  |  |  |

#### 6.2.47.3	Definition

The syntax of the VnfcMonitoringParameter data type shall comply with the following definition:

tosca.datatypes.nfv.VnfcMonitoringParameter: derived_from: tosca.datatypes.Root description: Represents information on virtualised resource related performance metrics applicable to the VNF. properties: name: type: string description: Human readable name of the monitoring parameter required: true performance_metric: type: string description: Identifies a performance metric to be monitored, according to ETSI GS NFV-IFA 027. required: true constraints: - valid_values: [ v_cpu_usage_mean_vnf, v_cpu_usage_peak_vnf, v_memory_usage_mean_vnf, v_memory_usage_peak_vnf, v_disk_usage_mean_vnf, v_disk_usage_peak_vnf, byte_incoming_vnf_int_cp, byte_outgoing_vnf_int_cp, packet_incoming_vnf_int_cp, packet_outgoing_vnf_int_cp, v_cpu_usage_mean, v_cpu_usage_peak,v_memory_usage_mean,v_memory_usage_peak, v_disk_usage_mean, v_disk_usage_peak, v_net_byte_incoming, v_net_byte_outgoing, v_net_packet_incoming, v_net_packet_outgoing, usage_mean_vStorage, usage_peak_vStorage ] collection_period: type: scalar-unit.time description: Describes the periodicity at which to collect the performance information. required: false constraints: - greater_than: 0 s

#### 6.2.47.4	Examples

None.

#### 6.2.47.5	Additional Requirements

None.

### 6.2.48	tosca.datatypes.nfv.VirtualLinkMonitoringParameter

#### 6.2.48.1	Description

This data type provides information on virtualised resource related performance metrics. Table 6.2.48.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.48.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkMonitoringParameter |
| --- | --- |
| Type Qualified Name | toscanfv: VirtualLinkMonitoringParameter |
| Type URI | tosca.datatypes.nfv.VirtualLinkMonitoringParameter |

#### 6.2.48.2	Properties

The properties of the VirtualLinkMonitoringParameter data type shall comply with the provisions set out in table 6.2.48.2-1.

Table 6.2.48.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Human readable name of the monitoring parameter. |
| performance_metric | yes | string | valid values: byte_incoming byte_outgoing packet_incoming packet_outgoing | Identifies a performance metric to be monitored. Performance metric values shall be set to following measurement names defined in clause 7.1 of ETSI GS NFV-IFA 027 [7] ,without appending a sub-counter: - ByteIncoming - ByteOutgoing - PacketIncoming - PacketOutgoing The VNFM collects these metrics from the VIM by aggregating the sub-counters of all virtual link ports attached to the virtual link to which the metrics apply. |
| collection_period | no | scalar-unit.time |  | Describes the periodicity at which to collect the performance information. |

#### 6.2.48.3	Definition

The syntax of the VirtualLinkMonitoringParameter data type shall comply with the following definition:

tosca.datatypes.nfv.VirtualLinkMonitoringParameter: derived_from: tosca.datatypes.Root description: Represents information on virtualised resource related performance metrics applicable to the VNF. properties: name: type: string description: Human readable name of the monitoring parameter required: true performance_metric: type: string description: Identifies a performance metric to be monitored. required: true constraints: - valid_values: [ byte_incoming, byte_outgoing, packet_incoming, packet_outgoing ] collection_period: type: scalar-unit.time description: Describes the periodicity at which to collect the performance information. required: false constraints: - greater_than: 0 s

#### 6.2.48.4	Examples

None.

#### 6.2.48.5	Additional Requirements

None.

### 6.2.49	tosca.datatypes.nfv.InterfaceDetails

#### 6.2.49.1	Description

The InterfaceDetails data type describes information used to access an interface exposed by a VNF. It corresponds to the interfaceDetails attribute of the VnfInterfaceDetails information element defined in ETSI GS NFV-IFA 011 [1]. Table 6.2.49.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.49.1-1: Type name, shorthand, and URI

| Shorthand Name | InterfaceDetails |
| --- | --- |
| Type Qualified Name | toscanfv:InterfaceDetails |
| Type URI | tosca.datatypes.nfv.InterfaceDetails |

#### 6.2.49.2	Properties

The properties of the InterfaceDetails data type shall comply with the provisions set out in table 6.2.49.2-1.

Table 6.2.49.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| uri_components | no | tosca.datatypes.nfv.UriComponents |  | Provides components to build a Uniform Resource Identifier (URI) where to access the interface end point. |
| interface_specific_data | no | map of string |  | Provides additional details that are specific to the type of interface considered. |

#### 6.2.49.3	Definition

The syntax of the InterfaceDetails data type shall comply with the following definition:

tosca.datatypes.nfv.InterfaceDetails: derived_from: tosca.datatypes.Root description: information used to access an interface exposed by a VNF properties: uri_components: type: tosca.datatypes.nfv.UriComponents description: Provides components to build a Uniform Resource Identifier (URI) where to access the interface end point. required: false interface_specific_data: type: map description: Provides additional details that are specific to the type of interface considered. required: false entry_schema: type: string

#### 6.2.49.4	Examples

See clause 6.10.12.

#### 6.2.49.5	Additional Requirements

None.

### 6.2.50	tosca.datatypes.nfv.UriComponents

#### 6.2.50.1	Description

The UriComponents data type describes information used to build a URI that complies with IETF RFC 3986 [8]. Table 6.2.50.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.50.1-1: Type name, shorthand, and URI

| Shorthand Name | UriComponents |
| --- | --- |
| Type Qualified Name | toscanfv:UriComponents |
| Type URI | tosca.datatypes.nfv.UriComponents |

#### 6.2.50.2	Properties

The properties of the UriComponents data type shall comply with the provisions set out in table 6.2.50.2-1.

Table 6.2.50.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scheme | yes | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the scheme component of a URI, as per IETF RFC 3986 [8] |
| authority | no | tosca.datatypes.nfv.UriAuthority |  | Corresponds to the authority component of a URI, as per IETF RFC 3986 [8] See note |
| path | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the path component of a URI, as per IETF RFC 3986 [8] |
| query | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the query component of a URI, as per IETF RFC 3986 [8] |
| fragment | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the fragment component of a URI, as per IETF RFC 3986 [8] |
| NOTE: If this property is not included while the URI scheme requires it, the VNFM is expected to generate it, based on knowledge of the network configuration of the external CP instance that provides the connectivity for this interface. |  |  |  |  |

#### 6.2.50.3	Definition

The syntax of the UriComponents data type shall comply with the following definition:

tosca.datatypes.nfv.UriComponents: derived_from: tosca.datatypes.Root description: information used to build a URI that complies with IETF RFC 3986 [8]. properties: scheme: type: string # shall comply with IETF RFC 3986 description: scheme component of a URI. required: true authority: type: tosca.datatypes.nfv.UriAuthority description: Authority component of a URI required: false path: type: string # shall comply with IETF RFC 3986 description: path component of a URI. required: false query: type: string # shall comply with IETF RFC 3986 description: query component of a URI. required: false fragment: type: string # shall comply with IETF RFC 3986 description: fragment component of a URI. required: false

#### 6.2.50.4	Examples

See clause 6.10.12.

#### 6.2.50.5	Additional Requirements

When this datatype is used to provide information for accessing APIs defined in ETSI GS NFV-SOL 002 [i.4], the path property may be included and the query and fragment properties shall be absent. The values of the scheme, authority and path properties form the {apiRoot} of the URI prefix.

### 6.2.51	tosca.datatypes.nfv.UriAuthority

#### 6.2.51.1	Description

The UriAuthority data type corresponds to the authority component of a URI as specified in IETF RFC 3986 [8]. Table 6.2.51.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.51.1-1: Type name, shorthand, and URI

| Shorthand Name | UriAuthority |
| --- | --- |
| Type Qualified Name | toscanfv:UriAuthority |
| Type URI | tosca.datatypes.nfv.UriAuthority |

#### 6.2.51.2	Properties

The properties of the UriAuthority data type shall comply with the provisions set out in table 6.2.51.2-1.

Table 6.2.51.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| user_info | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the user_info field of the authority component of a URI, as per IETF RFC 3986 [8]. For HTTP and HTTPS URIs, the provisions in sections 2.7.1 and 2.7.2 of IETF RFC 7230 [12] apply, respectively. |
| host | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the host field of the authority component of a URI, as per IETF RFC 3986 [8]. See note 1. |
| port | no | string | String values shall comply with IETF RFC 3986 [8] | Corresponds to the port field of the authority component of a URI, as per IETF RFC 3986 [8]. See note 2. |
| NOTE 1: If this property is not included the VNFM is expected to generate it, based on knowledge of the network configuration of the external CP instance that provides the connectivity for this interface. NOTE 2: If this property is not included the default port for the protocol declared by the scheme property of the parent UriComponents structure shall be used unless there are configuration mechanisms applied that are outside the scope of the present document. If no default port exists for the URI scheme, the port property shall be included unless there are configuration mechanisms applied that are outside the scope of the present document. |  |  |  |  |

#### 6.2.51.3	Definition

The syntax of the UriAuthority data type shall comply with the following definition:

tosca.datatypes.nfv.UriAuthority: derived_from: tosca.datatypes.Root description: information that corresponds to the authority component of a URI as specified in IETF RFC 3986 [8] properties: user_info: type: string # shall comply with IETF RFC 3986 description: user_info field of the authority component of a URI required: false host: type: string # shall comply with IETF RFC 3986 description: host field of the authority component of a URI required: false port: type: string # shall comply with IETF RFC 3986 description: port field of the authority component of a URI required: false

#### 6.2.51.4	Examples

See clause 6.10.12.

#### 6.2.51.5	Additional Requirements

When this datatype is used to provide information for accessing APIs defined in ETSI GS NFV-SOL 002 [i.4], the host property and port properties may be included and the user_info property shall not be included. If the host property is included and the value is a registered name, it is assumed that means are in place to resolve the host name to the correct IP address. If the host property is not included, it is assumed that the VNFM will use the IP address associated to one of the connection point instances created from the VnfExpCp and VduCp node types declared as a target of the SupportedVnfInterface policy.

NOTE:	This means that if multiple CP instances exist that were created from a particular VnfExtCp or VduCp node template, the VNFM may use any of them to attempt accessing the interface. If no reply is received because the selected CP instance if out of service or is not reachable, the VNFM is expected to try reaching the interface through another CP instance.

### 6.2.52	tosca.datatypes.nfv.VnfProfile

#### 6.2.52.1	Description

The VnfProfile data type is defined in clause 9.2.8 of the present document.

### 6.2.53	tosca.datatypes.nfv.ChecksumData

#### 6.2.53.1	Description

The ChecksumData data type describes information about the result of performing a checksum operation over some arbitrary data. Table 6.2.53.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.53.1-1: Type name, shorthand, and URI

| Shorthand Name | ChecksumData |
| --- | --- |
| Type Qualified Name | toscanfv:ChecksumData |
| Type URI | tosca.datatypes.nfv.ChecksumData |

#### 6.2.53.2	Properties

The properties of the ChecksumData data type shall comply with the provisions set out in table 6.2.53.2-1.

Table 6.2.53.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| algorithm | yes | string | Valid values: sha-224, sha-256, sha-384, sha-512 | Describes the algorithm used to obtain the checksum value, as described in [14]. |
| hash | yes | string |  | Contains the result of applying the algorithm indicated by the algorithm property to the data to which this ChecksumData refers. |

#### 6.2.53.3	Definition

The syntax of the ChecksumData data type shall comply with the following definition:

tosca.datatypes.nfv.ChecksumData: derived_from: tosca.datatypes.Root description: Describes information about the result of performing a checksum operation over some arbitrary data properties: algorithm: type: string description: Describes the algorithm used to obtain the checksum value required: true constraints: - valid_values: [sha-224, sha-256, sha-384, sha-512 ] hash: type: string description: Contains the result of applying the algorithm indicated by the algorithm property to the data to which this ChecksumData refers required: true

#### 6.2.53.4	Examples

<some_tosca_entity>: properties: checksum: algorithm: sha-256 hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

#### 6.2.53.5	Additional Requirements

None.

### 6.2.54	tosca.datatypes.nfv.VnfmInterfaceInfo

#### 6.2.54.1	Description

The VnfmInterfaceInfo data type describes information enabling the VNF instance to access the NFV-MANO interfaces produced by the VNFM (e.g. URIs and credentials). Table 6.2.54.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.54.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfmInterfaceInfo |
| --- | --- |
| Type Qualified Name | toscanfv:VnfmInterfaceInfo |
| Type URI | tosca.datatypes.nfv.VnfmInterfaceInfo |

#### 6.2.54.2	Properties

The properties of the VnfmInterfaceInfo data type shall comply with the provisions set out in table 6.2.54.2-1.

Table 6.2.54.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| interface_name | yes | string | Valid values vnf_lcm vnf_pm vnf_fm | Identifies an interface produced by the VNFM. |
| details | no | tosca.datatypes.nfv.InterfaceDetails |  | Provide additional data to access the interface endpoint (e.g. API URI prefix). |
| credentials | no | map of string |  | Provides credential enabling access to the interface. This property is reserved for future use in the present document. |

#### 6.2.54.3	Definition

The syntax of the VnfmInterfaceInfo data type shall comply with the following definition:

tosca.datatypes.nfv.VnfmInterfaceInfo: derived_from: tosca.datatypes.Root description: Describes information enabling the VNF instance to access the NFV-MANO interfaces produced by the VNFM properties: interface_name: type: string description: Identifies an interface produced by the VNFM. required: true constraints: - valid_values: [ vnf_lcm, vnf_pm, vnf_fm ] details: type: tosca.datatypes.nfv.InterfaceDetails description: Provide additional data to access the interface endpoint required: false credentials: type: map description: Provides credential enabling access to the interface required: false entry_schema: type: string

#### 6.2.54.4	Examples

None.

#### 6.2.54.5	Additional Requirements

None.

### 6.2.55	tosca.datatypes.nfv.OauthServerInfo

#### 6.2.55.1	Description

The OauthServerInfo data type describes information to enable discovery of the authorization server. This data type definition is reserved for future use in the present document. Table 6.2.55.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.55.1-1: Type name, shorthand, and URI

| Shorthand Name | OauthServerInfo |
| --- | --- |
| Type Qualified Name | toscanfv:OauthServerInfo |
| Type URI | tosca.datatypes.nfv.OauthServerInfo |

#### 6.2.55.2	Properties

None.

#### 6.2.55.3	Definition

The syntax of the OauthServerInfo data type shall comply with the following definition:

tosca.datatypes.nfv.OauthServerInfo: derived_from: tosca.datatypes.Root description: information to enable discovery of the authorization server #properties: #This data type definition is reserved for future use in the present document

#### 6.2.55.4	Examples

None.

#### 6.2.55.5	Additional Requirements

None.

### 6.2.56	tosca.datatypes.nfv.BootData

#### 6.2.56.1	Description

The BootData data type describes the information used to customize a virtualised compute resource at boot time. Table 6.2.56.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.56.1-1: Type name, shorthand, and URI

| Shorthand Name | BootData |
| --- | --- |
| Type Qualified Name | toscanfv:BootData |
| Type URI | tosca.datatypes.nfv.BootData |

#### 6.2.56.2	Properties

The properties of the BootData data type shall comply with the provisions set out in table 6.2.56.2-1.

Table 6.2.56.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| vim_specific_properties | no | tosca.datatypes.nfv.BootDataVimSpecificProperties |  | Properties used for selecting VIM specific capabilities when setting the boot data. |
| kvp_data | no | tosca.datatypes.nfv.KvpData |  | A set of key-value pairs for configuring a virtual compute resource. The mechanisms for conveying these key-value pairs to the virtual compute resource are out of the scope of the present document. An example of such mechanisms is the OpenStack metadata service defined in [i.13]. |
| content_or_file_data | no | tosca.datatypes.nfv.ContentOrFileData |  | A string content or a file for configuring a virtual compute resource. The mechanisms for conveying the string content or the file to the virtual compute resource are out of the scope of the present document. An example of such mechanisms is the OpenStack User-data service defined in [i.14]. |

#### 6.2.56.3	Definition

The syntax of the BootData data type shall comply with the following definition:

tosca.datatypes.nfv.BootData: derived_from: tosca.datatypes.Root description: describes the information used to customize a virtualised compute resource at boot time. properties: vim_specific_properties: type: tosca.datatypes.nfv.BootDataVimSpecificProperties description: Properties used for selecting VIM specific capabilities when setting the boot data. required: false kvp_data: type: tosca.datatypes.nfv.KvpData description: A set of key-value pairs for configuring a virtual compute resource. required: false content_or_file_data: type: tosca.datatypes.nfv.ContentOrFileData description: A string content or a file for configuring a virtual compute resource. required: false

#### 6.2.56.4	Examples

See clause 6.8.3.8.

#### 6.2.56.5	Additional Requirements

None.

### 6.2.57	tosca.datatypes.nfv.KvpData

#### 6.2.57.1	Description

The KvpData data type describes a set of key-value pairs information used to customize a virtualised compute resource by using only key-value pairs data. Table 6.2.57.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.57.1-1: Type name, shorthand, and URI

| Shorthand Name | KvpData |
| --- | --- |
| Type Qualified Name | toscanfv:KvpData |
| Type URI | tosca.datatypes.nfv.KvpData |

#### 6.2.57.2	Properties

The properties of the MetaData data type shall comply with the provisions set out in table 6.2.57.2-1.

Table 6.2.57.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| data | no | map of string |  | A map of strings that contains a set of key-value pairs that describes the information for configuring the virtualised compute resource. |

#### 6.2.57.3	Definition

The syntax of the KvpData data type shall comply with the following definition:

tosca.datatypes.nfv.KvpData: derived_from: tosca.datatypes.Root description: describes a set of key-value pairs information used to customize a virtualised compute resource at boot time by using only key-value pairs data. properties: data: type: map description: A map of strings that contains a set of key-value pairs that describes the information for configuring the virtualised compute resource. required: false entry_schema: type: string

#### 6.2.57.4	Examples

See clause 6.8.3.8.

#### 6.2.57.5	Additional Requirements

None.

### 6.2.58	tosca.datatypes.nfv.ContentOrFileData

#### 6.2.58.1	Description

The ContentOrFileData data type describes a string content or a file information used to customize a virtualised compute resource by using string content or file. Table 6.2.58.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.58.1-1: Type name, shorthand, and URI

| Shorthand Name | ContentOrFileData |
| --- | --- |
| Type Qualified Name | toscanfv:ContentOrFileData |
| Type URI | tosca.datatypes.nfv.ContentOrFileData |

#### 6.2.58.2	Properties

The properties of the ContentOrFileData data type shall comply with the provisions set out in table 6.2.58.2-1.

Table 6.2.58.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| data | no | map of string |  | A map of strings that contains a set of key-value pairs that carries the dynamic deployment values which used to replace the corresponding variable parts in the file as identify by a URL as described in source_path. Shall be present if "source_path" is present and shall be absent otherwise. See note 1. |
| content | no | string |  | The string information used to customize a virtualised compute resource at boot time. See note 2. |
| source_path | no | string |  | The URL to a file contained in the VNF package used to customize a virtualised compute resource. The content shall comply with IETF RFC 3986 [8]. See note 2. |
| destination_path | no | string |  | The URL where to inject a file indicated by the source_path property into the virtualised compute resource. The content shall comply with IETF RFC 3986 [8]. See note 3. |
| NOTE 1: It is the file processor (e.g. in the VNFM) responsibility to replace the corresponding variable parts in the file with the value carried in the data property, the variable parts in the file are start with $ and end with $, its content is the same character with one of the keys in the data property, for example, if one of the keys in 'data' is "https_proxy", somewhere in the file content there is $https_proxy$. NOTE 2: One and only one of the following properties shall be present: contents or source_path. NOTE 3: It is only present when a particular method is used for transferring boot information into a virtualised compute resource and source_path is also present. For example, such method can be the personality method as described in [i.15], and it has been deprecated since Openstack 12.0.0 (Stein). |  |  |  |  |

#### 6.2.58.3	Definition

The syntax of the ContentOrFileData data type shall comply with the following definition:

tosca.datatypes.nfv.ContentOrFileData: derived_from: tosca.datatypes.Root description: describes a string content or a file information used to customize a virtualised compute resource at boot time by using string content or file. properties: data: type: map description: A map of strings that contains a set of key-value pairs that carries the dynamic deployment values which used to replace the corresponding variable parts in the file as identify by a URL as described in source_path. Shall be present if "source_path" is present and shall be absent otherwise.. required: false entry_schema: type: string content: type: string description: The string information used to customize a virtualised compute resource at boot time. required: false source_path: type: string description: The URL to a file contained in the VNF package used to customize a virtualised compute resource. The content shall comply with IETF RFC 3986 [8]. required: false destination_path: type: string description: The URL address when inject a file into the virtualised compute resource. The content shall comply with IETF RFC 3986 [8]. required: false

#### 6.2.58.4	Examples

See clause 6.8.3.8.

#### 6.2.58.5	Additional Requirements

None.

### 6.2.59	tosca.datatypes.nfv.BootDataVimSpecificProperties

#### 6.2.59.1	Description

The BootDataVimSpecificProperties data type describes the VIM related information used for selecting VIM specific capabilities when setting the boot data when setting the boot data. Table 6.2.59.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.59.1-1: Type name, shorthand, and URI

| Shorthand Name | BootDataVimSpecificProperties |
| --- | --- |
| Type Qualified Name | toscanfv: BootDataVimSpecificProperties |
| Type URI | tosca.datatypes.nfv.BootDataVimSpecificProperties |

#### 6.2.59.2	Properties

The properties of the BootDataVimSpecificProperties data type shall comply with the provisions set out in table 6.2.59.2-1.

Table 6.2.59.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| vim_type | no | string |  | Discriminator for the different types of the VIM information. The set of permitted values is expected to change over time as new types or versions of VIMs become available. The ETSI NFV registry of VIM-related information [i.16] provides access to information about various VIM types. |
| properties | yes | map of string |  | Properties used for selecting VIM specific capabilities when setting the boot data. For example, it can set whether config_drive functionality is selected in case VIM support it. This property is reserved for future use in the present document. |

#### 6.2.59.3	Definition

The syntax of the BootDataVimSpecificProperties data type shall comply with the following definition:

tosca.datatypes.nfv.BootDataVimSpecificProperties: derived_from: tosca.datatypes.Root description: describes the VIM specific information used for selecting VIM specific capabilities when setting the boot data. properties: vim_type: type: string description: Discriminator for the different types of the VIM information. required: true properties: type: map description: Properties used for selecting VIM specific capabilities when setting the boot data entry_schema: type: string required: true

#### 6.2.59.4	Examples

None.

#### 6.2.59.5	Additional Requirements

None.

### 6.2.60	tosca.datatypes.nfv.VnfPackageChangeSelector

#### 6.2.60.1	Description

The VnfPackageChangeSelector data type describes the source and destination VNFDs as well as source deployment flavour for a change current VNF Package. Table 6.2.60.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.60.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfPackageChangeSelector |
| --- | --- |
| Type Qualified Name | toscanfv: VnfPackageChangeSelector |
| Type URI | tosca.datatypes.nfv.VnfPackageChangeSelector |

#### 6.2.60.2	Properties

The properties of the VnfPackageChangeSelector data type shall comply with the provisions set out in table 6.2.60.2-1.

Table 6.2.60.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| source_descriptor_id | yes | string |  | Identifier of the source VNFD and the source VNF package. |
| destination_descriptor_id | yes | string |  | Identifier of the destination VNFD and the destination VNF package. |
| source_flavour_id | yes | string |  | Identifier of the deployment flavour in the source VNF package for which this data type applies. |

#### 6.2.60.3	Definition

The syntax of the VnfPackageChangeSelector data type shall comply with the following definition:

tosca.datatypes.nfv.VnfPackageChangeSelector: derived_from: tosca.datatypes.Root description: data type describes the source and destination VNFDs as well as source deployment flavour for a change current VNF Package. properties: source_descriptor_id: type: string description: Identifier of the source VNFD and the source VNF package. required: true destination_descriptor_id: type: string description: Identifier of the destination VNFD and the destination VNF package. required: true source_flavour_id: type: string description: Identifier of the deployment flavour in the source VNF package for which this data type applies. required: true

#### 6.2.60.4	Examples

See clause 6.10.15.5.

#### 6.2.60.5	Additional Requirements

Either the source_descriptor_id or the destination_descriptor_id shall be equal to the vnfdId of the VNFD containing this version VnfPackageChangeSelector.

### 6.2.61	tosca.datatypes.nfv.VnfPackageChangeComponentMapping

#### 6.2.61.1	Description

The VnfPackageChangeComponentMapping data type describes a mapping between the identifier of a components or property in the source VNFD and the identifier of the corresponding component or property in the destination VNFD. Table 6.2.61.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.61.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfPackageChangeComponentMapping |
| --- | --- |
| Type Qualified Name | toscanfv: VnfPackageChangeComponentMapping |
| Type URI | tosca.datatypes.nfv.VnfPackageChangeComponentMapping |

#### 6.2.61.2	Properties

The properties of the VnfPackageChangeComponentMapping data type shall comply with the provisions set out in table 6.2.61.2-1.

Table 6.2.61.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| component_type | yes | string | valid_values: [vdu, cp, virtual_link, virtual_storage, deployment_flavour, instantiation_level, scaling_aspect] | The type of component or property. Possible values differentiate whether changes concern to some VNF component (e.g. VDU, internal VLD, etc.) or property (e.g. a Scaling Aspect, etc.). |
| source_id | yes | string |  | Identifier of the component or property in the source VNFD. |
| destination_id | yes | string |  | Identifier of the component or property in the destination VNFD. |
| description | no | string |  | Human readable description of the component changes. |

#### 6.2.61.3	Definition

The syntax of the VnfPackageChangeComponentMapping data type shall comply with the following definition:

tosca.datatypes.nfv.VnfPackageChangeComponentMapping: derived_from: tosca.datatypes.Root description: A mapping between the identifier of a components or property in the source VNFD and the identifier of the corresponding component or property in the destination VNFD. properties: component_type: type: string description: The type of component or property. Possible values differentiate whether changes concern to some VNF component (e.g. VDU, internal VLD, etc.) or property (e.g. a Scaling Aspect, etc.). constraints: - valid_values: [ vdu, cp, virtual_link, virtual_storage, deployment_flavour, instantiation_level, scaling_aspect ] required: true source_id: type: string description: Identifier of the component or property in the source VNFD. required: true destination_id: type: string description: Identifier of the component or property in the destination VNFD. required: true description: type: string description: Human readable description of the component changes. required: false

#### 6.2.61.4	Examples

See clause 6.10.15.5.

#### 6.2.61.5	Additional Requirements

None.

### 6.2.62	tosca.datatypes.nfv.VnfChangeCurrentPackageOperation
Configuration

#### 6.2.62.1	Description

The VnfChangeCurrentPackageOperationConfiguration data type represents information that affect the invocation of the change current VNF Package operation, as specified in ETSI GS NFV-IFA 011 [1]. This data type definition is reserved for future use in the present document. Table 6.2.62.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.62.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfChangeCurrentPackageOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfChangeCurrentPackageOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfChangeCurrentPackageOperationConfiguration |

#### 6.2.62.2	Properties

None.

#### 6.2.62.3	Definition

The syntax of the VnfChangeCurrentPackageOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfChangeCurrentPackageOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the change current VNF Package operation. # This data type definition is reserved for future use in the present document. # properties: # derived types are expected to introduce new properties, with their type derived from tosca.datatypes.nfv.VnfChangeCurrentPackageOperationConfiguration, with the same name as the operation designated to the ChangeCurrentVnfPackage request

#### 6.2.62.4	Examples

None.

#### 6.2.62.5	Additional Requirements

None.

### 6.2.63	tosca.datatypes.nfv.VnfCreateSnapshotOperationConfiguration

#### 6.2.63.1	Description

The VnfCreateSnapshotOperationConfiguration data type represents information that affect the invocation of the CreateVnfSnapshot operation, as specified in ETSI GS NFV-IFA 011 [1]. This data type definition is reserved for future use in the present document. Table 6.2.63.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.63.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfCreateSnapshotOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfCreateSnapshotOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfCreateSnapshotOperationConfiguration |

#### 6.2.63.2	Properties

None.

#### 6.2.63.3	Definition

The syntax of the VnfCreateSnapshotOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfCreateSnapshotOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the CreateVnfSnapshot operation # This data type definition is reserved for future use in the present document. # properties:

#### 6.2.63.4	Examples

None.

#### 6.2.63.5	Additional Requirements

None.

### 6.2.64	tosca.datatypes.nfv.VnfRevertToSnapshotOperationConfiguration

#### 6.2.64.1	Description

The VnfRevertToSnapshotOperationConfiguration data type represents information that affect the invocation of the RevertToVnfSnapshot operation, as specified in ETSI GS NFV-IFA 011 [1]. This data type definition is reserved for future use in the present document. Table 6.2.64.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.2.64.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfRevertToSnapshotOperationConfiguration |
| --- | --- |
| Type Qualified Name | toscanfv:VnfRevertToSnapshotOperationConfiguration |
| Type URI | tosca.datatypes.nfv.VnfRevertToSnapshotOperationConfiguration |

#### 6.2.64.2	Properties

None.

#### 6.2.64.3	Definition

The syntax of the VnfRevertToSnapshotOperationConfiguration data type shall comply with the following definition:

tosca.datatypes.nfv.VnfRevertToSnapshotOperationConfiguration: derived_from: tosca.datatypes.Root description: represents information that affect the invocation of the RevertToVnfSnapshot operation # This data type definition is reserved for future use in the present document. # properties:

#### 6.2.64.4	Examples

None.

#### 6.2.64.5	Additional Requirements

None.

## 6.3	Artifact Types

### 6.3.1	tosca.artifacts.nfv.SwImage

#### 6.3.1.1	Description

The SwImage artifact describes the software image which is directly loaded on the virtualisation container realizing of the VDU or is to be loaded on a virtual storage resource, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.3.1.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.3.1.1-1: Type name, shorthand, and URI

| Shorthand Name | SwImage |
| --- | --- |
| Type Qualified Name | toscanfv:SwImage |
| Type URI | tosca.artifacts.nfv.SwImage |

#### 6.3.1.2	Properties

The properties of the SwImage artifacts type shall comply with the provisions set out in table 6.3.1.2-1.

Table 6.3.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | yes | string |  | Name of this software image. |
| version | yes | string |  | Version of this software image. |
| provider | no | string |  | Provider of this software image. |
| checksum | yes | tosca.datatypes.nfv.ChecksumData |  | Checksum of the software image file. |
| container_format | yes | string | Valid values: [aki, ami, ari, bare, docker, ova, ovf] | The container format describes the container file format in which software image is provided. Description of valid values: aki: a kernel image ami: a machine image ari: a ramdisk image bare: the image does not have a container or metadata envelope docker: docker container format ova: OVF package in a tarfile ovf: OVF container format Future versions of the present document may extend the list of possible values. See note 1. |
| disk_format | yes | string | Valid values: [aki, ami, ari, iso, qcow2, raw, vdi, vhd, vhdx, vmdk] | The disk format of a software image is the format of the underlying disk image. Description of valid values: aki: a kernel image ami: a machine image ari: a ramdisk image iso: an archive format for the data contents of an optical disc, such as CD-ROM qcow2: a common disk image format, which can expand dynamically and supports copy on write raw: an unstructured disk image format vdi: a common disk image format vhd: a common disk image format vhdx: enhanced version of VHD format vmdk: a common disk image format Future versions of the present document may extend the list of possible values. See note 2. |
| min_disk | yes | scalar-unit.size | greater_or_equal: 0 B | The minimal disk size requirement for this software image. |
| min_ram | no | scalar-unit.size | greater_or_equal: 0 B | The minimal RAM requirement for this software image. |
| size | yes | scalar-unit.size |  | The size of this software image. |
| operating_system | no | string |  | Identifies the operating system used in the software image. |
| supported_virtualisation_environments | no | list of string |  | Identifies the virtualisation environments (e.g. hypervisor) compatible with this software image. |
| NOTE 1: The list of permitted values was taken from "Container formats" in [i.12]. NOTE 2: The list of permitted values was adapted from "Disk formats" in [i.12]. |  |  |  |  |

#### 6.3.1.3	Description

tosca.artifacts.nfv.SwImage: derived_from: tosca.artifacts.Deployment.Image description: describes the software image which is directly loaded on the virtualisation container realizing of the VDU or is to be loaded on a virtual storage resource properties: name: type: string description: Name of this software image required: true version: type: string description: Version of this software image required: true provider: type: string description: Provider of this software image required: false checksum: type: tosca.datatypes.nfv.ChecksumData description: Checksum of the software image file required: true container_format: type: string description: The container format describes the container file format in which software image is provided required: true constraints: - valid_values: [ aki, ami, ari, bare, docker, ova, ovf ] disk_format: type: string description: The disk format of a software image is the format of the underlying disk image required: true constraints: - valid_values: [ aki, ami, ari, iso, qcow2, raw, vdi, vhd, vhdx, vmdk ] min_disk: type: scalar-unit.size # Number description: The minimal disk size requirement for this software image required: true constraints: - greater_or_equal: 0 B min_ram: type: scalar-unit.size # Number description: The minimal RAM requirement for this software image required: false constraints: - greater_or_equal: 0 B size: type: scalar-unit.size # Number description: The size of this software image required: true operating_system: type: string description: Identifies the operating system used in the software image required: false supported_virtualisation_environments: type: list description: Identifies the virtualisation environments (e.g. hypervisor) compatible with this software image required: false entry_schema: type: string

### 6.3.2	tosca.artifacts.Implementation.nfv.Mistral

#### 6.3.2.1	Description

This artifact type represents a Mistral file that contains Mistral language [i.7] constructs that can be executed within a Mistral workbook. Support of this type is optional.

Table 6.3.2.1-1: Type name, shorthand, and URI

| Shorthand Name | Mistral |
| --- | --- |
| Type Qualified Name | toscanfv:Mistral |
| Type URI | tosca.artifacts.Implementation.nfv.Mistral |
| derived_from | tosca.artifacts.Implementation |

#### 6.3.2.2	Definition

The syntax of the Mistral artifact type shall comply with the following definition:

tosca.artifacts.Implementation.nfv.Mistral: derived_from: tosca.artifacts.Implementation description: artifacts for Mistral workflows mime_type: application/x-yaml file_ext: [ yaml ]

## 6.4	Capability Types

### 6.4.1	tosca.capabilities.nfv.VirtualBindable

#### 6.4.1.1	Description

The VirtualBindable capability indicates that the node that includes it can be pointed by a tosca.relationships.nfv.VirtualBindsTo relationship type which is used to model the VduHasCpd association illustrated in ETSI GS NFV-IFA 011 [1]. Table 6.4.1.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.4.1.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualBindable |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualBindable |
| Type URI | tosca.capabilities.nfv.VirtualBindable |

#### 6.4.1.2	Properties

None.

#### 6.4.1.3	Definition

The syntax of the VirtualBindable capability type shall comply with the following definition:

tosca.capabilities.nfv.VirtualBindable: derived_from: tosca.capabilities.Node description: Indicates that the node that includes it can be pointed by a tosca.relationships.nfv.VirtualBindsTo relationship type which is used to model the VduHasCpd association

### 6.4.2	tosca.capabilities.nfv.VirtualLinkable

#### 6.4.2.1	Description

The VirtualLinkable capability type is defined in clause 9.4.1 of the present document.

### 6.4.3	tosca.capabilities.nfv.VirtualCompute

#### 6.4.3.1	Description

The VirtualCompute capability type describes the capabilities related to virtual compute resources, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.4.3.1-1 specifies the declared names for this capability type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.4.3.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualCompute |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualCompute |
| Type URI | tosca.capabilities.nfv.VirtualCompute |

#### 6.4.3.2	Properties

The properties of the VirtualCompute capability type shall comply with the provisions set out in table 6.4.3.2-1.

Table 6.4.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| logical_node | no | map of tosca.datatypes.nfv.LogicalNodeData |  | The Logical Node requirements. |
| request_additional_capabilities | no | map of tosca.datatypes.nfv.RequestedAdditionalCapability |  | Describes additional capability for a particular VDU. |
| compute_requirements | no | map of string |  | Describes compute requirements. |
| virtual_memory | yes | tosca.datatypes.nfv.VirtualMemory |  | Describes virtual memory of the virtualised compute. |
| virtual_cpu | yes | tosca.datatypes.nfv.VirtualCpu |  | Describes virtual CPU(s) of the virtualised compute. |
| virtual_local_storage | no | list of tosca.datatypes.nfv.VirtualBlockStorageData |  | A list of virtual system disks created and destroyed as part of the VM lifecycle. |

#### 6.4.3.3	Definition

The syntax of the VirtualCompute capability type shall comply with the following definition:

tosca.capabilities.nfv.VirtualCompute: derived_from: tosca.capabilities.Node description: Describes the capabilities related to virtual compute resources properties: logical_node: type: map description: Describes the Logical Node requirements required: false entry_schema: type: tosca.datatypes.nfv.LogicalNodeData requested_additional_capabilities: type: map description: Describes additional capability for a particular VDU required: false entry_schema: type: tosca.datatypes.nfv.RequestedAdditionalCapability compute_requirements: type: map required: false entry_schema: type: string virtual_memory: type: tosca.datatypes.nfv.VirtualMemory description: Describes virtual memory of the virtualised compute required: true virtual_cpu: type: tosca.datatypes.nfv.VirtualCpu description: Describes virtual CPU(s) of the virtualised compute required: true virtual_local_storage: type: list description: A list of virtual system disks created and destroyed as part of the VM lifecycle required: false entry_schema: type: tosca.datatypes.nfv.VirtualBlockStorageData description: virtual system disk definition

### 6.4.4	tosca.capabilities.nfv.VirtualStorage

#### 6.4.4.1	Description

The VirtualStorage capability indicates that the node that includes it can be pointed by a tosca.relationships.nfv.AttachesTo relationship type which is used to model the VduHasVirtualStorageDesc association illustrated in ETSI GS NFV-IFA 011 [1]. Table 6.4.4.1-1 specifies the declared names for this capability type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.4.4.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualStorage |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualStorage |
| Type URI | tosca.capabilities.nfv.VirtualStorage |

#### 6.4.4.2	Definition

The syntax of the VirtualStorage capability type shall comply with the following definition:

tosca.capabilities.nfv.VirtualStorage: derived_from: tosca.capabilities.Root description: Describes the attachment capabilities related to Vdu.Storage

### 6.4.5	tosca.capabilities.nfv.TrunkBindable

#### 6.4.5.1	Description

The TrunkBindable capability indicates that the VduCp node that includes it can be pointed by a tosca.relationships.nfv.TrunkBindsTo relationship type which is used to model the trunkPortTopology illustrated in ETSI GS NFV-IFA 011 [1]. Table 6.4.5.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.4.5.1-1: Type name, shorthand, and URI

| Shorthand Name | TrunkBindable |
| --- | --- |
| Type Qualified Name | toscanfv:TrunkBindable |
| Type URI | tosca.capabilities.nfv.TrunkBindable |

#### 6.4.5.2	Properties

None.

#### 6.4.5.3	Definition

The syntax of the TrunkBindable capability type shall comply with the following definition:

tosca.capabilities.nfv.TrunkBindable: derived_from: tosca.capabilities.Node description: Indicates that the node that includes it can be pointed by a tosca.relationships.nfv.TrunkBindsTo relationship type which is used to model the trunkPortTopology. This capability is available only the trunk_mode properties value is “true”.

## 6.5	Requirements Types

None.

## 6.6	Relationship Types

### 6.6.1	tosca.relationships.nfv.VirtualBindsTo

#### 6.6.1.1	Description

This relationship type represents an association between Vdu.Compute and VduCp node types. Table 6.6.1.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.6.1.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualBindsTo |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualBindsTo |
| Type URI | tosca.relationships.nfv.VirtualBindsTo |

#### 6.6.1.2	Properties

None.

#### 6.6.1.3	Definition

The syntax of the VirtualBindsTo relationship type shall comply with the following definition:

tosca.relationships.nfv.VirtualBindsTo: derived_from: tosca.relationships.DependsOn description: Represents an association relationship between Vdu.Compute and VduCp node types valid_target_types: [ tosca.capabilities.nfv.VirtualBindable ]

### 6.6.2	tosca.relationships.nfv.VirtualLinksTo

#### 6.6.2.1	Description

The VirtualLinksTo relationship type is defined in clause 9.6.1 of the present document representing an association relationship between a VduCp and a VnfVirtualLink node type or a VnfExtCp and a VnfVirtualLink node type.

### 6.6.3	tosca.relationships.nfv.AttachesTo

#### 6.6.3.1	Description

This relationship type represents an association between the Vdu.Compute and one of the following node types: Vdu.VirtualBlockStorage, Vdu.VirtualObjectStorage or Vdu.VirtualFileStorage. Table 6.6.3.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.6.3.1-1: Type name, shorthand, and URI

| Shorthand Name | AttachesTo |
| --- | --- |
| Type Qualified Name | toscanfv:AttachesTo |
| Type URI | tosca.relationships.nfv.AttachesTo |

#### 6.6.3.2	Properties

None.

#### 6.6.3.3	Definition

The syntax of the AttachesTo relationship type shall comply with the following definition:

tosca.relationships.nfv.AttachesTo: derived_from: tosca.relationships.Root description: Represents an association relationship between the Vdu.Compute and one of the node types, Vdu.VirtualBlockStorage, Vdu.VirtualObjectStorage or Vdu.VirtualFileStorage valid_target_types: [ tosca.capabilities.nfv.VirtualStorage ]

### 6.6.4	tosca.relationships.nfv.TrunkBindsTo

#### 6.6.4.1	Description

This relationship type represents an association between a VduCp node used as a trunk port and other VduSubCp nodes used as subports of the same trunk. Table 6.6.4.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.6.4.1-1: Type name, shorthand, and URI

| Shorthand Name | TrunkBindsTo |
| --- | --- |
| Type Qualified Name | toscanfv: TrunkBindsTo |
| Type URI | tosca.relationships.nfv.TrunkBindsTo |

#### 6.6.4.2	Properties

None.

#### 6.6.4.3	Definition

The syntax of the TrunkBindsTo relationship type shall comply with the following definition:

tosca.relationships.nfv.TrunkBindsTo: derived_from: tosca.relationships.DependsOn description: Represents the association relationship between a VduCp node used as a trunk port and other VduSubCp nodes used as subports of the same trunk. valid_target_types: [ tosca.capabilities.nfv.TrunkBindable ]

## 6.7	Interface Types

### 6.7.1	tosca.interfaces.nfv.Vnflcm

#### 6.7.1.1	Description

The tosca.interfaces.nfv.Vnflcm contains a set of TOSCA operations corresponding to the following VNF LCM operations defined in ETSI GS NFV-IFA 007 [i.1]:

Instantiate VNF

Terminate VNF

Modify VNF information

Change VNF Flavour

Change External VNF Connectivity

Operate VNF

Heal VNF

Scale VNF

Scale VNF To Level

Create VNF Snapshot

Revert to VNF Snapshot

Change VNF current package

In addition, the VNFM shall also support TOSCA operations corresponding to preamble and postamble to the execution of the base operation. The name of these operations is constructed according to the following pattern:

<base_operation_name>_start for a preamble

<base_operation_name>_end for a postamble

The designations ("_start", "_end") in the name of TOSCA operations are postfixes so that related operations are adjacent in an alphabetical listing.

The tosca.interfaces.nfv.Vnflcm also contains a set of TOSCA notifications corresponding to the following VNF LCM
operations defined in ETSI GS NFV-IFA 007 [i.1]:

Change VNF current package

In addition, the VNFM shall also support TOSCA notifications corresponding to preamble and postamble to the base notification. The name of these notifications is constructed according to the following pattern:

- <base_notification_name>_start for a preamble

- <base_notification_name>_end for a postamble

Table 6.7.1.1-1 specifies the declared names for this interface type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.7.1.1-1: Type name, shorthand, and URI

| Shorthand Name | Vnflcm |
| --- | --- |
| Type Qualified Name | toscanfv:Vnflcm |
| Type URI | tosca.interfaces.nfv.Vnflcm |

#### 6.7.1.2	Definition

The syntax of the Vnflcm interface type shall comply with the following definition:

tosca.interfaces.nfv.Vnflcm: derived_from: tosca.interfaces.Root description: This interface encompasses a set of TOSCA operations corresponding to the VNF LCM operations defined in ETSI GS NFV-IFA 007 as well as to preamble and postamble procedures to the execution of the VNF LCM operations. operations: instantiate: description: Invoked upon receipt of an Instantiate VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters instantiate_start: description: Invoked before instantiate instantiate_end: description: Invoked after instantiate terminate: description: Invoked upon receipt Terminate VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters terminate_start: description: Invoked before terminate terminate_end: description: Invoked after terminate modify_information: description: Invoked upon receipt of a Modify VNF Information request modify_information_start: description: Invoked before modify_information modify_information_end: description: Invoked after modify_information change_flavour: description: Invoked upon receipt of a Change VNF Flavour request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters change_flavour_start: description: Invoked before change_flavour change_flavour_end: description: Invoked after change_flavour change_external_connectivity: description: Invoked upon receipt of a Change External VNF Connectivity request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters change_external_connectivity_start: description: Invoked before change_external_connectivity change_external_connectivity_end: description: Invoked after change_external_connectivity operate: description: Invoked upon receipt of an Operate VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters operate_start: description: Invoked before operate operate_end: description: Invoked after operate heal: description: Invoked upon receipt of a Heal VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters inputs: cause: type: string description: Indicates the reason why a healing procedure is required. required: false vnfc_instance_ids: type: list entry_schema: type: string description: List of VNFC instances requiring a healing action. required: false heal_start: description: Invoked before heal heal_end: description: Invoked after heal scale: description: Invoked upon receipt of a Scale VNF request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters inputs: type: type: string description: Indicates the type of the scale operation requested. constraints: - valid_values: [ scale_out, scale_in ] aspect: type: string description: Identifier of the scaling aspect. number_of_steps: type: integer description: Number of scaling steps to be executed. required: true constraints: - greater_than: 0 default: 1 scale_start: description: Invoked before scale scale_end: description: Invoked after scale scale_to_level: description: Invoked upon receipt of a Scale VNF to Level request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters inputs: instantiation_level: type: string description: Identifier of the target instantiation level of the current deployment flavour to which the VNF is requested to be scaled. Either instantiation_level or scale_info shall be provided. required: false scale_info: type: map # key: aspectId description: For each scaling aspect of the current deployment flavour, indicates the target scale level to which the VNF is to be scaled. Either instantiation_level or scale_info shall be provided. required: false entry_schema: type: tosca.datatypes.nfv.ScaleInfo scale_to_level_start: description: Invoked before scale_to_level scale_to_level_end: description: Invoked after scale_to_level create_snapshot: description: Invoked upon receipt of a Create VNF snapshot request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters create_snapshot_start: description: Invoked before create_snapshot create_snapshot_end: description: Invoked after create_snapshot revert_to_snapshot: description: Invoked upon receipt of a Revert to VNF snapshot request # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters revert_to_snapshot_start: description: Invoked before revert_to_snapshot revert_to_snapshot_end: description: Invoked after revert_to_snapshot change_current_package: description: Invoked by tosca.policies.nfv.VnfPackageChange # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters change_current_package_start: description: Invoked by tosca.policies.nfv.VnfPackageChange change_current_package_end: description: Invoked by tosca.policies.nfv.VnfPackageChange notifications: change_current_package_notification: description: Invoked upon receipt of a ChangeCurrentVnfPackage request change_current_package_start_notification: description: Invoked before the operation designated to changing the current VNF package change_current_package_end_notification: description: Invoked after the operation designated to changing the current VNF package

#### 6.7.1.3	Additional Requirements

All VNF supported LCM operations shall be listed in the service template, except "instantiate" and "terminate" that may be omitted, as specified in ETSI GS NFV-IFA 011 [1] for the supportedOperation attribute of a deployment flavour.

The implementation and inputs keynames specified in TOSCA-Simple-Profile-YAML-v1.3 [20] for an operation definition may be included for each operation listed in the Vnflcm interface definition.

If a TOSCA operation representing a VNF LCM operation is listed in the service template without an associated implementation, then it means that:

the VNF LCM operation is supported (i.e. this is the manifestation of the supportedOperation attribute as per ETSI GS NFV-IFA 011 [1]); and

the processing logic associated with the LCM operation is the default implementation provided by the VNFM.

If an implementation is associated to a TOSCA operation that represents a preamble or a postamble to a VNF LCM operation, the implementation logic is executed before or after the execution of the VNF LCM operation implementation, respectively.

The VNFM shall make available all parameters from the message invoking the VNF LCM operation as inputs to the corresponding TOSCA interface operations.

In the operation definitions on the Vnflcm interface, the additional_parameters (VNF-specific extension of the tosca.datatypes.nfv.VnfOperationAdditionalParameters) of the inputs section describes the name and type of the additional parameters (additionalParams) that can be submitted in the VNF LCM operation request. See an example in clause 6.2.43 (tosca.datatypes.nfv.VnfOperationAdditionalParameters).

The inputs keyname can also be used to specify additional input parameters for executing the TOSCA operation, beyond those received in the VNF LCM operation request. To distinguish them from the latter ones, such input parameters shall not be named "additional_parameters".

The implementation of the operation corresponding to preamble and postamble TOSCA operations (instantiate_start, instantiate_end, scale_start, scale_end, etc.), if present, shall be invoked with the same parameters as the corresponding base operations ones (instantiate, scale, etc.). The inputs of the operations corresponding to the postamble and preamble operations shall not be defined in the VNFD.

Starting with version 3.3.1 of the present document, the Vnflcm interface type definition grammar was changed to support notifications and operations. For backward compatibility, TOSCA-Simple-Profile-YAML-v1.3 [20], clause 3.7.5.5 specifies the provisions for handling the previous grammar. Support of the Release 2 Vnflcm interface type definition grammar can be removed in subsequent versions of the present document.

#### 6.7.1.4	Support of LCM scripts

In ETSI GS NFV-IFA 011 [1], the definition of the "LifeCycleManagementScript" information element of the VNFD associates scripts with events, where an event can be an external or an internal stimulus. These events are mapped to TOSCA operations of the VNF node type in the following way:

external stimuli are mapped to TOSCA operations corresponding to the VNF LCM operations defined in ETSI GS NFV-IFA 007 [i.1];

internal stimuli are mapped to preamble and postamble of these TOSCA operations;

events that cannot be mapped to these TOSCA operations (lcmTransitionEvent as described in ETSI GS NFV-IFA 011 [1], clause 7.1.13) can be mapped to further TOSCA operations by extending the TOSCA interface.

The LCM scripts can be regarded as artifacts that provide a VNF-specific implementation of the TOSCA operation corresponding to the stimulus. The script input parameters shall be provided to the script according to the declaration in the inputs field of the operation definition. The artifact type definition shall enable identifying the DSL used by the script. The artifact type definition for Python is provided in section 5.4.4.1 of TOSCA-Simple-Profile-YAML-v1.3 [20]. The artifact definition for Mistral is provided in clause 6.3.2 of the present document.

NOTE:	As all input parameters needed for operations corresponding to external and internal stimuli are defined in the "input parameters of the external stimuli operations", the VNF Designer is expected to make the list of parameters as complete as needed to handle not only the external stimuli but also the internal stimuli.

#### 6.7.1.5	Examples

The following example template fragments illustrate the concepts.

tosca_definitions_version: tosca_simple_yaml_1_3 imports: - .. node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF .. topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 .. interfaces: Vnflcm: operations: instantiate: ..

In the above example, as there is no implementation and inputs specified to the operations, the built-in implementation of the operation is invoked when the Instantiate VNF request is received on the LCM interface of the Or-Vnfm reference point. The received parameters (flavourId, instantiationLevelId, etc.) are passed to the built-in implementation (as flavour_id, instantiation_level_id).

tosca_definitions_version: tosca_simple_yaml_1_3 imports: - .. node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF .. topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 .. interfaces: Vnflcm: operations: instantiate: implementation: instantiate-script ..

In the above example, the instantiate-script is invoked when the Instantiate VNF request is received, passing the received parameters to it similarly to the previous example. This example does not imply a one-to-one mapping between operations and script names.

tosca_definitions_version: tosca_simple_yaml_1_3 imports: - .. node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF .. topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 .. interfaces: Vnflcm: operations: .. scale_start: pre-scale-script scale_end: post-scale-script ..

In the above example, LCM scripts are associated with the "scale start" and "scale end" internal stimuli. As no script is associated to the scale operation, its default implementation runs (after running the pre-scale-script, and before running the post-scale-script).

tosca_definitions_version: tosca_simple_yaml_1_3 imports: - .. node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF .. topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 .. interfaces: Vnflcm: operations: instantiate: implementation: instantiate-script inputs: script_input_1: value_1 script_input_2: value_2 artifacts: instantiate-script: description: Instantiate workflow script type: tosca.artifacts.Implementation.Python file: instantiate.py #repository: .. #deploy_path: .. ..

In the above example:

The inputs section provides additional input values to the instantiate-script (i.e. the manifestation of the scriptInput attribute of LifeCycleManagementScript as defined in ETSI GS NFV-IFA 011 [1]).

NOTE:	There is another kind of input called additional_parameters dedicated to the additional parameters (additionalParams) received in the message invoking the VNF LCM operation; this input is not illustrated by the above examples; see clause 6.2.43.4 on how to declare additional_parameters in the derived VNF node type.

TOSCA artifacts definition is used to convey the type of DSL used as a scripting language that is associated with an operation (i.e. the manifestation of the scriptDsl attribute of LifeCycleManagementScript as per ETSI GS NFV-IFA 011 [1]).

### 6.7.2	tosca.interfaces.nfv.VnfIndicator

#### 6.7.2.1	Description

The tosca.interfaces.nfv.VnfIndictor is an empty base interface type for deriving VNF specific interface types that include VNF indicator specific notifications.

Table 6.7.2.1-1 specifies the declared names for this interface type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.7.2.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfIndicator |
| --- | --- |
| Type Qualified Name | toscanfv:VnfIndicator |
| Type URI | tosca.interfaces.nfv.VnfIndicator |

#### 6.7.2.2	Definition

The syntax of the VnfIndicator interface type shall comply with the following definition:

tosca.interfaces.nfv.VnfIndicator: derived_from: tosca.interfaces.Root description: This interface is an empty base interface type for deriving VNF specific interface types that include VNF indicator specific notifications.

#### 6.7.2.3	Examples

See clause 6.8.1.9.

### 6.7.3	tosca.interfaces.nfv.ChangeCurrentVnfPackage

#### 6.7.3.1	Description

The tosca.interfaces.nfv.ChangeCurrentVnfPackage is an empty base interface type for deriving VNF specific interface types that include VNF Change Current VNF Package specific operation.

Table 6.7.3.1-1 specifies the declared names for this interface type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.7.3.1-1: Type name, shorthand, and URI

| Shorthand Name | ChangeCurrentVnfPackage |
| --- | --- |
| Type Qualified Name | toscanfv:ChangeCurrentVnfPackage |
| Type URI | tosca.interfaces.nfv.ChangeCurrentVnfPackage |

#### 6.7.3.2	Definition

The syntax of the ChangeCurrentVnfPackage interface type shall comply with the following definition:

tosca.interfaces.nfv.ChangeCurrentVnfPackage: derived_from: tosca.interfaces.Root description: This interface is an empty base interface type for deriving VNF specific interface types that include VNF Change Current VNF Package specific operation. # operations: # operation_name: name of a VNF-specific operation serving the Change current VNF Package request. # description: Invoked by tosca.policies.nfv.VnfPackageChange # inputs: # additional_parameters: # type: tosca.datatypes.nfv.VnfOperationAdditionalParameters # required: false # derived types are expected to introduce additional_parameters with its # type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters

#### 6.7.3.3	Examples

See clause 6.10.15.5.

## 6.8	Node Types

### 6.8.1	tosca.nodes.nfv.VNF

#### 6.8.1.1	Description

The VNF node type is the generic abstract type from which all VNF specific node types shall be derived to form, together with other node types, the TOSCA service template(s) representing the VNFD information element as defined in ETSI GS NFV-IFA 011 [1]. Table 6.8.1.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.1.1-1: Type name, shorthand, and URI

| Shorthand Name | VNF |
| --- | --- |
| Type Qualified Name | toscanfv:VNF |
| Type URI | tosca.nodes.nfv.VNF |

#### 6.8.1.2	Properties

The properties of the VNF node type shall comply with the provisions set out in table 6.8.1.2-1.

Table 6.8.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| descriptor_id | yes | string |  | Identifier of this VNFD information element. This attribute shall be globally unique. See note 3. The VNFD Identifier shall be used as the unique identifier of the VNF Package that contains this VNFD. Any modification of the content of the VNFD or the VNF Package shall result in a new VNFD Identifier. |
| descriptor_version | yes | string |  | Identifies the version of the VNFD. |
| provider | yes | string |  | Provider of the VNF and of the VNFD. |
| product_name | yes | string |  | Name to identify the VNF Product. Invariant for the VNF Product lifetime. |
| software_version | yes | string |  | Software version of the VNF. This is changed when there is any change to the software that is included in the VNF Package. |
| product_info_name | no | string |  | Human readable name for the VNF Product. Can change during the VNF Product lifetime. |
| product_info_description | no | string |  | Human readable description of the VNF Product. Can change during the VNF Product lifetime. |
| vnfm_info | yes | list of string |  | Identifies VNFM(s) compatible with the VNF described in this version of the VNFD. To indicate that a VNF can be managed by any ETSI NFV compliant VNFM, the string value shall be the concatenation of the string "etsivnfm" and the minimum version of ETSI GS NFV-SOL 002 [i.4] to be supported by this VNFM (e.g. etsivnfm:v2.3.1). If the VNF is compatible with multiple versions, multiple values may be included. See note 1. To indicate a specific VNFM product, the string value shall be the concatenation of the IANA enterprise number of the VNFM provider [5], followed by a product-specific string. |
| localization_languages | no | list of string | Valid values: string values that comply with IETF RFC 5646 [13] | Information about localization languages of the VNF (includes e.g. strings in the VNFD). This allows to provide one or more localization languages to support selecting a specific localization language at VNF instantiation time. |
| default_localization_language | no | string | Valid values: string values that comply with IETF RFC 5646 [13] | Default localization language that is instantiated if no information about selected localization language is available. Shall be present if "localizationLanguage" is present and shall be absent otherwise. |
| configurable_properties | no | tosca.datatypes.nfv.VnfConfigurableProperties |  | Describes the configurable properties of the VNF (e.g. related to auto scaling and auto healing). |
| modifiable_attributes | no | tosca.datatypes.nfv.VnfInfoModifiableAttributes |  | Describes the modifiable attributes of the VNF. |
| lcm_operations_configuration | no | tosca.datatypes.nfv.VnfLcmOperationsConfiguration |  | Describes the configuration parameters for the VNF LCM operations. |
| monitoring_parameters | no | map of tosca.datatypes.nfv.VnfMonitoringParameter |  | Describes monitoring parameters applicable to the VNF. See note 4. |
| flavour_id | yes | string |  | Identifier of this DF within the VNFD. |
| flavour_description | yes | string |  | Human readable description of the DF. |
| vnf_profile | no | tosca.datatypes.nfv.VnfProfile |  | Describes a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF. See note 2. |
| NOTE 1: When LCM scripts are used, the support of this minimum version might not be sufficient to ensure that the VNF can be managed by a VNFM. The support of the domain specific language(s) used by these LCM scripts is another criterion for determining the compatibility of the VNF with a VNFM. NOTE 2: This property is only used in an NSD service template when describing a VNF node template with the corresponding VnfProfile information. NOTE 3: The value of the descriptor_id string shall comply with an UUID format as specified in section 3 of [9]. NOTE 4: This property is only used in a VNFD service template when describing a VNF node template with the corresponding monitoring information. |  |  |  |  |

The syntax of the vnfm_info string values shall comply with the following ABNF [6] snippet:

value = any_etsi_nfv_compliant_product| product_specific

any_etsi_nfv_compliant_product = "etsivnfm" SEP "version"

version = "v" version_identifier

version_identifier = 1*2DIGIT DOT 1*2DIGIT DOT 1*2DIGIT

; the version identifier is encoded as a sequence of items of 1 or 2 digits separated by dots representing the 3 fields (major, technical and editorial) of the version of an ETSI deliverable.

product_specific = enterprise_number SEP product_specific_string

enterprise_number = 1*DIGIT

product_specific_string = *(ALPHA / DIGIT / "-" / ".")

SEP = ":"

DOT = "."

This implies that vnfm_info string values shall also comply with the pattern defined by the following regular expression [15]: (^etsivnfm:v[0-9]?[0-9]\.[0-9]?[0-9]\.[0-9]?[0-9]$)|(^[0-9]+:[a-zA-Z0-9.-]+$)

#### 6.8.1.3	Attributes

The attribute of the VNF node type shall comply with the provisions set out in table 6.8.1.3-1.

Table 6.8.1.3-1: Attributes

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scale_status | no | map of tosca.datatypes.nfv.ScaleInfo |  | Scale status of the VNF, one entry per aspect. Represents for every scaling aspect how "big" the VNF has been scaled w.r.t. that aspect. |

If the VNF supports VNF indicators, the VNF node type definition shall include one TOSCA attribute of a primitive type for each supported VNF indicator.

NOTE 1:	In this version of the present document, the type of VNF indicators is constrained to primitive types. This is due to the limitations in the TOSCA-Simple-Profile-YAML-v1.3 [20] to define conditions on attributes of complex types.

If the scale_status attribute is used in a VNF indicator policy, e.g. an auto-scale policy, the VNF specific node type definitions may include additional attribute definitions of type integer, one for each scaling aspect. See example in clause A.15.

NOTE 2:	As the scale_status attribute is complex, the scale_level property of the individual scaling aspects can be retrieved by passing a path to the get_attribute function: { get_attribute: [ SELF, scale_status, {scaling_aspect}, scale_level ] }. If the value of the scale_level property is needed in a constraint (tosca.policies.nfv.VnfIndicator), then the value needs to be retrieved in an indirect way. This is due to the limitation mentioned in the previous note.

VNF indicators may be defined in the VNFD to allow for the asynchronous notification of VNF specific information to the VNFM.

An attribute defined in the VNF node type for a VNF indicator holds the value for that indicator during the lifecycle of the VNF. A notification defined in the derived interface for VNF indicators (see clause 6.7.2) produces an output value which is assigned to the attribute, as per TOSCA Simple Profile in YAML 1.3 syntax. Examples of such assignments are shown in clause 6.8.1.9. Thus, the value of the VNF indicator may change every time a notification is received.

#### 6.8.1.4	Requirements

The requirements of the VNF node type shall comply with the provisions set out in table 6.8.1.4-1.

Table 6.8.1.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link |

#### 6.8.1.5	Capabilities

None.

#### 6.8.1.6	Definition

The syntax of the VNF node type shall comply with the following definition:

tosca.nodes.nfv.VNF: derived_from: tosca.nodes.Root description: The generic abstract type from which all VNF specific node types shall be derived to form, together with other node types, the TOSCA service template(s) representing the VNFD properties: descriptor_id: # instead of vnfd_id type: string # UUID description: Identifier of this VNFD information element. This attribute shall be globally unique required: true descriptor_version: # instead of vnfd_version type: string description: Identifies the version of the VNFD required: true provider: # instead of vnf_provider type: string description: Provider of the VNF and of the VNFD required: true product_name: # instead of vnf_product_name type: string description: Human readable name for the VNF Product required: true software_version: # instead of vnf_software_version type: string description: Software version of the VNF required: true product_info_name: # instead of vnf_product_info_name type: string description: Human readable name for the VNF Product required: false product_info_description: # instead of vnf_product_info_description type: string description: Human readable description of the VNF Product required: false vnfm_info: type: list required: true description: Identifies VNFM(s) compatible with the VNF entry_schema: type: string constraints: - pattern: (^etsivnfm:v[0-9]?[0-9]\.[0-9]?[0-9]\.[0-9]?[0-9]$)¦(^[0-9]+:[a-zA-Z0-9.-]+$) localization_languages: type: list description: Information about localization languages of the VNF required: false entry_schema: type: string #IETF RFC 5646 string default_localization_language: type: string #IETF RFC 5646 string description: Default localization language that is instantiated if no information about selected localization language is available required: false #configurable_properties: #type: tosca.datatypes.nfv.VnfConfigurableProperties #description: Describes the configurable properties of the VNF #required: false # derived types are expected to introduce configurable_properties # with its type derived from tosca.datatypes.nfv.VnfConfigurableProperties #modifiable_attributes: #type: tosca.datatypes.nfv.VnfInfoModifiableAttributes #description: Describes the modifiable attributes of the VNF #required: false # derived types are expected to introduce modifiable_attributes # with its type derived from # tosca.datatypes.nfv.VnfInfoModifiableAttributes lcm_operations_configuration: type: tosca.datatypes.nfv.VnfLcmOperationsConfiguration description: Describes the configuration parameters for the VNF LCM operations required: false monitoring_parameters: type: map # key: id entry_schema: type: tosca.datatypes.nfv.VnfMonitoringParameter description: Describes monitoring parameters applicable to the VNF. required: false flavour_id: type: string description: Identifier of the Deployment Flavour within the VNFD required: true flavour_description: type: string description: Human readable description of the DF required: true vnf_profile: type: tosca.datatypes.nfv.VnfProfile description: Describes a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF required: false attributes: scale_status: type: map # key: aspectId description: Scale status of the VNF, one entry per aspect. Represents for every scaling aspect how "big" the VNF has been scaled w.r.t. that aspect. entry_schema: type: tosca.datatypes.nfv.ScaleInfo requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [ 0, 1 ] # Additional requirements shall be defined in the VNF specific node type (deriving from tosca.nodes.nfv.VNF) corresponding to NS virtual links that need to connect to VnfExtCps interfaces: Vnflcm: type: tosca.interfaces.nfv.Vnflcm # VnfIndicator: # type: tosca.interfaces.nfv.VnfIndicator # derived types are expected to introduce Vnf Indicator interfaces # with their type derived from tosca.interfaces.nfv.VnfIndicator

#### 6.8.1.7	Artifact

None.

#### 6.8.1.8	Additional Requirements

For a given VNFD, a new VNF node type shall be defined following the below requirements:

- The node type shall be derived from: tosca.nodes.nfv.VNF.

The following properties listed in tosca.nodes.nfv.VNF where the "required:" field is set to "true" shall be included with their values indicated as constraints and as default values:

  - descriptor_id

  - descriptor_version

  - provider

  - product_name

  - software_version

  - vnfm_info

  - flavour_id

  - flavour_description

NOTE 1:	Indicating their values as default allows not to include them in property assignments in node templates, e.g. in the NSD.

An empty string shall be indicated as the default value of the flavour_description property.

The capabilities, requirements, interfaces of tosca.nodes.nfv.VNF shall be preserved.

Depending on the number of external connection points of the VNF that need to connect to NS virtual links, additional requirements for VirtualLinkable capability shall be defined with the occurrences set to [ 0, 1 ]. In this case, it is the VNFD author's choice to use the requirement for VirtualLinkable capability defined in the tosca.nodes.nfv.VNF node type or use only the additional requirements defined in the derived VNF specific node type. In the latter case, the virtual_link requirement should be included in the node type definition with occurrences [ 0, 0 ].

- If the external connection point exposes a VipCp, a new requirement for VirtualLinkableCapability using the VipVirtualLinksTo relationship shall be defined for this connection point.

The rule for naming this node type in the service template should be:

provider.product_name.software_version.descriptor_version, by concatenating the values of the corresponding properties of the created VNF node type.

NOTE 2:	If the software_version value or descriptor_version value contains a dot (i.e. "."), this character should be replaced with an underscore (i.e. "_").

If the VNF supports VNF indicators, the VNF node type definition shall include an interface definition of a VNF specific interface type indicating the mapping of notification outputs to the VNF node attributes and, optionally, tosca.policies.nfv.VnfIndicator policies that may invoke auto-scale or auto-heal operations. For each of the VNF indicators, the name of the notification output shall be the same as the name of the corresponding VNF attribute.

NOTE 3:	The notifications keyname in TOSCA interface is defined in TOSCA-Simple-Profile-YAML-v1.3 [20].

If "additionalParams" are expected in the Change current VNF Package request on the API (NFV-SOL 003 or NFV-SOL 002), then they shall be defined as "additional_parameters" inputs of the change_current_package operation on the Vnflcm interface (in case the same LCM script with the same set of "additionalParams" is suitable for all change paths) or the VNF-specific operations on the ChangeCurrentVnfPackage interface (in case different change paths require different LCM scripts potentially with different sets of "additionalParams").

VNF Providers shall use the following types to derive the VNF specific modifiable attributes and additional configurable properties:

tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions.

tosca.datatypes.nfv.VnfInfoModifiableAttributesMetadata.

tosca.datatypes.nfv.VnfAdditionalConfigurableProperties.

tosca.datatypes.nfv.VnfInfoModifiableAttributes.

tosca.datatypes.nfv.VnfConfigurableProperties.

See illustrative examples in clauses 6.8.1.9, 6.2.35.4 and 6.2.31.4.

In the derived VNF node type, the modifiable_attributes and configurable_properties (VNF-specific extension of the tosca.datatypes.nfv.VnfInfoModifiableAttributes and tosca.datatypes.nfv.VnfConfigurableProperties, respectively, by extending the above listed types) describe the name and type of the modifiable attributes (extensions and metadata) and configurable properties (vnfConfigurableProperties).

The modifiable_attributes and configurable_properties information provided in the node type is sufficient for the client of the VNF LCM API for providing values to these properties. A value provided via the VNF LCM API to such a property overrides the value (if any) assigned in the node template or defined as default value in the node type definition.

Node templates of the VNF specific node type shall not include the vnf_profile property when they are part of a VNFD service template.

For a given NSD, when describing a referenced VNFD as a node templates, the vnf_profile property shall be included with a valid value.

For a given NSD, when describing a referenced VNFD as a node templates, the monitoring_parameters property shall not be included.

#### 6.8.1.9	Example

Example usage of modifiable_attributes:

tosca_definitions_version: tosca_simple_yaml_1_3 node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: constraints: - valid_values: [ simple, complex ] modifiable_attributes: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes data_types: mycompany.datatypes.nfv.VnfInfoModifiableAttributes: derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributes properties: extensions: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions: derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions properties: http_proxy: type: string required: true https_proxy: type: string required: false

Example usage of lcm_operations_configuration:

Top level service template:

tosca_definitions_version: tosca_simple_yaml_1_3 node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: type: string constraints: - valid_values: [ simple, complex ] lcm_operations_configuration: scale: scaling_by_more_than_one_step_supported: true scale_to_level: arbitrary_target_levels_supported: true heal: causes: - service_unavailable - performance_degraded terminate: min_graceful_termination_timeout: 60 s max_recommended_graceful_termination_timeout: 600 s operate: min_graceful_stop_timeout: 60 s max_recommended_graceful_stop_timeout: 600 s

Example usage of describing a VNF node template with vnf_profile in an NSD TOSCA service template:

tosca_definitions_version: tosca_simple_yaml_1_3 description: an example of NSD TOSCA service template topology template: … node_templates: VNF_1: type: tosca.nodes.nfv.exampleVNF properties: flavour_id: small descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 provider: MyCompany product_name: SunshineDB software_version: 1.0 descriptor_version: 1.0 vnf_profile: instantiation_Level: level_1 min_number_of_instances: 2 max_number_of_instances: 6 # other properties omitted for brevity requirements: - virtual_link: NsVirtualLink

Example usage of VNF indicators attributes in VNF node type definition and VNF indicator notifications in interface definition:

tosca_definitions_version: tosca_simple_yaml_1_3 interface_types: tosca.nfv.interfaces.MyCompanyVnfIndicator derived_from: tosca.nfv.interfaces.VnfIndicator notifications: health: description: this notification is used to received asynchronous information of value change of the health_vnf_indicator utilization: description: this notification is used to received asynchronous information of value change of the utilization_vnf_indicator node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: flavour_id: constraints: - valid_values: [ simple, complex ] modifiable_attributes: type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes attributes: health_vnf_indicator: type: string constraints: - valid_values: [ green, red, yellow ] utilization_vnf_indicator: type: float constraints: - in_range [ 0.0, 100.0 ] interfaces: Vnflcm: type: tosca.interfaces.nfv.Vnflcm operations: # omitted for brevity VnfIndicator: type: tosca.interfaces.nfv.MyCompanyVnfIndicator notifications: health: output: health_vnf_indicator: [ SELF, health_vnf_indicator ] utilization: output: utilization_vnf_indicator: [ SELF, utilization_vnf_indicator ]

### 6.8.2	tosca.nodes.nfv.VnfExtCp

#### 6.8.2.1	Description

The VnfExtCp node type represents the VnfExtCpd information element as defined in ETSI GS NFV-IFA 011 [1], which describes a logical external connection point, exposed by this VNF enabling connecting with an external Virtual Link. Table 6.8.2.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.2.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfExtCp |
| --- | --- |
| Type Qualified Name | toscanfv:VnfExtCp |
| Type URI | tosca.nodes.nfv.VnfExtCp |

#### 6.8.2.2	Properties

The properties of the VnfExtCp node type shall comply with the provisions set out in table 6.8.2.2-1.

Table 6.8.2.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_network_interface_requirements | no | list of tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements |  | The actual virtual NIC requirements that is been assigned when instantiating the connection point. |

#### 6.8.2.3	Attributes

None.

#### 6.8.2.4	Requirements

The requirements of the VnfExtCp node type shall comply with the provisions set out in table 6.8.2.4-1.

Table 6.8.2.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| external_virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |
| internal_virtual_link | yes | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |

#### 6.8.2.5	Capabilities

None.

#### 6.8.2.6	Definition

The syntax of the VnfExtCp node type shall comply with the following definition:

tosca.nodes.nfv.VnfExtCp: derived_from: tosca.nodes.nfv.Cp description: Describes a logical external connection point, exposed by the VNF enabling connection with an external Virtual Link properties: virtual_network_interface_requirements: type: list description: The actual virtual NIC requirements that is been assigned when instantiating the connection point required: false entry_schema: type: tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements requirements: - external_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [0, 1] - internal_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [1, 1]

#### 6.8.2.7	Additional Requirements

A node template of this type is used to represent a VNF external connection point only in the case the VnfExtCp is connected to an internal virtual link. The node template has the following requirements:

internal_virtual_link requirement to allow to connect it to an internal virtual link;

external_virtual_link requirement to allow to connect it to an external virtual link.

In the case where a VNF external connection point is re-exposing a VduCp (internal connection point), the VduCp node type shall be used in the service template, instead of the VnfExtCp node type.

#### 6.8.2.8	Example

In a typical scenario, the VnfExtCp node template will be part of a service template representing a certain VNF deployment flavour. The service template substitutes for a VNF specific node type. In this substitution, the virtual_link requirement is mapped to the external_virtual_link requirement of the VnfExtCp node. This example is illustrated in clause A.3.3.

When a VNF external connection point re-exposes a Vdu connection point, the service template does not require an explicit node template of type VnfExtCp in a typical scenario where a VNF specific node type is substituted by a service template representing a certain VNF deployment flavour. In this substitution, the virtual_link requirement is mapped to the virtual_link requirement of the VduCp node. This example is illustrated in clause A.3.2.

### 6.8.3	tosca.nodes.nfv.Vdu.Compute

#### 6.8.3.1	Description

The Vdu.Compute node type describes the virtual compute part of a VDU which is a construct supporting the description of the deployment and operational behaviour of a VNFC, as defined in ETSI GS NFV-IFA 011 [1].

Table 6.8.3.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.3.1-1: Type name, shorthand, and URI

| Shorthand Name | Vdu.Compute |
| --- | --- |
| Type Qualified Name | toscanfv:Vdu.Compute |
| Type URI | tosca.nodes.nfv.Vdu.Compute |

#### 6.8.3.2	Properties

The properties of the Vdu.Compute node type shall comply with the provisions set out in table 6.8.3.2-1.

Table 6.8.3.2-1: Properties

| Name | Required | Type | Constraints | Status | Description |
| --- | --- | --- | --- | --- | --- |
| name | yes | string |  |  | Human readable name of the Vdu. |
| description | yes | string |  |  | Human readable description of the Vdu. |
| boot_order | yes | boolean |  |  | It indicates whether the order of the virtual_storage requirements is used as the boot index (the first requirement represents the lowest index and defines highest boot priority). If no boot order is indicated or the value is false, the default boot order defined in the VIM or NFVI shall be used. |
| nfvi_constraints | no | map of string |  |  | Describes constraints on the NFVI for the VNFC instance(s) created from this Vdu. For example, aspects of a secure hosting environment for the VNFC instance that involve additional entities or processes. This property is reserved for future use in the present document. |
| monitoring_parameters | no | map of tosca.datatypes.nfv.VnfcMonitoringParameter |  |  | Describes monitoring parameters applicable to a VNFC based on this VDU. |
| configurable_properties | no | map of tosca.datatypes.nfv.VnfcConfigurableProperties |  |  | Describes the configurable properties of all VNFC instances based on this VDU. |
| boot_data | no | tosca.datatypes.nfv.BootData |  |  | Contains the information used to customize a virtualised compute resource at boot time. See note. The bootData may contain variable parts that are replaced by deployment specific values before being sent to the VIM. For "volatile" parameters, i.e. those that exist only during the lifetime of an LCM operation occurrence, the parameters of each variable part shall be declared in a type derived from tosca.datatypes.nfv.VnfOperationAdditionalParameters. For "persistent" parameters, i.e. those that exist during the lifetime of the VNF instance beyond the lifetime of a single LCM operation occurrence, the parameters shall be declared in a type derived from tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions or tosca.datatypes.nfv.VnfConfigurableProperties. |
| vdu_profile | yes | tosca.datatypes.nfv.VduProfile |  |  | Defines additional instantiation data for the VDU.Compute node. |
| sw_image_data | no | tosca.datatypes.nfv.SwImageData |  | deprecated | Defines information related to a SwImage artifact used by this Vdu.Compute node. |
| NOTE: The boot_data structure passed to a VNFC instance cannot be changed after the boot time of the VNFC instance. |  |  |  |  |  |

#### 6.8.3.3	Attributes

None.

#### 6.8.3.4	Requirements

The requirements of the Vdu.Compute node type shall comply with the provisions set out in table 6.8.3.4-1.

Table 6.8.3.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_storage | no | tosca.capabilities.nfv.VirtualStorage |  | Describes storage requirements for a virtual_storage instance attached to the virtualisation container created from virtual_compute defined for this vdu |

#### 6.8.3.5	Capabilities

The capabilities of the Vdu.Compute node type shall comply with the provisions set out in table 6.8.3.5-1.

Table 6.8.3.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_compute | tosca.capabilities.nfv.VirtualCompute |  | Describes virtual compute resources capabilities. |
| virtual_binding | tosca.capabilities.nfv.VirtualBindable |  | Defines ability of VirtualBindable. See note. |
| NOTE: Due to current grammar limitations in TOSCA-Simple-Profile-YAML-v1.3 [20], the Vdu.Compute node template cannot limit the number of the virtual_binding capability occurrences. It is the VNFD author's responsibility to make sure that the number of VduCps connected to a particular Vdu.Compute node is compatible with the virtualised resources assigned to this node. |  |  |  |

#### 6.8.3.6	Definition

The syntax of the Vdu.Compute node type shall comply with the following definition:

tosca.nodes.nfv.Vdu.Compute: derived_from: tosca.nodes.Root description: Describes the virtual compute part of a VDU which is a construct supporting the description of the deployment and operational behavior of a VNFC properties: name: type: string description: Human readable name of the VDU required: true description: type: string description: Human readable description of the VDU required: true boot_order: type: boolean description: indicates whether the order of the virtual_storage requirements is used as the boot index (the first requirement represents the lowest index and defines highest boot priority) required: true default: false nfvi_constraints: type: map description: Describes constraints on the NFVI for the VNFC instance(s) created from this VDU. This property is reserved for future use in the present document. required: false entry_schema: type: string monitoring_parameters: type: map # key: id description: Describes monitoring parameters applicable to a VNFC instantiated from this VDU required: false entry_schema: type: tosca.datatypes.nfv.VnfcMonitoringParameter #configurable_properties: #type: tosca.datatypes.nfv.VnfcConfigurableProperties #required: false # derived types are expected to introduce # configurable_properties with its type derived from # tosca.datatypes.nfv.VnfcConfigurableProperties vdu_profile: type: tosca.datatypes.nfv.VduProfile description: Defines additional instantiation data for the VDU.Compute node required: true sw_image_data: type: tosca.datatypes.nfv.SwImageData description: Defines information related to a SwImage artifact used by this Vdu.Compute node required: false # property is required when the node template has an associated artifact of type tosca.artifacts.nfv.SwImage and not required otherwise status: deprecated boot_data: type: tosca.datatypes.nfv.BootData description: Contains the information used to customize a virtualised compute resource at boot time. The bootData may contain variable parts that are replaced by deployment specific values before being sent to the VIM. required: false capabilities: virtual_compute: type: tosca.capabilities.nfv.VirtualCompute occurrences: [ 1, 1 ] virtual_binding: type: tosca.capabilities.nfv.VirtualBindable occurrences: [ 1, UNBOUNDED ] requirements: - virtual_storage: capability: tosca.capabilities.nfv.VirtualStorage relationship: tosca.relationships.nfv.AttachesTo occurrences: [ 0, UNBOUNDED ]

#### 6.8.3.7	Additional requirements

Node templates of type tosca.nodes.nfv.Vdu.Compute may contain an artifact definition of type tosca.artifacts.nfv.SwImage. There shall be a maximum number of one such artifact definition in a tosca.nodes.nfv.Vdu.Compute node template. The node template name of type tosca.nodes.nfv.Vdu.Compute fulfills the purpose of the "SwImageDescId" attribute of the SwImageDesc information element in ETSI GS NFV-IFA 011 [1] and hence it will be used in APIs to identify the software image id from the VNFD perspective. See example in clause 6.8.3.8.

When VNF-specific configurable properties are defined at the VDU-level, VNF providers shall define a VNF/VDU specific Vdu.Compute node type, where the configurable_properties property has a datatype derived from tosca.datatypes.nfv.VnfcConfigurableProperties. See example in clause 6.2.10.4.

The VNF/VDU specific Vdu.Compute node type shall be defined as follows:

All properties listed in tosca.nodes.nfv.Vdu.Compute where the "required:" field is set to "true" shall be included.

The capabilities and requirements of tosca.nodes.nfv.Vdu.Compute shall be preserved.

The configurable_properties property shall have a datatype derived from tosca.datatypes.nfv.VnfcConfigurableProperties, according to the rules defined in clause 5.7.2 of the present document.

The definition of a VNF/VDU specific node type shall be included in one of the following yaml files:

In the yaml file which contains the corresponding VNF specific node type definition.

In low-level service templates or in the single TOSCA service template representing the VNFD in case of a single deployment flavour design with a single TOSCA service template.

In a standalone yaml file, to be imported from the low-level TOSCA service templates or from the single TOSCA service template representing the VNFD in case of a single deployment flavour design with a single TOSCA service template.

In any other VNF-specific files containing type definitions used by the VNFD TOSCA service template.

In the derived Vdu.Compute node type, the additional_vnfc_configurable_properties (VNF/VDU-specific extension of the tosca.datatypes.nfv.VnfAdditionalVnfcConfigurableProperties data type and the containing tosca.datatypes.nfv.VnfcConfigurableProperties data type) describe the name and type of the VNFC configurable properties.

The additional_vnfc_configurable_properties information provided in the node type is sufficient for the client of the VNF LCM API for providing values to these properties. A value provided via the VNF LCM API to such a property overrides the value (if any) assigned in the node template or defined as default value in the node type definition.

The node template name of type tosca.nodes.nfv.Vdu.Compute fulfills the purpose of the 'virtualComputeDescId' attribute of the virtualComputeDesc information element in ETSI GS NFV-IFA 011 [1] and hence it will be used in APIs to identify the virtual compute id (vnfdVirtualComputeDescId).

NOTE:	The use of the node template name of type tosca.nodes.nfv.Vdu.Compute for the 'virtualComputeDescId' attribute of the virtualComputeDesc information element in ETSI GS NFV-IFA 011 [1] implies in the present document version a one-to-one mapping of virtualComputeDesc with VDU. This deviates from the ETSI GS NFV-IFA 011 [1] modelling that defines a mapping where a virtualComputeDesc can be reused by one or more VDU, i.e. it implies a one-to-many mapping of virtualComputeDesc with VDU. This can have an impact in the determination of the number of compute flavours needed to be created with the VIM.

Use of sw_image_data property is deprecated. For backward compatibility, the sw_image_data property may be used in node templates of type tosca.nodes.nfv.Vdu.Compute containing an artifact definition of type tosca.artifacts.nfv.SwImage. The properties of the SwImage artifact as defined in clause 6.3.1.2 should be used instead. Either the sw_image_data property of the Vdu.Compute node template or the properties of the SwImage artefact shall be present but not both. Support of the sw_image_data property can be removed in subsequent versions of the present document.

#### 6.8.3.8	Example

This example illustrates boot data containing kvp_data by using modifiable_attributes.

tosca_definitions_version: tosca_simple_yaml_1_3

..

node_types:

mycompany.nodes.nfv.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

..

modifiable_attributes:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes

..

data_types:

mycompany.datatypes.nfv.VnfInfoModifiableAttributes:

derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributes

properties:

extensions:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions

required: false

mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions:

derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions

properties:

http_proxy:

type: string

required: true

https_proxy:

type: string

required: false

ip_address_1:

type: string

required: false

vm_Nname:

type: string

required: false

topology_template:

inputs:

extensions:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions

substitution_mappings:

node_type: mycompany.nodes.nfv.SunshineDB.1_0.1_0

..

node_templates:

vnf:

type: mycompany.nodes.nfv.SunshineDB.1_0.1_0

properties:

..

modifiable_attributes:

extensions: { get_input: extensions }

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

boot_data:

kvp_data:

data:

ip_address_1: { get_property: [vnf, modifiable_attributes, extensions, ip_address_1 ] }

..

This example illustrates boot data containing kvp_data by using configurable_properties.

tosca_definitions_version: tosca_simple_yaml_1_3

..

node_types:

mycompany.nodes.nfv.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

..

configurable_properties:

type: mycompany.datatypes.nfv.VnfConfigurableProperties

..

data_types:

mycompany.datatypes.nfv.VnfConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfConfigurableProperties

properties:

additional_configurable_properties:

type: mycompany.datatypes.nfv.VnfAdditionalConfigurableProperties

required: false

mycompany.datatypes.nfv.VnfAdditionalConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfAdditionalConfigurableProperties

properties:

host_name:

type: string

required: false

topology_template:

inputs:

configurable_properties:

type: mycompany.datatypes.nfv.VnfConfigurableProperties

substitution_mappings:

node_type: mycompany.nodes.nfv.SunshineDB.1_0.1_0

..

node_templates:

vnf:

type: mycompany.nodes.nfv.SunshineDB.1_0.1_0

properties:

..

configurable_properties: { get_input: configurable_properties }

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

boot_data:

kvp_data:

data:

ip_address_1: { get_property: [vnf, configurable_properties, additional_configurable_properties, host_name ] }

..

This example illustrates fetching the boot data value by using content_or_file_data.

tosca_definitions_version: tosca_simple_yaml_1_3

..

node_types:

mycompany.nodes.nfv.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

..

modifiable_attributes:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributes

..

data_types:

mycompany.datatypes.nfv.VnfInfoModifiableAttributes:

derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributes

properties:

extensions:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions

required: false

mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions:

derived_from: tosca.datatypes.nfv.VnfInfoModifiableAttributesExtensions

properties:

http_proxy:

type: string

required: true

https_proxy:

type: string

required: false

ip_address_1:

type: string

required: false

vm_name:

type: string

required: false

topology_template:

inputs:

extensions:

type: mycompany.datatypes.nfv.VnfInfoModifiableAttributesExtensions

substitution_mappings:

node_type: mycompany.nodes.nfv.SunshineDB.1_0.1_0

..

node_templates:

vnf:

type: mycompany.nodes.nfv.SunshineDB.1_0.1_0

properties:

..

modifiable_attributes:

extensions: { get_input: extensions }

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

boot_data:

content_or_file_data

contents: { concat: [ "#!/bin/bash\n",

"echo setting HTTP proxy to: ", { get_property: [vnf, modifiable_attributes, extensions, http_proxy ] }, "\n", "..."] }

This example illustrates fetching the boot data value from a file by using content_or_file_data.

tosca_definitions_version: tosca_simple_yaml_1_3

..

topology_template:

..

node_templates:

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

boot_data:

content_or_file_data

data:

http_proxy: { get_property: [vnf, modifiable_attributes, extensions, http_proxy ] }

source_path: { get_artifact : [ SELF, boot_data ] }

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: images/dbBackend.v1.0.1.qcow2

boot_data:

type: tosca.artifacts.example

file: implementation/templates/boot_data.file

..

This example illustrates fetching the boot data value from a file by using content_or_file_data with destination_path.

tosca_definitions_version: tosca_simple_yaml_1_3

..

topology_template:

..

node_templates:

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

boot_data:

content_or_file_data:

data:

vm_name: get_property: [vnf, modifiable_attributes, extensions, vm_name ]

source_path: { get_artifact : [ SELF, boot_data ] }

destination_path: /etc/

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: images/dbBackend.v1.0.1.qcow2

boot_data:

type: tosca.artifacts.example

file: implementation/templates/boot_data.file

..

This example illustrates the association of a software image artifact to a Vdu.Compute node. The name of the artifact "dbBackend" will be used in external APIs to identify the image.

tosca_definitions_version: tosca_simple_yaml_1_3

..

topology_template:

..

node_templates:

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

..

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: images/dbBackend.v1.0.1.qcow2

properties:

..

This example illustrates the association of a software image artifact to more than Vdu.Compute nodes. The name of the artifact "dbBackend and oamService" will be used in external APIs to identify the image of each Vdu.Compute node.

tosca_definitions_version: tosca_simple_yaml_1_3

..

topology_template:

..

node_templates:

dbBackend:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

..

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: images/dbBackend.v1.0.1.qcow2

properties:

oamService:

type: tosca.nodes.nfv.Vdu.Compute

properties:

..

..

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: images/dbBackend.v1.0.1.qcow2

properties:

..

### 6.8.4	tosca.nodes.nfv.Vdu.VirtualBlockStorage

#### 6.8.4.1	Description

The VirtualBlockStorage node type describes the specifications of requirements related to virtual block storage resources, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.8.4.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.4.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualBlockStorage |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualBlockStorage |
| Type URI | tosca.nodes.nfv.Vdu.VirtualBlockStorage |

#### 6.8.4.2	Properties

The properties of the VirtualBlockStorage node type shall comply with the provisions set out in table 6.8.4.2-1.

Table 6.8.4.2-1: Properties

| Name | Required | Type | Constraints | Status | Description |
| --- | --- | --- | --- | --- | --- |
| virtual_block_storage_data | yes | tosca.datatypes.nfv.VirtualBlockStorage |  |  | Describes the block storage characteristics. |
| sw_image_data | no | tosca.datatypes.nfv.SwImageData |  | deprecated | Defines information related to a SwImage artifact used by this Vdu.Compute node. |

#### 6.8.4.3	Attributes

None.

#### 6.8.4.4	Requirements

None.

#### 6.8.4.5	Capabilities

The capabilities of the VirtualBlockStorage node type shall comply with the provisions set out in table 6.8.4.5-1.

Table 6.8.4.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_storage | tosca.capabilities.nfv.VirtualStorage |  | Defines the capabilities of virtual_storage. |

#### 6.8.4.6	Definition

The syntax of the VirtualBlockStorage node type shall comply with the following definition:

tosca.nodes.nfv.Vdu.VirtualBlockStorage: derived_from: tosca.nodes.Root description: This node type describes the specifications of requirements related to virtual block storage resources properties: virtual_block_storage_data: type: tosca.datatypes.nfv.VirtualBlockStorageData description: Describes the block storage characteristics. required: true sw_image_data: type: tosca.datatypes.nfv.SwImageData description: Defines information related to a SwImage artifact used by this Vdu.Compute node. required: false # property is required when the node template has an associated artifact of type tosca.artifacts.nfv.SwImage and not required otherwise status: deprecated capabilities: virtual_storage: type: tosca.capabilities.nfv.VirtualStorage description: Defines the capabilities of virtual_storage.

#### 6.8.4.7	Additional requirements

Node templates of type tosca.nodes.nfv.Vdu.VirtualBlockStorage may contain an artifact definition of type tosca.artifacts.nfv.SwImage. There shall be a maximum number of one such artifact definition in a tosca.nodes.nfv.Vdu.VirtualBlockStorage node template. The node template name of type tosca.nodes.nfv.Vdu.VirtualBlockStorage fulfills the purpose of the "SwImageDescId" attribute of the SwImageDesc information element in ETSI GS NFV-IFA 011 [1] and hence it will be used in APIs to identify the software image id from the VNFD descriptor. See example in clause 6.8.3.8.

Use of sw_image_data property is deprecated. For backward compatibility, the sw_image_data property may be used in node templates of type tosca.nodes.nfv.Vdu.VirtualBlockStorage containing an artifact definition of type tosca.artifacts.nfv.SwImage. The properties of the SwImage artifact as defined in clause 6.3.1.2 should be used instead. Either the sw_image_data property of the VirtualBlockStorage node template or the properties of the SwImage artefact shall be present but not both. Support of the sw_image_data property can be removed in subsequent versions of the present document.

### 6.8.5	tosca.nodes.nfv.Vdu.VirtualObjectStorage

#### 6.8.5.1	Description

The VirtualObjectStorage node type describes the specifications of requirements related to virtual object storage resources, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.8.5.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.5.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualObjectStorage |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualObjectStorage |
| Type URI | tosca.nodes.nfv.Vdu.VirtualObjectStorage |

#### 6.8.5.2	Properties

The properties of the VirtualObjectStorage node type shall comply with the provisions set out in table 6.8.5.2-1.

Table 6.8.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_object_storage_data | yes | tosca.datatypes.nfv.VirtualObjectStorageData |  | Describes the object storage characteristics. |

#### 6.8.5.3	Attributes

None.

#### 6.8.5.4	Requirements

None.

#### 6.8.5.5	Capabilities

The capabilities of the VirtualObjectStorage node type shall comply with the provisions set out in table 6.8.5.5-1.

Table 6.8.5.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_storage | tosca.capabilities.nfv.VirtualStorage |  | Defines the capabilities of virtual_storage. |

#### 6.8.5.6	Definition

The syntax of the VirtualObjectStorage node type shall comply with the following definition:

tosca.nodes.nfv.Vdu.VirtualObjectStorage: derived_from: tosca.nodes.Root description: This node type describes the specifications of requirements related to virtual object storage resources properties: virtual_object_storage_data: type: tosca.datatypes.nfv.VirtualObjectStorageData description: Describes the object storage characteristics. required: true capabilities: virtual_storage: type: tosca.capabilities.nfv.VirtualStorage description: Defines the capabilities of virtual_storage.

#### 6.8.5.7	Additional requirements

None.

### 6.8.6	tosca.nodes.nfv.Vdu.VirtualFileStorage

#### 6.8.6.1	Description

The VirtualFileStorage node type describes the specifications of requirements related to virtual file storage resources, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.8.6.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.6.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualFileStorage |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualFileStorage |
| Type URI | tosca.nodes.nfv.Vdu.VirtualFileStorage |

#### 6.8.6.2	Properties

The properties of the VirtualFileStorage node type shall comply with the provisions set out in table 6.8.6.2-1.

Table 6.8.6.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_file_storage_data | yes | tosca.datatypes.nfv.VirtualFileStorageData |  | Describes the file storage characteristics. |

#### 6.8.6.3	Attributes

None.

#### 6.8.6.4	Requirements

The requirements of the VirtualFileStorage node type shall comply with the provisions set out in table 6.8.6.4-1.

Table 6.8.6.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_link | yes | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link |

#### 6.8.6.5	Capabilities

The capabilities of the VirtualFileStorage node type shall comply with the provisions set out in table 6.8.6.5-1.

Table 6.8.6.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_storage | tosca.capabilities.nfv.VirtualStorage |  | Defines the capabilities of virtual_storage. |

#### 6.8.6.6	Definition

The syntax of the VirtualFileStorage node type shall comply with the following definition:

tosca.nodes.nfv.Vdu.VirtualFileStorage: derived_from: tosca.nodes.Root description: This node type describes the specifications of requirements related to virtual file storage resources properties: virtual_file_storage_data: type: tosca.datatypes.nfv.VirtualFileStorageData description: Describes the file storage characteristics. required: true capabilities: virtual_storage: type: tosca.capabilities.nfv.VirtualStorage #description: Defines the capabilities of virtual_storage. requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [1, 1] # description: Describes the requirements for linking to virtual link

#### 6.8.6.7	Additional requirements

None.

### 6.8.7	tosca.nodes.nfv.Cp

#### 6.8.7.1	Description

The Cp node type is defined in clause 9.8.1 of the present document.

### 6.8.8	tosca.nodes.nfv.VduCp

#### 6.8.8.1	Description

A VduCp node type represents the VduCpd information element as defined in ETSI GS NFV-IFA 011 [1], which describes network connectivity between a VNFC instance (based on VDU) and an internal VL. Table 6.8.8.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.8.1-1: Type name, shorthand, and URI

| Shorthand Name | VduCp |
| --- | --- |
| Type Qualified Name | toscanfv:VduCp |
| Type URI | tosca.nodes.nfv.VduCp |

#### 6.8.8.2	Properties

The properties of the VduCp node type shall comply with the provisions set out in table 6.8.8.2-1.

Table 6.8.8.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| bitrate_requirement | no | integer | greater_or_equal: 0 | Bitrate requirement in bit per second on this connection point. |
| virtual_network_interface_requirements | no | list of tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements |  | Specifies requirements on a virtual network interface realizing the CPs instantiated from this CPD. |
| order | no | integer | greater_or_equal: 0 | The order of the NIC on the compute instance (e.g. eth2). See note. compute (aka multi vNICs) and ordering is desired, it is *mandatory* that all ports will be set with an order. value and. The order values shall represent a positive, arithmetic progression that starts with 0 (e.g. 0, 1, 2, …, n). If the property is not present, it shall be left to the VIM to assign the value when creating the instance. |
| vnic_type | no | string | Allowed values: normal, macvtap, direct, baremetal,virtio-forwarder, direct-physical, smart-nic | Describes the type of the virtual network interface realizing the CPs instantiated from this CPD. This is used to determine which mechanism driver(s) to be used to bind the port. |
| Note: When binding more than one port to a single. |  |  |  |  |

#### 6.8.8.3	Attributes

None.

#### 6.8.8.4	Requirements

The requirements of the VduCp node type shall comply with the provisions set out in table 6.8.8.4-1.

Table 6.8.8.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_binding | yes | tosca.capabilities.nfv.VirtualBindable |  | Describe the requirement for binding with VDU |
| virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link |

#### 6.8.8.5	Capabilities

The capabilities of the VduCp node type shall comply with the provisions set out in table 6.8.8.5-1. This capability is available only the trunk_mode properties value of this VduCp is “true” and there is at least one VduSubCp defined as subport of the same trunk.

Table 6.8.8.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| trunk_binding | tosca.capabilities.nfv.TrunkBindable |  | Defines ability of TrunkBindable. |

#### 6.8.8.6	Definition

The syntax of the VduCp node type shall comply with the following definition:

tosca.nodes.nfv.VduCp: derived_from: tosca.nodes.nfv.Cp description: describes network connectivity between a VNFC instance based on this VDU and an internal VL properties: bitrate_requirement: type: integer # in bits per second description: Bitrate requirement in bit per second on this connection point required: false constraints: - greater_or_equal: 0 virtual_network_interface_requirements: type: list description: Specifies requirements on a virtual network interface realizing the CPs instantiated from this CPD required: false entry_schema: type: tosca.datatypes.nfv.VirtualNetworkInterfaceRequirements order: type: integer description: The order of the NIC on the compute instance (e.g.eth2) required: false constraints: - greater_or_equal: 0 vnic_type: type: string description: Describes the type of the virtual network interface realizing the CPs instantiated from this CPD required: false constraints: - valid_values: [ normal, macvtap, direct, baremetal, virtio-forwarder, direct-physical, smart-nic ] capabilities: trunk_binding: # This capability is available only the trunk_mode properties value of this VduCp is “true” and there is at least one VduSubCp defined as subport of the same trunk. type: tosca.capabilities.nfv.TrunkBindable occurrences: [ 0, UNBOUNDED ] requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [0, 1] - virtual_binding: capability: tosca.capabilities.nfv.VirtualBindable relationship: tosca.relationships.nfv.VirtualBindsTo node: tosca.nodes.nfv.Vdu.Compute occurrences: [0, 1]

#### 6.8.8.7	Additional Requirements

The occurrence 0 of the virtual_binding requirement is applicable for node templates of tosca.nodes.nfv.VduSubCp node type derived from tosca.nodes.nfv.VduCp. For node templates of tosca.nodes.nfv.VduCp node type occurrence 1 applies.

### 6.8.9	tosca.nodes.nfv.VnfVirtualLink

#### 6.8.9.1	Description

The VnfVirtualLink node type represents the VnfVirtualLinkDesc information element as defined in ETSI GS NFV-IFA 011 [1], which describes the information about an internal VNF VL. Table 6.8.9.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.9.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfVirtualLink |
| --- | --- |
| Type Qualified Name | toscanfv:VnfVirtualLink |
| Type URI | tosca.nodes.nfv.VnfVirtualLink |

#### 6.8.9.2	Properties

The properties of the VnfVirtualLink node type shall comply with the provisions set out in table 6.8.9.2-1.

Table 6.8.9.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| connectivity_type | yes | ConnectivityType |  | Specifies the protocol exposed by the VL and the flow pattern supported by the VL. |
| description | no | string |  | Provides human-readable information on the purpose of the VL (e.g. control plane traffic). |
| test_access | no | list of string | Valid values: passive_monitoring, active_loopback | Test access facilities available on the VL. |
| vl_profile | yes | tosca.datatypes.nfv.VlProfile |  | Defines additional data for the VL: maximum and minimum bit rate requirements and QoS. |
| monitoring_parameters | no | map of tosca.datatypes.nfv.VirtualLinkMonitoringParameter |  | Describe monitoring parameters applicable to a VL instantiated from this node type. |

#### 6.8.9.3	Requirements

None.

#### 6.8.9.4	Capabilities

The capabilities of the VnfVirtualLink node type shall comply with the provisions set out in table 6.8.9.4-1.

Table 6.8.9.4-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_linkable | tosca.capabilities.nfv.VirtualLinkable |  | Defines ability of VirtualLinkable. |

#### 6.8.9.5	Definition

The syntax of the VnfVirtualLink node type shall comply with the following definition:

tosca.nodes.nfv.VnfVirtualLink: derived_from: tosca.nodes.Root description: Describes the information about an internal VNF VL properties: connectivity_type: type: tosca.datatypes.nfv.ConnectivityType description: Specifies the protocol exposed by the VL and the flow pattern supported by the VL required: true description: type: string description: Provides human-readable information on the purpose of the VL required: false test_access: type: list description: Test access facilities available on the VL required: false entry_schema: type: string constraints: - valid_values: [ passive_monitoring, active_loopback ] vl_profile: type: tosca.datatypes.nfv.VlProfile description: Defines additional data for the VL required: true monitoring_parameters: type: map #key: id entry_schema: type: tosca.datatypes.nfv.VirtualLinkMonitoringParameter description: Describes monitoring parameters applicable to the VL required: false capabilities: virtual_linkable: type: tosca.capabilities.nfv.VirtualLinkable

### 6.8.10	tosca.nodes.nfv.VipCp

#### 6.8.10.1	Description

A VipCp node type represents the VipCpd information element as defined in ETSI GS NFV-IFA 011 [1], which describes a connection point to allocate one or a set of virtual IP addresses. Table 6.8.10.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.10.1-1: Type name, shorthand, and URI

| Shorthand Name | VipCp |
| --- | --- |
| Type Qualified Name | toscanfv:VipCp |
| Type URI | tosca.nodes.nfv.VipCp |

#### 6.8.10.2	Properties

The properties of the VipCp node type shall comply with the provisions set out in table 6.8.10.2-1.

Table 6.8.10.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| vip_function | yes | string | valid values: high_availability, load_balance | Indicates the function the virtual IP address is used for: high availability or load balancing. See note. |
| NOTE: When used for high availability, only one of the internal VDU CP instances or VNF external CP instances that share the virtual IP is bound to the VIP address at a time, i.e. only one is configured in the external (to the VNF) router to receive the packets e.g. as a result of a G-ARP message previously sent by this instance. When used for load balancing purposes all CP instances that share the virtual IP are bound to it. A load balancing function sends the packet to one or the other, but not to both. |  |  |  |  |

#### 6.8.10.3	Attributes

None.

#### 6.8.10.4	Requirements

The requirements of the VipCp node type shall comply with the provisions set out in table 6.8.10.4-1.

Table 6.8.10.4-1: Requirements

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| target | yes | tosca.capabilities.Node |  | Describes the requirement for connecting to CP instances that share the virtual IP address. |
| virtual_link | yes | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link. |

#### 6.8.10.5	Definition

The syntax of the VipCp node type shall comply with the following definition:

tosca.nodes.nfv.VipCp: derived_from: tosca.nodes.nfv.Cp description: Describes a connection point to allocate one or a set of virtual IP addresses properties: vip_function: type: string description: "Indicates the function the virtual IP address is used for: high availability or load balancing. When used for high availability, only one of the internal VDU CP instances or VNF external CP instances that share the virtual IP is bound to the VIP address at a time. When used for load balancing purposes all CP instances that share the virtual IP are bound to it." required: true constraints: - valid_values: [ high_availability, load_balance ] requirements: - target: capability: tosca.capabilities.Node relationship: tosca.relationships.DependsOn occurrences: [ 1, UNBOUNDED ] - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VipVirtualLinksTo occurrences: [1, 1]

#### 6.8.10.6	Example

See clause A.13.

### 6.8.11	tosca.nodes.nfv.VduSubCp

#### 6.8.11.1	Description

A VduSubCp node type represents the Subport information element as defined in ETSI GS NFV-IFA 011 [1], which describes network connectivity between a VNFC instance (based on VDU) and an internal VL through a trunk port. Table 6.8.11.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.8.11.1-1: Type name, shorthand, and URI

| Shorthand Name | VduSubCp |
| --- | --- |
| Type Qualified Name | toscanfv:VduSubCp |
| Type URI | tosca.nodes.nfv.VduSubCp |

#### 6.8.11.2	Properties

The properties of the VduSubCp node type shall comply with the provisions set out in table 6.8.11.2-1.

Table 6.8.11.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| segmentation_type | no | string | Allowed values: vlan, inherit | Specifies the encapsulation type for the traffics coming in and out of the trunk subport. |
| segmentation_id | no | integer | greater_or_equal: 0 | Specifies the segmentation ID for the subport, which is used to differentiate the traffics on different networks coming in and out of the trunk port. If a value is provided here it may be overridden by a value provided at run time when the infrastructure doesn't support mapping of segmentation IDs. |

#### 6.8.11.3	Attributes

None.

#### 6.8.11.4	Requirements

The requirements of the VduSubCp node type shall comply with the provisions set out in table 6.8.11.4-1.

Table 6.8.11.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| trunk_binding | yes | tosca.capabilities.nfv.TrunkBindable |  | Describes the requirements for binding with trunk parent port. |

#### 6.8.11.5	Definition

The syntax of the VduSubCp node type shall comply with the following definition:

tosca.nodes.nfv.VduSubCp: derived_from: tosca.nodes.nfv.VduCp description: describes network connectivity between a VNFC instance based on this VDU and an internal VL through a trunk port properties: segmentation_type: type: string description: Specifies the encapsulation type for the traffics coming in and out of the trunk subport. required: false constraints: - valid_values: [ vlan, inherit ] segmentation_id: type: integer description: Specifies the segmentation ID for the subport, which is used to differentiate the traffics on different networks coming in and out of the trunk port. required: false constraints: - greater_or_equal: 0 requirements: - trunk_binding: capability: tosca.capabilities.nfv.TrunkBindable relationship: tosca.relationships.nfv.TrunkBindsTo node: tosca.nodes.nfv.VduCp occurrences: [1, 1]

#### 6.8.11.6	Example

See clause A.16.

## 6.9	Group Types

### 6.9.1	tosca.groups.nfv.PlacementGroup

#### 6.9.1.1	Description

PlacementGroup is used for describing the affinity or anti-affinity relationship applicable between the virtualisation containers to be created based on different VDUs, or between internal VLs to be created based on different VnfVirtualLinkDesc(s). Table 6.9.1.1-1 specifies the declared names for this group type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.9.1.1-1: Type name, shorthand, and URI

| Shorthand Name | PlacementGroup |
| --- | --- |
| Type Qualified Name | toscanfv:PlacementGroup |
| Type URI | tosca.groups.nfv.PlacementGroup |

#### 6.9.1.2	Properties

The properties of the PlacementGroup group type shall comply with the provisions set out in table 6.9.1.2-1.

Table 6.9.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | yes | string |  | Human readable description of the group |

#### 6.9.1.3	Definition

The syntax of the PlacementGroup group type shall comply with the following definition:

tosca.groups.nfv.PlacementGroup: derived_from: tosca.groups.Root description: PlacementGroup is used for describing the affinity or anti-affinity relationship applicable between the virtualisation containers to be created based on different VDUs, or between internal VLs to be created based on different VnfVirtualLinkDesc(s) properties: description: type: string description: Human readable description of the group required: true members: [ tosca.nodes.nfv.Vdu.Compute, tosca.nodes.nfv.VnfVirtualLink ]

#### 6.9.1.4	Additional Requirements

A group with type tosca.groups.nfv.PlacementGroup shall contain more than one member when used as the target of an AffinityRule or AntiAffinityRule policy.

#### 6.9.1.5	Examples

See clause 6.10.10.5.

## 6.10	Policy Types

### 6.10.1	tosca.policies.nfv.InstantiationLevels

#### 6.10.1.1	Description

The InstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour and including default instantiation level in term of the number of VNFC instances to be created as defined in ETSI GS NFV-IFA 011 [1]. This policy concerns the whole VNF (deployment flavour) represented by the topology_template and thus has no explicit target list. Table 6.10.1.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.1.1-1: Type name, shorthand, and URI

| Shorthand Name | InstantiationLevels |
| --- | --- |
| Type Qualified Name | toscanfv:InstantiationLevels |
| Type URI | tosca.policies.nfv.InstantiationLevels |

#### 6.10.1.2	Properties

The properties of the InstantiationLevels policy type shall comply with the provisions set out in table 6.10.1.2-1.

Table 6.10.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| levels | yes | map of tosca.datatypes.nfv.InstantiationLevel |  | Describes the various levels of resources that can be used to instantiate the VNF using this flavour. |
| default_level | no | string |  | The default instantiation level for this flavour. |

#### 6.10.1.3	Definition

The syntax of the InstantiationLevels policy type shall comply with the following definition:

tosca.policies.nfv.InstantiationLevels: derived_from: tosca.policies.Root description: The InstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour and including default instantiation level in term of the number of VNFC instances to be created as defined in ETSI GS NFV-IFA 011 [1]. properties: levels: type: map # key: levelId description: Describes the various levels of resources that can be used to instantiate the VNF using this flavour. required: true entry_schema: type: tosca.datatypes.nfv.InstantiationLevel constraints: - min_length: 1 default_level: type: string # levelId description: The default instantiation level for this flavour. required: false # required if multiple entries in levels

### 6.10.2	tosca.policies.nfv.VduInstantiationLevels

#### 6.10.2.1	Description

The VduInstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour in term of the number of VNFC instances to be created from each vdu.Compute. as defined in ETSI GS NFV-IFA 011 [1].

Table 6.10.2.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.2.1-1: Type name, shorthand, and URI

| Shorthand Name | VduInstantiationLevels |
| --- | --- |
| Type Qualified Name | toscanfv:VduInstantiationLevels |
| Type URI | tosca.policies.nfv.VduInstantiationLevels |

#### 6.10.2.2	Properties

The properties of the VduInstantiationLevels policy type shall comply with the provisions set out in table 6.10.2.2-1.

Table 6.10.2.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| levels | yes | map of tosca.datatypes.nfv.VduLevel |  | Describes the Vdu.Compute levels of resources that can be used to instantiate the VNF using this flavour. |

#### 6.10.2.3	Definition

The syntax of the VduInstantiationLevels policy type shall comply with the following definition:

tosca.policies.nfv.VduInstantiationLevels: derived_from: tosca.policies.Root description: The VduInstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour in term of the number of VNFC instances to be created from each vdu.Compute. as defined in ETSI GS NFV-IFA 011 [1] properties: levels: type: map # key: levelId description: Describes the Vdu.Compute levels of resources that can be used to instantiate the VNF using this flavour required: true entry_schema: type: tosca.datatypes.nfv.VduLevel constraints: - min_length: 1 targets: [ tosca.nodes.nfv.Vdu.Compute ]

#### 6.10.2.4	Additional Requirements

A VduInstantiationLevels policy shall contain an entry for each instantiation level (and only for them) defined in the InstantiationLevels policy.

### 6.10.3	tosca.policies.nfv.VirtualLinkInstantiationLevels

#### 6.10.3.1	Description

The VirtualLinkInstantiationLevels type is a policy type representing all the instantiation levels of virtual link resources to be instantiated within a deployment flavour as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.3.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.3.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkInstantiationLevels |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkInstantiationLevels |
| Type URI | tosca.policies.nfv.VirtualLinkInstantiationLevels |

#### 6.10.3.2	Properties

The properties of the VirtualLinkInstantiationLevels policy type shall comply with the provisions set out in table 6.10.3.2-1.

Table 6.10.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| levels | yes | map of tosca.datatypes.nfv.VirtualLinkBitrateLevel |  | Describes the virtual link levels of resources that can be used to instantiate the VNF using this flavour. |

#### 6.10.3.3	Definition

The syntax of the VirtualLinkInstantiationLevels policy type shall comply with the following definition:

tosca.policies.nfv.VirtualLinkInstantiationLevels: derived_from: tosca.policies.Root description: The VirtualLinkInstantiationLevels type is a policy type representing all the instantiation levels of virtual link resources to be instantiated within a deployment flavour as defined in ETSI GS NFV-IFA 011 [1]. properties: levels: type: map # key: levelId description: Describes the virtual link levels of resources that can be used to instantiate the VNF using this flavour. required: true entry_schema: type: tosca.datatypes.nfv.VirtualLinkBitrateLevel constraints: - min_length: 1 targets: [ tosca.nodes.nfv.VnfVirtualLink ]

#### 6.10.3.4	Additional Requirements

A VirtualLinkInstantiationLevels policy shall contain an entry for each instantiation level (and only for them) defined in the InstantiationLevels policy.

### 6.10.4	Void

### 6.10.5	tosca.policies.nfv.ScalingAspects

#### 6.10.5.1	Description

The ScalingAspects type is a policy type representing the scaling aspects used for horizontal scaling as defined in ETSI GS NFV-IFA 011 [1]. This policy concerns the whole VNF (deployment flavour) represented by the topology_template and thus has no explicit target list. Table 6.10.5.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.5.1-1: Type name, shorthand, and URI

| Shorthand Name | ScalingAspects |
| --- | --- |
| Type Qualified Name | toscanfv:ScalingAspects |
| Type URI | tosca.policies.nfv.ScalingAspects |

#### 6.10.5.2	Properties

The properties of the ScalingAspects policy type shall comply with the provisions set out in table 6.10.5.2-1.

Table 6.10.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspects | yes | Map of tosca.datatypes.nfv.ScalingAspect |  | Describe maximum scale level for total number of scaling steps that can be applied to a particular aspect. |

#### 6.10.5.3	Definition

The syntax of the ScalingAspects policy type shall comply with the following definition:

tosca.policies.nfv.ScalingAspects: derived_from: tosca.policies.Root description: The ScalingAspects type is a policy type representing the scaling aspects used for horizontal scaling as defined in ETSI GS NFV-IFA 011 [1] properties: aspects: type: map # key: aspectId description: Describe maximum scale level for total number of scaling steps that can be applied to a particular aspect required: true entry_schema: type: tosca.datatypes.nfv.ScalingAspect constraints: - min_length: 1

#### 6.10.5.4	Additional Requirements

A scaling aspect for which only one scaling delta is defined in VduScalingAspectDeltas and VirtualLinkBitrateScalingAspectDeltas policies is called a “uniform aspect”.  In the case of “uniform aspect”, the step_deltas properties of tosca.datatypes.nfv.ScalingAspect is optional. If step_deltas is included, the value shall be a list of entries of step_deltas.

#### 6.10.5.5	Examples

See clause A.6.

### 6.10.6	tosca.policies.nfv.VduScalingAspectDeltas

#### 6.10.6.1	Description

The VduScalingAspectDeltas type is a policy type representing the Vdu.Compute detail of an aspect deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.6.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.6.1-1: Type name, shorthand, and URI

| Shorthand Name | VduScalingAspectDeltas |
| --- | --- |
| Type Qualified Name | toscanfv:VduScalingAspectDeltas |
| Type URI | tosca.policies.nfv.VduScalingAspectDeltas |

#### 6.10.6.2	Properties

The properties of the VduScalingAspectDeltas policy type shall comply with the provisions set out in table 6.10.6.2-1.

Table 6.10.6.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspect: | yes | string |  | Represents the scaling aspect to which this policy applies. |
| deltas | yes | map of tosca.datatypes.nfv.VduLevel |  | Describes the Vdu.Compute scaling deltas to be applied for every scaling steps of a particular aspect. |

#### 6.10.6.3	Definition

The syntax of the VduScalingAspectDeltas policy type shall comply with the following definition:

tosca.policies.nfv.VduScalingAspectDeltas: derived_from: tosca.policies.Root description: The VduScalingAspectDeltas type is a policy type representing the Vdu.Compute detail of an aspect deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1] properties: aspect: type: string description: Represents the scaling aspect to which this policy applies required: true deltas: type: map # key: scalingDeltaId description: Describes the Vdu.Compute scaling deltas to be applied for every scaling steps of a particular aspect. required: true entry_schema: type: tosca.datatypes.nfv.VduLevel constraints: - min_length: 1 targets: [ tosca.nodes.nfv.Vdu.Compute ]

#### 6.10.6.4	Additional Requirements

In the case of “uniform aspect”, the deltas properties shall have only one entry.

#### 6.10.6.5	Examples

See clause A.6.

### 6.10.7	tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas

#### 6.10.7.1	Description

The VirtualLinkBitrateScalingAspectDeltas type is a policy type representing the VnfVirtualLink detail of an aspect deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.7.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.7.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkBitrateScalingAspectDeltas |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkBitrateScalingAspectDeltas |
| Type URI | tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas |

#### 6.10.7.2	Properties

The properties of the VirtualLinkBitrateScalingAspectDeltas policy type shall comply with the provisions set out in table 6.10.7.2-1.

Table 6.10.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspect | yes | string |  | Represents the scaling aspect to which this policy applies. |
| aspect_deltas | yes | map of tosca.datatypes.nfv.VirtualLinkBitrateLevel |  | Describes the VnfVirtualLink scaling deltas to be applied for every scaling steps of a particular aspect. |

#### 6.10.7.3	Definition

The syntax of the VirtualLinkBitrateScalingAspectDeltas policy type shall comply with the following definition:

tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas: derived_from: tosca.policies.Root description: The VirtualLinkBitrateScalingAspectDeltas type is a policy type representing the VnfVirtualLink detail of an aspect deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. properties: aspect: type: string description: Represents the scaling aspect to which this policy applies. required: true deltas: type: map # key: scalingDeltaId description: Describes the VnfVirtualLink scaling deltas to be applied for every scaling steps of a particular aspect. required: true entry_schema: type: tosca.datatypes.nfv.VirtualLinkBitrateLevel constraints: - min_length: 1 targets: [ tosca.nodes.nfv.VnfVirtualLink ]

#### 6.10.7.4	Additional Requirements

In the case of “uniform aspect”, the deltas properties shall have only one entry.

#### 6.10.7.5	Examples

See clause A.6.

### 6.10.8	tosca.policies.nfv.VduInitialDelta

#### 6.10.8.1	Description

The VduInitialDelta type is a policy type representing the Vdu.Compute detail of an initial delta used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.8.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.8.1-1: Type name, shorthand, and URI

| Shorthand Name | VduInitialDelta |
| --- | --- |
| Type Qualified Name | toscanfv:VduInitialDelta |
| Type URI | tosca.policies.nfv.VduInitialDelta |

#### 6.10.8.2	Properties

The properties of the VduInitialDelta policy type shall comply with the provisions set out in table 6.10.8.2-1.

Table 6.10.8.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| initial_delta: | yes | tosca.datatypes.nfv.VduLevel |  | Represents the initial minimum size of the VNF. |

#### 6.10.8.3	Definition

The syntax of the VduInitialDelta policy type shall comply with the following definition:

tosca.policies.nfv.VduInitialDelta: derived_from: tosca.policies.Root description: The VduInitialDelta type is a policy type representing the Vdu.Compute detail of an initial delta used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. properties: initial_delta: type: tosca.datatypes.nfv.VduLevel description: Represents the initial minimum size of the VNF. required: true targets: [ tosca.nodes.nfv.Vdu.Compute ]

#### 6.10.8.4	Examples

See clause A.6.

### 6.10.9	tosca.policies.nfv.VirtualLinkBitrateInitialDelta

#### 6.10.9.1	Description

The VirtualLinkBitrateInitialDelta type is a policy type representing the VnfVirtualLink detail of an initial deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.9.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.9.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkBitrateInitialDelta |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkBitrateInitialDelta |
| Type URI | tosca.policies.nfv.VirtualLinkBitrateInitialDelta |

#### 6.10.9.2	Properties

The properties of the VirtualLinkBitrateInitialDelta policy type shall comply with the provisions set out in table 6.10.9.2-1.

Table 6.10.9.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| initial_delta: | yes | tosca.datatypes.nfv.VirtualLinkBitrateLevel |  | Represents the initial minimum size of the VNF. |

#### 6.10.9.3	Definition

The syntax of the VirtualLinkBitrateInitialDelta policy type shall comply with the following definition:

tosca.policies.nfv.VirtualLinkBitrateInitialDelta: derived_from: tosca.policies.Root description: The VirtualLinkBitrateInitialDelta type is a policy type representing the VnfVirtualLink detail of an initial deltas used for horizontal scaling, as defined in ETSI GS NFV-IFA 011 [1]. properties: initial_delta: type: tosca.datatypes.nfv.VirtualLinkBitrateLevel description: Represents the initial minimum size of the VNF. required: true targets: [ tosca.nodes.nfv.VnfVirtualLink ]

#### 6.10.9.4	Examples

See clause A.6.

### 6.10.10	AffinityRule, AntiAffinityRule

#### 6.10.10.1	Description

The AffinityRule or AntiAffinityRule describes the affinity or anti-affinity rules applicable for the defined targets:

If there is only one node template with node type tosca.nodes.nfv.Vdu.Compute or tosca.nodes.nfv.VnfVirtualLink set as the targets, the AffinityRule or AntiAffinityRule applies between the virtualisation containers to be created based on a particular VDU, or between internal VLs to be created based on a particular VnfVirtualLinkDesc as described in ETSI GS NFV-IFA 011 [1].

If there are more than one node templates with node type tosca.nodes.nfv.Vdu.Compute or tosca.nodes.nfv.VnfVirtualLink set as the targets, or a group with type tosca.groups.nfv.PlacementGroup which contains more than one members set as targets, the AffinityRule or AntiAffinityRule applies between the virtualisation containers to be created based on different VDUs, or between internal VLs to be created based on different VnfVirtualLinkDesc(s) as described in ETSI GS NFV-IFA 011 [1].

Tables 6.10.10.1-1 and 6.10.10.1-2 specify the declared names for the policy types. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.10.1-1: Type name, shorthand, and URI

| Shorthand Name | AffinityRule |
| --- | --- |
| Type Qualified Name | toscanfv:AffinityRule |
| Type URI | tosca.policies.nfv.AffinityRule |

Table 6.10.10.1-2: Type name, shorthand, and URI

| Shorthand Name | AntiAffinityRule |
| --- | --- |
| Type Qualified Name | toscanfv:AntiAffinityRule |
| Type URI | tosca.policies.nfv.AntiAffinityRule |

#### 6.10.10.2	Properties

The properties of the AffinityRule and AntiAffinityRule types shall comply with the provisions set out in table 6.10.10.2-1.

Table 6.10.10.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scope | Yes | String | Possible values are "nfvi_pop", "zone", "zone_group", "nfvi_node", "network_link_and_node". | Specifies the scope of the local affinity rule. See note. |
| NOTE: When used in an anti-affinity relationship, the "network_link_and_node" scope is conceptually similar to link and node disjoint paths capabilities used commonly in network Traffic Engineering (TE). For example, as in Fast Reroute Resource Reservation Protocol Traffic Engineering (RSVP-TE) for Label-Switched Path (LSP) tunnels as introduced in IETF RFC 4090 [i.17]. |  |  |  |  |

#### 6.10.10.3	targets

The targets of the AffinityRule and AntiAffinityRule policy types shall comply with the provisions set out in table 6.10.10.3-1.

Table 6.10.10.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | Yes | string[] | Valid types: tosca.nodes.nfv.Vdu.Compute tosca.nodes.nfv.VnfVirtualLink tosca.groups.nfv.PlacementGroup | In case of LocalAffinityOrAntiAffinityRule as defined in ETSI GS NFV-IFA 011 [1], the valid type of the targets is tosca.nodes.nfv.Vdu.Compute or tosca.nodes.nfv.VnfVirtualLink. In case of affinityOrAntiAffinityGroup as defined in ETSI GS NFV-IFA 011 [1], the valid types of the targets are: tosca.nodes.nfv.Vdu.Compute and tosca.nodes.nfv.VnfVirtualLink or a tosca.groups.nfv.PlacementGroup. |

#### 6.10.10.4	Definition

The syntax of the AffinityRule policy type shall comply with the following definition:

tosca.policies.nfv.AffinityRule: derived_from: tosca.policies.Placement description: The AffinityRule describes the affinity rules applicable for the defined targets properties: scope: type: string description: scope of the rule is an NFVI_node, an NFVI_PoP, etc. required: true constraints: - valid_values: [ nfvi_node, zone, zone_group, nfvi_pop, network_link_and_node ] targets: [ tosca.nodes.nfv.Vdu.Compute, tosca.nodes.nfv.VnfVirtualLink, tosca.groups.nfv.PlacementGroup ]

The syntax of the AntiAffinityRule policy type shall comply with the following definition:

tosca.policies.nfv.AntiAffinityRule: derived_from: tosca.policies.Placement description: The AntiAffinityRule describes the anti-affinity rules applicable for the defined targets properties: scope: type: string description: scope of the rule is an NFVI_node, an NFVI_PoP, etc. required: true constraints: - valid_values: [ nfvi_node, zone, zone_group, nfvi_pop, network_link_and_node ] targets: [ tosca.nodes.nfv.Vdu.Compute, tosca.nodes.nfv.VnfVirtualLink, tosca.groups.nfv.PlacementGroup ]

#### 6.10.10.5	Examples

The following example template fragments illustrate the concepts:

node_templates: VDU_1: type: tosca.nodes.nfv.Vdu.Compute policies: policy_affinity_local_VDU_1: type: tosca.policies.nfv.AffinityRule targets: [ VDU_1 ] properties: scope: nfvi_node

The above example illustrates a local affinity rule for VDU_1.

node_template: VDU_1: type: tosca.nodes.nfv.Vdu.Compute VDU_2: type: tosca.nodes.nfv.Vdu.Compute groups: affinityOrAntiAffinityGroup_1: type: tosca.groups.nfv.PlacementGroup members: [ VDU_1, VDU_2 ] policies: policy_antiaffinity_group_1: type: tosca.policies.nfv.AntiAffinityRule targets: [ affinityOrAntiAffinityGroup_1 ] properties: scope: nfvi_node

The above example illustrates an anti-affinity policy among a group which contains VDU_1 and VDU_2 as members.

### 6.10.11	tosca.policies.nfv.Abstract.SecurityGroupRule

#### 6.10.11.1	Description

The Abstract.SecurityGroupRule policy type is defined in clause 9.10.1 of the present document.

### 6.10.12	tosca.policies.nfv.SupportedVnfInterface

#### 6.10.12.1	Description

The SupportedVnfInterface policy type represents interfaces produced by a VNF, the details to access them and the applicable connection points to use to access these interfaces. It corresponds to the VnfInterfaceDetails information element defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.12.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.12.1-1: Type name, shorthand, and URI

| Shorthand Name | SupportedVnfInterface |
| --- | --- |
| Type Qualified Name | toscanfv:SupportedVnfInterface |
| Type URI | tosca.policies.nfv.SupportedVnfInterface |

#### 6.10.12.2	Properties

The properties of the SupportedVnfInterface policy type shall comply with the provisions set out in table 6.10.12.2-1.

Table 6.10.12.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| interface_name | yes | string | Valid values vnf_indicator vnf_configuration | Identifies an interface produced by the VNF. |
| details | no | tosca.datatypes.nfv.InterfaceDetails |  | Provide additional data to access the interface endpoint (e.g. API URI prefix). |

#### 6.10.12.3	Definition

The syntax of the SupportedVnfInterface policy type shall comply with the following definition:

tosca.policies.nfv.SupportedVnfInterface: derived_from: tosca.policies.Root description: this policy type represents interfaces produced by a VNF, the details to access them and the applicable connection points to use to access these interfaces properties: interface_name: type: string description: Identifies an interface produced by the VNF. required: true constraints: - valid_values: [ vnf_indicator, vnf_configuration ] details: type: tosca.datatypes.nfv.InterfaceDetails description: Provide additional data to access the interface endpoint required: false targets: [ tosca.nodes.nfv.VnfExtCp, tosca.nodes.nfv.VduCp ]

#### 6.10.12.4	Additional requirements

The valid targets for this policy type shall be the node templates representing the connection point descriptors from which to instantiate the connection point instances through which the interfaces can be accessed. This may be a VnfExtCp node template or a VduCp node template when an internal connection point is re-exposed externally.

#### 6.10.12.5	Example

policies: policy_interface_1: type: tosca.policies.nfv.SupportedVnfInterface targets: [ MyVnfmFacingExtCp ] properties: interface_name: vnf_indicator details: uri_components: scheme: https authority: host: myvnf.example.com

### 6.10.13	tosca.policies.nfv.SecurityGroupRule

#### 6.10.13.1	Description

The SecurityGroupRule type is a policy type specifying the matching criteria for the ingress and/or egress traffic to and from visited connection points as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.13.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.13.1-1: Type name, shorthand, and URI

| Shorthand Name | SecurityGroupRule |
| --- | --- |
| Type Qualified Name | toscanfv:SecurityGroupRule |
| Type URI | tosca.policies.nfv.SecurityGroupRule |

#### 6.10.13.2	Properties

None.

#### 6.10.13.3	targets

The targets of the SecurityGroupRule policy types shall comply with the provisions set out in table 6.10.13.3-1.

Table 6.10.13.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | yes | string[] | Valid types: tosca.nodes.nfv.VduCp, tosca.nodes.nfv.VnfExtCp | Target connection points of VduCp and/or VnfExtCp. |

#### 6.10.13.4	Definition

The syntax of the SecurityGroupRule policy type shall comply with the following definition:

tosca.policies.nfv.SecurityGroupRule: derived_from: tosca.policies.nfv.Abstract.SecurityGroupRule description: The SecurityGroupRule type is a policy type specified the matching criteria for the ingress and/or egress traffic to/from visited connection points as defined in ETSI GS NFV-IFA 011 [1]. targets: [ tosca.nodes.nfv.VduCp, tosca.nodes.nfv.VnfExtCp ]

#### 6.10.13.5	Additional Requirements

None.

### 6.10.14	tosca.policies.nfv.VnfIndicator

#### 6.10.14.1	Description

The VnfIndicator is a base policy type for defining VNF indicator specific policies that define the conditions to assess and the action to perform when a VNF indicator changes value as defined in ETSI GS NFV-IFA 011 [1].

Table 6.10.14.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.14.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfIndicator |
| --- | --- |
| Type Qualified Name | toscanfv:VnfIndicator |
| Type URI | tosca.policies.nfv.VnfIndicator |

#### 6.10.14.2	Properties

The properties of the VnfIndicator policy type shall comply with the provisions set out in table 6.10.14.2-1.

Table 6.10.14.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| source | yes | string | valid values: vnf, em, both_vnf_and_em | Describe the source of the indicator. |

#### 6.10.14.3	Definition

The syntax of the VnfIndicator policy type shall comply with the following definition:

tosca.policies.nfv.VnfIndicator: derived_from: tosca.policies.Root description: The VnfIndicator policy type is a base policy type for defining VNF indicator specific policies that define the conditions to assess and the action to perform when a VNF indicator changes value as defined in ETSI GS NFV-IFA 011 [1]. properties: source: type: string description: Describe the source of the indicator. required: true constraints: - valid_values: [ vnf, em, both_vnf_and_em ] targets: [ tosca.nodes.nfv.VNF ]

#### 6.10.14.4	Additional requirements

The VNFD service template may include VNF indicator specific policies of VnfIndicator type with the following requirements:

- it shall include one or more trigger definitions which:

shall include an event with a value equal to the full name of a notification in the VnfIndicator interface definition of the VNF node where the policy applies;

may include a condition definition which can assert the value of vnf indicator attributes and other node attributes using arbitrary AND and OR combinations of the individual assertions;

may include an action invoking one or multiple operations of the Vnflcm interface;

the target shall be set to the node template to which the policy applies, i.e. to the node template of the VNF specific type present in the topology template that represents a particular deployment flavour.

### 6.10.15	tosca.policies.nfv.VnfPackageChange

#### 6.10.15.1	Description

The VnfPackageChange type is a policy type specifying the processes and rules to be used for performing the resource related tasks, to change VNF instance to a different VNF Package (destination package) as defined in ETSI GS NFV-IFA 011 [1]. Table 6.10.15.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 6.10.15.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfPackageChange |
| --- | --- |
| Type Qualified Name | toscanfv:VnfPackageChange |
| Type URI | tosca.policies.nfv.VnfPackageChange |

#### 6.10.15.2	Properties

The properties of the VnfPackageChange policy type shall comply with the provisions set out in table 6.10.15.2-1.

Table 6.10.15.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| selector | yes | tosca.datatypes.nfv.VnfPackageChangeSelector |  | Information to identify the source and destination VNFD for the change, and the related deployment flavours. |
| modification_qualifier | yes | string | up, down | Specifies the type of modification resulting from transitioning from srcVnfdId to dstVnfdId. The possible values are UP indicating that the destination VNF version is newer than the source version, DOWN indicating that the destination VNF version is older than the source version. |
| additional_modification_description | no | string |  | Additional information to qualify further the change between the two versions. |
| component_mappings | no | list of tosca.datatypes.nfv.VnfPackageChangeComponentMapping |  | Mapping information related to identifiers of components in source VNFD and destination VNFD that concern to the change process. |
| destination_flavour_id | yes | string |  | Identifies the deployment flavour in the destination VNF package for which this change applies. The flavour ID is defined in the destination VNF package. |

#### 6.10.15.3	Definition

The syntax of the VnfPackageChange policy type shall comply with the following definition:

tosca.policies.nfv.VnfPackageChange: derived_from: tosca.policies.Root description: policy type specifying the processes and rules to be used for performing the resource related tasks, to change VNF instance to a different VNF Package (destination package) properties: selector: type: tosca.datatypes.nfv.VnfPackageChangeSelector description: Information to identify the source and destination VNFD for the change, and the related deployment flavours. required: true modification_qualifier: type: string description: Specifies the type of modification resulting from transitioning from srcVnfdId to dstVnfdId. The possible values are UP indicating that the destination VNF version is newer than the source version, DOWN indicating that the destination VNF version is older than the source version. constraints: [ valid_values: [ up, down ] ] required: true additional_modification_description: type: string description: Additional information to qualify further the change between the two versions. required: false component_mappings: type: list entry_schema: type: tosca.datatypes.nfv.VnfPackageChangeComponentMapping description: Mapping information related to identifiers of components in source VNFD and destination VNFD that concern to the change process. required: false destination_flavour_id: type: string description: Identifies the deployment flavour in the destination VNF package for which this change applies. The flavour ID is defined in the destination VNF package. required: true

#### 6.10.15.4	Additional Requirements

The VnfPackageChange specific type policy shall have exactly one trigger with an event and an action.

The event value shall be set to change_current_package_notification notification of Vnflcm interface.

The action value shall be set to either change_current_package operation on the Vnflcm (in case the same LCM script or no LCM script with the same set of "additionalParams" or no "additionalParams" is suitable for all change paths)  or one of the VNF-specific operations on the ChangeCurrentVnfPackage interface (in case different change paths require different LCM scripts or no LCM script potentially with different sets of "additionalParams" or no "additionalParams").

The policy shall be applied when the actual values of the descriptor_id and flavour_id of the source VNF type and the descriptor_id of the destination VNF type match the source_descriptor_id, destination_descriptor_id and source_flavour_id properties, respectively, of the selector in the policy.

#### 6.10.15.5	Example

The following example template illustrates a VNFD which is supporting VNF package change info via using VnfPackageChange policy, additional_parameters input for each case of the selector and VNF specific interface (tosca.interfaces.nfv.ChangeCurrentVnfPackage), and other data types.  A policy definition of change_from_version_1 and change_to_version_1 has a trigger. The action will be taken when the conditions of the selector values are satisfied. The policy has an event associated to the notification of tosca.interfaces.nfv.Vnflcm.change_current_package_notification.

tosca_definitions_version: tosca_simple_yaml_1_3 imports: .. node_types: MyCompany.SunshineDB.2_0.2_0: derived_from: tosca.nodes.nfv.VNF properties: descriptor_id: type: string constraints: [ equal: ebc68e34-0cfa-40ba-8b45-9caa31f9dcb5 ] default: ebc68e34-0cfa-40ba-8b45-9caa31f9dcb5 provider: type: string constraints: [ equal: MyCompany ] default: MyCompany product_name: type: string constraints: [ equal: SunshineDB ] default: SunshineDB software_version: type: string constraints: [ equal: '2.0' ] default: '2.0' descriptor_version: type: string constraints: [ equal: '2.0' ] default: '2.0' flavour_id: type: string constraints: [ equal: default ] default: default .. interfaces: MyCompanyChangeCurrentVnfPackage: type: MyCompany.interfaces.nfv.ChangeCurrentVnfPackage interface_types: MyCompany.interfaces.nfv.ChangeCurrentVnfPackage derived_from: tosca.interfaces.nfv.ChangeCurrentVnfPackage operations: change_from_version_1: description: operation for change from version 1 to 2 inputs: additional_parameters: type: MyCompany.datatypes.nfv.VnfChangeFromVersion1AdditionalParameters change_to_version_1: description: operation for change from version 2 to 1 inputs: additional_parameters: type: MyCompany.datatypes.nfv.VnfChangeToVersion1AdditionalParameters data_types: .. MyCompany.datatypes.nfv.VnfChangeFromVersion1AdditionalParameters: derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters properties: parameter_1: type: string parameter_2: type: string MyCompany.datatypes.nfv.VnfChangeToVersion1AdditionalParameters: derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters properties: parameter_3: type: string parameter_4: type: string topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.2_0.2_0 .. node_templates: sunshine_db: type: MyCompany.SunshineDB.2_0.2_0 .. interfaces: MyCompanyChangeCurrentVnfPackage: operations: .. change_from_version_1: implementation: change-from-version-1.workbook.mistral.yaml change_to_version_1: implementation: change-to-version-1.workbook.mistral.yaml server: type: MyCompany.nodes.nfv.Vdu.Aux .. volume: type: tosca.nodes.nfv.Vdu.VirtualBlockStorage .. server_internal_cp: type: tosca.nodes.nfv.VduCp .. internal_vl: type: tosca.nodes.nfv.VnfVirtualLink .. policies: .. - instantiation_levels: type: tosca.policies.nfv.InstantiationLevels properties: levels: single: description: .. quadruple: description: .. default_level: single .. - change_from_version_1: type: tosca.policies.nfv.VnfPackageChange properties: selector: source_descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 destination_descriptor_id: ebc68e34-0cfa-40ba-8b45-9caa31f9dcb5 source_flavour_id: simple modification_qualifier: up additional_modification_description: .. component_mappings: - component_type: vdu source_id: dbBackend destination_id: server description: .. - component_type: virtual_storage source_id: mariaDbStorage destination_id: volume description: .. - component_type: instantiation_level source_id: single destination_id: default description: .. - component_type: deployment_flavour source_id: simple destination_id: default description: .. destination_flavour_id: default triggers: change_from_version_1: event: tosca.interfaces.nfv.Vnflcm.change_current_package_notification action: - call_operation: tosca.interfaces.nfv.MyCompanyChangeCurrentVnfPackage.change_from_version_1 - change_to_version_1: type: tosca.policies.nfv.VnfPackageChange properties: selector: source_descriptor_id: ebc68e34-0cfa-40ba-8b45-9caa31f9dcb5 destination_descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 source_flavour_id: default modification_qualifier: down additional_modification_description: .. component_mappings: - component_type: vdu source_id: server destination_id: dbBackend description: .. - component_type: virtual_storage source_id: volume destination_id: mariaDbStorage description: .. - component_type: instantiation_level source_id: default destination_id: single description: .. - component_type: deployment_flavour source_id: default destination_id: simple description: .. destination_flavour_id: simple triggers: change_to_version_1: event: tosca.interfaces.nfv.Vnflcm.change_current_package_notification action: - call_operation: tosca.interfaces.nfv.MyCompanyChangeCurrentVnfPackage.change_to_version_1 .. ..

The following example template illustrates a VNFD which is supporting VNF package change info via using VnfPackageChange policy, without additional_parameters input and Vnflcm interface operation. A policy definition of change_from_version_1 and change_to_version_1 has a trigger. The action will be taken when the conditions of the selector values are satisfied. The policy has an event associated to the notification of tosca.interfaces.nfv.Vnflcm.change_current_package_notification.

tosca_definitions_version: tosca_simple_yaml_1_3 imports: .. node_types: MyCompany.SunshineDB.2_0.2_0: derived_from: tosca.nodes.nfv.VNF properties: descriptor_id: type: string constraints: [ equal: ebc68e34-0cfa-40ba-8b45-9caa31f9dcb5 ] default: ebc68e34-0cfa-40ba-8b45-9caa31f9dcb5 provider: type: string constraints: [ equal: MyCompany ] default: MyCompany product_name: type: string constraints: [ equal: SunshineDB ] default: SunshineDB software_version: type: string constraints: [ equal: '2.0' ] default: '2.0' descriptor_version: type: string constraints: [ equal: '2.0' ] default: '2.0' flavour_id: type: string constraints: [ equal: default ] default: default .. interfaces: MyCompanyVnflcm: type: tosca.interfaces.nfv.Vnflcm topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.2_0.2_0 .. node_templates: sunshine_db: type: MyCompany.SunshineDB.2_0.2_0 .. interfaces: Vnflcm: operations: .. change_current_package: server: type: MyCompany.nodes.nfv.Vdu.Aux .. volume: type: tosca.nodes.nfv.Vdu.VirtualBlockStorage .. server_internal_cp: type: tosca.nodes.nfv.VduCp .. internal_vl: type: tosca.nodes.nfv.VnfVirtualLink .. policies: .. - instantiation_levels: type: tosca.policies.nfv.InstantiationLevels properties: levels: single: description: .. quadruple: description: .. default_level: single .. - change_from_version_1: type: tosca.policies.nfv.VnfPackageChange properties: selector: source_descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 destination_descriptor_id: ebc68e34-0cfa-40ba-8b45-9caa31f9dcb5 source_flavour_id: simple modification_qualifier: up additional_modification_description: .. component_mappings: - component_type: vdu source_id: dbBackend destination_id: server description: .. - component_type: virtual_storage source_id: mariaDbStorage destination_id: volume description: .. - component_type: instantiation_level source_id: single destination_id: default description: .. - component_type: deployment_flavour source_id: simple destination_id: default description: .. destination_flavour_id: default triggers: change_from_version_1: event: tosca.interfaces.nfv.Vnflcm.change_current_package_notification action: - call_operation: tosca.interfaces.nfv.Vnflcm.change_current_package - change_to_version_1: type: tosca.policies.nfv.VnfPackageChange properties: selector: source_descriptor_id: ebc68e34-0cfa-40ba-8b45-9caa31f9dcb5 destination_descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 source_flavour_id: default modification_qualifier: down additional_modification_description: .. component_mappings: - component_type: vdu source_id: server destination_id: dbBackend description: .. - component_type: virtual_storage source_id: volume destination_id: mariaDbStorage description: .. - component_type: instantiation_level source_id: default destination_id: single description: .. - component_type: deployment_flavour source_id: default destination_id: simple description: .. destination_flavour_id: simple triggers: change_to_version_1: event: tosca.interfaces.nfv.Vnflcm.change_current_package_notification action: - call_operation: tosca.interfaces.nfv.Vnflcm.change_current_package ..

## 6.11	VNFD TOSCA service template design

### 6.11.1	General

The TOSCA service template design for a VNFD in the general case uses two levels of service templates as described in clause 6.11.2. In this design, the top level contains an abstract VNF node template, i.e. without an implementation of the creation operation and is therefore substituted by one of the lower level service templates. This design is applicable regardless of whether the VNF has one or multiple deployment flavours.

In the particular case of a VNF with only one deployment flavour there is an alternative design which is described in clause 6.11.3 and which uses only one service template.

### 6.11.2	Single or multiple deployment flavour design with two levels of service templates

VNFD shall be implemented as one top-level service template and one or multiple lower level service templates, where each lower level service template represents a deployment flavour. A separate YAML file with a VNF specific node type definition which shall be derived from tosca.nodes.nfv.VNF node type as defined in clause 6.8.1 shall be provided and is also considered as a part of a VNFD. The top level service template shall be the main entry point of the VNF package as specified in ETSI GS NFV-SOL 004 [i.6], i.e. the Entry-definitions file. The file names of all the lower service templates shall be declared as the value of the Other-Definitions key as specified in TOSCA-Simple-Profile-YAML-v1.3 [20] in the TOSCA.meta file of the VNF package.

See clause A.2 for an example of VNFD design with multiple deployment flavours.

The top level service template shall comply with TOSCA-Simple-Profile-YAML-v1.3 [20] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.2;

an import statement referencing a yaml file which contains a VNF specific node type definition;

optionally, import statements referencing additional VNF-specific files containing only type definitions used by this TOSCA service template; and

a topology template with a node template of the VNF specific node type, which shall include the flavour_id and other properties and the requirements as defined in clause 6.8.1.

The deployment and lifecycle management of instances of this VNF node type is done by means of substitution by any of the lower level service templates as declared in the Other-Definitions of the TOSCA.meta file in the VNF package.

The lower level service template is an implementable TOSCA service template for the deployment of a specific deployment flavour. The lower level service template shall comply with TOSCA-Simple-Profile-YAML-v1.3 [20] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.2;

an import statement referencing a yaml file which contains a VNF specific node type definition which shall be derived from tosca.nodes.nfv.VNF node type as defined in clause 6.8.1;

optionally, import statements referencing additional VNF-specific files containing type definitions used by this TOSCA service template; and

a topology template describing the internal topology of the VNF with:

substitution_mappings indicating:

the same node type as defined in the VNF specific node type definition service template;

a flavour_id property and its value as defined in substitution_filter which identifies the DF corresponding to this low level template within the VNFD;

NOTE 1:	Starting with version 3.3.1 of the present document, the property_mapping grammar was changed to support substitution_filter. For backward compatibility, TOSCA-Simple-Profile-YAML-v1.3 [20], clause 3.8.8.3 specifies the provisions for handling the previous grammar. Support of the Release 2 property_mapping grammar can be removed in subsequent versions of the present document.

the mapping of the virtual_link requirements on external connection points;

a node template referencing the VNF specific node type, implementations of the operations of the LCM interface to be executed by the VNFM, if applicable; and

additional node templates of type Vdu.Compute (or a derived node type), Vdu.VirtualBlockStorage, Vdu.VirtualObjectStorage, Vdu.VirtualFileStorage VduCp, etc. that define the topology and composition of the VNF flavour.

NOTE 2:	The format and structure of a VNF package is defined in ETSI GS NFV-SOL 004 [i.6].

NOTE 3:	All the imported type definition files as indicated either in the top level service template or in any of the lower level service template are considered as parts of a VNFD.

When the flavour_id of a VNF has been chosen (e.g. through an input parameter of a VNF instantiation request received by a VNFM) among the values included in the VNF node type imported into the top level service template, it is then used as the filter for selecting a particular lower level TOSCA service template inside the VNF package as described in TOSCA-Simple-Profile-YAML-v1.3 [20].

### 6.11.3	Single deployment flavour design with one service template

In case of the single deployment flavour scenario with one service template design, the VNFD shall use TOSCA-Simple-Profile-YAML-v1.3 [20] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.2;

either a VNF specific node type definition derived from the tosca.nodes.nfv.VNF node type, as defined in clause 6.8.1 or an import statement referencing a file that contains such definition;

optionally, import statements referencing additional VNF-specific files containing type definitions used by this TOSCA service template; and

a topology template describing the internal topology of the VNF with:

substitution_mappings indicating the same VNF specific node type and the mapping of the virtual_link requirements on external connection points;

a node template of this VNF specific node type with the flavour_id and other properties and, if applicable, implementations of the operations of the LCM interface to be executed by the VNFM; and

additional node templates of type Vdu.Compute (or a derived node type), Vdu.VirtualBlockStorage, Vdu.VirtualObjectStorage, Vdu.VirtualFileStorage, VduCp, etc. that define the topology and composition of the VNF flavour.

See clause A.5 for an example of VNFD design with single deployment flavour.

NOTE 1:	The service template is deployed stand-alone, i.e. without performing a substitution. However including the substitution_mappings rule indicate its ability to substitute a node template of the VNF specific node type, which may appear in an NSD.

NOTE 2:	All the imported type definition files as indicated in the service template are considered as parts of a VNFD.

6.11.4	Package change (handling the Change current VNF Package request)

The Change current VNF Package operation, defined in ETSI GS NFV-SOL 002 [i.4] and ETSI GS NFV-SOL 003 [i.9], enables the NFVO to request the VNFM to change the current VNF Package, i.e. the VNF package on which a VNF instance is based, hence enabling VNF software modification. For such a request to be handled properly by the VNFM, the VNFD provides a policy of type tosca.policies.nfv.VnfPackageChange with the following characteristics:

there may be as many instances of this policy type in the VNFD as change paths are supported;

either the source or the destination of a change path shall be the current VNFD i.e. the one where the policy instance is present;

a given instance of this policy type is triggered when the combination of the source_descriptor_id, destination_descriptor_id,  source_flavour_id values in its selector property matches with the following:

source_descriptor_id: "vnfdId": attribute in VnfInstance

destination_descriptor_id: "vnfdId" attribute in ChangeCurrentVnfPkgRequest

source_flavour_id: "flavour" attribute in VnfInstance

the operation designated by the trigger action in the policy definition is invoked upon receiving the Change current VNF Package request on the API (ETSI GS NFV-SOL 003 [i.9] or ETSI GS NFV-SOL 002 [i.4]);

if "additionalParams" are to be received in the Change current VNF Package request on the API, then the input signature (additional_parameters) of the designated operation defines what "additionalParams" can be submitted as part of the operation request.

# 7	NSD TOSCA model

## 7.1	Introduction

The NSD information model specified by ETSI GS NFV-IFA 014 [2] is mapped to the TOSCA concepts. NSD occurrences are represented as TOSCA service templates, as defined in the TOSCA-Simple-Profile-YAML-v1.3 [20], to be used by the NFVO for managing the lifecycle of NS instances.

Table 7.1-1 shows an overview of the mapping between the main NSD information elements defined in ETSI GS NFV-IFA 014 [2] and TOSCA types defined in the present document. The definition of all TOSCA types for representing all information elements is described in the following clauses.

NOTE:	There is only part of NSD features as specified in ETSI GS NFV-IFA 014 [2] included in this version of the present document, the following are features not included:

NS scaling Aspect

Multiple Ns instantiation levels

VnfIndicators

Table 7.1-1: Mapping of ETSI GS NFV-IFA 014 [2] information elements with TOSCA types

| ETSI NFV Information Element ETSI GS NFV-IFA 014 [2] | TOSCA type | Derived from |
| --- | --- | --- |
| NSD | tosca.nodes.nfv.NS | tosca.nodes.Root |
| Sapd | tosca.nodes.nfv.Sap | tosca.nodes.Root |
| NsVirtualLinkDesc | tosca.nodes.nfv.NsVirtualLink | tosca.nodes.Root |
| Pnfd | tosca.nodes.nfv.PNF | tosca.nodes.Root |
| Vnfd | tosca.nodes.nfv.VNF | tosca.nodes.Root |
| Vnffgd | tosca.groups.nfv.VNFFG | tosca.groups.Root |

## 7.2	Data Types

### 7.2.1	Void

### 7.2.2	tosca.datatypes.nfv.VnfProfile

#### 7.2.2.1	Description

The VnfProfile data type is defined in clause 9.2.8 of the present document.

### 7.2.3	tosca.datatype.nfv.NsVlProfile

#### 7.2.3.1	Description

The NsVlProfile data type describes additional instantiation data for a given NsVirtualLink used in a specific NS deployment flavour. Table 7.2.3.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.3.1-1: Type name, shorthand, and URI

| Shorthand Name | NsVlProfile |
| --- | --- |
| Type Qualified Name | toscanfv:NsVlProfile |
| Type URI | tosca.datatypes.nfv.NsVlProfile |

#### 7.2.3.2	Properties

The properties of the NsVlProfile data type shall comply with the provisions set out in table 7.2.3.2-1.

Table 7.2.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| max_bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Specifies the maximum bitrate requirements for a VL instantiated according to this profile. |
| min_bitrate_requirements | yes | tosca.datatypes.nfv.LinkBitrateRequirements |  | Specifies the minimum bitrate requirements for a VL instantiated according to this profile. |
| qos | no | tosca.datatypes.nfv.NsVirtualLinkQos |  | Specifies the QoS requirements of a VL instantiated according to this profile. |
| service_availability_level | no | integer | greater_or_equal: 1 | If present, specifies the service availability level for the VL instance created from this profile. See note 2. |
| virtual_link_protocol_data | no | list of tosca.datatypes.nfv.NsVirtualLinkProtocolData |  | Specifies the protocol data for a virtual link. |
| NOTE 1: A virtualLinkDescId property, which exists in ETSI GS NFV-IFA 014 [2] is not needed, as the NsVlProfile is contained in the NsVirtualLink node. NOTE 2: The value '1' expresses the highest service availability level. |  |  |  |  |

#### 7.2.3.3	Definition

The syntax of the NsVlProfile data type shall comply with the following definition:

tosca.datatypes.nfv.NsVlProfile: derived_from: tosca.datatypes.Root description: Describes additional instantiation data for a given NsVirtualLink used in a specific NS deployment flavour. properties: max_bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Specifies the maximum bitrate requirements for a VL instantiated according to this profile. required: true min_bitrate_requirements: type: tosca.datatypes.nfv.LinkBitrateRequirements description: Specifies the minimum bitrate requirements for a VL instantiated according to this profile. required: true qos: type: tosca.datatypes.nfv.NsVirtualLinkQos description: Specifies the QoS requirements of a VL instantiated according to this profile. required: false service_availability_level: type: integer description: Specifies the service availability level for the VL instance created from this profile required: false constraints: - greater_or_equal: 1 virtual_link_protocol_data: type: list description: Specifies the protocol data for a virtual link. required: false entry_schema: type: tosca.datatypes.nfv.NsVirtualLinkProtocolData

#### 7.2.3.4	Examples

None.

#### 7.2.3.5	Additional Requirements

None.

### 7.2.4	tosca.datatypes.nfv.ConnectivityType

#### 7.2.4.1	Description

The ConnectivityType data type is defined in clause 9.2.4 of the present document.

### 7.2.5	tosca.datatypes.nfv.NsVirtualLinkQos

#### 7.2.5.1	Description

The NsVirtualLinkQoS describes QoS data type a given NsVirtualLink used in an NS deployment flavour. Table 7.2.5.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.5.1-1: Type name, shorthand, and URI

| Shorthand Name | NsVirtualLinkQos |
| --- | --- |
| Type Qualified Name | toscanfv:NsVirtualLinkQos |
| Type URI | tosca.datatypes.nfv.NsVirtualLinkQos |

#### 7.2.5.2	Properties

The properties of the NsVirtualLinkQos data type shall comply with the provisions set out in table 7.2.5.2-1.

Table 7.2.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| priority | no | integer | greater_or_equal: 0 | Specifies the priority level in case of congestion on the underlying physical links. |

#### 7.2.5.3	Definition

The syntax of the NsVirtualLinkQos data type shall comply with the following definition:

tosca.datatypes.nfv.NsVirtualLinkQos: derived_from: tosca.datatypes.nfv.Qos description: describes QoS data for a given VL used in a VNF deployment flavour properties: priority: type: integer constraints: - greater_or_equal: 0 description: Specifies the priority level in case of congestion on the underlying physical links required: false

#### 7.2.5.4	Examples

None.

#### 7.2.5.5	Additional Requirements

None.

### 7.2.6	tosca.datatypes.nfv.LinkBitrateRequirements

#### 7.2.6.1	Description

The LinkBitrateRequirements data type is defined in clause 9.2.5 of the present document.

### 7.2.7	Void

### 7.2.8	Void

### 7.2.9	Void

### 7.2.10	Void

### 7.2.11	tosca.datatypes.nfv.CpProtocolData

#### 7.2.11.1	Description

The CpProtocolData data type is defined in clause 9.2.6 of the present document.

### 7.2.12	tosca.datatypes.nfv.AddressData

#### 7.2.12.1	Description

The AddressData data type is defined in clause 9.2.3 of the present document.

### 7.2.13	tosca.datatypes.nfv.L2AddressData

#### 7.2.13.1	Description

The L2AddressData data type is defined in clause 9.2.1 of the present document.

### 7.2.14	tosca.datatypes.nfv.L3AddressData

#### 7.2.14.1	Description

The L3AddressData data type is defined in clause 9.2.2 of the present document.

### 7.2.15	tosca.datatypes.nfv.Qos

#### 7.2.15.1	Description

The Qos data type is defined in clause 9.2.7 of the present document.

### 7.2.16	tosca.datatypes.nfv.NsProfile

#### 7.2.16.1	Description

The NsProfile data type describes a profile for instantiating nested NSs which are constituents of an NS with a particular NS DF as defined in ETSI GS NFV-IFA 014 [2]. Table 7.2.16.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.16.1-1: Type name, shorthand, and URI

| Shorthand Name | NsProfile |
| --- | --- |
| Type Qualified Name | toscanfv:NsProfile |
| Type URI | tosca.datatypes.nfv.NsProfile |

#### 7.2.16.2	Properties

The properties of the NsProfile data type shall comply with the provisions set out in table 7.2.16.2-1.

Table 7.2.16.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ns_instantiation_level | no | string |  | Identifier of the instantiation level of the NS DF to be used for instantiation. If not present, the default instantiation level as declared in the NSD shall be used. |
| min_number_of_instances | yes | integer | greater_or_equal : 0 | Minimum number of instances of the NS based on this NSD that is permitted to exist for this NsProfile. |
| max_number_of_instances | yes | integer | greater_or_equal : 0 | Maximum number of instances of the NS based on this NSD that is permitted to exist for this NsProfile. |
| flavour_id | yes | string |  | Identifies the applicable network service DF within the scope of the NSD. |

#### 7.2.16.3	Definition

The syntax of the NsProfile data type shall comply with the following definition:

tosca.datatypes.nfv.NsProfile: derived_from: tosca.datatypes.Root description: describes a profile for instantiating NSs of a particular NS DF according to a specific NSD and NS DF. properties: ns_instantiation_level: type: string description: Identifier of the instantiation level of the NS DF to be used for instantiation. If not present, the default instantiation level as declared in the NSD shall be used. required: false min_number_of_instances: type: integer description: Minimum number of instances of the NS based on this NSD that is permitted to exist for this NsProfile. required: true constraints: - greater_or_equal: 0 max_number_of_instances: type: integer description: Maximum number of instances of the NS based on this NSD that is permitted to exist for this NsProfile. required: true constraints: - greater_or_equal: 0 flavour_id: type: string description: Identifies the applicable network service DF within the scope of the NSD. required: true

#### 7.2.16.4	Example

None.

#### 7.2.16.5	Additional Requirements

None.

### 7.2.17	tosca.datatypes.nfv.Mask

#### 7.2.17.1	Description

The Mask data type describes the value to be matched for a sequence of bits at a particular location in a frame. Table 7.2.17.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.17.1-1: Type name, shorthand, and URI

| Shorthand Name | Mask |
| --- | --- |
| Type Qualified Name | toscanfv:Mask |
| Type URI | tosca.datatypes.nfv.Mask |

#### 7.2.17.2	Properties

The properties of the Mask data type shall comply with the provisions set out in table 7.2.17.2-1.

Table 7.2.17.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| starting_point | yes | integer |  | Indicates the offset between the last bit of the source mac address and the first bit of the sequence of bits to be matched. |
| length | yes | integer |  | Indicates the number of bits to be matched. |
| value | yes | string |  | Provides the sequence of bit values to be matched. |

#### 7.2.17.3	Definition

The syntax of the Mask data type shall comply with the following definition:

tosca.datatypes.nfv.Mask: derived_from: tosca.datatypes.Root properties: starting_point: description: Indicates the offset between the last bit of the source mac address and the first bit of the sequence of bits to be matched. type: integer required: true length: description: Indicates the number of bits to be matched. type: integer required: true value: description: Provide the sequence of bit values to be matched. type: string required: true

#### 7.2.17.4	Examples

None.

### 7.2.18	tosca.datatypes.nfv.NsOperationAdditionalParameters

#### 7.2.18.1	Description

The NsOperationAdditionalParameters data type is an empty base type  for deriving data types for describing NS specific additional parameters that affect the invocation of NS Lifecycle Management operations, as defined in ETSI GS NFV-IFA 014 [2]. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20]. Table 7.2.18.1-1 specifies the declared names for this data type.

Table 7.2.18.1-1: Type name, shorthand, and URI

| Shorthand Name | NsOperationAdditionalParameters |
| --- | --- |
| Type Qualified Name | toscanfv:NsOperationAdditionalParameters |
| Type URI | tosca.datatypes.nfv.NsOperationAdditionalParameters |

#### 7.2.18.2	Properties

None.

#### 7.2.18.3	Definition

The syntax of the NsOperationAdditionalParameters data type shall comply with the following definition:

tosca.datatypes.nfv.NsOperationAdditionalParameters: derived_from: tosca.datatypes.Root description: Is an empty base type for deriving data types for describing NS-specific additional parameters to be passed when invoking NS lifecycle management operations #properties:

#### 7.2.18.4	Examples

tosca_definitions_version: tosca_simple_yaml_1_3 node_types: tosca.example_NS: derived_from: tosca.nodes.nfv.NS properties: .. interfaces: Nslcm: operations: instantiate: inputs: additional_parameters: type: MyCompany.datatypes.nfv.NsInstantiateAdditionalParameters scale: inputs: additional_parameters: type: MyCompany.datatypes.nfv.NsScaleAdditionalParameters heal: inputs: additional_parameters: type: MyCompany.datatypes.nfv.NsHealAdditionalParameters data_types: MyCompany.datatypes.nfv.NsInstantiateAdditionalParameters: derived_from: tosca.datatypes.nfv.NsOperationAdditionalParameters properties: parameter_1: type: string required: true parameter_2: type: string required: true default: value_2 MyCompany.datatypes.nfv.NsScaleAdditionalParameters: derived_from: tosca.datatypes.nfv.NsOperationAdditionalParameters properties: parameter_1: type: string required: true parameter_2: type: string required: true MyCompany.datatypes.nfv.NsHealAdditionalParameters: derived_from: tosca.datatypes.nfv.NsOperationAdditionalParameters properties: parameter_1: type: string required: true parameter_2: type: string required: true default: value_2

### 7.2.19	tosca.datatypes.nfv.NsMonitoringParameter

#### 7.2.19.1	Description

This data type is used to specify information on virtualised resource related performance metrics to be monitored at the NS level. Table 7.2.19.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.19.1-1: Type name, shorthand, and URI

| Shorthand Name | NsMonitoringParameter |
| --- | --- |
| Type Qualified Name | toscanfv:NsMonitoringParameter |
| Type URI | tosca.datatypes.nfv.NsMonitoringParameter |

#### 7.2.19.2	Properties

The properties of the NsMonitoringParameter data type shall comply with the provisions set out in table 7.2.19.2-1.

Table 7.2.19.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Human readable name of the monitoring parameter. |
| performance_metric | yes | string | valid values: byte_incoming_sap byte_outgoing_sap packet_incoming_sap packet_outgoing_sap byte_incoming byte_outgoing packet_incoming packet_outgoing | Identifies a performance metric to be monitored. Performance metric values shall be either set to: A measurement name defined in clause 7.3 of ETSI GS NFV-IFA 027 [7]. In this case the NFVO computes these measurements from lower-level metrics collected from the VIM. A measurement name defined in clause 7.1 of ETSI GS NFV-IFA 027 [7], without appending a sub-counter. In this case the NFVO collects these metrics from the VIM for all network resources allocated to all NS virtual links. |
| collection_period | no | scalar-unit.time |  | Describes the periodicity at which to collect the performance information. |

#### 7.2.19.3	Definition

The syntax of the NsMonitoringParameter data type shall comply with the following definition:

tosca.datatypes.nfv.NsMonitoringParameter: derived_from: tosca.datatypes.Root description: Represents information on virtualised resource related performance metrics applicable to the NS. properties: name: type: string description: Human readable name of the monitoring parameter required: true performance_metric: type: string description: Identifies a performance metric to be monitored, according to ETSI GS NFV-IFA 027. required: true constraints: - valid_values: [byte_incoming_sap, byte_outgoing_sap, packet_incoming_sap, packet_outgoing_sap, byte_incoming, byte_outgoing, packet_incoming, packet_outgoing ] collection_period: type: scalar-unit.time description: Describes the periodicity at which to collect the performance information. required: false

#### 7.2.19.4	Examples

None.

#### 7.2.19.5	Additional Requirements

None.

### 7.2.20	tosca.datatypes.nfv.VnfMonitoringParameter

The VnfMonitoringParameter data type is defined in clause 9.2.9 of the present document.

### 7.2.21	tosca.datatypes.nfv.NsVirtualLinkProtocolData

#### 7.2.21.1	Description

The NsVirtualLinkProtocolData data type describes one protocol layer and associated protocol data for a given virtual link used in a specific NS deployment flavour. Table 7.2.21.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.21.1-1: Type name, shorthand, and URI

| Shorthand Name | NsVirtualLinkProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:NsVirtualLinkProtocolData |
| Type URI | tosca.datatypes.nfv.NsVirtualLinkProtocolData |

#### 7.2.21.2	Properties

The properties of the NsVirtualLinkProtocolData data type shall comply with the provisions set out in table 7.2.21.2-1.

Table 7.2.21.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| associated_layer_protocol | yes | string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | Identifies one of the protocols a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire) as specified by the connectivity_type property. |
| l2_protocol_data | no | tosca.datatypes.nfv.NsL2ProtocolData |  | Specifies the L2 protocol data for a virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L2 protocol and shall be absent otherwise. |
| l3_protocol_data | no | tosca.datatypes.nfv.NsL3ProtocolData |  | Specifies the L3 protocol data for this virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L3 protocol and shall be absent otherwise. |

#### 7.2.21.3	Definition

The syntax of the NsVirtualLinkProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.NsVirtualLinkProtocolData: derived_from: tosca.datatypes.Root description: describes one protocol layer and associated protocol data for a given virtual link used in a specific NS deployment flavour properties: associated_layer_protocol: type: string description: Identifies one of the protocols a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire) as specified by the connectivity_type property. required: true constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] l2_protocol_data: type: tosca.datatypes.nfv.NsL2ProtocolData description: Specifies the L2 protocol data for a virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L2 protocol and shall be absent otherwise. required: false l3_protocol_data: type: tosca.datatypes.nfv.NsL3ProtocolData description: Specifies the L3 protocol data for this virtual link. Shall be present when the associatedLayerProtocol attribute indicates a L3 protocol and shall be absent otherwise. required: false

#### 7.2.21.4	Examples

None.

#### 7.2.21.5	Additional Requirements

None.

### 7.2.22	tosca.datatypes.nfv.NsL2ProtocolData

#### 7.2.22.1	Description

The NsL2ProtocolData data type describes L2 protocol data for a given virtual link used in a specific NS deployment flavour. Table 7.2.22.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.22.1-1: Type name, shorthand, and URI

| Shorthand Name | NsL2ProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:NsL2ProtocolData |
| Type URI | tosca.datatypes.nfv.NsL2ProtocolData |

#### 7.2.22.2	Properties

The properties of the NsL2ProtocolData data type shall comply with the provisions set out in table 7.2.22.2-1.

Table 7.2.22.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Identifies the network name associated with this L2 protocol. |
| network_type | no | string | Valid values: flat, vlan, vxlan, gre | Specifies the network type for this L2 protocol. The value may be overridden at run-time. |
| vlan_transparent | no | boolean | default: false | Specifies whether to support VLAN transparency for this L2 protocol or not. |
| mtu | no | integer | greater_than : 0 | Specifies the maximum transmission unit (MTU) value for this L2 protocol. |
| segmentation_id | no | string |  | If present, specifies a specific virtualised network segment, which depends on the network type. For e.g. VLAN ID for VLAN network type and tunnel ID for GRE/VXLAN network types. See note. |
| NOTE: If this property is included in the NSD, the property value shall be provided at run-time, unless a default value is provided at design time in the NSD. If a default value is provided at design-time, this value may be overridden at run-time. |  |  |  |  |

#### 7.2.22.3	Definition

The syntax of the NsL2ProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.NsL2ProtocolData: derived_from: tosca.datatypes.Root description: describes L2 protocol data for a given virtual link used in a specific NS deployment flavour. properties: name: type: string description: Identifies the network name associated with this L2 protocol. required: false network_type: type: string description: Specifies the network type for this L2 protocol. The value may be overridden at run-time. required: false constraints: - valid_values: [ flat, vlan, vxlan, gre ] vlan_transparent: type: boolean description: Specifies whether to support VLAN transparency for this L2 protocol or not. required: false default: false mtu: type: integer description: Specifies the maximum transmission unit (MTU) value for this L2 protocol. required: false constraints: - greater_than: 0 segmentation_id: type: string description: Specifies a specific virtualised network segment, which depends on the network type. For e.g., VLAN ID for VLAN network type and tunnel ID for GRE/VXLAN network types required: false

#### 7.2.22.4	Examples

None.

#### 7.2.22.5	Additional Requirements

None.

### 7.2.23	tosca.datatypes.nfv.NsL3ProtocolData

#### 7.2.23.1	Description

The NsL3ProtocolData data type describes L3 protocol data for a given virtual link used in a specific NS deployment flavour. Table 7.2.23.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.23.1-1: Type name, shorthand, and URI

| Shorthand Name | NsL3ProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:NsL3ProtocolData |
| Type URI | tosca.datatypes.nfv.NsL3ProtocolData |

#### 7.2.23.2	Properties

The properties of the NsL3ProtocolData data type shall comply with the provisions set out in table 7.2.23.2-1.

Table 7.2.23.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Identifies the network name associated with this L3 protocol. |
| ip_version | yes | string | Valid values: ipv4 ipv6 | Specifies IP version of this L3 protocol. The value of the ip_version property shall be consistent with the value of the layer_protocol in the connectivity_type property of the virtual link node. |
| cidr | yes | string |  | Specifies the CIDR (Classless Inter-Domain Routing) of this L3 protocol. The value may be overridden at run-time. |
| ip_allocation_pools | no | list of tosca.datatypes.nfv.NsIpAllocationPool |  | Specifies the allocation pools with start and end IP addresses for this L3 protocol. The value may be overridden at run-time. |

#### 7.2.23.3	Definition

The syntax of the NsL3ProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.NsL3ProtocolData: derived_from: tosca.datatypes.Root description: describes L3 protocol data for a given virtual link used in a specific NS deployment flavour. properties: name: type: string description: Identifies the network name associated with this L3 protocol. required: false ip_version: type: string description: Specifies IP version of this L3 protocol. The value of the ip_version property shall be consistent with the value of the layer_protocol in the connectivity_type property of the virtual link node. required: true constraints: - valid_values: [ ipv4, ipv6 ] cidr: type: string description: Specifies the CIDR (Classless Inter-Domain Routing) of this L3 protocol. The value may be overridden at run-time. required: true ip_allocation_pools: type: list description: Specifies the allocation pools with start and end IP addresses for this L3 protocol. The value may be overridden at run-time. required: false entry_schema: type: tosca.datatypes.nfv.NsIpAllocationPool

#### 7.2.23.4	Examples

None.

#### 7.2.23.5	Additional Requirements

None.

### 7.2.24	tosca.datatypes.nfv.NsIpAllocationPool

#### 7.2.24.1	Description

The NsIpAllocationPool data type specifies a range of IP addresses. Table 7.2.24.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.24.1-1: Type name, shorthand, and URI

| Shorthand Name | NsIpAllocationPool |
| --- | --- |
| Type Qualified Name | toscanfv:NsIpAllocationPool |
| Type URI | tosca.datatypes.nfv.NsIpAllocationPool |

#### 7.2.24.2	Properties

The properties of the NsIpAllocationPool data type shall comply with the provisions set out in table 7.2.24.2-1.

Table 7.2.24.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| start_ip_address | yes | string |  | The IP address to be used as the first one in a pool of addresses derived from the cidr block full IP range |
| end_ip_address | yes | string |  | The IP address to be used as the last one in a pool of addresses derived from the cidr block full IP range |

#### 7.2.24.3	Definition

The syntax of the NsIpAllocationPool data type shall comply with the following definition:

tosca.datatypes.nfv.NsIpAllocationPool: derived_from: tosca.datatypes.Root description: Specifies a range of IP addresses properties: start_ip_address: type: string description: The IP address to be used as the first one in a pool of addresses derived from the cidr block full IP range required: true end_ip_address: type: string description: The IP address to be used as the last one in a pool of addresses derived from the cidr block full IP range required: true

#### 7.2.24.4	Examples

None.

#### 7.2.24.5	Additional Requirements

None.

### 7.2.25	tosca.datatypes.nfv.NsScalingAspect

#### 7.2.25.1	Description

The NsScalingAspect data type describes the details of an aspect used for horizontal scaling. Table 7.2.25.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.25.1-1: Type name, shorthand, and URI

| Shorthand Name | NsScalingAspect |
| --- | --- |
| Type Qualified Name | toscanfv:NsScalingAspect |
| Type URI | tosca.datatypes.nfv.NsScalingAspect |

#### 7.2.25.2	Properties

The properties of the NsScalingAspect data type shall comply with the provisions set out in table 7.2.25.2-1.

Table 7.2.25.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | yes | string |  | Human readable name of the NS scaling aspect. |
| description | yes | string |  | Human readable description of the NS scaling aspect. |
| ns_scale_levels | yes | map of tosca.datatypes.nfv.NsLevels |  | Description of the NS levels for this scaling aspect. |

#### 7.2.25.3	Definition

The syntax of the NsScalingAspect data type shall comply with the following definition:

tosca.datatypes.nfv.NsScalingAspect: derived_from: tosca.datatypes.Root description: describes the details of an aspect used for horizontal scaling properties: name: type: string description: Human readable name of the aspect required: true description: type: string description: Human readable description of the aspect required: true ns_scale_levels: type: map description: Description of the NS levels for this scaling aspect. required: true key_schema: type: integer # Integer type in order to number the levels. First level is level 0. entry_schema: type: tosca.datatypes.nfv.NsLevels

#### 7.2.25.4	Examples

See clause A.17.

#### 7.2.25.5	Additional Requirements

None.

### 7.2.26	tosca.datatypes.nfv.NsLevels

#### 7.2.26.1	Description

The NsLevels data type describes the Ns levels. Table 7.2.26.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.2.26.1-1: Type name, shorthand, and URI

| Shorthand Name | NsLevels |
| --- | --- |
| Type Qualified Name | toscanfv:NsLevels |
| Type URI | tosca.datatypes.nfv.NsLevels |

#### 7.2.26.2	Properties

The properties of the NsLevels data type shall comply with the provisions set out in table 7.2.26.2-1.

Table 7.2.26.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | yes | string |  | Human readable description of the NS level. |

#### 7.2.26.3	Definition

The syntax of the NsLevels data type shall comply with the following definition:

tosca.datatypes.nfv.NsLevels: derived_from: tosca.datatypes.Root description: describes the Ns levels properties: description: type: string description: Human readable description of the Ns level required: true

#### 7.2.26.4	Examples

See clause A.17.

#### 7.2.26.5	Additional Requirements

None.

## 7.3	Artifact Types

None.

## 7.4	Capabilities Types

### 7.4.1	tosca.capabilities.nfv.VirtualLinkable

#### 7.4.1.1	Description

The VirtualLinkable capability type is defined in clause 9.4.1 of the present document.

### 7.4.2	tosca.capabilities.nfv.Forwarding

#### 7.4.2.1	Description

The Forwarding capability type describes the capabilities related to nodes which can be pointed by tosca.relationships.nfv.ForwardTo relationship type. Table 7.4.2.1-1 specifies the declared names for this capability type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

NOTE:	The forwarding capability represents the ability of a CP or SAP to receive and forward traffic flows. Traffic flows can be received by a CP from an NS Virtual Link and forwarded to the VNF or PNF to which the CP is attached. Symmetrically, traffic flows can be received by an external CP of the VNF or PNF and forwarded to an NS Virtual Link. Traffic flows can be received by a SAP from an external link and forwarded to the NS to which the SAP is attached. Symmetrically, traffic flows can be received from the NS to which the SAP is attached and forwarded to an external link. An ingress CP is an external CP that forwards traffic to a VNF, PNF or NS while and egress CP is an external CP that forwards traffic outside a VNF, PNF or NS. The same CP may but need not play both roles.

Table 7.4.2.1-1: Type name, shorthand, and URI

| Shorthand Name | Forwarding |
| --- | --- |
| Type Qualified Name | toscanfv:Forwarding |
| Type URI | tosca.capabilities.nfv.Forwarding |

#### 7.4.2.2	Properties

None.

#### 7.4.2.3	Definition

The syntax of the Forwarding capability type shall comply with the following definition:

tosca.capabilities.nfv.Forwarding: derived_from: tosca.capabilities.Root

## 7.5	Requirements Types

None.

## 7.6	Relationship Types

### 7.6.1	tosca.relationships.nfv.VirtualLinksTo

#### 7.6.1.1	Description

The VirtualLinksTo relationship type is defined in clause 9.6.1 of the present document representing an association relationship between the VNF or PNF or Sap of a Nested NS and NsVirtualLink node types when used in an NSD.

### 7.6.2	tosca.relationships.nfv.ForwardTo

#### 7.6.2.1	Description

The ForwardTo relationship type represents an association between two node types which are a part of NFP. Table 7.6.2.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.6.2.1-1: Type name, shorthand, and URI

| Shorthand Name | ForwardTo |
| --- | --- |
| Type Qualified Name | toscanfv:ForwardTo |
| Type URI | tosca.nodes.relationships.ForwardTo |

#### 7.6.2.2	Properties

None.

#### 7.6.2.3	Definition

The syntax of the ForwardTo relationship type shall comply with the following definition:

tosca.relationships.nfv.ForwardTo: derived_from: tosca.relationships.Root valid_target_types: [ tosca.capabilities.nfv.Forwarding ]

## 7.7	Interface Types

### 7.7.1	tosca.interfaces.nfv.Nslcm

#### 7.7.1.1	Description

The tosca.interfaces.nfv.Nslcm interface type contains a set of TOSCA operations corresponding to the following NS LCM operations defined in ETSI GS NFV-IFA 013 [i.8]:

Instantiate NS

Scale NS

Update NS

Heal NS

Terminate NS

The interface also contains TOSCA operations corresponding to preamble and postamble to the execution of the aforementioned base operations. The name of these operations is constructed according to the following pattern:

<base_operation_name>_start for a preamble

<base_operation_name>_end for a postamble

The designations ("_start", "_end") in the name of TOSCA operations are postfixes so that related operations are adjacent in an alphabetical listing.

Table 7.7.1.1-1 specifies the declared names for this interface type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.7.1.1-1: Type name, shorthand, and URI

| Shorthand Name | Nslcm |
| --- | --- |
| Type Qualified Name | toscanfv:Nslcm |
| Type URI | tosca.interfaces.nfv.Nslcm |

#### 7.7.1.2	Definition

The syntax of the Nslcm interface type shall comply with the following definition:

tosca.interfaces.nfv.Nslcm: derived_from: tosca.interfaces.Root description: This interface encompasses a set of TOSCA operations corresponding to NS LCM operations defined in ETSI GS NFV-IFA 013. as well as to preamble and postamble procedures to the execution of the NS LCM operations. operations: instantiate_start: description: Preamble to execution of the instantiate operation instantiate: description: Base procedure for instantiating an NS, corresponding to the Instantiate NS operation defined in ETSI GS NFV-IFA 013. # inputs: # additional_parameters: # type: tosca.datatypes.nfv.NsOperationAdditionalParameters # required: false instantiate_end: description: Postamble to the execution of the instantiate operation terminate_start: description: Preamble to execution of the terminate operation terminate: description: Base procedure for terminating an NS, corresponding to the Terminate NS operation defined in ETSI GS NFV-IFA 013. terminate_end: description: Postamble to the execution of the terminate operation update_start: description: Preamble to execution of the update operation update: description: Base procedure for updating an NS, corresponding to the Update NS operation defined in ETSI GS NFV-IFA 013. update_end: description: Postamble to the execution of the update operation scale_start: description: Preamble to execution of the scale operation scale: description: Base procedure for scaling an NS, corresponding to the Scale NS operation defined in ETSI GS NFV-IFA 013. # inputs: # additional_parameters: # type: tosca.datatypes.nfv.NsOperationAdditionalParameters # required: false scale_end: description: Postamble to the execution of the scale operation heal_start: description: Preamble to execution of the heal operation heal: description: Base procedure for healing an NS, corresponding to the Heal NS operation defined in ETSI GS NFV-IFA 013. # inputs: # additional_parameters: # type: tosca.datatypes.nfv.NsOperationAdditionalParameters # required: false heal_end: description: Postamble to the execution of the heal operation

#### 7.7.1.3	Additional Requirements

The implementation and inputs keynames specified in TOSCA-Simple-Profile-YAML-v1.3 [20] for an operation definition may be included for each operation listed in the Nslcm interface definition.

When a TOSCA operation representing an NS LCM operation does not have an associated implementation keyname, the default implementation provided by the NFVO for this NS LCM operation applies.

The NSD consumer shall make available all parameters from the message invoking the NS LCM operation as inputs to the corresponding TOSCA interface operations. The inputs keyname can be used to specify additional input parameters for executing the operation.

In the operation definitions on the Nslcm interface, the additional_parameters (NS-specific extension of the tosca.datatypes.nfv.NsOperationAdditionalParameters) of inputs section describes the name and type of the additional parameters that can be submitted in the NS LCM operation request. Refer example in clause 7.2.17.

The implementation of preamble and postamble TOSCA operations (e.g. instantiate_start), if present, is invoked with the same parameters as the corresponding base TOSCA operation (e.g. instantiate). The inputs of the preamble and postamble operations shall not be defined in the NSD.

If an implementation is associated to a TOSCA operation that represents a preamble or a postamble to an NS LCM operation, the implementation logic is executed before or after the execution of the NS LCM operation implementation, respectively.

Starting with version 3.3.1 of the present document, the Nslcm interface type definition grammar was changed to support notifications and operations. For backward compatibility, TOSCA-Simple-Profile-YAML-v1.3 [20], clause 3.7.5.5 specifies the provisions for handling the previous grammar. Support of the Release 2 Nslcm interface type definition grammar can be removed in subsequent versions of the present document.

#### 7.7.1.4	Support of LCM scripts

In ETSI GS NFV-IFA 014 [2], the definition of the "LifeCycleManagementScript" information element of the NSD associates LCM scripts with events, where an event can be an external or an internal stimulus. These events are mapped to TOSCA operations of the NS node type in the following way:

external stimuli are mapped to TOSCA operations corresponding to the NS LCM operations defined in ETSI GS NFV-IFA 013 [i.8];

internal stimuli are mapped to preamble and postamble of these TOSCA operations.

LCM scripts can be regarded as artifacts that provide an NS-specific implementation of the TOSCA operation corresponding to the stimulus.

The script input parameters shall be provided to the script according to the declaration in the inputs field of the operation definition. The artifact type definition shall enable identifying the DSL used by the script. The artifact type definition for Python is provided in section 5.4.4.1 of TOSCA-Simple-Profile-YAML-v1.3 [20]. The artifact definition for Mistral is provided in clause A.7.2 of the present document.

#### 7.7.1.5	Examples

The following example template fragments illustrate the concept. An LCM script is associated with the instantiate_end operation. As no LCM script is associated to the instantiate operation, its default implementation runs and before running the post-instantiate-script. The inputs section of the instantiate_end operation definition provides additional input values to the post-instantiate-script, and the TOSCA artifacts definition conveys the type of DSL used as a scripting language.

tosca_definitions_version: tosca_simple_yaml_1_3 imports: - .. node_types: MyCompany.SunshineVPN.1_0.1_0: derived_from: tosca.nodes.nfv.NS .. topology_template: substitution_mappings: node_type: MyCompany.SunshineVPN.1_0.1_0 .. node_templates: SunshineVPN: type: MyCompany.SunshineVPN.1_0.1_0 .. interfaces: Nslcm: operations: instantiate_end: implementation: post-instantiate-script inputs: script_input_1: value_1 script_input_2: value_2 artifacts: post-instantiate-script: description: Instantiate workflow script type: tosca.artifacts.Implementation.Python file: instantiate.py #repository: .. #deploy_path: .. ..

## 7.8	Node Types

### 7.8.1	tosca.nodes.nfv.NS

#### 7.8.1.1	Description

The NFV Network Service (NS) node type describes an NS in terms of deployment, operational behaviour, and requirements, as defined by ETSI GS NFV-IFA 014 [2]. Table 7.8.1.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.8.1.1-1: Type name, shorthand, and URI

| Shorthand Name | NS |
| --- | --- |
| Type Qualified Name | toscanfv:NS |
| Type URI | tosca.nodes.nfv.NS |

#### 7.8.1.2	Properties

The properties of the NS node type shall comply with the provisions set out in table 7.8.1.2-1.

Table 7.8.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| descriptor_id | yes | string |  | Identifier of this NS descriptor. See note 2. |
| designer | yes | string |  | Identifies the designer of the NSD. |
| version | yes | string |  | Identifies the version of the NSD. |
| name | yes | string |  | Provides the human readable name of the NSD. |
| invariant_id | yes | string |  | Identifies an NSD in a version independent manner. This attribute is invariant across versions of NSD. See note 2. |
| flavour_id | yes | string |  | Identifier of this NS DF within the NSD. |
| ns_profile | no | tosca.datatypes.nfv.NsProfile |  | Specifies a profile of an NS, when this NS is used as nested NS within another NS. See note 1. |
| service_availability_level | no | integer | greater_or_equal: 1 | If present, specifies the service availability level for the NS instance. See note 3. |
| NOTE 1: This property is only used in an NS node template, when it is representing a nested NS within another NS. NOTE 2: The value of the descriptor_id string shall comply with an UUID format as specified in section 3 of IETF RFC 4122 [9]. NOTE 3: The value '1' expresses the highest service availability level. |  |  |  |  |

#### 7.8.1.3	Attributes

None.

#### 7.8.1.4	Requirements

The requirements of the NS node type shall comply with the provisions set out in table 7.8.1.4-1.

Table 7.8.1.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link |

#### 7.8.1.5	Capabilities

None.

#### 7.8.1.6	Definition

The syntax of the NS node type shall comply with the following definition:

tosca.nodes.nfv.NS: derived_from: tosca.nodes.Root properties: descriptor_id: type: string # UUID description: Identifier of this NS descriptor required: true designer: type: string description: Identifies the designer of the NSD. required: true version: type: string description: Identifies the version of the NSD. required: true name: type: string description: Provides the human readable name of the NSD. required: true invariant_id: # UUID type: string description: Identifies an NSD in a version independent manner. This attribute is invariant across versions of NSD required: true flavour_id: type: string description: Identifier of the NS Deployment Flavour within the NSD required: true ns_profile: type: tosca.datatypes.nfv.NsProfile description: Specifies a profile of a NS, when this NS is used as nested NS within another NS. required: false service_availability_level: type: integer description: Specifies the service availability level for the NS instance. required: false constraints: - greater_or_equal: 1 requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo node: tosca.nodes.nfv.NsVirtualLink occurrences: [ 0, 1 ] interfaces: Nslcm: type: tosca.interfaces.nfv.Nslcm

#### 7.8.1.7	Artifact

None.

#### 7.8.1.8	Additional requirements

For a given NSD, a new NS node type shall be defined following the below requirements:

- The node type shall be derived from: tosca.nodes.nfv.NS.

All properties listed in tosca.nodes.nfv.NS where the "required:" field is set to "true" shall be included with their values indicated as constraints.

Properties listed in in tosca.nodes.nfv.NS where the "required:" field is set to "false" may be included.

The capabilities, requirements, interfaces of tosca.nodes.nfv.NS shall be preserved.

Depending on the number of SAPs of the NS, additional requirements for VirtualLinkable capability shall be defined with the occurrences set to [ 0, 1 ]. In this case, it is the NSD author's choice to use the requirement for VirtualLinkable capability defined in the tosca.nodes.nfv.NS node type or use only the additional requirements defined in the derived NS specific node type. In the latter case, the virtual_link requirement should be included in the node type definition with occurrences [ 0, 0 ].

### 7.8.2	tosca.nodes.nfv.Sap

#### 7.8.2.1	Description

The Service Access Point (SAP) node type describes a connection point where an NS can be accessed, as defined by ETSI GS NFV-IFA 014 [2]. Table 7.8.2.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.8.2.1-1: Type name, shorthand, and URI

| Shorthand Name | Sap |
| --- | --- |
| Type Qualified Name | toscanfv:Sap |
| Type URI | tosca.nodes.nfv.Sap |

#### 7.8.2.2	Properties

The properties applied to Sap node are derived from Cp node type as defined in clause 9.8.1 of the present document.

#### 7.8.2.3	Attributes

None.

#### 7.8.2.4	Requirements

The requirements of the Sap node type shall comply with the provisions set out in table 7.8.2.4-1.

Table 7.8.2.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| external_virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |
| internal_virtual_link | yes | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability |

#### 7.8.2.5	Capabilities

None.

#### 7.8.2.6	Definition

The syntax of the Sap node type shall comply with the following definition:

tosca.nodes.nfv.Sap: derived_from: tosca.nodes.nfv.Cp description: node definition of SAP. requirements: - external_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [0, 1] - internal_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [1, 1]

#### 7.8.2.7	Additional requirements

A node template of this type is used to represent a SAP only in the case the Sap is connected to an NsVirtualLink inside an NSD. The node template has the following requirements:

internal_virtual_link requirement to allow to connect it to an NsVirtualLink inside an NSD;

external_virtual_link requirement to allow to connect it to an NsVirtualLink outside an NSD.

In the case where a Sap is exposed by a VNF external connection point, a PNF external connection point or a Sap of the nested NS, the Sap node type does not apply.

#### 7.8.2.8	Example

In a typical scenario, the Sap node template will be part of a service template representing a certain NS deployment flavour. The service template substitutes for a NS specific node type. In this substitution, the virtual_link requirement is mapped to the external_virtual_link requirement of the Sap node. This example is illustrated in clause A.7.3.

When a Sap re-exposes a VNF external connection point, the service template does not require an explicit node template of type Sap in a typical scenario where a NS specific node type is substituted by a service template representing a certain NS deployment flavour. In this substitution, the virtual_link requirement is mapped to the virtual_link requirement of the VNF node. This example is illustrated in clause A.7.2.

### 7.8.3	tosca.nodes.nfv.NsVirtualLink

#### 7.8.3.1	Description

The NsVirtualLink node type represents the NsVirtualLinkDesc information element as defined in ETSI GS NFV-IFA 014 [2], which describes the requirements for a virtual link of a network service. Table 7.8.3.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.8.3.1-1: Type name, shorthand, and URI

| Shorthand Name | NsVirtualLink |
| --- | --- |
| Type Qualified Name | toscanfv:NsVirtualLink |
| Type URI | tosca.nodes.nfv.NsVirtualLink |

#### 7.8.3.2	Properties

The properties of the NsVirtualLink node type shall comply with the provisions set out in table 7.8.3.2-1.

Table 7.8.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| vl_profile | yes | tosca.datatype.nfv.NsVlProfile |  | Specifies instantiation parameters for a virtual link of a particular NS deployment flavour. |
| connectivity_type | yes | tosca.datatypes.nfv.ConnectivityType |  | Network service virtual link connectivity type. |
| test_access | no | list of string | Valid values: passive_monitoring, active_loopback | Test access facilities available on the VL. |
| description | no | string |  | Human readable information on the purpose of the virtual link (e.g. VL for control plane traffic). |

#### 7.8.3.3	Attributes

None.

#### 7.8.3.4	Requirements

None.

#### 7.8.3.5	Capabilities

The capabilities of the NsVirtualLink node type shall comply with the provisions set out in table 7.8.3.5-1.

Table 7.8.3.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_linkable | tosca.capabilities.nfv.VirtualLinkable |  | VirtualLinkable capability |

#### 7.8.3.6	Definition

The syntax of the NsVirtualLink node type shall comply with the following definition:

tosca.nodes.nfv.NsVirtualLink: derived_from: tosca.nodes.Root description: node definition of Virtual Links properties: vl_profile: type: tosca.datatypes.nfv.NsVlProfile # only covers min/max bitrate requirements description: Specifies instantiation parameters for a virtual link of a particular NS deployment flavour. required: true connectivity_type: type: tosca.datatypes.nfv.ConnectivityType required: true test_access: type: list description: Test access facilities available on the VL required: false entry_schema: type: string constraints: - valid_values: [ passive_monitoring, active_loopback ] description: type: string required: false description: Human readable information on the purpose of the virtual link (e.g. VL for control plane traffic). capabilities: virtual_linkable: type: tosca.capabilities.nfv.VirtualLinkable

#### 7.8.3.7	Artifact

None.

#### 7.8.3.8	Additional Requirements

None.

#### 7.8.3.9	Example

None.

### 7.8.4	tosca.nodes.nfv.Cp

#### 7.8.4.1	Description

The Cp node type is defined in clause 9.8.1 of the present document.

### 7.8.5	tosca.nodes.nfv.NfpPositionElement

#### 7.8.5.1	Description

The NfpPositionElement node type represents the NfpPositionElement information element as defined in ETSI GS NFV-IFA 014 [2], which describes one or two CPD(s) or SAPD(s) for a given Vnf, Pnf or Ns. Table 7.8.5.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

A NfpPositionElement node type has a requirement for a forwarding capability to be exposed by the VNFs, PNFs or NSs, in order to re-expose this capability to an NfpPosition node type.

NOTE:	The NfpPosition and NfpPositionElement node types of the VNFFG model describe the entities in VIM for enabling packets/frames to traverse the constituent VNFs, PNFs or Nested NSs of the Network Forwarding Path.

Table 7.8.5.1-1: Type name, shorthand, and URI

| Shorthand Name | NfpPositionElement |
| --- | --- |
| Type Qualified Name | toscanfv:NfpPositionElement |
| Type URI | tosca.nodes.nfv.NfpPositionElement |

#### 7.8.5.2	Properties

None.

#### 7.8.5.3	Attributes

None.

#### 7.8.5.4	Requirements

The requirements of the NfpPositionElement node type shall comply with the provisions set out in table 7.8.5.4-1.

Table 7.8.5.4-1: Requirements

| Name | Required | Capability Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| profile_element | yes | tosca.capabilities.nfv.Forwarding |  | Describes the requirement for the constituent of the NfpPositionElement. |

#### 7.8.5.5	Capabilities

The capabilities of the NfpPositionElement node type shall comply with the provisions set out in table 7.8.5.5-1.

Table 7.8.5.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| forwarding | tosca.capabilities.nfv.Forwarding |  | NfpPositionElement forwarding capability |

#### 7.8.5.6	Definition

The syntax of the NfpPositionElement node type shall comply with the following definition:

tosca.nodes.nfv.NfpPositionElement derived_from: tosca.nodes.Root description: node definition of NfpPositionElement capabilities: forwarding: type: tosca.capabilities.nfv.Forwarding requirements: - profile_element: capability: tosca.capabilities.nfv.Forwarding relationship: tosca.relationships.nfv.ForwardTo occurrences: [ 1, 2 ] #When the number of occurrences is 1, the ingress and egress traffic is associated to a single VnfExtCp or Sap; When the number of occurrences is 2, the ingress VnfExtCp or Sap is associated to the first value and the egress VnfExtCp or Sap is associated to the second value.

#### 7.8.5.7	Artifact

None.

#### 7.8.5.8	Additional Requirements

The valid node types for the "profile_element" requirements shall be limited to VNF, PNF, NS and SAP.

#### 7.8.5.9	Example

None.

### 7.8.6	tosca.nodes.nfv.NFP

#### 7.8.6.1	Description

The NFP node type associates traffic flow criteria to a list of descriptors associated to the connection points and service access points to be visited by traffic flows matching these criteria. Table 7.8.6.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.8.6.1-1: Type name, shorthand, and URI

| Shorthand Name | NFP |
| --- | --- |
| Type Qualified Name | toscanfv:NFP |
| Type URI | tosca.nodes.nfv.NFP |

#### 7.8.6.2	Properties

None.

#### 7.8.6.3	Attributes

None.

#### 7.8.6.4	Requirements

The requirements of the NFP node type shall comply with the provisions set out in table 7.8.6.4-1.

Table 7.8.6.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| nfp_position | yes | tosca.capabilities.nfv.Forwarding |  |  |

#### 7.8.6.5	Capabilities

None.

#### 7.8.6.6	Definition

The syntax of the NFP node type shall comply with the following definition:

tosca.nodes.nfv.NFP: derived_from: tosca.nodes.Root description: node definition of NFP requirements: - nfp_position: capability: tosca.capabilities.nfv.Forwarding node: tosca.nodes.nfv.NfpPosition relationship: tosca.relationships.nfv.ForwardTo occurrences: [ 1, UNBOUNDED ]

### 7.8.7	tosca.nodes.nfv.NfpPosition

#### 7.8.7.1	Description

The NfpPosition node type describes the reference of one or more NfpPositionElements and rules on how to route traffic flows among VnfExtCp or SAP instances corresponding to these elements. Table 7.8.7.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

NOTE:	The NfpPosition and NfpPositionElement node types of the VNFFG model describe the entities in VIM for enabling packets/frames to traverse the constituent VNFs, PNFs or Nested NSs of the Network Forwarding Path.

Table 7.8.7.1-1: Type name, shorthand, and URI

| Shorthand Name | NfpPosition |
| --- | --- |
| Type Qualified Name | toscanfv:NfpPosition |
| Type URI | tosca.nodes.nfv.NfpPosition |

#### 7.8.7.2	Properties

The properties of the NfpPosition node type shall comply with the provisions set out in table 7.8.7.2-1.

Table 7.8.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| forwarding_behaviour | no | string | Possible values: "all", "lb", "ff" | Identifies a rule to apply to forward traffic to CP or SAP instances corresponding to the referenced NfpPositionElement(s). The minimum list of rules to be supported shall include: all = Traffic flows shall be forwarded simultaneously to all CP or SAP instances created from the referenced CP profile(s). lb (load balancing) = Traffic flows shall be forwarded to one CP or SAP instance created from the referenced CP profile(s) selected based on a load-balancing algorithm. The following value may be used as well: ff (fast failover) = Traffic flows shall be forwarded to the next CP or SAP in case they cannot be forwarded to a CP or SAP instance created from the referenced CP profile(s). See note. |
| forwarding_behaviour_input_parameters | no | map of string |  | Provides input parameters to configure the forwarding behaviour (e.g. identifies a load balancing algorithm). This property is reserved for future use in the present document. |
| NOTE: When no rules are provided and there are multiple CP or SAP instances corresponding to the referenced CP profile(s), the VIM and/or the NFVI are expected to apply NFP-independent rules determined by means outside the scope of the present document. |  |  |  |  |

#### 7.8.7.3	Attributes

None.

#### 7.8.7.4	Requirements

The requirements of the NfpPosition node type shall comply with the provisions set out in table 7.8.7.4-1.

Table 7.8.7.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| element | yes | tosca.capabilities.nfv.Forwarding |  | Specifies that an NfpPosition requires a node that has a forwarding capability. |

#### 7.8.7.5	Capabilities

The capabilities of the NfpPosition node type shall comply with the provisions set out in table 7.8.7.5-1.

Table 7.8.7.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| forwarding | tosca.capabilities.nfv.Forwarding |  | NfpPosition forwarding capability |

#### 7.8.7.6	Definition

The syntax of the NfpPosition node type shall comply with the following definition:

tosca.nodes.nfv.NfpPosition: derived_from: tosca.nodes.Root description: node definition of NFP position properties: forwarding_behaviour: type: string description: Identifies a rule to apply to forward traffic to CP or SAP instances corresponding to the referenced NfpPositionElement(s). constraints: - valid_values: [ all, lb, ff ] required: false # forwarding_behaviour_input_parameters: # description: Provides input parameters to configure the forwarding behaviour. # type: map # required: false # entry_schema: # type: string capabilities: forwarding: type: tosca.capabilities.nfv.Forwarding requirements: - element: capability: tosca.capabilities.nfv.Forwarding node: tosca.nodes.nfv.NfpPositionElement relationship: tosca.relationships.nfv.ForwardTo occurrences: [ 1, UNBOUNDED ]

#### 7.8.7.7	Artifact

None.

#### 7.8.7.8	Additional Requirements

None.

#### 7.8.7.9	Example

See clause A.14.

### 7.8.8	tosca.nodes.nfv.Forwarding

#### 7.8.8.1	Description

The Forwarding node type represents a point in the NS topology that can participate as a forwarding target in a network forwarding path. A template of this type is inserted between a virtual link (VirtualLinkable) requirement of a VNF/PNF node template (in effect, an external connection point of the VNF/PNF) or a Sap and the virtual link template satisfying this requirement. Table 7.8.8.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

NOTE:	The Forwarding node type is only used for the VNFFGD design. A node template with this type is only present in an NSD if at least one template of the VNFFG group type is included in the NSD.

Table 7.8.8.1-1: Type name, shorthand, and URI

| Shorthand Name | Forwarding |
| --- | --- |
| Type Qualified Name | toscanfv:Forwarding |
| Type URI | tosca.nodes.nfv.Forwarding |

#### 7.8.8.2	Properties

None.

#### 7.8.8.3	Attributes

None.

#### 7.8.8.4	Requirements

The requirements of the Forwarding node type shall comply with the provisions set out in table 7.8.8.4-1.

Table 7.8.8.4-1: Requirements

| Name | Capability Type | Constraints | Description |
| --- | --- | --- | --- |
| virtual_link | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirement for linking to a virtual linkable node type. |

#### 7.8.8.5	Capabilities

The capabilities of the Forwarding node type shall comply with the provisions set out in table 7.8.8.5-1.

Table 7.8.8.5-1: Capabilities

| Name | Type | Constraints | Description |
| --- | --- | --- | --- |
| forwarding | tosca.capabilities.nfv.Forwarding |  | The forwarding capability exposed by the node. |
| virtual_linkable | tosca.capabilities.nfv.VirtualLinkable |  | The virtual linkable capability exposed by the node. |

#### 7.8.8.6	Definition

The syntax of the Forwarding node type shall comply with the following definition:

tosca.nodes.nfv.Forwarding: derived_from: tosca.nodes.Root capabilities: virtual_linkable: type: tosca.capabilities.nfv.VirtualLinkable forwarding: type: tosca.capabilities.nfv.Forwarding occurrences: [ 1, 2 ] #When the number of occurrences is 1, the ingress and egress traffic is associated to a single VnfExtCp, PnfExtCp or Sap; When the number of occurrences is 2, the ingress VnfExtCp, PnfExtCp or Sap is associated to the first value and the egress VnfExtCp, PnfExtCp or Sap is associated to the second value. requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo

#### 7.8.8.7	Artifact

None.

#### 7.8.8.8	Additional Requirements

None.

#### 7.8.8.9	Example

See clause A.14.

## 7.9	Group Types

### 7.9.1	tosca.groups.nfv.NsPlacementGroup

#### 7.9.1.1	Description

The NsPlacementGroup group type is used for describing the affinity or anti-affinity relationship applicable between VNF instances created using different VNFDs, the Virtual Link instances created using different VLDs or the nested NS instances created using different NSDs when used in an NSD.

Table 7.9.1.1-1 specifies the declared names for this group type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.9.1.1-1: Type name, shorthand, and URI

| Shorthand Name | NsPlacementGroup |
| --- | --- |
| Type Qualified Name | toscanfv:NsPlacementGroup |
| Type URI | tosca.groups.nfv.NsPlacementGroup |

#### 7.9.1.2	Properties

The properties of the NsPlacementGroup group type shall comply with the provisions set out in table 7.9.1.2-1.

Table 7.9.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | yes | string |  | Human readable description of the group |

#### 7.9.1.3	Definition

The syntax of the NsPlacementGroup group type shall comply with the following definition:

tosca.groups.nfv.NsPlacementGroup: derived_from: tosca.groups.Root description: NsPlacementGroup is used for describing the affinity or anti-affinity relationship applicable between VNF instances created using different VNFDs, the Virtual Link instances created using different VLDs or the nested NS instances created using different NSDs when used in a NSD. properties: description: type: string description: Human readable description of the group required: true members: [tosca.nodes.nfv.VNF, tosca.nodes.nfv.NsVirtualLink, tosca.nodes.nfv.NS]

#### 7.9.1.4	Additional Requirements

A group with type tosca.groups.nfv.NsPlacementGroup shall contain more than one member when used as the target of an AffinityRule or AntiAffinityRule policy.

### 7.9.2	tosca.groups.nfv.VNFFG

#### 7.9.2.1	Description

The VNF Forwarding Graph (VNFFG) group type describes a topology of the NS or a portion of the NS and optionally forwarding rules, applicable to the traffic conveyed over this topology, as defined by ETSI GS NFV-IFA 014 [2]. Table 7.9.2.1-1 specifies the declared names for this group type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.9.2.1-1: Type name, shorthand, and URI

| Shorthand Name | VNFFG |
| --- | --- |
| Type Qualified Name | toscanfv:VNFFG |
| Type URI | tosca.groups.nfv.VNFFG |

#### 7.9.2.2	Properties

The properties of the VNFFG group type shall comply with the provisions set out in table 7.9.2.2-1.

Table 7.9.2.2-1: properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | yes | string |  | Human readable description of the group |

#### 7.9.2.3	Definition

The syntax of the VNFFG group type shall comply with the following definition:

tosca.groups.nfv.VNFFG: derived_from: tosca.groups.Root description: the VNFFG group type describes a topology of the NS or a portion of the NS, and optionally forwarding rules, applicable to the traffic conveyed over this topology properties: description: type: string description: Human readable description of the group required: true members:[ tosca.nodes.nfv.NFP, tosca.nodes.nfv.VNF, tosca.nodes.nfv.PNF, tosca.nodes.nfv.NS, tosca.nodes.nfv.NsVirtualLink, tosca.nodes.nfv.NfpPositionElement ]

#### 7.9.2.4	Additional Requirements

None.

#### 7.9.2.5	Example

See clause A.14.

## 7.10	Policy Types

### 7.10.1	NsAffinityRule, NsAntiAffinityRule

#### 7.10.1.1	Description

The NsAffinityRule and NsAntiAffinityRule policy describes the affinity or anti-affinity rules applicable for the defined target.

If there is only one node template with node type tosca.nodes.nfv.VNF or tosca.nodes.nfv.NsVirtualLink set as the targets, the NsAffinityRule or NsAntiAffinityRule applies between the instances to be created based on the same VNFD, or between VLs to be created based on the same VLD as described in ETSI GS NFV-IFA 014 [2].

If there are more than one node templates with node type tosca.nodes.nfv.VNF or tosca.nodes.nfv.NsVirtualLink or tosca.nodes.nfv.NS set as the targets, or a group with type tosca.groups.nfv.PlacementGroup which contains more than one members set as targets, the NsAffinityRule or NsAntiAffinityRule applies between VNF instances created using different VNFD, the Virtual Link instances created using different VLD or the nested NS instances created using different NSD. as described in ETSI GS NFV-IFA 014 [2].

Tables 7.10.1.1-1 and 7.10.1.1-2 specify the declared names for the policy types. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.1.1-1: Type name, shorthand, and URI

| Shorthand Name | NsAffinityRule |
| --- | --- |
| Type Qualified Name | toscanfv:NsAffinityRule |
| Type URI | tosca.policies.nfv.NsAffinityRule |

Table 7.10.1.1-2: Type name, shorthand, and URI

| Shorthand Name | NsAntiAffinityRule |
| --- | --- |
| Type Qualified Name | toscanfv:NsAntiAffinityRule |
| Type URI | tosca.policies.nfv.NsAntiAffinityRule |

#### 7.10.1.2	Properties

The properties of the NsAffinityRule and NsAntiAffinityRule types shall comply with the provisions set out in table 7.10.1.2-1.

Table 7.10.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| scope | Yes | String | Valid values : "nfvi_pop", "zone", "zone_group", "nfvi_node", "network_link_and_node". | Specifies the scope of the local affinity rule. See note. |
| NOTE: When used in an anti-affinity relationship, the "network_link_and_node" scope is conceptually similar to link and node disjoint paths capabilities used commonly in network Traffic Engineering (TE). For example, as in Fast Reroute Resource Reservation Protocol Traffic Engineering (RSVP-TE) for Label-Switched Path (LSP) tunnels as introduced in IETF RFC 4090 [i.17]. |  |  |  |  |

#### 7.10.1.3	Targets

The targets of the NsAffinityRule and NsAntiAffinityRule policy types shall comply with the provisions set out in table 7.10.1.3-1 when used in an NSD.

Table 7.10.1.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | Yes | string[] | Valid types: tosca.nodes.nfv.VNF, tosca.nodes.nfv.NsVirtualLink, tosca.nodes.nfv.NS, tosca.groups.nfv.NsPlacementGroup | In case of LocalAffinityOrAntiAffinityRule as defined in ETSI GS NFV-IFA 014 [2], the valid type of the targets is tosca.nodes.nfv.VNF or tosca.nodes.nfv.NsVirtualLink. In case of affinityOrAntiAffinityGroup as defined in ETSI GS NFV-IFA 014 [2], the valid types of the targets are: tosca.nodes.nfv.VNF, tosca.nodes.nfv.NsVirtualLink and tosca.nodes.nfv.NS or a tosca.groups.nfv.NsPlacementGroup. |

#### 7.10.1.4	Definition

The syntax of the NsAffinityRule policy type shall comply with the following definition:

tosca.policies.nfv.NsAffinityRule: derived_from: tosca.policies.Placement description: The NsAffinityRule describes the affinity rules applicable for the defined targets properties: scope: type: string description: Specifies the scope of the local affinity rule. required: true constraints: - valid_values: [ nfvi_node, zone, zone_group, nfvi_pop, network_link_and_node ] targets: [tosca.nodes.nfv.VNF, tosca.nodes.nfv.NsVirtualLink, tosca.nodes.nfv.NS, tosca.groups.nfv.NsPlacementGroup ]

The syntax of the NsAntiAffinityRule policy type shall comply with the following definition:

tosca.policies.nfv.NsAntiAffinityRule: derived_from: tosca.policies.Placement description: The NsAntiAffinityRule describes the anti-affinity rules applicable for the defined targets properties: scope: type: string description: Specifies the scope of the local affinity rule.. required: true constraints: - valid_values: [ nfvi_node, zone, zone_group, nfvi_pop, network_link_and_node ] targets: [tosca.nodes.nfv.VNF, tosca.nodes.nfv.NsVirtualLink, tosca.nodes.nfv.NS, tosca.groups.nfv.NsPlacementGroup ]

#### 7.10.1.5	Examples

The following example template fragments illustrate the concepts:

node_templates: VNF_1: type: tosca.nodes.nfv.exampleVNF policies: policy_affinity_local_VNF_1: type: tosca.policies.nfv.AffinityRule targets: [ VNF_1 ] properties: scope: nfvi_node

The above example illustrates a local affinity rule for all the instances of VNF_1.

node_template: VNF_1: type: tosca.nodes.nfv.Vdu.exampleVNF_1 VNF_2: type: tosca.nodes.nfv.Vdu.exampleVNF_2 groups: affinityOrAntiAffinityGroup_1: type: tosca.groups.nfv.PlacementGroup members: [ VNF_1, VNF_2 ] policies: policy_antiaffinity_group_1: type: tosca.policies.nfv.AntiAffinityRule targets: [ affinityOrAntiAffinityGroup_1 ] properties: scope: nfvi_node

The above example illustrates an anti-affinity policy among a group which contains VNF_1 and VNF_2 as members.

### 7.10.2	tosca.policies.nfv.NsSecurityGroupRule

#### 7.10.2.1	Description

The NsSecurityGroupRule policy type when used in an NSD specifies the matching criteria for the ingress and/or egress traffic to and from visited SAPs. Table 7.10.2.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.2.1-1: Type name, shorthand, and URI

| Shorthand Name | NsSecurityGroupRule |
| --- | --- |
| Type Qualified Name | toscanfv:NsSecurityGroupRule |
| Type URI | tosca.policies.nfv.NsSecurityGroupRule |

#### 7.10.2.2	Properties

None

#### 7.10.2.3	targets

The targets of the SecurityGroupRule policy types shall comply with the provisions set out in table 7.10.2.3-1.

Table 7.10.2.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | yes | string[] | Valid types: tosca.nodes.nfv.Sap. | Target connection points of Sap. |

#### 7.10.2.4	Definition

The syntax of the NsSecurityGroupRule policy type shall comply with the following definition:

tosca.policies.nfv.NsSecurityGroupRule: derived_from: tosca.policies.nfv.Abstract.SecurityGroupRule description: The NsSecurityGroupRule type is a policy type specified the matching criteria for the ingress and/or egress traffic to/from visited SAPs. targets: [ tosca.nodes.nfv.Sap ]

#### 7.10.2.5	Additional Requirements

None.

### 7.10.3	tosca.policies.nfv.NfpRule

#### 7.10.3.1	Description

The NfpRule policy type represents the NFP rule attribute of the Nfpd information element as defined in ETSI GS NFV-IFA 014 [2], which describes the conditions that shall be met in order for the NFP to be applicable to the packet. Table 7.10.3.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.3.1-1: Type name, shorthand, and URI

| Shorthand Name | NfpRule |
| --- | --- |
| Type Qualified Name | toscanfv:NfpRule |
| Type URI | tosca.policies.nfv.NfpRule |

#### 7.10.3.2	Properties

The properties of the NFP policy type shall comply with the provisions set out in table 7.10.3.2-1.

Table 7.10.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ether_destination_address | no | string |  | Indicates a destination Mac address. |
| ether_source_address | no | string |  | Indicates a source Mac address. |
| ether_type | no | string | ipv4, ipv6 | Indicates the protocol carried over the Ethernet layer. |
| vlan_tag | no | list of string |  | Indicates a VLAN identifier in an IEEE 802.1Q-2014 tag [16]. Multiple tags can be included for QinQ stacking. |
| protocol | no | string |  | Indicates the L4 protocol, For IPv4 [17] this corresponds to the field called "Protocol" to identify the next level protocol. For IPv6 [18] this corresponds to the field called the "Next Header" field. Permitted values: Any keyword defined in the IANA [19] protocol registry, e.g.: TCP UDP ICMP. |
| dscp | no | string |  | For IPv4 [17] a string of "0" and "1" digits that corresponds to the 6-bit Differentiated Services Code. Point (DSCP) field of the IP header. For IPv6 [18] a string of "0" and "1" digits that corresponds to the 6 differentiated services bits of the traffic class header field. |
| source_port_range | no | range | 0 - 65535 | Indicates a range of source ports. |
| destination_port_range | no | range | 0 - 65535 | Indicates a range of destination ports. |
| source_ip_address_prefix | no | string |  | Indicates the source IP address range in CIDR format. |
| destination_ip_address_prefix | no | string |  | Indicates the destination IP address range in CIDR format. |
| extended_criteria | no | list of tosca.datatypes.nfv.Mask |  | Indicates values of specific bits in a frame. |

#### 7.10.3.3	Targets

The targets of the NfpRule policy types shall comply with the provisions set out in table 7.10.3.3-1.

Table 7.10.3.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | yes | string[] | Valid types: tosca.nodes.nfv.NFP. | The NFPs to which the rule applies. |

#### 7.10.3.4	Definition

The syntax of the NfpRule policy type shall comply with the following definition:

tosca.policies.nfv.NfpRule: derived_from: tosca.policies.Root description: policy definition of NfpRule properties: ether_destination_address: description: Indicates a destination Mac address. type: string required: false ether_source_address: description: Indicates a source Mac address. type: string required: false ether_type: description: Indicates the protocol carried over the Ethernet layer. type: string constraints: - valid_values: [ ipv4, ipv6 ] required: false vlan_tag: description: Indicates a VLAN identifier in an IEEE 802.1Q-2014 tag [16]. Multiple tags can be included for QinQ stacking. type: list entry_schema: type: string required: false protocol: description: 'Indicates the L4 protocol, For IPv4 [17] this corresponds to the field called "Protocol" to identify the next level protocol. For IPv6 [18] this corresponds to the field is called the "Next Header" field. Permitted values: Any keyword defined in the IANA [19] protocol registry.' type: string required: false dscp: description: For IPv4 [17] a string of "0" and "1" digits that corresponds to the 6-bit Differentiated Services Code Point (DSCP) field of the IP header. For IPv6 [18] a string of "0" and "1" digits that corresponds to the 6 differentiated services bits of the traffic class header field. type: string required: false source_port_range: description: Indicates a range of source ports. type: range required: false constraints: - in_range: [ 0, 65535 ] destination_port_range: description: Indicates a range of destination ports. type: range required: false constraints: - in_range: [ 0, 65535 ] source_ip_address_prefix: description: Indicates the source IP address range in CIDR format. type: string required: false destination_ip_address_prefix: description: Indicates the destination IP address range in CIDR format. type: string required: false extended_criteria: description: Indicates values of specific bits in a frame. type: list entry_schema: type: tosca.datatypes.nfv.Mask required: false targets: [ tosca.nodes.nfv.NFP ]

#### 7.10.3.5	Example

None.

### 7.10.4	tosca.policies.nfv.NsMonitoring

#### 7.10.4.1	Description

The NsMonitoring policy type is a policy type representing the virtualised resource related performance metrics to be monitored during the lifetime of network service instance as defined in ETSI GS NFV-IFA 014 [2] and ETSI GS NFV-IFA 027 [7]. Table 7.10.4.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.4.1-1: Type name, shorthand, and URI

| Shorthand Name | NsMonitoring |
| --- | --- |
| Type Qualified Name | toscanfv:NsMonitoring |
| Type URI | tosca.policies.nfv.NsMonitoring |

#### 7.10.4.2	Properties

The properties of the NsMonitoring policy type shall comply with the provisions set out in table 7.10.4.2-1.

Table 7.10.4.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ns_monitoring_parameters | yes | map of tosca.datatypes.nfv.NsMonitoringParameter |  | Specifies a virtualised resource related performance metric to be monitored on the NS level. |

#### 7.10.4.3	targets

The targets of the NsMonitoring policy types shall comply with the provisions set out in table 7.10.4.3-1.

Table 7.10.4.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | yes | string | Valid types: tosca.nodes.nfv.NS | Specifies the services node type(s) to which the monitoring policy applies. |

#### 7.10.4.4	Definition

The syntax of the NsMonitoring policy type shall comply with the following definition:

tosca.policies.nfv.NsMonitoring: derived_from: tosca.policies.Root description: Policy type is used to identify information to be monitored during the lifetime of a network service instance as defined in ETSI GS NFV-IFA 014 [2]. properties: ns_monitoring_parameters: type: map #key: id description: Specifies a virtualised resource related performance metric to be monitored on the NS level. required: true entry_schema: type: tosca.datatypes.nfv.NsMonitoringParameter constraints: - min_length: 1 targets: [ tosca.nodes.nfv.NS ]

#### 7.10.4.5	Additional Requirements

When a policy of this type is specified in an NS service template, the targets set shall only include NS node template names that correspond to this NS or to a nested NS.

### 7.10.5	tosca.policies.nfv.VnfMonitoring

#### 7.10.5.1	Description

The VnfMonitoring policy type is a policy type representing the virtualised resource related performance metrics to be monitored during the lifetime of VNF instance as defined in ETSI GS NFV-IFA 014 [2] and ETSI GS NFV-IFA 027 [7]. Table 7.10.5.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.5.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfMonitoring |
| --- | --- |
| Type Qualified Name | toscanfv:VnfMonitoring |
| Type URI | tosca.policies.nfv.VnfMonitoring |

#### 7.10.5.2	Properties

The properties of the VnfMonitoring policy type shall comply with the provisions set out in table 7.10.5.2-1.

Table 7.10.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| vnf_monitoring_parameters | yes | map of tosca.datatypes.nfv.VnfMonitoringParameter |  | Specifies a virtualised resource related performance metric to be monitored on the VNF level. |

#### 7.10.5.3	targets

The targets of the VnfMonitoring policy types shall comply with the provisions set out in table 7.10.5.3-1.

Table 7.10.5.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | yes | string | Valid types: tosca.nodes.nfv.VNF | Specifies the VNF node type(s) to which the monitoring policy applies. |

#### 7.10.5.4	Definition

The syntax of the VnfMonitoring policy type shall comply with the following definition:

tosca.policies.nfv.VnfMonitoring: derived_from: tosca.policies.Root description: Policy type is used to identify information to be monitored during the lifetime of a VNF instance as defined in ETSI GS NFV-IFA 014 [2]. properties: vnf_monitoring_parameters: type: map #key: id description: Specifies a virtualised resource related performance metric to be monitored on the NS level. required: true entry_schema: type: tosca.datatypes.nfv.VnfMonitoringParameter constraints: - min_length: 1 targets: [ tosca.nodes.nfv.VNF ]

#### 7.10.5.5	Additional Requirements

When a policy of this type is specified in an NS service template, the targets set shall only include VNF node template names representing constituent VNFs for the NS deployment flavour corresponding to this NS.

### 7.10.6	tosca.policies.nfv.Abstract.SecurityGroupRule

#### 7.10.6.1	Description

The Abstract.SecurityGroupRule policy type is defined in clause 9.10.1 of the present document.

### 7.10.7	tosca.policies.nfv.NsScalingAspects

#### 7.10.7.1	Description

The NsScalingAspects type is a policy type representing the scaling aspects used for horizontal scaling as defined in ETSI GS NFV-IFA 014 [2]. This policy concerns the whole NS (deployment flavour) represented by the topology_template and thus has no explicit target list. Table 7.10.7.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.7.1-1: Type name, shorthand, and URI

| Shorthand Name | ScalingAspects |
| --- | --- |
| Type Qualified Name | toscanfv:NsScalingAspects |
| Type URI | tosca.policies.nfv.NsScalingAspects |

#### 7.10.7.2	Properties

The properties of the NsScalingAspects policy type shall comply with the provisions set out in table 7.10.5.2-1.

Table 7.10.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspects | yes | Map of tosca.datatypes.nfv.NsScalingAspect |  | Describe the details of a particular aspect including the corresponding NS levels. |

#### 7.10.7.3	Definition

The syntax of the NsScalingAspects policy type shall comply with the following definition:

tosca.policies.nfv.NsScalingAspects: derived_from: tosca.policies.Root description: The ScalingAspects type is a policy type representing the scaling aspects used for horizontal scaling as defined in ETSI GS NFV-IFA 014 [2] properties: aspects: type: map # key: aspectId description: Describe the details of a particular aspect including the corresponding NS levels. required: true entry_schema: type: tosca.datatypes.nfv.NsScalingAspect constraints: - min_length: 1

#### 7.10.7.4	Examples

See clause A.17.

### 7.10.8	tosca.policies.nfv.VnfToLevelMapping

#### 7.10.8.1	Description

The VnfToLevelMapping type is a policy type representing the number of VNF instances that correspond to every NS level of a particular aspect, as defined in ETSI GS NFV-IFA 014 [2]. Table 7.10.8.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.8.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfToLevelMapping |
| --- | --- |
| Type Qualified Name | toscanfv: VnfToLevelMapping |
| Type URI | tosca.policies.nfv.VnfToLevelMapping |

#### 7.10.8.2	Properties

The properties of the VnfToLevelMapping policy type shall comply with the provisions set out in table 7.10.8.2-1.

Table 7.10.8.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspect: | yes | string |  | Represents the scaling aspect to which this policy applies. |
| number_of_instances | yes | map of integer |  | Describes the number of VNF instances to be deployed for each NS level of a particular aspect. The first level is level 0. |

#### 7.10.8.3	Definition

The syntax of the VnfToLevelMapping policy type shall comply with the following definition:

tosca.policies.nfv.VnfToLevelMapping: derived_from: tosca.policies.Root description: The VnfToLevelMapping type is a policy type representing the number of VNF instances to be deployed at each NS level, as defined in ETSI GS NFV-IFA 014 [2] properties: aspect: type: string description: Represents the scaling aspect to which this policy applies required: true number_of_instances: type: map # key: Ns level description: Number of VNF instances to be deployed for each NS level. required: true key_schema: type: integer # First level is level 0. entry_schema: type: integer constraints: - min_length: 1 targets: [ tosca.nodes.nfv.VNF ]

#### 7.10.8.4	Examples

See clause A.17.

### 7.10.9	tosca.policies.nfv.NsToLevelMapping

#### 7.10.9.1	Description

The NsToLevelMapping type is a policy type representing the number of  instances of a nested NS that correspond to every NS level of the composite NS of a particular aspect, as defined in ETSI GS NFV-IFA 014 [2]. Table 7.10.9.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.9.1-1: Type name, shorthand, and URI

| Shorthand Name | NsToLevelMapping |
| --- | --- |
| Type Qualified Name | toscanfv: NsToLevelMapping |
| Type URI | tosca.policies.nfv.NsToLevelMapping |

#### 7.10.9.2	Properties

The properties of the NsToLevelMapping policy type shall comply with the provisions set out in table 7.10.9.2-1.

Table 7.10.9.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspect: | yes | string |  | Represents the scaling aspect to which this policy applies. |
| number_of_instances | yes | map of integer |  | Describes the number of NS instances of a nested NS to be deployed for each NS level of the composite NS of a particular aspect. The first level is level 0. |

#### 7.10.9.3	Definition

The syntax of the NsToLevelMapping policy type shall comply with the following definition:

tosca.policies.nfv.NsToLevelMapping: derived_from: tosca.policies.Root description: The NsToLevelMapping type is a policy type representing the number of NS instances of a nested NS to be deployed at each NS level of the composite NS, as defined in ETSI GS NFV-IFA 014 [2] properties: aspect: type: string description: Represents the scaling aspect to which this policy applies required: true number_of_instances: type: map # key: Ns level description: Number of NS instances of a nested NS to be deployed for each NS level of the composite NS. required: true key_schema: type: integer # First level is level 0. entry_schema: type: integer constraints: - min_length: 1 targets: [ tosca.nodes.nfv.NS ]

#### 7.10.9.4	Examples

See clause A.17.

### 7.10.10	tosca.policies.nfv.VirtualLinkToLevelMapping

#### 7.10.10.1	Description

The VirtualLinkToLevelMapping type is a policy type representing the bitrate requirements of a VL for every NS level of a particular aspect, as defined in ETSI GS NFV-IFA 014 [2]. Table 7.10.10.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.10.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkToLevelMapping |
| --- | --- |
| Type Qualified Name | toscanfv: VirtualLinkToLevelMapping |
| Type URI | tosca.policies.nfv.VirtualLinkToLevelMapping |

#### 7.10.10.2	Properties

The properties of the VirtualLinkToLevelMapping policy type shall comply with the provisions set out in table 7.10.10.2-1.

Table 7.10.10.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| aspect: | yes | string |  | Represents the scaling aspect to which this policy applies. |
| bit_rate_requirements | yes | map of tosca.datatypes.nfv. LinkBitrateRequirements |  | Describes the bitrate requirements of a VL for each NS level of a particular aspect. First level is level 0. |

#### 7.10.10.3	Definition

The syntax of the VirtualLinkToLevelMapping policy type shall comply with the following definition:

tosca.policies.nfv.VirtualLinkToLevelMapping: derived_from: tosca.policies.Root description: The VirtualLinkToLevelMapping type is a policy type representing the number of NS instances of a nested NS to be deployed at each NS level of the composite NS, as defined in ETSI GS NFV-IFA 014 [2] properties: aspect: type: string description: Represents the scaling aspect to which this policy applies required: true bit_rate_requirements: type: map # key: Ns level description: Bitrate requirements of a VL for each NS level. required: true key_schema: type: integer # First level is level 0. entry_schema: type: tosca.datatypes.nfv.LinkBitrateRequirements constraints: - min_length: 1 targets: [ tosca.nodes.nfv.NsVirtualLink ]

#### 7.10.10.4	Examples

See clause A.17.

### 7.10.11	tosca.policies.nfv.NsInstantiationLevels

#### 7.10.11.1	Description

The NsInstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour and including default instantiation level in term of the number of VNF instances and nested NS instances to be created as defined in ETSI GS NFV-IFA 014 [2]. This policy concerns the whole NS(deployment flavour) represented by the topology_template and thus has no explicit target list. Table 7.10.11.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.11.1-1: Type name, shorthand, and URI

| Shorthand Name | NsInstantiationLevels |
| --- | --- |
| Type Qualified Name | toscanfv:NsInstantiationLevels |
| Type URI | tosca.policies.nfv.NsInstantiationLevels |

#### 7.10.11.2	Properties

The properties of the NsInstantiationLevels policy type shall comply with the provisions set out in table 7.10.11.2-1.

Table 7.10.11.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ns_levels | yes | map of tosca.datatypes.nfv.NsLevels |  | Describes the various levels of resources that can be used to instantiate the NS using this flavour. |
| default_level | no | string |  | The default instantiation level for this flavour. |

#### 7.10.11.3	Definition

The syntax of the NsInstantiationLevels policy type shall comply with the following definition:

tosca.policies.nfv.NsInstantiationLevels: derived_from: tosca.policies.Root description: The NsInstantiationLevels type is a policy type representing all the instantiation levels of resources to be instantiated within a deployment flavour and including default instantiation level in term of the number of VNF and nested NS instances to be created as defined in ETSI GS NFV-IFA 014 [2]. properties: ns_levels: type: map # key: levelId description: Describes the various levels of resources that can be used to instantiate the VNF using this flavour. required: true entry_schema: type: tosca.datatypes.nfv.NsLevels constraints: - min_length: 1 default_level: type: string # levelId description: The default instantiation level for this flavour. required: false # required if multiple entries in ns_levels

### 7.10.12	tosca.policies.nfv.VnfToInstantiationLevelMapping

#### 7.10.12.1	Description

The VnfToIntantiationLevelMapping type is a policy type representing the number of VNF instances that correspond to every NS instantiation level, as defined in ETSI GS NFV-IFA 014 [2]. Table 7.10.12.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.12.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfToInstantiationLevelMapping |
| --- | --- |
| Type Qualified Name | toscanfv: VnfToInstantiationLevelMapping |
| Type URI | tosca.policies.nfv.VnfToInstantiationLevelMapping |

#### 7.10.12.2	Properties

The properties of the VnfToInstantiationLevelMapping policy type shall comply with the provisions set out in table 7.10.12.2-1.

Table 7.10.12.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| number_of_instances | yes | map of integer |  | Describes the number of VNF instances to be deployed for each NS instantiation level. |

#### 7.10.12.3	Definition

The syntax of the VnfToInstantiationLevelMapping policy type shall comply with the following definition:

tosca.policies.nfv.VnfToInstantiationLevelMapping: derived_from: tosca.policies.Root description: The VnfToInstantiationLevelMapping type is a policy type representing the number of VNF instances to be deployed at each NS instantiation level, as defined in ETSI GS NFV-IFA 014 [2] properties: number_of_instances: type: map # key: Ns instantiation level description: Number of VNF instances to be deployed for each NS instantiation level. required: true entry_schema: type: integer constraints: - min_length: 1 targets: [ tosca.nodes.nfv.VNF ]

#### 7.10.12.4	Examples

See clause A.17.

#### 7.10.12.5	Additional requirements

There shall be one VnfToInstantiationLevelMapping policy defined for each VNF of the NS. If no instances of a given VNF have to be deployed at NS instantiation time for a certain instantiation level, the number_of_instances in the corresponding map entry shall be set to 0.

### 7.10.13	tosca.policies.nfv.NsInstantiationToLevelMapping

#### 7.10.13.1	Description

The NsToInstantiationLevelMapping type is a policy type representing the number of  instances of a nested NS that correspond to every NS instantiation level of the composite NS, as defined in ETSI GS NFV-IFA 014 [2]. Table 7.10.13.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.13.1-1: Type name, shorthand, and URI

| Shorthand Name | NsToInstantiationLevelMapping |
| --- | --- |
| Type Qualified Name | toscanfv: NsToInstantiationLevelMapping |
| Type URI | tosca.policies.nfv.NsInstantiationToLevelMapping |

#### 7.10.13.2	Properties

The properties of the NsToInstantiationLevelMapping policy type shall comply with the provisions set out in table 7.10.13.2-1.

Table 7.10.13.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| number_of_instances | yes | map of integer |  | Describes the number of NS instances of a nested NS to be deployed for each NS instnatiation level of the composite NS. |

#### 7.10.13.3	Definition

The syntax of the NsToInstantiationLevelMapping policy type shall comply with the following definition:

tosca.policies.nfv.NsInstantiationToLevelMapping: derived_from: tosca.policies.Root description: The NsToInstantiationLevelMapping type is a policy type representing the number of NS instances of a nested NS to be deployed at each NS instantiation level of the composite NS, as defined in ETSI GS NFV-IFA 014 [2] properties: number_of_instances: type: map # key: Ns instantiation level description: Number of NS instances of a nested NS to be deployed for each NS instantiation level of the composite NS. required: true entry_schema: type: integer constraints: - min_length: 1 targets: [ tosca.nodes.nfv.NS ]

#### 7.10.13.4	Examples

See clause A.17.

#### 7.10.13.5	Additional requirements

There shall be one VnfToInstantiationLevelMapping policy defined for each nested NS of the composite NS. If no instances of a given nested NS  have to be deployed at NS instantiation time for a certain instantiation level, the number_of_instances in the corresponding map entry shall be set to 0.

### 7.10.14	tosca.policies.nfv.VirtualLinkToInstantiationLevelMapping

#### 7.10.14.1	Description

The VirtualLinkToInstantiationLevelMapping type is a policy type representing the bitrate requirements of a VL for every NS instantiation level of a particular aspect, as defined in ETSI GS NFV-IFA 014 [2]. Table 7.10.14.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 7.10.14.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkToInstantiationLevelMapping |
| --- | --- |
| Type Qualified Name | toscanfv: VirtualLinkToInstantiationLevelMapping |
| Type URI | tosca.policies.nfv.VirtualLinkToInstantiationLevelMapping |

#### 7.10.14.2	Properties

The properties of the VirtualLinkToInstantiationLevelMapping policy type shall comply with the provisions set out in table 7.10.14.2-1.

Table 7.10.14.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| bit_rate_requirements | yes | map of tosca.datatypes.nfv. LinkBitrateRequirements |  | Describes the bitrate requirements of a VL for each NS instantiation level. |

#### 7.10.14.3	Definition

The syntax of the VirtualLinkToInstantiationLevelMapping policy type shall comply with the following definition:

tosca.policies.nfv.VirtualLinkToInstantiationLevelMapping: derived_from: tosca.policies.Root description: The VirtualLinkToInstantiationLevelMapping type is a policy type describing the bitrate requirements of a VL at each NS instantiation level of the composite NS, as defined in ETSI GS NFV-IFA 014 [2] properties: bit_rate_requirements: type: map # key: Ns instantiation level description: Bitrate requirements of a VL for each NS instantiation level. required: true entry_schema: type: tosca.datatypes.nfv. LinkBitrateRequirements constraints: - min_length: 1 targets: [ tosca.nodes.nfv.NsVirtualLink ]

#### 7.10.14.4	Examples

See clause A.17.

## 7.11	NSD TOSCA service template design

### 7.11.1	General

The TOSCA service template design for an NSD in the general case uses two levels of service templates as described in clause 7.11.2. In this design, the top level contains an abstract NS node template, i.e. without an implementation of the creation operation and is therefore substituted by one of the lower level service templates. This design is applicable regardless of whether the NS has one or multiple deployment flavours.

In the particular case of an NS with only one deployment flavour there is an alternative design which is described in clause 7.11.3 and which uses only one service template.

### 7.11.2	Single or multiple deployment flavour design with two levels of service templates

NSD shall be implemented as one top-level service template and one or multiple lower level service templates, where each lower level service template represents a deployment flavour. A separate YAML file with an NS specific node type definition which shall be derived from tosca.nodes.nfv.NS node type as defined in clause 7.8.1 shall be provided and is also considered as a part of an NSD. The top level service template shall be the main entry point of the NSD file structure as specified in ETSI GS NFV-SOL 007 [i.11], i.e. the Entry-definitions file. The file names of all the lower service templates shall be declared as the value of the Other-Definitions key in the TOSCA.meta file of the NSD file structure as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

See clause A.11 for an example of NSD design with multiple deployment flavours.

The top level service template shall comply with TOSCA-Simple-Profile-YAML-v1.3 [20] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.3;

an import statement referencing a yaml file which contains an NS specific node type definition;

a topology template with a node template of the NS-specific node type, which shall include the flavour_id and other properties and the requirements as defined in clause 7.8.1, excluding the 'service_availability_level' properties; and

optionally, import statements referencing additional NS-specific files containing only type definitions used by this TOSCA service template.

The deployment and lifecycle management of instances of this NS node type is done by means of substitution by any of the lower level service templates as declared in the Other-Definitions of the TOSCA.meta file in the NSD file structure.

The lower level service template is an implementable TOSCA service template for the deployment of a specific deployment flavour.

The lower level service template shall comply with TOSCA-Simple-Profile-YAML-v1.3 [20] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.3;

an import statement referencing a yaml file which contains an NS specific node type definition which shall be derived from tosca.nodes.nfv.NS node type as defined in clause 7.8.1;

one of more import statements respectively referencing the yaml file which contains the included VNF specific node type definition if any or the included PNF specific node type definition if any or the included NS specific node type definition if any;

- If the imported files contain a topology template, this topology template shall be ignored during the parsing of the NSD. To facilitate parsing the NSD, the imported files may be included in the NSD file structure as specified in ETSI GS NFV-SOL 007 [i.11], in which case the node type definitions they contain shall be identical to those contained in the corresponding VNF package, PNF archive and NSD file structure.

NOTE 1: 	These node type definitions are only used to parse the NSD. For the LCM operations of the corresponding VNF or nested NS, such node type definitions do not apply, only the files included in the related VNF package or NSD file structure will be used.

optionally, import statements referencing additional NS-specific files containing type definitions used by this TOSCA service template; and

a topology template describing the internal topology of the NS with:

substitution_mappings indicating:

the same node type as defined in the NS specific node type definition service template;

a flavour_id property and its value as defined in substitution_filter which identifies the DF corresponding to this low level template within the NSD;

NOTE 2:	Starting with version 3.3.1 of the present document, the property_mapping grammar was changed to support substitution_filter. For backward compatibility, TOSCA-Simple-Profile-YAML-v1.3 [20], clause 3.8.8.3. specifies the provisions for handling the previous grammar. Support of the Release 2 property_mapping grammar can be removed in subsequent versions of the present document.

the mapping of the virtual_link requirements on SAPs;

a node template referencing the NS specific node type containing the 'service_availability_level' property with a value, if applicable, implementations of the operations of the LCM interface to be executed by the NFVO, if applicable, and

additional node templates of type VNF, PNF, NS, NsVirtualLink, Sap, etc. that define the topology and composition of the NS flavour, the dependency requirements as defined in TOSCA-Simple-Profile-YAML-v1.3 [20] may be used between different VNF node templates, or between a VNF node template and a nested NS node template, or between different nested NS node templates to specify the order in which instances of the VNFs and/or nested NSs have to be created.

Either the service_availability_level property in the NS node template or the service_availability_level attribute in any of the VNF node templates may be provided, but not both. Either the service_availability_level property in the NS node template or the service_availability_level attribute in any of the NsVirtualLink node templates may be provided, but not both.

NOTE 3:	The format and structure of an NSD file structure is defined in ETSI GS NFV-SOL 007 [i.11].

NOTE 4:	All the imported type definition files as indicated either in the top level service template or in any of the lower level service template are considered as parts of an NSD.

When the flavour_id of an NS has been chosen (e.g. through an input parameter of an NS instantiation request received by a NFVO) among the values included in the NS node type imported into the top level service template, it is then used as the filter for selecting a particular lower level TOSCA service template inside the NSD file structure as described in TOSCA-Simple-Profile-YAML-v1.3 [20].

### 7.11.3	Single deployment flavour design with one service template

In case of the single deployment flavour scenario with one service template design, the NSD shall use TOSCA-Simple-Profile-YAML-v1.3 [20] and shall include:

- an import statement referencing the TOSCA types definition file as defined in clause B.3;

- one of more import statements respectively referencing the yaml file which contains the included VNF specific node type definition if any or the included PNF specific node type definition if any or the included NS specific node type definition if any;

- If the imported files contain a topology template, this topology template shall be ignored during the parsing of the NSD. To facilitate parsing the NSD, the imported files may be included in the NSD file structure as specified in ETSI GS NFV-SOL 007 [i.11], in which case the node type definitions they contain shall be identical to those contained in the corresponding VNF package, PNF archive and NSD file structure.

- NOTE 1: 	These node type definitions are only used to parse the NSD. For the LCM operations of the corresponding VNF or nested NS, such node type definitions do not apply, only the files included in the related VNF package or NSD file structure will be used.

- optionally, import statements referencing additional NS-specific files containing only type definitions used by this TOSCA service template;

- an NS specific node type definition derived from the tosca.nodes.nfv.NS node type, as defined in clause 7.8.1; and

- a topology template describing the internal topology of the NS with:

substitution_mappings indicating the same NS specific node type and the mapping of the virtual_link requirements on SAPs;

a node template of this NS specific node type with the flavour_id and other properties and, if applicable, implementations of the operations of the LCM interface to be executed by the NFVO; and

additional node templates of type VNF, PNF, NS, NsVirtualLink, Sap, etc. that define the topology and composition of the NS flavour, the dependency requirements as defined in TOSCA-Simple-Profile-YAML-v1.3 [20] may be used between different VNF node templates, or between a VNF node template and a nested NS node template, or between different nested NS node templates to specify the order in which instances of the VNFs and/or nested NSs have to be created.

See clause A.8 for an example of NSD design with single deployment flavour.

NOTE 2:	The service template is deployed stand-alone, i.e. without performing a substitution. However including the substitution_mappings rule indicates its ability to substitute a node template of the NS specific node type, which may appear in an NSD.

NOTE 3:	All the imported type definition files as indicated in the service template are considered as parts of an NSD.

# 8	PNFD TOSCA model

## 8.1	Introduction

The PNFD information model specified by ETSI GS NFV-IFA 014 [2] is mapped to the TOSCA concepts. It represents as TOSCA topology template to be used by NFVO for preparing network connection.

Table 8.1-1 shows the TOSCA Type "derived from" values used when applying the TOSCA-Simple-Profile-YAML-v1.3 [20] to the PNFD.

Table 8.1-1: Mapping of ETSI GS NFV-IFA 014 [2] information elements with TOSCA types

| ETSI NFV Information Element ETSI GS NFV-IFA 014 [2] | TOSCA type | Derived from |
| --- | --- | --- |
| Pnf | tosca.nodes.nfv.PNF | tosca.nodes.Root |
| PnfExtCpd (PNF External Connection Point) | tosca.nodes.nfv.PnfExtCp | tosca.nodes.Root |

Figure 8.1-1 provides an overview of the TOSCA node types used to build a service template for a PNFD.

Figure 8.1-1: Service template PNFD overview

## 8.2	Data Types

### 8.2.1	tosca.datatypes.nfv.CpProtocolData

#### 8.2.1.1	Description

The CpProtocolData data type is defined in clause 9.2.6 of the present document.

### 8.2.2	tosca.datatypes.nfv.AddressData

#### 8.2.2.1	Description

The AddressData data type is defined in clause 9.2.3 of the present document.

### 8.2.3	tosca.datatypes.nfv.L2AddressData

#### 8.2.3.1	Description

The L2AddressData data type is defined in clause 9.2.1 of the present document.

### 8.2.4	tosca.datatypes.nfv.L3AddressData

#### 8.2.4.1	Description

The L3AddressData data type is defined in clause 9.2.2 of the present document.

### 8.2.5	tosca.datatypes.nfv.LocationInfo

#### 8.2.5.1	Description

The LocationInfo data type represents geographical information on the location where a PNF is deployed as specified in ETSI GS NFV-IFA 011 [1]. Table 8.2.5.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 8.2.5.1-1: Type name, shorthand, and URI

| Shorthand Name | LocationInfo |
| --- | --- |
| Type Qualified Name | toscanfv:LocationInfo |
| Type URI | tosca.datatypes.nfv.LocationInfo |

#### 8.2.5.2	Properties

The properties of the LocationInfo data type shall comply with the provisions set out in table 8.2.5.2-1.

Table 8.2.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| country_code | yes | string |  | Shall be a two-letter ISO 3166 [10] country code in capital letters. |
| civic_address_element | no | list of tosca.datatypes.nfv.CivicAddressElement |  | Elements composing the civic address where the PNF is deployed. |
| geographic_coordinates | no | tosca.datatypes.nfv.GeographicCoordinates |  | Geographic coordinates (e.g. Altitude, Longitude, Latitude) where the PNF is deployed. |

#### 8.2.5.3	Definition

The syntax of the LocationInfo data type shall comply with the following definition:

tosca.datatypes.nfv.LocationInfo: derived_from: tosca.datatypes.Root description: Represents geographical information on the location where a PNF is deployed. properties: country_code: type: string # two-letter ISO 3166 country code description: Country code required: true civic_address_element: type: list entry_schema type: tosca.datatypes.nfv.CivicAddressElement description: Elements composing the civic address where the PNF is deployed. required: false geographic_coordinates: type: tosca.datatypes.nfv.GeographicCoordinates description: Geographic coordinates (e.g. Altitude, Longitude, Latitude) where the PNF is deployed. required: false

#### 8.2.5.4	Examples

<some_tosca_entity>: properties: geographical_location_info: country_code: FR civic_address_element: - element_1 ca_type: 3 ca_value: Paris

#### 8.2.5.5	Additional Requirements

None.

### 8.2.6	tosca.datatypes.nfv.CivicAddressElement

#### 8.2.6.1	Description

The CivicAddressElement data type represents an element of a civic location as specified in IETF RFC 4776 [11]. Table 8.2.6.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 8.2.6.1-1: Type name, shorthand, and URI

| Shorthand Name | CivicAddressElement |
| --- | --- |
| Type Qualified Name | toscanfv: CivicAddressElement |
| Type URI | tosca.datatypes.nfv.CivicAddressElement |

#### 8.2.6.2	Properties

The properties of the CivicAddressElement data type shall comply with the provisions set out in table 8.2.6.2-1.

Table 8.2.6.2-1

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ca_type | yes | string |  | Describe the content type of caValue. The value of caType shall comply with section 3.4 of IETF RFC 4776 [11]. |
| ca_value | yes | string |  | Content of civic address element corresponding to the caType. The format caValue shall comply with section 3.4 of IETF RFC 4776 [11]. |

#### 8.2.6.3	Definition

The syntax of the CivicAddressElement data type shall comply with the following definition:

tosca.datatypes.nfv.CivicAddressElement: derived_from: tosca.datatypes.Root description: Represents an element of a civic location as specified in IETF RFC 4776 [11]. properties: ca_type: type: string # RFC4776 description: caType as per RFC4776 required: true ca_value: type: string # RFC4776 description: caValue as per RFC4776. required: true

#### 8.2.6.4	Examples

See clause 8.2.5.4.

#### 8.2.6.5	Additional Requirements

None.

### 8.2.7	tosca.datatypes.nfv.GeographicCoordinates

#### 8.2.7.1	Description

The GeographicCoordinates data type represents a geographic coordinate location as specified in IETF RFC 6225 [21]. Table 8.2.7.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 8.2.7.1-1: Type name, shorthand, and URI

| Shorthand Name | GeographicCoordinates |
| --- | --- |
| Type Qualified Name | toscanfv: GeographicCoordinates |
| Type URI | tosca.datatypes.nfv.GeographicCoordinates |

#### 8.2.7.2	Properties

The properties of the GeographicCoordinates data type shall comply with the provisions set out in table 8.2.7.2-1.

Table 8.2.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| latitude_uncertainty | no | string |  | Describe the content of latitude_uncertainty. The value of latitude_uncertainty shall comply with LatUnc in section 2.3 of IETF RFC 6225 [21]. |
| latitude | yes | string |  | Describe the content of latitude. The value of latitude shall comply with Latitude in section 2.3 of IETF RFC 6225 [21]. |
| longitude_ uncertainty | no | string |  | Describe the content of longitude_uncertainty. The value of longitude_uncertainty shall comply with LongUnc in section 2.3 of IETF RFC 6225 [21]. |
| longitude | yes | string |  | Describe the content type of longitude. The value of longitude shall comply with Longitude in section 2.3 of IETF RFC 6225 [21]. |
| altitude_type | yes | string |  | Describe the content type of altitude_type. The value of altitude_type shall comply with AType in section 2.4 of IETF RFC 6225 [21]. |
| altitude_uncertainty | no | string |  | Describe the content of altitude_uncertainty. The value of altitude_uncertainty shall comply with AltUnc in section 2.4 of IETF RFC 6225 [21]. |
| altitude | yes | string |  | Describe the content of altitude. The value of altitude shall comply with Altitude in section 2.4 of IETF RFC 6225 [21]. |

#### 8.2.7.3	Definition

The syntax of the GeographicCoordinates data type shall comply with the following definition:

tosca.datatypes.nfv.GeographicCoordinates: derived_from: tosca.datatypes.Root description: Represents an element of a geographic coordinate location as specified in IETF RFC 6225 [21]. properties: latitude_uncertainty: type: string # RFC 6225 description: LatUnc as per RFC 6225 required: false latitude: type: string # RFC 6225 description: Latitude value as per RFC 6225 required: true longitude_uncertainty: type: string # RFC 6225 description: LongUnc as per RFC 6225 required: false longitude: type: string # RFC 6225 description: Longitude value as per RFC 6225 required: true altitude_type: type: string # RFC 6225 description: AType value as per RFC 6225 required: true altitude_uncertainty: type: string # RFC 6225 description: AltUnc as per RFC 6225 required: false altitude: type: string # RFC 6225 description: Altitude value as per RFC 6225 required: true

#### 8.2.7.4	Examples

None.

#### 8.2.7.5	Additional Requirements

None.

## 8.3	Artifact Types

None.

## 8.4	Capabilities Types

### 8.4.1	tosca.capabilities.nfv.VirtualLinkable

#### 8.4.1.1	Description

The VirtualLinkable capability type is defined in clause 9.4.1 of the present document.

## 8.5	Requirements Types

None.

## 8.6	Relationship Types

### 8.6.1	tosca.relationships.nfv.VirtualLinksTo

#### 8.6.1.1	Description

The VirtualLinksTo relationship type is defined in clause 9.6.1 of the present document representing an association relationship between a PNF external connection point and an NsVirtualLink node type.

## 8.7	Interface Types

None.

## 8.8	Node Types

### 8.8.1	tosca.nodes.nfv.PNF

#### 8.8.1.1	Description

The Physical Network Function (PNF) node type describes a PNF in terms of deployment behaviour requirements, which it contains PNFD identifier, version and functional description and so on as defined by ETSI GS NFV-IFA 014 [2]. Table 8.8.1.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 8.8.1.1-1: Type name, shorthand, and URI

| Shorthand Name | PNF |
| --- | --- |
| Type Qualified Name | tosca:PNF |
| Type URI | tosca.nodes.nfv.PNF |

#### 8.8.1.2	Properties

The properties of the PNF node type shall comply with the provisions set out in table 8.8.1.2-1.

Table 8.8.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| descriptor_id | yes | string |  | Identifier of this PNFD information element. It uniquely identifies the PNFD. See note. |
| function_description | yes | string |  | Describes the PNF function. |
| provider | yes | string |  | Identifies the provider of the PNFD. |
| version | yes | string |  | Identifies the version of the PNFD. |
| descriptor_invariant_id | yes | string |  | Identifier of this PNFD in a version independent manner. This attribute is invariant across versions of PNFD. See note. |
| name | yes | string |  | Name to identify the PNFD. |
| geographical_location_info | no | tosca.datatype.nfv.LocationInfo |  | Provides information about the geographical location (e.g. geographic coordinates or address of the building, etc.) of the PNF. |
| NOTE: The value of the descriptor_id string and the value of the descriptor_invariant_id string shall comply with an UUID format as specified in section 3 of [9]. |  |  |  |  |

#### 8.8.1.3	Attributes

None.

#### 8.8.1.4	Requirements

The requirements of the VNF node type shall comply with the provisions set out in table 8.8.1.4-1.

Table 8.8.1.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Describes the requirements for linking to virtual link |

#### 8.8.1.5	Capabilities

None.

#### 8.8.1.6	Definition

The syntax of the PNF node type shall comply with the following definition:

tosca.nodes.nfv.PNF: derived_from: tosca.nodes.Root properties: descriptor_id: # instead of pnfd_id type: string # UUID required: true description: Identifier of this PNFD information element. It uniquely identifies the PNFD. function_description: type: string required: true description: Describes the PNF function. provider: type: string required: true description: Identifies the provider of the PNFD. version: type: string required: true description: Identifies the version of the PNFD. descriptor_invariant_id: # instead of pnfd-invariant-id type: string # UUID required: true description: Identifier of this PNFD in a version independent manner. This attribute is invariant across versions of PNFD. name: type: string required: true description: Name to identify the PNFD. geographical_location_info: type: tosca.datatypes.nfv.LocationInfo required: false description: Provides information about the geographical location (e.g. geographic coordinates or address of the building, etc.) of the PNF. requirements: - virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [ 0, 1 ] # Additional requirements shall be defined in the PNF specific node type (deriving from tosca.nodes.nfv.PNF) corresponding to NS virtual links that need to connect to PnfExtCps

#### 8.8.1.7	Artifact

None.

#### 8.8.1.8	Additional Requirements

For a given PNFD, a new PNF node type shall be defined following the below requirements:

- The node type shall be derived from: tosca.nodes.nfv.PNF.

All properties listed in tosca.nodes.nfv.PNF where the "required:" field is set to "true" shall be included with their values indicates as constraints.

The requirements of tosca.nodes.nfv.PNF shall be preserved.

Depending on the number of external connection points of the PNF that need to connect to NS virtual links, additional requirements for VirtualLinkable capability shall be defined. In this case, it is the PNFD author's choice to use the requirement for VirtualLinkable capability defined in the tosca.nodes.nfv.PNF node type or use only the additional requirements defined in the derived PNF specific node type. In the latter case, the virtual_link requirement should be included in the node type definition with occurrences [ 0, 0 ].

#### 8.8.1.9	Example

See clause A.10.

### 8.8.2	tosca.nodes.nfv.PnfExtCp

#### 8.8.2.1	Description

The PnfExtCp node type describes the characteristics of an external interface, a.k.a. an external CP, where to connect the PNF to a VL, as defined by ETSI GS NFV-IFA 014 [2]. Table 8.8.2.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 8.8.2.1-1: Type name, shorthand, and URI

| Shorthand Name | PnfExtCp |
| --- | --- |
| Type Qualified Name | toscanfv:PnfExtCp |
| Type URI | tosca.nodes.nfv.PnfExtCp |

#### 8.8.2.2	Properties

The properties applied to PnfExtCp node are derived from Cp node type.

#### 8.8.2.3	Attributes

None.

#### 8.8.2.4	Requirements

The requirements of the PnfExtCp node type shall comply with the provisions set out in table 8.8.2.4-1.

Table 8.8.2.4-1: Requirements

| Name | Required | Capability type | Constraints | Description |
| --- | --- | --- | --- | --- |
| external_virtual_link | no | tosca.capabilities.nfv.VirtualLinkable |  | Specifies that CP instances require to be connected to a node that has a VirtualLinkable capability. |

#### 8.8.2.5	Capabilities

None.

#### 8.8.2.6	Definition

The syntax of the PnfExtCp node type shall comply with the following definition:

tosca.nodes.nfv.PnfExtCp: derived_from: tosca.nodes.nfv.Cp description: node definition of PnfExtCp. requirements: - external_virtual_link: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [0, 1]

### 8.8.3	tosca.nodes.nfv.Cp

#### 8.8.3.1	Description

The Cp node type is defined in clause 9.8.1 of the present document.

## 8.9	Group Types

None.

## 8.10	Policy Types

### 8.10.1	tosca.policies.nfv.PnfSecurityGroupRule

#### 8.10.1.1	Description

The PnfSecurityGroupRule policy type when used in a PNFD specifies the matching criteria for the ingress and/or egress traffic to and from visited PNF external connection points. Table 8.10.1.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 8.10.1.1-1: Type name, shorthand, and URI

| Shorthand Name | PnfSecurityGroupRule |
| --- | --- |
| Type Qualified Name | toscanfv:PnfSecurityGroupRule |
| Type URI | tosca.policies.nfv.PnfSecurityGroupRule |

#### 8.10.1.2	Properties

None.

#### 8.10.1.3	targets

The targets of the SecurityGroupRule policy types shall comply with the provisions set out in table 8.10.1.3-1.

Table 8.10.1.3-1: Targets

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| targets | yes | string[] | Valid types: tosca.nodes.nfv.PnfExtCp | Target connection points of PnfExtCp |

#### 8.10.1.4	Definition

The syntax of the NsSecurityGroupRule policy type shall comply with the following definition:

tosca.policies.nfv.PnfSecurityGroupRule: derived_from: tosca.policies.nfv.Abstract.SecurityGroupRule description: The PnfSecurityGroupRule type is a policy type specified the matching criteria for the ingress and/or egress traffic to/from visited PNF external connection points. targets: [ tosca.nodes.nfv.PnfExtCp ]

#### 8.10.1.5	Additional Requirements

None.

### 8.10.2	tosca.policies.nfv.Abstract.SecurityGroupRule

#### 8.10.2.1	Description

The Abstract.SecurityGroupRule policy type is defined in clause 9.10.1 of the present document.

## 8.11	PNFD TOSCA service template design

### 8.11.1	General

One single TOSCA service template is used to design a PNFD which shall comply with TOSCA-Simple-Profile-YAML-v1.3 [20] and includes:

- an import statement referencing the TOSCA types definition file as defined in clause B.4;

- a PNF specific node type definition derived from the tosca.nodes.nfv.PNF node type, as defined in clause 8.8.1; and

- a topology template describing the internal topology of the PNF with:

substitution_mappings indicating the same PNF specific node type and the mapping of the virtual_link requirements on PNF external connection point;

a node template of this PNF specific node type with the properties as defined in tosca.nodes.nfv.PNF; and

additional node templates of type PnfExtCp that define the connection information of the PNF.

See clause A.10 for an example of PNFD design.

NOTE:	The service template is deployed stand-alone, i.e. without performing a substitution. However including the substitution_mappings rule indicate its ability to substitute a node template of the PNF specific node type, which may appear in an NSD.

# 9	Common Definitions

## 9.1	Introduction

This clause defines the TOSCA type definitions which are used by at least two types of deployment templates among those identified in clause 5.1.

## 9.2	Data Types

### 9.2.1	tosca.datatypes.nfv.L2AddressData

#### 9.2.1.1	Description

The L2AddressData data type describes the information on the MAC addresses to be assigned to a connection point as defined in ETSI GS NFV-IFA 011 [1]. Table 9.2.1.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.1.1-1: Type name, shorthand, and URI

| Shorthand Name | L2AddressData |
| --- | --- |
| Type Qualified Name | toscanfv:L2AddressData |
| Type URI | tosca.datatypes.nfv.L2AddressData |

#### 9.2.1.2	Properties

The properties of the L2AddressData data type shall comply with the provisions set out in table 9.2.1.2-1.

Table 9.2.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| mac_address_assignment | yes | boolean |  | Specify if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility. |

#### 9.2.1.3	Definition

The syntax of the L2AddressData data type shall comply with the following definition:

tosca.datatypes.nfv.L2AddressData: derived_from: tosca.datatypes.Root description: Describes the information on the MAC addresses to be assigned to a connection point. properties: mac_address_assignment: type: boolean description: Specifies if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility required: true

#### 9.2.1.4	Examples

<some_tosca_entity>: properties: l2_address_data: mac_address_assignment: true

#### 9.2.1.5	Additional Requirements

None.

### 9.2.2	tosca.datatypes.nfv.L3AddressData

#### 9.2.2.1	Description

The L3AddressData data type supports providing information about Layer 3 level addressing scheme and parameters applicable to a CP, as defined in ETSI GS NFV-IFA 011 [1]. Table 9.2.2.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.2.1-1: Type name, shorthand, and URI

| Shorthand Name | L3AddressData |
| --- | --- |
| Type Qualified Name | toscanfv:L3AddressData |
| Type URI | tosca.datatypes.nfv.L3AddressData |

#### 9.2.2.2	Properties

The properties of the L3AddressData data type shall comply with the provisions set out in table 9.2.2.2-1.

Table 9.2.2.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| ip_address_assignment | yes | boolean |  | Specify if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility. |
| floating_ip_activated | yes | boolean |  | Specify if the floating IP scheme is activated on the Connection Point or not. |
| ip_address_type | no | string | Valid values: ipv4, ipv6 | Define address type. The address type should be aligned with the address type supported by the layer_protocols properties of the connection point. |
| number_of_ip_address | no | Integer | greater_than: 0 | Minimum number of IP addresses to be assigned. |

#### 9.2.2.3	Definition

The syntax of the L3AddressData data type shall comply with the following definition:

tosca.datatypes.nfv.L3AddressData: derived_from: tosca.datatypes.Root description: Provides information about Layer 3 level addressing scheme and parameters applicable to a CP properties: ip_address_assignment: type: boolean description: Specifies if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility required: true floating_ip_activated: type: boolean description: Specifies if the floating IP scheme is activated on the Connection Point or not required: true ip_address_type: type: string description: Defines address type. The address type should be aligned with the address type supported by the layer_protocols properties of the connection point required: false constraints: - valid_values: [ ipv4, ipv6 ] number_of_ip_address: type: integer description: Minimum number of IP addresses to be assigned required: false constraints: - greater_than: 0

#### 9.2.2.4	Examples

<some_tosca_entity>: properties: l3_address_data: ip_address_assignment: true floating_ip_activated: true ip_address_type: ipv4 number_of_ip_address: 4

#### 9.2.2.5	Additional Requirements

None.

### 9.2.3	tosca.datatypes.nfv.AddressData

#### 9.2.3.1	Description

The AddressData data type describes information about the addressing scheme and parameters applicable to a CP, as defined in ETSI GS NFV-IFA 011 [1]. Table 9.2.3.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.3.1-1: Type name, shorthand, and URI

| Shorthand Name | AddressData |
| --- | --- |
| Type Qualified Name | toscanfv:AddressData |
| Type URI | tosca.datatypes.nfv.AddressData |

#### 9.2.3.2	Properties

The properties of the AddressData data type shall comply with the provisions set out in table 9.2.3.2-1.

Table 9.2.3.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| address_type | yes | string | Valid values: mac_address ip_address. | Describes the type of the address to be assigned to a connection point The content type shall be aligned with the address type supported by the layerProtocol property of the connection point. |
| l2_address_data | no | tosca.datatypes.nfv.L2AddressData | Shall be present when the address_type is mac_address. | Provides the information on the MAC addresses to be assigned to a connection point. |
| l3_address_data | no | tosca.datatypes.nfv.L3AddressData | Shall be present when the address_type is ip_address. | Provides the information on the IP addresses to be assigned to a connection point. |

#### 9.2.3.3	Definition

The syntax of the AddressData data type shall comply with the following definition:

tosca.datatypes.nfv.AddressData: derived_from: tosca.datatypes.Root description: Describes information about the addressing scheme and parameters applicable to a CP properties: address_type: type: string description: Describes the type of the address to be assigned to a connection point. The content type shall be aligned with the address type supported by the layerProtocol property of the connection point required: true constraints: - valid_values: [ mac_address, ip_address ] l2_address_data: type: tosca.datatypes.nfv.L2AddressData description: Provides the information on the MAC addresses to be assigned to a connection point. required: false l3_address_data: type: tosca.datatypes.nfv.L3AddressData description: Provides the information on the IP addresses to be assigned to a connection point required: false

#### 9.2.3.4	Examples

<some_tosca_entity>: properties: address_data: address_type: ip_address l3_address_data: ip_address_assignment: true floating_ip_activated: true ip_address_type: ipv4 number_of_ip_address: 4

#### 9.2.3.5	Additional Requirements

None.

### 9.2.4	tosca.datatypes.nfv.ConnectivityType

#### 9.2.4.1	Description

The ConnectivityType data type describes the protocol exposed by a virtual link and the flow pattern supported by the virtual link, as defined in ETSI GS NFV-IFA 011 [1]. Table 9.2.4.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.4.1-1: Type name, shorthand, and URI

| Shorthand Name | ConnectivityType |
| --- | --- |
| Type Qualified Name | toscanfv:ConnectivityType |
| Type URI | tosca.datatypes.nfv.ConnectivityType |

#### 9.2.4.2	Properties

The properties of the ConnectivityType shall comply with the provisions set out in table 9.2.4.2-1.

Table 9.2.4.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| layer_protocols | yes | list of string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | Identifies the protocol a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire). The top layer protocol of the virtualLink protocol stack shall always be provided. The lower layer protocols may be included when there are specific requirements on these layers. |
| flow_pattern | no | string | Valid values: line, tree, mesh | Identifies the flow pattern of the connectivity. |

#### 9.2.4.3	Definition

The syntax of the ConnectivityType data type shall comply with the following definition:

tosca.datatypes.nfv.ConnectivityType: derived_from: tosca.datatypes.Root description: describes additional connectivity information of a virtualLink properties: layer_protocols: type: list description: Identifies the protocol a virtualLink gives access to (ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire). The top layer protocol of the virtualLink protocol stack shall always be provided. The lower layer protocols may be included when there are specific requirements on these layers. required: true entry_schema: type: string constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] flow_pattern: type: string description: Identifies the flow pattern of the connectivity required: false constraints: - valid_values: [ line, tree, mesh ]

#### 9.2.4.4	Examples

<some_tosca_entity>: properties: connectivity_type: layer_protocol: - ipv4 flow_pattern: mesh

#### 9.2.4.5	Additional Requirements

None.

### 9.2.5	tosca.datatypes.nfv.LinkBitrateRequirements

#### 9.2.5.1	Description

The LinkBitrateRequirements data type describes the requirements in terms of bitrate for a virtual link. Table 9.2.5.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.5.1-1: Type name, shorthand, and URI

| Shorthand Name | LinkBitrateRequirements |
| --- | --- |
| Type Qualified Name | toscanfv:LinkBitrateRequirements |
| Type URI | tosca.datatypes.nfv.LinkBitrateRequirements |

#### 9.2.5.2	Properties

The properties of the LinkBitrateRequirements data type shall comply with the provisions set out in table 9.2.5.2-1.

Table 9.2.5.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| root | yes | integer | greater_or_equal: 0 | Specifies the throughput requirement in bits per second of the link (e.g. bitrate of E-Line, root bitrate of E-Tree, aggregate capacity of E-LAN). |
| leaf | no | integer | greater_or_equal: 0 | Specifies the throughput requirement in bits per second of leaf connections to the link when applicable to the connectivity type (e.g. for E-Tree and E-LAN branches). |

#### 9.2.5.3	Definition

The syntax of the LinkBitrateRequirements data type shall comply with the following definition:

tosca.datatypes.nfv.LinkBitrateRequirements: derived_from: tosca.datatypes.Root description: describes the requirements in terms of bitrate for a virtual link properties: root: type: integer # in bits per second description: Specifies the throughput requirement in bits per second of the link (e.g. bitrate of E-Line, root bitrate of E-Tree, aggregate capacity of E-LAN). required: true constraints: - greater_or_equal: 0 leaf: type: integer # in bits per second description: Specifies the throughput requirement in bits per second of leaf connections to the link when applicable to the connectivity type (e.g. for E-Tree and E LAN branches). required: false constraints: - greater_or_equal: 0

#### 9.2.5.4	Examples

None.

#### 9.2.5.5	Additional Requirements

None.

### 9.2.6	tosca.datatypes.nfv.CpProtocolData

#### 9.2.6.1	Description

The CpProtocolData data type describes and associates the protocol layer that a CP uses together with other protocol and connection point information. Table 9.2.6.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.6.1-1: Type name, shorthand, and URI

| Shorthand Name | CpProtocolData |
| --- | --- |
| Type Qualified Name | toscanfv:CpProtocolData |
| Type URI | tosca.datatypes.nfv.CpProtocolData |

#### 9.2.6.2	Properties

The properties of the CpProtocolData data type shall comply with the provisions set out in table 9.2.6.2-1.

Table 9.2.6.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| associated_layer_protocol | yes | string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | One of the values of the property layer_protocols of the CP. |
| address_data | no | list of tosca.datatypes.nfv.AddressData |  | Provides information on the addresses to be assigned to the CP. |

#### 9.2.6.3	Definition

The syntax of the CpProtocolData data type shall comply with the following definition:

tosca.datatypes.nfv.CpProtocolData: derived_from: tosca.datatypes.Root description: Describes and associates the protocol layer that a CP uses together with other protocol and connection point information properties: associated_layer_protocol: type: string description: One of the values of the property layer_protocols of the CP required: true constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] address_data: type: list description: Provides information on the addresses to be assigned to the CP entry_schema: type: tosca.datatypes.nfv.AddressData required: false

#### 9.2.6.4	Examples

None.

#### 9.2.6.5	Additional Requirements

None.

### 9.2.7	tosca.datatypes.nfv.Qos

#### 9.2.7.1	Description

The QoS describes QoS data type a given VL used in a VNF deployment flavour. Table 9.2.7.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.7.1-1: Type name, shorthand, and URI

| Shorthand Name | Qos |
| --- | --- |
| Type Qualified Name | toscanfv:Qos |
| Type URI | tosca.datatypes.nfv.Qos |

#### 9.2.7.2	Properties

The properties of the Qos data type shall comply with the provisions set out in table 9.2.7.2-1.

Table 9.2.7.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| latency | yes | scalar-unit.time | greater_than: 0 s | Specifies the maximum latency. |
| packet_delay_variation | yes | scalar-unit.time |  | Specifies the maximum jitter. |
| packet_loss_ratio | no | float | in_range: [0,1] | Specifies the maximum packet loss ratio. |

#### 9.2.7.3	Definition

The syntax of the Qos data type shall comply with the following definition:

tosca.datatypes.nfv.Qos: derived_from: tosca.datatypes.Root description: describes QoS data for a given VL used in a VNF deployment flavour properties: latency: type: scalar-unit.time #Number description: Specifies the maximum latency required: true constraints: - greater_than: 0 s packet_delay_variation: type: scalar-unit.time #Number description: Specifies the maximum jitter required: true constraints: - greater_or_equal: 0 s packet_loss_ratio: type: float description: Specifies the maximum packet loss ratio required: false constraints: - in_range: [ 0.0, 1.0 ]

#### 9.2.7.4	Examples

None.

#### 9.2.7.5	Additional Requirements

None.

### 9.2.8	tosca.datatypes.nfv.VnfProfile

#### 9.2.8.1	Description

The VnfProfile data type describes a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF as defined in ETSI GS NFV-IFA 014 [2]. Table 9.2.8.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.8.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfProfile |
| --- | --- |
| Type Qualified Name | toscanfv:VnfProfile |
| Type URI | tosca.datatypes.nfv.VnfProfile |

#### 9.2.8.2	Properties

The properties of the VnfProfile data type shall comply with the provisions set out in table 9.2.8.2-1.

Table 9.2.8.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| instantiation_Level | no | string |  | Identifier of the instantiation level of the VNF DF to be used for instantiation. If not present, the default instantiation level as declared in the VNFD shall be used. |
| min_number_of_instances | yes | integer | greater_or_equal : 0 | Minimum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. |
| max_number_of_instances | yes | integer | greater_or_equal : 0 | Maximum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. |
| service_availability_level | no | integer | greater_or_equal: 1 | If present, specifies the service availability level for the VNF instance created from this profile. See note. |
| NOTE: The value '1' expresses the highest service availability level. |  |  |  |  |

#### 9.2.8.3	Definition

The syntax of the VnfProfile data type shall comply with the following definition:

tosca.datatypes.nfv.VnfProfile: derived_from: tosca.datatypes.Root description: describes a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF. properties: instantiation_level: type: string description: Identifier of the instantiation level of the VNF DF to be used for instantiation. If not present, the default instantiation level as declared in the VNFD shall be used. required: false min_number_of_instances: type: integer description: Minimum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. required: true constraints: - greater_or_equal: 0 max_number_of_instances: type: integer description: Maximum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. required: true constraints: - greater_or_equal: 0 service_availability_level: type: integer description: Specifies the service availability level for the VNF instance created from this profile. required: false constraints: - greater_or_equal: 1

#### 9.2.8.4	Example

None.

#### 9.2.8.5	Additional Requirements

None.

### 9.2.9	tosca.datatypes.nfv.VnfMonitoringParameter

#### 9.2.9.1	Description

This data type provides information on virtualised resource related performance metrics applicable to VNF. Table 9.2.9.1-1 specifies the declared names for this data type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.2.9.1-1: Type name, shorthand, and URI

| Shorthand Name | VnfMonitoringParameter |
| --- | --- |
| Type Qualified Name | toscanfv:VnfMonitoringParameter |
| Type URI | tosca.datatypes.nfv.VnfMonitoringParameter |

#### 9.2.9.2	Properties

The properties of the VnfMonitoringParameter data type shall comply with the provisions set out in table 9.2.9.2-1.

Table 9.2.9.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| name | no | string |  | Human readable name of the monitoring parameter. |
| performance_metric | yes | string | valid values: v_cpu_usage_mean_vnf v_cpu_usage_peak_vnf v_memory_usage_mean_vnf v_memory_usage_peak_vnf v_disk_usage_mean_vnf v_disk_usage_peak_vnf byte_incoming_vnf_ext_cp byte_outgoing_vnf_ext_cp packet_incoming_vnf_ext_cp packet_outgoing_vnf_ext_cp | Identifies a performance metric to be monitored. Performance metric values shall be set to a measurement name defined in clause 7.2 of ETSI GS NFV-IFA 027 [7], without appending a sub-counter. In this case the VNFM computes these measurements from lower-level metrics collected from the VIM. |
| collection_period | no | scalar-unit.time |  | Describes the periodicity at which to collect the performance information. |

#### 9.2.9.3	Definition

The syntax of the VnfMonitoringParameter data type shall comply with the following definition:

tosca.datatypes.nfv.VnfMonitoringParameter: derived_from: tosca.datatypes.Root description: Represents information on virtualised resource related performance metrics applicable to the VNF. properties: name: type: string description: Human readable name of the monitoring parameter required: true performance_metric: type: string description: Identifies a performance metric to be monitored, according to ETSI GS NFV-IFA 027. required: true constraints: - valid_values: [ v_cpu_usage_mean_vnf, v_cpu_usage_peak_vnf, v_memory_usage_mean_vnf, v_memory_usage_peak_vnf, v_disk_usage_mean_vnf, v_disk_usage_peak_vnf, byte_incoming_vnf_ext_cp, byte_outgoing_vnf_ext_cp, packet_incoming_vnf_ext_cp, packet_outgoing_vnf_ext_cp ] collection_period: type: scalar-unit.time description: Describes the periodicity at which to collect the performance information. required: false constraints: - greater_than: 0 s

#### 9.2.9.4	Examples

See clause A.8.

#### 9.2.9.5	Additional Requirements

None.

## 9.3	Artifact Types

None.

## 9.4	Capabilities Types

### 9.4.1	tosca.capabilities.nfv.VirtualLinkable

#### 9.4.1.1	Description

A node type that includes the VirtualLinkable capability indicates that it can be pointed by tosca.relationships.nfv.VirtualLinksTo relationship type. which is used to model the association between a VduCpd and an intVirtualLinkDesc and the association between a VnfExtCpd and an intVirtualLinkDesc as specified in ETSI GS NFV-IFA 011 [1] as well as the association represented by the NsVirtualLinkConnectivity information element in ETSI GS NFV-IFA 014 [2]. Table 9.4.1.1-1 specifies the declared names for this capability type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.4.1.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinkable |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinkable |
| Type URI | tosca.capabilities.nfv.VirtualLinkable |

#### 9.4.1.2	Properties

None.

#### 9.4.1.3	Definition

The syntax of the VirtualLinkable capability type shall comply with the following definition:

tosca.capabilities.nfv.VirtualLinkable: derived_from: tosca.capabilities.Node description: A node type that includes the VirtualLinkable capability indicates that it can be pointed by tosca.relationships.nfv.VirtualLinksTo relationship type

### 9.4.2	Void

## 9.5	Requirements Types

None.

## 9.6	Relationship Types

### 9.6.1	tosca.relationships.nfv.VirtualLinksTo

#### 9.6.1.1	Description

This relationship type represents an association between the VduCp and VnfVirtualLink node types or the association between either a VnfExtCp, a PnfExtCp or a Sap and an NsVirtualLink node types. Table 9.6.1.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.6.1.1-1: Type name, shorthand, and URI

| Shorthand Name | VirtualLinksTo |
| --- | --- |
| Type Qualified Name | toscanfv:VirtualLinksTo |
| Type URI | tosca.relationships.nfv.VirtualLinksTo |

#### 9.6.1.2	Properties

None.

#### 9.6.1.3	Definition

The syntax of the VirtualLinksTo relationship type shall comply with the following definition:

tosca.relationships.nfv.VirtualLinksTo: derived_from: tosca.relationships.DependsOn description: Represents an association relationship between the VduCp and VnfVirtualLink node types or the association between either a VnfExtCp, a PnfExtCp or a Sap and an NsVirtualLink node types. valid_target_types: [ tosca.capabilities.nfv.VirtualLinkable ]

### 9.6.2	Void

### 9.6.3	tosca.relationships.nfv.VipVirtualLinksTo

#### 9.6.3.1	Description

This relationship type represents an association between the VipCp and a VnfVirtualLink node types or between the former and an NsVirtualLink node types. Table 9.6.3.1-1 specifies the declared names for this relationship type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.6.3.1-1: Type name, shorthand, and URI

| Shorthand Name | VipVirtualLinksTo |
| --- | --- |
| Type Qualified Name | toscanfv:VipVirtualLinksTo |
| Type URI | tosca.relationships.nfv.VipVirtualLinksTo |

#### 9.6.3.2	Properties

None.

#### 9.6.3.3	Definition

The syntax of the VipVirtualLinksTo relationship type shall comply with the following definition:

tosca.relationships.nfv.VipVirtualLinksTo: derived_from: tosca.relationships.DependsOn description: Represents an association relationship between the VipCp and a VnfVirtualLink node types or between the former and a NsVirtualLink node types. valid_target_types: [ tosca.capabilities.nfv.VirtualLinkable ]

## 9.7	Interface Types

None.

## 9.8	Node Types

### 9.8.1	tosca.nodes.nfv.Cp

#### 9.8.1.1	Description

A Cp node type represents the Cpd information element as defined in ETSI GS NFV-IFA 011 [1], which describes network connectivity to a compute resource or a VL. This is an abstract type used as parent for the various Cp node types. Table 9.8.1.1-1 specifies the declared names for this node type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.8.1.1-1: Type name, shorthand, and URI

| Shorthand Name | Cp |
| --- | --- |
| Type Qualified Name | toscanfv:Cp |
| Type URI | tosca.nodes.nfv.Cp |

#### 9.8.1.2	Properties

The properties of the Cp node type shall comply with the provisions set out in table 9.8.1.2-1.

Table 9.8.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| layer_protocols | yes | list of string | Valid values: ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire | Identifies which protocol the connection point uses for connectivity purposes. |
| role | no | string | valid values: root, leaf | Identifies the role of the port in the context of the traffic flow patterns in the VNF or parent NS. For example a VNF with a tree flow pattern within the VNF will have legal cpRoles of ROOT and LEAF. |
| description | no | string |  | Provides human-readable information on the purpose of the connection point (e.g. connection point for control plane traffic). |
| protocol | no | list of tosca.datatypes.nfv.CpProtocolData |  | Provides information on the addresses to be assigned to the connection point(s) instantiated from this Connection Point Descriptor. |
| trunk_mode | no | boolean |  | Information about whether the CP instantiated from this Cp is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. Absence of this property implies that trunkMode is not configured for the Cp i.e. It is equivalent to boolean value "false". |

#### 9.8.1.3	Attributes

None.

#### 9.8.1.4	Requirements

None.

#### 9.8.1.5	Capabilities

None.

#### 9.8.1.6	Definition

The syntax of the Cp node type shall comply with the following definition:

tosca.nodes.nfv.Cp: derived_from: tosca.nodes.Root description: Provides information regarding the purpose of the connection point properties: layer_protocols: type: list description: Identifies which protocol the connection point uses for connectivity purposes required: true entry_schema: type: string constraints: - valid_values: [ ethernet, mpls, odu2, ipv4, ipv6, pseudo-wire ] role: #Name in ETSI GS NFV-IFA 011: cpRole type: string description: Identifies the role of the port in the context of the traffic flow patterns in the VNF or parent NS required: false constraints: - valid_values: [ root, leaf ] description: type: string description: Provides human-readable information on the purpose of the connection point required: false protocol: type: list description: Provides information on the addresses to be assigned to the connection point(s) instantiated from this Connection Point Descriptor required: false entry_schema: type: tosca.datatypes.nfv.CpProtocolData trunk_mode: type: boolean description: Provides information about whether the CP instantiated from this Cp is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. Absence of this property implies that trunkMode is not configured for the Cp i.e. It is equivalent to boolean value "false". required: false

#### 9.8.1.7	Additional requirements

The 'protocol' property shall not be included in a derived PnfExtCp node and shall be included in all other cases.

## 9.9	Group Types

None.

## 9.10	Policy Types

### 9.10.1	tosca.policies.nfv.Abstract.SecurityGroupRule

#### 9.10.1.1	Description

The Abstract.SecurityGroupRule type represents an abstract policy type without any target requirements. Table 9.10.1.1-1 specifies the declared names for this policy type. These names shall be used as specified in TOSCA-Simple-Profile-YAML-v1.3 [20].

Table 9.10.1.1-1: Type name, shorthand, and URI

| Shorthand Name | Abstract.SecurityGroupRule |
| --- | --- |
| Type Qualified Name | toscanfv:Abstract.SecurityGroupRule |
| Type URI | tosca.policies.nfv.Abstract.SecurityGroupRule |

#### 9.10.1.2	Properties

The properties of the Abstract.SecurityGroupRule policy type shall comply with the provisions set out in table 9.10.1.2-1.

Table 9.10.1.2-1: Properties

| Name | Required | Type | Constraints | Description |
| --- | --- | --- | --- | --- |
| description | no | string |  | Human readable description of the security group rule. |
| direction | yes | string | ingress, egress | The direction in which the security group rule is applied. The direction of 'ingress' or 'egress' is specified against the associated CP. I.e. 'ingress' means the packets entering a CP, while 'egress' means the packets sent out of a CP. |
| ether_type | yes | string | ipv4, ipv6 | Indicates the protocol carried over the Ethernet layer. |
| protocol | yes | string | see note | Indicates the protocol carried over the IP layer. Permitted values: any protocol defined in the IANA protocol registry [19], e.g. TCP, UDP, ICMP, etc. |
| port_range_min | yes | integer | 0 - 65535 | Indicates minimum port number in the range that is matched by the security group rule. If a value is provided at design-time, this value may be overridden at run-time based on other deployment requirements or constraints. |
| port_range_max | yes | integer | 0 – 65535 | Indicates maximum port number in the range that is matched by the security group rule. If a value is provided at design-time, this value may be overridden at run-time based on other deployment requirements or constraints. |
| NOTE: "protocol" constraints values: hopopt, icmp, igmp, ggp, ipv4, st, tcp, cbt, egp, igp, bbn_rcc_mon, nvp_ii, pup, argus, emcon, xnet, chaos, udp, mux, dcn_meas, hmp, prm, xns_idp, trunk_1, trunk_2, leaf_1, leaf_2, rdp, irtp, iso_tp4, netblt, mfe_nsp, merit_inp, dccp, 3pc, idpr, xtp, ddp, idpr_cmtp, tp++, il, ipv6, sdrp, ipv6_route, ipv6_frag, idrp, rsvp, gre, dsr, bna, esp, ah, i_nlsp, swipe, narp, mobile, tlsp, skip, ipv6_icmp, ipv6_no_nxt, ipv6_opts, cftp, sat_expak, kryptolan, rvd, ippc, sat_mon, visa, ipcv, cpnx, cphb, wsn, pvp, br_sat_mon, sun_nd, wb_mon, wb_expak, iso_ip, vmtp, secure_vmtp, vines, ttp, iptm, nsfnet_igp, dgp, tcf, eigrp, ospfigp, sprite_rpc, larp, mtp, ax.25, ipip, micp, scc_sp, etherip, encap, gmtp, ifmp, pnni, pim, aris, scps, qnx, a/n, ip_comp, snp, compaq_peer, ipx_in_ip, vrrp, pgm, l2tp, ddx, iatp, stp, srp, uti, smp, sm, ptp, isis, fire, crtp, crudp, sscopmce, iplt, sps, pipe, sctp, fc, rsvp_e2e_ignore, mobility, udp_lite, mpls_in_ip, manet, hip, shim6, wesp, rohc. |  |  |  |  |

#### 9.10.1.3	Definition

The syntax of the Abstract.SecurityGroupRule policy type shall comply with the following definition:

tosca.policies.nfv.Abstract.SecurityGroupRule: derived_from: tosca.policies.Root description: The Abstract.SecurityGroupRule type represents an abstract policy type without any target requirements properties: description: type: string description: Human readable description of the security group rule. required: false direction: type: string description: The direction in which the security group rule is applied. The direction of 'ingress' or 'egress' is specified against the associated CP. I.e., 'ingress' means the packets entering a CP, while 'egress' means the packets sent out of a CP. required: true constraints: - valid_values: [ ingress, egress ] default: ingress ether_type: type: string description: Indicates the protocol carried over the Ethernet layer. required: true constraints: - valid_values: [ ipv4, ipv6 ] default: ipv4 protocol: type: string description: Indicates the protocol carried over the IP layer. Permitted values include any protocol defined in the IANA protocol registry, e.g. TCP, UDP, ICMP, etc. required: true constraints: - valid_values: [ hopopt, icmp, igmp, ggp, ipv4, st, tcp, cbt, egp, igp, bbn_rcc_mon, nvp_ii, pup, argus, emcon, xnet, chaos, udp, mux, dcn_meas, hmp, prm, xns_idp, trunk_1, trunk_2, leaf_1, leaf_2, rdp, irtp, iso_tp4, netblt, mfe_nsp, merit_inp, dccp, 3pc, idpr, xtp, ddp, idpr_cmtp, tp++, il, ipv6, sdrp, ipv6_route, ipv6_frag, idrp, rsvp, gre, dsr, bna, esp, ah, i_nlsp, swipe, narp, mobile, tlsp, skip, ipv6_icmp, ipv6_no_nxt, ipv6_opts, cftp, sat_expak, kryptolan, rvd, ippc, sat_mon, visa, ipcv, cpnx, cphb, wsn, pvp, br_sat_mon, sun_nd, wb_mon, wb_expak, iso_ip, vmtp, secure_vmtp, vines, ttp, iptm, nsfnet_igp, dgp, tcf, eigrp, ospfigp, sprite_rpc, larp, mtp, ax.25, ipip, micp, scc_sp, etherip, encap, gmtp, ifmp, pnni, pim, aris, scps, qnx, a/n, ip_comp, snp, compaq_peer, ipx_in_ip, vrrp, pgm, l2tp, ddx, iatp, stp, srp, uti, smp, sm, ptp, isis, fire, crtp, crudp, sscopmce, iplt, sps, pipe, sctp, fc, rsvp_e2e_ignore, mobility, udp_lite, mpls_in_ip, manet, hip, shim6, wesp, rohc ] default: tcp port_range_min: type: integer description: Indicates minimum port number in the range that is matched by the security group rule. If a value is provided at design-time, this value may be overridden at run-time based on other deployment requirements or constraints. required: true constraints: - greater_or_equal: 0 - less_or_equal: 65535 default: 0 port_range_max: type: integer description: Indicates maximum port number in the range that is matched by the security group rule. If a value is provided at design-time, this value may be overridden at run-time based on other deployment requirements or constraints. required: true constraints: - greater_or_equal: 0 - less_or_equal: 65535 default: 65535

#### 9.10.1.4	Additional Requirements

The design of security group rule follows a permissive model where all security group rules applied to a connection point are dealt with in an "OR" logic fashion, i.e. the traffic is allowed if it matches any security group rule applied to this connection point.

Annex A (informative):
Examples

# A.1	Deployment flavour design mapping

## A.1.1	Introduction

This clause describes the main design principle for VNF/NS deployment flavour and the mapping between VNF/NS deployment flavour elements to TOSCA concept.

## A.1.2	Design principle for VNF deployment flavour

Each Deployment flavour as specified in ETSI GS NFV-IFA 011 [1] describes a given deployment configuration of a VNF in terms of its internal topology and resource needs. Different deployment flavours can define different topologies of the same VNF, with different scalingAspect, different VDUs and different internal connectivity requirements. The idea of VNF deployment flavour as specified in [1] is that each deployment flavour describes the required vduProfiles and virtualLinkProfiles which are the additional instantiation data for the given VDUs and virtualLinks, once a specific deployment flavour has been chosen at the instantiation time, in order to successfully deploy the given VDUs and virtualLinks, the bindings between the VDU with the corresponding VduProfile and the virtualLinkDesc with the corresponding virtualLinkProfile are required.

To achieve the concept of deployment flavour by using TOSCA, the main design principle is to describe each deployment flavour as a standalone implementable TOSCA service template, and binding VDU and virtualLink with the corresponding VduProfile and virtualLinkProfile, respectively, together at the design time. Once a specific deployment flavour has been chosen at the instantiation time, the corresponding TOSCA service template will be used for deploying the VNF with the given deployment flavour.

Figure A.1.2-1 shows the general principle for a VNF deployment flavour design.

Figure A.1.2-1: General principle for a VNF deployment flavour design

## A.1.3	Design principle for NS deployment flavour

The design principle for NSD deployment flavour is the same with the VNF deployment flavour design. Each NS deployment flavour is described as a standalone implementable TOSCA service template, the constituent VNF, virtualLink, nested NS and PNF is bound with the corresponding VnfProfile, VirtualLinkProfile, NsProfile and virtual_link requirements respectively together at the design time. Once a specific deployment flavour has been chosen at the instantiation time, the corresponding TOSCA service template will be used for deploying the NS with the given deployment flavour.

Figure A.1.3-1 shows the general principle for a NS deployment flavour design.

Figure A.1.3-1: General principle for a NS deployment flavour design

# A.2	VNFD with deployment flavour modelling design example

Deployment flavours are represented as deployable TOSCA topology templates. This way one VNF service template represents one deployment flavour, and different deployment flavours are described by different VNF service templates. This is in line with the idea that different deployment flavours can define different topologies of the same VNF, with different scaling aspects, different VDUs and different internal connectivity.

In order to represent a VNF a top-level service template is used. This top-level service template contains a topology template with only an abstract VNF node which defines the common parts of the different deployment flavours (such as product information, modifiable attributes and parts of the lifecycle management interface definition). It also sets a constraint on the deployment flavour property (the required value of the flavour_id property); this constraint comes from the VNF instantiation request which contains a flavour_id selected among those available in the VNFD.

As a result, the VNFM will look into the available further service templates representing the different VNF deployment flavours of the VNF and use the one that has the matching flavour_id property value to substitute for the abstract VNF. These are the low-level service templates.

Figure A.2-1: VNFD overview with deployment flavour

A VNFD contains a TOSCA top-level Service Template as entry point in the VNF package and one or more TOSCA low-level Service templates representing the different deployment flavours (see figure A.2-1). The VNFD is interpreted by an NFVO or VNF manager. In this example, the templates describe two variants of the VNF each corresponding to a deployment flavour: a simple one and a complex one. The simple VNF consists of one server: a DB backend whereas, the complex VNF variant consists of minimum three DB backend servers and one serviceNode, which may be scaled out in one-size increments.

SunshineDB: VNFD-top level

sunshine.vnfd.tosca.yaml

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, non-scalable

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

- sunshineVNF.yaml # contains the VNF node type definition

topology_template:

inputs:

flavour_id:

type: string

description: VNF deployment flavour selected by the consumer. It is provided in the API

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

properties:

flavour_id: { get_input: flavour_id }

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider: MyCompany

product_name: SunshineDB

software_version: '1.0'

descriptor_version: '1.0'

vnfm_info:

- '0:MyCompany-1.0.0'

# requirements:

#- virtual_link_backend # mapped in lower-level templates

#- virtual_link_service # mapped in lower-level templates

# get_input function would be used to process the template at run time to access the selected flavour id. If the TOSCA functions are not supported by VNFM, the above function may not be needed, what other mechanisms can be used by VNFM which provide the same function are out of scope of the present document.

The sunshineVNF.yaml file has the following content:

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, non-scalable

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

data_types:

MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters:

derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters

properties:

parameter_1:

type: string

required: true

default: value_1

parameter_2:

type: string

required: true

default: value_2

node_types:

MyCompany.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: SunshineDB ]

default: SunshineDB

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ valid_values: [ simple, complex ] ]

default: simple

flavour_description:

type: string

default: "" #empty string

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

occurrences: [ 0, 0 ]

- virtual_link_backend:

capability: tosca.capabilities.nfv.VirtualLinkable

occurrences: [ 0, 1 ]

- virtual_link_service:

capability: tosca.capabilities.nfv.VirtualLinkable

occurrences: [ 0, 1 ]

interfaces:

Vnflcm:

type: tosca.interfaces.nfv.Vnflcm

operations:

instantiate:

inputs:

additional_parameters:

type: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters

The vnf node template in the sunshine.vnfd.tosca.yaml file is abstract and is subject to substitution; the lower-level templates in the subsequent sections provide these substitutions. The actual lower-level template is selected based on the node type and a value constraint on the flavour_id property.

Each low level service template contains a node template of type MyCompany.SunshineDB.1_0.1_0 with implementation of the LCM interfaces.

SunshineDB (simple): Lower level

sunshinedbsimple.vnfd.tosca.yaml

This example illustrates one Vdu.Compute nodes (dbBackend) with two connection points and two virtual links (see figure A.2-2). The flavour_id is "Simple".

dbBackend

dbBackendInternalCp

internalVl

dbBackendCp

externalVl

VNF

Figure A.2-2: SunshineDB (simple): Lower level

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, simple

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

- sunshineVNF.yaml # contains the VNF node type definition

node_types:

MyCompany.nodes.nfv.Vdu.Aux:

derived_from: tosca.nodes.nfv.Vdu.Compute

properties:

configurable_properties:

type: MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties

required: false

data_types:

MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfcConfigurableProperties

properties:

additional_vnfc_configurable_properties:

type: MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties

required: true

MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties

properties:

name_prefix_in_vim:

type: string

required: true

default: "MyCustomer"

dns_server:

type: string

required: true

default: "90.200.250.57"

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

substitution_filter:

properties:

- flavour_id: { equal: simple }

requirements:

virtual_link_backend: [ dbBackendCp, virtual_link ]  # IPv4 for SQL

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

properties:

flavour_description: A simple flavour

interfaces:

Vnflcm:

operations:

instantiate:

implementation: instantiate.workbook.mistral.yaml

terminate:

implementation: terminate.workbook.mistral.yaml

dbBackend:

type: MyCompany.nodes.nfv.Vdu.Aux

properties:

name: dbBackend

description: dbBackend compute node

nfvi_constraints:

key_1: value_1

key_2: value_2

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 8192 MiB

virtual_cpu:

cpu_architecture: x86

num_virtual_cpu: 2

virtual_cpu_clock: 1800 MHz

requirements:

- virtual_storage: mariaDbStorage

mariaDbStorage:

type: tosca.nodes.nfv.Vdu.VirtualBlockStorage

properties:

virtual_block_storage_data:

size_of_storage: 100 GB

rdma_enabled: true

..

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: maria.db.image.v1.0.qcow2

properties:

name: Software of Maria Db

version: '1.0'

checksum:

algorithm: sha-256

hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

container_format: bare

disk_format: qcow2

min_disk: 2 GB

min_ram: 8192 MiB

size: 2 GB

operating_system: Linux

supported_virtualisation_environments:

- KVM

dbBackendCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access the DB on IPv4

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: dbBackend

# - virtual_link: # the target node is determined in the NSD

dbBackendInternalCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: dbBackend

- virtual_link: internalVl

internalVl:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

flow_pattern: mesh

description: Internal Virtual link in the VNF

vl_profile:

max_bitrate_requirements:

root: 100000

leaf: 20000

min_bitrate_requirements:

root: 10000

leaf: 10000

SunshineDB (complex): Lower level

This example illustrates two Vdu.Compute nodes (dbBackend, and serviceNode) with two connection points and two virtual links (see figure A.2-3). The flavour_id is "complex".

dbBackend

dbBackendInternalCp

internalVl

serviceNode

serviceNodeInternalCp

serviceNodeCp

dbBackendCp

externalVl

VNF

NOTE:	The single external VL above illustrates that both serviceNodeCp and dbBackendCp are connected to the same external VL. Alternatively, external connection points can be connected to separate external VLs.

Figure A.2-3: SunshineDB (complex): Lower level

sunshinedbcomplex.vnfd.tosca.yaml

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, complex

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

- sunshineVNF.yaml # contains the VNF node type definition

node_types:

MyCompany.nodes.nfv.Vdu.Aux:

derived_from: tosca.nodes.nfv.Vdu.Compute

properties:

configurable_properties:

type: MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties

required: false

data_types:

MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfcConfigurableProperties

properties:

additional_vnfc_configurable_properties:

type: MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties

required: true

MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties

properties:

name_prefix_in_vim:

type: string

required: true

default: "MyCustomer"

dns_server:

type: string

required: true

default: "90.200.250.57"

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

substitution_filter:

properties:

- flavour_id: { equal: complex }

requirements:

virtual_link_backend: [ dbBackendCp, virtual_link ]  # IPv4 for SQL

virtual_link_service: [ serviceNodeCp, virtual_link ]  # IPv4 for SSH

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

properties:

flavour_description: A complex flavour

interfaces:

Vnflcm:

operations:

instantiate:

implementation: instantiate.workbook.mistral.yaml

terminate:

implementation: terminate.workbook.mistral.yaml

heal:

implementation: heal.workbook.mistral.yaml

dbBackend:

type: MyCompany.nodes.nfv.Vdu.Aux

properties:

name: dbBackend

description: dbBackend compute node

nfvi_constraints:

key_1: value_1

key_2: value_2

vdu_profile:

min_number_of_instances: 3

max_number_of_instances: 4

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 8192 MiB

virtual_cpu:

cpu_architecture: x86

num_virtual_cpu: 2

virtual_cpu_clock: 1800 MHz

requirements:

- virtual_storage: mariaDbStorage

serviceNode:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: serviceNode

description: brief description about serviceNode

nfvi_constraints:

key_3: value_3

key_4: value_4

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 8192 MiB

virtual_cpu:

cpu_architecture: x86

num_virtual_cpu: 2

virtual_cpu_clock: 1800 MHz

requirements:

- virtual_storage: mariaDbStorage

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: maria.db.image.v1.0.qcow2

properties:

name: Software of Maria Db

version: '1.0'

checksum:

algorithm: sha-256

hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

container_format: bare

disk_format: qcow2

min_disk: 2 GB

min_ram: 8192 MiB

size: 2 GB

operating_system: Linux

supported_virtualisation_environments:

- KVM

mariaDbStorage:

type: tosca.nodes.nfv.Vdu.VirtualBlockStorage

properties:

virtual_block_storage_data:

size_of_storage: 100 GB

rdma_enabled: true

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: maria.db.image.v1.0.qcow2

properties:

name: Software of Maria Db

version: '1.0'

checksum:

algorithm: sha-256

hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

container_format: bare

disk_format: qcow2

min_disk: 2 GB

min_ram: 8192 MiB

size: 2 GB

operating_system: Linux

supported_virtualisation_environments:

- KVM

dbBackendCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access the DB on IPv4

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: dbBackend

#- virtual_link: # the target node is determined in the NSD

dbBackendInternalCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: dbBackend

- virtual_link: internalVl

serviceNodeCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access the DB on IPv4

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: serviceNode

#- virtual_link: # the target node is determined in the NSD

serviceNodeInternalCp:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on VL

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

requirements:

- virtual_binding: serviceNode

- virtual_link: internalVl

internalVl:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

flow_pattern: mesh

description: Internal VL

vl_profile:

max_bitrate_requirements:

root: 100000

leaf: 20000

min_bitrate_requirements:

root: 10000

leaf: 10000

# A.3	VNF external connection point

## A.3.1	General

A VNF external connection point may either be an internal connection point (a VDU connection point) which is re-exposed externally, i.e. it may be connected to an external virtual link, or a virtual link port. In the latter case the external connection point node has an association relationship of type VirtualLinksTo to the internal virtual link node.

The following clauses illustrate the use of both models of VNF external connection point, re-exposure of an internal connection point and connected to an internal virtual link.

## A.3.2	External connection point re-exposing an internal connection point

In this case there is no need for a VnfExtCp node template. When substituting the VNFD low level service template for the abstract VNF node, the virtual_link requirement of the abstract VNF node is mapped to the VDU connection point's virtual_link requirement. This is shown in figure A.3.2-1. In this case the VNF Service Template does not include a node template of type tosca.nodes.nfv.VnfExtCp, but the functionality of it is provided by a VDU connection point.

Figure A.3.2-1: VNFD with an VDU connection point acting as VnfExtCp

The following snippet shows the relevant part of the service template:

tosca_definitions_version: tosca_simple_yaml_1_3

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

- exampleVNF.yaml # contains the VNF node type definition

node_types:

tosca.nodes.nfv.exampleVNF:

derived from: tosca.nodes.nfv.VNF

# properties:

# omitted for brevity

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

...

topology_template:

...

substitution_mappings:

node_type: tosca.nodes.nfv.exampleVNF

substitution_filter:

properties:

- flavour_id: { equal: simple }

requirements:

virtual_link: [vduCp_B2, virtual_link]

node_templates:

..

ExampleVNF:

type: tosca.nodes.nfv.exampleVNF

properties:

flavour_description: A simple flavour

interfaces:

Vnflcm:

operations:

instantiate:

implementation: instantiate.workbook.mistral.yaml

terminate:

implementation: terminate.workbook.mistral.yaml

vduCp_B2:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

# other properties omitted for brevity

requirements:

- virtual_binding: VDU-B

# - virtual_link: # mapped to virtual_link requirement of VNF node

# other node template definitions (VDU-A, VDU-B, intVL, etc.):

VDU-A:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: VDU-A

description: VDU-A description

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

vduCp_A:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: true

requirements:

- virtual_binding: VDU-A

- virtual_link: intVL

intVL:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

vl_profile:

max_bitrate_requirements:

root: 1000000

leaf: 100000

min_bitrate_requirements:

root: 100000

leaf: 10000

vduCp_B:

type: tosca.nodes.nfv.VduCp

properties:

layer_protocols: [ ipv4 ]

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: true

requirements:

- virtual_binding: VDU-B

- virtual_link: intVL

VDU-B:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: VDU-B

description: VDU-B description

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

The following snippet shows the relevant part of the file containing the imported node type definition:

tosca_definitions_version: tosca_simple_yaml_1_3

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

node_types:

tosca.nodes.nfv.exampleVNF:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: ExampleVNF ]

default: ExampleVNF

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ valid_values: [ simple, complex ] ]

default: simple

flavour_description:

type: string

default: "" #empty string

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

# other properties omitted for brevity

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

## A.3.3	External connection point connected to an internal virtual link

In this case a VnfExtCp node template is needed. When substituting the VNFD low level service template for the abstract VNF node type, the virtual_link requirement of the abstract VNF node maps to the external_virtual_link requirement of the VnfExtCp node. The internal_virtual_link requirement of the VnfExtCp node is fulfilled with the corresponding capability of the internal VirtualLink node. This is shown in figure A.3.3-1.

Figure A.3.3-1: VNFD with a VnfExtCp connected to an internal virtual link

The following snippet shows the corresponding node template definition.

The node type definition is assumed to be identical to the example in clause A.3.2.

tosca_definitions_version: tosca_simple_yaml_1_3

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

- exampleVNF.yaml # contains the VNF node type definition

topology_template:

...

substitution_mappings:

node_type: tosca.nodes.nfv.exampleVNF

substitution_filter:

properties:

- flavour_id: { equal: simple }

requirements:

virtual_link: [myMRFExtCp, external_virtual_link]

node_templates:

ExampleVNF:

type: tosca.nodes.nfv.exampleVNF

properties:

flavour_description: A simple flavour

interfaces:

Vnflcm:

operations:

instantiate:

implementation: instantiate.workbook.mistral.yaml

terminate:

implementation: terminate.workbook.mistral.yaml

myMRFExtCp:

type: tosca.nodes.nfv.VnfExtCp

properties:

layer_protocols: [ ipv4 ]

# other properties omitted for brevity

...

requirements:

# - external_virtual_link: # mapped to virtual_link requirement of VNF node

- internal_virtual_link: intVL-A

intVL-A:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

flow_pattern: mesh

description: Internal VL

vl_profile:

max_bitrate_requirements:

root: 1000000

leaf: 100000

min_bitrate_requirements:

root: 100000

leaf: 10000

qos:

latency: 100 ms

packet_delay_variation: 80 ms

packet_loss_ratio: 0.00001

# Other node templates e.g. VDU-A, VDU-B, VduCP_A, etc.:

...

# A.4	VNFD modelling design example by using TOSCA composition

The following example in figure A.4-1 shows a VNF descriptor contains three VDUs, which are interconnected by two virtualLinks.

Figure A.4-1: Example of using substitution mapping for the VNFD design

In this example, a separate service template is used to describe the composition of Vdu.Compute, Vdu.VirtualBlockStorage and VduCp, and then substituted as a node template in a VNFD service template. tosca.nodes.nfv.groupVDU_A, tosca.nodes.nfv.groupVDU_B, tosca.nodes.nfv.groupVDU_C types are used for substitution_mapping.

tosca_definitions_version: tosca_simple_yaml_1_3 imports: - etsi_nfv_sol001_vnfd_types.yaml # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001 topology_template: description: Template of a VNFD example node_templates: Group_VDU_A: type: tosca.nodes.nfv.groupVDU_A capabilities: virtual_compute: properties: virtual_memory: virtual_mem_size: 1 GB virtual_cpu: num_virtual_cpu: 1 requirements: - virtual_link: VL_2 Group_VDU_B: type: tosca.nodes.nfv.groupVDU_B capabilities: virtual_compute: properties: virtual_memory: virtual_mem_size: 1 GB virtual_cpu: num_virtual_cpu: 1 requirements: - virtual_link: VL_2 - virtual_link1: VL_1 Group_VDU_C: type: tosca.nodes.nfv.groupVDU_C # the description of this type is described # in another service template. capabilities: virtual_compute: properties: virtual_memory: virtual_mem_size: 1 GB virtual_cpu: num_virtual_cpu: 1 requirements: - virtual_link: VL_1 VL_1: type: tosca.nodes.nfv.VnfVirtualLink properties: connectivity_type: layer_protocols: [ ipv4 ] vl_profile: max_bitrate_requirements: root: 1000000 leaf: 100000 min_bitrate_requirements: root: 100000 leaf: 10000 # other properties omitted here for brevity VL_2: type: tosca.nodes.nfv.VnfVirtualLink properties: connectivity_type: layer_protocols: [ ipv4 ] vl_profile: max_bitrate_requirements: root: 1000000 leaf: 100000 min_bitrate_requirements: root: 100000 leaf: 10000 # other properties omitted here for brevity

The TOSCA service template example of Group_VDU C is showing in figure A.4-2.

Figure A.4-2: Example of composition Vdu.Compute, Vdu.VirtualBlockStorage and VduCp
together using TOSCA substitution mapping

tosca_definitions_version: tosca_simple_yaml_1_3 description: composition of Vdu.Compute, Vdu.VirtualBlockStorage and VduCp. topology_template: substitution_mappings: node_type: tosca.nodes.nfv.groupVDU_C # substituted as a node type requirements: virtual_link: [ internalCpd, virtual_link ] capabilities: virtual_compute: [vduC_compute, virtual_compute] virtual_storage: [vduC_storage, virtual_storage] node_templates: vduC_compute: type: tosca.nodes.nfv.Vdu.Compute properties: name: vduC_compute description: vduC_compute .. vdu_profile: min_number_of_instances: 1 max_number_of_instances: 1 # other properties omitted here for brevity capabilities: virtual_compute: properties: virtual_memory: virtual_mem_size: 1 GB virtual_cpu: num_virtual_cpu: 1 artifacts: sw_image: # omitted here for brevity requirements: - virtual_storage: vduC_storage vduC_storage: type: tosca.nodes.nfv.Vdu.VirtualBlockStorage properties: virtual_block_storage_data: size_of_storage: 2 GB # other properties omitted here for brevity internalCpd: type: tosca.nodes.nfv.VduCp properties: layer_protocols: [ ipv4 ] # protocol: [ associated_layer_protocol: ipv4 ] # trunk_mode: false # properties omitted here for brevity requirements: # - virtual_link: - virtual_binding: vduC_compute

The following template fragment provides the group definitions used in the above examples.

tosca_definitions_version: tosca_simple_yaml_1_3

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of VNFD types as defined in NFV SOL 001 GS

node_types:

tosca.nodes.nfv.groupVDU:

description: Abstract group VDU.

derived_from: tosca.nodes.Root

capabilities:

virtual_compute:

description: Describes virtual compute resources capabilities.

type: tosca.capabilities.nfv.VirtualCompute

virtual_storage:

description: Defines the capabilities of virtual_storage.

type: tosca.capabilities.nfv.VirtualStorage

requirements:

- virtual_link:

description: Describes the requirements for linking to virtual link.

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

node: tosca.nodes.nfv.VnfVirtualLink

occurrences: [ 1, 1 ]

tosca.nodes.nfv.groupVDU_A:

description: Abstract group VDU A.

derived_from: tosca.nodes.nfv.groupVDU

tosca.nodes.nfv.groupVDU_B:

description: Abstract group VDU B.

derived_from: tosca.nodes.nfv.groupVDU

requirements:

- virtual_link1:

description: Describes the requirements for linking to virtual link.

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

node: tosca.nodes.nfv.VnfVirtualLink

occurrences: [ 1, 1 ]

tosca.nodes.nfv.groupVDU_C:

description: Abstract group VDU C.

derived_from: tosca.nodes.nfv.groupVDU

# A.5	VNFD with Single deployment flavour modelling design example

In this example, there is one deployment flavour applied to this VNFD, and TOSCA-Simple-Profile-YAML-v1.3 [20] is used for designing and processing this VNFD TOSCA model.

The one service template design illustrated by this example is only applicable when the VNF has only one deployment flavour.

The service template is the main entry point in the VNF Package, i.e. the Entry-definitions file, and is deployed as a stand-alone service template, i.e. without substituting for a node template. However, the service template still contains substitution_mappings to indicate its ability to substitute for a node template of the specific node type.

The service template contains a node template of type MyCompany.SunshineDB.1_0.1_0. The node template contains the properties defined in the node type definition and implementations for the LCM interfaces.

sunshinesimple.vnfd.tosca.yaml

This example illustrates a VNF with one VDU.Compute nodes (dbBackend) with two VDU connection points and one VNF virtual link (see figure A.5-1). The VNF is also connected to an external virtual link. The flavour_id is "simple".

Figure A.5-1: SunshineDB (simple)

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, simple

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001 for a VNFD

data_types:

MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters:

derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters

properties:

segmentation_id_of_internalVl:

type: string

required: true

default: 1-4095

parameter_2:

type: string

required: true

default: value_2

MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfcConfigurableProperties

properties:

additional_vnfc_configurable_properties:

type: MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties

required: true

MyCompany.datatypes.nfv.AuxVnfcAdditionalConfigurableProperties:

derived_from: tosca.datatypes.nfv.VnfcAdditionalConfigurableProperties

properties:

name_prefix_in_vim:

type: string

required: true

default: "MyCustomer"

dns_server:

type: string

required: true

default: "90.200.250.57"

node_types:

MyCompany.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: SunshineDB ]

default: SunshineDB

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ equal: simple ]

default: simple

flavour_description:

type: string

default: ""

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

interfaces:

Vnflcm:

type: tosca.interfaces.nfv.Vnflcm

operations:

instantiate:

inputs:

additional_parameters:

type: MyCompany.datatypes.nfv.VnfInstantiateAdditionalParameters

#terminate:

MyCompany.nodes.nfv.Vdu.Aux:

derived_from: tosca.nodes.nfv.Vdu.Compute

properties:

configurable_properties:

type: MyCompany.datatypes.nfv.AuxVnfcConfigurableProperties

required: false

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

requirements:

virtual_link: [ dbBackendIpv4, virtual_link ] # IPv4 for SQL

inputs:

segmentation_id_of_internalVl:

type: string

required: true

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider: MyCompany

product_name: SunshineDB

software_version: '1.0'

descriptor_version: '1.0'

vnfm_info:

- '0:MyCompany-1.0.0'

flavour_id: simple

flavour_description: 'vnf simple flavour description'

interfaces:

Vnflcm:

operations:

instantiate:

implementation: instantiate.workbook.mistral.yaml

terminate:

implementation: terminate.workbook.mistral.yaml

heal:

implementation: heal.workbook.mistral.yaml

dbBackend:

type: MyCompany.nodes.nfv.Vdu.Aux

properties:

name: dbbackend

description: dbBackend

nfvi_constraints:

key_1: value_1

key_2: value_2

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 4

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 8192 MiB

virtual_cpu:

cpu_architecture: x86

num_virtual_cpu: 2

virtual_cpu_clock: 1800 MHz

requirements:

- virtual_storage: mariaDbStorage

mariaDbStorage:

type: tosca.nodes.nfv.Vdu.VirtualBlockStorage

properties:

virtual_block_storage_data:

size_of_storage: '200 GB'

rdma_enabled: true

artifacts:

sw_image:

type: tosca.artifacts.nfv.SwImage

file: maria.db.image.v1.0.qcow2

properties:

name: Software of Maria Db

version: '1.0'

checksum:

algorithm: sha-256

hash: b9c3036539fd7a5f87a1bf38eb05fdde8b556a1a7e664dbeda90ed3cd74b4f9d

container_format: bare

disk_format: qcow2

min_disk: 2 GB

min_ram: 8192 MiB

size: 2 GB

operating_system: Linux

supported_virtualisation_environments:

- KVM

dbBackendInternalCp:

type: tosca.nodes.nfv.VduCp

properties:

protocol: [associated_layer_protocol: ipv4 ]

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

requirements:

- virtual_binding: dbBackend

- virtual_link: internalVl

internalVl:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

flow_pattern: mesh

test_access: []

description: Internal Virtual link in the VNF

vl_profile:

max_bitrate_requirements:

root: 100000

leaf: 20000

min_bitrate_requirements:

root: 10000

leaf: 10000

virtual_link_protocol_data:

- associated_layer_protocol: ethernet

l2_protocol_data:

network_type: vlan

segmentation_id: { get_input: segmentation_id_of_internalVl }

dbBackendIpv4:

type: tosca.nodes.nfv.VduCp

properties:

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access the DB on IPv4

requirements:

#- virtual_link: # the target node is determined in the NSD

- virtual_binding: dbBackend

# A.6	Scaling and Instantiation Level examples

## A.6.1	ScalingAspect and InstantiationLevels policies with uniform delta

This example shows instantiationLevels, and ScalingAspect policies types in the complex scaling scenario where scaling aspect delta is based on "uniform delta" values , where it has only one entry.  For a uniform aspect, the example 1 shows that step_deltas is omitted, whereas the example 2 shows step_deltas is included as a list of entries.

Figure A.6.1-1: Complex scaling example with uniform delta

Example 1

tosca_definitions_version: tosca_simple_yaml_1_3

description: Complex scaling example (uniform delta value) described with policies

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

topology_template:

node_templates:

vdu_1:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 5

nfvi_constraints          key_1: value_1

key_2: value_2

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

vdu_2:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 7

nfvi_constraints:

key_1: value_1

key_2: value_2

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

vdu_3:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 5

nfvi_constraints:

key_1: value_1

key_2: value_2

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

vdu_4:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 9

nfvi_constraints:

key_1: value_1

key_2: value_2

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

vl_1:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

vl_profile:

min_bitrate_requirements:

root: 1000000

max_bitrate_requirements:

root: 2000000

vl_2:

type: tosca.nodes.nfv.VnfVirtualLink

properties:

connectivity_type:

layer_protocols: [ ipv4 ]

vl_profile:

min_bitrate_requirements:

root: 1000000

max_bitrate_requirements:

root: 4000000

policies:

- scaling_aspects:

type: tosca.policies.nfv.ScalingAspects

properties:

aspects:

database:

name: ..

description: ..

max_scale_level: 2

call_proc:

name: ..

description: ..

max_scale_level: 4

- vdu_1_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ vdu_1 ]

- vdu_1_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: database

deltas:

delta_1:

number_of_instances: 2

targets: [ vdu_1 ]

- vdu_2_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ vdu_2 ]

- vdu_2_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: database

deltas:

delta_1:

number_of_instances: 3

targets: [ vdu_2 ]

- vl_1_bitrate_initial_delta:

type: tosca.policies.nfv.VirtualLinkBitrateInitialDelta

properties:

initial_delta:

bitrate_requirements:

root: 1000000

targets: [ vl_1 ]

- vl_1_bitrate_scaling_aspect_deltas:

type: tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas

properties:

aspect: database

deltas:

delta_1:

bitrate_requirements:

root: 1000000

targets: [ vl_1 ]

- vdu_3_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ vdu_3 ]

- vdu_3_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: call_proc

deltas:

delta_1:

number_of_instances: 1

targets: [ vdu_3 ]

- vdu_4_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ vdu_4 ]

- vdu_4_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: call_proc

deltas:

delta_1:

number_of_instances: 2

targets: [ vdu_4 ]

- vl_2_bitrate_initial_delta:

type: tosca.policies.nfv.VirtualLinkBitrateInitialDelta

properties:

initial_delta:

bitrate_requirements:

root: 1000000

targets: [ vl_2 ]

- vl_2_bitrate_scaling_aspect_deltas:

type: tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas

properties:

aspect: call_proc

deltas:

delta_1:

bitrate_requirements:

root: 1000000

targets: [ vl_2 ]

- instantiation_levels:

type: tosca.policies.nfv.InstantiationLevels

properties:

levels:

instantiation_level_1:

description: ..

scale_info:

database:

scale_level: 0

call_proc:

scale_level: 0

instantiation_level_2:

description: ..

scale_info:

database:

scale_level: 2

call_proc:

scale_level: 3

default_level: instantiation_level_1

- vdu_1_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

instantiation_level_2:

number_of_instances: 5

targets: [ vdu_1 ]

- vdu_2_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

instantiation_level_2:

number_of_instances: 7

targets: [ vdu_2 ]

- vl_1_instantiation_levels:

type: tosca.policies.nfv.VirtualLinkInstantiationLevels

properties

levels:

instantiation_level_1:

bitrate_requirements:

root: 30000

instantiation_level_2:

bitrate_requirements:

root: 30000

targets: [ vl_1 ]

- vdu_3_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

instantiation_level_2:

number_of_instances: 4

targets: [ vdu_3 ]

- vdu_4_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

instantiation_level_2:

number_of_instances: 7

targets: [ vdu_4 ]

- vl_2_instantiation_levels:

type: tosca.policies.nfv.VirtualLinkInstantiationLevels

properties:

levels:

instantiation_level_1:

bitrate_requirements:

root: 30000

instantiation_level_2:

bitrate_requirements:

root: 60000

targets: [ vl_2 ]

Example 2:  This example is same as the example 1, except for ScalingAspects, which shows step_deltas list of entries.

topology_template:

. . .

policies:

- scaling_aspects:

type: tosca.policies.nfv.ScalingAspects

properties:

aspects:

database:

name: ..

description: ..

max_scale_level: 2

step_deltas:

- delta_1

- delta_1

call_proc:

name: ..

description: ..

max_scale_level: 4

step_deltas:

- delta_1

- delta_1

- delta_1

- delta_1

. . .

## A.6.2	ScalingAspect and InstantationLevels policies with non-uniform deltas

This example shows instantiationLevel, ScalingAspect policies and group types in the complex scaling scenario where scaling aspect delta is based on "delta" (non-uniform) values for Processing Auxiliary VNFC.

Figure A.6.2-1: Complex scaling example with non-uniform delta

For simplicity, virtual link is not illustrated and instantiation level is one level (default) in this example, as these properties are already illustrated in clause A.6.1.

The "Proc" aspect contains a uniform delta part ("processing" and a non-uniform part ("processing_auxiliary").

The "DB" aspect contains a non-uniform part ("db"), and no delta information for "oam".

tosca_definitions_version: tosca_simple_yaml_1_3

description: Complex example (uniform and non-uniform delta value) described with policies

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001

topology_template:

node_templates:

db:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

vdu_profile:

min_number_of_instances: 2

max_number_of_instances: 6

nfvi_constraints:

key_1: value_1

key_2: value_2

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

oam:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

nfvi_constraints:

key_1: value_1

key_2: value_2

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

processing:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

vdu_profile:

min_number_of_instances: 4

max_number_of_instances: 12

nfvi_constraints:

key_1: value_1

key_2: value_2

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

processing_auxiliary:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: ..

description: ..

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 3

nfvi_constraints:

key_1: value_1

key_2: value_2

capabilities:

virtual_compute:

properties:

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

policies:

- scaling_aspects:

type: tosca.policies.nfv.ScalingAspects

properties:

aspects:

database:

name: ..

description: ..

max_scale_level: 2

step_deltas:

- delta_1

- delta_1

proc:

name: ..

description: ..

max_scale_level: 4

step_deltas:

- delta_1

- delta_2

- delta_1

- delta_2

- db_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 2

targets: [ db ]

- db_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: database

deltas:

delta_1:

number_of_instances: 2

targets: [ db ]

- oam_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ oam ]

- processing_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 4

targets: [ processing ]

- processing_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: proc

deltas:

delta_1:

number_of_instances: 2

delta_2:

number_of_instances: 2

targets: [ processing ]

- processing_auxiliary_initial_delta:

type: tosca.policies.nfv.VduInitialDelta

properties:

initial_delta:

number_of_instances: 1

targets: [ processing_auxiliary ]

- processing_auxiliary_scaling_aspect_deltas:

type: tosca.policies.nfv.VduScalingAspectDeltas

properties:

aspect: proc

deltas:

delta_1:

number_of_instances: 1

delta_2:

number_of_instances: 0

targets: [ processing_auxiliary ]

- instantiation_levels:

type: tosca.policies.nfv.InstantiationLevels

properties:

levels:

instantiation_level_1:

description: ..

scale_info:

database:

scale_level: 0

proc:

scale_level: 0

- db_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 2

targets: [ db ]

- oam_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

targets: [ oam ]

- processing_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 4

targets: [ processing ]

- processing_auxiliary_instantiation_levels:

type: tosca.policies.nfv.VduInstantiationLevels

properties:

levels:

instantiation_level_1:

number_of_instances: 1

targets: [ processing_auxiliary ]

# A.7	Service Access Point

## A.7.1	General

A SAP may either be a VNF external connection point (or a PNF external connection points or a Sap of a nested NS of this NS) which is exposed as CP of one of the NS constituents (VnfExtCp, PnfExtCp, nested NS Sap externally, or be exposed as a new CP on NS virtual link. This is shown in figure A.7.1-1. In the latter case the Sap node has an association relationship of type VirtualLinksTo to the NsVirtualLink node.

Figure A.7.1-1: Overview of SAP options

The following clauses illustrate the use of both models of SAP, re-exposure of a VNF external connection point and connected to an NS virtual link.

## A.7.2	VNF External connection point exposing as a SAP

In this case, Sap node template is not required. When substituting the NSD service template for the abstract NS node, the virtual_link requirement of the abstract NS node is mapped to the VNF external connection point's virtual_link requirement. This is shown in figure A.7.2-1. In this case the NS Service Template does not include a node template of type tosca.nodes.nfv.Sap, but the functionality of it is provided by a VNF external connection point.

Figure A.7.2-1: VNF_2 External connection point (VnfExtCp) exposed as a Sap

The following snippet shows the relevant part of the NS service template:

tosca_definitions_version: tosca_simple_yaml_1_3

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in ETSI GS NFV-SOL 001

- example_VNF2.yaml

node_types:

tosca.nodes.nfv.exampleNS

derived_from: tosca.nodes.nfv.NS

# properties:

# properties omitted for brevity

requirements:

- virtual_link:

capability:

tosca.capabilitites.nfv.VirtualLinkable

...

topology_template:

...

substitution_mappings:

node_type: tosca.nodes.nfv.exampleNS

requirements:

virtual_link: [VNF_2, virtual_link_2] # the External connection point of VNF_2

# will be used as the Sap of this NS

node_templates:

VNF_2:

type: tosca.nodes.nfv.exampleVNF2

# properties omitted for brevity

requirements:

- virtual_link_1: NsVirtualLink_1 # connects to the External connection

# point which maps to the

# virtual_link_1 requirement of VNF_2

# - virtual_link_2: # mapped to virtual_link requirement of NS node

NsVirtualLink_1:

type: tosca.nodes.nfv.NsVirtualLink

properties:

vl_profile:

min_bitrate_requirements:

root: 100000

max_bitrate_requirements:

root: 200000

connectivity_type:

layer_protocols: [ ipv4 ]

# other node template definitions

#...

## A.7.3	SAP connected to an NS virtual link

In this case, a Sap node template is required. When substituting the NSD service template for the abstract NS node type, the virtual_link requirement of the abstract NS node maps to the virtual_link requirement of the Sap node. This is shown in figure A.7.3-1.

Figure A.7.3-1: SAP_1 exposed as new CP on an internal virtual link

The following snippet shows the corresponding node template definition:

tosca_definitions_version: tosca_simple_yaml_1_3

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in ETSI GS NFV-SOL 001

node_types:

tosca.nodes.nfv.exampleNS

derived_from: tosca.nodes.nfv.NS

# properties:

# properties omitted for brevity

requirements:

- virtual_link:

capability:

tosca.capabilitites.nfv.VirtualLinkable

topology_template:

...

substitution_mappings:

node_type: tosca.nodes.nfv.exampleNS

requirements:

virtual_link: [SAP_1, external_virtual_link] #SAP_1 is the Sap of the NS

node_templates:

SAP_1:

type: tosca.nodes.nfv.Sap

properties:

protocol: [ associated_layer_protocol: ipv4 ]

trunk_mode: false

layer_protocols: [ ipv4 ]

# other properties omitted for brevity

requirements:

- internal_virtual_link: NsVirtualLink_1

# - external_virtual_link: # map to virtual_link requirement of the NS node

NsVirtualLink_1:

type: tosca.nodes.nfv.NsVirtualLink

properties:

vl_profile:

min_bitrate_requirements:

root: 100000

max_bitrate_requirements:

root: 200000

connectivity_type:

layer_protocols: [ ipv4 ]

# Other node templates

#...

# A.8	NSD with Single deployment flavour modelling design example

In this example, there is one deployment flavour applied to this NSD, and TOSCA-Simple-Profile-YAML-v1.3 [20] is used for designing and processing this NSD TOSCA model.

The one service template design illustrated by this example is only applicable when the NS has only one deployment flavour.

The service template is the main entry point in the NSD file structure, i.e. the Entry-definitions file, and is deployed as a stand-alone service template, i.e. without substituting for a node template. However, the service template still contains substitution_mappings to indicate its ability to substitute for a node template of the specific node type.

example_NS.yaml

This example illustrates an NS which contains two VNF: VNF_1 and VNF_2, they connect through NsVirtualLink_1. One of the VnfExtCp of VNF_2 is exposed as the Sap of this NS. The flavour_id is "simple". In the VNF_2 node template, it defines a dependency requirement to VNF_1, which indicates that all the instances of VNF_1 have to be deployed first before deploy the instances of VNF_2. NsMonitoring and VnfMonitoring policies represent information to be monitored during the lifetime of a network service and VNF instances.

VNF_1

NsVirtualLink_1

Ext_NsVirtualLink

NS

VNF_2

Figure A.8-1: example_NS (simple)

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, simple

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of NSD related TOSCA types as defined in ETSI GS NFV-SOL 001

- example_VNF1.yaml  # uri of the yaml file which contains the definition of tosca.nodes.nfv.example_VNF1, this file might be included in the NSD file structure

- example_VNF2.yaml  # uri of the yaml file which contains the definition of tosca.nodes.nfv.example_VNF2, this file might be included in the NSD file structure

data_types:

MyCompany.datatypes.nfv.NsInstantiateAdditionalParameters:

derived_from: tosca.datatypes.nfv.NsOperationAdditionalParameters

properties:

parameter_1:

type: string

required: true

default: value_1

parameter_2:

type: string

required: true

default: value_2

node_types:

tosca.example_NS:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

name:

type: string

constraints: [ equal: ExampleService ]

default: ExampleService

version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

invariant_id:

type: string

constraints: [ equal: 1111-2222-aaaa-bbbb ]

default: 1111-2222-aaaa-bbbb

flavour_id:

type: string

constraints: [ equal: simple ]

default: simple

interfaces:

Nslcm:

type: tosca.interfaces.nfv.Nslcm

operations:

instantiate:

inputs:

additional_parameters:

type: MyCompany.datatypes.nfv.NsInstantiateAdditionalParameters

topology_template:

substitution_mappings:

node_type: tosca.example_NS

requirements:

virtual_link: [ VNF_2, virtual_link_2 ] # the External connection point of

# VNF_2 is exposed as the Sap

node_templates:

my_service:

type: tosca.example_NS

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer: MyCompany

name: ExampleService

version: '1.0'

invariant_id: 1111-2222-aaaa-bbbb

flavour_id: simple

interfaces:

Nslcm:

operations:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: NsVirtualLink_1

VNF_2:

type: tosca.nodes.nfv.example_VNF2

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: NsVirtualLink_1

# - virtual_link_2: # map to virtual_link requirement of the NS node

- dependency: VNF_1

NsVirtualLink_1:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

policies:

- my_service_NsMonitoring:

type: tosca.policies.nfv.NsMonitoring

properties:

ns_monitoring_parameters:

ns_monitoring_1111: #key map to the id of this monitoring parameter

name: MyService_byte_incoming_sap

performance_metric: byte_incoming_sap

collection_period: 1 s

ns_monitoring_2222: #key map to the id of this monitoring parameter

name: MyService_byte_outgoing_sap

performance_metric: byte_outgoing_sap

collection_period: 1 s

targets: [ my_service ]

- VNF_1_VnfMonitoring:

type: tosca.policies.nfv.VnfMonitoring

properties:

vnf_monitoring_parameters:

vnf_1_monitoring_1111: #key map to the id of this monitoring parameter

name: VNF_1_v_cpu_usage_mean_vnf

performance_metric: v_cpu_usage_mean_vnf

collection_period: 1 s

vnf_1_monitoring_2222: #key map to the id of this monitoring parameter

name: VNF_1_v_disk_usage_mean_vnf

performance_metric: v_disk_usage_mean_vnf

collection_period: 1 s

targets: [ VNF_1 ]

- VNF_2_VnfMonitoring:

type: tosca.policies.nfv.VnfMonitoring

properties:

vnf_monitoring_parameters:

vnf_2_monitoring_1111: #key map to the id of this monitoring parameter

name: VNF_2_v_cpu_usage_mean_vnf

performance_metric: v_cpu_usage_mean_vnf

collection_period: 1 s

vnf_2_monitoring_2222: #key map to the id of this monitoring parameter

name: VNF_2_v_disk_usage_mean_vnf

performance_metric: v_disk_usage_mean_vnf

collection_period: 1 s

vnf_2_monitoring_3333: #key map to the id of this monitoring parameter

name: VNF_2_v_memory_usage_mean_vnf

performance_metric: v_memory_usage_mean_vnf

collection_period: 1 s

targets: [ VNF_2 ]

The above example illustrates three policies (1 NS-level policy and 2 VNF-level policies). Each of the policies has different monitoring parameters. In the case, where the values of vnf_monitoring_parameters are the same across all constituent VNFs, a single VnfMonitoring policy can be used with a targets referencing all VNF node types.

# A.9	Mapping between NFV IM and TOSCA concepts

## A.9.1	Introduction

This clause describes the mapping between the NFV information model and the TOSCA concepts.

## A.9.2	Mapping between ETSI GS NFV-IFA 011 IM and TOSCA concepts

Table A.9.2-1 illustrates the mapping between the information model as specified in ETSI GS NFV-IFA 011 [1] and the corresponding TOSCA concepts.

Table A.9.2-1: Mapping between ETSI GS NFV-IFA 011 [1] IM and TOSCA concepts

| ETSI GS NFV-IFA 011 [1] information element | TOSCA concept |
| --- | --- |
| VNFD (clause 7.1.2) | TOSCA service template(s) in the VNF package |
| VnfExtCpd (clause 7.1.4) | node template with type tosca.nodes.nfv.VnfExtCp or tosca.nodes.nfv.VduCp |
| VnfLcmOperationsConfiguration (clause 7.1.5) | property of VNF node type with data type tosca.datatypes.nfv.VnfLcmOperationsConfiguration and/or inputs additional_parameters of the corresponding operation in the Vnflcm interface with type tosca.datatypes.nfv.VnfOperationAdditionalParameters |
| Vdu (clause 7.1.6) | node template with type tosca.nodes.nfv.Vdu.Compute |
| VLd (clause 7.1.7) | node template with type tosca.nodes.nfv.VnfVirtualLink |
| DeploymentFlavour (clause 7.1.8) | lower level service template(s) in the VNF package |
| VduProfile (clause 7.1.8.3) | property of Vdu.Compute node type with data type tosca.datatypes.nfv.VduProfile |
| VirtualLinkProfile (clause 7.1.8.4) | property of VnfVirtualLink node type with data type tosca.datatypes.nfv.VlProfile |
| VirtualLinkDescFlavour (clause 7.1.8.5) | Only qos attribute in VirtualLinkDescFlavour has been defined as property of the VlProfile data type |
| InstantiationLevel (clause 7.1.8.7) | policy with type tosca.policies.nfv.InstantiationLevels |
| VduLevel (clause 7.1.8.9) | policy with type tosca.policies.nfv.VduInstantiationLevels |
| LocalAffinityOrAntiAffinityRule (clause 7.1.8.11) | policy with type tosca.policies.nfv.AffinityRule or tosca.policies.nfv.AntiAffinityRule |
| AffinityOrAntiAffinityGroup (clause 7.1.8.12) | policy with type tosca.policies.nfv.AffinityRule or tosca.policies.nfv.AntiAffinityRule |
| VirtualComputeDesc (clause 7.1.9.2.2) | VirtualCompute capability of the Vdu.Compute node template |
| VirtualStorageDesc with BlockStorageData (clause 7.1.9.4.3) | node template with type tosca.nodes.nfv.Vdu.VirtualBlockStorage |
| VirtualStorageDesc with ObjectStorageData (clause 7.1.9.4.4) | node template with type tosca.nodes.nfv.Vdu.VirtualObjectStorage |
| VirtualStorageDesc with FileStorageData (clause 7.1.9.4.5) | node template with type tosca.nodes.nfv.Vdu.VirtualFileStorage |
| ScalingAspect (clause 7.1.10.2) | policy with type tosca.policies.nfv.ScalingAspects |
| ScalingDelta (clause 7.1.10.4) | policy with type tosca.policies.nfv.VduScalingAspectDeltas and/or tosca.policies.nfv.VirtualLinkBitrateScalingAspectDeltas |
| VnfIndicator (clause 7.1.11.2) | interface with type tosca.interfaces.nfv.VnfIndicator and related attributes of the VNF node type. |
| MonitoringParameter (clause 7.1.11.3) | property of the VNF, Vdu.Compute and VnfVirtualLink node types |
| VnfConfigurableProperties (clause 7.1.12) | property of VNF node template with data type tosca.datatypes.nfv.VnfConfigurableProperties |
| LifeCycleManagementScript (clause 7.1.13) | interface with type tosca.interfaces.nfv.Vnflcm or tosca.interfaces.nfv.ChangeCurrentVnfPackage |
| VnfInfoModifiableAttributes (clause 7.1.14) | property of VNF node template with data type tosca.datatypes.nfv.VnfInfoModifiableAttributes |
| VnfPackageChangeInfo (clause 7.1.15) | policy with type tosca.policies.nfv.VnfPackageChange, and tosca.interfaces.nfv.ChangeCurrentVnfPackage or tosca.interfaces.nfv.Vnflcm |

## A.9.3	Mapping between ETSI GS NFV-IFA 014 IM and TOSCA concepts

Table A.9.3-1 illustrates the mapping between the information model as specified in ETSI GS NFV-IFA 014 [2] and the corresponding TOSCA concepts.

Table A.9.3-1: Mapping between ETSI GS NFV-IFA 014 [2] IM and TOSCA concepts

| ETSI GS NFV-IFA 014 [2] information element | TOSCA concept |
| --- | --- |
| NSD (clause 6.2.2) | TOSCA service template(s) in the NSD file structure |
| Sapd (clause 6.2.3) | node template with type tosca.nodes.nfv.Sap or the virtual_link requirement in the substitution_mapping |
| MonitoredData (clause 6.2.6) | policy with type tosca.policies.nfv.VnfMonitoring and tosca.policies.nfv.NsMonitoring |
| VnfIndicatorData (clause 6.2.7) | Not supported in this version of the present document |
| LifeCycleManagementScript (clause 6.2.9) | Interface with type Nslcm |
| NsDf (clause 6.3) | lower level service template(s) in the NSD file structure |
| VnfProfile (clause 6.3.3) | property of VNF node template with data type tosca.datatypes.nfv.VnfProfile |
| VirtualLinkProfile (clause 6.3.4) | property of NsVirtualLink node template with data type tosca.datatypes.nfv.VlProfile |
| AffinityOrAntiAffinityGroup (clause 6.3.5) | policy with type tosca.policies.nfv.NsAffinityRule or tosca.policies.nfv.NsAntiAffinityRule |
| NsVirtualLinkConnectivity (clause 6.3.7) | requirement and capability between PnfExtCp or VnfExtCp and NsVirtualLink node templates |
| NsLevel (clause 6.3.9) | Not supported in this version of the present document |
| NsScaleInfo (clause 6.3.10) | Not supported in this version of the present document |
| NsProfile (clause 6.3.11) | property of NS node template with data type tosca.datatypes.nfv.NsProfile |
| Dependencies (clause 6.3.12) | dependency requirements |
| Vnffgd (clause 6.4.2) | group with type tosca.groups.nfv.VNFFG |
| Nfpd (clause 6.4.3) | node template with type tosca.nodes.nfv.NFP |
| NfpPositionElement (clause 6.4.6) | node template with type tosca.nodes.nfv.NfpPositionElement |
| NfpPositionDesc(clause 6.4.5) | node template with type tosca.nodes.nfv.NfpPosition |
| NsVirtualLinkDesc (clause 6.5.2) | node template with type tosca.nodes.nfv.NsVirtualLink |
| VirtualLinkDf (clause 6.5.4) | Only the serviceAvaibilityLevel and qos attributes in VirtualLinkDf have been defined as properties of the NsVlProfile data type. The flavour id is not mapped to a property as there is only one flavour of a particular NS virtual link per NS service template. |
| Pnfd (clause 6.6.2) | A standalone TOSCA service template |
| PnfExtCpd (clause 6.6.4) | node template with type tosca.nodes.nfv.PnfExtCp |
| NsScalingAspect (clause 6.7.2) | Not supported in this version of the present document |
| VnfToLevelMapping (clause 6.7.4) | Not supported in this version of the present document |
| VirtualLinkToLevelMapping (clause 6.7.5) | Not supported in this version of the present document |
| NsToLevelMapping (clause 6.7.6) | Not supported in this version of the present document |

# A.10	PNFD modelling design example

In this example, TOSCA-Simple-Profile-YAML-v1.3 [20] is used for designing and processing a PNFD TOSCA model.

The service template contains a node template of type MyCompany.examplePnf.1_0 which represents the main part of the PNF and a node template of type tosca.nodes.nfv.PnfExtCp representing the PNF external connection point.

examplePnf.yaml

This example illustrates a PNF with one external connection point, pnfExtCp_1. All the rest parts of the PNF is described as a single box, e.g. called pnf_mainPart in the PNFD.

Figure A.10-1: examplePnf

tosca_definitions_version: tosca_simple_yaml_1_3

description: the service template of a PNFD

imports:

- etsi_nfv_sol001_pnfd_types.yaml

node_types:

MyCompany.examplePnf.1_0:

derived_from: tosca.nodes.nfv.PNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a2233 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a2233

function_description:

type: string

default: an example PNF

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_invariant_id:

type: string

constraints: [ equal: 1111-2222-ccaa-bbdd ]

default: 1111-2222-ccaa-bbdd

name:

type: string

constraints: [ equal: ExamplePnf ]

default: ExamplePnf

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

topology_template:

substitution_mappings:

node_type: MyCompany.examplePnf.1_0

requirements:

virtual_link: [ pnfExtCp_1, external_virtual_link ]

node_templates:

pnf_mainPart:

type: MyCompany.examplePnf.1_0

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a2233

function_description: an example PNF

provider: MyCompany

version: 1.0

descriptor_invariant_id: 1111-2222-ccaa-bbdd

name: ExamplePnf

pnfExtCp_1:

type: tosca.nodes.nfv.PnfExtCp

properties:

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: External connection point to access this pnf

requirements:

# - external_virtual_link:

- dependency: pnf_mainPart

# A.11	NSD with Multiple deployment flavour modelling design example

Deployment flavours are represented as deployable TOSCA topology templates. This way one NS service template represents one NS deployment flavour, and different deployment flavours are described by different NS service templates. This is in line with the concept that different deployment flavours can define different topologies of the same NS, with different scaling aspects, different constituent VNFs, PNFs and nested NSs, and different internal connectivity.

In order to represent an NS, a top-level service template is used. This top-level service template contains a topology template with only an abstract NS node which defines the common parts of the different deployment flavours (such as designer, version and parts of the lifecycle management interface definition). It also sets a constraint on the deployment flavour property (the required value of the flavour_id property); this constraint comes from the NS instantiation request which contains a flavour_id selected among those available in the NSD.

As a result, the NFVO will look into the available further service templates representing the different NS deployment flavours of the NS and use the one that has the matching flavour_id property value to substitute for the abstract NS. These are the low-level service templates.

Figure A.11-1: NSD overview with multiple deployment flavour

An NSD contains a TOSCA top-level Service Template as entry point in the NSD file structure and one or more TOSCA low-level Service templates representing the different deployment flavours (see figure A.11-1). The NSD is interpreted by an NFVO. In this example, the templates describe two variants of the NS each corresponding to a deployment flavour: a small and a big one. The small NS consists of two VNFs one NS Virtual link and, the big VNF variant consists of three VNFs and one NS Virtual link.

NSD-top level MyExampleNs.yaml

tosca_definitions_version: tosca_simple_yaml_1_3

description: my service

imports:

- etsi_nfv_sol001_nsd_types.yaml

- MyExampleNs_Type.yaml # contains the NS node type definition

topology_template:

inputs:

flavour_id:

type: string

description: NS deployment flavour selected by the consumer. It is provided in the SOL005 API

node_templates:

myexampleNs:

type: tosca.MyExampleNS

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer: MyCompany

name: ExampleService

version: '1.0'

invariant_id: 1111-2222-aaaa-bbbb

flavour_id: {get_input: flavour_id }

# get_input function would be used by a TOSCA orchestrator at run time to access the flavour_id. If the deployment is not done by a TOSCA orchestrator above function may not be needed

# requirements:

#- virtual_link # mapped in lower-level templates

The MyExampleNs_Type.yaml file has the following content:

tosca_definitions_version: tosca_simple_yaml_1_3

description: type definition of tosca.MyExampleNS

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in ETSI GS NFV-SOL 001

data_types:

MyCompany.datatypes.nfv.NsInstantiateNsAdditionalParameters:

derived_from: tosca.datatypes.nfv.NsOperationAdditionalParameters

properties:

parameter_1:

type: string

required: true

default: value_1

parameter_2:

type: string

required: true

default: value_2

node_types:

tosca.MyExampleNS:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ valid_values: [ b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ] ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer:

type: string

constraints: [ valid_values: [ MyCompany] ]

default: MyCompany

name:

type: string

constraints: [ valid_values: [ ExampleService ] ]

default: ExampleService

version:

type: string

constraints: [ valid_values: [ '1.0' ] ]

default: '1.0'

invariant_id:

type: string

constraints: [ valid_values: [ 1111-2222-aaaa-bbbb ] ]

default: 1111-2222-aaaa-bbbb

flavour_id:

type: string

constraints: [ valid_values: [ small, big ] ]

default: small

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

interfaces:

Nslcm:

type: tosca.interfaces.nfv.Nslcm

operations:

instantiate:

inputs:

additional_parameters:

type: MyCompany.datatypes.nfv.NsInstantiateNsAdditionalParameters

required: false

The NS node template in the myexample_NS.yaml file is abstract and is subject to substitution; the lower-level templates in the subsequent sections provide these substitutions. The actual lower-level template is selected based on the node type and a value constraint on the flavour_id property.

Each low level service template contains a node template of type tosca.MyExampleNS with implementation of the LCM interfaces.

MyExampleNs (small): Lower level, contains 2 VNFs and 1 NS virtual link.

Figure A.11-2: MyExampleNs (simple): Lower level

MyExampleNs_small.yaml

tosca_definitions_version: tosca_simple_yaml_1_3

description: myExampleNs with small flavour

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in ETSI GS NFV-SOL 001

- MyExampleNs_Type.yaml # contains the NS node type definition

- example_VNF1.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF1 node type definition, this file might be included in the NSD file structure

- example_VNF2.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF2 node type definition, this file might be included in the NSD file structure

topology_template:

substitution_mappings:

node_type: tosca.MyExampleNS

substitution_filter:

properties:

- flavour_id: { equal: small }

requirements:

virtual_link: [ VNF_1, virtual_link_2 ]

node_templates:

MyExampleNS:

type: tosca.MyExampleNS

properties:

#

interfaces:

Nslcm:

operations:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: Ns_VL

# - virtual_link_2: # map to virtual_link requirement of the NS node

VNF_2:

type: tosca.nodes.nfv.example_VNF2

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: Ns_VL

Ns_VL:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

MyExampleNs (big): Lower level, contains 3 VNFs and 1 NS virtual link.

Figure A.11-3: MyExampleNs (big): Lower level

MyExampleNs_big.yaml

tosca_definitions_version: tosca_simple_yaml_1_3

description: myExampleNs with big flavour

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in ETSI GS NFV-SOL 001

- MyExampleNs_Type.yaml # contains the NS node type definition

- example_VNF1.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF1 node type definition, this file might be included in the NSD file structure

- example_VNF2.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF2 node type definition, this file might be included in the NSD file structure

- example_VNF3.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF3 node type definition, this file might be included in the NSD file structure

topology_template:

substitution_mappings:

node_type: tosca.MyExampleNS

substitution_filter:

properties:

- flavour_id: { equal: big }

requirements:

virtual_link: [ VNF_1, virtual_link_2 ]

node_templates:

MyExampleNS:

type: tosca.MyExampleNS

properties:

#

interfaces:

Nslcm:

operations:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

scale:

implementation: scale.workbook.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: Ns_VL

# - virtual_link_2: # map to virtual_link requirement of the NS node

VNF_2:

type: tosca.nodes.nfv.example_VNF2

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: Ns_VL

VNF_3:

type: tosca.nodes.nfv.example_VNF3

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: Ns_VL

Ns_VL:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

# A.12	NSD with nested NS design example

A TOSCA service template representing an NSD may contain a node template of some specific NS node type as one of its constituents. The latter is a nested NS. When the containing NS is deployed, the node template of the nested NS is substituted by the topology template representing the nested NS.

Figure A.12-1 illustrates a network service NS_1 that consists of one VNF (VNF_1), one NsVirtualLink (NS_VL_1_and one nested NS (NS_2).

The nested NS consists of two VNFs (VNF_3 and VNF_4) and one NsVirtualLink (NS_VL_2).

Figure A.12-1: Example of a network service containing a nested network service

tosca_definitions_version: tosca_simple_yaml_1_3

description: myExampleNs with small flavour

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in ETSI GS NFV-SOL 001

- MyExampleNs_Type.yaml # contains the NS node type definition

- MyExampleNS_2.yaml # uri of the yaml file which contains the tosca.myExample.NS_2 node type definition, this file might be included in the NSD file structure of NS_1

- example_VNF1.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF1 node type definition, this file might be included in the NSD file structure of NS_1

topology_template:

substitution_mappings:

node_type: tosca.MyExampleNS

substitution_filter:

properties:

- flavour_id: { equal: small }

requirements:

virtual_link: [ VNF_1, virtual_link_2 ]

node_templates:

NS_1:

type: tosca.MyExampleNS

interfaces:

Nslcm:

operations:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link_1: NS_VL_1

# - virtual_link_2: # map to virtual_link requirement of the NS node

NS_2:

type: tosca.myExample.NS_2

properties:

descriptor_id: c1bb0ab8-deab-4fa7-95ed-4840d70a3574

designer: MyCompany

version: 1.0.0.0

name: myExample2Service

invariant_id: aaaa-bbbb-cccc-dddd

ns_profile:

ns_instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

flavour_id: simple

NS_VL_1:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

The contents of MyExampleNs_Type.yaml file with the node type definition are as follows:

tosca_definitions_version: tosca_simple_yaml_1_3

description: type definition of tosca.MyExampleNS

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA types as defined in ETSI GS NFV-SOL 001

node_types:

tosca.MyExampleNS:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ valid_values: [ b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ] ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer:

type: string

constraints: [ valid_values: [ MyCompany] ]

default: MyCompany

name:

type: string

constraints: [ valid_values: [ ExampleService ] ]

default: ExampleService

version:

type: string

constraints: [ valid_values: [ '1.0' ] ]

default: '1.0'

invariant_id:

type: string

constraints: [ valid_values: [ 1111-2222-aaaa-bbbb ] ]

default: 1111-2222-aaaa-bbbb

flavour_id:

type: string

constraints: [ valid_values: [ small, big ] ]

default: small

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

interfaces:

Nslcm:

type: tosca.interfaces.nfv.Nslcm

The following snippet shows the service template representing the NSD NS_2. In this example, NS_2 supports one single deployment flavour.

MyExampleNS_2.yaml:

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, simple

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of NSD related TOSCA types as defined in ETSI GS NFV-SOL 001

- example_VNF3.yaml # uri of the yaml file which contains the definition of tosca.nodes.nfv.example_VNF3, this file might be included in the NSD file structure

- example_VNF4.yaml # uri of the yaml file which contains the definition of tosca.nodes.nfv.example_VNF4, this file might be included in the NSD file structure

node_types:

tosca.myExample.NS_2:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ valid_values: [ c1bb0ab8-deab-4fa7-95ed-4840d70a3574 ] ]

default: c1bb0ab8-deab-4fa7-95ed-4840d70a3574

designer:

type: string

constraints: [ valid_values: [ MyCompany] ]

default: MyCompany

name:

type: string

constraints: [ valid_values: [ myExample2Service ] ]

default: myExample2Service

version:

type: string

constraints: [ valid_values: [ '1.0.0.0' ] ]

default: '1.0.0.0'

invariant_id:

type: string

constraints: [ valid_values: [ aaaa-bbbb-cccc-dddd ] ]

default: aaaa-bbbb-cccc-dddd

flavour_id:

type: string

constraints: [ valid_values: [ simple ] ]

default: simple

topology_template:

substitution_mappings:

node_type: tosca.myExample.NS_2

requirements:

virtual_link: [ VNF_4, virtual_link_2 ] # the External connection point of

# VNF_2 is exposed as the Sap

node_templates:

NS_2:

type: tosca.myExample.NS_2

interfaces:

Nslcm:

operations:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_3:

type: tosca.nodes.nfv.example_VNF3

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: NS_VL_2

VNF_4:

type: tosca.nodes.nfv.example_VNF4

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: NS_VL_2

# - virtual_link_2: # map to virtual_link requirement of the NS node

NS_VL_2:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

The following template fragment is part of the content in the etsi_nfv_example_vnf4.yaml. Template fragments for etsi_nfv_example_vnf1.yaml, etsi_nfv_example_vnf2.yaml and etsi_nfv_example_vnf3.yaml are available in clause A.14.

tosca_definitions_version: tosca_simple_yaml_1_3

description: Example VNF4 type

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of VNFD types as defined in NFV SOL 001 GS

node_types:

tosca.nodes.nfv.example_VNF4:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1184 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1184

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: Example_VNF4 ]

default: Example_VNF4

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ equal: simple ]

default: simple

flavour_description:

type: string

default: ""

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

requirements:

- virtual_link_1:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

occurrences: [ 1, 1 ]

- virtual_link_2:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

occurrences: [ 1, 1 ]

# A.13	Virtual IP address connection point

Virtual IP address connection points (VipCps) are used to allocate one or multiple IP addresses that are shared by other CP instances, which may be instances of the same or of different VduCp or VnfExtCp nodes.

Load balancing

In the following example two or more instances of a particular VNFC are created. The respective instances of the VduCp, in addition to their default IP address which is assigned according to the 'protocol' property, share a virtual IP address. The multiple instances are created for load sharing purposes.

In this particular example the VduCp is re-exposed as VnfExtCp. Therefore the VipCp is also re-exposed as VnfExtCp.

Figure A.13-1: VNFD with a VDU connection point acting as VnfExtCp
and sharing a virtual IP address

In this example the VduCp and  the VipCp are exposed as VnfExtCps. Thus, the VNF abstract node has two requirements for a VirtualLinkable capability. One of them uses the VirtualLinksTo relationship and the other one uses the VipVirtualLinksTo relationship. Both requirements are considered in the substitution mapping.

The following service template shows the relevant parts of the TOSCA VNFD corresponding to figure A.13-1. For simplicity, a single deployment flavour VNF is assumed.

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, simple

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001 for a VNFD

node_types:

MyCompany.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: SunshineDB ]

default: SunshineDB

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ valid_values: [ simple ] ]

default: simple

flavour_description:

type: string

default: ""

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

interfaces:

Vnflcm

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

occurrences: [ 0, 1 ]
      - virtual_link_vip:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VipVirtualLinksTo

occurrences: [ 0, 1 ]

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

requirements:

virtual_link: [ Vdu-A-Cp, virtual_link ]

virtual_link_vip: [ VipCp, virtual_link ]

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

# properties:

# omitted for brevity

# interfaces:

# omitted for brevity

VDU-A:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: VDU-A

description: VDU-A description

vdu_profile:

min_number_of_instances: 2

max_number_of_instances: 5

# other properties omitted for brevity

capabilities:

virtual_compute:

properties: # following properties are required.

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

# requirements:

# omitted for brevity

Vdu-A-Cp:

type: tosca.nodes.nfv.VduCp

properties:

protocol: [associated_layer_protocol: ipv4 ]

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

requirements:

- virtual_binding: VDU-A

#- virtual_link:  # the target node is determined in the NSD

VipCp:

type: tosca.nodes.nfv.VipCp

properties:

vip_function: load_balance

protocol:

- associated_layer_protocol: ipv4

address_data:

- address_type: ip_address

l3_address_data:
                   ip_address_assignment: true

floating_ip_activated: false

number_of_ip_address: 1

trunk_mode: false

layer_protocols: [ ipv4 ]

description: >

Virtual IP connection point. It holds one IP address shared by all instances (between 2 and 5 according to the vdu_profile) of the Vdu-A-Cp node. Floating IP address is not used in the VipCp. Thus, incoming packets are forwarded with unmodified destination address to one of the instances of Vdu-A-Cp. A router external to the VNF with Equal-Cost Multi-Path (ECMP) load balancing functionality is assumed to be properly configured to route the packets accordingly to the available instances applying load balancing, i.e. one packet is only forwarded to one instance.

requirements:

- target: Vdu-A-Cp

# - virtual_link:  # the target node is determined in the NSD

High availability

In the following example, a VNF uses two VNFCs to provide high availability of a service. One of them is the active one receiving IP packets, the other one is in stand-by mode. The VNF logic determines which VNFC is the active and which is the stand-by. The respective VduCp instances, in addition to their default IP address which is assigned according to the 'protocol' property, share a virtual IP address. At any point in time, only one of the VduCp instances, the one belonging to the active VNFC, is bound to the virtual IP address, i.e. only one receives the packets. During the life of the VNF the binding may change, for example in case of failure of the active VNFC, or if determined by the VNF logic. In order to bind the virtual IP address, the active VNFC sends a gratuitous ARP (G-ARP) message with the mapping of the VIP address to its MAC address. A router external to the VNF updates its routing tables when receiving the G-ARP and thereafter routes packets that have the virtual IP address as destination address to the active VNFC.

In this particular example the VduCps are re-exposed as VnfExtCps. Therefore the VipCp is also re-exposed as VnfExtCp.

Figure A.13-2: VNFD with two VDU connection points acting as VnfExtCps
and sharing a virtual IP address

In this example the two VduCps and  the VipCp are exposed as VnfExtCps. Therefore the VNF abstract node has three requirements for a VirtualLinkable capability. Two of them use the VirtualLinksTo relationship and the third one uses the VipVirtualLinksTo relationship. The three of them are considered in the substitution mapping.

The following service template shows the relevant parts of the TOSCA VNFD corresponding to figure A.13-2. For simplicity,  a single deployment flavour VNF is assumed.

tosca_definitions_version: tosca_simple_yaml_1_3

description: Relational database, simple

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of TOSCA VNFD types as defined in ETSI GS NFV-SOL 001 for a VNFD

node_types:

MyCompany.SunshineDB.1_0.1_0:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: SunshineDB ]

default: SunshineDB

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ valid_values: [ simple ] ]

default: simple

flavour_description:

type: string

default: ""

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

# interfaces:

# omitted for brevity

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

occurrences: [ 0, 1 ]

- virtual_link_sby:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

occurrences: [ 0, 1 ]
      - virtual_link_vip:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VipVirtualLinksTo

occurrences: [ 0, 1 ]

topology_template:

substitution_mappings:

node_type: MyCompany.SunshineDB.1_0.1_0

requirements:

virtual_link: [ Vdu-A-Cp, virtual_link ]

virtual_link_sby: [ Vdu-B-Cp, virtual_link ]

virtual_link_vip: [ VipCp, virtual_link ]

node_templates:

SunshineDB:

type: MyCompany.SunshineDB.1_0.1_0

# properties:

# omitted for brevity

# interfaces:

# omitted for brevity

VDU-A:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: VDU-A

description: VDU-A description

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

# other properties omitted for brevity

capabilities:

virtual_compute:

properties: # following properties are required.

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

# requirements:

# omitted for brevity

VDU-B:

type: tosca.nodes.nfv.Vdu.Compute

properties:

name: VDU-B

description: VDU-B description

vdu_profile:

min_number_of_instances: 1

max_number_of_instances: 1

# other properties omitted for brevity

capabilities:

virtual_compute:

properties: # following properties are required.

virtual_memory:

virtual_mem_size: 1 GB

virtual_cpu:

num_virtual_cpu: 1

# requirements:

# omitted for brevity

Vdu-A-Cp:

type: tosca.nodes.nfv.VduCp

properties:

protocol: [associated_layer_protocol: ipv4 ]

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

requirements:

- virtual_binding: VDU-A

#- virtual_link:  # the target node is determined in the NSD

Vdu-B-Cp:

type: tosca.nodes.nfv.VduCp

properties:

protocol: [associated_layer_protocol: ipv4 ]

trunk_mode: false

layer_protocols: [ ipv4 ]

role: leaf

description: Internal connection point on an VL

requirements:

- virtual_binding: VDU-B

#- virtual_link:  # the target node is determined in the NSD

VipCp:

type: tosca.nodes.nfv.VipCp

properties:

vip_function: high_availability

protocol:

- associated_layer_protocol: ipv4

address_data:

- address_type: ip_address

l3_address_data:
                   ip_address_assignment: true

floating_ip_activated: true

number_of_ip_address: 1

trunk_mode: false

layer_protocols: [ ipv4 ]

description: >

Virtual IP connection point. It holds one IP address shared by the instances of the Vdu-A-Cp and the Vdu-B-Cp nodes (one instance of each). Floating IP address is used. Thus, incoming packets are first NATed to the virtual IP address and then forwarded with the virtual IP address as destination address to the instance of Vdu-A-Cp or Vdu-B-Cp that currently has the address binding.

requirements:

- target: Vdu-A-Cp

- target: Vdu-B-Cp

# - virtual_link:  # the target node is determined in the NSD

In the example above, the VipCp uses a floating IP address. Thus, the incoming packets are expected to have the floating IP as destination address and they are first NATed to the virtual IP address and then forwarded to the instance of the VduCp that currently has the binding to the virtual IP address.

If the VipCp does not use floating IP address, the incoming packets are expected to have the virtual IP address as destination address.

# A.14	NSD VNF Forwarding Graph design example

The following template fragment illustrates a VNF FG data model for a Network Service. The NS consists of VNF_1, VNF_2, VNF_3 and NsVirtualLink_1 as its constituents. VNF_1, VNF_2 and VNF_3 node templates have virtual link requirements pointing to node templates of the type tosca.nodes.nfv.Forwarding defined in clause 7.8.8 which in turn have virtual link requirements pointing to the NS virtual links or to external virtual links (i.e. transport links beyond the SAPs) to which these VNFs are attached.

Figure A.14-1: Example Network Forwarding Path

Figure A.14-2: Example VNFFG_1 group with constituent elements

tosca_definitions_version: tosca_simple_yaml_1_3 description: VNF FG Model for example_NS imports: - etsi_nfv_sol001_nsd_types.yaml # all of NSD related TOSCA types as defined in ETSI GS NFV-SOL 001 - etsi_nfv_example_vnf1.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF1 node type definition, this file might be included in the NSD file structure - etsi_nfv_example_vnf2.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF2 node type definition, this file might be included in the NSD file structure - etsi_nfv_example_vnf3.yaml # uri of the yaml file which contains the tosca.nodes.nfv.example_VNF3 node type definition, this file might be included in the NSD file structure data_types: MyCompany.datatypes.nfv.NsInstantiateAdditionalParameters: derived_from: tosca.datatypes.nfv.NsOperationAdditionalParameters node_types: tosca.example_NS: derived_from: tosca.nodes.nfv.NS properties: descriptor_id: type: string constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ] default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 designer: type: string constraints: [ equal: MyCompany ] default: MyCompany name: type: string constraints: [ equal: ExampleService ] default: ExampleService version: type: string constraints: [ equal: '1.0' ] default: '1.0' invariant_id: type: string constraints: [ equal: 1111-2222-aaaa-bbbb ] default: 1111-2222-aaaa-bbbb flavour_id: type: string constraints: [ equal: simple ] default: simple interfaces: Nslcm: type: tosca.interfaces.nfv.Nslcm operations: instantiate: inputs: additional_parameters: type: MyCompany.datatypes.nfv.NsInstantiateAdditionalParameters requirements: - virtual_link: capability: tosca.capabilitites.nfv.VirtualLinkable occurrences: [ 0, 0 ] - virtual_link_2: capability: tosca.capabilitites.nfv.VirtualLinkable occurrences: [ 0, 1 ] - virtual_link_3: capability: tosca.capabilitites.nfv.VirtualLinkable occurrences: [ 0, 1 ] - virtual_link_4: capability: tosca.capabilitites.nfv.VirtualLinkable occurrences: [ 0, 1 ] topology_template: substitution_mappings: node_type: tosca.example_NS requirements: virtual_link_2: [ VNF_1_forward_2, virtual_link ] # the requirement of SAP_1 virtual_link_3: [ VNF_3_forward_6, virtual_link ] # the requirement of SAP_2 virtual_link_4: [ VNF_2_forward_4, virtual_link ] # the requirement of SAP_3 node_templates: my_service: type: tosca.example_NS properties: descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 designer: MyCompany name: ExampleService # . . . interfaces: Nslcm: operations: instantiate: implementation: instantiate.workflow.yaml terminate: implementation: terminate.workflow.yaml NsVirtualLink_1: type: tosca.nodes.nfv.NsVirtualLink properties: # . . . VNF_1: type: tosca.nodes.nfv.example_VNF1 properties: descriptor_id: b1bb0ce7-2222-4fa7-95ed-4840d70a1179 descriptor_version: "1.0.0" # . . . requirements: - virtual_link: VNF_1_forward_1 - virtual_link_2: VNF_1_forward_2 VNF_1_forward_1: type: tosca.nodes.nfv.Forwarding requirements: - virtual_link: NsVirtualLink_1 VNF_1_forward_2: type: tosca.nodes.nfv.Forwarding requirements: - virtual_link: NsVirtualLink_1 VNF_2: type: tosca.nodes.nfv.example_VNF2 properties: descriptor_id: b1bb0ce7-2222-4fa7-95ed-4840d70a1182 descriptor_version: "1.0.0" # . . . requirements: - virtual_link: VNF_2_forward_3 - virtual_link_4: VNF_2_forward_4 VNF_2_forward_3: type: tosca.nodes.nfv.Forwarding # map to virtual_link requirement of the NS node VNF_2_forward_4: type: tosca.nodes.nfv.Forwarding requirements: - virtual_link: NsVirtualLink_1 VNF_3: type: tosca.nodes.nfv.example_VNF3 properties: descriptor_id: b1bb0ce7-2222-4fa7-95ed-4840d70a1177 descriptor_version: "1.0.0" # . . . requirements: - virtual_link: VNF_3_forward_5 - virtual_link_3: VNF_3_forward_6 VNF_3_forward_5: type: tosca.nodes.nfv.Forwarding requirements: - virtual_link: NsVirtualLink_1 VNF_3_forward_6: type: tosca.nodes.nfv.Forwarding # NfpPositionElement (Service Function) for VNF_1 Element_1: type: tosca.nodes.nfv.NfpPositionElement requirements: - profile_element: node: VNF_1_forward_1 capability: forwarding - profile_element: node: VNF_1_ forward_2 capability: forwarding # NfpPositionElement (Service Function) for VNF_2 Element_2: type: tosca.nodes.nfv.NfpPositionElement requirements: - profile_element: node: VNF_2_ forward_3 capability: forwarding - profile_element: node: VNF_2_ forward_4 capability: forwarding # NfpPositionElement (Service Function) for VNF_3 Element_3: type: tosca.nodes.nfv.NfpPositionElement requirements: - profile_element: node: VNF_3_forward_5 capability: forwarding - profile_element: node: VNF_3_forward_6 capability: forwarding # NfpPosition_1 with Element_1 and Element_2 as constituents NfpPosition_1: type: tosca.nodes.nfv.NfpPosition properties: forwarding_behaviour: lb requirements: - element: Element_1 - element: Element_2 # NfpPosition_2 with Element_3 as constituents NfpPosition_2: type: tosca.nodes.nfv.NfpPosition properties: forwarding_behaviour: all requirements: - element: Element_3 Nfp_1: type: tosca.nodes.nfv.NFP requirements: - nfp_position: NfpPosition_1 - nfp_position: NfpPosition_2 policies: - NfpRule_1: type: tosca.policies.nfv.NfpRule properties: ether_destination_address: 00:0a:95:9d:68:16 ether_source_address: 00:A0:C9:14:C8:29 ether_type: ipv4 vlan_tag: - 10 - 20 - 30 protocol: tcp dscp: 101111 source_port_range: [ 5000, 15000 ] destination_port_range: [ 800, 8080 ] source_ip_address_prefix: 10.10.10.0 destination_ip_address_prefix: 125.1.12.111 extended_criteria: - starting_point: 3 length: 4 value: 1000 targets: [ Nfp_1 ] groups: VNFFG_1: type: tosca.groups.nfv.VNFFG properties: description: VNF Forwarding Graph for example_NS members: [ Nfp_1, VNF_1, VNF_2, VNF_3, NsVirtualLink_1, Element_1, Element_2, Element_3 ]

The following template fragment is part of the content in the etsi_nfv_example_vnf1.yaml.

tosca_definitions_version: tosca_simple_yaml_1_3 description: VNF Descriptor for VNF1 imports: - etsi_nfv_sol001_vnfd_types.yaml # all of VNFD related TOSCA types as defined in ETSI GS NFV-SOL 001 node_types: tosca.nodes.nfv.example_VNF1: derived_from: tosca.nodes.nfv.VNF properties: descriptor_id: type: string constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1181 ] default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1181 provider: type: string constraints: [ equal: MyCompany ] default: MyCompany product_name: type: string constraints: [ equal: Example_VNF1 ] default: Example_VNF1 software_version: type: string constraints: [ equal: '1.0' ] default: '1.0' descriptor_version: type: string constraints: [ equal: '1.0' ] default: '1.0' flavour_id: type: string constraints: [ equal: simple ] default: simple flavour_description: type: string default: "" vnfm_info: type: list entry_schema: type: string constraints: [ equal: '0:MyCompany-1.0.0' ] default: [ '0:MyCompany-1.0.0' ] requirements: - virtual_link_1: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [ 1, 1 ] - virtual_link_2: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [ 0, 1 ] topology_template: substitution_mappings: node_type: tosca.nodes.nfv.example_VNF1 requirements: virtual_link: [ cp_02, external_virtual_link ] virtual_link_2: [ cp_01, external_virtual_link ] node_templates: cp_01: type: tosca.nodes.nfv.VnfExtCp properties: # . . . ...... cp_02: type: tosca.nodes.nfv.VnfExtCp properties: # . . . ......

The following template fragment is part of the content in the etsi_nfv_example_vnf2.yaml.

tosca_definitions_version: tosca_simple_yaml_1_3 description: VNF Descriptor for VNF2 imports: - etsi_nfv_sol001_vnfd_types.yaml # all of VNFD related TOSCA types as defined in ETSI GS NFV-SOL 001 node_types: tosca.nodes.nfv.example_VNF2: derived_from: tosca.nodes.nfv.VNF properties: descriptor_id: type: string constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1182 ] default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1182 provider: type: string constraints: [ equal: MyCompany ] default: MyCompany product_name: type: string constraints: [ equal: Example_VNF2 ] default: Example_VNF2 software_version: type: string constraints: [ equal: '1.0' ] default: '1.0' descriptor_version: type: string constraints: [ equal: '1.0' ] default: '1.0' flavour_id: type: string constraints: [ equal: simple ] default: simple flavour_description: type: string default: "" vnfm_info: type: list entry_schema: type: string constraints: [ equal: '0:MyCompany-1.0.0' ] default: [ '0:MyCompany-1.0.0' ] requirements: - virtual_link_1: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [ 0, 1 ] - virtual_link_2: capability: tosca.capabilities.nfv.VirtualLinkable relationship: tosca.relationships.nfv.VirtualLinksTo occurrences: [ 1, 1 ] topology_template: substitution_mappings: node_type: tosca.nodes.nfv.example_VNF2 requirements: virtual_link: [ cp_04, external_virtual_link ] virtual_link_4: [ cp_03, external_virtual_link ] node_templates: cp_03: type: tosca.nodes.nfv.VnfExtCp properties: # . . . ...... cp_04: type: tosca.nodes.nfv.VnfExtCp properties: # . . . ......

The following template fragment is part of the content in the etsi_nfv_example_vnf3.yaml.

tosca_definitions_version: tosca_simple_yaml_1_3

description: VNF Descriptor for VNF3

imports:

- etsi_nfv_sol001_vnfd_types.yaml  # all of VNFD related TOSCA types as defined in ETSI GS NFV-SOL 001

node_types:

tosca.nodes.nfv.example_VNF3:

derived_from: tosca.nodes.nfv.VNF

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1183 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1183

provider:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

product_name:

type: string

constraints: [ equal: Example_VNF3 ]

default: Example_VNF3

software_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

descriptor_version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

flavour_id:

type: string

constraints: [ equal: simple ]

default: simple

flavour_description:

type: string

default: ""

vnfm_info:

type: list

entry_schema:

type: string

constraints: [ equal: '0:MyCompany-1.0.0' ]

default: [ '0:MyCompany-1.0.0' ]

requirements:

- virtual_link_1:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

occurrences: [ 1, 1 ]

- virtual_link_2:

capability: tosca.capabilities.nfv.VirtualLinkable

relationship: tosca.relationships.nfv.VirtualLinksTo

occurrences: [ 0, 1 ]

topology_template:

substitution_mappings:

node_type: tosca.nodes.nfv.example_VNF3

requirements:

virtual_link:   [ cp_05, external_virtual_link ]

virtual_link_3: [ cp_06, external_virtual_link ]

node_templates:

cp_05:

type: tosca.nodes.nfv.VnfExtCp

properties:

# . . .

cp_06:

type: tosca.nodes.nfv.VnfExtCp

properties:

# . . .

# A.15	Auto-scale and auto-heal design with the use of VNF indicators

This clause illustrates an example of a VNF with two VNF indicators, one VNF indicator related to the utilization of the VNF and one related to the VNF health.

The modelling and handling of the indicator in the VNFD service template involves the following definitions:

A VNF specific VNF indicator interface type definition that includes one notification for each VNF indicator.

New attribute definition included in the VNF specific node type definition for each of the indicators.

Two attributes to hold the values of the current scale level of the 'call_proc' and 'database' scaling aspects.

NOTE:	It is assumed that this VNF has the same scaling aspects as defined in the example in clause A.6 ('call_proc' and 'database').

An interface definition of the VNF specific Vnflcm type with input definitions of additional_parameters for the heal operation.

An interface definition of the VNF specific VNF indicator type with an output definition for each notificaton indicating the attribute modified by the output of the notification.

AVNF specific auto_scale and  auto_heal policy definitions of tosca.nfv.policies.VnfIndicator type Each policy includes a trigger with an event associated to the notification, as well as condition and action.

tosca_definitions_version: tosca_simple_yaml_1_3 interface_types: MyCompany.nfv.interfaces.VnfIndicator derived_from: tosca.interfaces.nfv.VnfIndicator notifications: utilization: description: this notification is used to received asynchronous information of value change of the utilization_vnf_indicator outputs: utilization_vnf_indicator: type: float health: description: this notification is used to received asynchronous information of value change of the health_vnf_indicator outputs: health_vnf_indicator: type: string node_types: MyCompany.SunshineDB.1_0.1_0: derived_from: tosca.nodes.nfv.VNF properties: descriptor_id: type: string constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ] default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 provider: type: string constraints: [ equal: MyCompany ] default: MyCompany product_name: type: string constraints: [ equal: SunshineDB ] default: SunshineDB software_version: type: string constraints: [ equal: '1.0' ] default: '1.0' descriptor_version: type: string constraints: [ equal: '1.0' ] default: '1.0' flavour_id: type: string constraints: [ valid_values: [ simple ] ] default: simple flavour_description: type: string default: "" vnfm_info: type: list entry_schema: type: string constraints: [ equal: '0:MyCompany-1.0.0' ] default: [ '0:MyCompany-1.0.0' ] attributes: utilization_vnf_indicator: type: float description: holds the value of the utilization VNF indicator. It is assigned the output value of the utilization notification. health_vnf_indicator: type: string description: holds the value of the health VNF indicator. It is assigned the output value of the health notification. call_proc_scale_level: type: integer constraints: - in_range: [ 1, 4 ] database_scale_level: type: integer constraints: - in_range: [ 1, 2 ] interfaces: Vnflcm: operations: instantiate: terminate: scale: heal: inputs: additional_parameters: type: MyCompany.datatypes.nfv.HealAdditionalParameters required: false VnfIndicator: type: MyCompany.nfv.interfaces.VnfIndicator notifications: utilization: outputs: utilization_vnf_indicator: [ SELF, utilization_vnf_indicator ] health: outputs: health_vnf_indicator: [ self, health_vnf_indicator ] data_types: MyCompany.datatypes.nfv.HealAdditionalParameters: derived_from: tosca.datatypes.nfv.VnfOperationAdditionalParameters properties: parameter_1: type: string required: false policy_types: MyCompany.policies.nfv.VnfIndicator: derived_from: tosca.policies.nfv.VnfIndicator topology_template: substitution_mappings: node_type: MyCompany.SunshineDB.1_0.1_0 .. node_templates: SunshineDB: type: MyCompany.SunshineDB.1_0.1_0 attributes: call_proc_scale_level: { get_attribute: [ SELF, scale_status, call_proc, scale_level ] } database_scale_level: { get_attribute: [ SELF, scale_status, database, scale_level ] } policies: - scaling_aspects: type: tosca.policies.nfv.ScalingAspects properties: aspects: database: name: .. description: .. max_scale_level: 2 step_deltas: - delta_1 call_proc: name: .. description: .. max_scale_level: 4 step_deltas: - delta_1 - auto_scale: type: tosca.policies.nfv.VnfIndicator properties: source: vnf triggers: scale_out: event: tosca.interfaces.nfv.VnfIndicator.utilization # full name of a notification in the VnfIndicator interface condition: - utilization_vnf_indicator: [ { greater_or_equal: 60.0 } ] - call_proc_scale_level: [ { less_than: 3 } ] action: - call_operation: operation: tosca.interfaces.nfv.Vnflcm.scale inputs: - type: value: scale_out - aspect: value: call_proc - number_of_steps: value: 1 # optional scale_in: event: tosca.interfaces.nfv.VnfIndicator.utilization # full name of a notification in the VnfIndicator interface condition: - utilization_vnf_indicator: [ { less_or_equal: 20.0 } ] - call_proc_scale_level: [ { greater_than: 0 } ] action: - call_operation: operation: tosca.interfaces.nfv.Vnflcm.scale inputs: - type: value: scale_in - aspect: value: call_proc targets: [SunshineDB ] - auto_heal: type: tosca.policies.nfv.VnfIndicator properties: source: vnf triggers: red: event: tosca.interfaces.nfv.VnfIndicator.health # full name of a notification in the VnfIndicator interface condition: - health_vnf_indicator: red action: - call_operation: operation: tosca.interfaces.nfv.Vnflcm.heal inputs: - cause: value: no_service yellow: event: tosca.interfaces.nfv.VnfIndicator.health condition: - health_vnf_indicator: yellow action: - call_operation: operation: tosca.interfaces.nfv.Vnflcm.heal inputs: - cause: value: degraded_service - auto_heal_with_additional_parameters: type: MyCompany.policies.nfv.VnfIndicator properties: source: vnf triggers: red_or_yellow: event: tosca.interfaces.nfv.VnfIndicator.health # full name of a notification in the VnfIndicator interface condition: or: - health_vnf_indicator: [ { equal: red } ] - health_vnf_indicator: [ { equal: yellow } ] action: - call_operation: operation: tosca.interfaces.nfv.Vnflcm.heal inputs: - cause: value: problem - additional_parameters: value: parameter_1: value_1 ..

# A.16	VDU connection point in trunk mode

With supporting trunk mode, a VM can differentiate between traffic of many networks by different encapsulation types and Ids, instead of using many vNICs. Connection points of a VDU are combined into one VIF by turning one connection point into a trunk parent port and the other connection points into trunk subports of the same trunk. A trunk logical model is a collection of connection points. Different infrastructure technologies may have different way to implement trunk logical model.

The main concept of difference between subport and parent port:

•	The parent port can associate to the VM and a network.

•	The parent port can be associated by subports.

•	The subport can not associate directly to the VM but through the parent port and a network.

•	The traffic of sub-port can be encapsulated as required.

In the following example, there are three connection points are needed in initial deployment stage, one VdpCp node taken as trunk port and two VduSubCp nodes taken as trunk subports. With respect to VduSubCp node, requirement virtual_binding, if existing, is useless because the trunk_binding have priority over it.

Figure A.16-1: VNFD with a VDU has three connection points group as trunk

The following service template shows the relevant parts of the TOSCA VNFD corresponding to figure A.y-1

tosca_definitions_version: tosca_simple_yaml_1_3

...

node_types:

tosca.nodes.nfv.exampleVNF:

derived from: tosca.nodes.nfv.VNF

# omitted for brevity

node_templates:

...

VduwithTrunk:

type:  tosca.nodes.nfv.Vdu.Compute

properties:

name: VduwithTrunk

description: compute node with trunk port

# omitted for brevity

TrunkParentPort:

type: tosca.nodes.nfv.VduCp

properties:

trunk_mode: true

# omitted for brevity

requirements:

- virtual_binding: VduwithTrunk

- virtual_link: intVL1

TrunkSubPort1:

type: tosca.nodes.nfv.VduSubCp

properties:

# omitted for brevity

segmentation_type: vlan

segmentation_id: 10

requirements:

- trunk_binding: TrunkParentPort

- virtual_link: intVL2

TrunkSubPort2:

type: tosca.nodes.nfv.VduSubCp

properties:

# omitted for brevity

segmentation_type: vlan

segmentation_id: 100

requirements:

- trunk_binding: TrunkParentPort

- virtual_link: intVL3

intVL1:

# omitted for brevity

intVL2:

# omitted for brevity

intVL3:

# omitted for brevity

...

# A.17	NS scaling

This annex shows the use of the NS policies to define the scaling aspects and NS scale levels.

The NSD  NS_1 consists of the following components:

- Three VNFDs specified with the VNF node templates:  FrontEnd,  CentralProcess and Database

- One nested NSD specified with the NS node template: SupportFrontEnd

- One NsVirtualLinkDesc specified with the NS VL node template: NS_VL_1.

Figure A.17-1 shows the two scaling aspects and the two instantiation levels defined in this NS.

Figure A17-1: NS scaling aspects and instantiation levels

tosca_definitions_version: tosca_simple_yaml_1_3

description: myExampleNs with scaling aspects

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA NSD types as defined in ETSI GS NFV-SOL 001

- MyScalableNs.yaml # contains the NS node type definition.

- SupportFrontEndNs.yaml # uri of the yaml file which contains the MyProviderCompany.NS.SupportFrontEndNs node type definition, this file might be included in the NSD file structure of NS_1.

- FrontEnd_1_0.yaml # uri of the yaml file which contains the MyCompany.VNF.FrontEnd_1_0 node type definition, this file might be included in the NSD file structure of NS_1.

- Database_1_0.yaml # uri of the yaml file which contains the MyCompany.VNF.Database_1_0 node type definition, this file might be included in the NSD file structure of NS_1.

- CentralProcess_1_0.yaml # uri of the yaml file which contains the MyCompany.VNF.CentralProcess_1_0 node type definition, this file might be included in the NSD file structure of NS_1.

topology_template:

substitution_mappings:

node_type: MyProviderCompany.NS.MyScalableNs

substitution_filter:

properties:

- flavour_id: { equal: small }

requirements:

virtual_link: [ FrontEnd, virtual_link_2 ]

node_templates:

NS_1:

type: MyProviderCompany.NS.MyScalableNS

interfaces:

Nslcm:

operations:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

FrontEnd:

type: MyCompany.VNF.FrontEnd_1_0

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 4

requirements:

- virtual_link_1: NS_VL_1

# - virtual_link_2: # map to virtual_link requirement of the NS node

Database:

type: MyCompany.VNF.Database_1_0

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 2

requirements:

- virtual_link_1: NS_VL_1

CentralProcess:

type: MyCompany.VNF.CentralProcess_1_0

properties:

# no property assignments needed for required properties that have a default value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 2

requirements:

- virtual_link_1: NS_VL_1

SupportFrontEnd:

type: MyProviderCompany.NS.SupportFrontEndNs

properties:

descriptor_id: c1bb0ab8-deab-4fa7-95ed-4840d70a3574

designer: MyCompany

version: 1.0.0.0

name: myExample2Service

invariant_id: aaaa-bbbb-cccc-dddd

ns_profile:

ns_instantiation_level: level_1

min_number_of_instances: 0

max_number_of_instances: 1

flavour_id: simple

requirements:

- virtual_link_1: NS_VL_1

NS_VL_1:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

policies:

- scaling_aspects:

type: tosca.policies.nfv.nsScalingAspects

properties:

aspects:

database:

name: DatabaseScalingAspect

description: This scaling aspect is used to scale the Database VNF

ns_scale_levels:

0:

description: first NS level for the database scaling aspect

1:

description: second NS level for the database scaling aspect

processor:

name: ProcessorScalingAspect

description: This scaling aspect is used to scale the FrontEnd and CentralProcess VNFs and the SupportFrontEnd nested NS

ns_scale_levels:

0:

description: first NS level for the processor scaling aspect

1:

description: second NS level for the processor scaling aspect

2:

description: second NS level for the processor scaling aspect

- FrontEndVnfToLevelMapping:

type: tosca.policies.nfv.VnfToLevelMapping

properties:

aspects: processor

number_of_instances:

0: 1

1: 3

2: 4

targets: [ FrontEnd ]

- CentralProcessVnfToLevelMapping:

type: tosca.policies.nfv.VnfToLevelMapping

properties:

aspects: processor

number_of_instances:

0: 1

1: 1

2: 2

targets: [ CentralProcess ]

- DatabaseVnfToLevelMapping:

type: tosca.policies.nfv.VnfToLevelMapping

properties:

aspects: database

number_of_instances:

0: 1

1: 2

targets: [ Database ]

- SupportFrontEndNsToLevelMapping

type: tosca.policies.nfv.NsToLevelMapping

properties:

aspects: processor

number_of_instances:

0: 0

1: 0

2: 1

targets: [ SupportFrontEnd ]

- ns_instantiation_levels

type: tosca.policies.NsInstantiationLevels

properties:

ns_levels:

instantiation_level_1:

description: low capacity instantiation level

instantiation_level_2:

description: high capacity instantiation level

default_level: instantiation_level_1

- FrontEnd_instantiation_levels

type: tosca.policies.VnfToInstantiationLevelMapping

properties:

number_of_instances:

instantiation_level_1: 1

instantiation_level_2: 3

targets: [ FrontEnd ]

- CentralProcess_instantiation_levels

type: tosca.policies.VnfToInstantiationLevelMapping

properties:

number_of_instances:

instantiation_level_1: 1

instantiation_level_2: 1

targets: [ CentralProcess ]

- Database_instantiation_levels

type: tosca.policies.VnfToInstantiationLevelMapping

properties:

number_of_instances:

instantiation_level_1: 1

instantiation_level_2: 1

targets: [ Database ]

- SupportFrontEnd_instantiation_levels

type: tosca.policies.NsToInstantiationLevelMapping

properties:

number_of_instances:

instantiation_level_1: 0

instantiation_level_2: 0

targets: [ SupportFrontEnd ]

The contents of MyScalableNs_Type.yaml file with the node type definition are as follows:

tosca_definitions_version: tosca_simple_yaml_1_3

description: type definition of tosca.MyScalableNS

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of TOSCA types as defined in ETSI GS NFV-SOL 001

node_types:

tosca.MyScalableNS:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ valid_values: [ b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ] ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer:

type: string

constraints: [ valid_values: [ MyCompany] ]

default: MyCompany

name:

type: string

constraints: [ valid_values: [ ExampleService ] ]

default: ExampleService

version:

type: string

constraints: [ valid_values: [ '1.0' ] ]

default: '1.0'

invariant_id:

type: string

constraints: [ valid_values: [ 1111-2222-aaaa-bbbb ] ]

default: 1111-2222-aaaa-bbbb

flavour_id:

type: string

constraints: [ valid_values: [ small, big ] ]

default: small

requirements:

- virtual_link:

capability: tosca.capabilities.nfv.VirtualLinkable

interfaces:

Nslcm:

type: tosca.interfaces.nfv.Nslcm

Annex B (normative):
etsi_nfv_sol001_type definitions

# B.1	Purpose

All type definitions specified in clauses 6, 7, 8 and 9 of the present document are gathered in four definition files.

The file names are structured as follows:

etsi_nfv_sol001_common_types.yaml, for the common type definitions provided in clause 9 which are used by at least two types of deployment templates among those identified in clause 5.1;

etsi_nfv_sol001_vnfd_types.yaml, for the definitions provided in clause 6 and only used in a VNFD service template design;

etsi_nfv_sol001_nsd_types.yaml, for the definitions provided in clause 7 and only used in an NSD service template design;

etsi_nfv_sol001_pnfd _types.yaml, for the definitions provided in clause 8 and only used in a PNFD service template design.

# B.2	VNFD type definitions file

All type definitions specified in clause 6 of the present document are contained in the file etsi_nfv_sol001_vnfd_types.yaml of archive gs_nfv-sol001v030301p0.zip which accompanies the present document.

This file is also available at the following URL:

https://forge.etsi.org/rep/nfv/SOL001/raw/v3.3.1/etsi_nfv_sol001_vnfd_types.yaml.

NOTE 1:	The file etsi_nfv_sol001_vnfd_types.yaml includes a TOSCA import definition referencing etsi_nfv_sol001_common_types.yaml file. If the later file is included in the VNF package, the import definition can reference the local file using appropriate path in the VNF package.

This file is a TOSCA service template that only contains definitions. The template_version in the metadata section within this template is structured as x.y.z, where x, y and z represent the version of this file and are set respectively to "3", "3" and "1" for this version of the present document. In subsequent versions of the present document, "x", "y" and "z" in the template_version will be incremented only if there are changes in the VNFD type definitions.

A TOSCA service template representing a VNFD complying with the present document shall contain import statement referencing this file, as defined in clause 5.6.1.

NOTE 2:	This file may, but need not, be included in the VNF Package.

# B.3	NSD type definitions file

All type definitions specified in clause 7 of the present document are contained in the file etsi_nfv_sol001_nsd_types.yaml of archive gs_nfv-sol001v030301p0.zip which accompanies the present document.

This file is also available at the following URL:

https://forge.etsi.org/rep/nfv/SOL001/raw/v3.3.1/etsi_nfv_sol001_nsd_types.yaml.

NOTE 1:	The file etsi_nfv_sol001_nsd_types.yaml includes a TOSCA import definition referencing etsi_nfv_sol001_common_types.yaml file. If the later file is included in the NSD file archive, the import definition can reference the local file using appropriate path in the NSD file archive.

This file is a TOSCA service template that only contains definitions. The template_version in the metadata section within this template is structured as x.y.z, where x, y and z represent the version of this file and are set respectively to "3", "3" and "1" for this version of the present document. In subsequent versions of the present document, "x", "y" and "z" in the template_version will be incremented only if there are changes in the NSD type definitions.

A TOSCA service template representing an NSD complying with the present document shall contain import statement referencing this file, as defined in clause 5.6.2.

NOTE 2:	This file may, but need not, be included in the NSD file archive.

# B.4	PNFD type definitions file

All type definitions specified in clause 8 of the present document are contained in the file etsi_nfv_sol001_pnfd_types.yaml of archive gs_nfv-sol001v030301p0.zip which accompanies the present document.

This file is also available at the following URL:

https://forge.etsi.org/rep/nfv/SOL001/raw/v3.3.1/etsi_nfv_sol001_pnfd_types.yaml.

NOTE:	The file etsi_nfv_sol001_pnfd_types.yaml includes a TOSCA import definition referencing etsi_nfv_sol001_common_types.yaml file.

This file is a TOSCA service template that only contains definitions. The template_version in the metadata section within this template is structured as x.y.z, where x, y and z represent the version of this file and are set respectively to "3", "3" and "1" for this version of the present document. In subsequent versions of the present document, "x", "y" and "z" in the template_version will be incremented only if there are changes in the PNFD type definitions.

A TOSCA service template representing a PNFD complying with the present document shall contain import statement referencing this file, as defined in clause 5.6.3.

# B.5	Common type definitions file

The type definitions as specified in clause 9 and used by at least two types of deployment templates are contained in the file etsi_nfv_sol001_common_types.yaml of archive gs_nfv-sol001v030301p0.zip which accompanies the present document.

This file is also available at the following URL:

https://forge.etsi.org/rep/nfv/SOL001/raw/v3.3.1/etsi_nfv_sol001_common_types.yaml.

This file is a TOSCA service template that only contains definitions. The template_version in the metadata section within this template is structured as x.y.z, where x, y and z represent the version of this file and are set respectively to "3", "3" and "1" for this version of the present document. In subsequent versions of the present document, "x", "y" and "z" in the template_version will be incremented only if there are changes in the common type definitions.

NOTE:	This file may, but need not, be included in the VNF package or NSD file archive.

Annex C (normative):
Conformance

# C.1	Purpose

The present document specifies a data model for the VNFD, the NSD and the PNFD, by using the grammar defined in the TOSCA-Simple-Profile-YAML-v1.3 [20]. This annex specifies the requirements to be fulfilled for claiming conformance to the present document.

# C.2	NFV TOSCA YAML service template

A VNFD, an NSD or a PNFD conforms to the present document if it complies with all the requirements below:

- A VNFD conformant to the present document shall comply with the requirements in clause 6 of the present document and to the specification of the elements of the T TOSCA-Simple-Profile-YAML-v1.3 [20] it uses, unless otherwise stated in clause 6 of the present document.

An NSD conformant to the present document shall comply with the requirements in clause 7 of the present document and to the specification of the elements of the TOSCA-Simple-Profile-YAML-v1.3 [20] it uses, unless otherwise stated in clause 7 of the present document.

A PNFD conformant to the present document shall comply with the requirements in clause 8 of the present document and to the specification of the elements of the TOSCA-Simple-Profile-YAML-v1.3 [20] it uses, unless otherwise stated in clause 8 of the present document.

When using or referring to the TOSCA normative types listed in table C.2-1, it is valid according to the definitions given in clauses 6, 7, 8 and 9 of the present document and to section 5 of the TOSCA-Simple-Profile-YAML-v1.3 [20].

Table C.2-1: TOSCA normative types used in the present document

| Types | VNFD | NSD | PNFD |
| --- | --- | --- | --- |
| tosca.datatypes.Root | X | X | X |
| tosca.artifacts.Deployment.Image | X |  |  |
| tosca.artifacts.Implementation | X |  |  |
| tosca.capabilities.Root | X | X |  |
| tosca.capabilities.Node | X | X | X |
| tosca.relationships.Root | X | X |  |
| tosca.relationships.DependsOn | X | X | X |
| tosca.interfaces.Root | X | X |  |
| tosca.nodes.Root | X | X | X |
| tosca.groups.Root | X | X |  |
| tosca.policies.Root | X | X | X |
| tosca.policies.Placement | X | X |  |

A VNFD conformant to the present document shall comply with VNFD TOSCA service template design specified in clause 6.11 of the present document.

A NSD conformant to the present document shall comply with NSD TOSCA service template design specified in clause 7.11 of the present document.

A PNFD conformant to the present document shall comply with PNFD TOSCA service template design specified in clause 8.11 of the present document.

A VNFD and NSD conformant to the present document shall comply with rules for Type extension defined in clause 5.7 of the present document.

# C.3	NFV TOSCA processor

A processor or program conforms to the present document as NFV TOSCA processor for VNFD, NFV TOSCA processor for NSD or NFV TOSCA processor for PNFD if it complies with all the requirements below:

1)	It can parse and recognize the elements of any VNFD, NSD or PNFD that conform to the present document, and shall generate errors for those documents that fail to conform to the present document.

2)	It shall comply with all requirements and implement the semantics associated with the definitions specified in clauses 6, 7, 8 and 9 of the present document.

3)	It shall resolve the import definitions, as described in clause 5.6 of the present document.

Annex D (informative):
Mapping between properties of TOSCA types and API attributes

# D.1	Introduction

This annex provides the mapping between properties of TOSCA types defined in the present document and defined in the following API specifications: ETSI GS NFV-SOL 002 [i.4], ETSI GS NFV-SOL 003 [i.9], and ETSI GS NFV-SOL 005 [i.10].

NOTE:	See also annex A "Mapping operations to protocol elements" of ETSI GS NFV-SOL 002 [i.4], ETSI GS NFV-SOL 003 [i.9] and ETSI GS NFV-SOL 005 [i.10] for each operation.

# D.2	VNFD-related constructs

Table D.2-1 provides the mapping between the properties of TOSCA types related to the VNFD and API attributes, which include: resource or notification data type (and referenced structured data type when available), attribute name and type in the resource or notification data type, and the interface operation in which the data type is used.

NOTE:	In the "Data model" column of the table, an arrow "->" indicates the navigation through the resource, notification and referenced structured data types.

Table D.2-1: Mapping of API attributes and TOSCA constructs

| ETSI GS NFV-SOL 001 (the present document) | SOL APIs |  |  |
| --- | --- | --- | --- |
| Type and Property or entity name | Type and attribute name | Data model | Operation (see note 1) |
| tosca.nodes.nfv.VNF -> descriptor_id | (Identifier) vnfdId | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfInfoModificationRequest VnfInfoModifications | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query/Read VNF Package Info (see SOL003 and SOL005) |
|  |  | VnfPackageOnboardingNotification | Notify about VNF Package (see SOL003 and SOL005) |
|  |  | VnfPackageChangeNotification | Notify about VNF Package (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | UpdateNsRequest -> InstantiateVnfData | Update NS (see SOL005) |
|  |  | NsLcmOperationOccurrenceNotification -> AffectedVnf | Notify about NS LCM (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> provider | (String) vnfProvider | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query/Read VNF Package Info (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter -> vnfProductsFromProviders | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter -> vnfProductsFromProviders | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> product_name | (String) vnfProductName | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query/Read VNF Package Info (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> software_version | (Version) vnfSoftwareVersion | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query/Read VNF Package Info (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts -> versions | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts -> versions | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> descriptor_version | (Version) vnfdVersion | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfInfoModifications | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | VnfPkgInfo | Create VNF package (see SOL005) Query/Read VNF Package Info (see SOL003 and SOL005) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | PkgmSubscriptionRequest -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts -> versions | Subscription about VNF Package (see SOL003 and SOL005) |
|  |  | PkgmSubscription -> PkgmNotificationsFilter -> vnfProductsFromProviders -> vnfProducts -> versions | Subscription about VNF Package (see SOL003 and SOL005) |
| tosca.nodes.nfv.VNF -> configurable_properties | (KeyValuePairs) vnfConfigurableProperties | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | VnfInfoModificationRequest | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfInfoModifications |  |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | ChangeCurrentVnfPkgRequest | Change current VNF package (see SOL002 and SOL003) |
| tosca.nodes.nfv.VNF ->flavour_id | (IdentifierInVnfd) flavourId | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | InstantiateVnfRequest | Instantiate VNF (see SOL002 and SOL003) |
|  |  | GrantRequest | Grant Lifecycle Operation (see SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) vnfFlavourId | UpdateNsRequest -> instantiateVnfData | Update NS (see SOL005) |
|  | (IdentifierInVnfd) newFlavourId | ChangeVnfFlavourRequest | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | UpdateNsRequest -> ChangeVnfFlavourData | Update NS (see SOL005) |
| tosca.policies.nfv.ScalingAspects ->aspects[key] | (IdentifierInVnfd) aspectId | VnfInstance -> instantiatedVnfInfo -> ScaleInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | ScaleVnfRequest | Scale VNF (see SOL002 and SOL003) |
|  |  | ScaleVnfToLevelRequest -> ScaleInfo | Scale VNF to level (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> ScaleInfo | Query NS (see SOL005) |
| tosca.datatypes.nfv.ScaleInfo ->scale_level | (Integer) scaleLevel | VnfInstance -> instantiatedVnfInfo -> ScaleInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | ScaleVnfToLevelRequest -> ScaleInfo | Scale VNF to level (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> ScaleInfo | Query NS (see SOL005) |
| Node template name of type tosca.nodes.nfv.VnfExtCp | (IdentifierInVnfd) cpdId | VnfInstance -> instantiatedVnfInfo -> VnfExtCpInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | InstantiateVnfRequest -> ExtVirtualLinkData -> VnfExtCpData | Instantiate VNF (see SOL002 and SOL003) |
|  |  | ChangeVnfFlavourRequest -> ExtVirtualLinkData -> VnfExtCpData | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | ChangeExtVnfConnectivityRequest -> ExtVirtualLinkData -> VnfExtCpData | Change External VNF Connectivity (see SOL002 and SOL003) |
|  |  | Grant ->ExtVirtualLinkData ->VnfExtCpData | Grant Lifecycle Operation (see SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> VnfExtCpInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) resourceTemplateId | GrantRequest -> ResourceDefinition | Grant Lifecycle Operation (see SOL003) |
| Node template name of type tosca.nodes.nfv.VnfVirtualLink | (IdentifierInVnfd) vnfVirtualLinkDescId | VnfInstance -> instantiatedVnfInfo -> ExtManagedVirtualLinkInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfInstance -> instantiatedVnfInfo -> VnfVirtualLinkResourceInfo | Query VNF (see SOL003) |
|  |  | InstantiateVnfRequest -> ExtManagedVirtualLinkData | Instantiate VNF (see SOL002 and SOL003) |
|  |  | ChangeVnfFlavourRequest -> ExtManagedVirtualLinkData | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> resourceChanges -> AffectedVirtualLink | Get Operation Status (see SOL002 and SOL003) |
|  |  | Grant -> ExtManagedVirtualLinkData | Grant Lifecycle Operation (see SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> ExtManagedVirtualLinkInfo | Query NS (see SOL005) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> VnfVirtualLinkResourceInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) vnfVirtualLinkDescId | UpdateNsRequest -> InstantiateVnfData -> ExtManagedVirtualLinkData | Update NS (see SOL005) |
| tosca.datatypes.nfv.VnfMonitoringParameter | (IdentifierInVnfd) id | VnfInstance -> instantiatedVnfInfo -> MonitoringParameter | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> MonitoringParameter | Query NS (see SOL005) |
| tosca.datatypes.nfv.VnfMonitoringParameter ->name | (String) name | VnfInstance -> instantiatedVnfInfo -> MonitoringParameter | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> MonitoringParameter | Query NS (see SOL005) |
| tosca.nodes.nfv.VNF ->localization_languages | (String) localizationLanguage | VnfInstance -> instantiatedVnfInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo | Query NS (see SOL005) |
| Node template name of type tosca.nodes.nfv.Vdu.Compute | (IdentifierInVnfd)vduId | VnfInstance -> instantiatedVnfInfo -> VnfcResourceInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfInstance -> instantiatedVnfInfo -> VnfcInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> resourceChanges -> AffectedVnfc | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> AffectedVnfc | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> VnfcResourceInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) vnfdVirtualComputeDescId | Grant -> vimAssets -> VimConputeResourceFlavour | Grant Lifecycle Operation (see SOL003) |
| Node template name of type tosca.nodes.nfv.VduCp | (IdentifierInVnfd) cpdId | VnfInstance -> instantiatedVnfInfo -> VnfcResourceInfo -> vnfcCpInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfInstance -> instantiatedVnfInfo -> VnfExtCpInfo See note 3. | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | InstantiateVnfRequest -> ExtVirtualLinkData -> VnfExtCpData See note 3. | Instantiate VNF (see SOL002 and SOL003) |
|  |  | ChangeVnfFlavourRequest -> ExtVirtualLinkData -> VnfExtCpData See note 3. | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | ChangeExtVnfConnectivityRequest -> ExtVirtualLinkData -> VnfExtCpData See note 3. | Change External VNF Connectivity (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> VnfExtCpInfo See note 3. | Query NS (see SOL005) |
|  | (IdentifierInVnfd) resourceTemplateId See note 3. | GrantRequest -> ResourceDefinition See note 3. | Grant Lifecycle Operation (see SOL003) |
| Node template name of type tosca.nodes.nfv.Vdu.VirtualBlockStorage Node template name of type tosca.nodes.nfv.Vdu.VirtualObjectStorage Node template name of type tosca.nodes.nfv.Vdu.VirtualFileStorage | (IdentifierInVnfd) virtualStorageDescId | VnfInstance -> instantiatedVnfInfo -> VirtualStorageResourceInfo | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | VnfLcmOpOcc -> resourceChanges -> AffectedVirtualStorage | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> AffectedVirtualStorage | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance -> instantiatedVnfInfo -> VirtualStorageResourceInfo | Query NS (see SOL005) |
|  | (IdentifierInVnfd) resourceTemplateId | GrantRequest -> ResourceDefinition | Grant Lifecycle Operation (see SOL003) |
| tosca.datatypes.nfv.VnfInfoModifiableAttributes ->metadata | (KeyValuePairs) metadata | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | CreateVnfRequest | Create VNF Identifier (see SOL002 and SOL003) |
|  |  | VnfInfoModificationRequest | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfInfoModifications |  |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
| tosca.datatypes.nfv.VnfInfoModifiableAttributes ->extensions | (KeyValuePairs) extensions | VnfInstance | Create VNF Identifier (see SOL002 and SOL003) Query VNF (see SOL002 and SOL003) |
|  |  | NsInstance -> VnfInstance | Query NS (see SOL005) |
|  |  | InstantiateVnfRequest | Instantiate VNF (see SOL002 and SOL003) |
|  |  | VnfInfoModificationRequest | Modify VNF Information (see SOL002 and SOL003) |
|  |  | VnfInfoModifications |  |
|  |  | VnfLcmOpOcc -> VnfInfoModifications | Get Operation Status (see SOL002 and SOL003) |
|  |  | VnfLcmOperationOccurrenceNotification -> VnfInfoModifications | Notify about VNF LCM (see SOL002 and SOL003) |
|  |  | ChangeCurrentVnfPkgRequest | Change current VNF package (see SOL002 and SOL003) |
| tosca.policies.nfv.InstantiationLevel ->levels[key] | (IdentifierInVnfd) instantiationLevelId | InstantiateVnfRequest | Instantiate VNF (see SOL002 and SOL003) |
|  |  | ScaleVnfToLevelRequest | Scale VNF to level (see SOL002 and SOL003) |
|  |  | ChangeVnfFlavourRequest | Change VNF Flavour (see SOL002 and SOL003) |
|  |  | GrantRequest | Grant Lifecycle Operation (see SOL003) |
|  |  | UpdateNsRequest -> changeVnfFlavourData See note 4. | Update NS (see SOL005) |
|  | (IdentifierInVnfd) vnfInstantiationLevelId | UpdateNsRequest -> InstantiateVnfData See note 4. | Update NS (see SOL005) |
|  |  | ScaleNsRequest -> ScaleVnfData -> ScaleToLevelData | Scale NS (see SOL005) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the instantiate operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | InstantiateVnfRequest | Instantiate VNF (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the scale operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | ScaleVnfRequest | Scale VNF (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the 'scale to level' operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | ScaleVnfToLevelRequest | Scale VNF to level (see [i.4] and [i.9])l |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the 'change vnf flavour operation' of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | ChangeVnfFlavourRequest | Change VNF Flavour (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the operate operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | OperateVnfRequest | Operate VNF (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the heal operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | HealVnfRequest | Heal VNF (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the 'change external vnf connectivity' operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | ChangeExtVnfConnectivityRequest | Change External VNF Connectivity (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the terminate operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | TerminateVnfRequest | Terminate VNF (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the create snapshot operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | CreateVnfSnapshotRequest | CreateVNFSnapshot (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the revert to snapshot operation of the Vnflcm interface See note 2 | (KeyValuePairs) additionalParams | RevertToVnfSnapshotRequest | RevertToVNFSnapshot (see SOL002 and SOL003) |
| Properties of tosca.datatypes.nfv.VnfOperationAdditionalParameters in the inputs of the Change current VNF operation of the Vnflcm or ChangeCurrentVnfPackage interface. See note 2 | (KeyValuePairs) additionalParams | ChangeCurrentVnfPkgRequest | Change current VNF package (see SOL002 and SOL003) |
| name of type tosca.nodes.nfv.Vdu.Compute Node template name of type tosca.nodes.nfv.Vdu.VirtualBlockStorage | (IdentifierInVnfd) id | VnfPkgInfo -> VnfPackageSoftwareImageInfo | Create VNF Package Info (see SOL005) Query VNF Package Info (see SOL005) |
| Name of attribute for VNF indicator in VNF node | (IdentifierInVnfd) Id | VnfIndicator | Get indicator value (see SOL002 and SOL003) |
|  |  | VnfIndicatorValueChangeNotification | Notify about VNF indicator value change (see SOL002 and SOL003). |
| Name of attribute for VNF indicator in VNF | (String) Name | VnfIndicator | Get indicator value (see SOL002 and SOL003) |
|  | (IdentifierInVnfd) vnfIndicatorId | VnfIndicatorValueChangeNotification | Notify about VNF indicator value change (see SOL002 and SOL003). |
| Value of attribute for VNF indicator in VNF node | (Object) value | VnfIndicator | Get indicator value (see SOL002 and SOL003) |
|  |  | VnfIndicatorValueChangeNotification | Notify about VNF indicator value change (see SOL002 and SOL003). |
| Node template name of type tosca.nodes.nfv.Vdu.Compute Node template name of type tosca.nodes.nfv.Vdu.VirtualBlockStorage | (IdentifierInVnfd) vnfdSoftwareImageId | Grant -> vimAssets -> VimSoftwareImage | Grant Lifecycle Operation (see SOL003) |
| tosca.datatypes.nfv.VnfPackageChangeSelector -> destination_descriptor_id | (Identifier) vnfdId | ChangeCurrentVnfPkgRequest | Change current VNF package (see SOL002 and SOL003) |
| NOTE 1: The entry "SOL002" in the "Operation" column refers to ETSI GS NFV-SOL 002 [i.4], "SOL003" refers to ETSI GS NFV-SOL 003 [i.9] and "SOL005" refers to ETSI GS NFV-SOL 005 [i.10]. NOTE 2: This is an empty base type to be extended to a VNF specific type per LCM operation. The extended VNF specific and LCM operation specific type is the one that actually maps to the additionalParams in the API. NOTE 3: Only when VduCp is re-exposed as VnfExtCp. NOTE 4: Additional mappings corresponding to input parameter sets of VNF LCM operations in UpdateNS requests may be added in future versions of the present document. |  |  |  |

# D.3	NSD-related constructs

Table D.3-1 provides the mapping between the properties of TOSCA types related to the NSD and API attributes, which include: resource or notification data type (and referenced structured data type when available), attribute name and type in the resource or notification data type, and the interface operation in which the data type is used.

NOTE:	In the "Data model" column of the table, an arrow "->" indicates the navigation through the resource, notification and referenced structured data types.

Table D.3-1: Mapping of API attributes and TOSCA constructs

| ETSI GS NFV-SOL001 | SOL APIs |  |  |  |
| --- | --- | --- | --- | --- |
| Type and Property or entity name | Type and attribute name | Data model | Operation (see note 1) |
| tosca.nodes.nfv.NS -> descriptor_id | (Identifier) nsdIds | LccnSubscriptionRequest -> LifecycleChangeNotificationsFilter -> NsInstanceSubscriptionFilter | Subscription to NS LCM notifications (see SOL005) |
|  | (Identifier) nsdId | NsdInfo | Create NSD Update NSD Query/read NSD (see SOL005) |
|  |  | NsdOnboardingNotification | Notification about NSD management (see SOL005) |
|  |  | NsdOnboardingFailureNotification |  |
|  |  | NsdChangeNotification |  |
|  |  | NsdDeletionNotification |  |
|  |  | NsdmSubscriptionRequest -> NsdmNotificationsFilter | Subscription about NSD management (see SOL005) |
|  |  | NsdmSubscription -> NsdmNotificationsFilter |  |
|  |  | CreateNsRequest | Create NS (see SOL005) |
|  |  | NsInstance | Create NS Query NS Delete NS (see SOL005) |
|  |  | NsLcmOpOcc -> AffectedNs | Query/read about NS LCM operation occurrence (see SOL005) |
|  |  | NsLcmOperationOccurrenceNotification -> AffectedNs | Notification about NS LCM (see SOL005) |
|  | (Identifier) newNsdId | UpdateNsRequest -> AssocNewNsdVersionData | Update NS (see SOL005) |
| tosca.nodes.nfv.NS -> name | (String) nsdName | NsdInfo | Create NSD Update NSD Query/read NSD (see SOL005) |
|  |  | NsdmSubscriptionRequest -> NsdmNotificationsFilter | Subscription about NSD management (see SOL005) |
|  |  | NsdmSubscription -> NsdmNotificationsFilter |  |
| tosca.nodes.nfv.NS -> version | (Version) nsdVersion | NsdInfo | Create NSD Update NSD Query/read NSD (see SOL005) |
|  |  | NsdmSubscriptionRequest -> NsdmNotificationsFilter | Subscription about NSD management (see SOL005) |
|  |  | NsdmSubscription -> NsdmNotificationsFilter |  |
| tosca.nodes.nfv.NS -> designer | (String) nsdDesigner | NsdInfo | Create NSD Update NSD Query/read NSD (see SOL005) |
|  |  | NsdmSubscriptionRequest -> NsdmNotificationsFilter | Subscription about NSD management (see SOL005) |
|  |  | NsdmSubscription -> NsdmNotificationsFilter |  |
| tosca.nodes.nfv.NS -> invariant_id | (Identifier) nsdInvariantId | NsdInfo | Create NSD Update NSD Query/read NSD (see SOL005) |
|  |  | NsdmSubscriptionRequest -> NsdmNotificationsFilter | Subscription about NSD management (see SOL005) |
|  |  | NsdmSubscription -> NsdmNotificationsFilter |  |
| tosca.nodes.nfv.NS -> flavour_id | (IdentifierInNsd) flavourId | NsInstance | Create NS Query NS Delete NS (see SOL005) |
|  | (IdentifierInNsd) nsFlavourId | InstantiateNsRequest | Instantiate NS (see SOL005) |
|  | (IdentifierInNsd) newNsFlavourId | UpdateNsRequest -> ChangeNsFlavourData | Update NS (see SOL005) |
| Node template name of type tosca.nodes.nfv.NsVirtualLink | (IdentifierInNsd) nsVirtualLinkDescId | NsLcmOperationOccurrenceNotification -> AffectedVirtualLink | Notification about NS LCM (see SOL005) |
|  |  | NsInstance -> NsVirtualLinkInfo | Create NS Query NS Delete NS (see SOL005) |
| (No related property in type "tosca.datatype.nfv.NsVlProfile") See note 2 | (IdentifierInNsd) nsVirtualLinkProfileId | NsInstance -> NsVirtualLinkInfo | Create NS Query NS Delete NS (see SOL005) |
|  | (IdentifierInNsd) vlProfileId | NsLcmOperationOccurrenceNotification -> AffectedVirtualLink | Notification about NS LCM (see SOL005) |
| See note 2 | (IdentifierInNsd) nsInstantiationLevelId | InstantiateNsRequest | Instantiate NS (see SOL005) |
|  | (IdentifierInNsd) instantiationLevelId | UpdateNsRequest -> ChangeNsFlavourData | Update NS (see SOL005) |
|  | (IdentifierInNsd) nsInstantiationLevel | ScaleNsRequest -> ScaleNsData -> ScaleNsToLevelData | Scale NS (see SOL005) |
| Node template name in NSD of type derived from tosca.nodes.nfv.NS | (IdentifierInNsd) nsProfileId | UpdateNsRequest -> NestedNsInstanceData | Update NS (see SOL005) |
|  |  | InstantiateNsRequest -> NestedNsInstanceData | Instantiate NS (see SOL005) |
|  |  | InstantiateNsRequest -> ParamsForNestedNs |  |
| Node template name in NSD of type derived from tosca.nodes.nfv.VNF | (IdentifierInNsd) vnfProfileId | NsLcmOpOcc -> AffectedVnf | Query/read information about NS LCM (see SOL005) |
|  |  | NsLcmOperationOccurrenceNotification -> AffectedVnf | Notification about NS LCM (see SOL005) |
|  |  | NsInstance -> AffinityOrAntiAffinityRule | Create NS Query NS Delete NS (see SOL005) |
|  |  | InstantiateNsRequest -> AffinityOrAntiAffinityRule | Instantiate NS (see SOL005) |
|  |  | InstantiateNsRequest -> ParamsForVnf |  |
|  |  | InstantiateNsRequest -> VnfLocationConstraint |  |
|  |  | InstantiateNsRequest -> VnfInstanceData |  |
|  |  | ScaleNsRequest -> ScaleNsData -> VnfLocationConstraint | Scale NS (see SOL005) |
|  |  | ScaleNsRequest -> ScaleNsData -> VnfInstanceData |  |
|  |  | UpdateNsRequest -> VnfInstanceData | Update NS (see SOL005) |
| tosca.nodes.nfv.PNF -> descriptor_id | (Identifier) pnfdIds | LccnSubscriptionRequest -> LifecycleChangeNotificationsFilter -> NsInstanceSubscriptionFilter | Subscription to NS LCM (see SOL005) |
|  | (Identifier) pnfdId | PnfdOnboardingNotification | Notification about NSD management (see SOL005) |
|  |  | PnfdOnboardingFailureNotification |  |
|  |  | PnfdDeletionNotification |  |
|  |  | NsLcmOpOcc -> AffectedPnf | Query/read information about NS LCM (see SOL005) |
|  |  | NsLcmOperationOccurrenceNotification -> AffectedPnf | Notification about NS LCM (see SOL005) |
|  |  | NsInstance -> PnfInfo | Create NS Query NS Delete NS (see SOL005) |
|  |  | InstantiateNsRequest -> AddPnfData | Instantiate NS (see SOL005) |
|  |  | UpdateNsRequest -> AddPnfData | Update NS (see SOL005) |
| tosca.nodes.nfv.PNF -> name | (String) pnfdName | NsInstance -> PnfInfo | Create NS Query NS Delete NS (see SOL005) |
| tosca.nodes.nfv.PNF -> version | (Version) pnfdVersion | NsInstance -> PnfInfo | Create NS Query NS Delete NS (see SOL005) |
| tosca.nodes.nfv.PNF -> provider | (String) pnfdProvider | NsInstance -> PnfInfo | Create NS Query NS Delete NS (see SOL005) |
| tosca.nodes.nfv.PNF -> descriptor_invariant_id | (Identifier) pnfdInvariantId | NsInstance -> PnfInfo | Create NS Query NS Delete NS (see SOL005) |
| Node template name in NSD of type derived from tosca.nodes.nfv.PNF | (IdentifierInNsd) pnfProfileId | NsLcmOpOcc -> AffectedPnf | Query/read information about NS LCM Fail NS LCM (see SOL005) |
|  |  | NsLcmOperationOccurrenceNotification -> AffectedPnf | Notification about NS LCM (see SOL005) |
|  |  | NsInstance -> PnfInfo | Create NS Query NS Delete NS (see SOL005) |
|  |  | InstantiateNsRequest -> AddPnfData | Instantiate NS (see SOL005) |
|  |  | UpdateNsRequest -> AddPnfData | Update NS (see SOL005) |
| Node template name of type tosca.nodes.nfv.Cp | (IdentifierInNsd) cpdId | InstantiateNsRequest -> AddPnfData -> PnfExtCpData | Instantiate NS (see SOL005) |
|  |  | UpdateNsRequest -> AddPnfData -> PnfExtCpData | Update NS (see SOL005) |
|  |  | UpdateNsRequest -> ModifyPnfData -> PnfExtCpData |  |
|  |  | NsInstance -> PnfInfo -> PnfExtCpInfo | Create NS Query NS Delete NS (see SOL005) |
| Group template name of type tosca.groups.nfv.VNFFG | (IdentifierInNsd) vnffgdId | NsLcmOpOcc -> AffectedVnffg | Query/read information about NS LCM Fail NS LCM (see SOL005) |
|  |  | NsLcmOperationOccurrenceNotification -> AffectedVnffg | Notification about NS LCM (see SOL005) |
|  |  | UpdateNsRequest -> AddVnffgData | Update NS (see SOL005) |
|  |  | UpdateNsRequest -> UpdateVnffgData -> VnffgInfo |  |
| See note 2 | (IdentifierInNsd) aspectId | ScaleNsRequest -> ScaleNsData -> ScaleNsByStepsData | Scale NS (see SOL005) |
|  | (IdentifierInNsd) nsScalingAspectId | ScaleNsRequest -> ScaleNsData -> ScaleNsToLevelData -> NsScaleInfo |  |
| Name of the policy of type tosca.policies.nfv.NsMonitoring | (IdentifierInNsd) id | NsInstance -> NsMonitoringParameter | Create NS Query NS Delete NS (see SOL005) |
| tosca.policies.nfv.NsMonitoring -> ns_monitoring_parameters -> name | (String) name | NsInstance -> NsMonitoringParameter | Create NS Query NS Delete NS (see SOL005) |
| See note 2 | (Identifier) vnfPkgIds | NsdInfo | Create NSD Update NSD Query/read NSD (see SOL005) |
|  |  | NsdmSubscriptionRequest -> NsdmNotificationsFilter | Subscription about NSD management (see SOL005) |
|  |  | NsdmSubscription -> NsdmNotificationsFilter |  |
| See note 2 | (IdentifierInNsd) healScript | HealNsRequest -> HealNsData | Heal NS (see SOL005) |
| See note 2 | (IdentifierInNsd) aspectId | ScaleNsRequest -> ScaleNsData -> ScaleNsByStepsData | Scale NS (see SOL005) |
|  | (IdentifierInNsd) nsScalingAspectId | ScaleNsRequest -> ScaleNsData -> ScaleNsToLevelData -> NsScaleInfo |  |
| Node template name of type tosca.nodes.nfv.Sap | (IdentifierInNsd) sapdId | NsInstance -> SapInfo | Query NS (see SOL005) |
|  |  | InstantiateNsRequest -> SapData | Instantiate NS (see SOL005) |
|  |  | UpdateNsRequest -> SapData | Update NS (see SOL005) |
|  |  | NsLcmOpOcc -> AffectedSap | Query/read information about NS LCM operation occurrence (see SOL005) |
|  |  | NsLcmOperationOccurrenceNotification -> AffectedSap | Notification about NS LCM (see SOL005) |
| NOTE 1: The entry "SOL005" refers to ETSI GS NFV-SOL 005 [i.10]. NOTE 2: The corresponding TOSCA construct is not included in the present document, the mapping may be updated in future versions of the present document. |  |  |  |  |

Annex E (informative):
TOSCA Imperative workflows

# E.1	Purpose

This annex specifies TOSCA Imperative workflows for the NSD and the VNFD by using the grammar defined in TOSCA Simple Profile-YAML-v1.3 [20].

# E.2	TOSCA Imperative workflows for the NSD

## E.2.1	Introduction

TOSCA Imperative workflows based on TOSCA-Simple-Profile-YAML-v1.3 [20] may be used by the NFVO to fulfil the NS LCM operations described in ETSI GS NFV-IFA 013 [i.8]. TOSCA Imperative workflows provide an additional method for implementation of LCM operations in the Nslcm interface defined in clause 7.7.1.1 of the present document.

NOTE:	Even if TOSCA Imperative workflows is described in the NSD, the NFVO will still process the NSD with Nslcm operations as defined in clause 7.7.1.1. Since this is an additional method for implementation of NS LCM operations, execution of workflows instead of NS LCM operations is optional and up to the NFVO implementation.

TOSCA Imperative workflows defined in the NSD describe procedures for the NFVO to manage the lifecycle of network services.

Workflows are comprised of steps associated with the NS LCM operations and additional steps that are preamble and postamble to the execution of the former steps. The name of the preamble and postamble steps is constructed according to the following pattern:

< NS_LCM_base_operation_workflow_name>_start_<step_name> for preamble steps

< NS_LCM_base_operation_workflow_name>_end_<step_name> for postamble steps

Preamble steps are specified before the execution of workflow steps. Postamble steps are specified after the execution of workflow steps.

External and internal stimuli described in clause 7.7.1.4 of the present document, are mapped to workflows as below:

External stimuli are mapped to TOSCA Imperative workflows, i.e. <NS_LCM_base_operation_workflow_name>

Internal stimuli are mapped to preamble and postamble steps of the workflow

## E.2.2	Definition of an NS workflow

The syntax of TOSCA Imperative workflows for LCM operations on the NS has the following definition:

workflows: description: TOSCA Imperative workflows corresponding to NS LCM operations defined in ETSI GS NFV-IFA 013. instantiate: description: This workflow is invoked upon receipt of an Instantiate NS request # inputs: steps: instantiate_start_<step_name>: # Invoked before steps for instantiate LCM operation # steps for instantiate workflow instantiate_end_<step_name> : # Invoked after steps for instantiate LCM operation terminate: description: This workflow is invoked upon receipt of Terminate NS request # inputs: steps: terminate_start_<step_name> : # Invoked before steps for terminate LCM operation # steps for terminate workflow terminate_end_<step_name>: # Invoked after steps for terminate LCM operation

## E.2.3	Examples

The following example template fragment, based on clause A.8 of the present document, illustrates the use of TOSCA Imperative workflows for NS LCM operations.

When the NFVO executes TOSCA Imperative workflows in the NSD, it uses standard APIs for LCM operations defined in the Or-Vnfm interface and delegates the task to VNFM; The VNFM in turn executes corresponding TOSCA Operations on the VNF, as explained in clause 6.7.1 of the present document.

NOTE 1:	The NSD consumer makes available all parameters from the message invoking the NS base LCM operation as inputs to the corresponding TOSCA workflows.  The additional parameters for NS base LCM operations are defined as workflow inputs.

NOTE 2:	It is out of scope of the present document to specify mapping of SOL003/SOL005 API execution results with the success and failure of workflows.

tosca_definitions_version: tosca_simple_yaml_1_3

description: NS TOSCA Imperative Workflows

imports:

- etsi_nfv_sol001_nsd_types.yaml  # all of NSD related TOSCA types as defined in ETSI GS NFV-SOL 001

data_types:

MyCompany.datatypes.nfv.NsInstantiateAdditionalParameters:

derived_from: tosca.datatypes.nfv.NsOperationAdditionalParameters

properties:

parameter_1:

type: string

required: true

default: value_1

parameter_2:

type: string

required: true

default: value_2

node_types:

tosca.example_NS:

derived_from: tosca.nodes.nfv.NS

properties:

descriptor_id:

type: string

constraints: [ equal: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177 ]

default: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer:

type: string

constraints: [ equal: MyCompany ]

default: MyCompany

name:

type: string

constraints: [ equal: ExampleService ]

default: ExampleService

version:

type: string

constraints: [ equal: '1.0' ]

default: '1.0'

invariant_id:

type: string

constraints: [ equal: 1111-2222-aaaa-bbbb ]

default: 1111-2222-aaaa-bbbb

flavour_id:

type: string

constraints: [ equal: simple ]

default: simple

topology_template:

substitution_mappings:

node_type: tosca.example_NS

requirements:

virtual_link: [ VNF_2, virtual_link_2 ] # the External connection point of

# VNF_2 is exposed as the Sap

node_templates:

# This abstract node template enables the NSD author to use Nslcm scripts if he does #not use workflows.

my_service:

type: tosca.example_NS

properties:

descriptor_id: b1bb0ce7-ebca-4fa7-95ed-4840d70a1177

designer: MyCompany

name: ExampleService

version: '1.0'

invariant_id: 1111-2222-aaaa-bbbb

flavour_id: simple

interfaces:

Nslcm:

operations:

instantiate:

implementation: instantiate.workflow.yaml

terminate:

implementation: terminate.workflow.yaml

VNF_1:

type: tosca.nodes.nfv.example_VNF1

properties:

# no property assignments needed for required properties that have a default #value assigned in the node type definition, e.g. descriptor_id

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 2

max_number_of_instances: 6

requirements:

- virtual_link: NsVirtualLink_1

# Additional parameters input to be defined in the VNFD of VNF_1.

interfaces:

Vnflcm:

operations:

instantiate: . . .

terminate: . . .

VNF_2:

type: tosca.nodes.nfv.example_VNF2

properties:

flavour_id: simple

vnf_profile:

instantiation_level: level_1

min_number_of_instances: 1

max_number_of_instances: 3

requirements:

- virtual_link_1: NsVirtualLink_1

- virtual_link_2: # map to virtual_link requirement of the NS node

- dependency: VNF_1

# Additional parameters input to be defined in the VNFD of VNF_2.

interfaces:

Vnflcm:

operations:

instantiate: . . .

terminate: . . .

NsVirtualLink_1:

type: tosca.nodes.nfv.NsVirtualLink

properties:

connectivity_type:

layer_protocols: [ipv4]

flow_pattern: mesh

vl_profile:

max_bitrate_requirements:

root: 1000

min_bitrate_requirements:

root: 1000

workflows:

instantiate: #instantiate workflow

inputs:

additional_parameters:

type: tosca.datatypes.nfv.NsInstantiateAdditionalParameters

required: false

steps:

# preamble steps for instantiate operation. These correspond to preparatory #steps internal to the NFVO before instantiate operation

instantiate_start_step_1:

. .  .

on_success:

create_VNF_1

create_VNF_2

# steps for instantiate workflow

create_VNF_1:   # Step: Instantiate VNF_1

target: VNF_1

activities:

call_operation: tosca.interfaces.nfv.Vnflcm.instantiate

# invoking Vnflcm.instantiate operation enables NFVO to use internal implementation of #Vnflcm.instantiate operation which results in an ETSI GS NFV-SOL 003 API call towards #the VNFM to call VnfInstantiate operation. This enables VNFM to execute LCM operations #to deploy VNF_1

on_success:

instantiate_end_step_1

. . .

create_VNF_2:   # Step: Instantiate VNF_2

target: VNF_2

activities:

call_operation: tosca.interfaces.nfv.Vnflcm.instantiate

# invoking Vnflcm.instantiate operation enables NFVO to use internal implementation of #Vnflcm.instantiate operation which results in an ETSI GS NFV-SOL 003 API call towards #the VNFM to call VnfInstantiate operation. This enables VNFM to execute LCM operations #to deploy VNF_2

on_success:

instantiate_end_step_1

. . .

# postamble steps for instantiate operation. These correspond to closing steps #internal to the NFVO after instantiate operation.

instantiate_end_step_1:

. . .

terminate: #terminate workflow

steps:

# preamble steps for terminate operation. These correspond to preparatory #steps internal to the NFVO before terminate operation.

terminate_start_step_1:

. . .

on_success:

terminate_VNF_1

terminate_VNF_2

# steps for terminate workflow

terminate_VNF_1:   # Step: Terminate VNF_1

target: VNF_1

activities:

call_operation: tosca.interfaces.nfv.Vnflcm.terminate

# invoking Vnflcm.terminate operation enables NFVO to use internal implementation of #Vnflcm.terminate operation which results in an ETSI GS NFV-SOL 003 API call towards #the VNFM to call VnfTerminate operation. This enables VNFM to execute LCM operations #to terminate VNF_1

on_success:

terminate_end_step_1

. . .

terminate_VNF_2:   # Step: Terminate VNF_2

target: VNF_2

activities:

call_operation: tosca.interfaces.nfv.Vnflcm.terminate

# invoking Vnflcm.terminate operation enables NFVO to use internal implementation of #Vnflcm.terminate operation which results in an ETSI GS NFV-SOL 003 API call towards #the VNFM to call VnfTerminate operation. This enables VNFM to execute LCM operations #to terminate VNF_2

on_success:

terminate_end_step_1

. . .

# post amble steps for terminate operation. These correspond to closing steps #internal to the NFVO after terminate operation.

terminate_end_step_1:

. . .

NOTE 3:	As the on_success keyword is not used between steps inside the workflow for NS LCM base operation, the order of execution is decided by the NFVO.

NOTE 4:	As the on_failure keyword is not present inside the workflow for NS LCM base operation, the error handling is decided by the NFVO.

Annex F (informative):
Non-Backward Compatible Changes in the GS

# F.1	Introduction

This annex provides the list of non-backward compatible changes during the development of the present document.

A change introduced in version n of the present document is non-backward compatible if a service template written according to a previous version n-1 of the present document, i.e. a service template that has not been updated according to this change, is invalid with respect to this change for a NFVO/VNFM compliant to version n of the present document.

This annex focuses on compatibility from a descriptor view point. It does not evaluate whether a change made to the present document leads to non-backward compatible changes on the APIs referenced in annex D of the present document.

# F.2	Non-Backward Compatible changes between version 2.7.1 and 2.6.1

Table F.2-1 provides a list of non-backward compatible changes between version 2.6.1 and version 2.7.1 of the present document.

Table F.2-1: Non-backward compatible changes

| No. | Description | Clause |
| --- | --- | --- |
| 1 | nfvi_constraints type changed from list of string to map of string. | 6.8.3 |
| 2 | The definition of SecurityGroupRule is changed in version 2.7.1, a new policy type tosca.policies.nfv.Abstract.SecurityGroupRule is introduced in the definition, which SecurityGroupRule policy is derived from. | 6.10.13 |
| 3 | PlacementGroup used to be applied for both VNFD and NSD in version 2.6.1, but in version 2.7.1, it is only applied to VNFD. | 6.9 |
| 4 | AffinityRule, AntiAffinityRule used to be applied for both VNFD, NSD in version 2.6.1, but in version 2.7.1, it is only applied to VNFD. | 6.10.10 |
| 5 | SecurityGroupRule policy type used to be applied for VNFD, PNFD and NSD in version 2.6.1, but in version 2.7.1, it is only applied to VNFD. | 6.10.13 |
| 6 | boot_order type changed from list of string to Boolean. | 6.8.3 |
| 7 | boot_data type changed from string to BootData data type. | 6.8.3 |

# F.3	Non-Backward Compatible changes between version 2.8.1 and 3.3.1

Table F.3-1 provides a list of non-backward compatible changes between version 2.8.1 and version 3.3.1 of the present document.

Table F.3-1: Non-backward compatible changes

| No. | Description | Clause |
| --- | --- | --- |
| 1 | Multiple VNF deployment flavour design changed from using TOSCA v 1.2 grammar to TOSCA v 1.3 grammar: - in the top level service templates, removing the imports for lower level service templates. See note. - in the low level service templates, use of substitution filter instead of property mapping. | 6.11.2 |
| 2 | Multiple NS deployment flavour design changed from using TOSCA v 1.2 grammar to TOSCA v 1.3 grammar: - in the top level service templates, removing the imports for lower level service templates. See note. - in the low level service templates, use of substitution filter instead of property mapping. | 7.11.2 |
| NOTE: Instead the low level service templates are declared in the Other-Definitions of the TOSCA.meta file as specified in TOSCA-Simple-Profile-YAML-v1.3 [20]. |  |  |

# F.4	Non-Backward Compatible changes between version 3.3.1 and 3.5.1

Table F.4-1 provides a list of non-backward compatible changes between version 3.3.1 and version 3.5.1 of the present document.

Table F.4-1: Non-backward compatible changes

| No. | Description | Clause |
| --- | --- | --- |
| 1 | In VNF, Vdu.Compute and VnfVirtualLink node type definition, the type of monitoring_parameters changed from list to map. | 6.8.1, 6.8.3, 6.8.9 |
| 2 | In NsMonitoring policy definition, the type of ns_monitoring_parameters changed from list to map. | 7.10.4 |
| 3 | In VnfMonitoring policy definition, the type of vnf_monitoring_parameters changed from list to map. | 7.10.5 |

              - Annex G (informative):
Change History

| Date | Version | Information about changes |
| --- | --- | --- |
| 2016.05 | 0.0.1 | Implemented NFVSOL(16)000005r1_GS_NFV_SOL001_ToC |
| 2016.07 | 0.02 | Implemented NFVSOL(16)000026r1, NFVSOL(16)000027r1, NFVSOL(16)000028r2 |
| 2017.09 | 0.1.0 | Implemented NFVSOL(17)000539r3, NFVSOL(17)000540r3, NFVSOL(17)000542r2, NFVSOL(17)000544r1 |
| 2017.10 | 0.2.0 | Implemented NFVSOL(17)000543r3, NFVSOL(17)000566, NFVSOL(17)000567r1, NFVSOL(17)000568r1, NFVSOL(17)000569 Editorial modification for clause numbering and format |
| 2017.11 | 0.3.0 | Clean-up done by editHelp Implemented NFVSOL(17)000545r4, NFVSOL(17)000559r2, NFVSOL(17)000560r2, NFVSOL(17)000570r1, NFVSOL(17)000575r1, NFVSOL(17)000616r1, NFVSOL(17)000641 |
| 2017.12 | 0.4.0 | Implemented NFVSOL(17)000642r1, NFVSOL(17)000675r1, NFVSOL(17)000756 |
| 2018.01 | 0.5.0 | Implemented NFVSOL(17)000621r6, NFVSOL(17)000676r3, NFVSOL(17)000677r2, NFVSOL(17)000736r1, NFVSOL(18)000004r2 |
| 2018.03 | 0.6.0 | Implemented NFVSOL(18)000049, NFVSOL(18)000025r1, NFVSOL(18)00048R1, NFVSOL(18)00040, NFVSOL(18)00023r2, NFVSOL(18)00024r2, NFVSOL(18)00029r1, NFVSOL(18)00063r5, NFVSOL(18)000077, NFVSOL(18)000046r2, NFVSOL(18)000074r2, NFVSOL(17)0000611r5, NFVSOL(18)000041r1, NFVSOL(18)000038r3, NFVSOL(18)000028, NFVSOL(18)000044r2, NFVSOL(18)000055r2, NFVSOL(18)000045r1 |
| 2018.03 | 0.6.1 | Implemented NFVSOL(18)000052r1, NFVSOL(18)000094r5, NFVSOL(18)000043r3, NFVSOL(18)0000112r1, NFVSOL(18)0000117r2, NFVSOL(18)0000124r1, NFVSOL(18)0000129r1, NFVSOL(18)000042r2, NFVSOL(18)0000115r2 |
| 2018.04 | 0.6.2 | Implemented NFVSOL(18)000113r2, NFVSOL(18)000119r2, NFVSOL(18)000121, NFVSOL(18)000135, NFVSOL(18)000116r1, NFVSOL(18)000157, NFVSOL(18)000158r1 |
| 2018.05 | 0.6.3 | Implemented NFVSOL(18)000168, NFVSOL(18)000169r2, NFVSOL(18)000173, NFVSOL(18)000174r2 |
| 2018.05 | 0.7.0 | Implemented NFVSOL(18)000156r7, NFVSOL(18)000142r3, NFVSOL(18)000147, NFVSOL(18)000193r2, NFVSOL(18)000201, NFVSOL(18)000202, NFVSOL(18)000203r2, NFVSOL(18)000205r1, NFVSOL(18)000160r1, NFVSOL(18)000199r2, NFVSOL(18)000200r3, NFVSOL(18)000192r1, NFVSOL(18)000194r2, NFVSOL(18)000231r1, NFVSOL(18)000223r1, NFVSOL(18)000183r2 |
| 2018.06 | 0.8.0 | Implemented NFVSOL(18)000287, NFVSOL(18)00012r11, NFVSOL(18)000197r2, NFVSOL(18)000198r3, NFVSOL(18)000286, NFVSOL(18)000292, NFVSOL(18)000294r1, NFVSOL(18)000295r1, NFVSOL(18)000301r2, NFVSOL(18)000302r2, NFVSOL(18)000240r1, NFVSOL(18)000253r3, NFVSOL(18)000254r2, NFVSOL(18)000256r2 |
| 2018.06 | 0.9.0 | Implemented NFVSOL(18)000288r2 Adding etsi_nfv_sol001_vnfd_0_9_0_type.yaml and SOL001 Graphics v0_9_0.pptx in the draft GS zip package Editorial changes for all the TOSCA type definitions |
| 2018.08 | 0.10.0 | Implemented NFVSOL(18)000331r4, NFVSOL(18)000335r2, NFVSOL(18)000351r1, NFVSOL(18)000374r1, NFVSOL(18)000376, NFVSOL(18)000382, NFVSOL(18)000395r1, NFVSOL(18)000402r2, NFVSOL(18)000404, NFVSOL(18)000405r2, NFVSOL(18)000406r1, NFVSOL(18)000408r2, NFVSOL(18)000409r1, NFVSOL(18)000411r1, NFVSOL(18)000413, NFVSOL(18)000416r2, NFVSOL(18)000422r4, NFVSOL(18)000423r4, NFVSOL(18)000424r2, NFVSOL(18)000425r2, NFVSOL(18)000430r1, NFVSOL(18)000336r4, NFVSOL(18)000380, NFVSOL(18)000387r2, NFVSOL(18)000394r1, NFVSOL(18)000410r2, NFVSOL(18)000447r3, NFVSOL(18)000427, NFVSOL(18)000403r3, NFVSOL(18)000420r4, NFVSOL(18)000393r1, NFVSOL(18)000415, NFVSOL(18)000398r1, NFVSOL(18)000399r3, NFVSOL(18)000414r2, NFVSOL(18)000379r3, NFVSOL(18)000428r5, NFVSOL(18)000429r4, NFVSOL(18)000465r1, NFVSOL(18)000479, NFVSOL(18)000375r4, NFVSOL(18)000488r2, NFVSOL(18)000492r1 |
| 2018.09 | 0.11.0 | Implemented NFVSOL(18)000486r7, NFVSOL(18)000495r1, NFVSOL(18)000497r3, NFVSOL(18)000498, NFVSOL(18)000500r3, NFVSOL(18)000503r1, NFVSOL(18)000504r1, NFVSOL(18)000505r1, NFVSOL(18)000508, NFVSOL(18)000514r2, NFVSOL(18)000515r1, NFVSOL(18)000516r1, NFVSOL(18)000524r1, NFVSOL(18)000529r2, NFVSOL(18)000530r1, NFVSOL(18)000536r1, NFVSOL(18)000538r1, NFVSOL(18)000541r3, NFVSOL(18)000544r1，NFVSOL(18)000545r1 |
| 2018.10 | 0.12.0 | Implemented NFVSOL(18)000507r3, NFVSOL(18)000621r4, NFVSOL(18)000547r1, NFVSOL(18)000567r1, NFVSOL(18)000574, NFVSOL(18)000579, NFVSOL(18)000590r1 |
| 2018.11 | 0.13.0 | Implemented NFVSOL(18)000563r5, NFVSOL(18)000575r5, NFVSOL(18)000586r2, NFVSOL(18)000587r1, NFVSOL(18)000589r2, NFVSOL(18)000591, NFVSOL(18)000592, NFVSOL(18)000604r2, NFVSOL(18)000606r1, NFVSOL(18)000607r5, NFVSOL(18)000608, NFVSOL(18)000609, NFVSOL(18)000612, NFVSOL(18)000614r1, NFVSOL(18)000615, NFVSOL(18)000616r2, NFVSOL(18)000617r3, NFVSOL(18)000619r3, NFVSOL(18)000620r5, NFVSOL(18)000628r2, NFVSOL(18)000629r1, NFVSOL(18)000630, NFVSOL(18)000631r1, NFVSOL(18)000632, NFVSOL(18)000634, NFVSOL(18)000635, NFVSOL(18)000636r3, NFVSOL(18)000637, NFVSOL(18)000655r2, NFVSOL(18)000658r3, NFVSOL(18)000659, NFVSOL(18)000660, NFVSOL(18)000665r1, NFVSOL(18)000666, NFVSOL(18)000678r1, NFVSOL(18)000679, NFVSOL(18)000682, NFVSOL(18)000684r1, NFVSOL(18)000687 |
| 2019.03 | 2.5.2 | Implemented NFVSOL(19)000063r4, NFVSOL(19)000068r1, NFVSOL(19)000069r2, NFVSOL(19)000070r1, NFVSOL(19)00080, NFVSOL(19)000107r1, NFVSOL(19)000120r1, NFVSOL(19)000121, NFVSOL(19)00039r5, NFVSOL(19)00087r2, NFVSOL(19)00067r5, NFVSOL(19)00077, NFVSOL(19)00082r3, NFVSOL(19)00085, NFVSOL(19)00086, NFVSOL(19)000106r1 |
| 2019.03 | 2.5.3 | Implemented NFVSOL(19)000084r4, NFVSOL(19)0000101r2, NFVSOL(19)0000165, NFVSOL(19)0000166, NFVSOL(19)0000167r1, NFVSOL(19)0000170r4, NFVSOL(19)0000173, NFVSOL(19)0000163, NFVSOL(19)0000119 |
| 2019.03.22 | 2.5.4 | Editorial modification made by ETSI Secretariat allowing to structure the SOL repository on ETSI Forge in a future proof and maintainable way: Forge structure updated: "v2.6.1" tag created Yaml filenames updated: version numbers removed from filenames (still included in file header) Import statements updated: version number removed from imported filenames. Draft updated: Updated all references to yaml files Updated the forge URLs In B.1: removed the sentence explaining the meaning of x_y_z_ in the filename structure |
| 2019.04.23 | 2.5.5 | 2 comments were raised during the Remote Consensus approval: both requesting to implement the WG SOL approved Change Request in NFVSOL(19)000229r3 onto the final SOL001 draft (see these 2 comments in the RC report). The present version implements NFVSOL(19)000229r3: adding machine readable meta info inside the yaml file indicating the SOL001 release version to which they apply + other editorial changes |
| 2019.05 | 2.6.2 | Implemented NFVSOL(19)000162r1, NFVSOL(19)000194r2, NFVSOL(19)000222r2, NFVSOL(19)000241r2, NFVSOL(19)000242r2 |
| 2019.06 | 2.6.3 | Implemented NFVSOL(19)000160r9, NFVSOL(19)000239r3, NFVSOL(19)000248, NFVSOL(19)000262r2, NFVSOL(19)000263r3, NFVSOL(19)000268r3, NFVSOL(19)000269, NFVSOL(19)000270r3, NFVSOL(19)000303r1, NFVSOL(19)000307, NFVSOL(19)000279, NFVSOL(19)000280, NFVSOL(19)000296r3, NFVSOL(19)000301r1, NFVSOL(19)000305r1, NFVSOL(19)000338, NFVSOL(19)000340, NFVSOL(19)000342r1, NFVSOL(19)000344r2, NFVSOL(19)000345r4, NFVSOL(19)000266r7 |
| 2019.08 | 2.6.4 | Implemented NFVSOL(19)000325r8, NFVSOL(19)000346r6, NFVSOL(19)000347r2, NFVSOL(19)000380r1, NFVSOL(19)000383r1, NFVSOL(19)000384r1, NFVSOL(19)000385, NFVSOL(19)000389r1, NFVSOL(19)000428r2, NFVSOL(19)000449 |
| 2019.09 | 2.6.5 | Implemented NFVSOL(19)000386r8, NFVSOL(19)000408r4, NFVSOL(19)000451r8, NFVSOL(19)000559 |
| 2019.11 | 2.6.6 | Implemented NFVSOL(19)000727r1_SOL001Ed271_Miscellaneous_corrections, NFVSOL(19)000642r1_SOL001ed271_Annex_Mapping_table_for_SOL_API_NSD_related_cons, NFVSOL(19)000645r3_SOL001ed271_NBWC_issue_list_annex, NFVSOL(19)000700r1_SOL001ed271_updating_mapping_table_in_A_9, NFVSOL(19)000577r2_SOL001Ed271_-_Standards_Configurable_Properties, NFVSOL(19)000593_SOL001ed271_adding_introduction_in_clasue_9_1, NFVSOL(19)000595_SOL001ed271_resolving_requirement_occurannce_issue, NFVSOL(19)000597r3_SOL001ed271_deployment_flavour_related_CSAR_design_for_NSD, NFVSOL(19)000598r2_SOL001ed271_TOSCA_Imperative_workflows_NSD_Editor_s_Notes, NFVSOL(19)000599r2_SOL001ed271_TOSCA_Imperative_workflows_Example_Editors_Notes, NFVSOL(19)000607r2_SOL001ed271_VNFFG_clause_6_8_2_6_Editor_s_note_handling, NFVSOL(19)000608r2_SOL001ed271_VNFFG_clause_7_8_2_6_Editor_s_note_handling, NFVSOL(19)000609r2_SOL001ed271_VNFFG_clause_7_8_5_1_Editor_s_notes_handling, NFVSOL(19)000610_SOL001ed271_VNFFG_clause_7_8_6_1_Editor_s_note_handling, NFVSOL(19)000611r2_SOL001ed271_adding_TOSCA-Simple-Profile-yaml-v1_3_reference, NFVSOL(19)000616_SOL001ed271_ConnectivityType, NFVSOL(19)000617r3_SOL001ed271_nfviConstraint, NFVSOL(19)000633_SOL001ed271_VnfIndicators_editor_s_notes_resolution, NFVSOL(19)000636r2_SOL001ed271_NS_workflow_defintion_update, NFVSOL(19)000702_SOL001ed271_adding_reference_for_SwImageData, NFVSOL(19)000703r1_SOL001ed271_forwarding_behaviour_input_parameters, NFVSOL(19)000704r1_SOL001ed271_correction_on_NfpPositionElement_definition, NFVSOL(19)000705r1_SOL001ed271_correction_on_descriptor_id, NFVSOL(19)000741r1_SOL001ed271_NfpPositionElement_node_type_improvement, NFVSOL(19)000596r3_SOL001ed271_deployment_flavour_related_CSAR_design_for_VNFD, NFVSOL(19)000725r2_SOL001ed271_vnfd_common_yaml_file_for_v1_3, NFVSOL(19)000744r2_SOL001ed271_Clause_7_1_Introduction_improvement, NFVSOL(19)000566r9_SOL001ed271_adding_boot_data_type, NFVSOL(19)000674r1_SOL001Ed271_Monitoring_Parameters_-_Alignement_with_IFA027 |
| 2019.11 | 2.6.7 | Implemented NFVSOL(19)000780r3_SOL001ed271__VnfIndicator_id_Issue_1, NFVSOL(19)000784r1_SOL001ed271_solving_remaining_editor_s_notes, NFVSOL(19)000785r1_SOL001ed271_solving_remaining_monitoring_issues, NFVSOL(19)000789_SOL001ed271_adding_boot_data_in_NBWC_list_table, NFVSOL(19)000719r12_SOL001ed271_forwarding_capability_for_VNF_node_type |
| 2020.02 | 3.0.1 | Implemented NFVSOL(19)000710r3_SOL001ed331_deployment_flavour_related_CSAR_design_for_VNFD, NFVSOL(19)000792r1_SOL001ed331_FEAT10_Add_specification_for_Multi-Site_Connectivity_Services, NFVSOL(19)000799r2_SOL001ed331_FEAT05_Adding_priority_to_NSD |
| 2020.02 | 3.0.2 | Implemented: NFVSOL(19)000714r4_SOL001ed271_support_of_auto-scaling_with_use_of_VNF_indicator, NFVSOL(20)000018r2_SOL001ed331_deployment_flavour_related_CSAR_design_for_NSD, NFVSOL(20)000067r1_SOL001ed331_FEAT16_Adding_SAL_to_NSD |
| 2020.04 | 3.0.3 | Implemented: NFVSOL(20)000016r6_SOL001ed331_FEAT02_VnfPackageChange, NFVSOL(20)000040r6_SOL001ed331_Criteria_for_backward_compatibility_of_changes, NFVSOL(20)000117_SOL001ed331_Vnflcm_update_TOSCA_1_3_grammar, NFVSOL(20)000172r1_SOL001ed331_VNF_specific_datatypes_naming_rules, NFVSOL(20)000239r2_SOL001ed331_release_3_mirror_adding_NS_DF_design_principle_in_annex, NFVSOL(20)000242_SOL001ed331_updating_annex_C, NFVSOL(20)000285_SOL001ed331_VNF_specific_types_naming_rules rapporteur changes: editorial, changing "will" to "with" in some places in annex C.2 and C.3 editorial, removing extra space in some of the TOSCA type definitions |
| 2020.05 | 3.0.4 | Implemented: NFVSOL(20)000295r3_SOL001ed331_FEAT02_ChangeCurrentVnfPackage_interface, NFVSOL(20)000320r6_SOL001ed331_FEAT15_VNF_Snapshot, NFVSOL(20)000354r1_SOL001ed331_FEAT02_ChangeCurrentVnfPackage_AnnexA_D_mapping, NFVSOL(20)000327r1_SOL001ed331_rel3_mirror_corrections_of_specific_node_tpye, NFVSOL(20)000353_SOL001ed331_Vnflcm_interface_EN_cleanup, NFVSOL(20)000376r2_SOL001ed331_mirror_of_375_PNFD_geographic_coordinates_suppor, NFVSOL(20)000378r2_SOL001ed331_rel3_mirror_add_VNF_related_type_names_in_clause, NFVSOL(20)000380r1_SOL001ed331_rel3_mirror_add_NS_related_type_names_in_clause_7, NFVSOL(20)000382r1_SOL001ed331_rel3_mirror_clarification_of_using_VnfMonitoring, NFVSOL(20)000384_SOL001ed331_rel3_mirror_clarification_on_ip_address_type, NFVSOL(20)000386_SOL001ed331_rel3_mirror_clarification_on_VirtualLinkable_cap, NFVSOL(20)000194r3_SOL001ed331_release_3_mirror_example_corrections |
| 2020.06 | 3.0.5 | Implemented: NFVSOL(20)000267r3_SOL001ed331_VNF_node_type_definitions, NFVSOL(20)000355r2_SOL001ed331_examples_cleanup_tosca1_3_interfaces, NFVSOL(20)000356_SOL001ed331_example_cleanup_to_tosca_1_3, NFVSOL(20)000357_SOL001ed331_all_type_defintions_metadata_cleanup, NFVSOL(20)000394r2_SOL001ed331_clause6_11_cleanup_tosca1_3, NFVSOL(20)000395r4_SOL001ed331_sw_image_data_align_with_TOSCA_1_3, NFVSOL(20)000396r5_SOL001ed331_removing_reference_of_TOSCA_1_1_and_1_2, NFVSOL(20)000397r1_SOL001ed331_clause_7_11_align_tosca_1_3, NFVSOL(20)000399r1_SOL001ed331_clause_8_11_align_tosca_1_3, NFVSOL(20)000517r1_SOL001ed331_Rel-3_mirror-_fixing_optional_properties, NFVSOL(20)000527r4_SOL001ed331_update_annex_F, NFVSOL(20)000597_SOL001ed331_resolve_remaining_issues_for_TOSCA_reference, NFVSOL(20)000598r1_SOL001ed331_resolve_editor_note_for_package_change. Undo the implementation of NFVSOL(19)000799r2, supporting for FEAT5 will be removed from this version |
| 2020.09 | 3.3.2 | Implemented: NFVSOL(20)000387r3_SOL001ed341_adding_virtualLinkProtocolData_for_NsVirtualLink, NFVSOL(20)000617r7_SOL001ed341_Use_of_Credentials_datatype, NFVSOL(20)000618_SOL001ed341_VnfPackageChange_corrections, NFVSOL(20)000674_SOL001ed341_modifiable_attributes_example_correction, NFVSOL(20)000676r1_SOL001ed341_update_extension_rule |
| 2020.11 | 3.3.3 | Implemented: NFVSOL(20)000723r4_SOL001Ed341_Fixing_examples, NFVSOL(20)000359r5_SOL001ed331_support_of_trunk_port_topology, NFVSOL(20)000679r2_SOL001ed341_monitoring_parameter_identifier, NFVSOL(20)000716r2_SOL001Ed341_Clarification_on_node_type_definitions_in_an_NSD, NFVSOL(20)000737r1_SOL001Ed341_Ambiguous_use_of__may_not_, NFVSOL(20)000744r2_SOL001ed341_uniform_delta_correction, NFVSOL(20)000747r1_SOL001ed351_fix_TOSCA_YAML_version_reference, NFVSOL(20)000749r1_SOL001ed341_Adding_NS_scaling_aspects_instantiation_levels, NFVSOL(20)000752r6_SOL001ed351_support_using_VnfConfigurableProperties_for_boot_data, NFVSOL(20)000767_SOL001ed351_Removal_of_constraint_in_VnfIndicator_attribute |

# History

| Document history |  |  |
| --- | --- | --- |
| V3.3.1 | September 2020 | Publication |